3GPP TSG RAN WG1 #112		          R1-2301616

Athens, Greece, February 27th – March 3rd, 2023

Source: 	MediaTek Inc.

Title:	Discussion on SL operation on FR2

Agenda item:	9.4.3

Document for:	Discussion & Decision

## Introduction

Based on the SL evolution WID’s guidance as agreed in RAN#97 meeting [1], e.g., determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18, it has been extensively discussed in RAN#98 meeting with the following SL FR2 evolution WID updated, and the detailed contents can refer to the approved WID contribution [2]. According to the updated WID guidance, SL FR2 discussion only focus on the study purpose without specification work.

*** Unchanged text is omitted *** ### Objective of SI or Core part WI or Testing part WI *** …………….. *** - Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only. *** …………… ***

For the evaluation methodology part, it has been extensively discussed in the Q4 2022 and achieved the following agreements/conclusions in last meeting [3]:

Conclusion: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, indoor layout with cluster-based topology is up to companies. Further discussion on the evaluations assumptions for cluster-based topology is not expected. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor layout, do not support UE-to-UE 2D distance smaller than 10m. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor layout, in the pathloss model for UMi – Street Canyon in TR38.901, antenna height of base station (hBS) is replaced by antenna height of UE (hUT). Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, optionally support performance metric of UE satisfaction as section 7.2 in TR38.838 for XR traffic evaluation. Conclusion When reporting the simulation results for sidelink operation on FR2, companies should report the used beamwidth. Conclusion In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the UE antenna array configurations other than the one defined in Table 6.1.4-7 of TR37.885 are not precluded.

However, there still have some issues discussed in the last meeting and without consensus due to limited meeting time, e.g., whether to support SL relay case for SL FR2 evaluation. In this contribution, we will further discuss the remaining issues and provide some simulation results based on previous agreements.

## Discussion

### 2.1 Evaluation methodology related

For the Rel-18 SL evolution, the purpose of introducing the SL operation on FR2 is to increase the data rate efficiently for expending the applicability of NR SL to commercial use cases, e.g., XR based interactive service via SL communication, SL tethering, etc. Thus, the evaluation methodology for SL FR2 shall focus on the commercial use case.

Observation 1: The evaluation methodology for SL FR2 shall focus on the commercial use case.

Regarding to the commercial deploying scenario for SL FR2, in addition to consider the transmission between device to device only topology, another commercial use case that gNB serves relay UE device through Uu interface and relay UE forwards traffic to remote UE through SL FR2 should also be considered. The deployment topology is provided in Figure 1.

Figure 1 SL FR2 relay deployment

The relay UE device (e.g., CPE, smart phone) has higher antenna capability to achieve high data rate on Uu interface. The remote UE device with reduced antenna capability (e.g., XR glasses, wearable devices) connects to the relay device through SL FR2 with short distance transmission. Several benefits are observed in this deployment:

- FR2 massive pathloss is overcame by short distance transmission between relay UE and remote UE

- High spectrum efficiency is achieved on Uu interface when gNB serves the high-end relay UE instead of the reduced capability remote UE

- High data rate is achieved on PC5 interface with availability of larger frequency resource on FR2

- Larger serving coverage is achieved for remote UE to connect to network

- Less power is consumed by remote UE through short distance packet transmission instead of transmitting packet directly to the gNB

From our understanding, this is a beneficial deployment of SL FR2 in the network. With this deployment achieving high data rate, network is able to provide XR based interactive service to XR wearable device.

Observation 2: SL FR2 relay is a typical scenario for SL commercial use case to provide XR based service to low-end wearable (e.g., XR glasses) UEs.

Based on the above discussions and observation, the following proposal is suggested:

Proposal 1: Relay topology shall be defined for SL FR2 evaluation.

### 2.2 Simulation results

In the following evaluation results, the indoor layout topology agreed in RAN1 #110-b e-meeting is adopted. The unicast traffic and traffic model of FTP model3 with low BO is used in the analog beam number (coarse beam, fine beam) evaluation between TX UE and RX UE. Based on the agreements, a total of 20 SL pairs are dropped randomly on indoor topology, where the SL UE pairing threshold satisfies the maximum pairing distance < 10m. The other detailed evaluation configuration is summarized in Table 1.

Table 1. Summary of evaluation configurations for indoor scenario

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Subcarrier Spacing | 120 kHz |
| Bandwidth | 100 MHz |
| Deployment Scenario | Indoor Office as agreement |
| Mobility | 3 km/hr |
| UE Antenna Configuration | Smart phone UE M,N,P,Mg,Ng=(2,4,2,1,2) 4 port with dV,dH=(0.5,0.5)λ Wearable device M,N,P,Mg,Ng=(2,2,1,1,1) with dV,dH=(0.5,0.5)λ for 1 TX M,N,P,Mg,Ng=(2,2,2,1,1) with dV,dH=(0.5,0.5)λ for 2 RX The UE utilizes only 1 panel at a given moment. |
| UE Antenna element gain | 5 dBi |
| UE Power Limitation | 23dBm |
| UE Noise Figure | 10 dB |
| Traffic Model | Option 1: FTP Model 3 Option 2: XR traffic model |
| UE Receiver | MMSE-IRC |
| UE Pairing threshold | By maximum distance |
| Number of UE pairs | 4 pairs/20MHz For 100MHz BW, total of 20 SL pairs |

Different analog beam number configured at TX UE side and at RX UE side are evaluated. For the direction of UE analog beam steering, the following parameters are configured per panel:

Configure 1 : 1 Beam (Single antenna element is activated to form an omni-directional beam)

Configure 2 : 4 Beam

  - Azimuth (Horizontal) angle [-pi/2, pi/2], 2 beam

  - Zenith (Vertical) angle: [pi/4, 3*pi/4], 2 beam

Configure 3 : 8 Beam

  - Azimuth (Horizontal) angle [-3*pi/8, -pi/8, pi/8, 3*pi/8], 4 beam

  - Zenith (Vertical) angle: [pi/4, 3*pi/4], 2 beam

The UE selects one beam based on beam management result for data transmission. The UPT performance is provided in Figure 2.

Figure 2 The UPT performance evaluation by different configured beam number

The simulation result shows that finer TX beam and finer RX beam provides better UPT performance. UPT degradation is observed when TX UE or RX UE uses omni-directional beam for data transmission or reception.

Next, different SL pairs number with different analog beam number configured at TX UE side and at RX UE side are evaluated. The number of SL pairs number in the system is increased from 20 to 40 pairs. The UE selects one beam based on beam management result for data transmission. The UPT performance is provided in Figure 3.

Figure 3 The UPT performance evaluation by increasing SL pairs number

The simulation result shows that coarse TX beam and coarse RX beam suffers from UPT degradation when number of SL pairs in the system increases. Meanwhile, finer TX and RX beam transmission introduces less interference to the system which improves system capacity.

Based on the simulation, the following observation is achieved:

Observation 3: Finer TX beam and finer RX beam are necessary to improve UPT performance, where initial beam pairing and beam management procedure should be studied.

With TX UE uses fine beam transmission and RX UE uses fine beam reception, different beam management period configurations are evaluated. By configured beam management period, RX UE reports best beam pair within the configured 8 TX beam and 8 RX beam to TX UE for data transmission. The UPT performance is provided in Figure 4.

Figure 4 The UPT performance evaluation by different beam management period

The simulation result shows that in indoor office scenario where UE pairs have low velocity and short serving distance, the UPT degradation by extending beam management period is marginal. SL FR2 beam management is expected to adopt longer beam management period or aperiodic beam management to maintain the data beam pair after the initial beam pairing procedure is done.

Based on the simulation, the following observation is achieved:

Observation 4: Longer beam management period or aperiodic beam management to maintain data beam pair are required for SL FR2 beam management

## Conclusion

In this contribution, it further discusses the SL FR2 issues with following observations and proposals:

Observation 1: The evaluation methodology for SL FR2 shall focus on the commercial use case.

Observation 2: SL FR2 relay is a typical scenario for SL commercial use case to provide XR based service to low-end wearable (e.g., XR glasses) UEs.

Observation 3: Finer TX beam and finer RX beam are necessary to improve UPT performance, where initial beam pairing and beam management procedure should be studied.

Observation 4: Longer beam management period or aperiodic beam management to maintain data beam pair are required for SL FR2 beam management

Proposal 1: Relay topology shall be defined for SL FR2 evaluation

## Reference

- RP-221938, WID revision: NR sidelink evolution, OPPO, RAN#97-e, September 12 – 16, 2022.

- RP-222806, WID revision: NR sidelink evolution, OPPO, RAN#98-e, December 12 – 16, 2022.

- RAN1 Chairman’s Notes, RAN1 #111, November 2022.