3GPP TSG-RAN WG1 #112	R1-2301634

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:		9.5.1.3

Source:	MediaTek Inc.

Title:	Resource allocation for SL-PRS

Document for:		Discussion

# Introduction

During the study item, the agreements related to resource allocation are listed below,

Agreement With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility Agreement For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. Agreement Update the agreed TP into the conclusion section of the TR 38.859 as follows: For the solutions for sidelink positioning, - The following 2 operation scenarios are recommended for normative work - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning. - RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work. - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT - For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation is recommended for normative work. - For the support of the above methods the following measurements are recommended for normative work: - SL-PRS based Rx-Tx measurement - SL-PRS based RSTD measurement - SL-PRS based RSRP measurement - SL-PRS based RSRPP measurement - SL-PRS based RTOA measurement - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement - A new sidelink reference signal (SL-PRS) is recommended for normative work. - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point. - SCI can be used for reserving/indicating one or more SL-PRS resources - With regards to the power control for SL-PRS at least Open Loop PC is recommended for normative work. - Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation. - For resource allocation mechanism for SL-PRS in Scheme 2, a sensing based resource allocation, or a random resource selection, or both, should be introduced, where the legacy designs for UE autonomous resource allocation are used as a starting point. - With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work. - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work. Agreement A dedicated SL-PRS resource pool is (pre-)configured in the only SL BWP of a carrier. Agreement Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

In this contribution, we provide our view for the resource allocation design.

# Dedicated resource pool design

The resources in a resource pool are specified in both time and frequency domain. For dedicated resource pool for allocating SL-PRS, the following could be considered,

The basic unit in time domain for dedicated resource pool is a slot

A slot for dedicated resource pool contains the control region for PSCCH and the symbols for allocating SL-PRS

The SL-PRS allocation within a slot of dedicated resource pool could be both IFDMA and TDM based

The allocation in IFDMA manner could be due to the comb structure of SL-PRS. Then each UE could be allocated with a comb index

A resource with staggered structure has different RE offset in different symbol. The comb index could be based on first symbol for allocating a resource, since the RE offset pattern has a rule

The allocation in TDM manner within a slot could be due to the short duration for a SL-PRS resource

The short duration of a resource could be realized by partial staggering structure to reduce the symbol number for a resource

All the cast type of SL-PRS can be allocated within a same dedicated resource pool

If the dedicated resource pool transmits SL-PRS and PSCCH only, then the SL-PRS BW could be equal to that of the pool. otherwise, the resource utilization rate becomes lower and the way to improve the utilization rate is to reduce the BW of the pool.

Further, when SL-PRS BW is equal to that of the pool, the lowest PRB of the PSCCH associated to the SL-PRS transmission may not be the same as that of the SL-PRS.

Proposal 2-1: The basic unit in time domain for dedicated resource pool is a slot

Proposal 2-2: A slot for dedicated resource pool contains the control region for PSCCH and the symbols for allocating SL-PRS

Proposal 2-3: The SL-PRS allocation within a slot of dedicated resource pool could be both IFDMA and TDM based, which is RS structure based

Proposal 2-4: All the cast type of SL-PRS can be allocated within a same dedicated resource pool

Proposal 2-5: Since it is dedicated, the SL-PRS BW is expected to be equal to that of the dedicated resource pool. If not, the resource pool’s bandwidth could be reduced

# Resource allocation design

The resource allocation for SL-PRS transmission could consider the following methods,

  - A single SCI could be considered for SL-PRS allocation in dedicated resource pool

  - Let’s say UE A and UE B are performing the measurements for RTT. When the UE A transmits SL-PRS and reserves the time/frequency resources for future transmission, the UE A may also reserve time/frequency resources as the recommendation for the transmission of UE B

    - This method may reduce the latency, since UE A has performed sensing to reserve the resource for UE B’s transmission

      - The RTT measurement maybe critical to avoid accidental collision between UEs. Then the latency due to higher layer information exchange and sensing may need to be reduced

    - UE B could select the resource reserved by UE A, or select the resource based on the own sensing for the transmission

Proposal 3-1: A single SCI could be considered for SL-PRS allocation in dedicated resource pool

Proposal 3-2: When a UE reserves the resource for future transmission in SCI, the UE may also reserve the resource for the transmission of another UE

# Conclusion

Proposal 2-1: The basic unit in time domain for dedicated resource pool is a slot

Proposal 2-2: A slot for dedicated resource pool contains the control region for PSCCH and the symbols for allocating SL-PRS

Proposal 2-3: The SL-PRS allocation within a slot of dedicated resource pool could be both IFDMA and TDM based, which is RS structure based

Proposal 2-4: All the cast type of SL-PRS can be allocated within a same dedicated resource pool

Proposal 2-5: Since it is dedicated, the SL-PRS BW is expected to be equal to that of the dedicated resource pool. If not, the resource pool’s bandwidth could be reduced

Proposal 3-1: A single SCI could be considered for SL-PRS allocation in dedicated resource pool

Proposal 3-2: When a UE reserves the resource for future transmission in SCI, the UE may also reserve the resource for the transmission of another UE