3GPP TSG RAN WG1 Meeting #112		R1- 2301644

Athens, Greece, Feb 27th – Mar 3rd, 2023

Agenda item:       9.1.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	CSI enhancement for high/medium UE velocities and CJT

Document for:     Discussion and Decision

# 1	Introduction

In RAN1#111 [2], progress was made in several design aspects of the three CSI features and open issues were also identified and prioritised for discussion, summarised in the following table:

Table 1. Priority issues to be addressed in RAN1#112.

|  | Issue | Topic |
| --- | --- | --- |
| 1 | Type-II CJT | W2 quantization: working assumption on Alt3 |
| 2 |  | Finalize details for mode-1: Alt1 vs 2 vs 3 |
| 3 |  | Finalize Parameter Combination (including supported NL values) |
| 4 |  | Finalize UCI parameter design |
| 5 |  | Initial discussion on CBSR |
| 6 |  | Initial discussion on CSI omission |
| 6 | Type-II Doppler | FFS: Whether to prioritize/down-select refinement of Rel-16 vs Rel-17 Type-II |
| 7 |  | Finalize CQI: if X>1 is supported, CQI/PMI/slot correspondence |
| 8 |  | Finalize remaining parameter issues: last  value, need for additional Q values, d values, need for additional N4 values, need for additional K values |
| 9 |  | Finalize W2_tilde quantization (N4>1) |
| 10 |  | Finalize NNZC bitmap design |
| 11 |  | Initial discussion on Parameter Combination |
| 11 | TDCP | Finalize down-selection of parameter type (Alt A1 vs A2 vs B) |

In this paper we discuss the open issues that were identified for each of the three features, elaborate on our proposals and present some simulation results based on the agreed EVM assumptions.

# 2	Type-II CJT

## 2.1	Issue 1: working assumption on Alt 3

In RAN1#110bis-e, the following working assumption was agreed on the number of reference amplitudes for the quantisation of W2

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer:

- Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

  - FFS: Amplitude quantization table enhancement

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

- Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111)

  - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

- If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

The justification for supporting one reference amplitude per polarisation per TRP was to account for the possible RSRP difference between TRPs in some scenario. However, Type-II-CJT already allows for the UE flexibility of dynamically select and report a subset N≤NTRP of the configured TRPs and exclude TRPs with large RSRP gap from the best TRP.

Besides, the gNB also has the flexibility of selecting a subset NTRP of TRPs to configure for reporting from a larger set of cooperating TRPs and exclude TRPs that are known from previously reported measurements to have poor RSRP for a UE. Therefore, supporting more that two amplitude references, one per polarisation across TRPs, seem unnecessary.

- Revert the working assumption on Alt 3 for the number of amplitude references in W2 quantisation.

## 2.2	Issue 2: FD bases determination for mode-1

In offline email discussion after RAN1#110bis-e, the following alternatives were proposed regarding the reporting of Wf,n for mode-1.

Agreement

On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes:

- Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources from a gNB-configured set of FD basis candidates

- Alt2. Wf,n independently selected across N CSI-RS resources from a gNB-configured set of FD basis candidates (without any per-CSI-RS-resource FD basis selection offset)

- Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources from a gNB-configured set of FD basis candidates.

For all the above alternatives, the legacy FD basis selection indication scheme (combinatorial-based for N3≤19, window-based for N3>19) is applied on each selected FD basis.

Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

The main use case for mode-1 is CJT scenarios where different distances between TRPs and UE may cause relative offsets of the power delay profiles between TRPs. Reporting an FD basis offset with respect to a reference TRP allows to compensate for these delay differences and maximise the alignment between FD basis vectors across TRPs. Note that the frequency selectivity characteristics and the delay spread of the channel from cooperating TRPs are expected to be very similar and the main difference between their propagation channels is their relative delay.

An FD basis offset, φn of TRP n relative to a reference TRP corresponds to subband-level co-phasing between TRPs along a phase ramp as formulated in the description of Alt 1 and Alt 3. φn can be either integer in the value set {0,1,…,N3-1} or fractional in case of oversampling. The advantage of oversampling is to better capture the energy of the channel signature in the frequency domain, in a similar way as oversampling in the spatial domain improves the accuracy of the beam selection. Note that for single-TRP Type-II CSI, oversampling in the frequency domain is a UE implementation choice that is transparent to the gNB because the same phase multiplication applied to all the CSI-RS ports does not change a precoder. Hence, the choice of an orthogonal group for the FD basis vectors from an oversampled codebook does not need reporting for single-TRP Type-II CSI. Similarly, an FD basis vector offset within an orthogonal group and applied to all CSI-RS ports does not need reporting for single-TRP Type-II CSI. However, for CJT Type-II CSI, the choice of an orthogonal group for FD basis vector selection in case of oversampling, and/or the application of an FD basis vector offset within the selected orthogonal group needs to be reported if these offsets are TRP-specific, because different offsets may apply to different groups of CSI-RS ports corresponding to different TRPs.

- The FD basis offsets, φn, of Alt 1 can be integer or fractional, in case of oversampling. The choice of an orthogonal group and/or the application of an FD basis vector offset within the selected orthogonal group needs to be reported if these offsets are TRP-specific.

When compare Alt 2 and Alt 3, they both use separate reporting of FD basis vectors per TRP, with the only difference that Alt 3 applies a per-TRP FD basis offset. However, for N3>19, i.e., when R=2, applying a window of length 2Mν and initial point Minitial per TRP, with Alt 2, is functionally equivalent to applying an FD basis vector offset, with Alt 3.

When comparing Alt 1 and Alt 2 in simulations there is no noticeable advantage in reporting TRP-specific FD basis vectors. This is explained by the similarity between delay spreads of different TRPs in a typical scenario and the fact that the dominant FD component for each TRP can be reported without need for TRP-specific Wf,n. On the other hand, Alt 1 has advantages over Alt 2 in performance, particularly when using fractional offset values, in overhead, because it reports a single TRP-common set of vectors instead of N separate sets and in complexity as a single set of vectors can be used for W2 calculation.

- Alt 2 and Alt 3 are expected to have the same performance because both use separate reporting of FD basis vectors per TRP and are functionally equivalent for N3>19, i.e., for R=2, when applying per-TRP Minitial.

- Alt 1 has advantages over Alt 2 in performance, particularly when using fractional offset values, in overhead, because it reports a single TRP-common set of vectors instead of N separate sets and in complexity as a single set of vectors can be used for W2 calculation.

To confirm the analysis above, we compared throughput for Alt 1 and Alt 2 from SLS results. We tested an Outdoor 1 scenario with 4 CSI-RS ports per TRP, RMa channel model at 700MHz and parameter combination 2 for Rel-16 Type-II. The results are shown in Table 2 for different oversampling factors and confirm the superiority of Alt 1 to Alt 2/Alt 3 in terms of performance, especially when using fractional offsets.

Table 2. Throughput comparison between Alt 1 and Alt 2/Alt 3 for mode 1.

|  | Alt 2/Alt 3 | Alt1 (O3=1) | Alt1 (O3=4) | Alt1 (O3=8) | Alt1 (O3=16) |
| --- | --- | --- | --- | --- | --- |
| Mean UE SE | 3.65 | 3.70 | 4.35 | 4.47 | 4.52 |
| Cell edge SE | 1.15 | 1.19 | 1.69 | 1.78 | 1.82 |

- In terms of throughput performance, Alt 1 shows about 1.3% and 3.5% gain in mean and cell edge throughput, respectively, over Alt 2/Alt 3, when using integer offsets, i.e., no oversampling. This gain increases significantly with fractional offsets, i.e., with oversampling, and is about 19% and 47% in mean and cell edge throughput, respectively, with an oversampling factor O4=4.

- Support Alt 1 for mode 1 in Type-II-CJT CSI reporting.

## 2.3	Issue 3: parameters combinations and NL values

In RAN1#110bis-e and RAN1#111 it was agreed to adopt the legacy codebook parameters of Rel16 and Rel17 Type-II codebooks and that the RRC parameters: Mν, β,N1,N2,R are TRP-common, whereas the number of SD basis vectors, Ln, may be different between TRPs and configured by NL combinations: {L1,…,LNTRP}, for Rel-16 Type-II extension, and {α1,…,αNTRP} for Rel-17 Type-II extension. A UE can select and report a combination out of NL. The following was agreed

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling

- When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations

  - NL =1 is one of the supported candidate values

  - FFS: Other supported value(s) of NL, and its respective UE capability

  - FFS: The supported combinations of values for {L1, ..., LNTRP}

- Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of  PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2.

- The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and

  - for Rel-17-based refinement, the gNB configures a set of NL combinations for {α1, ...,αNTRP}  Ln=αnPCSI-RS/2 where αn=12,34, 1

FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived

Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

When

- NL=1, the gNB configures a single combination of Ln (or αn). In this case there is no need for indication of the selected combination in Part 1 CSI

- NL>1, the total number of SD vectors configured across TRPs,  n=1NTRPLn, is variable and so is K0 for a configured value of β. In this case, it becomes difficult to define a parameter combination because of the dynamic selection of the values of Ln by a UE, such that the dynamic range of feedback overhead can be very large for the same configuration.  This problem of having different values of K0 for a given β can be avoided by allowing only combinations with the same total number of beams n=1NTRPLn.

Therefore, it makes sense to maintain the legacy pair {pν,β} in the definition of a parameter combination and discuss the supported combinations {L1,…, LNTRP} separately

- Support the definition of parameter combinations by means of the legacy pair {pν,β} without adding new parameters. Consider the supported combinations of {L1,…,LNTRP} separately.

Regarding the parameter R, the following was agreed in RAN1#111

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter R, the supported value(s) from the legacy specification are reused.

- FFS: whether additional value 4 can also be added

A higher value of R allows higher resolution in delays, such that, for example, a UE can compensate for different propagation delays across TRPs, at the expense of higher UE complexity due to the larger number of FD basis vectors. However, higher accuracy in delay compensation can be achieved with much lower complexity by reporting FD basis offset. Hence, values of R larger than 2 seem unnecessary.

- Support only parameter values: R=1,2.

## 2.4	Issue 5-6: CBSR and UCI omission

Codebook subset restriction is a feature intended to allow the gNB to restrict the use of certain spatial beams (hard CBSR) or the power allocated to certain beams (soft CBSR) in the reported PMI.

UCI omission is an emergency procedure that is indented to truncate the reported PMI when the PUSCH/PDCCH resource is not enough to fit the CSI payload in full.

Both features can be largely reused from Rel-16 with a simple extension to support multiple CSI-RS resources for Type-II-CJT

- Regarding CBSR and UCI omission for Type-II-CJT, support a simple extension from Rel-16 to multiple CMRs.

# 3	Type-II Doppler

## 3.1	Issue 7: X>1 CQIs per subband per report

In RAN1#111, it was agreed to support at least one CQI per subband, with three alternatives for the association between time slot(s) and CQI

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives:

- Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices

- Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices

- Alt2B.  The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices

Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB

The three alternatives are illustrated in Figure 1

- Alt 1. The CQI is associated to the entire CSI reporting window WCSI formed by N4⋅d slots and all N4 W2 matrices

- Alt 2A. The CSI is associated to the first slot, l, of the reporting window, and the first W2

- Alt 2B. The CSI is associated to the first d slots and to the first W2

Figure 1. Alternative association between X=1 CQI, slot(s) and W2's

Alt 1 offers the least mismatched between CQI and PMI, whilst Alt 2A is the simplest for UE complexity because CQI calculation is done on a single slot. Alt 2B has the disadvantage of requiring multi-slot CQI calculation without the benefit of Alt 1. To maintain legacy behaviour, Alt 2A may be supported as basic feature, and an optional feature may be introduced whereby the CQI is calculated on two slots, the first slot and the middle slot of the reporting window.

Regarding the issue of reporting multiple CQIs per subband, although this is beneficial for the gNB in matching CQI and PMIs, UE complexity can be also considered and a compromise solution between accuracy and complexity may be that of supporting, as optional feature, X=2 CQIs per subband. The association between CQIs and slots and CQIs and W2’s can follow the X=1 optional case, with one CQI associated with the first slot, l, of the reporting window and the first W2, and the second CQI associated with the middle slot, l+WCSI-1, and the N4/2-th W2.

- Support X=1 CQI per subband as basic feature with association to the first slot of the CSI reporting window and first W2. Support, as optional features, X=1 with association to the first and middle slot, and first and middle W2, and X=2 with each CQI associated, respectively, to the first and middle slot of the CSI reporting window and to the first and middle W2.

## 3.2	Issue 8: remaining parameter issues

To achieve good CSI prediction, the length of the prediction filters, the time separation between CSI-RS resources and the length of the training phase all play a role. We need to predict N4 PMIs in a CSI reporting window of size WCSI=dN4.  Thus, for that is reasonable to have 1) sufficient CSI-RS periodicity TCSI-RS 2) sufficient CSI-RS resources prior the prediction. In addition, the main challenge is how to avoid incurring in excessive radio resource overhead, thus minimizing the amount of resources required for the CSI-RS generation while keeping good CSI prediction. We can think of three different cases for the setup of the CSI-RS transmission:

- a dense P/SP-CSI-RS scheme with TCSI-RS=P ,  P=d<δ+WCSI slots

- a scheme in which for a set of Nf  P/SP-CSI-RS signals with TCSI-RS=P,  P=δ+WCSI slots, there is one set of AP-CSI-RS burst with K burst signals, in which the signals in the burst CSI-RS are separated by m=d slots, configured to cover WCSI=dN4, with K=N4 or

- a P/SP-CSI-RS scheme with  TCSI-RS=P,  P=δ+WCSI slots, without any triggered burst.

To illustrate the utilization of CSI-RS resources and how to define the necessary scheme to achieve a reasonable CSI prediction, we may imagine initially a very simple case. In the first case, we consider a dense P/SP-CSI-RS scheme (See Figure 2) with TCSI-RS=P,  P=d<δ+WCSI. If the P/SP-CSI-RS sequence is dense enough, we may have a very good CSI prediction. In particular, it would be highly desired that  TCSI-RS≤Tc , where  Tc corresponds to the coherence time of the channel. As it is well-known, Tc is inversely proportional to the doppler spread characteristics of the channel represented by a maximum doppler shift frequency fd,max, i.e., Tc∝1/fd,max. In that context, Tc corresponds to a physical quantity to represent the channel variability in the time domain. During a coherence time window Tc, the temporal channel correlation is supposed to change very slow without dropping below a certain threshold, thus having TCSI-RS<Tc is a desirable condition to facilitate CSI prediction. However, the major issue to implement this P/SP-CSI-RS scheme is indeed that having a short period TCSI-RS implies a high density of CSI-RS resources required in time to achieve such good results. In the ideal case implementation, we would like to have TCSI-RS=P=d slots, in order to track and granularly predict the required N4 PMIs, while we exploit the periodical time correlation properties of the channel. Nonetheless, this contradicts with the assumption in which d=m∈{1,2} as for larger WCSI windows we will require larger values of d if we want to avoid to have an excessive large value for N4. If the agreed value of d is particularly small, this makes this approach not very practical from the implementation point of view. However, if we allow larger values of d, this implementation might make sense for TCSI-RS=P=d slots enabling CSI for larger WCSI and predicting each of the N4 PMIs spaced by d slots.

We may modify the scheme presented previously and illustrated in Figure 2 by combining a P-CSI-RS with an AP-CSI-RS burst. In Figure 3, we illustrate how we may increase the length of TCSI-RS in order to enable TCSI-RS=P>d  slots. We may, for instance, add the AP-CSI-RS triggered bursts within the WCSI prediction window, which might allow granular calculation of the CSI predictor with d slots spacing for the set of N4 PMIs. In this case a certain ratio of Nf  P-CSI-RS must be transmitted before the single AP-CSI-RS burst set is allocated. In order to facilitate the prediction of the set of N4 PMIs, each AP-CSI-RS burst is composed by K signals, in which K=N4. Likewise, another alternative to have TCSI-RS=P>d  slots avoiding to use the AP-CSI-RS burst consists of using a scheme as depicted in Figure 4. In this scheme we have a continuous P/SP-CSI-RS scheme with  TCSI-RS=P, P=δ+WCSI slots. This case requires a much less dense CSI-RS configuration in comparison with the case previously discussed in Figure 3. However, the question that might arise is how to obtain the N4 PMIs in between two CSI-RS measurements in this scheme. In order to do that, implementation might consider using any mechanism of interpolation to obtain such PMIs. This might work reasonably well for some speed range in which channel still has reasonable time correlation properties depending on the already mentioned variability of channel represented by Tc and the requirement to have a reasonable TCSI-RS.

We suggest now to focus on the two latter schemes mentioned previously and depicted in Figure 3 and Figure 4.  In order to illustrate these ideas, we provide some exemplary results as shown in Figure 5. In this experiment, we have a setup with TCSI-RS=P=20 ms, N4=8, d=m= 2 ms and δ=4 ms with a speed of 10km/h for an UMa channel realization at 2GHz with 10MHz bandwidth (4PRB/subband). The empirical cumulative distribution function (CDF) exhibits the cosine similarity for the resulting rank 1 precoding matrices obtained after having applied different prediction methodologies. Precoding matrices obtained are not CSI compressed, we consider unquantized eigenvectors in this example. Zero Order Holder (ZoH) corresponds to the baseline (depicted in blue in Figure 5), in which the set of N4 precoding matrices are obtained from repeating the latest obtained channel measurement on each of the points of the window WCSI. The second exhibited case corresponds to the case combining P/SP-CSI-RS TCSI-RS=P>d  slots, P=δ+WCSI,  with an AP-CSI-RS burst (depicted in red in Figure 5) in which the burst is composed by N4=K signals spaced by d=m= 2ms. Finally, we depict the case with P/SP-CSI-RS case TCSI-RS=P>d  slots, P=δ+WCSI and no AP-CSI-RS burst. A simple mechanism of interpolation is used to obtain the set of N4 precoding matrices in this case.

Figure 2. Illustration of a P/SP- CSI-RS schemes in which TCSI-RS=P ,  P=d<δ+WCSI

Figure 3. Illustration of a set of Nf  P/SP-CSI-RS signals spaced P slots accompanied with one AP-CSI-RS burst set with K signals, in which the triggered burst CSI-RS signals are separated by d slots, configured to cover WCSI=dK, with  K=N4.

Figure 4. Illustration of a P/SP-CSI-RS schemes in which TCSI-RS=P, P=δ+WCSI

We have produced results in two different time snapshots a) 30ms after the CSI predictor has been calculated and b) 200 ms later after the CSI predictor has been calculated. In order to initialize the CSI predictor we consider a P/SP-CSI-RS with Nf=16 measurements spaced by P=20ms. We may notice from results that just after the CSI predictor has been calculated 30ms later, the scheme with  AP-CSI-RS outperforms the scheme with P/SP-CSI-RS only based on interpolation. However, when we calculate the results after 200ms, the CSI predictor obtained with the AP-CSI-RS burst is quite degraded and the interpolation seems to be still working. This lead us to think that a mechanism with AP-CSI-RS burst requires a more frequent refreshment rate, which implies to send the AP-CSI-RS burst very often. For this reason, this idea from the radio resource utilization point of view seems not fully practical in that sense. On the other hand, the idea based on interpolation seems reasonable at first glance, however it must be used carefully as performance depends on the capability of interpolation into the WCSI window, and the accuracy of the curve fitting, which is physically challenged by the variability of channel.

Based on the previous results, this justifies delving deeper in the CSI prediction case with TCSI-RS=P>d  slots, P=δ+WCSI with interpolation. For that, we have done a simulation study to understand how the UE speed and TCSI-RS  might impact the functioning of this scheme. In the simulation results presented in Figure 6, we calculate the CSI prediction for 3 different UE speeds with 10, 20 and 30km/h. In addition, we choose a value of TCSI-RS in which interpolation provides some performance gain. For the CSI, we use a Rel16 Enhanced Type II scheme with no TD compression (i.e. all N4 PMIs and respective W2 are transmitted). We may notice that for high speed at 30km/h we must choose TCSI-RS= 5ms which means a very dense P/SP-CSI-RS scheme. Nonetheless the gain is remarkably high, i.e., 9% for mean UE Thrp, 26% for UE cell-edge Thrp vs. ZoH scheme. On the other hand, for the configurations with 10 and 20km/h we have relaxed the TCSI-RS with 20 and 10ms respectively. However, as we may observe, the attainable gain is highly impacted by the time length of  TCSI-RS and WCSI. These results looks very interesting and it leads us to study more in detail the nature of the variability of channel, how the channel aging behaves in time and as a consequence how CSI prediction performance decays as we increase the size of WCSI. This study is necessary for further support the idea of interpolation in P/SP-CSI-RS scheme and look for other alternatives.

| (a) | (b) |
| --- | --- |

Figure 5. Illustration of a CSI Prediction performance by means of P/SP-CSI-RS with Interpolation (Yellow) and P/SP-CSI+RS +AP-CSI-RS burst (Red) after a) 30slots after having calculated the CSI predictor b) 200slots after having calculated the CSI predictor.

Figure 6. Comparison of performance gain vs. ZoH at different speeds (10, 20, 30km/h) for a P/SP-CSI-RS scheme with a given TCSI-RS  (5, 10, 20ms respectively) using interpolation to obtain the N4 PMIs inside WCSI.

## 3.3	Issue 9: W2 quantisation (N4>1)

In RAN1#111, two proposals were discussed regarding W2 quantisation, based on a simple extension of the Rel-16 Type-II design. The extension deals with the additional compression in the Doppler domain with Q DD basis vectors, by defining one reference phase and one SCI across the 2L×Mν×Q bitmap and two reference amplitudes, one per polarisation across all FD and DD basis vectors.

Proposal 2.I.1 from RAN1#111: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme:

- Alphabets for amplitude and phase

- Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients

Proposal 2.I.2 from RAN1#111: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer:

- One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors

- One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2)

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

Our preference is to support this simple extension as outlined in the proposals above, as we do not see any reason to complicate the existing design.

- Regarding the quantisation of W2, support proposals 2.I.1 and 2.I.2 from RAN1#111

## 3.4	Issue 10: NNZC and bitmap design

In RAN1#111, several proposals were made to optimise the bitmap design, as reported below.

Proposal 2.E.2

For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select only one from the following alternatives:

- Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv

- Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code

- Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector.

- Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector.

- Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector.

- Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L - s1-s0+ mint1-t0, Q - t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

Our preference is to reuse the existing Rel-16 design with simple extension to accommodate the new DD dimension and avoid lengthy discussions on bitmap optimisation. This is not likely to yield any significant overhead reduction.

- Regarding the bitmap design, support Alt1, i.e., the bitmap size is 2LMν for each of the Q DD basis vectors.

# 4	TDCP

## 4.1	Issue 12: TDCP parameter type

In RAN1#111 the following agreement and conclusion were taken for down-selection regarding the TDCP quantity definition.

Agreement

For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112:

- AltA.1 (Doppler spread) as described in R1-2210523

- AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources

  - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP)

  - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,…

  - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0 , where  ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n.

- AltB (TD correlation profile) as described in R1-2210523

Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming:

- Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD

  - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports.

  - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead

  - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric.

  - Other scenarios of the agreed use cases can optionally be simulated

- Based on the agreed EVM for sTRP and mTRP

Note: Different alternatives may or may not apply to different use cases

FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high

FFS: TDCP parameter(s) signalled with respect to each alternative

In the summary document [6], descriptions are provided for Alt A.1 and Alt B. The three methods are summarised as follows:

- UE calculates the time-correlation of the N strongest channel delays and estimates the Doppler spread for each delay by fitting a model-based correlation (Bessel function) in MMSE sense. UE reports fd,max-fd,min.

- A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources.

- UE calculates wideband time-correlation and reports the amplitude samples in quantised form.

In terms of complexity, complexity of method A1 is higher than the other two, because of the need to calculate multiple error functions, for each of the N dominant channel delays, and fit a model-based time-correlation in terms of MMSE.

Experiment setup

We consider the use case of a gNB deciding to switch between Type-I and Type-II CSI reporting based on the TDCP report. We assume the time-correlation is measured from L+1 aperiodic TRS resources separated by Ts=5ms, where L is the configured number of time correlation lags other than 0. We assume TDCP quantity is reported every 100ms, whereas both Type-I and Type-II CSIs are available every 20ms. The simulated channel is CDL-A, with 10MHz bandwidth, and SU-MIMO downlink transmission.

For method A.2, a UE reports the location of M=1 strongest peak, by using with log2(L+1) bits: k0∈{0,…,L-1}. The metric used by the gNB for method A.2 is Doppler speed defined as follows: MA2=fdfcc, where fd=k02(L+1)Ts. A typical threshold value used with this method is 10km/h.

For method B, a UE reports the amplitude of the L-th time-correlation lag, aL, which coincides with the metric used by the gNB. A typical threshold value used with this method is 0.98.

Parameter sweep values are as follows: number of lags L: 1,2,…,15; Threshold for method B aL amplitude: 0.9,0.91,…,0.99; threshold doppler speed: L+1 values depending on the resolution that L allows (see fd description above).

For the first comparison of Methods A1, A2, and B, we do the following: for each UE based on first L+1 TRS channel measurements we calculate the doppler speed estimation based on Methods A1, A2 and B. Then we compare estimated doppler speed with Threshold speed parameter and make type-I vs type-II decision: if doppler speed estimation vd is smaller or equal to Threshold speed, then we choose type-II, otherwise type-I. The best sets of parameters from the sweep for each method are shown on Figure 7. First, we choose parameters that maximize spectral efficiency (the top plots). After that we choose parameters to maximize throughput. Final throughput measurement is based on the full simulated time (100ms), therefore Type-I vs Type-II choice is made “retrospectively”, only once, and longer lag values do not influence the performance directly.

Figure 7. Methods A1, A2, B performance in terms of raw throughput (left) and spectral efficiency (right); parameters from the sweep are chosen to maximize spectral efficiency (top) or raw throughput. NB: Throughput measurement is based on the all simulated time (100ms), therefore Type-I vs Type-II choice is made “retrospectively” and larger number of lags do not influence the performance.

In reality the numbers of lags can influence performance of all Methods since we need to make measurements and only after that report TDCP and make a decision. Since immediate throughput values have very high variance, we cannot rely on it as on a good measure to choose best parameters sets for Methods A1, A2, B. Here and further, we will be choosing parameters which maximize mean spectral efficiency across all UEs and different speeds and make a choice between Type-I and Type-II several times for every UE, if not stated otherwise. The switch can only happen every 20ms and not before UE measures all TRSs from the burst. Therefore, larger values of L can delay the decision.

Now, we show performance for scenario where multiple switches in time between Type-I and Type-II are possible based on TDCP report on Figure 8. Now, best number of lags L values are smaller since it is more beneficial to make choice to switch from Type-II to Type-I faster and possibly make transition from Type-II (here we always start with Type-I) to Type-II.

What we can also note from Figure 8 is the fact that Type-I shows better spectral efficiency only after around 30kmph UE speed, but for raw throughput Type-I start showing better results after around 12kmph UE speed. This can be explained by the fact that in our simulations HARQ and OLLA were turned off and therefore spectral efficiency should be a better performance metric then raw throughput.

Figure 8. Methods A1, A2, B performance in terms of spectral efficiency (top) and throughput prediction (bottom); parameters from the sweep are chosen to maximize spectral efficiency. NB: ideal and worst switching are done based on per slot spectral efficiency value and do not appear to be “ideal” in terms of throughput prediction.

Now, we show on the bubble plots performance for methods A1 and A2 (see Figure 9), and for method B (see Figure 10) for all the parameters in the sweep, where the size of bubbles shows mean spectral efficiency across all UEs and different speeds. Method A2 shows better performance than Method A1 in terms of mean spectral efficiency across all the tested parameters (see Figure 9). For method B only single parameter is important for the performance, which is the lag power threshold. Number of lags used is not important for Type-I vs Type-II switching.

- Method A2 shows better performance than Method A1 in terms of mean spectral efficiency across all the tested parameters.

Here, on both Figure and Figure top row of bubbles reflect the trivial decision since threshold speed here is a maximal resolution for the doppler speed estimation and leads to always choosing Type-II (as doppler speed is always smaller or equal than maximal resolution value). On Figure these top row bubbles also become smaller as for larger number of legs delays this trivial switch to Type-II degrading the performance. On Figure to better show that Method B almost always works better for smaller number of lags we decided to show spectral efficiency for the single “retrospective” decision between Type-I and Type-II, so the performance degradation with larger values of L is not because of delayed switching (as on Figure 9), which is reflected in the fact that top row bubbles are not getting smaller with growing L. We can also note that bubbles with different colors (which show Method B threshold for lag power) have different sizes, so we need to optimize only for that parameter.

Figure 9. Comparing Method A1 vs A2 mean spectral efficiency (across all UE speeds) for different numbers of lags and threshold speeds.

Figure 10. Effect of different numbers of lags and threshold speeds for Method B. Single switch retrospective choice of Type-I or Type-II.

For Method B we found out that L=1 in most of cases shows better performance (see Figure 10), therefore, we compare performance only for AC Threshold B with L=1 parameter on Figure 11. In Method B for low UE speeds smaller thresholds work better for lower speeds and higher threshold values work better for higher speeds with switch point at around 38 kmph.

Figure 11. Effect of changing threshold for Method B.

- For method B only single parameter is important for the performance, which is lag power threshold. Number of lags used is not important for Type-I vs Type-II switching.

- In Method B for low UE speeds smaller thresholds work better for lower speeds and higher threshold values work better for higher speeds with switch point at around 38 kmph.

- For Method B higher values of lag power threshold show better performance across different UE speeds.

- Depending on the use case optimal lag power threshold value can be different.

- Quantization settings can influence the accuracy of the reporting values and as a consequence use case performance.

- Support method B for TDCP reporting of one correlation lag amplitude. Optionally, also consider reporting amplitude and phase of one or more correlation lags.

- Consider quantisation methods for the reported time correlation amplitude.

# 5	Conclusion

Hereafter is a summary of observations and proposals for Type-II-CJT enhancement in FDD.

Observation 1	The FD basis offsets, φn, of Alt 1 can be integer or fractional, in case of oversampling. The choice of an orthogonal group and/or the application of an FD basis vector offset within the selected orthogonal group needs to be reported if these offsets are TRP-specific.

Observation 2	Alt 2 and Alt 3 are expected to have the same performance because both use separate reporting of FD basis vectors per TRP and are functionally equivalent for N3>19, i.e., for R=2, when applying per-TRP Minitial.

Observation 3	Alt 1 has advantages over Alt 2 in performance, particularly when using fractional offset values, in overhead, because it reports a single TRP-common set of vectors instead of N separate sets and in complexity as a single set of vectors can be used for W2 calculation.

Observation 4	In terms of throughput performance, Alt 1 shows about 1.3% and 3.5% gain in mean and cell edge throughput, respectively, over Alt 2/Alt 3, when using integer offsets, i.e., no oversampling. This gain increases significantly with fractional offsets, i.e., with oversampling, and is about 19% and 47% in mean and cell edge throughput, respectively, with an oversampling factor O4=4.

Proposal 1	Revert the working assumption on Alt 3 for the number of amplitude references in W2 quantisation.

Proposal 2	Support Alt 1 for mode 1 in Type-II-CJT CSI reporting.

Proposal 3	Support the definition of parameter combinations by means of the legacy pair {pν,β} without adding new parameters. Consider the supported combinations of {L1,…,LNTRP} separately.

Proposal 4	Support only parameter values: R=1,2.

Proposal 5	Regarding CBSR and UCI omission for Type-II-CJT, support a simple extension from Rel-16 to multiple CMRs.

Hereafter is a summary of observations and proposals for Type-II-Doppler enhancement.

Proposal 6	Support X=1 CQI per subband as basic feature with association to the first slot of the CSI reporting window and first W2. Support, as optional features, X=1 with association to the first and middle slot, and first and middle W2, and X=2 with each CQI associated, respectively, to the first and middle slot of the CSI reporting window and to the first and middle W2.

Proposal 7	Regarding the quantisation of W2, support proposals 2.I.1 and 2.I.2 from RAN1#111

Proposal 8	Regarding the bitmap design, support Alt1, i.e., the bitmap size is 2LMν for each of the Q DD basis vectors.

Hereafter is a summary of observations and proposals for TRS-based TDCP reporting enhancement.

Observation 5	Method A2 shows better performance than Method A1 in terms of mean spectral efficiency across all the tested parameters.

Observation 6	For method B only single parameter is important for the performance, which is lag power threshold. Number of lags used is not important for Type-I vs Type-II switching.

Observation 7	In Method B for low UE speeds smaller thresholds work better for lower speeds and higher threshold values work better for higher speeds with switch point at around 38 kmph.

Observation 8	For Method B higher values of lag power threshold show better performance across different UE speeds.

Observation 9	Depending on the use case optimal lag power threshold value can be different.

Observation 10	Quantization settings can influence the accuracy of the reporting values and as a consequence use case performance.

Proposal 9	Support method B for TDCP reporting of one correlation lag amplitude. Optionally, also consider reporting amplitude and phase of one or more correlation lags.

Proposal 10	Consider quantisation methods for the reported time correlation amplitude.

# References

- RP-213598, Moderator (Samsung), “New WID: MIMO Evolution for Downlink and Uplink,” RAN#94e.

- RAN1 Chair’s Notes, RAN1#111.

- R1-2205289, Moderator (Samsung), “EVM for Rel-18 MIMO CSI enhancements”, RAN1#109-e.

- R1-2212169, Nokia, Nokia Shanghai Bell, “CSI enhancement for high/medium UE velocities and CJT”, RAN1#111

- R1-2210063, Nokia, Nokia Shanghai Bell, “CSI enhancement for high/medium UE velocities and CJT”, RAN1#110bis-e.

- R1-2210523, Moderator (Samsung), “Summary of TDCP Alternatives for Comparison”, RAN1#110bis-e.

# Appendix A	SLS assumptions for Type-II-CJT

Table 3

| Parameters | Scenarios |  |  |
| --- | --- | --- | --- |
|  | Outdoor 1 | Outdoor 2A, intra-site | Outdoor 2A, inter-site |
| Inter-site distances | 1.7 km | 200 m | 200 m |
| Carrier frequencies | 0.7 GHz | 2 GHz | 2 GHz |
| Channel type | RMa | DU | DU |
| System bandwidth | 10MHz |  |  |
| BS Transmit Power | Macro: 46 dBm RRH: 46 dBm | Macro: 46 dBm | Macro: 46 dBm |
| BS Height | Macro: 35 m RRH: 35m | Macro: 25m | Macro: 25m |
| BS Antenna Configuration | 4 ports: (M,N,P,Mg,Ng,Mp,Np) = (8,2,2,1,1,1,2) 100 mechanical elevation tilt | 4 ports: (M,N,P,Mg,Ng,Mp,Np) = (8,2,2,1,1,1,2) 16 ports: (M,N,P,Mg,Ng,Mp,Np) = (8,4,2,1,1,2,4) 100 mechanical elevation tilt | 16 ports: (M,N,P,Mg,Ng,Mp,Np) = (8,4,2,1,1,2,4) 100 mechanical elevation tilt |
| UE Distribution | 100% outdoor | 100%, 20% outdoor | 20% outdoor |
| UE Antenna Configuration | 2 Rx: (M,N,P) = (1,1,2) | 4 Rx: (M,N,P) = (1,2,2) | 4 Rx: (M,N,P) = (1,2,2) |
| UE speed | 3 kmph |  |  |
| Traffic Model | FTP Model 1: target resource utilisation (RU) as specified in the results |  |  |
| Receiver | Non-ideal 2RX MMSE | Non-ideal 4RX MMSE | Non-ideal 4RX MMSE |
| CJT scheduling set size | 4 TRPs (intra-sector), 12 TRPs (inter-sector) | 3 TRPs | 9 TRPs |
| CJT reporting set size (NTRP=N) | Up to 4 TRPs, gNB configured | Up to 3 TRPs, gNB configured | Up to 4 TRPs, gNB configured |

# Appendix B	SLS assumptions for Type-II-Doppler

Table 4

| Parameter | Value |
| --- | --- |
| Duplexing | FDD |
| Scenario | Dense Urban |
| Carrier frequency | 2GHz |
| Inter-BS distance | 200m |
| BS antenna configuration | 16TX: (8,4,2,1,1,2,4) 100 mechanical elevation tilt |
| UE antenna configuration | 4RX: (1,2,2,1,1,1,2) |
| BS Tx power | 41 dBm |
| BS antenna height | 25m |
| SCS | 15kHz |
| Number of RBs | 52 |
| Simulation bandwidth | 10 MHz |
| DL MIMO | Max Rank-2 MU-MIMO |
| CSI feedback | Rel-16 Type-II |
| Traffic model | FTP Model 1 |
| UE distribution | 100% outdoor |
| UE speed | 10//20/30 km/h |
| UE receiver | Nonideal 4Rx MMSE |
| CSI-RS period | TCSI-RS=20/10/5ms respectively to each of the UE speeds. |
| Channel prediction parameters | d=m∈1,2slots, N4∈4, 8, n+δ ∈1,2slots, TCSI-RS=5, 10ms resp. d=m=2 slots, N4=8, n+δ=4 slots, TCSI-RS=20ms |