3GPP TSG RAN WG1 #112		R1-2301664

Athens, Greece, February 27th – March 5th, 2023

Agenda Item:	9.2.1

Source:	Indian Institute of Tech (M), CEWiT, IIT Kanpur

Title:	Identifying Procedures for General Aspects of AI/ML Frameworks

Document for:	Discussion

# Introduction

In RAN1 #111 [1], the following agreements on the general aspects of AI/ML framework have been reached.

Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Functionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs Working Assumption Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately. Working Assumption Terminology; Description. Model update; Process of updating the model parameters and/or model structure of a model. Model parameter update; Process of updating the model parameters of a model.

# Discussion

In this contribution, we provide a discussion on the general aspects of AI/ML Frameworks for Rel 18. Specifically, this contribution focusses on expanding on prior agreements [1, 2, 3] related to model switching, UE capabilities as well as dataset generation.

Proposal 1: A study on model switching scenarios and corresponding switching procedures should be considered.

In the application of AI/ML models to real-time wireless deployments, we envision the following two cases.

- Case 1: A UE or NW will have multiple AI/ML models, one for each use-case

- Case 2: Within the same use-case, a UE or NW will have multiple AI/ML models

Case 1 comes from the fact that clearly the same AI/ML model cannot support, for example, both CSI compression and positioning estimation. Case 2 comes from the fact that even within the same use case, different wireless deployment scenarios will exhibit different characteristics that have to be learnt by the model. Multiple AI/ML models will be better equipped to extract the scenario specific features than a single AI/ML model.

We believe that it would be beneficial to have multiple AI/ML models. A study is required to investigate the switching between these multiple AI/ML models. The study should determine procedures for the following:

- Step 1: When a current AI/ML model is no longer applicable

- Step 2: Signalling to request AI/ML model switching

- Step 3: Signalling to indicate AI/ML model switch

- Step 4: Report new AI/ML model configuration/parameters

Step 1 also includes the identification of criteria based on which models are switched. This includes identification of both use-case based switch or scenario based switch. An alternative outcome of Step 1 could be that switching needs to occur from an AI/ML model back to a non-AI/ML model. In this case, steps 2, 3 and 4 will adapt accordingly.

Proposal 2: A study should be considered to determine procedures for the signalling of UE capabilities, specific to AI/ML.

We believe that any AI/ML framework should take into account, the UE capabilities such as memory, compute power (existence of GPUs, NPUs etc), power consumption etc. We propose a study to determine signalling procedures to report AI/ML-specific UE capabilities.

Proposal 3: A study on generating and including field datasets for training and testing is necessary.

Simulations are a fast and efficient way to get started with AI/ML model training and testing. However, it is our understanding that training and testing data obtained purely from software simulations may not be sufficient to encompass all the characteristics of various scenarios and impairments. Therefore, we recommend the inclusion of field data, captured from real-time over-the-air sources. The study should also determine the amount of field data to be included in the training and testing i.e., the ratio of simulated data to field data.

# Conclusion

In this contribution, we provided a discussion on the general aspects of AI/ML framework for Rel 18. The discussion has resulted in the following proposals.

Proposal 1: A study on model switching scenarios and corresponding switching procedures should be considered.

Proposal 2: A study should be considered to determine procedures for the signalling of UE capabilities, specific to AI/ML.

Proposal 3: A study on generating and including field datasets for training and testing is necessary.

# References

[1] Chair’s Notes, RAN1 #111, November 2022

[2] Chair’s Notes, RAN1 #110, August 2022

[3] Chair’s Notes, RAN1 #109-e, May 2022