3GPP TSG RAN WG1 #112		R1-2301690

Athens, Greece, 27 February – 03 March 2023

Agenda item:	9.2.4.1

Source:		CEWiT

Title:			Evaluation on AI/ML for Positioning Accuracy Enhancement

Document for:	Discussion and Decision

1. Introduction

In this contribution concern related to Evaluation on AI/ML for Positioning accuracy enhancement, Agenda Item 9.2.4.1 is present. At RAN1 #111 meeting, some agreements on simulation assumption, KPI and further research direction have been concluded, which are listed below:

Agreement Study how AI/ML positioning accuracy is affected by user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the Evaluation. Agreement For reporting the model input dimension NTRP * Nport * Nt of CIR (Channel Impulse Response) and PDP, Nt refers to Nt consecutive time domain samples. - If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒N’t) time domain samples set to zero, then companies report value Nt in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input. Agreement For reporting the model input dimension NTRP * Nport * Nt : - If the model input is CIR, then each input value of CIR is a complex number i.e. it contains two real values, either {real, imaginary} or {magnitude, phase}. - If the model input is PDP, then each input value of PDP is a real value. Agreement At least for model inference of AI/ML assisted positioning, evaluate and report the AI/ML model output, including (a) the type of information (e.g., ToA, RSTD, AoD, AoA, LOS/NLOS indicator) to use as model output, (b) soft information vs hard information, (c) whether the model output can reuse existing measurement report (e.g., NRPPa, LPP). Agreement For AI/ML assisted positioning, evaluate the three constructions: - Single-TRP, same model for N TRPs - Single-TRP, N models for N TRPs - Multi-TRP (i.e., one model for N TRPs) Note: Individual company may evaluate one or more of the three constructions. Agreement For AI/ML assisted approach, study the performance of model monitoring metrics at least where the metrics are obtained from inference accuracy of model output.

2. Observation and Proposals:

In this contribution, we present our simulation results and observations to demonstrate the performance gain of applying AI/ML technology onto positioning for various scenarios.

Observation-1: When the training dataset and inference dataset are from different scenario the performance of AI/ML model is poor.

Observation-2: The major challenge in AI/ML based positioning is good quality for model training and testing/validation is a good quality data with accurate information.

Observation-3: From analysis we observe that the user area density (training dataset size ) has high imact on training of AI/ML model to get a better performance accuracy.

Observation-4: In AI/ML based positioning method, it is not clear as how to ensure that the provided training dataset is utilized in such a manner which ensures the optimal performance of AI/ML model.

Observation-5: From the evaluation results, it is observed that Direct AI/ML-based positioning provides reliable positioning accuracy under both heavy and moderate NLOS conditions.

Observation-6:  When the inference dataset and the training dataset are from different drops, Direct AI/ML-based positioning model provides poor performance.

Fig-1: cdf plot for different drop of training and inference data

Observation-7: When there is a SNR mismatch between the inference dataset and training dataset the AI/ML based positioning model performance  degrades.

Fig-2: cdf plot for SNR mismatch of training and inference data

Observation-8: From the evaluation result, it is observes that the performance of AI/ML model for inference dataset with channel estimation error is better than the  inference dataset without channel estimation error.

Fig-3: cdf plot for channel estimation error

From our study and simulation results we have following proposals:

Proposal-1: For the evaluation on AI/ML-based Direct positioning results, support both CIR and PDP as the model inputs.

Proposal-2:  For  reporting the model input dimension NTRP  * Nport * N’t  of CIR and PDP it is import to report,how to report timing information for N’t.

Proposal-3: For AI/ML based positioning evaluate the impact of SNR mismatch and channel estimation error on the positioning accuracy of the AI/ML model.

References

[1] RAN1 Chairman’s notes, RAN1#111, Toulouse, France, November 14 – 18, 2022

# Appendix A

Simulation assumptions for the evaluated sub use cases:

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Channel model |  | InF-DH | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (1, 1, 1 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Penetration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull. |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |  |
| UE mobility |  | 3km/h |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |
| Clutter parameters: {density , height ,size } |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |