3GPP TSG RAN WG1 #112			R1-2301735

Athens, Greece, February 27th – March 3rd, 2023

Source:	WILUS Inc.

Title:	Discussion on PHY channel design framework for SL on unlicensed spectrum

Agenda item:	9.4.1.2

Document for:	Discussion/Decision

- Introduction

From the RAN1#109-e meeting to RAN1#111 meeting, the followings were agreed on physical channel design framework for SL-U [1]-[4].

Agreement at RAN1#109-e SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement at RAN1#109-e For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement at RAN1#110 For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement at RAN1#110 For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission Agreement at RAN1#110bis-e For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets Agreement at RAN1#110bis-e To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS: - Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH - Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3) Agreement at RAN1#110bis-e Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement at RAN1#110bis-e Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details Agreement at RAN1#110bis-e For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details Agreement at RAN1#110bis-e Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission Agreement at RAN1#110bis-e At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer Agreement at RAN1#110bis-e At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details Agreement at RAN1#111 Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. Agreement at RAN1#111 For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs Agreement at RAN1#111 For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement. - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement. - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource. - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements. - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it Agreement at RAN1#111 To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity Agreement at RAN1#111 Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured. - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured. - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period. - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number. Agreement at RAN1#111 Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions. Agreement at RAN1#111 Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets

In this contribution, we discuss a PHY channel design for NR sidelink on unlicensed spectrum and provide our views.

Discussion on channel access for NR sidelink on unlicensed spectrum

Interlace RB-based PSCCH/PSSCH transmission

From the RAN guidance, RAN1 study phase should focus on FR1 unlicensed bands such as n46 and n96/n012 and is to be completed by RAN#98. According to the regulation on FR1 unlicensed spectrum, devices shall meet Occupied Channel Bandwidth (OCB) requirement that transmissions in a channel shall occupy 80% or more of the channel bandwidth. From the coverage perspective, it might be beneficial for the device to maintain the high output power and fulfill the regulations on maximum EIRP, and power spectral density. For these aspects, the interlaced allocation as one of resource allocation mechanisms was adopted in NR-U Rel-16 and LTE-LAA Rel-14. Considering available subchannel size(s) for each transmission of PSCCH/PSSCH which can be allocated on one or more contiguous sub-channels, PSCCH/PSSCH in sidelink unlicensed cannot satisfy the OCB requirement. It was agreed at the RAN1#110 meeting that interlaced RB-based transmission for PSCCH/PSSCH on unlicensed spectrum is supported. The details how to support interlaced transmission such as RB based interlaced transmission or sub-channel based interlaced transmission was discussed from the RAN1#110 to RAN1#111 meeting. The followings for further study were agreed at RAN1#111 meeting regarding mapping between sub-channel and interlace for interlace RB-based transmission for PSCCH and PSSCH in SL-U.

Agreement at RAN1#111 For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs

Similar to the method in which RB-based interlaced defined in NR-U is allocated across RB-sets regardless of RBs for intra-cell guard band, when one subchannel can occupy at least one or K interlaced in SL-U, it might be desirable to be able to span 1 or multiple RB set(s) allocated in a resource pool, rather than being confined within 1 RB set.

- Proposal 1: For interlace RB-based transmission for PSCCH and PSSCH in SL-U, we support option 3 that 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool regarding mapping between sub-channel and interlace.

  - 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool.

  - Support that the used interlace index(s) in different RB sets are always the same.

Considering that LBT is performed per 20 MHz channel bandwidth for a fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U, interlaced allocation should be reused for PSSCH resource allocation and RB set indication might be also reused if intra-cell guard configuration is configured for PSSCH resource allocation for wideband operation on SL-U. Therefore, we support to explicitly indicating the used sub-channel index(s) and RB set index(s) regarding frequency domain resource indication for interlace RB-based PSSCH transmission.

- Proposal 2: For interlace RB-based transmission for PSCCH and PSSCH in SL-U,

  - Support explicitly indicating the used sub-channel index(s) and RB set index(s)

    - The RB set indication might be used for wideband operation on SL-U if intra-cell guard configuration is configured for PSSCH resource allocation.

And it was agreed that PRBs within intra-cell guard band of two adjacent RB sets are used in the same way as Rel-16 NR-U. The receiver UE cannot know whether or not channel access procedures by the transmitter UE to transmit PSCCH/PSSCH is successful. From the perspective of the UE receiving the PSCCH, it would be difficult to know in which RB set the transmitter UE transmits the PSCCH or whether the transmission is performed in one or more RB sets after the transmitter UE performs channel access successfully. In order to reduce blind detection for determining the number of RB sets in which the PSCCH is transmitted from the standpoint of the UE receiving the PSCCH, to limit the transmission of the PSCCH to at least one RB set except intra-cell guard band PRBs was agreed at the RAN1#110bis-e meeting. However, it was not concluded yet whether or not such PRBs within intra-cell guard band of two adjacent RB sets are used for PSFCH/S-SSB transmission. If the RB-based interlaced transmission for PSFCH/S-SSB transmission is not introduced for SL-U and repetition for PSFCH/S-SSB over multiple RB sets is introduced, it would not be necessary to discuss on this. Therefore, after conclusion of the method regarding the transmission of S-SSB within the RB set to satisfy OCB and PSD, it may be desirable to determine whether to use PRBs within intra-cell guard band of two adjacent RB sets. And similarly, with respect to whether to use PRBs within intra-cell guard band of two adjacent RB sets for the PSFCH transmission, it seems desirable to further discuss after deciding whether to transmit the PSFCH in multiple RB sets or not.

- Proposal 3: Regarding whether or not to use PRBs within intra-cell guard band of two adjacent RB sets for S-SSB and/or PSFCH transmission,

  - After conclusion of the method regarding the transmission of S-SSB within the RB set to satisfy OCB and PSD, it may be desirable to determine whether to use PRBs within intra-cell guard band of two adjacent RB sets.

  - With respect to whether to use PRBs within intra-cell guard band of two adjacent RB sets for the PSFCH transmission, it seems desirable to further discuss after deciding whether to transmit the PSFCH in multiple RB sets or not.

PSFCH transmission

Different from PSCCH/PSSCH which can be allocated on one or more contiguous sub-channels, each PSFCH associated with PSSCH is occupied with 1 PRB using PUCCH format 0. To meet OCB requirement, it may be desirable to allocate the PSFCH in the form of an RB-based interlaced transmission. In Rel-16 NR-U, enhanced PUCCH format 0 can be considered as a starting point for RB-based interlaced PSFCH transmission. Unlike the method in NR-U in which PUCCH resources are dynamically indicated by scheduling DCI, in NR sidelink, a set of continuous PRBs for PSFCH occasions is used for PSFCH transmission. This is to simplify the sidelink HARQ-ACK feedback procedure by allowing all sidelink UEs to have a common understanding of HARQ-ACK feedback, and to improve the reliability of SL HARQ-ACK feedback by avoiding interference from non-HARQ feedback SL transmission to HARQ feedback SL transmission. Therefore, PUCCH format 0 is considered as a starting point for RB-based interlace PSFCH transmission, but it should be further studied and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for NR sidelink unlicensed operation.

- Proposal 4: PUCCH format 0 using RB-based interlace for PSFCH transmission should be supported, but it should be further discussed and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for SL-U.

- Proposal 5: To meet OCB and PSD requirement for PSFCH transmission, each PSFCH transmission occupies an interlace similar to PUCCH in NR-U. Different PRBs within an interlace should be applied to allocate rotating CS shift for the PAPR/CM reduction.

- Proposal 6: Considering LBT failure for PSFCH transmission, it should be adopted that PSFCH resource is dynamically indicated which is Alt-2 in the previous meeting.

When having one RB based interlaced structure and one interlace has the same index between different RB sets, PSFCH transmission can be transmitted to multiple RB sets. If the NR-U downlink-based multiple channel access method is considered as the transmission of S-SSB and transmission of PSFCH in the previous meeting, when channel access is successful in at least one RB set, the design method of the RB based interlaced PSFCH should be considered so that a PRB to which at least one PSFCH is allocated exists in multiple RB sets in order to allow transmission of the PSFCH.

- Proposal 7: The method of the RB based interlaced PSFCH should be considered so that a PRB to which at least one PSFCH is allocated exists in multiple RB sets in order to allow transmission of the PSFCH when channel access is successful in at least one RB set from the perspective of the channel access by a given UE.

Additional candidate S-SSB occasions

In the previous meeting, it was discussed regarding number and location(s) of additional candidate S-SSB occasions, Tx UE behaviors, whether or not additional candidate S-SSB occasions is belonging to RP, whether OCB exemption is applicable or not to SSB transmission and SSB structure in frequency domain to be satisfied with OCB requirement. The followings were agreed at RAN1#111 meeting.

Agreement at RAN1#111 Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured. - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured. - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period. - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number. Agreement at RAN1#111 Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions. Agreement at RAN1#111 For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement. - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement. - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource. - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements. - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it

Regarding whether or not additional candidate S-SSB occasions is belonging to RP, channel access used in unlicensed spectrum is a procedure performed at the transmitting end, and whether to use resources in candidate S-SSB occasions is determined according to the success of channel access. And each Tx UE cannot predict in advance whether or not the other RAT’s node coexisting in the unlicensed spectrum will transmit with respect to the channel condition in the candidate S-SSB occasions. In addition, the Tx UE cannot inform the Rx UE whether or not the S-SSB is transmitted in candidate S-SSB occasions, and the Rx UE cannot know the result of the channel access at the Tx UE. In NR-U, a discovery burst transmission window is configured for a UE, one or more candidate SSB transmission locations are set, and the UE assumes that one or more candidate SSB transmissions is transmitted at the corresponding locations corresponding to the SSB index set from the gNB regardless of whether or not transmission from the gNB is performed. Assuming that an SSB with one index is transmitted from locations, reception from UE is attempted. However, at candidate SSB transmission locations, the UE attempts to receive SSBs, and the corresponding resources are regarded as SSB resources. And, when receiving an SSB having the same SSB index, the UE does not expect reception of one or more SSB with same SSB index within the discovery burst transmission window. Therefore, it may be desirable not to include additional candidate S-SSB occasions configured to mitigate channel access failure in unlicensed spectrum even for SL-U in the resource pool for SL PSCCH/PSSCH transmission resources.

- Proposal 8: It may be desirable not to include additional candidate S-SSB occasions configured to mitigate channel access failure for SL-U in the resource pool for SL PSCCH/PSSCH transmission resources.

Regarding the number and location(s) of additional candidate S-SSB occasions, in NR-U, in order to reduce UE’s complexity due to SSB reception performed by UEs at every slot, a discovery burst transmission window is configured to the UE and the UE attempts SSB reception within the discovery burst transmission window. Similarly, it may be desirable to configure the S-SSB transmission window in the SL-U to the UE. And, by (pre-)configuring candidate S-SSB occasions and the number of S-SSBs to be transmitted within the S-SSB transmission window, the Rx UE can assume that S-SSB transmission is transmitted at the corresponding candidate S-SSB occasions according to the number of (pre-)configured S-SSBs and the Rx UE attempts reception of the S-SSB.

- Proposal 9: We support option 5 that the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number.

When S-SSB transmission cannot be performed due to LBT failure on candidate S-SSB occasions corresponding to the number of (pre-)configured S-SSBs from the point of view of the S-SSB Tx UE, S-SSB transmission can be attempted by performing channel access on subsequent S-SSB occasions.

- Proposal 10: We support Alt 4 that upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions.

Conclusion

In this contribution, we have discussed a PHY channel design for NR sidelink on unlicensed spectrum. As a conclusion, we summarize our view as follows:

- Proposal 1: For interlace RB-based transmission for PSCCH and PSSCH in SL-U, we support option 3 that 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool regarding mapping between sub-channel and interlace.

  - 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool.

  - Support that the used interlace index(s) in different RB sets are always the same.

- Proposal 2: For interlace RB-based transmission for PSCCH and PSSCH in SL-U,

  - Support explicitly indicating the used sub-channel index(s) and RB set index(s)

    - The RB set indication might be used for wideband operation on SL-U if intra-cell guard configuration is configured for PSSCH resource allocation.

- Proposal 3: Regarding whether or not to use PRBs within intra-cell guard band of two adjacent RB sets for S-SSB and/or PSFCH transmission,

  - After conclusion of the method regarding the transmission of S-SSB within the RB set to satisfy OCB and PSD, it may be desirable to determine whether to use PRBs within intra-cell guard band of two adjacent RB sets.

  - With respect to whether to use PRBs within intra-cell guard band of two adjacent RB sets for the PSFCH transmission, it seems desirable to further discuss after deciding whether to transmit the PSFCH in multiple RB sets or not.

- Proposal 4: PUCCH format 0 using RB-based interlace for PSFCH transmission should be supported, but it should be further discussed and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for SL-U.

- Proposal 5: To meet OCB and PSD requirement for PSFCH transmission, each PSFCH transmission occupies an interlace similar to PUCCH in NR-U. Different PRBs within an interlace should be applied to allocate rotating CS shift for the PAPR/CM reduction.

- Proposal 6: Considering LBT failure for PSFCH transmission, it should be adopted that PSFCH resource is dynamically indicated which is Alt-2 in the previous meeting.

- Proposal 7: The method of the RB based interlaced PSFCH should be considered so that a PRB to which at least one PSFCH is allocated exists in multiple RB sets in order to allow transmission of the PSFCH when channel access is successful in at least one RB set from the perspective of the channel access by a given UE.

- Proposal 8: It may be desirable not to include additional candidate S-SSB occasions configured to mitigate channel access failure for SL-U in the resource pool for SL PSCCH/PSSCH transmission resources.

- Proposal 9: We support option 5 that the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number.

- Proposal 10: We support Alt 4 that upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions.

References

[1] Final Report of 3GPP TSG RAN WG1 #109-e meeting v1.0.0

[2] Final Report of 3GPP TSG RAN WG1 #110 meeting v1.0.0

[3] Final Report of 3GPP TSG RAN WG1 #110bis-e meeting v1.0.0

[4] Draft Report of 3GPP TSG RAN WG1 #111 meeting v0.1.0