3GPP TSG RAN WG1 #112		R1-2301745

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.3.2

Source:	Indian Institute of Tech (M), CEWiT

Title:	Discussion on subband non-overlapping full duplex

Document for:	Discussion

# Introduction

A new study item named ‘Discussion on subband non-overlapping full duplex’, was approved to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in the RAN#94 e-meeting. The main agenda is to identify applicable and relevant deployment scenarios and to study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD.

# Discussion on SBFD

Different agreements have been reached in previous meeting i.e., RAN1#111. Those agreements are summarized below.

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

In this contribution, we provide a discussion on the study and enhancements required for implementing SBFD in a network based on the agreements in the previous meetings.

2.1 Enhancement in flexible slots

In existing NR, flexible slots can be configured to be UL or DL. SBFD allows allocation of both DL and UL in the same slot. Thus, there is a high probability of collision of UL and DL messages. Both UL transmission and DL reception in an SBFD symbol can be regulated semi-statically or dynamically. However, a UE can only conduct the UL transmission or DL reception at the same time. As a consequence, prioritizing rules may be necessary for the UE to choose only one of the overlapped UL transmission and DL reception when collisions are detected.

Observation 1: In case of flexible slots, the UE has simultaneous DL & UL messages being allocated to it. So, there might be chance of collision of DL and UL messages.  For Example: The gNB semi statically configures UL subband in a symbol. The same symbol might be configured with a periodic CSI-RS in DL. In this case, the UE needs to decide whether to perform CSI-RS reception or UL transmission. Thus, priority rules need to be defined in the specification for such collisions.

Proposal 1: There should be a study to examine the DL and UL message collision in flexible slots by using gNB schedulers for dynamically schedule the messages for that slot. The gNB scheduler can configure and schedule the UL/DL transmission in a symbol.

  - PRACH related enhancement

Observation 2: Allowing PRACH transmission in SBFD symbols increases the number of consecutive slots. Repetition of PRACH signal increases the UL coverage.

Proposal 2: Initial access on SBFD symbols needs to be studied since allowing initial access in SBFD symbols can improve PRACH latency as it decreases initial access delay unlike legacy TDD.

2.3 Enhancement related to cross-link interference (CLI)

UE- UE and gNB-gNB CLI exist in both SBFD and dynamic TDD. However, in SBFD sub-band level CLI is present. Rel. 16 CLI management does not take into account subband level CLI. Hence, measurement at sub-band level is required. It could be useful for a gNB to be informed of the ranges of interferences associated in a DL receiver for UE-to-UE CLI handling, and to execute DL scheduling and data transmission correspondingly. A UE operating in duplex mode in Rel-18 SBFD will experience co-channel inter-subband/intra-subband CLI and adjacent-channel CLI from different UEs in the same cell or neighbour cells. To ensure DL functionality, CLI mitigation strategies must be addressed. UE-to-UE inter-cell co-channel inter-subband CLI is defined as the interference created by the aggressor UE transmitting in a carrier to the victim UE receiving DL subband in the next cell in the same carrier.

Observation 3: The SBFD scenario is different from legacy TDD and dynamic TDD. Hence more specifications are required in the case of UE-UE and gNB-gNB CLI measurement, mitigation and handling.

Proposal 3: Since victim UE subbands are impacted by inter cell inter UE CLI, the victim UEs should measure CLI within those subbands for UE-UE CLI measurement.

2.4 Enhancement related to UL and DL channels in SBFD and non SBFD slot

Some channels like PDCCH can be present in both SBFD and non SBFD slots. PDCCH resources are dynamically adapted depending on the presence of SBFD symbols and the accompanying DL-UL frequency domain partition. For instance, the UE could be given multiple PDCCH CORESET configurations such as one associated with DL-only slots/symbols and another with SBFD slots/symbols. The CORESET that will be utilized is determined by the UE depending on the slot/symbol type.

Proposal 4: The impact of UL and DL channels being allocated for both SBFD and non SBFD slots needs to be studied.

# Conclusion

We made the following observations and proposals based on our discussion.

Observation 1: In case of flexible slots, the UE has simultaneous DL & UL messages being allocated to it. So, there might be chance of collision of DL and UL messages.  For Example: The gNB semi statically configures UL subband in a symbol. The same symbol might be configured with a periodic CSI-RS in DL. In this case, the UE needs to decide whether to perform CSI-RS reception or UL transmission. Thus, priority rule needs to be defined in the specification for such collisions.

Proposal 1: There should be a study to examine the DL and UL message collision in flexible slots by using gNB schedulers for dynamically schedule the messages for that slot. The gNB scheduler can configure and schedule the UL/DL transmission in a symbol.

Observation 2: Allowing PRACH transmission in SBFD symbols increases the number of consecutive slots. Repetition of PRACH signal increase the UL coverage.

Proposal 2: Initial access on SBFD symbols needs to be studied since allowing initial access in SBFD symbols can improve PRACH latency as it decreases initial access delay unlike legacy TDD.

Observation 3: The SBFD scenario is different from legacy TDD and dynamic TDD. Hence require more specifications in case of UE- UE and gNB-gNB CLI mitigation and handling. The CLI need to be measured for the CLI mitigation and handling.

Proposal 3: As victim UEs subband are getting impacted by inter cell inter UE CLI, So the victim UEs should measure CLI within subband for UE-UE CLI measurement.

Proposal 4: The impact of UL and DL channels being allocated for both SBFD and non SBFD slots needs to be studied.

# References

[1] Chair’s Notes, RAN1 #111, November 2022

[2] Chair’s Notes, RAN1 #110, August 2022

[3] Chair’s Notes, RAN1 #109-e, May 2022