3GPP TSG RAN WG1 #112-e	R1-2301764

Athens, Greece, February 27th – March 3rd, 2023

# Title:	[Draft] Reply LS on RB set configuration

# Response to:	R1-2300006 (R3-226781) LS on RB set configuration

# Release:	Rel-17

# Work Item:	NR_IAB_enh-Core

Source:	Ericsson [RAN1]

To:	RAN3

Cc:

Contact Person:

Name:	Magnus Åström

E-mail Address:	magnus.astrom@ericsson.com

1. Overall Description:

RAN1 thanks RAN3 for the LS on RB set configuration. In the LS, RAN3 inquiries about which one of the below understandings is the correct one:

- Understanding 1: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the SCS-specific offset defined in the NR Carrier List IE based on the SCS included in the RB Set Configuration IE.

- Understanding 2: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the smallest SCS-specific frequency offset across all numerologies provided by the NR Carrier List IE for this cell. Note that in this understanding, the start RB index does not depend on the SCS value included in the RB Set Configuration IE in TS 38.473.

With respect to the above, RAN1 would like to provide the following clarification:

Understanding 2 is correct, i.e., the start RB index for the Rel-17 IAB-DU HSNA resource configuration depends on Point A (the centre of the lowest subcarrier of common RB 0, TS 38.211) plus the smallest SCS-specific frequency offset across all numerologies (in case multiple numerologies are configured).

2. Actions:

To RAN3 group:

ACTION:

RAN1 respectfully asks RAN3 to take the above into account.

3. Date of Next RAN1 Meetings:

TSG WG RAN1 #112-bis-e	April 17th – 26th, 2023		Online

TSG WG RAN1 #113		May 22nd – 26th, 2023		Incheon, Korea