3GPP TSG-RAN WG1 Meeting #112	R1-2301856

Athens, Greece, 27th Feb – 3rd Mar, 2023

Agenda item:		7.2

Source:	Moderator (Nokia)

Title:	Moderator summary #1 on Rel-16 UL Tx Switching related issues

WI code:	NR_RF_FR1-Core

Document for:		Discussion and Decision

# Introduction

This document summarizes the three documents on Rel-16 WI NR_RF_FR1-Code on Uplink Tx Switching maintenance. The following documents are considered:

Uplink TX switching – switching gap details

R1-2300317	38.214 CR draft on Clarification to the switching gap location of the UL Tx Switching, Nokia, Nokia Shanghai Bell

R1-2300325	On the switching gap location of the UL Tx Switching	Nokia, Nokia Shanghai Bell

R1-2301384	Clarification of Rel-16 UL Tx Switching	Qualcomm Incorporated

To be moderated by Karri (Nokia).

- Issues identified

## Issue #1: Uplink Tx Switching period location

Documents R1-2300317 and R1-2300325 raise the issue of the uplink Tx Switching period location.

- 38.214 defines the duration of the switching gap (switching period) NTx1-Tx2 but it does not define its location in time even though section 6.1.6 claims to be defining the location of the gap.

- 38.101-1 defines the switching gap (switching period) placing on one of the two carriers relative to a reference point, but it does not define the reference point. The 38.101-1 definition can be read as only specifying the UE behaviour when there gNB is not guaranteeing unused UL symbols for absorbing the gap.

Two different cases can be identified

- The gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching gap

- The gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching gap

For the case 1 the symbols not scheduled/configured to transmit on either of the two uplinks can absorb the switching gap. There is no need to cut the end of the switch-from carrier’s transmission or the start of the switch-to carrier’s transmission, and the uplinkTxSwitchingPeriodLocation does not need to impact the UE behaviour. However, neither 38.214 nor 38.101-1 seem to provide a clear definition that in this case the switching gap does not map on symbols that were supposed to be transmitted.

For the case 2 the UE does not have a sufficient number of symbols to absorb the gap. One or the other UL transmission needs to be cut. The uplinkTxSwitchingPeriodLocation defines which uplink transmission is cut to provide sufficient switching gap.

Alt 1-1: The uplink switching gap (switching period) ends at the nominal start time of the uplink transmission that triggered the switch.

- If the network ensures that there is no transmission for a sufficient time duration prior to the transmission that triggered the switch, no transmission is dropped.

- If the network does not ensure this, then the uplinkTxSwitchingPeriodLocation determines whether the transmission on the switch-from or on the switch-to carrier is dropped.

Alt 1-2: The uplink switching gap (switching period) location depends on the uplinkTxSwitchingPeriodLocation configuration and the direction of the switch.

- If the uplinkTxSwitchingPeriodLocation places the switching gap on the switch-to carrier: the switching gap starts at the nominal start of the transmission that triggered the switch. The beginning of that transmission is always dropped due it always colliding with the switching gap.

- If the uplinkTxSwitchingPeriodLocation places the switching gap on the switch-from carrier: the switching gap ends at the nominal start of the transmission that triggered the switch. The end of the previous transmission is dropped if it collides with the switching gap.

### Issue #1, Round #1

Q1-1: What is in your view the role of uplinkTxSwitchingPeriodLocation in case the gNB provides sufficient number of unused symbols between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier?

| Company | Comment |
| --- | --- |
| Nokia, NSB | In our view, when the scheduler provides sufficient gap to absorb the switching period, the configuration of the uplinkTxSwitchingPeriodLocation has no impact. It only affects the case when the scheduler did not allocate enough unused symbols to absorb the switching period. |
| NTT DOCOMO | As moderator described, we also think that there is no need to cut the end of the switch-from carrier’s transmission or the start of the switch-to carrier’s transmission, and the uplinkTxSwitchingPeriodLocation does not need to impact the UE behaviour in this case (case 1). |
| Ericsson | Share the same view as Moderator. |
| ZTE | We share similar views as Nokia. But we think it is safer to confirm this with RAN4. It is worth noting that the following working assumption depends on the accurate location of switching period. If the location of switching period is up to UE, then how the gNB can know whether there are two UL Tx switching within two consecutive reference slots? Agreement Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands. - The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band - (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us} |
| Qualcomm | We have same understanding that this uplinkTxSwitchingPeriodLocation has no impact in the above case. |
| vivo | Share the same view as Moderator. uplinkTxSwitchingPeriodLocation is used to determine the location when there is no sufficient gap. If there is a sufficient gap, there is no need to cut the scheduled UL transmission. |
| Apple | We also have same understanding that for the case when scheduling gap is longer than the switching period, then switching period location configuration has no impact |
| MTK | We share the same understanding as the moderator. |
| Samsung | We share the same view as the moderator. |
| Huawei, HiSilicon | The same understanding as the moderator. Better to have a RAN1 conclusion to capture RAN1 understanding on it. |

Q1-2: When the gNB provides sufficient number of unused symbols between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, do you agree that the UE should not drop any UL symbols that were supposed to be transmitted? Do you think this case is clear in the specification, and if you do, which specification part defines this?

| Company | Comment |
| --- | --- |
| Nokia, NSB | Agree that the UE should not drop any UL transmissions due to UL Tx Switching if the gNB guaranteed enough unused UL symbols to absorb the switching period. We think the specification (either 38.214, 38.101-1 or both) should be clarified on this. |
| NTT DOCOMO | As our answer to Q1-1, we agree that UE should not drop any UL symbols due to Tx switching in case 1. We think it is not clear yet in specifications, and it is good to clarify this in either 38.101-1 (e.g., 6.3A.3/6.3C.3) or 38.214 (6.1.6). |
| Ericsson | Agree. We somewhat share the same view as DCM. In RAN4 spec, it is not clear how the timing is done in reference to T0. They use “I” which is not in our understanding T0. It is helpful to clarify that T0 is after the switching gap and denotes the start of UL transmission , no matter if enough gap is provided or not. |
| ZTE | Similar view as Nokia. |
| Qualcomm | We agree that UE should not drop any scheduled/configured transmission if the unused symbols are sufficiently larger than its reported switching period. This is unfortunately not explicitly specified in current standards even we think the above is consensus across companies. We are ok to clarify this in either specification if majority companies prefer. |
| Vivo | Agree with Moderator that any UL should not be dropped. We think the switching period location is no clear in current spec, and we are ok to clarify this aspect either in RAN1 or RAN4 spec. |
| Apple | Agree and it is okay to clarify this in specification |
| MTK | Agree with Nokia. |
| LGE | We have similar view with Nokia. Regardless of the answer for the 1st question, we think it is not clear in the current specification as the answer for the 2nd question. |
| Samsung | We share the same view as the moderator. |
| Huawei, HiSilicon | In 38.101-1, the green bars in the concerned figures (e.g. figure 6.3A.3.3.2-1a) only mean the scheduled resources as usual (referring to the figures 6.3.3.6-4 and 6.3.3.6-5). The current specifications are clear because the back-to-back green bars only means no scheduled gap between two transmissions. But we are open to make the specs plainer and clearer. S6.3A Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1 S6.3.3.6 Figure 6.3.3.6-4: Consecutive SRS time mask for the case when power change is required and when 60 kHz SCS is used in FR1, when the transient period is 10 µs Figure 6.3.3.6-5: FR1 Time mask for 15 kHz and 30 kHz SCS for the case when consecutive SRS switching usage is between antenna switching & other sets |

Q1-3: 38.214 6.1.6 claims to define the location of the switching gap in subclauses 6.1.6.x, but no such definition or reference to 38.101-1 in those clauses. Should this be clarified?

| Company | Comment |
| --- | --- |
| Nokia, NSB | Yes. 38.214 should either define the switching period location, or at least refer to 38.101-1. If 38.214 is not updated to define the location, then RAN1 should ask RAN4 to clarify this in their specs. |
| NTT DOCOMO | We basically thought that exact location of the switching gap does not necessary to be defined in case 1. In that sense, we need to clarify that all descriptions about the switching gap in specifications are only for case 2. However, one potential issue for this approach would be about DL interruption caused by UL Tx switching, e.g., if exact switching period location is not defined for case 1, exact location of DL interruption caused by the UL Tx switching would be unclear. It is defined in 38.133 8.2.2.2.10 that “The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period located in either NR carrier 1 or carrier 2 as indicated in RRC signalling”. Even in case 1, UE would be allowed to cause DL interruption due to switching as specified in 38.133 while there is no impact on UL in case 1. We would like to discuss on this issue to decide whether the location of the switching gap should be clarified or not. |
|  | Yes. The reason is DL interruption as DCM clearly explained. |
| ZTE | as pointed out by NTT DOCOMO, considering this issue is also related to DL interruption, it is better to send LS to RAN4 and ask RAN4 to clarify this. RAN1 spec can refer to RAN4 spec for information. |
| Qualcomm | We are ok to clarify this in either specification. However, the potential RAN4 impact is preferrable to be discussed and agreed in RAN4. |
| vivo | Yes. |
| Apple | Agree with NTT DOCOMO and ZTE that this issue is related to DL interruption and could be clarified in RAN4. |
| MTK | Same view as ZTE. RAN1 should ask RAN4 to clarify this in their specs. |
| LGE | Yes. It would be better to clarify this. |
| Samsung | Agree with ZTE and NTT DoCoMo. Should send LS to RAN4 and ask RAN4 to clarify this in 38.101-1. |
| Huawei, HiSilicon | If a scheduled gap is sufficiently large, then no UL interruption should be on any scheduled UL transmissions. It does not matter where the exact switching location in time domain is. Therefore, the exact location is not specified and thus make more room for UE implementation. Regarding DL interruption, we are open to let RAN4 discuss it. But in our understanding, DL interruption is introduced to relax RAN4 requirements and tests on UE DL reception for those impacted symbols. It is different from UL interruption that a gNB scheduler is not able to guarantee no overlaps between DL reception and UL transmission. In typical case, DL traffic and DL signalling would occupy most of symbols. Therefore, it seems no gain to specify switching location for DL interruption but causes limitation to UE implementation. |

### Issue #1, Round #1 Summary (NEW)

On Q1-1: Commenting companies share the same view that if the network provides a sufficient number of unused uplink symbols to absorb the switching period, the configuration of uplinkTxSwitchingPeriodLocation has no impact.

On Q1-2:

- Commenting companies agree that if the network provides a sufficient number of unused uplink symbols to absorb the switching period, no UL symbols should be dropped

- Commenting companies are supporting or at least open to clarify the specification in this regard (obviously subject to actually being able to agree on the exact clarification)

On Q1-3:

- Many commenting companies indicate that a spec clarification is preferred.

- Several companies note that the exact location of the switching period should be defined also when the network ensures sufficient duration of no scheduled UL symbols due to potential impact on DL reception, while at least one company thinks that the exact location of the switch is not necessary even for knowing where in time the DL interruption is.

- Several companies suggest RAN1 to ask RAN4 to clarify 38.101-1.

### Issue #1, Proposal after Round #1 (NEW)

Proposed agreement:

- If the gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period,

  - The time of no UL transmission allocated absorbs the switching period

  - Neither of the uplink transmissions (the one ending on the switch-from carrier nor the one starting on the switch-to carrier) are interrupted by the switching period.

  - The setting of uplinkTxSwitchingPeriodLocation has no impact.

  - The end of the switching period is located at the start of the UL transmission on the switch-to carrier

- Send an LS to RAN4 requesting RAN4 to, in this regard, clarify TS38.101-1 subclauses 6.3A.3.3.2 and 6.3C.3.1 for CA, and SUL based UL Tx Switching, and to TS38.101-3 subclause 6.3B.4.1 for EN-DC.

- Change the sentence in the 38.214 6.1.6 to refer to 38.101 accordingly as below

  - the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively, and the location of the switching gap is defined in [8, TS 38.101-3 and TS 38.101-1].

## Issue #2: UL skipping relation to Uplink Tx Switching

Document raises R1-2301384 the issue of UL skipping relation to UL Tx switching. If a scheduled or configured UL transmission is skipped, this may or may not be considered as a transmission that implies a Tx switch.

Two possible behaviours are identified for UL Tx Switching when an UL transmission is skipped:

Alt 2-1: A skipped UL transmission does not cause a Tx switch

Alt 2-2: A skipped UL transmission causes a Tx switch as if the transmission was not skipped

As noted in the R1-2301384, the current specification can be understood to refer to an actual transmission when determining the Tx switching, but it may be fair to say that UL skipping was not considered when the specification text was created. As noted on-line, due to the scheduling timeline the network may not be aware of whether an UL transmission is skipped at the time it needs to issue the scheduling DCI for the UL transmission following the UL transmission that may or may not get skipped, making it impossible for the gNB to consider skipped UL transmissions when determining the UE’s transmitter chain arrangement.

During the discussion it was also mentioned that if the gNB anyway always assumes Tx switching as if UL transmission is never skipped and also ensures sufficient number of unused symbols are provided to absorb the switching gap, it may not matter if the UE sometimes did not need the gap and possibly both UE behaviours are fine.

### Issue #2, Round #1

Q2-1: Three possible RAN1 outcomes can be identified. Please provide your views on them, and if you identify additional possibilities, please elaborate:

  - Clarify that a skipped UL transmission does not cause a Tx Switch; if you think this is the desired RAN1 outcome, please elaborate what is the assumed gNB behaviour that takes this UE behaviour into account.

  - Clarify that a skipped UL transmission causes a Tx Switch as if the skipping did not take place; gNB always assumes a gap is needed.

  - Both UE behaviours are possible; if you think this is the desired RAN1 outcome, please elaborate what is the assumed gNB behaviour that takes both UE behaviours into account.

| Company | Comment |
| --- | --- |
| Nokia, NSB | The gNB scheduler cannot know if an UL transmission is to be skipped or not when it needs to issue the DCI for the transmission following the transmission that maybe skipped. Hence we don’t think a) makes sense from the system perspective even if it may seem logical when solely focusing on the UE implementation.. We’d be OK with b). If it is clear that for a reasonable gNB implementation both UE behaviours work without needing to differentiate the two, we could be OK with c), but wish to discuss this more first. |
| NTT DOCOMO | We also think that the gNB cannot know whether an UL transmission is to be skipped or not, and hence b) (i.e., conservative assumption) would be reasonable. |
| Ericsson | We also think (b) is reasonable and can be reflected in spec. |
| ZTE | If UE transmits uplink transmission but the gNB missed this transmission, gNB cannot identify whether it is because 1) UE skipped this UL transmission or 2) UE transmitted this uplink transmission but gNB missed. To avoid this confusion from gNB side, it is better to have the same assumption for both 1) and 2). Thus, b) is our understanding and we think the current spec “If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0 -Toffset or based on a higher layer configuration(s):” has already reflected this. But we are open to further clarify it if majority companies prefer to do so. |
| Qualcomm | Thanks for the good discussion, and we are glad to confirm the necessity to discuss this issue as companies have different interpretations on this case. Among above possibilities, we prefer a) which seems doesn’t need specification updates. BS could determine whether the UL transmission is skipped or not based on the UL transmission detection. The current specification requests MAC to trigger UE by telling PHY some data to transmit and then Tx chain switching is triggered. Without data to transmit, there is not any procedure to trigger the Tx switching from MAC to PHY. This is with some similarity to UCI mux for skip case, ultimately solution was to add clause in MAC spec. Alt. b) would require MAC spec changes as PHY needs MAC to tigger the switch for the skipped UL based on current procedure. With concern on large spec impact of b), we prefer a), especially after we completed Rel-16 spec for quite a long time. For the concern of missing detection of UL transmission, please note this detection is DMRS based which is with reasonable reliability. This mis-alignment possibility between gNB and UE always exists as UE might fail detecting UL grant. Given this consideration, we don’t think misdetection of UL transmission in configured resource would be a high risk. |
| Vivo | We think a) is aligned with the current spec “If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0 -Toffset or based on a higher layer configuration(s)”. Before the CG, gNB cannot know whether an UL transmission is to be skipped or not, thus it should ensure sufficient gap between the CG and the preceding UL when it wants to avoid UL interruption due to potential TX switching. gNB will know whether the CG is skipped after detecting the UL CG and then figure out the current TX state of UE side. |
| Apple | We share similar view as QC and our preference is also a) and agree with their justification on how gNB can determine whether the UL transmission is skipped or not. |
| MTK | We tend to agree with QC that the MAC-PHY coordination requires a real transmission to trigger Tx switching. Our preference is (a) and the assumed gNB behavior is described in vivo’s comment. |
| LGE | In our view, Alt a) seems to be aligned with the current specification. Actually, we think two options may work. Only required action is a clarification on it (and fixed something if needed). Slightly prefer no spec update. |
| Samsung | Agree with NTT DoCoMo and (b) is acceptable to us. |

### Issue #2, Round #1 Summary (NEW)

On Q2-1: Commenting companies are split on whether UE behaviour a) or b) is acceptable:

- 5 companies think that a skipped UL transmission should not cause a Tx Switch

- 5 companies think that a skipped UL transmission should cause a Tx Switch as if the skipping did not take place

It is also noted that the current specification can be understood to assume behaviour a). The fact that the gNB cannot know in time if a transmission was skipped or not (and thus what is the state of the UE transmitters) was said to be OK; if the gNB always assumes that there is a switch due to the skipped transmission, and assumes there is always a need for a switch after the skipped transmission, then UE implementation according to a) works fine. It is the moderator’s understanding that with that sort of network behaviour the UE can be implemented according to a) or b) and the end result is the same. Still, unless there is a strong motivation to define that both implementations are allowed, it maybe better just to conclude that

### Issue #2, Proposal after Round #1 (NEW)

Proposed conclusion:

- If an UL transmission is skipped due to UL skipping, from the UL Tx Switching perspective this is a non-transmission, i.e. a skipped transmission does not cause an UL Tx Switch

- No specification change needed

# Conclusion

To be written

# References

- R1-2300317, 38.214 CR draft on Clarification to the switching gap location of the UL Tx Switching, Nokia, Nokia Shanghai Bell

- R1-2300325	On the switching gap location of the UL Tx Switching	Nokia, Nokia Shanghai Bell

- R1-2301384	Clarification of Rel-16 UL Tx Switching	Qualcomm Incorporated