3GPP TSG-RAN WG1 Meeting #112	  R1-2301936

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.2.2.1

Source:	Moderator (Huawei)

Title:	Summary#1 for CSI evaluation of [112-R18-AI/ML]

Document for:	Discussion and Decision

Introduction

In RAN#94-e plenary meeting, a new SID on AI/ML for air-interface was approved for Rel-18 [1], where the use cases include CSI feedback enhancement, and evaluations would be performed for this use case:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This document summarizes the key issues discussed under agenda item 9.2.2.1, and aims to discuss a set of issues for the evaluation of the AI/ML-based CSI feedback enhancements in RAN1#112.

1st round email discussions

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table. Please update your contact information on top of the last meeting if needed.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Samsung | Ameha | amehat.abebe@samsung.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| ZTE | Lun Li | li.lun1@zte.com.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Intel | Victor Sergeev | victor.sergeev@intel.com |
| Ericsson | Mattias Frenne | Mattias.frenne@ericsson.com |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| AT&T | Isfar Tariq Salam Akoum | isfar.tariq@att.com salam.akoum@att.com |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Mavenir | Ali Fatih Demir | ali.demir@mavenir.com |
| NTT DOCOMO | Haruhi Echigo Liu Liu | haruhi.echigo.fw@nttdocomo.com liul@docomolabs-beijing.com.cn |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| IIT Kanpur | Abhishek Kumar Singh | Abhishekks@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| vivo | Jianming Wu | jianming.wu@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |

Generic issues on evaluation methodology

Summary of views from companies

### 2.1-1: Remaining issues of the EVM table

N/A

### 2.1-2: Metrics

- SGCS

- Rank>1 case

  - Method 1: Average over all layers: CAICT,

SGCS=E1N×Ki=1Nj=1K||wijHwij ||||wij||  ||wij||2

  - Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

    - CAICT: For SGCS calculation for rank>1, method 1(average over all layers) is proposed as baseline

      - It is difficult to find a unified method for GCS/SGCS calculation with different weights for different layers.

  - Method 2: Weighted average over all layers: Samsung, CATT, Fujitsu,

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

    - Fujitsu: For example, consider the case that we want to compare the performance of two pairs of layer-common two-sided AI/ML model for the case that rank = 2. The SGCSs for the two layers are x1 and x2 for AI/ML model 1, and are y1 and y2 for AI/ML model 2, which satisfy y1>x1>x2>y2. In this case, it is difficult to decide which of the two AI/ML models is better by Method 3 only.

- Views of companies

  - Not to adopt an additional method except for Method 3: Huawei, Hisilicon, Apple,

    - Huawei, Hisilicon: For the SGCS calculation under rank>1 cases, no additional method needs to be introduced except Method 3 (SGCS is separately calculated for each layer) as already adopted

    - Apple: For rank>1, report SGCS separately for each layer. No need to define average or weighed average (i.e., method 1 and method 2) for higher rank

  - Whether Method 1 or Method 2 is used is up to companies OPPO, LG AT&T

    - OPPO: For rank>1, Method 3 is selected for calibration, whether Method 1 or Method 2 is used is up to companies

    - LG: For generating SGCS when rank>1, Method 1 or method 2 can be optionally reported by company

    - AT&T: For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, company can optionally report either Method 1 or Method 2 as an additional metric

  - Method 1 is adopt as an additional method except for Method 3: CAICT

    - CAICT: For SGCS calculation for rank>1, method 1(average over all layers) is proposed as baseline

  - Method 2 is adopt as an additional method except for Method 3: CATT, Samsung, Fujitsu, FUTUREWEI

    - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement, adopt SGCS for rank>1 as weighted average over all ranks

    - Fujitsu: For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, normalized weighted SGCS should be selected as an intermediate KPI

    - CATT: For the evaluation of AI/ML based CSI feedback enhancement, as the intermediate KPI for rank>1 cases, if a down-selected metric between the following two methods is adopted, Method 2 is preferred

      - A layer corresponds to a larger eigenvalue is expected to have higher contribution than a layer corresponds to a smaller eigenvalue. Therefore, for the calculation method of SGCS, Method 2 is preferred than Method 1.

    - FUTUREWEI: For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, besides the agreed-upon Method 3, SGCS results calculated using Method 2 (weighted average across all layers) may be optionally adopted by companies.

- Other intermediate KPIs

- Alt.2A: Numerical spectral efficiency ratio/relative achievable rate (RAR) (for rank >= 1): Ericsson, Lenovo Fraunhofer

RARγ,L=EH1Ff=1FlogdetI+γPfHHfHHfPfEH1Ff=1FlogdetI+γPfHHfHHfPf

    - where Hf~[NRx×NTx] is the complex channel matrix for RB f

    - F is the total number of RBs,

    - Pf~[NTx×L] is the reported precoding matrix for RB f,

    - Pf~[NTx×L] is the optimal (SVD-based) precoding matrix for RB f,

    - L is the number of MIMO layers the KPI is evaluated for,

    - γ is the SNR-value;

  - Lenovo: As one intermediate KPI, to evaluate the efficiency of the estimated precoders, we suggest to use relative achievable rate (RAR) defined as

- RARγ=EHi1Bj=1Bk=1Klog21+γHijTvijk2 EHi1Bj=1Bk=1Klog21+γHijTvijk2

    - where γ is the SNR and channels are normalized, i.e.,  E|Hi|=1 is assumed to be normalized. For simplification, RARγ can report at γ=1, i.e.,

- RAR1=EHi1Bj=1Bk=1Klog21+HijTvijk2 EHi1Bj=1Bk=1Klog21+HijTvijk2

    - Note also, if the metric is intended for comparison between different implementations which use the same “H”, then the denominator can be removed (as it will be the same between different models).

  - Fraunhofer: If explicit CSI is not provided at the gNB, the metrics based on spectral efficiency (differential or relative) are proposed for performance evaluation and comparison.

- Other intermediate results are optional/not needed:

  - Huawei, HiSilicon: For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which has been agreed as the baseline metrics, other intermediate KPIs are not considered as baseline for metrics, and can be optionally considered and reported by companies

  - OPPO: For intermediate KPI, use SGCS as the evaluation metric for calibration

    - Other intermediate KPIs are not suggested

  - Samsung: RRSNR as intermediate KPI may be advantageous on providing better emulation of the MU-MIMO DL throughput performance than GCS in some receiver assumptions. If the UE performs inter-layer interference nulling, this advantage is lost.  This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG

  - Samsung: In MU-MIMO scenario, when the gNB do not directly apply the reconstructed precoder, RRSNR does not give a practical advantage in terms of emulating the DL MU-MIMO throughput performance than SGCS. This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG

  - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement, deprioritize discussion on additional intermediate KPIs

  - LG: Intermediate KPI other than SGCS and NMSE is not necessary

- Provision of upper bound with ideal CSI

- Huawei, HiSilicon: For the evaluation of CSI enhancements, the additional baseline based on ideal CSI (e.g., ideal eigenvector) can be introduced, which is taken as an upper bound for performance comparison.

  - With the same overhead of CSI feedback and in terms of mean UPT for rank 2, compared to the mean UPT corresponding to the ideal CSI

    - Rel-16 Type II codebook has 37%/25%/19% margin under the payload size of 80/163/341 bits, respectively

    - AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin under the payload size of 80/163/341 bits, respectively

    - AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin under the payload size of 80/163/341 bits, respectively

- Ericsson: Companies are encouraged to provide optional genie based upper bound performance metrics obtained using ideal CSI per subband

- ZTE: Throughput baseline based on ideal CSI (e.g., ideal eigenvector) should be adopted as an additional throughput baseline at least for calibration purpose

  - For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is quite small for average UPT, as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling

  - For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is 7% for 5% tail UPT

  - With regard to up to rank 2, for average UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 6% under the case of 50% RU, and 11% under the case of 70% RU.

  - With regard to up to rank 2, for 5% tail UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 30% under the case of 50% RU, and 42% under the case of 70% RU.

  - With regard to up to rank 4, for 5% tail UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 30% under the case of 50% RU, and 55% under the case of 70% RU.

  - With regard to up to rank 4, for average UPT, the margin of AI-based CSI recovery to ideal CSI feedback is 28% under the case of 50% RU, and 32% under the case of 70% RU.

  - With regard to up to Rank 4, for average sector throughput, the margin between AI-based CSI recovery and ideal CSI feedback is 22% under full buffer traffic.

  - With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 48% under full buffer traffic.

- FUTUREWEI: For the evaluation of CSI enhancements, companies may optionally provide the additional throughput upper-bound baseline assuming ideal CSI (e.g., ideal eigenvector) is available

- AT&T: For the evaluation of CSI enhancements, companies to provide the additional throughput baseline based on ideal CSI (e.g., ideal eigenvector), which is taken as an upper bound for performance comparison

- LG: UPT upper bound assuming ideal CSI can be optionally reported by company

- Eventual KPIs

- Throughput per complexity unit: MediaTek

  - MediaTek: To appreciate low-complex, yet high-performing, AI/ML model designs, a KPI measuring throughput per complexity unit would be beneficial.

    - We suggest defining new KPIs incorporating both throughput and complexity to appreciate AI/ML models with a balanced design, such as Throughput per FLOPs, Throughput per MACs, or Throughput per number of trainable parameters

- Additional baseline ZTE

  - ZTE: Throughput baseline of non-AI/ML approaches with enhancements on CSI feedback (e.g. Wideband (WB) covariance matrix, full rank information) can be considered as an additional baseline for performance comparison

- Capability/complexity related KPIs

- Processing complexity

  - Ericsson: for FLOPs we present the per-layer count followed by (an upper bound in parenthesis) where the latter is counted as executing all 6 different MIMO-layer models for encoders and as executing 4 different MIMO-layer models for decoders; and for number of parameters and storage we present the total number for all 6 different MIMO-layer models.

  - NVIDIA: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for CSI feedback enhancements

    - Increasing hardware performance can support successively more complex AI/ML models. For example, GPU inference performance has improved by 317x in 8 years (2012-2020), more than doubling each year.

  - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement, while reporting the computational complexity of pre-processing and post-processing the following are considered

    - Estimated raw channel matrix per each frequency unit as an input for pre-processing

    - Precoding vectors per each  frequency unit as an output of post-processing

    - Note: frequency unit can be set to 4 RB or 2 RB for 15KHz SCS and 30KHz SCS, respectively

- Number of training samples

  - MediaTek: Number of training samples to reach a certain performance KPI can itself be used as a KPI to quantify trainability of AI/ML models

### 2.1-3: Dataset related issues

Source of dataset

- MediaTek: For generalization study, the mixed datasets should be subject of further discussions to determine the exact contribution of each sub-dataset into the mixed one according to real-world settings.

- MediaTek: Considering the potential number of mixing dimensions, offer public datasets to facilitate study on generalization aspects of AI/ML models

- OPPO: Companies are encouraged to disclose their utilized dataset(s) and reference model(s)

  - FFS: to establish common dataset(s) and/or reference model(s) for performance calibration and drawing final conclusions.

- NVIDIA Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for CSI feedback enhancement

### 2.1-4: Other views/findings

1st round email discussions

### 2.2-1: Metrics

- Issue#2-1 (High priority) SGCS calculation for rank>1 – additional method

Moderator note: 2 companies prefer not to adopt additional method except for Method 3 which is already agreed. 3 companies prefer to additionally adopt Method 2 or Method 3 up to companies. 1 company prefer Method 1, 4 companies prefer Method 2.

Based on the above inputs as well as the polling of the last meeting, let’s try the following three options: Method 2 is additionally adopted, or one additional method between Method 1 and Method 2 is optionally adopted up to companies, or we have no consensus on this issue.

Proposal 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, for the other two methods:

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

One of the following option is preferred:

- Option 1: Company can report Method 2 as an additional metric.

- Option 2: Company can optionally report either Method 1 or Method 2 as an additional metric.

- Option 3: There is no consensus on whether to adopt an additional method.

| Option 1 | Support/Can accept | CATT, Fujitsu |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | Vivo, Futurewei, OPPO, NTT DOCOMO, LG Electronics, AT&T |
|  | Object/Concern | Qualcomm |
| Option 3 | Support/Can accept | MediaTek, vivo, IIT Kanpur, CATT, Futurewei, ETRI, ZTE, LG Electronics, Huawei/HiSi, Spreadtrum, Nokia/NSB, Intel |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | This issue has lasted quite long time. If it is still controversial at this meeting, then it is Moderator’s feeling that Option 3 is the natural outcome on this issue, since we are going to complete the template and collect results from companies. |
| MediaTek | We don’t think additional KPIs are necessary |
| Lenovo | We are okay with option one, but also okay not to include this metric and report the metric per layer or use RAR instead of that. |
| vivo | This issue is not that crucial, and we should not spend much time on it. We are supportive of Option 2, but if it is still controversial, we can take Option 3. |
| IIT Kanpur | As with agreed method 3, SGCS for individual layers is already calculated hence interpretation of this information is implementation specific and hence there is no consensus on adopting an additional method out of method 1 and method 2. |
| CATT | Support Option 1 but can live with Option 3. |
| Futurewei | Our view is that Option 2 is acceptable, however, if it’s still controversial, then there is no consensus on this topic. |
| ETRI | We share same view with MediaTek. |
| OPPO | We think option 1 and option 2 can be optionally reported. |
| NTT DOCOMO | In our evaluation results, we adopted Method 1 for simplicity. According to discussion at the last meeting, it seems more companies do not want any additional method except for Method 3(independent calculation for each layer). Then, we are fine with it. |
| Qualcomm | If it is left optional to companies and there is no agreement on a common method that results in a single metric, then it would be difficult to compare the results across companies. |
| ZTE | We think it is not necessary to adopt additional metrics. |
| LG Electronics | Support option 2, but can live with option 3. |
| AT&T | We support Option 2. |
| Fujitsu | Our view is that an additional SGCS is needed, and Method 2 is preferred. Method 3 by itself is not enough as an intermediate KPI. A single number, rather than multiple ones, is preferred as an intermediate KPI. For example, in the case of rank = 2, the SGCSs for the two layers are x1 and x2 for model #1, and those for model #2 are y1 and y2, which satisfy y1>x1>x2>y2. In this case, it is difficult to decide which of the two AI/ML models is better by Method 3 only. In Method 2, the layers are weighted differently by their associated singular values. While in Method 1, they are weighted equally. We think that it is more reasonable to have them weighted differently. The reasons are as follows. On one hand, simulation results show that the SGCS of the layer with a large singular value is larger than that with a small singular value. On the other hand, the layer associated with a larger singular value appears more frequently because the probability of scheduling UEs with a small number of layers is much bigger than more layers. So the layer with a larger singular value should be weighted higher, which is characterized in Method 2 but not Method 1. |

- Issue#2-2 (High priority) Other intermediate KPIs-motivation

Moderator note: In the 110bis-e meeting, we have achieved a working assumption on the layer disorder issue, when SGCS is adopted as the intermediate KPI for the rank>1 case. Two meetings have passed since then, and as far as Moderator knows, there is no simulation results observing the disorder issue. So, let’s try to confirm the working assumption and close this issue.

Proposal 2.2.2: Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

| Company | View |
| --- | --- |
| Lenovo | Agree |
| vivo | Agree. It should be confirmed. |
| CATT | Support. |
| Futurewei | Agree. |
| ETRI | We agree |
| OPPO | Agree |
| NTT DOCOMO | We support to confirm the WA. |
| ZTE | Agree. |
| LG Electronics | Support |
| Huawei/HiSi | Agree |
| Spreadtrum | Support |
| Nokia/NSB | Ok |
| AT&T | Support |
| Fujitsu | Agree. |

- Issue#2-2a (High priority) Other intermediate KPIs-options

3 companies propose to adopt RAR as the additional metric other than SGCS/NMSE which have been agreed, with the motivation of 1) having a closer metric to the network throughput [27], and 2) SGCS gains are huge for layer 3 and 4 while corresponding RAR gains are modest [4]. 4 companies prefer not to introduce other intermediate KPIs.

The intention of the following proposal is to provide two options: one is to adopt RAR as the additional metric for companies to report, while the other is not to consider additional intermediate KPI other than SGCS/NMSE.

- Note: As there is limited input from other companies for how to set the parameter γ to complete the formula of RAR, the value of γ directly refers to Lenovo’s contribution [27]. If you agree on the formula of RAR but have a different view on the parameter value, please also provide your comments in the table.

Proposal 2.2.3: For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, consider one of the following options:

- Option 1: RAR is adopted as the additional intermediate KPI

  - Relative achievable rate (RAR)

RARγ,L=EH1Ff=1FlogdetI+γPfHHfHHfPfEH1Ff=1FlogdetI+γPfHHfHHfPf

where Hf~[NRx×NTx] is the complex channel matrix for RB f

F is the total number of RBs,

Pf~[NTx×L] is the reported precoding matrix for RB f,

Pf~[NTx×L] is the optimal (SVD-based) precoding matrix for RB f,

L is the number of MIMO layers the KPI is evaluated for,

γ is the SNR-value;

  - the value of γ=1 is assumed for RAR calculation

- Option 2: NO additional intermediate KPI is adopted.

| Option 1 | Support/Can accept | Lenovo, Qualcomm (comment) |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | MediaTek, vivo, CATT, Futurewei, OPPO, NTT DOCOMO, ZTE, LG Electronics, Huawei/HiSi, Spreadtrum, Nokia/NSB, Fujitsu, Intel |
|  | Object/Concern |  |
| Option 3 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | This issue has lasted quite long time. If it is still controversial at this meeting, then it is Moderator’s feeling that there is NO CONSENSUS on the additional intermediate KPI, since we are going to complete the template and collect results from companies. |
| MediaTek | We do not support adopting new KPIs related to CSI reconstruction accuracy or expected throughput. |
| Lenovo | Option 1. We believe RAR could be a illustrative metric to show the more realistic benefit of AI/ML. In fact, since RAR considers “H”, it can show how much misalignment of the estimated precoder-vector reduces the achievable rate. |
| NTT DOCOMO | For the intermediate KPI, our opinion is that one KPI is enough for obtaining some preliminary observations. We should rely on system-level simulations for the final results. |
| Qualcomm | It may be useful to evaluate the metric for different ranges of SNRs. Hence, we suggest to add “Other values of γ are not precluded”. |
| ZTE | There is no need to adopt a new intermediate KPI for calibration. |

- Issue#2-3 (High priority) Provision of upper bound with ideal CSI

Moderator note: In this meeting, 5 companies raised that we can consider/optionally consider the simulation results based on ideal CSI. Besides, ZTE [5] and Huawei [3] have provided simulation results including the throughput with the ideal CSI.

Based on the views of the last meeting, and the inputs for this meeting, the same proposal of the last meeting is raised as below.

Proposal 2.2.4: For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on ideal CSI (e.g., ideal eigenvector), which is taken as an upper bound for performance comparison

| Support/Can accept | MediaTek, Lenovo, vivo, IIT Kanpur, CATT, Futurewei, OPPO, NTT DOCOMO, Qualcomm (comment), ZTE, LG Electronics, Huawei/HiSi, Spreadtrum, AT&T, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | This issue has lasted 3 meetings. If it is still controversial at this meeting, then it is Moderator’s feeling that there is NO CONSENSUS on the upper bound with ideal CSI, since we are going to complete the template and collect results from companies. |
| Lenovo | It is illustrative to optionally report the throughput upper-bound. So, we generally support this proposal. |
| vivo | We are fine with the throughput baseline. It is up to the companies for reporting. |
| IIT KANPUR | An additional throughput baseline based on ideal CSI can be provided by companies which is taken as upper bound for performance comparison |
| NTT DOCOMO | We support the proposal and provide the throughput baseline based on ideal CSI for this meeting. Throughput based on ideal CSI can show the upper bound of the performance from CSI accuracy. |
| Qualcomm | It would be helpful to clarify that “ideal CSI” in this proposal refers to CSI with ideal (i.e., lossless) compression, and that channel estimation should still be realistic. |
| ZTE | Since the legacy eType II approach shows average performance and differs from companies which is difficult to calibrate among companies, and the upper bound may not have too much difference, it could make cross checking on eventual KPIs (e.g. UPT, throughput) much easier. Furthermore, it is helpful to compare the AI performance with the ideal CSI feedback to know how much gain is left. From our perspective, we suggest additional throughput baseline based on ideal CSI (e.g., ideal eigenvector) at least for calibration purpose. |
| LG Electronics | Support. It is upto company. |

- Issue#2-4 (Medium priority) Clarification for processing complexity

Moderator note: In the 109-e meeting, we agreed in 9.2.2.1 that FLOPs is adopted as the metric to evaluate the complexity.

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

In 110b-e meeting, we agreed in 9.2.1 that the complexity includes pre/post-processing.

Agreement The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency)

In this meeting, Samsung [29] brings up that the idea that there may be some ambiguity on the boundary for the calculation of the pre-processing (see below).

[Samsung] However, while reporting the complexity of pre- and post-processing, the reference input and output format of the pre-processing and post-processing operations, respectively, could be ambiguous. For example, raw channel matrices or eigenvectors can be considered as input for pre-processing. In order to clarify this point we propose the following: Proposal #1: For the evaluation of the AI/ML based CSI feedback enhancement, while reporting the computational complexity of pre-processing and post-processing the following are considered. - Estimated raw channel matrix per each frequency unit as an input for pre-processing - Precoding vectors per each frequency unit as an output of post-processing Note: frequency unit can be set to 4 RB or 2 RB for 15KHz SCS and 30KHz SCS, respectively

A question is then raised accordingly:

Question 2.2.1: For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, do you agree that we need to clarify the boundary of calculating the complexity metric (i.e., FLOPs)? E.g.,

- For the input of the CSI prediction, or input of the CSI generation part, the pre-processing starts at the raw channel matrix (i.e., includes the decomposition from channel matrix to eigenvectors)

- For the input of the CSI reconstruction part, the pre-processing includes the dequantization.

- For the output of the CSI generation part, the post-processing includes the quantization.

- For the output of the CSI reconstruction part, the ends at the precoding vectors.

| Company | View |
| --- | --- |
| MediaTek | We are okay with the boundaries. |
| vivo | We believe that, clarifying the pre-processing and post-processing is necessary. We are fine with bullet-1 and 4, but not fine with bullet-2 and 3. This is because the pre-processing and post-processing, in general, are not involved during the training procedure. However, the quantization and dequantization are involved during the training procedure if we want to harvest all AI/ML gain. |
| IIT Kanpur | We agree on fact that there is need to clarify the boundary of calculating the complexity metric along with need to mention the complete block chain eg. Block chain for (1 BS , 1UE) and for (1Bs, M UE) may require an additional pre or post processing blocks. This will be helpful in standardizing and also comparing Processing Chain. |
| OPPO | We are general okay about this question, but our question is how to evaluate and report the computational complexity of pre-processing and post-processing (e.g. eigenvector decomposition from raw channel). |
| NTT DOCOMO | We agree to clarify the boundary. |
| AT&T | We support the proposal to have clarify the boundary for computing complexity metric. |
| Fujitsu | We share a similar view to that of OPPO. |

### 2.2-2: Others

Question 2.2.2: Do you think there are additional high priority issues or EVM parameters which are generic to all sub use cases and have not been discussed/captured in previous sub-sections?

| Company | View |
| --- | --- |

Specific evaluation methodology for CSI compression sub use case

Summary of views from companies

### 3.1-1: Evaluation findings from companies

CSI compression

- Findings on CSI compression

- Findings on spatial-frequency domain CSI compression

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 7%-10.3% gain on rank 1 under full buffer traffic.

    - 8.9%-14.8% gain on rank 2 under full buffer traffic.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 2.2%-2.5% gain on rank 1 under FTP traffic and 50% RU.

    - 3.2%-7.8% gain on rank 1 under FTP traffic and 70% RU.

    - 4.8%-8.8% gain on rank 1 under FTP traffic and 80% RU.

    - 5.4%-10.1% gain on rank 2 under FTP traffic and 50% RU.

    - 7.2%-14.4% gain on rank 2 under FTP traffic and 70% RU.

    - 7.5%-14.5% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 2.2%-7% gain on rank 1 under FTP traffic and 50% RU.

    - 6.4%-14.6% gain on rank 1 under FTP traffic and 70% RU.

    - 7.1%-16.3% gain on rank 1 under FTP traffic and 80% RU.

    - 4.3%-13.3% gain on rank 2 under FTP traffic and 50% RU.

    - 8.1%-18% gain on rank 2 under FTP traffic and 70% RU.

    - 9.1%-23.7% gain on rank 2 under FTP traffic and 80% RU.

  - Qualcomm AI/ML based CSI compression gives UL overhead gains of around 50% for mean throughputs (at a mean throughput around 14.4 Mbps) and 25% for cell-edge throughputs (around 2.6 Mbps).

  - Qualcomm For Bursty Traffic

    - At 10% RU, we see a 65% improvement in UL overhead at a mean user experience of around 150 Mbps. At similar improvement in UL overhead, the edge user experience is around 80 Mbps.

    - At 40% RU, we observe a 40% improvement in UL overhead at a mean user experience of 88 Mbps. At an edge user experience of around 38 Mbps, the gain in UL overhead is 55%.

    - At 80% RU, we observe a 63% improvement in UL overhead at a mean user experience of around 45 Mbps, and an edge user experience of around 11 Mbps

  - Ericsson:	The upper bounds of the gain (over eType-II ParComb 1) of CSI compression for the presented encoder-decoder pair configuration is 3.4% in rank-2 mean RAR and 7.0% in rank-4 mean RAR.

  - Ericsson:	Initial tests shows that a decoder-heavy scenario is more acceptable than an encoder-heavy scenario, but neither are as good as a well-balanced (and better hyper-parameter tuned) model.

  - Apple: Transformer based AE can achieve better SGCS consistently comparing to type II codebook. Around 40% overhead reduction is observed

  - ZTE: For rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the same feedback overhead, AI-based CSI recovery can obtain 7%-8% SGCS gains

  - ZTE: For Rank 1, AI-based CSI reconstruction achieves minor average UPT gain, where less than 1% average UPT gain is obtained over Rel-16 eTypeII with the same feedback overhead

  - ZTE: For rank 1, AI-based CSI reconstruction achieves 2% ~ 20% performance gain for 5% tail UPT, and the margin of AI-based CSI recovery to ideal CSI feedback is 7%.

  - ZTE: For up to rank 2, with the same feedback overhead, AI based CSI reconstruction has about 10%-37% performance gain for 5% tail UPT over the Rel-16 eType II under the case of 50% RU and 10%-25% performance gain for 5% tail UPT can be obtained by AI based CSI recovery under the case of 70% RU.

  - ZTE: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

  - ZTE: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU

  - ZTE: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 7%-13% performance gain for 5% tail UPT over the Rel-16 eType II under the case of 50% RU and 13%-22% performance gain for 5% tail UPT can be obtained by AI-based CSI recovery under the case of 70% RU.

  - ZTE: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank 4.

  - ZTE: For up to Rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 9%-14% average sector throughput gains over the Rel-16 eType II under full buffer traffic.

  - ZTE: For up to Rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 4%-7% performance gain for 5% tail UPT over the Rel-16 eType II under full buffer traffic

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, AI/ML models achieve 5.43% GCS gain over eType II codebook in terms of CSI feedback accuracy. The GCS gain ranges from 3.75% to 6.47% for 100~300 bits of CSI feedback.

  - MediaTek: At 0.85 GCS, the AI/ML model is able to approximately reduce feedback overhead by 36%.

  - Nokia: The transformer-based CSI compression model outperforms baseline Rel-16 eTypeII codebook performance based on the SGCS metric.

  - Xiaomi: AI based CSI compression shows around 23% performance gain on average SE compared with eType2 codebook

  - CATT: Compared to Rel-16 Type II codebook based CSI feedback, obvious performance gain can be achieved by CSI feedback with proposed scalable AI/ML model for rank=1, 2, 3, 4:

    - SGCS can be improved by 0.02~0.22 under the same CSI feedback payload;

    - Payload can be saved by 30%~60% bits under the same SGCS.

  - NTT DOCOMO: AI approach could get closer to the upper bound than Rel-16 Type II codebook. Also, AI approach is closer to the upper bound performance when traffic load is low, and payload is high

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same or similar number of feedback bits.

    - With the same or similar number of feedback bits, AI based approach could obtain 4%~40% performance gain over traditional codebook in the square of generalized cosine similarity

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits when achieving the same CSI or higher accuracy

    - With similar performance in the square of generalized cosine similarity, AI based approach could reduce 30%~60% feedback bits

  - InterDigital: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of SGCS

  - InterDigital: The AI/ML-based CSI compression using transformer model achieves a favorable performance-complexity-memory tradeoff

  - InterDigital: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of average throughput

  - InterDigital: The SGCS gain of the Transformer-based model over the CNN-based model (CsiNet) is 28% while the throughput gain is only 5%. The significant gap between the throughput and SGCS gain may suggest that using SGCS as an intermediate KPI is not a good indicator of the throughput gain

  - CAICT: From preliminary results, AI based spatial-frequency domain CSI compression shows good SGCS performance at least for rank=1

  - China Telecom: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same feedback bits.

  - China Telecom: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits with the similar CSI accuracy.

  - ETRI: With an Autoencoder using a previously developed neural network structure, CsiNet, there are significant improvements in terms of SGCS compared to the baseline (eTypeII) in CSI compression sub use case.

  - ETRI: With the PCA-based AI model, there are significant improvements in terms of SGCS compared to the baseline (eTypeII) and AE-based AI model in the CSI compression sub-use case

  - Intel: ML based Autoencoder can outperform Rel-16 eType II codebook for Layer 1 and Layer 2 case in all overhead regimes for InH, Dense Urban Macro and Dense Urban Micro deployments

  - Intel:

    - Up to 13% gain for average UE throughput and up to 17% gain for cell-edge UE throughput can be achieved for AI-ML CSI comparing to eType II PMI codebook

    - Up to ~100 bits overhead reduction can be achieved using the AI-ML CSI with the same performance as for eType II PMI codebook

  - Spreadtrum: The AI based CSI compression and recovery can achieve better SGCS performance and lower feedback bits cost than Rel-16 eType II codebook

- Findings on spatial-frequency domain CSI compression with additional past CSI (Temporal-spatial-frequency domain)

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression with additional past CSI as input can outperform Rel-16 Type II codebook with

    - 18.3%-25.4% gain on rank 1 under full buffer traffic.

    - 23.3%-30.2% gain on rank 2 under full buffer traffic.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

    - 7.7%-14.9% gain on rank 1 under FTP traffic and 80% RU.

    - 16.6%-28.6% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

    - 10%-28.4% gain on rank 1 under FTP traffic and 80% RU.

    - 17%-39.2% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT for rank 2, compared to the mean UPT corresponding to the ideal CSI,

    - Rel-16 Type II codebook has 37%/25%/19% margin under the payload size of 80/163/341 bits, respectively.

    - AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin under the payload size of 80/163/341 bits, respectively.

    - AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin under the payload size of 80/163/341 bits, respectively.

  - MediaTek: Model with historical channel/latent vectors as input in both encoder and decoder can learn the time-correlated feature and thus get better performance

- Findings on capability-related KPI

  - Samsung: The number of FLOPs to perform the AE operations is much larger than eType II

- Preliminary trends observed by more than one company

  - Different effects of AI/ML on different layers: The benefit of AI/ML method is more/less obvious in higher rank CSI compression due to better efficiency on compression than legacy Type II. Huawei, Hisilicon, Ericsson, ZTE, OPPO InterDigital

    - Huawei, Hisilicon: For the rank=2 case, AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more gains over the Rel-16 Type II on the 2nd layer than on the 1st layer.

    - Ericsson:	It’s notable that SGCS gains are huge for layer 3 and 4 while corresponding RAR gains are modest.

    - ZTE: For Rank 4, AI-based CSI reconstruction shows performance gain in SGCS per layer over the Rel-16 eType II

    - ZTE: For Rank 4, AI-based CSI reconstruction shows larger performance gains in Layer 3/4 than Layer 1/2 in terms of SGCS with the assumption of the same feedback overhead of each layer.

    - ZTE: The case of rank>1 should be prioritized in future study.

    - OPPO: Compared to rank 1 achieving 5%~8% SGCS gain and 1%~3% SLS throughput gain, AI based CSI feedback achieves 8%~16% SGCS gain and 4%~10% SLS throughput gain for rank 2.

    - InterDigital: For the rank two case, the AI/ML-based CSI compression using transformer model achieves higher SGCS than the Rel-16 Type II with 20% reduction in overhead. Also, the achievable gain over Rel-16 Type II on the first layer is higher than that of the second layer

  - Performance of different layers: 1st layer has higher SGCS than later layers Huawei, Hisilicon, ZTE, MediaTek CATT

    - Huawei, Hisilicon: For the rank=2 case, the 1st layer achieves higher SGCS than that of the 2nd layer as the eigenvectors of the 2nd layer are more sparse

    - MediaTek: For rank-2 channels in Dataset 1-Dataset 3, EVs of layer 0 are more correlative across frequency and antenna domains compared to EVs of layer 1.

  - Impact of CSI payload size: Compared to higher feedback overhead, AI based CSI feedback achieves larger gain with lower feedback overhead.

    - OPPO: Compared to higher feedback overhead achieving 1% for rank 1 and 4% for rank 2 SLS throughput gain, AI based CSI feedback achieves larger SLS throughput gain with lower feedback overhead, about 3% for rank 1 and 10% for rank 2

  - Impact of traffic load/RU: AI/ML approaches show better throughput performance over Rel-16 Type II CB with heavy traffic load/full buffer. Huawei, Hisilicon, ZTE, OPPO,

    - Huawei, Hisilicon: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook on high RU than low RU

    - ZTE: For up to rank 2, with the same feedback overhead, AI based CSI reconstruction has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

    - ZTE: For up to rank 4, with the same feedback overhead, AI-based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU.

    - ZTE: AI/ML approaches show better average throughput performance with heavy traffic load for up to rank 2

    - ZTE: AI/ML approaches show better throughput performance with heavy traffic load for up to rank 4.

    - OPPO: Compared to FTP model achieving 1%~3% SLS throughput gain for rank 1, AI based CSI feedback achieves larger SLS throughput gain for full buffer model about 3%~6%.

  - Effects on cell edge UEs: AI/ML can achieve more gains for cell edge UEs (than average). Huawei, Hisilicon,

    - Huawei, Hisilicon: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook for cell edge UPT than average UPT

Other views/findings

- NVIDIA: AI/ML based algorithms for CSI compression (e.g., using autoencoders) should be selected as a sub-use case for evaluation

- BJTU: The performance, FLOPs and parameters of the joint channel estimation and CSI compression method are better than the separated channel estimation and CSI compression method

### 3.1-2: AI/ML training methods

- General findings/views

- Lenovo: For FDD systems with network-based model training as well as Type-3 training collaboration, siginaling the CSI training data from the UE to the network is needed.

- Lenovo: Evaluate schemes related to transfer of CSI-dataset for different stages of the LCM

- Lenovo: Evaluate the following CSI training data signaling techniques

  - Alt1. Proprietary signaling via non-3GPP techniques

  - Alt2. Legacy CSI-dataset feedback where the NR codebook-based CSI is utilized as CSI training data

  - Alt3. Explicit CSI-dataset feedback via enhanced 3GPP-based signaling of the CSI training data

- Fraunhofer: The same AI-model can be trained by type 1 or type 2, therefore type 1 and type 2 training may result in the same performance. However, in type2, two vendors located at different places are used for the training, this causes a lot of offline overhead for data exchange and synchronization

- Fraunhofer: If the confidentiality of the details of the two-sided model is crucial, which means the UE or gNB vendors concern about revealing their data and/or implementation to each other, type 3 is the only applicable option

- Fraunhofer: Some of the AI models, which require joint training, cannot be implemented using separate training, therefore type 3 or any separate training method does not outperform the joint training approaches e.g., type 1, 2

- Fraunhofer: Separate training type 3 can be applied when there is no full coordination between the UE and gNB vendors

- Unmatched/restricted model pairing

- Huawei, Hisilicon: For the performance comparison of training Type 2/3, the learning capability difference of model backbone itself should not be attributed to the potential performance loss due to the specific training behaviors.

- Ericsson: Unmatched models are defined as the case when decoder and encoder use different Neural Network architectures (e.g. CNN and Transformer respectively)

- Ericsson: Unbalanced two sided models are defined as when two ML-models have significantly (>4x) different sizes of their Neural Network architectures in terms of FLOPS and/or number of parameters. Unbalanced models are further categorized into “encoder heavy” and “decoder heavy” when the two models in question are an encoder and a decoder

- Ericsson: Companies are encouraged to investigate the performances of encoder vs decoder heavy models and first vs second heavy models as well as balanced models especially for Type 3 training and for Type 2 training with the relation to UE first vs NW first training and the multi-vendor training cases

- MediaTek: Report UE’s gain/loss and gNB’s gain/loss separately for unmatched encoder-decoder pairs in any training strategy

- MediaTek: To measure the performance loss/gain achieved by a training strategy for a certain encoder(s)-decoder(s) setting, use the performance of the same setting with Type 2 (joint) training as the baseline

- MediaTek: For more than one encoder or decoder, use performance of single-encoder single-decoder with the same training type to measure how the number of encoders/decoders affect the performance of final AI/ML AEs in inference

- Apple: When mis-matched model is used, a simple fully connected encoder model with one hidden layer, together with transformer-based decoder, perform better than other combinations

- Lenovo: Although smaller AI/ML models have lower overhead, they may experience performance loss when they are trained for all different UE-link types

- Lenovo: Study the performance of AI/ML models for more restrictive scenarios/configurations, e.g., different UE-link-types (UE groups).

- Boundary between Type 2 and Type 3

- General views/findings

  - Huawei, HiSilicon: The boundary between Type 2 and Type 3 is whether the information exchange is required in each FP/BP loop or not:

- Type 2 training requires information exchange in each FP/BP loop.

- Type 3 training doesn’t require information exchange in each FP/BP loop.

  - Huawei, HiSilicon: If one side is trained first, and the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, it belongs to Type 2 training.

  - Ericsson: Extend the Type 2 training definitions so it implies that gradient is passed between the sides while in Type 3 training, the latent space variable is passed between the sides

- Frozen after trained

  - Ericsson: There is no noticeable performance degradation on using frozen/non-trainable decoder compared to the case of both encoder and the decoder are trainable

  - Qualcomm

    - gNB-first type3-alt training: gNB-Dec is trained first and then UE-Enc is trained with a frozen gNB-Dec

      - This approach may require exchange of activation (latent vector), gradient and corresponding target for loss function computation

    - UE-first type3-alt training: UE-Enc is trained first and then gNB-Dec is trained with a frozen UE-Enc

      - This approach may require exchange of activation (latent vector) and corresponding input

- Descriptions/Findings for Type 1

- Huawei, Hisilicon: For both joint training Type 1 and joint training Type 2, compared to the ideal model pairing, suboptimal performance may be suffered under restricted model pairing due to the non-cooperative model structure development or misaligned target performance metric between the Network vendor and the UE vendor

- vivo: Study the performance and overhead of per-cell (region) model transfer in CSI compression

- Descriptions/Findings for Type 2

- General views/findings

  - Huawei, Hisilicon: For the evaluation of Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), analyze the feasibility for the following two cases of N>1 NW part models to M>1 UE part models (i.e., Case 4), and companies to report the training method/order, including the preference and details to achieve:

    - Case A: Simultaneously train all N Network part models and M UE part models involving all vendors

    - Case B: Sequentially train N Network part models and M UE part models in a pairwise manner

    - Note: The loss function should be separately generated for N>1 NW part models.

  - Huawei, Hisilicon: For Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), challenges are observed for both of the two training methods/orders of Case A (Simultaneously train all N Network part models and M UE part models involving all vendors) and Case B (Sequentially train N Network part models and M UE part models in a pairwise manner)

    - For Case A, there may be challenges on aligning the training timelines of all involved Network vendors and UE vendors.

    - For Case B, fairness may be harmed for later trained Network-UE pairs where only one side may be able to update parameters during the joint training.

- Ericsson: For both Type 2 and Type 3 training, also evaluate the case with N>1 and M>1

- Samsung: For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), for multi-vendors training Case 2, i.e., one NW part model and M>1 separate UE part models, and Case 3, i.e., one UE part model and N>1 separate NW part models, evaluate and compare the following two training scenarios

  - Scenario 1: training in a single session

  - Scenario 2: training in multiple sequential training sessions, i.e, one UE part and one network part at each training session.

- Different structures between NW part and UE part (NW and UE are not aware of the AI/ML model of each other) Huawei, Hisilicon, vivo, Qualcomm, MediaTek,

  - Huawei, Hisilicon: Different backbones: 1 NW part model (TF) to M=3 UE part models (TF#1, TF#2, CNN). Same backbone but different structures: N=2 NW part models to M=2 UE part models all with TF but different number of layers.

  - vivo: the basic model structures for CSI generation parts are Transformer, CNN, and MLP, while a Transformer CSI reconstruction part is adopted

  - MediaTek: Define a mechanism/threshold to identify and avoid certain vulnerable pairings of encoders and decoders

Findings

  - Qualcomm: Type 2 offline training of the UE-side model and NW-side model is feasible even if the ML model structure of the UE-side and NW-side models are not matched.

  - MediaTek: UE and gNB vendors equally suffer from the performance loss of the unmatched pairs in the joint training. In average for all unmatched pairs, UE losses 2.23% performance and gNB losses 2.26%

  - MediaTek: Overall, joint training on all pairs caused 1.68% performance loss

- One common NW part at NW to M>1 UE parts of different UEs Huawei, Hisilicon, Ericsson, vivo, Qualcomm, MediaTek

Solutions/views

  - Huawei, Hisilicon: M=3.

  - Ericsson: M=2.

  - vivo: M=3.

  - Qualcomm: M=3

  - MediaTek: M=4

Findings

  - Huawei, Hisilicon: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 1.1%~6.5% for one Network part model to M=3 UE part models with different backbones or different structures

  - vivo: One common CSI reconstruction part could be trained to match multiple CSI generation parts of different UEs in training collaboration type 2 at the cost of some performance loss

  - vivo: Considering one common CSI reconstruction part matching three CSI generation parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduces from 0.075 to 0.052, i.e., losing about 30% performance gain.

  - Qualcomm: It is feasible to use Type 2 offline training to train a common NW-side model together with separate UE-side models without any performance impact when compared to training a separate NW-side model for each UE-side model.

  - MediaTek: With training type 2, multi-encoder single-decoder setting have inferior performance compared to single-encoder single-decoder setting.

  - MediaTek: In pair-to-pair comparison with joint training strategy of single-encoder single-decoder setting, using four encoders (instead of one) causes 0.7% performance loss.

  - MediaTek: Employing multi-encoder training strategies, UEs and gNBs lose ~2.4% performance in average.

  - MediaTek: In both multi-decoder and multi-encoder training strategies, matched encoder-decoder pairs may promote themselves at the cost of degrading the performance of unmatched encoder or decoder.

- N>1 NW parts of different NWs to one common UE part at UE vivo, MediaTek

Solutions/views

  - MediaTek: Assign higher priority to single-encoder multi-decoder setting compared to multi-encoder single-training with joint training strategy.

  - MediaTek: N=4

  - vivo: N=3

Findings

  - vivo: One common CSI generation part could be trained to match multiple CSI reconstruction parts of different networks in training collaboration type 2 at the cost of some performance loss

  - vivo: Considering one common CSI generation part matching three CSI reconstruction parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.061, i.e., losing about 19% performance gain.

  - MediaTek: In single-encoder multi-decoder training strategy, we observe 0.3% performance loss compared to the joint training. Also, degradation for both UE and gNB vendors are ~2.0%.

  - MediaTek: In both multi-decoder and multi-encoder training strategies, matched encoder-decoder pairs may promote themselves at the cost of degrading the performance of unmatched encoder or decoder

- N>1 NW parts of different NWs to M>1 UE parts at UE Huawei, Hisilicon, Ericsson

Solutions/views

  - Huawei, Hisilicon: all of the 4 models (UE 1, UE 2, Network 1 and Network 2) are jointly trained using simultaneous training method, i.e., Case A as discussed in Section 2.3.3.

  - Ericsson: N=2,M=2; N=2,M=3

Findings

  - Huawei, Hisilicon: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 0.1%~2.6% for N=2 Network part models to M=2 UE part models with different structures.

  - Ericsson: Using intermediate KPIs, the joint multi-vendor training (Type 2) seems to come with some minor performance deterioration compared to the single vendor (N=M=1) case.  With N=2 and M=2, the performance degradation in SGCS for layer 1 is in the order of 0.6% to 1.4%, for layer 2 in the order of 1.5% to 2.3%. The performance in rank-2 RAR decreased in the order of 0.9% to 1.2%.

  - Ericsson: The performance deterioration for multi-vendor training cannot be directly connected to the number of jointly trained vendors. In our tests. The case N=2 and M=3 gives marginally better results than N=2 and M=2. Moreover, the former is even decoder-heavy, as the third encoder included in the training is an order of magnitude smaller than the decoders, and even than the other encoders

- Loss function generation of N>1 CSI reconstruction parts of different NWs

  - MediaTek: Discuss how to calculate a joint loss to avoid adverse bias toward the matched pairs

    - In our pilot study, we use a joint loss that is simply calculated by averaging over all individual losses of gNBs

    - there are cases that the matched encoder-decoder pairs promote themselves at the cost of degrading the performance of unmatched encoders or decoders. In this regard, we believe a simple average has the risk of biasing the common encoder/decoder to its matched decoder/encoder.

  - Ericsson: This produces four different reconstructions, corresponding to the combinations (EncA – DecA), (EncA – DecC), (EncB – DecA), and (EncB – DecC.). A joint loss is computed from these 4 losses.

- Dataset interaction/training order

  - MediaTek: Study different parameters’ update scheduling for multi-encoder or multi-decoder settings using any training strategy

    - The UEs can share a common dataset or employ UE-specific datasets, also UE/gNB vendors may undergo concurrent, alternating, and sequential updating schedule

- Overhead calculation

  - vivo: Overhead in information exchange for training collaboration type 2 grows linearly with the number of iterations at training stage

    - vivo Overhead ≈ # of epoch*(forward-propagation information + back-propagation information + input data)

- Descriptions/Findings for Type 3

- Direction 1: Sequential training starting with Network side training (NW-first training) Huawei, HiSilicon, Qualcomm, Ericsson, vivo, MediaTek, Samsung, ZTE, CATT, Xiaomi, CMCC, OPPO Fujitsu

Findings

  - Qualcomm: gNB-first type3 training with dataset exchange performs worse compared to type1 and type3 with activation/gradient exchange (type3-alt), since type3-alt training is based on end-to-end loss minimization in contrast to latent space loss minimization which is used in type3

  - Ericsson: For training Type 3 and N=M=1 case, NW-first training works well with a minor performance degradation compared to training Type 1. UE first training show in these evaluations a slight performance degradation.

  - MediaTek: In single-encoder single-decoder setting using gNB-first separate training, matched pairs not only do not experience performance loss, but they also reach a gain compared to joint training.

  - Xiaomi: When AI model structure is known, separate training can achieve similar performance with joint training

  - CMCC: With large enough dataset samples at UE side, separate training could achieve similar SGCS as joint training

  - Fujitsu: For the Case 1 of Type 3 training, only a negligible SGCS degradation (0.0001~0.0011) is observed compared to joint training

How to achieve N NW part models to M UE part models (Multiple NW parts of different NWs to one common UE part at UE) Huawei, HiSilicon, MediaTek, Samsung, Fujitsu

  - Solution description

    - Huawei, HiSilicon: UE uses a mix of N=2 datasets generated by two different Networks to train a common CSI generation part

  - Findings

    - Huawei, HiSilicon: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training when the separate training considers N>1 Network part models to one common UE part model with a different backbone or a different structure from any of the N Network part models.

    - MediaTek: On average, using gNB-first separate training for single-encoder multi-decoder setting degrades the performance by -58.89% compared to joint training for single-encoder single-decoder setting

    - MediaTek: On average, using gNB-first separate training for single-encoder multi-decoder setting degrades the performance by -58.63% compared to joint training for single-encoder multi-decoder setting

    - MediaTek: On average, using gNB-first separate training for single-encoder multi-decoder setting degrades the performance by -58.97% compared to joint training for single-encoder multi-decoder setting

    - MediaTek: On average, UE’s gain is -59.63% from gNB-first separate training for single-encoder multi-decoder

    - MediaTek: On average, gNB’s gain is -59.57% from gNB-first separate training for single-encoder multi-decoder setting

    - Samsung: In NW-first training, i.e., Type 3 training between one UE part model and N>1 separate NW part models, severe performance degradation is observed when the multiple NW parts are from different backbone

    - Fujitsu: For the Case 3 of Type 3 training, where the training at NW side is performed at first, the SGCS degradation is negligibly 0.003~0.006 compared to that of Case 1 of Type 3 training if the backbone of the AI/ML model is transformer.

- Direction 2: Sequential training starting with UE side training (UE-first training) Huawei, HiSilicon, vivo, Qualcomm, Ericsson, MediaTek, ZTE, Nokia, CATT OPPO Fujitsu

  - MediaTek: Give higher priority to UE-first separate training if separate training is adopted as the main training framework

    - gNB-first separate training has inferior performance compared to UE-first separate training for any number of encoders and decoders participating in the training session

  - Nokia: Continue further study of the separate training case for mismatched models

Findings

  - Huawei, HiSilicon: For AI/ML-based CSI compression, there is minor margin (<1%) between the SGCS of the UE first separate training and the SGCS of the joint training even when the Network-side CSI reconstruction part has a different structure or backbone with the UE-side CSI reconstruction part

    - This observation applies regardless when the dataset for the Network side is only a subset of or equal to the dataset for the UE side

  - Qualcomm: UE-first type3 training with dataset or activation exchange can achieve the same performance of Type1 training

  - Qualcomm: Joint and sequential training (e.g., UE-first type3) training achieves similar SGCS performance for rank >1

  - Ericsson: For training Type 3 and N=M=1 case, NW-first training works well with a minor performance degradation compared to training Type 1. UE first training show in these evaluations a slight performance degradation.

  - MediaTek: For single-encoder single-decoder setting, UE-first separate training does not necessarily reach an inferior performance compared to joint training. Matched pairs experience 1.2% improvement

  - Nokia: For UE-first Type 3 separate training with matched encoder/decoder models, results indicate little performance degradation relative to Type I joint training

  - Fujitsu: For the Case 1 of Type 3 training, only a negligible SGCS degradation (0.0001~0.0011) is observed compared to joint training

How to achieve N NW part models to M UE part models (One common NW part at NW to multiple UE parts of different UEs) vivo, Qualcomm, MediaTek, Fujitsu

  - Solution description:

    - Vivo: One gNB could collect paired input/output data from multiple UEs and then train one CSI reconstruction part based on a mixed dataset of all collected data.

  - Findings

    - vivo: There would be non-negligible performance loss for the case where one common CSI reconstruction/generation part is trained to match multiple CSI reconstruction/generation parts of different UEs in training collaboration type 3 (e.g., considering one common CSI reconstruction part to three CSI generation part and each UE sharing 50,000 samples with NW, the performance loss in SGCS is around 0.04).

    - vivo: Performance of one common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts of different networks/UEs is affected by the amount of exchanged data from each network/UE.

    - vivo: Performance loss in supporting common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts gets worse as the number of supported UEs/networks increases

    - Qualcomm Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training

    - MediaTek: On average, using UE-first separate training for multi-encoder single-decoder setting degrades the performance by -36.82% compared to joint training for single-encoder single-decoder setting

    - MediaTek: Using UE-first separate training for multi-encoder single-decoder setting degrades the performance by -36.66% compared to joint training for multi-encoder single-decoder setting

    - MediaTek: Using UE-first separate training for multi-encoder single-decoder setting degrades the performance by -37.45% compared to joint training in multi-encoder single-decoder setting

    - MediaTek: The main contributing factor in performance degradation of UE-first separate training for multi-encoder single-decoder setting is the number of encoders and not the training strategy itself

    - MediaTek: On average, UE’s gain is -37.84% from UE-first separate training for multi-encoder single-decoder setting

    - MediaTek: On average, gNB’s gain is -37.99% from UE-first separate training for multi-encoder single-decoder setting

    - Fujitsu: For the Case 2 of Type 3 training, where the training at UE side is performed at first, the SGCS degradation is 0.11~0.03 compared to that of Case 1 of Type 3 training if the backbone of the AI/ML model is transformer.

- Different structures between NW part and UE part for NW-first and UE-first (NW and UE are not aware of the AI/ML model of each other)

  - Solution description: 4 cases are raised by companies in the evaluations

    - Case 1 (baseline): Same backbone/hyperparameters/quantization method at NW ane UE Huawei, HiSilicon, vivo, MediaTek, CATT Xiaomi, CMCC OPPO Xiaomi ZTE Nokia Fujitsu

    - Case 2: Same backbone and quantization method at NW and UE, but different hyperparameters (e.g., different number of layers) between NW and UE Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT Nokia Fujitsu

    - Case 3: Same quantization method at NW and UE, but different backbones between NW and UE (e.g., one side is Transformer, the other side is CNN/ResNet) Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT, Xiaomi, CMCC Xiaomi Fujitsu

    - Case 4: Different backbones (and hyperparameters)/quantization methods between NW and UE vivo

  - Solution description-different dataset size: dataset used for the second training entity is only a subset (e.g., including CSI only corresponding to the second training entity) of the dataset used for the first training entity (e.g., including CSI corresponding to multiple second training entities): Huawei, HiSilicon, vivo CATT, CMCC OPPO

    - Huawei, HiSilicon: For NW-first training, two cases are evaluated:

      - Case 1: The shared dataset includes the full dataset (300K samples) at the training of the NW side, i.e., input and label of the NW side CSI generation part

      - Case 2: UE only uses its own CSI as labels (100K samples) and the corresponding CSI feedback shared by NW as inputs

    - Huawei, HiSilicon: For UE-first training, two cases are evaluated:

      - Case 1: The set of information includes the full dataset (300K samples) at the training of the UE side, i.e., the input and label of the UE side CSI reconstruction part

      - Case 2: Network only uses its own CSI as labels (100K samples) and the corresponding CSI feedback shared by UE as inputs

    - vivo: Each UE reports 10,000, 50,000, or 300,000 data samples for separate training, and the gNB combines all reported data to train the CSI reconstruction model

    - CATT For sequential training, we also provided simulation results with dataset in step 2 which is the only half size of dataset in step 1.

    - CMCC For case 2, 3 and 4, the decoder part at network side is the same as that of case 1, and the encoder parts at the UE side are trained based on Transformer, EVCsiNet and MLP-Mixer respectively using different number of dataset samples, i.e., 154K, 100K, 50K, 10K, 5K.

  - OPPO: for 600k, 300k, 100k and 50k training data, the total type 3 training overhead is about 1909M, 954M, 319M and 160M

Views on different structures/dataset sizes

  - Huawei, HiSilicon: practically if a vendor always uses a state-of-the-art model structure with excellent learning capability, it will naturally achieve the optimized end-to-end performance when pairing with the opposite side which also adopts optimized model structure; e.g., Transformer backbone would be naturally preferred by both sides rather than a suboptimal CNN/MLP/ResNet model.

  - MediaTek: In the UE-first separate training strategy, UE should inform gNB about the type of its architecture

    - In the UE-first separate training strategy, unmatched decoders may struggle to leverage the latent features provided by a pre-trained TF-based encoder

  - MediaTek: In the gNB-first separate training strategy, gNB should inform UE vendor at least about the type of its dropped encoder’s architecture

    - Unlike UE-first separate training strategy, gNB-first separate training strategy degrades the performance of unmatched pairs

  - Fujitsu: For the evaluation of the Type 3 training, evaluate the effect of the choice of backbone of AI/ML model on the performance of Type 3 training

Findings on different structures/dataset sizes

  - Huawei, HiSilicon: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the separate training and the SGCS of the joint training even when the UE-side CSI generation part has a different structure or backbone with the Network-side CSI generation part

    - This observation applies regardless when the input dataset for the UE side is only a subset of or equal to the input dataset for the Network side

  - vivo: There is a slight performance loss compared with joint training when all model configurations are aligned and enough data is exchanged (e.g., the same amount of data as that for local training)

  - vivo: If the quantization/dequantization method for separately trained CSI generation/reconstruction model does not match, the performance will drop significantly (more than 0.1 SGCS in our experiment).

  - vivo: In NW-first training, if the backbone structure for separately trained CSI generation/reconstruction models does not match, the performance will drop significantly (more than 0.1 SGCS in our experiment).

  - MediaTek: UE-first separate training does not degrade the performance of unmatched pairs (w.r.t. joint training). It shows a negligible improvement of 0.93%

  - MediaTek: In UE-first separate training, UE and gNB both experience 0.7% performance loss compared to their matched pairs which are trained via joint training strategy

  - MediaTek: In single-encoder single-decoder setting using gNB-first separate training, UE and gNB respectively experience 1.52% and 1.42% performance loss compared to their matched designed trained via joint training strategy

  - MediaTek: In the gNB-first separate training strategy, unmatched encoders may fail to replace the gNB’s TF-based encoder (cannot establish similar mapping from CSI to latent space). A significant degradation of -12.05% is observed in the performance for such pairings.

  - CATT: For separate training for AI/ML based CSI compression, compared to joint training

    - Similar performance can be achieved when the UE side CSI generation part and network side CSI reconstruction part use aligned AI/ML model structure (both use transformer based AI/ML model with 6 layers);

    - Performance loss is tiny when both the UE side CSI generation part and network side CSI reconstruction part use transformer based AI/ML model, but with different number of layers;

    - Performance loss can be obtained (2% ~ 6.6%) when one of the UE side CSI generation part and network side CSI reconstruction part uses transformer based AI/ML model and the other one uses ResNet based AI/ML model

  - CATT: For sequential separate training, compared to dataset in step 2 has the same size as dataset in step 1, minor performance loss can be seen for dataset CSI reconstruction part has half size of dataset in step 1

  - Xiaomi: When different AI backbone is used for CSI generation part and CSI reconstruction part, performance of separate training decreased compared with joint training

  - CMCC: When the number of dataset samples at UE side decreases, the SGCS of separate training will also decrease.

  - CMCC: When the generation part at UE side and the reconstruction part at network side have the same AI algorithms or model structures, to ensure separate training achieve similar SGCS as joint training, the requirement of number of dataset samples at UE side is much lower than the requirement when the AI algorithm or model structure is different between UE side and network side.

  - OPPO: Compared with type 1 training, NW-first type 3 training has 0.9%~6.6% and UE-first type 3 training has 0.3%~3.6% SGCS performance loss with 160M~1909M overhead, respectively

  - Fujitsu: For the Case 1 of Type 3 training, by varying the backbones of AI/ML models and fixing other conditions, it is observed that the performance of transformer models is superior to that of a convolutional neural network (CNN).

- Direction 3: Parallel training

Solution description

  - CATT: Parallel training means the AI/ML model at the UE side and the AI/ML model at the network side are trained separately, with no distinguishable sequential order. In other words, parallel training is order-agnostic training. One example of parallel training is as follows:

    - UE side trains the UE side CSI generation part with dataset #A;

    - Network side trains the network side CSI reconstruction part based on dataset #B;

    - Dataset #A and dataset #B can be the same or not

  - CATT: Parallel training with both the UE side CSI generation part and the network side CSI reconstruction part trained with the same dataset of {Channel, target CSI}, where “channel” is used as the input, and “target CSI” is used as output for the training of the UE side CSI generation part; and “target CSI” is used as the input, and “channel” is used as output for the training of the network side CSI reconstruction part

  - CATT: For separate training, parallel training is supported for further studied and evaluation

  - ETRI: For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the parallel training:

    - Step 1: The dataset for training or reference dataset for latent space alignment is optionally shared between the NW and UE sides. The dataset can be delivered either from NW to UE sides or from UE to NW sides.

    - Step 2: The encoder is trained at UE or UE-side server using its own dataset. At the same time, the decoder is trained at NW using its own dataset. A regulation to have geometric similarities between different training entities (e.g., isometry regulation [9]) can be applied if needed.

    - Step 3: Alignment of the latent space between different entities is applied if needed.Views

Findings

  - CATT: For separate training, similar performance can be achieved by parallel training and sequential training.

  - ETRI: The PCA-based AI models show almost no performance degradation when parallel learning is conducted even with a small number of common reference data

- Other views/findings for Type 3

  - Lenovo: Study methods for generating representative datasets for evaluation of multi-vendor scenarios.

  - Indian Institute of Tech: Training UE part model and NW part model with datasets based on different scenarios will result in a depletion of model performance

  - Indian Institute of Tech: For multi-vendor scenarios with different CSI compression models, sharing parameters at the NW reduces computation time and in turn power consumption. However, it may result in a performance drop of the system

  - Indian Institute of Tech: For multi-vendor scenarios with different CSI compression models, sharing trainable parameters at the network side allows for a more efficient implementation of the CSI compression model

  - Indian Institute of Tech: A specifier must be sent from all the UEs to the NW informing the type of CSI model used for compression. This will be used by the NW to optimize the number of parameters to be shared

  - Indian Institute of Tech: Training UE part model and NW part model with datasets based on different output dimensions will result in a depletion of model performance

  - Indian Institute of Tech: For multi-vendor scenarios with different output dimensions, sharing parameters at the NW reduces computation time and in turn power consumption. However, it may result in a performance drop of the system

  - Indian Institute of Tech: A specifier must be sent from all the UEs to the NW informing the output dimension used for compression. This will be used by the NW to optimize the number of parameters to be shared

- Quantization / dequantization in training

- Quantization aware training/ Quantization non-aware training

Solution description

  - Case 0: Training & inference w/o quantization

    - Xiaomi: Training without quantization, inference without quantization

  - Case 1: Quantization non-aware training: where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training Huawei, Hisilicon, Ericsson, vivo, Qualcomm, MediaTek, Nokia, Apple, Mavenir

    - MediaTek: Adopt training-non-aware non-learnable scalar quantization (TNA-NL-SQ) as the baseline for evaluation of all other quantization methods

    - Apple: we train the transformer base auto-encoder without quantization, the floating-point latent dimension is set to 30. After that, we apply scaler codebook and vector quantization codebook which quantize one float point value to 2 bits

  - Quantization-aware training: where quantization/dequantization is involved in the training process.

    - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase Ericsson, vivo, Qualcomm, MediaTek, Nokia Apple Xiaomi, Mavenir

      - Apple: the VQ codebook is trained using Linde-Buzo-Gray (LBG) algorithm based on floating point encoder output

      - Xiaomi: Case 2-1: Scalar quantization

    - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase Huawei, Hisilicon, vivo, Qualcomm, MediaTek, Nokia Xiaomi

      - Huawei, Hisilicon: the dictionary/codebook for the vector quantization is generated by applying the clustering algorithm to the outputs of the well-trained CSI generation part.

      - MediaTek: As the VQ is a training-non-aware method in essence, we only use it in the inference stage.

      - Xiaomi: Case 2-2: Vector quantization

Views

  - vivo: Quantization method at UE side and dequantization method at NW side should be aligned for training collaboration type2 and 3 to achieve a satisfying performance

  - vivo: Study the method and procedure to align quantization/dequantization method at different entities for CSI compression

  - Nokia: Define quantization loss as the difference between the reconstruction metric (SGCS or NMSE) obtained from a model trained without quantization and a model which incorporates quantization whether through quantization aware or unaware training. Report the loss in dB

  - Apple: Consider specified fixed VQ codebook for different CSI payload, to simply quantization alignment for training collaboration type 2 and 3.

  - Xiaomi: The case of training without quantization and inference without quantization should be evaluated as the upper bound

  - OPPO: For training collaborative Type3, study whether/how to align the quantization and dequantization method between UE and NW

  - Fujitsu: Quantization aware training is considered as the priority training strategy for the evaluation of AI/ML based CSI compression feedback

  - Fujitsu: In quantization aware training, it is suggested that we increase the number of floating-point outputs for a fixed number of output bit numbers

  - Fujitsu: For the evaluation of the quantization aware training, study and compare the throughput achieved by the approaches that the quantizers are updated (Case 2-2) or not (Case 2-1) during the training phase. This provides evidence for studying which of the two should be considered as the priority method.

  - Mavenir: Quantization aware training should be chosen as a better trade-off between performance and overhead when applying scalar quantization

  - Mavenir: The average quantization bit number of 4bit/float is a good trade-off between performance and overhead

Findings

  - Huawei, Hisilicon: For AI/ML-based CSI compression, quantization non-aware training shows significant performance loss compared to quantization aware training

  - Ericsson: Quantization aware training provides better performance and/or fewer quantization bits for the same performance, compared to the quantization non-aware training

  - Ericsson: In training Type 3 quantization-aware-training performs better compared to the quantization non-aware training

  - vivo: Quantization non-aware training only achieves good performance when the averaged quantization bit is large (e.g., >= 4bits/float). When the averaged quantization bit is small (e.g., <= 2bits/float), the performance loss is significant

  - vivo: Performance of quantization non-aware training could be significantly lower than that of quantization aware training (more than 0.1 in SGCS in our considered configurations)

  - Qualcomm: Quantization non-aware training (case-1) leads to noticeable performance degradation compared with quantization aware training (case-2)

  - Qualcomm: For quantization aware training, fixed or pre-configured quantization (case 2-1) is more sensitive to quantization’s parameters/configuration compared with trainable quantization (case 2-2). That is, quantization’s parameters/configuration in case 2-1 need to be carefully chosen to align with statistical distribution of latent vector (z), otherwise performance is degraded.

  - Qualcomm: Trainable quantization offers more flexibility and better performance compared to fixed quantization, e.g., trainable vector quantization can improve the performance.

  - MediaTek: Training awareness improves the gain of quantization methods by 6.5%.

  - MediaTek: Learnability improves the gain of quantization methods by 5.1%.

  - Nokia: Quantization-aware training performs better than quantization-unaware training due to the ability to optimize the model to the quantization method within the training process

  - Nokia: If the quantization design and properties match the distribution of the encoder outputs, Case-1 training can provide acceptable performance and adapt easily to different feedback sizes without the need to change the AE model

  - Apple: For quantization aware training, fixed VQ codebook can achieve similar performance comparing to joint VQ codebook/auto-encoder design

  - Xiaomi: There is around 7% performance loss caused by quantization

  - Fujitsu: For scalar uniform quantization, the performance of quantization aware training is better than that of quantization non-aware training, and the performance loss of quantization non-aware training is reduced with the increasing quantization bits and feedback payloads

    - Compared with quantization aware training, there is a 0.01~0.03 SGCS loss for quantization non-aware training under various feedback payloads

  - Mavenir: Quantization loss is specific to average quantization bit number and smaller average quantization bit means larger quantization loss.

  - Mavenir: Quantization aware training is better than quantization non-aware training for 2 and 4 bit/float cases, but worse for 6 bit/float cases.

  - Mavenir: The worst case is Quantization non-aware training with 2bits/float which shows ~32% performance degradation (evaluated using intermediate KPI GCS) when applying scalar quantization.

  - Mavenir: The best case is Quantization non-aware training with 6bits/float can cause ~0.3% performance degradation (evaluated using intermediate KPI GCS) when applying scalar quantization.

  - Mavenir: If average quantization bit is larger than 6bits/float, both quantization aware-training and non-aware training shows similar performance as non-quantization case.

  - NVIDIA: Both quantization aware training and quantization non-aware training have been agreed to be evaluated for AI/ML based CSI compression

Preliminary trends observed by more than one company

  - For AI/ML-based CSI compression, quantization non-aware training shows clear performance loss compared to quantization aware training Huawei, Hisilicon, Ericsson, vivo, Qualcomm MediaTek, Nokia, Fujitsu

- Scalar quantization/vector quantization

Design of SQ

  - Nokia: For scalar quantization, we consider both uniform quantization and non-uniform quantization, where non-uniform quantization uses μ-law quantization with μ=150.

  - vivo: To define a specific scalar quantization rule, we can directly define the number of bits assigned to each float. For example, we can use a vector [2, 2, …, 2, 3, …, 3, …, 4] to express a scalar quantization method, which assigns 2 bits to the first several float number, 3 bits to the next several float number, and 4 bits to the last several float number. The most trivial scalar quantization method is to uniformly assign K bits for all float numbers in a sequence.

  - ETRI: For example, allocating {8,8,4,4,3,3,1,1,0,0,0,…} bits to the latent variables to generate 32bits of CSI feedback payload and {10,9,8,6,6,5,4,4,3,3,2,2,1,1,0,0,…} bits to generate 64 bits of CSI feedback payload.

Findings for VQ

  - Fujitsu: For a fixed number of payload size, the number of floating-point outputs of the neural network followed by a small-resolution quantizer is larger than that followed by a high-resolution quantizer. This is helpful in increasing the SGCS performance if quantization aware training is used.

Design of VQ

  - MediaTek: Designing VQ should be done by UE, and gNB can optionally fine-tune its CSI reconstruction part.

  - MediaTek: The UE should inform gNB about its VQ design to maintain the alignment for quantization and dequantization parts.

  - Nokia: For vector quantization, the dimension of each segment is given by D and B is the number of bits allocated to the VQ codebook, where B = D × Feedback bits/element.  For Case 1, the K-means algorithm is used to determine the VQ codebook. For Case 2-2, the VQ codebook elements are jointly adapted with the encoder and decoder parameters by incorporating the VQ performance into the loss function.

  - vivo: The whole sequence to be quantized will usually be partitioned into several segments, as it is difficult to directly quantize the whole sequence…For example, 80 float variables can be partitioned into 16 sub-sequences, each of which is of size 5. Correspondingly, we can set 16 quantization codebooks, each of which will be used to quantize one segment…The quantization procedure is to select one codeword in the codebook that most represents the input, and the most common criteria is to select the one with the least MSE distance to the input.

Findings for VQ

  - MediaTek: Updating the entire AI/ML model through re-training/fine-tuning in the presence of TNA-VQ will degrade overall CSI reconstruction accuracy.

  - MediaTek: Fine-tuning CSI reconstruction part of AI/ML model in the presence of TNA-VQ improves the CSI reconstruction accuracy gain over TNA-NL-SQ by 62.7%.

  - MediaTek: In overall, fine-tuning gain is not significant. It is 1.2% gain compared to non-tuned AI/ML model

  - MediaTek: VQ is very sensitive to small changes in the distribution of CSI generation part’s output.

  - MediaTek: Re-using VQ codebook even for the AI/ML models of the same structure degrades its gain by 79.6% on average

  - Nokia: A slight modification of the AI encoder layer can lead to alteration of the latent vector element statistics

Comparison between SQ and VQ

  - vivo: Vector quantization with optimized codebook can achieve slightly better performance (e.g., by about 0.009 in SGCS in our considered configurations) than scalar quantization with fixed codebook

  - vivo: Performance of vector quantization with randomly initialization and fixed codebook can be slightly inferior to that of scalar quantization with fixed codebook (e.g., by about 0.0065 in SGCS in our considered configurations)

  - MediaTek: On average, TA-NL-SQ, TA-L-SQ, and TNA-VQ achieve 7.8%, 11.6%, and 4.1% gain over TNA-NL-SQ, respectively.

  - MediaTek: SQ methods of all kinds, on average, outperform VQ by 1.7%.

  - MediaTek: Give higher priority to SQ methods given their performance, less sensitivity, and less alignment efforts in multi-vendor ecosystems

  - Nokia: Vector quantization is found to have a smaller quantization loss than both uniform and non-uniform scalar quantization

  - Nokia: Possible impact of the final layer architecture of the AI encoder on the statistics of the latent vector needs to be investigated from the perspective of the corresponding quantization scheme and its performance

  - Nokia: A scalar quantization scheme can be still useful for some cases

  - Nokia: Keep a scalar quantization scheme as possible quantization options for AI/ML-empowered CSI feedback compression scheme

  - Xiaomi: Vector quantization and scalar quantization can achieve similar performance

  - Fujitsu: Under the method of quantization aware training, compared with the fixed scalar uniform quantization method (Case 2-1), better performance is achieved in a updated quantization approach (Case 2-2), where the vector quantization is updated together with the AI/ML models during the training phase.

- Alignment on vector quantization/dequantization for separate training

Solution description

  - Approach 1 (Quantization non-aware training)

    - Qualcomm UE server trains the encoder without quantization and shares the dataset (z, Vtarget); Vector quantizer (VQ) is trained with the decoder at NW-side training entity

  - Approach 2 (Quantization-aware training)

    - Case 1: output of CSI generation part (input of CSI reconstruction part) is shared before quantization Qualcomm ZTE Fujitsu

    - Case 2: output of CSI generation part (input of CSI reconstruction part) is shared after quantization Qualcomm ZTE Fujitsu

Views

  - Fujitsu: For the evaluation of the Type 3 training, evaluate the effect of the choice of quantizer on the performance of Type 3 training, from the perspectives of

    - Training method: quantization aware training or quantization non-aware training.

    - Quantization method: scalar quantization or vector quantization

Findings

  - Qualcomm: Training UE encoder without quantization and generating the separate training based on this encoder may lead to some performance degradation compared to encoder training with quantization

  - Qualcomm: Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training

  - ZTE: For Type 3 training with NW-first, the scheme with the shared output before quantization has similar performance to the scheme with the shared output after quantization. For Type 3 training with UE-first, the scheme with the shared input before quantization has similar performance to the scheme with the shared input after quantization

  - ZTE: For both Type 3 training with NW-first and Type 3 training with UE-first, two cases have similar performance. Type 3 training with NW-first has nearly the same performance as Type 1 training, while Type 3 training with UE-first has a slightly lower performance than Type 1 training

  - Fujitsu: There is negligible performance difference between two quantization behaviors for both NW-first and UE-first separate training

### 3.1-3: AI/ML model settings

- AI/ML model settings for rank>1

Some companies have discussed the issue on how to set up AI/ML models for multiple ranks situation. Some companies analyzed detailed methods, which can be summarized as follows:

- Option1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference. Apple ZTE,

  - Apple: the input is the PMI of the corresponding rank, and output is the reconstructed precoding matrixes

  - Apple: Rank specific AI model perform depends on loss function. If average SGCS is used in loss function, the model averages out the SGCS of each layer. With rank 4 model, the layer 1 and layer 2 SGCS suffer significant loss comparing to layer specific and layer common.  Higher model complexity and higher storage requirement for rank specific model is observed.

  - MediaTek: rank-specific designs are not suited for UE vendors as stacking layers, to process them jointly, will drastically increases AI/ML models’ complexity.

- Option2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference., ETRI

  - Apple: input is the measured channel matrix, and output is RI together with PMI corresponding to the RI

  - Apple: Rank common AI model needs further clarification

  - ETRI: the rank-common AI/ML model operates on the entire input (input of multiple layers)

- Option3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference. vivo Ericsson MediaTek CATT, Apple InterDigital

  - Option3-1 (layer specific and rank common): For a specific layer, the layer specific model is applied for all rank values. Ericsson

  - Option3-2 (layer specific and rank specific): For a specific layer, different layer specific models are applied for different rank values. Ericsson

    - MediaTek: Given the possible number of scenarios and configurations, it is infeasible to train and deploy a dedicated AI/ML model for each. In fact, it is the objective of generalization efforts to avoid dedicated designs like layer-specific and rank-specific AI/ML models.

- Option4 (layer common): For a specific layer, the unified AI/ML model is applied for all rank values. Huawei, Hisilicon, vivo Apple MediaTek, CATT, ZTE ETRI NTT DOCOMO InterDigital, LG

  - Apple: The AI model can be trained using only layer 1 eigen-vector as input or using mixture of layers’ eigen vector.

  - InterDigital: a unified model trained on samples from both layers, where the first and the second eigenvectors of each channel sample are derived and then used as two separate training samples

  - Option4-1 (layer common and rank common): A unified AI/ML model is trained and applied for each layer to perform individual inference. Huawei, Hisilicon NTT DOCOMO

  - Option4-2 (layer common and rank specific): For a specific layer, different layer common models are applied for different rank values. NTT DOCOMO

Solution/views

  - Huawei, Hisilicon: For the evaluation of the AI/ML-based CSI compression sub use cases with rank >=1, companies are encouraged to report the specific option adopted for AI/ML model settings to adapt to ranks/layers. Rank common and layer common AI/ML model is adopted in our evaluation

  - vivo: For rank > 1 cases, study option3 (layer specific) and option4 (layer common).

    - One common model for all payloads trained with the data from all layers

    - One specific model for each payload trained with the data from all layers

  - MediaTek: Between layer-specific and layer-common AI/ML models settings for rank>1, down select layer-common AI/ML models.

  - ZTE: For rank>1, two cases on model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point

    - Case 1: Layer common-A single model is applied to all layers and all ranks

    - Case 2: Rank specific- Multiple models are trained and each model is applied to a specific rank

  - CATT: For AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers from the view of model training and model inference respectively

    - For model training

      - Option 1(layer common, rank common): A unified AI/ML model is trained with data from all layers of all ranks.

      - Option 2(layer common, rank specific): Separated AI/ML models are trained per rank value with data from all layers of the rank.

      - Option 3(layer specific, rank common): Separated AI/ML models are trained per layer value with data from all ranks.

      - Option 4(layer specific, rank specific): Separated AI/ML models are trained per layer value per rank value.

    - For model inference

      - Option 1(layer common, rank common): A unified AI/ML model is applied for all layer values and all rank values to perform inference.

      - Option 2(layer common, rank specific): Separated AI/ML models are applied per rank value to perform individual inference, while the same AI/ML model is applied for the layers of a specific rank to perform inference.

      - Option 3(layer specific, rank common): Separated AI/ML models are applied per layer value to perform individual inference, while the same AI/ML models are applied for the same layer value of all ranks to perform inference.

      - Option 4(layer specific, rank specific): Separated AI/ML models are applied per layer value per rank value to perform individual inference.

  - CATT: For AI/ML based CSI feedback, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable to rank 2.

    - Therefore layer specific AI/ML model should be considered.

  - Apple: For AI based CSI compression, layer common provides reasonable performance gain and minimum storage overhead. Adopt layer common approach as a starting point for AI based CSI design

  - ETRI: For the evaluation of AI/ML-based CSI feedback for CSI compression, companies provide the details of the model structure for multiple ranks

  - AT&T: For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

    - Option 1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference.

      - FFS on the reported complexity and storage

      - FFS: input/output type

    - Option 2 (rank common): A unified AI/ML model is trained and applied for various adaptive ranks to perform inference.

      - FFS: input/output type

    - Option 3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

      - FFS on the reported complexity and storage

      - Note: input/output type is Precoding matrix

      - Companies to report the setting is layer specific and rank common (For a specific layer, the same model is applied for all rank values), or layer specific and rank specific (For a specific layer, different models are applied for different rank values)

    - Option 4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

      - Note: input/output type is Precoding matrix

      - Companies to report whether the setting is layer common and rank common (A unified AI/ML model is applied for each layer to perform individual inference), or layer common and rank specific (For a specific layer, different models are applied for different rank values)

    - Other options not precluded.

  - LG: For AI/ML model setting for rank>1, option 4 (layer common) can be a baseline

Findings

  - vivo: Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model

  - Apple: Layer specific AI model can achieve better performance comparing to layer common AI model, with 4 times storage overhead

  - MediaTek: Layer-common AI/ML model respectively achieves 1.18% and 1.55% SGCS gain for layer 0 and layer 1 of rank-2 channels compared to layer-specific AI/ML models.

  - ZTE: For rank =2, AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II.

  - ZTE: Case 1(layer common) can achieve better performance than Case 2 (rank specific).

  - NTT DOCOMO: Directly applying AI/ML model trained on rank2 dataset to infer Rank1 dataset has little influence on SGCS performance, when the payload size is sufficiently large

  - InterDigital: For the rank one case, the AI/ML-based CSI compression using eigenvectors of channels as the input outperforms the Rel-16 Type II at similar overhead

  - InterDigital: For the rank one case, the AI/ML-based CSI compression using the eigenvector layer-common model achieves the best SGCS performance relative to the layer-specific and rank-specific models

  - InterDigital: The AI/ML-based CSI compression using the eigenvector layer-common model generalizes well when tested under two layers, and achieves the best SGCS performance relative to the layer-specific and rank-specific models for both rank one and rank two transmission

  - ETRI: Rank-common or layer-common AI/ML models can process inputs with different numbers of ranks using a single model, allowing for a smaller model size

- Input/output CSI format

- Option 1: Raw channel matrix. NVIDIA, FUTUREWEI InterDigital

- Option 2: Eigenvector. Huawei, Hisilicon, vivo, CATT, Samsung, Nokia, MediaTek, OPPO, Xiaomi, Apple, ZTE, CMCC, FUTUREWEI, NVIDIA, ETRI, Google, China Telecom, Spreadtrum, Fraunhofer Intel InterDigital,

  - Option 2A: Legacy-like PMI (e.g., Type I-like, Type II-like CSI). Ericsson, Intel, vivo

  - Option 2B: Eigenvector of additional past CSI. Huawei, Hisilicon, MediaTek

- Views/findings

  - Google: The study of the input of CSI compression should prioritize the input based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1.

  - InterDigital: The AI/ML-based CSI compression using eigenvectors as an input for the layer-common model consistently outperforms the AI/ML model using full CSI as an input and the Rel-16 Type II, in terms of SGCS under the rank one transmission case

  - FUTUREWEI: For CSI feedback compression sub use case, AI/ML based approach using eigenvectors as input/output without applying quantization on encoded output achieved slightly better performance compared with the baseline approach, i.e., Rel.16 Type II codebook

  - FUTUREWEI: For CSI feedback compression sub use case, AI/ML based approach using channel matrix as input/output without applying quantization on encoded output achieved comparable Mean UPT and 5% UPT performance compared with the baseline approach, i.e., Rel.16 Type II codebook

  - FUTUREWEI: For CSI feedback compression sub use case, AI/ML based approach using either eigenvectors or channel matrix as input/output with vector quantization can significantly reduce air-interface overhead compared with Rel.16 Type II codebook-based approach, i.e., 13 bits vs. 242 bits (94.6% overhead reduction) while achieving decent performance

  - NVIDIA: Both autoencoders with raw channel matrix as input and autoencoders with eigenvector(s) of raw channel matrix as input have been agreed to be evaluated

- Quantization/dequantization method:

- Views

  - ETRI: For the evaluation of AI/ML-based CSI compression use case, companies to report the details on the quantization/dequantization including:

    - Functional separability of compression and quantization

    - Configuration of quantization/dequantization block (Scalar or vector quantization, fixed or dynamic codebook)

    - Quantization aware/non-aware training of the AI models

  - FUTUREWEI: Vector quantization using codebook size 8192 is applied to the encoder output first to generate quantized output with 13 overhead bits then the quantized result is used as input to the decoder.

- Findings

  - ETRI: AI/ML models with a functionally separable quantization block can adjust CSI payload sizes by using different quantization configurations

  - FUTUREWEI: For CSI feedback compression sub use case, AI/ML based approach using eigenvectors as input/output with 13 overhead bits after applying quantization on encoded output achieved comparable Mean UPT, however, the 5% UPT is slightly degraded (5.5% degradation) compared with the baseline approach, i.e., Rel.16 Type II codebook-based approach with 242 overhead bits

  - FUTUREWEI: For CSI feedback compression sub use case, AI/ML based approach using channel matrix as input/output with 13 overhead bits after applying quantization on encoded output achieved comparable Mean UPT and 5% UPT performance compared to the baseline approach, i.e., Rel.16 Type II codebook-based approach with 242 overhead bits.

- Other views

### 3.1-4: EVM related issues for CSI compression

- CSI payload alignment

In the previous meetings, two options have been discussed on the calculation of the payload size for AI/ML-based solution and legacy TypeII CB.

- Option 1: Payload size is calculated based on the maximum rank. CATT(1st) ZTE(baseline), LG

- Option 2: Payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. FFS the following understandings

  - Option 2a: The above-mentioned “CSI payload” is calculated as each CSI reported payload with a given rank Ericsson, Apple Samsung(reported)

  - Option 2b: The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank Huawei, Hisilicon, Qualcomm, CATT(2nd)  ZTE(optional) Samsung(reported) CAICT,

- Views from companies:

  - Huawei, Hisilicon: For the performance comparison between AI/ML-based CSI compression and legacy TypeII codebook, use tables in section 6.3.2.1.2 of TS 38.212 to calculate CSI payload.

  - Huawei, Hisilicon: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE

    - The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank.

  - Qualcomm: For the evaluation of the AI/ML based CSI compression sub use case, the CSI feedback overhead should be computed based on the rank indicated by the UE

  - Ericsson: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy Type-II codebook, for SLS and LLS the payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. The“CSI payload” is calculated as each CSI reported payload at the given rank.

  - Apple: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, the payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

    - The “CSI payload” is calculated as each CSI reported payload with a given rank, not the max allowed bits at the given rank

  - CATT: Align the interpretation on payload size for AI/ML-based CSI compression and the legacy Type II codebook

  - CATT: For the CSI payload size calculation for the evaluation of AI/ML-based CSI compression and the legacy TypeII codebook based CSI feedback, one of the following two options is considered

    - Option 1(1st preferred): Payload size is calculated based on the maximum allowed bits for the maximum rank;

    - Option 2b(2nd preferred): Payload size is calculated as the weighted average of CSI payload per rank, with the following calculation formula:

C=j=1KRjCj

      - where Cj is the maximum allowed bits for rank j, K is the maximum rank of the UE, and Rj is the ratio of CSI reports with rank j among all CSI reports

  - ZTE: For CSI payload size calculation, Option 1 is adopted as baseline evaluation metric and Option 2b is up to companies to report

  - ZTE: For up to Rank 2, Maximum overhead shows similar performance compared with Weighted Avg. Overhead

  - ZTE: With regard to up to Rank 4, Maximum overhead shows similar performance compared with Weighted Avg. Overhead

  - Samsung: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, the payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE

    - Note: Whether the CSI payload is calculated as each CSI reported payload with a given rank or as max allowed bits at the given rank can additionally be reported

  - CAICT: Option 2 proposed by moderator in last meeting for CSI payload calculation should be used as baseline

  - LG: Payload size is calculated based on the maximum rank

- Template for simulation results collection

- General views/findings

  - Intel: RAN1 to separately discuss tables for different usages, including AI-ML model description, SGCS and UPT performance comparison with eType II PMI codebook, generalization performance, different training types, etc.

    - Later RAN1 can endorse spreadsheet file with full template of results collection, where information (e.g., AI-ML model description) can be reused for different tables

  - Intel: For evaluation results collection table,

    - At least some fields of the table including number of parameters, storage and complexity of the AI-ML model shall be separately specified per CSI payload value

    - Remove fields corresponding to performance gains and support fields for absolute performance of eType II PMI codebook as well as AI-ML CSI (for intermediate metrics and UPT)

  - Lenovo: A single AI/ML design may use different training scheme in different stages of LCM. e.g., initial training, model update, model fine-tuning.

  - Lenovo: The KPIs that are important for evaluation of the training scheme may be different depending on:

    - Lifecycle stage during which training is performed (e.g., initial training, model update, …)

    - Where the training is performed (e.g., UE, on gNB, on other node(s) in the cloud)

    - If training needs only simulated data or offline field dataset or online (e.g., (near) real-time) dataset is needed.

  - Lenovo: When discussing Template for reporting the evaluation results, consider requirements for all stages of the LCM (not only the inference stage), and all different deployment types. As one example, important combinations of the following quadruple should be considered when we discuss evaluation template

    - (Training scheme, Lifecycle stage, Training entity, Dataset type) where

    - Training scheme: Type 1, Type 2, Type 3

    - Lifecycle stage: Initial Training, Model update, Fine-tuning

    - Training entity: UE, gNB, both UE and gNB, cloud node(s)

    - Dataset type: Simulated dataset, Offline field dataset, Online (near-real time)

- Range of payload size X/Y/Z

  - Summarized payload sizes from submitted results

|  | rank 1 |  | rank 2 |  | rank 3 |  | rank 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | min | max | min | max | min | max | min | max |
| Huawei, Hisilicon | 60 | 240 | 80 | 390 |  |  |  |  |
| Qualcomm | 134 | 234 | 134 | 234 |  |  |  |  |
| Ericsson | 62 | 66 | 111 | 113 | 99 | 100 | 111 | 127 |
| vivo | 80 | 230 |  |  |  |  |  |  |
| Nokia | 50 | 310 |  |  |  |  |  |  |
| ZTE | 60 | 340 | 120 | 650 |  |  | 120 | 750 |
| Spreadtrum | 50 | 250 |  |  |  |  |  |  |
| OPPO | 67 | 335 | 67 | 335 |  |  |  |  |
| CATT | 50 | 250 | 80 | 480 | 80 | 450 | 100 | 480 |
| ChinaTelecom | 48 | 120 |  |  |  |  |  |  |
| Fujitsu | 80 | 280 |  |  |  |  |  |  |
| Intel | 48 | 278 | 48 | 278 |  |  |  |  |
| CMCC | 32 | 120 |  |  |  |  |  |  |
| ETRI | 49 | 187 |  |  |  |  |  |  |
| InterDigital | 164 | 275 | 315 | 315 |  |  |  |  |
| Apple | 60 | 280 | 60 | 280 | 60 | 280 | 60 | 280 |
| NTT DOCOMO | 44 | 304 | 88 | 608 |  |  |  |  |
| MTK | 50 | 300 |  |  |  |  |  |  |

- Template without generalization/scalability

  - Xiaomi: The parameter and metric for complexity of AI model training e.g., the training related overhead, the required dataset size for convergence, number of epoch times, batch size and learning rate are captured in the template table,

  - Xiaomi: AI model structure e.g., the number of layers is provided in the template table at least for case 2 of Type 3 separate training

  - Lenovo: We suggest adding the following rows to the template we use for evaluation of the model. Note that, similar to some other rows as “Pre-processing” and “Post-processing”, we can have NA for some of the KPIs if they are not applicable to the proposed scheme. To shorten the table, we have removed some of the rows which are already agreed upon. Then new additions are marked with different color

Table 7. Template for evaluation of the performance of AI/ML models

|  |  | Source 1 |
| --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |
|  | Pre-processing |  |
|  | Post-processing |  |
|  | FLOPs/M |  |
|  | Number of parameters/M |  |
|  | [Storage /Mbytes] |  |
| CSI reconstruction part | AL/ML model backbone |  |
|  | [Pre-processing] |  |
|  | [Post-processing] |  |
|  | FLOPs/M |  |
|  | Number of parameters/M |  |
|  | [Storage /Mbytes] |  |
| Dataset description | Train/k |  |
|  | Test/k |  |
|  | Ground-truth CSI quantization method |  |
| Benchmark |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Gain for 5% UPT | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Data collection overhead | Initial training |  |
|  | Model monitoring |  |
|  | Model adaptation (update/switching/…) |  |
| Data collection latency | Initial training |  |
|  | Model monitoring |  |
|  | Model adaptation (update/switching/…) |  |
| Model Delivery overhead | Initial model |  |
|  | After Model adaptation (update/switching/…) |  |
| Model Delivery latency | Initial model |  |
|  | After Model adaptation (update/switching/…) |  |
| Model Training | Training type |  |
|  | Training overhead (e.g., type2 gradient exchange) |  |
|  | … |  |
| … |  |  |
| FFS others |  |  |

- Template for training types

  - Huawei, Hisilicon: From the evaluation perspective, Type 1/2/3 can be categorized into the following classes

    - Joint training with ideal model pairing. This corresponds to the idea joint training of 1 Network model to 1 UE model where the CSI generation part and the CSI reconstruction part can be regarded to be jointly designed/developed by a single side, so that performance can be regarded as optimal.

    - Joint training with restricted model pairing. As analyzed above, this includes 1 Network vs 1 UE with restricted model pairing or multi-vendor case for Type 1/2.

    - Separate training. Type 3 separate training can be also regarded as restricted model pairing.

  - Huawei, Hisilicon: For the AI/ML-based CSI compression sub use case,

    - The template achieved in the working assumption in the RAN1#111 meeting is considered for companies to report the evaluation results for joint training with ideal model pairing.

    - The template of Table A.2 in the Appendix is considered for companies to report the evaluation results for training Type 2.

    - The template of Table A.3 in the Appendix is considered for companies to report the evaluation results for training Type 3.

Table A.2 Template for CSI compression with training Type 2

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Train/k |  |  |
|  | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Case 1 (baseline): NW#1-UE#1 | UE part AL/ML model backbone/structure |  |  |
|  | Network part AL/ML model backbone/structure |  |  |
| ... (other NW-UE combinations for Case 1) |  |  |  |
| Case 2 (1 NW part to M>1 UE parts) | UE#1 part training dataset |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | NW part model backbone/structure |  |  |
| Case 3 (N>1 NW parts to 1 UE part) | NW#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | UE part model backbone/structure |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| Case 1: NW#1-UE#1: Intermediate KPI | Payload X |  |  |
|  | Payload Y |  |  |
|  | Payload Z |  |  |
| … (other NW-UE combinations for Case 1) |  |  |  |
| Case 2: Intermediate KPI | Payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | Payload X, NW-UE#M |  |  |
|  | Payload Y … |  |  |
|  | Payload Z … |  |  |
| Case 3: Intermediate KPI | Payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | Payload X, NW#N-UE |  |  |
|  | Payload Y … |  |  |
|  | Payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

Table A.3 Template for CSI compression with training Type 3

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Benchmark: NW#1-UE#1 joint training | UE part AL/ML model backbone/structure |  |  |  |
|  | Network part AL/ML model backbone/structure |  |  |  |
|  | Training dataset size |  |  |  |
| ... (other NW-UE combinations for benchmark) |  |  |  |
| Case 1-NW first training | NW part AL/ML model backbone/structure |  |  |  |
|  | UE#1 part model backbone/structure |  |  |  |
|  | UE#1 part training dataset size |  |  |  |
|  | … |  |  |  |
|  | UE#M part model backbone/structure |  |  |  |
|  | UE#M part training dataset size |  |  |  |
| Case 1-UE first training | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset size |  |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset size |  |  |  |
|  | UE part model backbone/structure |  |  |
| Case 2-NW first training | NW#1 part model backbone/structure |  |  |  |
|  | … |  |  |  |
|  | NW#N part model backbone/structure |  |  |  |
|  | UE part model backbone/structure |  |  |  |
|  | UE part training dataset size |  |  |  |
| Case 3-UE first training | UE#1 part model backbone/structure |  |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE part AL/ML model backbone/structure |  |  |
|  | NW part training dataset size |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| NW#1-UE#1 joint training: Intermediate KPI | Payload X |  |  |  |
|  | Payload Y |  |  |  |
|  | Payload Z |  |  |  |
| … |  |  |  |
| Case 1-NW first training: Intermediate KPI | Payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | Payload X, NW-UE#M |  |  |
|  | Payload Y … |  |  |
|  | Payload Z … |  |  |
| Case 1-UE first training: Intermediate KPI | Payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | Payload X, NW#N-UE |  |  |
|  | Payload Y … |  |  |
|  | Payload Z … |  |  |
| Case 2-NW first training: Intermediate KPI | Payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | Payload X, NW#N-UE |  |  |
|  | Payload Y … |  |  |
|  | Payload Z … |  |  |
| Case 3-NW first training: Intermediate KPI | Payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | Payload X, NW-UE#M |  |  |
|  | Payload Y … |  |  |
|  | Payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

  - Xiaomi: Adopt Table 1 as template for simulation results collection at least for case 2 of Type 3 separate training

    - For Type 3 separate training, when not aligned AI/ML model structures between NW side and UE side are used, it was agreed that companies to report the AI/ML structures for the UE part model and NW part model, e.g., different backbone, or same backbone but different structure (e.g., number of layers).

Table 1. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [Type 3/case 2]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |  |
|  | AI/ML model structure e.g., number of layers |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
|  | Convergence time of AI model training, e.g., the number of epoch times, batch size, learning rate |  |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |  |
|  | AI/ML model structure e.g., number of layers |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
|  | Convergence time of AI model training, e.g., the number of epoch times, batch size, learning rate |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
| Dataset description | Train/k Note: Various dataset size can be considered. |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method |  |  |  |
|  | Training related overhead |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

- Template for generalization/scalability

  - Huawei, Hisilicon: For the AI/ML-based CSI compression sub use case, the template of Table A.4 in the Appendix is considered for companies to report the evaluation results with model generalization/scalability.

Table A.4 Template for CSI compression with model generalization

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| Intermediate KPI #1, layer 1 | CSI payload X |  |  |
|  | CSI payload Y |  |  |
|  | CSI payload Z |  |  |
| Intermediate KPI #1, layer 2 | CSI payload X |  |  |
|  | CSI payload Y |  |  |
|  | CSI payload Z |  |  |
| … |  |  |  |
| Intermediate KPI #2, layer 1 | CSI payload X |  |  |
|  | CSI payload Y |  |  |
|  | CSI payload Z |  |  |
| Intermediate KPI #2, layer 2 | CSI payload X |  |  |
|  | CSI payload Y |  |  |
|  | CSI payload Z |  |  |
| … |  |  |  |
| Generalization Case 2 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| Generalization Case 3 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| Fine-tuning case (optional) | Train (setting, size/k) |  |  |
|  | Fine-tune (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| FFS others |  |  |  |

  - FUTUREWEI: When reporting AI/ML model generalization evaluation results for CSI feedback enhancements, companies to consider aligning the reporting attributes depicted in Table 2-1.

Note: for reporting/sharing evaluation results, separate entries should be used to indicate the result for:

    - Benchmark: the benchmark result is the performance achieved assuming the AI/ML model is trained using dataset from the target scenario/configuration (this may be considered as performance upper-bound)

    - Target scenario: this is the performance result achieved when the AI/ML model is trained using dataset from the source scenario/configuration then tested on the dataset from the target scenario after applying generalization techniques if applicable.

Table 2-1: Model generalization evaluation report attributes

|  |  | Source 1 | Source 2 |
| --- | --- | --- | --- |
| Scenario | Train |  |  |
|  | Test |  |  |
| Configuration | Train |  |  |
|  | Test |  |  |
| Channel estimation | Train |  |  |
|  | Test |  |  |
| Input/output type of AI/ML model | Train |  |  |
|  | Test |  |  |
| Dataset size | Train |  |  |
|  | Test |  |  |
|  | Used for generalization purpose |  |  |
| CSI payload | Train |  |  |
|  | Test |  |  |
| Intermediate KPI 1 | Benchmark |  |  |
|  | On Test scenario/config. |  |  |
| Intermediate KPI 2 | Benchmark |  |  |
|  | On Test scenario/config. |  |  |
| Eventual/final KPI: Mean UPT | Benchmark |  |  |
|  | On Test scenario/config. |  |  |
| Eventual/final KPI: 5% UPT | Benchmark |  |  |
|  | On Test scenario/config. |  |  |
| Mechanism applied | Pre/post-processing applied in Training |  |  |
|  | Pre/post-processing applied in Testing |  |  |
|  | Others (e.g., naïve transfer, fine tuning, adaptation) |  |  |

- High resolution ground-truth CSI labels

Solutions/views Huawei, Hisilicon, vivo, Ericsson, Qualcomm, ZTE, Apple, Nokia, Fujitsu, Lenovo

- Huawei, Hisilicon: evaluation results for the two quantization methods (scalar quantization and quantization by Rel-16 TypeII-like codebook generation method with new parameters) for the ground-truth CSI are provided

  - Float32/Float16/8 bits scalar quantization

  - Rel-16 TypeII CB with new parameters: L=10, p=0.9, beta=0.31, amplitude: 4 bits, phase: 6 bits

- Ericsson: we present results showing the accuracy (as measured in SGCS and RAR respectively) for different choices of L, M and subband size. Non-zero coefficients are presented with both “non-quantized” format and 7-bit quantization format. beta=1 and 0.5 are presented.

- vivo: Rel-16 TypeII CB with new parameters: L=12, M=6, beta=1.0. Rel-16 TypeII CB with legacy parameters: L=4, M=4, beta=0.75

- Qualcomm: For training data collection, specifying new/larger values of R16 Type II parameters to achieve higher resolution of the ground truth CSI needs clear justification

- ZTE:

  - eType-II PC9: L = 8, pv=0.75, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

  - eType-II PC10: L = 10, pv=0.9, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

- Nokia: Study the tradeoff between system performance and data collection overhead, especially for the use case of data collection for the purpose of building datasets for model training.

- Apple: Adopt 8-bit scaler quantization for ground true data quantization

- Fujitsu:

  - Parameter Set #1: L = 6, Mv=7, pv=0.5, β=0.5, Reference amplitude = 4 bits, Differential amplitude = 4 bits, Phase = 4 bits

  - Parameter Set #2: L = 12, Mv=13, pv=0.95, β=0.5, Reference amplitude = 4 bits, Differential amplitude = 4 bits, Phase = 4 bits

- Fujitsu: High-resolution codebook quantization of the right singular vectors of the spatial-frequency-domain channel matrix, e.g., Rel-16 type II-like method with new parameter values, can be used in the dataset construction for finetuning

- Fujitsu: In order to achieve a low-overhead dataset transferring in over-the-air-training/monitoring , the codebook-based quantization approach should be further studied.

- Lenovo: Evaluate the following CSI training data formats:

  - Alt-A. Legacy codebook-based dataset points generated via multiple occasions of NR codebook-based CSI feedback

  - Alt-B. High-resolution codebook-based dataset points generated via high-resolution variants of NR-based CSI codebooks

  - Alt-C. Floating point representation of raw CSI data, e.g., raw channel matrices or sets of channel eigenvectors

Findings

- Huawei, Hisilicon: For AI/ML-based CSI compression, compared with the training dataset quantized by Float32 format, other high resolution ground-truth CSI quantization methods with lower overhead show minor SGCS loss with remarkable overhead reduction,

  - Training dataset quantized by 8 bits scalar quantization has <0.9% SGCS loss but reduces 75% overhead of training dataset delivery.

  - Training dataset quantized by Rel-16 TypeII CB with new parameters has <0.7% SGCS loss but reduces 96% overhead of training dataset delivery.

- Ericsson: For higher layers (layer 3 and 4), the true Tx-eigenvector changes quickly over frequency, such that the eigenvectors extracted with 4 RBs/subband has lost 15-20% in SGCS. However, in terms of RAR the loss is only about 2-3%

- Ericsson: The legacy CSI reporting was shown by SLS to reach 70% UTP while increasing L and M can reach 80%. With non-quantized eType-II coefficients, 87% can be reached at a high overhead cost.

- Apple: For ground true data quantization, 8 bit quantization achieve the best tradeoff between overhead, performance and complexity

- vivo: High resolution R16-eType II codebook with large L, M, beta (for example, L=12, M=6, beta = 1.0) performs well for ground-truth CSI quantization

- Qualcomm: The SGCS achieved with a training dataset quantized using R16 Type II PC 8 is very close to the SGCS achieved using the ideal (floating point) training dataset for the rank 1 case with dense urban scenario.

- ZTE: The R16 Type II method with larger L values and larger pv values than legacy eType-II PCs has the possibility to achieve high resolution quantization with low overhead

- ZTE: New parameters combinations for enhanced R16 Type II method should be supported to achieve high resolution CSI with acceptable overhead for ground-truth CSI collection.

- Nokia: The need and details of ground-truth quantization are dependent on the use cases for the data.

- Fujitsu: For finetuning, an excellent performance can be achieved by the dataset composed by the high-resolution codebook quantization, i.e., Rel-16 type II-like method with new parameter values, of the right singular vectors of the spatial-frequency-domain channel matrix

- Fujitsu: The performance of finetuning is almost the same from using right singular vectors of channel matrices and their high-resolution codebook quantization, e.g., Rel-16 type II-like method with new parameter values

- Fujitsu: It is observed that there is a significant overhead reduction of transferring a codebook-based dataset than a dataset composed by channel vectors of floating-point numbers for separate training. So, it is worth to study codebook-based quantization method in order to achieve a low-overhead dataset transferring in separate training

- CQI calculation

- ZTE: Further study different CQI determination methods based on the following categorization

  - Case 1: CQI is calculated based on the output CSI from the realistic channel estimation, including

    - Case 1a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference

    - Case 1b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

  - Case 2: CQI is NOT calculated based on the output CSI from the realistic channel estimation, including

    - Case 2a: CQI is calculated based on target CSI with realistic channel measurement

    - Case 2b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

      - Case 2b-1 Potential CQI compensation based on some assistance of network indication if configured

      - Case 2b-2 Potential CQI compensation based on monitored performance

  - Case 2c: CQI is calculated based on traditional codebook

- ZTE: With regard to average sector throughput gain, Case 1a, Case 2b-1 and Case 2b-2 have similar performance, and these three cases are obviously higher than Case 2a and eType-II scheme; Case 2a with low feedback overhead has worse performance than eType-II scheme, and Case 2a with high feedback overhead has better performance than eType-II scheme

- ZTE: For 5% tail UE throughput gain, the performance increases according to the sequence of Case 2a, eType-II scheme, Case 2b-2, Case 2b-1, and Case 1b, where the performance of Case 2a is 6% - 15% lower than that of eType-II scheme

- Lenovo: Assuming two-sided AI models for CSI compression under training collaboration Type 3, further enhancements are needed to ensure precise CQI characterization in the presence of mismatch between the nominal decoder (at UE side) and the actual decoder (at network side).

- RI calculation

Solution description

- Apple: 3 RI calculation methods are considered

  - Sequential approach:

    - Step 1: RI is determined first, based on ideal eigen vector that optimized the PHY abstraction metric.

    - Step2: PMI is searched based on determined RI, for both e-type II and AI based approach.

  - Joint approach:

    - For e-type II codebook, exhaustive search the best PMI for each rank, then select the PMI and RI combination that optimize the PHY abstraction metric.

    - For AI based approach, similar search is done. This assumes UE knows the AI decoder, and UE runs encoder for each layer before RI is selected. Note: the assumption of UE knows the AI decoder may not be realistic.

  - Hybrid approach:

    - For e-type II codebook, the joint approach with exhaustive search is used.

    - For AI based method, the sequential approach is used.

Findings

- Apple: System level performance gain depends on RI selection algorithm

  - When ideal eigen-vector is used to select RI for both e-type II codebook and AI based method, for configuration 1,

    - At 57% RU, 13.3% cell edge and 13.5% cell average performance gain is observed.

    - At 77% RU, 13.7% cell edge and 16% cell average performance gain is observed.

  - When RI selection for e-type II is based selected e-type II codeword (joint RI/PMI search), or AI decoder output, 4.9% cell edge gain and 4.8% cell average gain is observed at 74% RU.

  - When e-type II RI selection is based on selected codebook, and AI RI selection is based on ideal eigen-vector, performance loss is observed

- Others

CSI dataset separation in evaluations

To have an accurate test dataset, we should make sure that for a given UE not only the test samples but also the samples close in time with the test samples are not present in the training data. This issue has been discussed in the previous meetings, and some companies raised it for this meeting.

- ZTE: For CSI dataset construction, dataset should be generated in diverse drops rather than in divers TTIs at least for calibration purpose (e.g., only one TTI can be used for generating training dataset in a single drop).

KPI

### 3.1-5: Generalization for CSI compression

- Generalization over scenarios (dimensions of the input/output of the AI/ML model are unchanged)

- Various deployment scenarios (UMa/UMi/InH): Huawei, HiSilicon, vivo, Xiaomi, ZTE, OPPO, CATT, CMCC, Intel, NVIDIA, Spreadtrum Fraunhofer,

Findings on generalization verification

  - Huawei, HiSilicon: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa/UMi) for inference (i.e., generalization Case 2), its performance may be degraded compared to inference under Scenario#A (i.e., generalization Case 1), but mixing the dataset over the three scenarios for AI/ML model training (i.e., generalization Case 3) is helpful to improve the generalization

  - vivo: CSI compression models trained on a mixed UMi and InH dataset generalizes good over both UMi and InH scenario (generalization case3).

  - vivo: The best ratio of mixing data from each scenario is not necessarily to be evenly divided. For example, when considering mixing UMi and InH data, more samples from UMi scenario (e.g., 250k UMi samples and 25k InH samples) are required to achieve better performance.

  - ZTE: Case 3(AI model trained with mixed deployment scenarios) shows good generalization performance for various deployment scenarios.

  - Xiaomi: AI/ML model shows good generalization performance in different scenarios

  - OPPO: For different scenarios, the SGCS degradation is slight (about 1%~3%) when training set and testing set are mismatching

  - OPPO: For different scenarios, training on mixing dataset can improve the generalization performance of AI/ML model

  - CATT: For the generalization of AI/ML based CSI feedback, the following is observed

    - For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small

    - For applying AI/ML model in InH, the AI/ML model trained based on a dataset collected in UMa/UMi slightly outperforms the AI/ML model trained based on a dataset collected in InH

    - For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH. Training the AI/ML model with mixed data of UMa and InH can alleviate the performance gap

  - Intel: If dataset with both UMa and UMi channel models is used for training (Case 3) then performance loss is marginal comparing to training and testing on aligned dataset (Case 1)

  - CMCC: The AI model trained with mixed dataset across various scenarios might have some performance loss comparing with dedicated model.

  - NVIDIA: Evaluation results show that it is much easier for the autoencoders to compress CSI in CDL-C than in dense urban scenario, as the link level channel model CDL-C has fixed angle values and represents only a single channel realization while the system level channel in the dense urban scenario is much more sophisticated

  - NVIDIA: Evaluation results show the autoencoders trained in the sophisticated dense urban scenario perform well in CDL-C, illustrating the generalization capability of the AI/ML models

  - Spreadtrum: The AI based CSI compression and recovery trained under different scenarios can also achieve better SGCS performance than Rel-16 eType II codebook

  - Fraunhofer: If the AI/ML model is trained for a mixture of scenarios including different channel environments, there might be a performance loss compared to the case that the AI/ML model is trained only by specific data set

  - Fraunhofer:With the same acceptable NMSE, the NLoS channel requires smaller compression ratio than LoS channel. In other words, for NLoS, more feedback overhead is needed

Preliminary trends observed by more than one company

  - AI model generalizes well from UMa to UMi and vice versa. Huawei, HiSilicon, vivo Samsung, ZTE, CATT, Xiaomi

    - vivo: CSI compression models trained on UMi generalizes well over UMa scenario (generalization case2).

    - Samsung: AI model generalizes well from UMa to UMi and vice versa.

    - ZTE: The AI model trained in the scenario of UMa/UMi shows good generalization performance to InH dataset, while the AI model trained in the scenario of InH shows generalization degradation to UMa/UMi dataset.

    - CATT: For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small. For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH.

  - It is relatively difficult to generalize from Uma/UMi to InH. vivo, Samsung

    - vivo: CSI compression models trained on UMi generalizes poorly over InH scenario (generalization case2), and vice versa

    - Samsung: It is relatively difficult to generalize from UMa or UMi to InH.

  - It is relatively difficult to generalize from InH to Uma/UMi. Huawei, HiSilicon, ZTE, CATT

- Various outdoor/indoor UE distributions for UMa/UMi: Huawei, HiSilicon, vivo, Qualcomm, Spreadtrum

Findings on generalization verification

  - Huawei, HiSilicon: For generalization over indoor/outdoor UE distribution ratios,

    - AI/ML model trained by any indoor/outdoor UE distribution ratio shows similar performance on the outdoor testing dataset.

    - With the decrease of the indoor channel ratio for the training dataset, the trained AI/ML model brings decreased performance on the indoor testing dataset.

  - Qualcomm: Training on a dataset constructed by mixing the datasets of multiple scenarios enables the same ML model to perform well during inference in each of the scenarios

  - vivo: For a generic model (non-optimized for a specific area/cell), AI model trained in complicated channel environment (more indoor users) has good generalization ability

  - vivo: The performance of AI model depends on the deployment environment

  - Spreadtrum: For the inference performance in the UMa scenario with 8:2 outdoor/indoor UE distributions ratio, AI/ML model trained by 8:2 and 2:8 indoor/outdoor UE distribution ratio shows similar performance

Preliminary trends observed by more than one company

  - For generalization Case 2, AI model generalizes well for training with indoor (or higher indoor ratio) and inference with outdoor (or higher outdoor ratio), while a poor generalization performance is observed for the other way around. Huawei, HiSilicon, vivo, Qualcomm

- Various carrier frequency: vivo,

Findings on generalization verification

  - vivo: CSI compression models trained on one carrier frequency generalizes well over other carrier frequencies below 6GHz (generalization case2).

Preliminary trends observed by more than one company

  - AI model trained under one carrier frequency generalizes well over other carrier frequencies (e.g., from the set of 2GHz, 3.5GHz, 4GHz, 5.5GHz) vivo,

- Various Tx/Rx antenna spacing: vivo,

Findings on generalization verification

  - vivo: When compressing precoding matrix, CSI compression models generalize well over different RX antenna spacing (generalization case2).

  - vivo: For a generic model (non-optimized for a specific zone/site), there is obvious performance loss for antenna spacing mismatch of training data.

Effect of pre-processing

  - Issue: Performance loss for antenna spacing mismatch of training data

  - Solution: Pre-processing to achieve a small AI models with spatial domain and frequency domain compression

  - Findings: For a generic model (non-optimized for a specific zone/site), there is obvious performance loss for antenna spacing mismatch of training data and this performance loss may decrease when the model size is reduced with reasonable pre-processing vivo

- Various antenna virtualization (TxRU mapping): Huawei, HiSilicon, vivo,

Views

  - vivo: Further study the generalization on antenna spacing and antenna virtualization, by using pre-processing mechanism

Findings on generalization verification

  - Huawei, HiSilicon: For generalization over various TxRU mapping methods including (8,8,2,1,1,2,8) and (2,8,2,1,1,2,8), the AI/ML model trained by the mixed datasets show moderate performance on each of the TxRU mapping methods.

  - vivo: For a generic model (non-optimized for a specific zone/site), SGCS performance of AI model may degrade slightly from 128 antennas with virtualization to 32 antennas without virtualization

  - vivo: For a generic model (non-optimized for a specific zone/site), in the case of 32 antennas, AI model trained with 32 antennas may have similar SE performance compared with AI model trained with 128 antennas and settled in the case of 128 antennas (generalization case 2).

- Generalization over scenarios – fine-tuning

Views on fine-tuning

- OPPO: Regarding the EVM for fine-tuning, the following factors should be considered at least

  - Size of fine-tuning dataset

  - Sampling distribution of fine-tuning dataset

  - Diversity between fine-tuning dataset and original dataset

  - Fine-tuning delay

  - Performance gain

  - Other aspects related to fine-tuning

- OPPO: For the baseline of fine-tuning evaluation, direct training on fine-tuning dataset from random initialization and inference on the testing dataset should be considered as a benchmark

- Lenovo: Other than the average performance, companies are encouraged to report the performance of the AI/ML model for each link-types (UE groups)

  - At least initially, UE link-types (UE groups) can be defined as below. Other link-types are for FFS

  - link-Type1: Indoor/O2I

  - link-Type2: Outdoor-car/NLOS

  - link-Type3: Outdoor-car/LOS

Findings

  - Lenovo: Due to the non-uniform nature of number of UEs link-types in EVM, the average performance is biased towards one particular UE link-type

  - Lenovo: CDF of throughput/intermediate KPIs cannot effectively show the performance of all link-types.

- Lenovo: Having UEs with different link-types, model adaptation (e.g., fine-tuning, model update) may improve the performance of the AI/ML especially for small-size models. Of course, the gain and the cost associated with such model-adaptation should be evaluated.

- Lenovo: Model adaptation (e.g., fine-tuning, model update) for example for UEs of different link-types should be considered especially for small-size models.

- Lenovo: Study the over-the-air training of the models, especially for model update and model adaptation, which needs less amount of data exchange.

- Lenovo: Performance and cost associated with model adaptation (e.g., fine-tuning, model update) should be evaluated for different UE link-types

Findings on fine-tuning

- Huawei, HiSilicon: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa) for inference, its performance can be improved by applying fine-tuning using a relatively small dataset from Scenario#B.

- Ericsson:	By only fine-tuning the decoder, the performance degradation resulting from training with the wrong quantization, can be improved from -5.5% to -1.3%.

- Xiaomi: The fine-tuning procedure can be used to improve the performance

- ZTE: The AI model trained in the scenario of InH and fine-tuned with the dataset from a scenario of UMa/UMi shows 1%-5% SGCS performance gain over the AI model trained only in the scenario of InH under the testing dataset from the scenario of UMa/UMi.

- ZTE: With the increased samples of fine-tuning dataset, the AI model shows better generalization performance

- OPPO: For AI model trained on UMa and fine-tuned on UMi, the fine-tuning gain on SGCS performance is small with insufficient UMi fine-tuning dataset

- OPPO: For AI model trained on UMa and fine-tuned on CDL-C, the fine-tuning gain on SGCS performance is about 3.6% for larger fine-tuning dataset, which vanishes when fine-tuning dataset is small

- OPPO: For AI model trained on UMa and fine-tuned on UMi and CDL-C, the fine-tuning with insufficient datasets cannot achieve the equivalent SGCS performance as upper-bound

  - Note: upper-bound indicates using sufficient training set, which is from the same scenario as testing set.

- OPPO: Fine-tuning converges faster than baseline for both CDL-C#2k and CDL-C#20k datasets. Compared with baseline, fine-tuning achieves about 27.9% SGCS performance gain on smaller dataset CDL-C#2k and similar SGCS performance gain on larger dataset CDL-C#20k.

  - Note: baseline indicates using directly trained on CDL-C#2k and CDL-C#20k with random AI model initialization as starting point.

- Fujitsu: There is a huge penalty of the performance if the AI/ML-based CSI generation part and the AI/ML-based CSI reconstruction part are mismatched in the sense that they are trained using the datasets from different scenarios

- Fujitsu: The performance of the finetuning is very similar to that of joint training in terms of the SGCS

- Generalization over Configurations (dimensions of the input/output of the AI/ML model are different, i.e., scalability needed)

A number of companies evaluated the generalization under varying configurations.

- General views

  - Solutions to achieve scalability

    - vivo: Study the following three methods for generalization of input dimension

      - Option 1: use large dimension AI/ML model in small dimension cases: zero-padding

      - Option 2: use small dimension AI/ML model in large dimension cases: grouping

      - Option 3: use pre-processing to fix the input dimension: angle-delay domain compression

    - vivo: Study the pre-processing methods to support one CSI generation part with fixed input dimensions at UE side

    - OPPO: For scalability evaluation, zero-padding, clipping and truncation can be considered for pre-processing and post-processing

- Various bandwidths/frequency granularities: Ericsson, vivo, Qualcomm, OPPO, Xiaomi, ZTE, CMCC, ETRI, Fujitsu

Input/output dimension cases:

  - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

    - X1/X2/X3 =16 ports & 6 subbands/ 16 ports & 12 subbands / 32 ports & 12 subbands: NTT DOCOMO

Solutions:

  - Solution 1: Pre-processing to angle-delay domain.

    - vivo: For the preprocessing AI/ML model, angle-delay compression is used for preprocessing and 4 top strong beams on each polarization and 4 top strong paths are selected, which means the input dimension is 8 * 4 complex coefficients

  - Solution 2: Training with mixed variable subband configurations. E.g., training the AI model using random subband patterns in addition to the full subband case. Qualcomm

  - Solution 3: Adapt the subband size according to BW size (to keep the input dimension unchanged). E.g., 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz. ZTE, Ericsson

    - Ericsson: The evaluation agreement is to double the number of RBs per subband for the ground-truth calculation when doubling the considered bandwidth from 10 to 20 MHz

  - Solution 4: Zero padding. vivo Xiaomi, OPPO, CMCC ETRI, Fujitsu

    - vivo: For the normal AI model, the input is 13 subbands and 32 ports and zero-padding is used for less input dimension.

    - OPPO: the AI/ML model trained on dataset from Configuration#A with 13 sub-band can be inferenced/tested on dataset from Configuration#B with 8 sub-band and zero-padding on the sub-band domain

    - CMCC: we train AI model with the samples composed of the eigenvectors of 12 subbands and apply this AI model to test the samples composed of the eigenvectors of 8 subbands. To achieve better generalization performance, we will pre-process the samples composed of the eigenvectors of 8 subbands before inputting these samples into generation part, i.e., we will perform padding zero at the end of each sample

    - Fujitsu: the input of CSI generation part will be filled with zero value along the sub-band dimension and/or the antenna port dimension if input dimensions are less than pre-configured values

  - Solution 5: PCA/restoration. ETRI

    - For PCA based AI model for CSI compression, restoration is performed at the last stage of the Decoder. The restoration is to remove noises of the reconstructed channel data, where the noises are induced during downsampling, dimension reduction, and quantization…Transformer network is one of Recurrent Neural Network (RNN) and RNNs can process variable length of data sequences. The restoration NN in Decoder gets (reconstructed) eigenvectors of subbands as an input sequence and puts restored eigenvectors of subbands as an output sequence... The PCA based AI/ML model in the section 2.2.3 supports various sizes of input and output because the restoration NN is based on Transformer network which can get and generate variable lengths of sequences.

  - Solution 6: Adaptation layer CMCC, NTT DOCOMO

    - CMCC: a pair of down-sampling (DS-x block) and up-sampling (US-x block) is corresponding to one number of subbands while the EN block and the DE block are shared among all the configured subband numbers…We train the AI model with a mixed data, which compose of the eigenvectors of 10 subbands, 6 subbands and 4 subbands

    - NTT DOCOMO: the adaptation layer is introduced before the encoder and after the decoder

Findings on generalization verification

  - vivo: Zero-padding is feasible for subband number generalization while its performance degrades dramatically in port number generalization

  - vivo: Pre-processing performs well for both subband number generalization and port number generalization

  - Qualcomm: Training with mixed variable subband configurations achieve robust performance across all possible subband configurations including arbitrary number of subbands and arbitrary subband patterns

  - Qualcomm: Training with mixed variable subband configurations outperforms specific training with specific subband configuration

  - Ericsson:	The gains of the AE model over baseline are quite stable when changing the ground truth granularity, while the absolute KPI-values changes

  - Ericsson:	The adjustment of subband size in the pre-processing allows for easy reconfiguration and generalization of the AI/ML-model. The improvements over baseline for 20 MHz bandwidth are almost the same as for the 10 MHz case for which training data was provided

  - ZTE: AI/ML approaches can achieve good generalization performance for the case that the training dataset and testing dataset are generated with different bandwidth configurations but with the same model input size

  - Xiaomi: AI/ML model shows good generalization performance in different sizes of subband and subband number

  - CMCC: The AI model trained under one number of subbands might have some performance loss when performing interference on CSI compression of a different number of subbands

  - CMCC: For scalability over different number of subbands, the AI model with adaptation layers only suffers a little SGCS performance degradation compared with subband number-specific model

  - CMCC: For scalability over different numbers of subbands, the LSTM based AI model will suffer great SGCS performance degradation compared with subband number-specific model; however, when using mixed data to train a unified LSTM based AI model, it could achieve similar SGCS performance as subband number-specific model

  - NTT DOCOMO: The input-scalable model by training on mixed dataset with padding can achieve the similar performance as separately trained models

  - ETRI: For operation of AI/ML model over various bandwidths (or subband sizes), it is required that the AI/ML model to support variable sizes of input and output

  - ETRI: The AE-based AI/ML for CSI compression can infer a smaller bandwidth size than the training dataset using zero-padding in the inference phase

  - ETRI: In the evaluation of AI/ML for CSI compression over various bandwidths, the AE-based AI model showed performance decreases on untrained bandwidth (Case 2) compared to the trained bandwidth (Case 1)

  - ETRI: In the evaluation of AI/ML for CSI compression over various bandwidths, the PCA-based AI model showed less reduction in performance compared to the AE-based model. In addition, it is shown that the amount of performance degradation decreased as the feedback length increased

  - Fujitsu: For generalization/scalability of AI/ML model over the different number of sub-bands, the zero-padding method can achieve good performances in terms of the SGCS when the AI/ML model is trained with mixed data

- Various Tx/Rx antenna port layouts: Huawei, HiSilicon, vivo, Qualcomm, ZTE, Apple, CATT, OPPO, NTT DOCOMO, Fujitsu

Input/output dimension cases:

  - Case 1: One scalable CSI generation part with N>1 separate CSI reconstruction parts each with fixed dimensions

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

      - Findings: Using common encoder with gNB-specific decoder achieves higher SGCS than using common encoder with common decoder

  - Case 2: M>1 separate CSI generation parts each with fixed dimensions to one scalable CSI reconstruction part

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

      - Findings: Using common decoder with UE-specific encoder achieves higher SGCS than using common decoder with common encoder

  - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

    - X1/X2 =16/32 ports: Huawei, HiSilicon vivo CATT ZTE

    - 2x8, 4x4 and 2x4: Qualcomm

    - [8 2 2] and [4 4 2]: Apple

    - X1/X2/X3 =16 ports & 6 subbands/ 16 ports & 12 subbands / 32 ports & 12 subbands NTT DOCOMO

Solutions

  - Solution 1A: training a common AI model using mixed data set of 2x8, 4x4 and 2x4 antenna configurations. Qualcomm

  - Solution 1B: Case1: training on [8,2,2], testing on [8,2,2]; Case2: training on [8,2,2], testing on [4,4,2]; Case3: training on mixed dataset of [8,2,2] and [4,4,2], testing on [8,2,2]/[4,4,2]. Apple

  - Solution 2: Zero padding to 32 ports (and dataset mixing). Huawei, HiSilicon, ZTE, OPPO, Fujitsu

    - Huawei, HiSilicon: The input dimension of the CSI generation part is set as up to the maximum antenna port number (32), and zeroes are padded to match the input dimension in case of 16 ports input. Symmetrically, the output dimension of CSI reconstruction part is set as up to the maximum antenna port number (32) and the output is truncated from the tail to generate the output of 16 ports.

    - OPPO: the AI/ML model trained on dataset from Configuration#A with 32 port can be inferenced/tested on dataset from Configuration#B with 16 port and zero-padding on the antenna port domain

  - Solution 3: Adaptation layer CATT

    - CATT: Fully connected layers are used for linear pre-transforming (LPT-x block) and linear transforming (LT-x block) for the purpose of unifying input/output dimensions and probability distribution of eigenvectors from different port numbers.

  - Solution 4: Pre-processing to angle-delay domain.

    - vivo: For the preprocessing AI/ML model, angle-delay compression is used for preprocessing and 4 top strong beams on each polarization and 4 top strong paths are selected, which means the input dimension is 8 * 4 complex coefficients

Findings on generalization verification

  - Huawei, HiSilicon: For scalability of AI/ML-based CSI compression over various antenna port numbers, the SGCS margin between the port-specific models and the unified scalable model supporting 2 different antenna port numbers is less than 0.9%.

    - Pre/post processing of padding/truncation is used to achieve the scalability.

  - vivo: Zero-padding is feasible for subband number generalization while its performance degrades dramatically in port number generalization

  - vivo: Pre-processing performs well for both subband number generalization and port number generalization

  - Qualcomm: Training with mixed antenna configuration achieves robust performance across all antenna configurations in the training

  - Qualcomm: Training with mixed antenna configuration outperforms specific training with specific antenna configuration

  - ZTE: Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers

  - ZTE: AI model trained with the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

  - Apple: For generalization study case 2, when the autoencoder is trained in UMa with [8 2 2] antenna port layout, and test with [4 4 2] antenna port layout, large performance loss is observed

  - Apple: For generalization study case 3, when the autoencoder is trained in mixed dataset with [8 2 2] and [4 4 2] antenna port layout, and test with [4 4 2] antenna port layout, similar performance is observed as case 1.

  - OPPO: The scalability performance of AI/ML model for various antenna ports and CSI feedback payloads can be improved by trained on mixing datasets

  - Fujitsu: For generalization/scalability of AI/ML model over the different number of antenna ports, the zero-padding method can achieve good performances in terms of the SGCS when the AI/ML model is trained with mixed data

- Various CSI feedback payloads: Huawei, HiSilicon, vivo, Qualcomm, Xiaomi, CATT, OPPO, CMCC, ETRI, NTT DOCOMO,

General views of companies:

  - CATT: For AI/ML-based CSI compression, scalable AI model is considered

Input/output dimension cases:

  - Case 1: One scalable CSI generation part with N>1 separate CSI reconstruction parts each with fixed dimensions

    - vivo: Y1/Y2/Y3 =176/199/223 bits or 132/176/199 bits or 132/176/223 bits

    - Xiaomi: Y1/Y2/Y3 =120/180/240 bits

    - CMCC: Y1/Y2=32/48 bits; Y1/Y2/Y3=32/48/120 bits

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

  - Case 2: M>1 separate CSI generation parts each with fixed dimensions to one scalable CSI reconstruction part

    - OPPO: X1&Y1/X2&Y2 =16ports & 49bit / 32ports & 67bit

  - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

    - Huawei, HiSilicon: Y1/Y2/Y3/Y4=60/120/168/240 bits.

    - CATT: payload = 20, 40 … 320 bits

    - NTT DOCOMO: Y1/Y2/Y3=44/104/304 bits

Solutions:

  - Solution 1: Payload truncation. vivo, Xiaomi OPPO CMCC

    - vivo: Further study the scalability over output dimensions, e.g., CSI payload truncation mechanism with N>1 separate CSI reconstruction parts

    - Xiaomi: different feedback payloads are obtained by cutting off the tail of the maximum 240 bit. The loss function is the average results of different decoder parts

    - OPPO: When the AI/ML model trained on Configuration#A is adopted on Configuration#B, the first LB bits can be reserved, and the latter LA-LB bits can be truncated during the interface feedback. Then, the truncated  LA-LB bits can be regarded as default 0 or 1 for the decoder input.

    - CMCC: The dimensions of the output of generation part is designed based on the maximum feedback bits, and before outputting from the generation part, some extra bits will be dropped

  - Solution 2: Variable subband configurations with variable payload Qualcomm

    - Option 1-two payload configurations are considered (i.e., encoder output dimension = 32 and 64) and are trained at the same time;

    - Option 2-training using contiguous patterns with random number of subbands, and the number of subbands are randomly generated;

    - Option 3-Similar to Option 2 but arbitrary subband pattern is considered in the training

  - Solution 3: Adaptation layer Huawei, HiSilicon, CATT, NTT DOCOMO

    - Huawei, HiSilicon: Adaptation layers in the AI/ML models are used to adjust the dimension of CSI generation output and CSI reconstruction input, where different adaptation layers correspond to separate output/input dimensions to be consistent with the CSI payload sizes.

    - CATT: fully-connected layers are used for both down-sampling (DS-x block) and up-sampling (US-x block). At the inference phase, only one branch is activated according to the configured payload.

    - NTT DOCOMO three pairs of fully connected layers with different units are used

  - Solution 4: Adjusting quantization bit lengths ETRI

  - ETRI: The AE based AI Model for CSI compression can generate various CSI feedback payload sizes by controlling quantization bit lengths.

      - For example, the AE based AI Model can generates 86 bits of CSI feedback payload by quantizing 43 output nodes of Encoder by 2 bits each output. By changing quantization bit lengths of each output node of Encoder, the AI Model can generate different payload sizes

  - Solution 5: PCA/restoration. ETRI

    - ETRI: The PCA based AI Model can further change payload sizes by controlling number of reduced dimensions to represent the input data (e.g., eigenvector) and number of bits to quantize each dimension. For example, an input eigenvector can be reduced to 4 dimensions and quantized using 4, 4, 2 and 2 bits for each dimensions, respectively, then 12 bits of CSI payload can be generated

Views from companies

  - Xiaomi: RAN1 study pre-processing mechanisms for the input of decoder to improve the AI model generalization performance on various feedback payloads

Findings on generalization/scalability verification

  - Huawei, HiSilicon: For scalability of AI/ML-based CSI compression over various CSI payload sizes, the SGCS margin between the payload-specific models and the unified scalable model supporting 4 different CSI payload sizes is less than 1%.

    - Adaptation layers in the AI/ML model are used to achieve the scalability

  - vivo: Truncation on output dimension provides scalability across different payload for the same encoder under generalization case3.

  - vivo: For truncation method, the closer the supported payload configurations are, the better performance model achieves (e.g., model supporting payload 223, 199, and 175 outperforms model supporting payload 223, 176, 132).

  - vivo: Methods with good scalability for several output configurations will sacrifice some performance compared with dedicated model.

  - Qualcomm: Smaller number of subbands can achieve comparable results to the larger number of subbands with half of reporting payload

  - Xiaomi: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads

  - OPPO: The scalability performance of AI/ML model for various antenna ports and CSI feedback payloads can be improved by trained on mixing datasets

  - CATT: Compared with a family of layer-common AI/ML models, the scalable AI/ML model (SCsiNet) can achieve a similar performance and can significantly reduce storage memory and model transferring overhead

  - CATT: Compared to Rel-16 Type II codebook based CSI feedback, 3%~11% throughput improvement under the same CSI feedback payload can be achieved by proposed scalable AI/ML model

  - CMCC: The unified AI model of one common encoder and multiple specific decoders performs well across different number of feedback bits.

  - NTT DOCOMO: Implementing the payload-scalable model by adding adaptor layers after the encoder introduces minor performance loss on SGCS

  - ETRI: The PCA-based AI Model for CSI compression can generate various CSI feedback payload sizes by controlling bit allocations (quantization).

  - ETRI: In the evaluation of AI/ML for CSI compression over various CSI feedback payload sizes, the PCA-based model is more eligible to generate various lengths of CSI feedback payload than the AE-based model

  - ETRI: In the evaluation of AI/ML for CSI compression over various CSI feedback payload sizes, using different lengths of feedback than training did not result in significant performance degradations for both AI models

- Various ranks/layers: Huawei, HiSilicon, vivo, ZTE, Apple, MediaTek, NTT DOCOMO, ETRI

Solution:

  - Rank common model (Option 2) ETRI

    - ETRI: the rank-common AI/ML model operates on the entire input (input of multiple layers)

  - Layer common model (Option 4). E.g., a unified AI/ML model is trained and applied for each layer to perform individual inference. Huawei, HiSilicon, vivo, ZTE, MediaTek ETRI NTT DOCOMO

Elaboration

    - Option 4-A: A unified AI/ML model is trained with the dataset for layer 1 (and 2) (Case 1/2) only; Apple CATT ZTE

    - Option 4-B: A unified AI/ML model is trained with the mixed dataset for multi-layers (Case 3) Apple CATT ZTE

Views

    - MediaTek: Between all options of AI/ML models settings for rank>1, down select layer-common setting.

    - NTT DOCOMO: the layer-common model trained with the rank 2 data set is applied to the inference of rank 1 channel dataset

Findings on generalization verification

  - vivo: Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model.

  - vivo: In the case of rank = 2, assigning a less payload for the second layer can benefit to reduce the total CSI payload with less performance loss.

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, re-usage of AI/ML models (case 2 of generalization) at layer level causes 1.29% GCS degradation

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, re-usage of AI/ML models (case 2 of generalization) at rank level causes 1.13% GCS degradation

  - MediaTek: It is feasible to generalize a layer-common AI/ML model across ranks with negligible performance loss

  - MediaTek: On average, a unified AI/ML model (generalization case 3) not only does not degrade the feedback accuracy, but it also achieves 0.46% higher GCS accuracy compared to the dedicated AI/ML models for both layers

  - MediaTek: On average, a unified AI/ML model (generalization case 3) shows 5.8% higher GCS accuracy for EVs of layer 0 compared to those belonging to layer 1. The similar trend has also been observed among the dedicated AI/ML models

  - ZTE: The AI/ML model shows a good generalization capability across layers in the following cases

    - Trained with data from all layers and tested for different layers

    - Trained with data from the first layer and tested for different layers

    - Trained with data from the first two layers and tested for different layers

  - NTT DOCOMO: Directly applying AI/ML model trained on rank2 dataset to infer Rank1 dataset has little influence on SGCS performance, when the payload size is sufficiently large.

- Overall proposals on scalability

  - Qualcomm: For the evaluation of generalization of AI model to variable configurations, consider the following in data set generation:

    - For subband generalization, generate N>=1 random patterns (either contiguous or non-contiguous) for each data sample in the training set. The full subband pattern can be used in addition.

    - For antenna configuration generalization, mix data sample generated based on M antenna configuration with equal proportion.

    - Same configuration in the testing set and training set

As the other direction as opposed to achieving generalization, one company raised to adopt overfitting Per-cell (region) model with small size. In Moderator’s understanding, this is a different direction of studying AI/ML models as opposed to applying a generalized model with potentially large model size or requiring large training dataset. The principle of the per–area/cell/region model is to train a much smaller and overfitting model with smaller training dataset and apply it to a specific area, while on the other hand it may potentially not be applicable to other areas.

- Per-area/cell/region model

- General views

  - vivo: Study the performance and overhead of per-cell (region) model transfer in CSI compression

  - vivo: Consider to capture observations from field data test into TR

- Model size/structure

  - vivo: From initial results for field test, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical zone/site specific optimization.

- Overhead of model update

  - vivo: If the model structure of CSI generation part is simple(e.g., one-layer MLP), overhead of the model updating procedure will be very small (probably less than 100kB)

- Data collection

  - vivo: Training per-area models requires to enhance the data collection mechanism by some assistance information. Cell ID/sector ID or some other information that could represent the collecting area should be assigned to the corresponding data during dataset delivery. However, there could be some concerns on user privacy, UE storage, power consumption or overhead.

- Channel model

  - vivo: Here we consider using data where the channel has spatial consistency characteristics. Each UE generates random variables with spatial consistency based on its own geographic location at the T=0, both the cluster specific random variables and the correlation distance for spatial consistency procedure a follow 38.901.

- Findings on performance

  - vivo: From initial results for spatial consistency data, performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance when per cell model is used

  - vivo: From initial results for field test, as the model applicable area increases, simple model structure can still achieve good performance

- Other views/findings on generalization verification

- Qualcomm: The performance of AI/ML-based CSI compression using two-sided model can vary considerably if there is a discrepancy between the training data and inference data due to device-side variations

  - The data corresponding to different types of devices may have different characteristics. The source of such differences could be from device construction, RF aspects, implementation differences across vendors or device models or chipsets, etc.

- Nokia: Study various model architectures for generalization performance, including an assessment of the trade-off between performance and model complexity

- Nokia: Study the scalability of CSI compression models with parameters including gNB antenna array configuration, bandwidth/subcarrier spacing/subband count, and feedback overhead configuration.  Collect and summarize the results of these analyses

- OPPO: Suggest to study generalization issue and scalability issue separately

  - Focus on the same input and output CSI dimension with different configuration(s)/scenario(s) for generalization performance evaluation

  - Focus on different input and output CSI dimensions with different configuration(s) for scalability performance evaluation

    - E.g., different numbers of antenna ports, different number of sub-bands and different CSI feedback payloads

- OPPO: Suggest to construct some typical datasets with aligned scenarios/configuration(s) to draw the conclusion on generalization performance

  - Companies to report the details of utilized scenarios/configurations in the current stage

- OPPO: Suggest to construct some typical datasets with aligned configuration(s) to draw the conclusion on scalability performance

  - Companies to report the details of utilized methods and configurations in the current stage

- Fraunhofer:The performance of the AI/ML-based CSI compression should be evaluated on different datasets including multiple scenarios used for training

### 3.1-6: Monitoring

Methodology

- vivo: Discuss and develop an evaluation methodology for performance monitoring approaches. Following options can be considered as a starting point

  - linking metrics/results for specific monitoring methods to intermediate KPI results to see their relevance

  - modeling an environment changing procedure where models may be outdated and measuring the accuracy for different monitoring methods via system KPIs (e.g., throughput)

- Qualcomm: Training samples are partitioned into two groups … the distribution of the distance of a sample from samples belonging to an unmatched group is biased from the distribution of the distance to samples belonging to its matched group…we first divide the testing samples to matched samples and unmatched samples where matched samples are those belong to group 1 and unmatched samples are those do not belong to group 1. Then, we further divide matched samples and unmatched samples based on the distance to group 1.

  - Findings: Model monitoring based on ground-truth provided by UE to the network requires large signalling overhead and may be sensitive to large latency

- Qualcomm: Study mechanisms to monitor the performance of the AI/ML model to detect whether the data observed during inference is outside the distribution of the dataset used to train the model

- ZTE: Intermediate KPI based monitoring can be a starting point for evaluation on model performance monitoring

  - Findings: For model performance monitoring at UE side, monitored CSI can be used as reconstruction output at UE to achieve a good monitoring accuracy

- ZTE: For model performance monitoring, the following two cases can be considered

  - Case 1: Model performance monitoring at UE side

  - Case 2: Model performance monitoring at NW side

Ground-truth CSI-based monitoring

- ZTE: For model performance monitoring at NW side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good monitoring accuracy

  - With the model monitoring method, UE reports a reference CSI, and gNB calculates the SGCS between the reconstructed CSI and the reference CSI. gNB determines AI model performance is invalid when the SGCS between the reconstructed CSI and the reference CSI is less than the threshold, otherwise valid.

    - Thresholds of 0.7 and 0.8 are used respectively

    - the CSI of eType-II PC8 and the CSI of eType-II PC10 are used as the reference CSI respectively

Input-based monitoring

- Qualcomm: Model monitoring based on metrics derived by comparison between input samples at inference and training samples can have strong relationship with the inference accuracy. As a result, input-based monitoring appears promising

- Qualcomm: Study specification impact of input-based model monitoring on the UE-side by comparing input samples at inference time to the training samples

Legacy-CSI-based monitoring

- Qualcomm: compare the ML-based reconstructed CSI with the CSI obtained from some other reference CSI feedback scheme in similar channel conditions. As an example, a legacy non-AI/ML CSI feedback scheme can be used as the reference scheme

  - Findings: Model performance monitoring based on using a legacy CSI feedback scheme as a reference can detect model accuracy failure reliably and efficiently

- Qualcomm: For model performance monitoring, specification change for reporting the target CSI with high resolution requires clear justification as it may incur additional overhead

Proxy model-based monitoring

Solution description

- MediaTek: Instead of using the actual CSI reconstruction part of the gNB, UE can use a simplified proxy CSI reconstruction which results lower, but drifted, intermediate KPI compared to the actual one.

Views/findings

- MediaTek: It is feasible to design a proxy CSI reconstruction for which monitoring its drifted KPI detects the changes of working environment

- MediaTek: The structures of the proxy and actual CSI reconstruction parts do not need to match either in size or type (which further secures proprietariness of gNB’s CSI reconstruction).

- MediaTek: If intermediate-KPI-based monitoring is adopted as the main monitoring method, prioritize UE-side monitoring with proxy CSI reconstruction.

1st round email discussions

### 3.2-1: AI/ML training methods

- Issue#3-1 (Medium priority) Boundary between Type 2 and Type 3

Moderator note: In the last meeting, there is controversy on the boundary between Type 2 and Type 3. Background can refer to the question discussed in the last meeting, which is also pasted below:

#111 meeting Question 3.3.2: For the evaluation of training types, which of Type 2 and Type 3 should it belong to, if one side is trained first, and the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides?

For this meeting, Qualcomm thinks it belongs to Type 3 [31], while Huawei [3] and Ericsson [4] hold the view that it belongs to Type 2, and Type 2 should be clarified that gradient is passed between the sides. Thus, we can continue this discussion and clarification on the boundary between Type 2 and Type 3. To facilitate the discussions, Moderator copied the procedures for Type 2 and Type 3, respectively (Type 3 only NW first is provided as an example).

Conclusion For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example: - For each FP/BP loop, - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side - Step 3: UE side trains the CSI generation part based on the BP information from NW side - Note: the dataset between UE side and NW side is aligned. - Other Type 2 training approaches are not precluded and reported by companies

Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded and reported by companies

To Moderator’s understanding, this behavior may not be precisely matching with the wording of either Type 2, or Type 3:

- as shown in Type 2, in highlighted part it says NW trains the CSI reconstruction part, while this new behavior froze the NW side parameter updating in Step B; as shown in Type 3, in highlighted part it says NW shares the UE side with the information, while this new behavior needs NW and UE to exchange FP and BP (rather than a one way sharing).

- From the examples in the bracket of Type 2/3, this new behavior is more close to Type 2 as in Step B it also asks NW and UE to exchange FP/BP.

Therefore, we can continue the discussion on whether/how this new behavior is categorized.

Question 3.2.1: For the evaluation of training types of CSI compression, which of Type 2 and Type 3 should it belong to, if

- Step A: first side is trained, and after the training for the first side is finished

- Step B: the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, while the first side is frozen

| Type 2 | Support/Can accept | MediaTek, vivo, ETRI, OPPO, LG Electronics, Huawei/HiSi, Spreadtrum, Fujitsu |
| --- | --- | --- |
|  | Object/Concern | Qualcomm |
| Type 3 | Support/Can accept | IIT Kanpur, Qualcomm |
|  | Object/Concern |  |
| Not support this new behavior | Support/Can accept |  |
|  | Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| MediaTek | We believe if the BP crosses two entities, the method should belong to training type 2. |
| vivo | From our understanding, if a training method for CSI compression with two-sided models involves exchanging FP/BP information, it belongs to type2. So we prefer the mentioned training method is a special subcase of type2. |
| IIT Kanpur | As one sided is trained first and frozen then second side is then trained hence it belongs to Type 3 Type 2 requires simultaneous training of both side with exchanging of information over each (FP,BP) loop. Above view is given for 1 UE and 1BS pair. |
| ETRI | We think it is more close to Type 2 since it requires forward/backward propagations. |
| OPPO | This training type belongs to Type 2 training, since it also requires gradient exchange over the air interface. Moreover, according to previous agreement in 9.2.2.2, the Type 2 training should be discussed in lower priority. |
| Qualcomm | The conclusions cited above are not definitions but are examples of the Type 2 and Type 3 procedures for evaluation purpose (as the wording mentions “following procedure is considered as an example”). The agreement defining the training types in 9.2.2.2 states that Type 2 is joint training, and that “Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation”. However, in the method under discussion, the reconstruction model is trained in Step A and the generation model is trained afterwards in Step B. Therefore, this does not qualify as Type 2. This is just another example procedure for Type 3 training. In fact, it has some advantages compared to Type 3 training with dataset exchange. It is not clear why it should not be supported. Moreover, this training method shares a lot of similarities with Type 3 and has many differences from Type 2. Treating it as Type 2 training, will make the pros / cons / applicability discussion very confusing. For example, when capturing results (as discussed in Question 3.2.9), the results for this training type should be captured together with separate training, as this is one form of separate training. |
| Fujitsu | We agree with FL that it does not match exactly with either Type 2 or 3 training. However, considering the gradient exchange is involved in the training process for multiple epochs, it is more close to Type 2 training in terms of potential STD impacts, and hence can be categorized into Type 2 training. |

- Issue#3-1a (Low priority) Boundary between Type 2 and Type 3-how it works

Moderator note: To Moderator’s curiosity, whether it can support multi-NW and multi-UE. E.g., as for each side, when it finishes the training, it will be frozen when being trained in the next pairwise training. When it moves to Step E, as all UEs and all NWs are frozen, whether/how these two entities can be paired is questionable. Similar question is raised in Huawei’s contribution [3].

Question 3.2.2: For the new behavior raised in Question 3.2.1, whether/how such sequential training behavior can support the training of N>1 NWs and M>1 UEs?

| Company | View |
| --- | --- |
| Moderator | Not necessary to reply this question if you are not in favor of this new behavior. |
| vivo | We believe that such training method is designed to support one NW-side model to multiple UE-side models. It cannot support the case of one UE-side model to multiple NW-side models. |
| Qualcomm | It is not clear why the concern raised in this example is connected with the training procedure discussed in Question 3.2.1. Even for sequential training with dataset exchange, the same concern can be raised. Also, the motivation for focusing on this situation is not clear. Steps C, D and E are not for initial training but more for backward-compatible sequential training. The comparison with joint type 2 training across all vendors is not clear. During steps D and E, the UE#2 will have to train two different CSI generation models to address the problem raised. It is not clear why it is assumed that the UE side must develop a common CSI generation model for both network vendors. In practice, a single gNB may have to interact with different UEs simultaneously and this may require it to have a common CSI reconstruction model to work with both UEs. However, the corresponding requirement on the UE-side is not clear as the same UE does not typically interact with the CSI reconstruction model of multiple gNBs. |

- Issue#3-1b (On hold) Boundary between Type 2 and Type 3-conclusion on the boundary

Moderator note: Based on the discussions above, we may have a further clarification on the boundary between Type 2 and Type 3.

Proposed conclusion 3.2.1: To clarify the definition of training Type 2 and training Type 3,

- Type 2 requires the information exchange between two sides in each FP/BP loop, while Type 3 requires the sharing of the information in form of dataset from one side to the other side

- Issue#3-2 (Medium priority) Type 2 training – N>1 NW to M>1 UEs

Moderator note: This issue has been discussed in the last meeting. In this meeting, Huawei [3] raised that two training orders for N NW part models to M UE part models: Option 1 is the N NW part models to M UE part models are simultaneously trained, and Option 2 is they are sequentially trained in a pair-wise manner. To Moderator’s understanding, there is no need to make down-selection in the evaluation part, and companies can report how they achieve the joint training.

| Option 1 simultaneously trained (separate loss function) | Option 2 sequentially trained in pair-wise manner (after the training of first pair between UE 1 and NW 1, AI/ML model of UE 1 need to be frozen when training the next pair between UE 1 and NW 2) |
| --- | --- |

For how to generate the loss function, there are two views: Huawei [3] implied the loss function is generated separately for per NW vendor, while Ericsson [4] mentioned the loss function is jointly generated over N NW vendors. As how to generate loss function over N NW vendors would lead to different performance/convergence of the AI/ML models, companies can report how it is achieved in the simulation.

Therefore the following question is raised.

Proposal 3.2.1: For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), besides the evaluation of Case 2 (one NW part model and M>1 separate UE part models) and Case 3 (one UE part model and N>1 separate NW part models), N>1 NW part models to M>1 UE part models are additionally considered as an evaluation case (i.e., Case 4), and companies to report

- The training method/order, including the preference and details to achieve

  - Case A: Simultaneously train all N Network part models and M UE part models involving all vendors

  - Case B: Sequentially train N Network part models and M UE part models in a pairwise manner

    - Note: there is a different understanding on the training type categorization of Case B

  - Others

- The generation of loss function, e.g., whether loss function is jointly generated over N>1 NW part models, or separately generated for N>1 NW part models.

| Company | View |
| --- | --- |
| Moderator | This issue depends on the conclusion of the boundary between Type 2 and Type 3 as in Issue#3-1, but you can provide your comments to other parts of the proposal in parallel. |
| MediaTek | We support the proposal |
| vivo | In fact, we doubt the meaning of evaluating one UE-side model to multiple NW-side models for type2, as the joint training of one UE-side model to multiple NW-side models requires that one UE connects to multiple gNBs simultaneously and the involved gNBs should also communicate with each other, which is difficult realize in practice. |
| OPPO | Based on previous agreements in 9.2.2.2, the Type 2 training should be discussed in lower priority. |
| Fujitsu | We share a similar view to that of OPPO. |

- Issue#3-3 (Medium priority) Type 3 training – parallel training

Moderator note: In this meeting, CATT [15] raised a parallel training method, where UE and NW train the CSI generation part and CSI reconstruction part, respectively, based on a common dataset of {Channel, target CSI}, with no distinguishable sequential order. A similar idea is raised by ETRI [22].

An example of the procedure provided by CATT in the last meeting email discussion is given in below:

Step 1: A joint training model with CSI generation part + CSI reconstruction part is trained. Step 2: The set of information includes input and label of the UE side CSI generation part is generated by the joint trained CSI generation part in step 1, and the set of information includes input and label of the NW side CSI reconstruction part is generated by the joint trained CSI reconstruction part in step 1, where the same dataset for CSI feedback is used for the output of CSI generation part and input of CSI reconstruction part. Step 3: The UE side trains UE side CSI generation part with the set of information includes input and output of the CSI generation part generated in step 2 and the NW side trains NW side CSI reconstruction part with the set of information includes input and output of the CSI reconstruction part generated in step 2.

Based on these inputs, the same question as the last meeting is raised for continuous discussion of companies.

Question 3.2.3: For the evaluation of an example of Type 3 (Separate training at NW side and UE side), do you think it is needed to further introduce an additional training order of parallel training given in below, except for the sequential training orders of NW-first training and UE-first training as already concluded?

- Step 1: A pair of reference models including reference UE part model and reference NW part model are jointly trained.

- Step 2: A set of information (Set-UE) is generated from the reference UE part model (e.g., its input and label) trained in Step 1, and a set of information (Set-NW) is generated from the reference NW part model (e.g., its input and label) trained in Step 1.

- Step 3: The UE side trains the UE part model with Set-UE, and the NW side trains the NW part model with Set-NW.

| Company | View |
| --- | --- |
| Moderator | Please also provide your view/comments if you have a different understanding to the procedure of the parallel training in the question. |
| MediaTek | This approach can be decoupled into NW-first and UE-first at the same time. We do not think it requires a new category, neither any new evaluation. It can be categorized to type 3. |
| vivo | We think the evaluation of such separate training method should deprioritized. |
| ETRI | We support to consider the parallel training as another approach of Type 3. However, the proposed text only allows the parallel training using the reference model. In addition, we have a view that the parallel training can also be achieved by the structured latent space. We propose to capture the following steps which are modified from our companion’s contribution[R1-2301041]. - Step 1: The dataset for training or the reference dataset for latent space alignment is optionally shared between the NW and UE sides. - Step 2: The encoder is trained at UE-side using the shared dataset for training or its own dataset. At the same time, the decoder is trained at NW-side using the shared dataset for training or its own dataset. - Step 3: Alignment of the latent space between different entities using the shared reference dataset for latent space alignment is applied at either UE and NW-sides if needed. |
| OPPO | Our question is where Step 1 is performed? |
| NTT DOCOMO | There is no performance gain of parallel training compared to the sequential training. In addition, more steps are necessary to enable the parallel training. More justification is necessary to study it. |
| Fujitsu | Parallel training can be another Type 3 training, but we think that there is no need to evaluate this case. We should focus on evaluating NW-first and UE-first Type 3 training. |

- Issue#3-4 (Medium priority) Restricted/unmatched model pairing

Moderator note: In this meeting, the restricted/unmatched model pairing has been discussed and evaluated by companies (see Section 3.1-2 Unmatched/restricted model pairing). From Huawei [3], the restricted model pairing is defined as the pairing of a powerful/complicated model and a lame/simple model, which is due to the non-ideal model pairing due to non-cooperative model structure development or misaligned target performance metric between the Network vendor and the UE vendor; this combination will lead to inferior performance to the case where both sides have powerful models. Ericsson [4], MediaTek [34], and Apple [30] provide some simulation results to evaluate different combinations of model structures, including the unmatched model pairs, e.g., TF & TF, TF & FC, TF & CNN, CNN&CNN, etc. In addition, MediaTek [34] proposes to report the gain/loss for unmatched model pairs.

Maybe the modeling of the restricted/unmatched model pairing (i.e., what combinations to use and which combination is taken as baseline of ideal model pairing) can be up to companies, since it is hard to cross check over companies on the performance of each combination. But on the other hand it is interesting to investigate how much performance loss is achieved due to the restricted/unmatched pairing, thus a question is raised to collect views from companies.

Question 3.2.4: For the evaluation of CSI compression, which of the following options do you prefer, to evaluate the unmatched model pairing under the same training Type?

- Option 1: Up to companies to evaluate and compare, and no need for agreed upon simulation cases.

- Option 2: Agree upon simulation cases, and companies are encouraged to compare with matched model pairing, and report the performance accordingly.

  - FFS the clear boundary between matched/unmatched, and the list of model combinations to be simulated.

- Note: Unmatched model means different model architectures/backbones between the UE part model and the NW part model (e.g., CNN & Transformer), due to, e.g., the non-cooperative model development between the two sides.

| Option 1 | Support/Can accept | ETRI, OPPO, NTT DOCOMO, Qualcomm, LG Electronics, Huawei/HiSi, Spreadtrum, Intel |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | MediaTek, vivo (with comment for “unmatched model”), OPPO, Fujitsu |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | We have different understanding for “unmatched model”. From our view, the model structures for CSI generation part and CSI reconstruction part are not necessarily to be the same for joint training. If jointly trained, a CNN CSI generation part and a TF CSI reconstruction part can also achieve satisfying performance. We believe the issue of “unmatched model” only appears in type3 (i.e., separate training), referring to the case that the actual model has a different structure with the original model (e.g., for NW-first training, UE utilizes a CNN CSI generation model to learn the paired input-output data of a TF CSI generation model). |
|  | As we see that the matched model paring does not always guarantee the good performance in the non-cooperative model development, we think that it can be informatively provided by companies and no need for agreed EVMs. |
| OPPO | We think both options are okay and slightly prefer Option 1. This simulation cases can be optionally evaluated by companies. And how to realize the unmatched model architectures/backbones between the UE part and NW part can also be reported by companies. |

- Issue#3-5 (on hold) Type 2 training – benchmark of joint training w/ matched model pairing

Moderator note: As discussed in Issue#3-4, if the outcome is Option 2, i.e., to determine the simulation cases to compare matched model pairing and unmatched model pairing, then we can continue the discussion of adding an additional benchmark for Type 2 training, since generally Type 2 is assumed with unmatched model pairing due to non-cooperative model development between the two sides.

- Issue#3-6 (Medium priority) Type 3 training –joint training benchmark

Moderator note: In the RAN1#111 meeting, 3 simulation cases for training Type 3 have been agreed. For this meeting, Moderator observed that a number of companies considered a benchmark of joint training to compare the performance loss due to the separate training. Therefore, on top of the agreed cases, it is Moderator’s feeling that maybe we may introduce a benchmark of Case 0 of joint training for performance comparison, so that the performance loss can be better compared and captured in the template.

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model … - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models … - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models …

Question 3.2.5: For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, do you agree to add one more Case 0 with joint training as benchmark/upper bound for performance comparison?

- FFS the relationship between the pair of models for Type 3 and the pair of models for Case 0

| Support/Can accept | MediaTek, vivo,OPPO, NTT DOCOMO, Qualcomm, LG Electronics, Huawei/HiSi, Spreadtrum, AT&T, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | We support the benchmark. In our evaluation, we have already used training type 2 counterpart of any scenario as the benchmark |
| vivo | Performance of joint training with the same model configuration can be provided as a benchmark for separate training. |
| OPPO | Agree to utilize joint training as benchmark/upper bound. But it seems not necessary to add it as Case 0, since we can use Type 1 training as the benchmark. |

- Issue#3-6a (On hold) Type 3 training – Model pair for joint training benchmark

Moderator note: For the FFS part of Issue#3-6, in this meeting, MediaTek suggests using the joint training with the same model pair setting as the benchmark for performance comparison [34]. To Moderator’s understanding, the potential performance loss of Type 3 is due to two reasons: 1) the unmatched model pairing as addressed in Issue#3-4, and 2) the separate training with dataset sharing. Therefore, taking the same setting of the model pair to compare joint training and separate training seems a way to separate the two factors.

Question 3.2.6: For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, if one more Case 0 with joint training is added as benchmark/upper bound for performance comparison, the structures for the pair of NW part model/UE part model for Case 0 are the same with the Type 3 case to be compared.

### 3.2-2: AI/ML model settings

- Issue#3-7 (High priority) AI/ML model settings for rank>1

Moderator note: The issue of multi-rank model has been discussed for several meetings. From the inputs of this meeting, the preferences are still diverse, so it seems a realistic way is to let companies report which option is chosen in their evaluation rather than making a down selection at the moment.

Proposal 3.2.2: For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

  - FFS: input/output type

- Option 3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is layer specific and rank common (For a specific layer, the same model is applied for all rank values), or layer specific and rank specific (For a specific layer, different models are applied for different rank values)

- Option 4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is layer common and rank common (A unified AI/ML model is applied for each layer to perform individual inference), or layer common and rank specific (For a specific layer, different models are applied for different rank values)

- Other options not precluded.

| Support/Can accept | MediaTek, vivo (support Option-2 and 4), CATT, Futurewei, ETRI, OPPO, NTT DOCOMO, Qualcomm (comment), LG Electronics, Huawei/HiSi, Spreadtrum, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | Support. |
| vivo | For option-2, even we have a single model commonly for all ranks, the performance degradation can be negligible. For option-3, if the model per layer corresponds to different payload size, the overall overhead can be reduced without performance degradation. |
| CATT | As we clarify in our contribution, rank common/specific and layer common/specific can be viewed from training or can be viewed from inference. The actual valid combination between training and inference may be more than what is listed. But if it is up to companies’ report, the proposal is OK to us, as long as companies clearly share their training and inference strategy. |
| Futurewei | We prefer Option 4 (layer common) but can accept other Options. |
| NTT DOCOMO | Generally fine with the proposal. However, the example of layer common and rank specific looks misleading to us. We understand that layer common and rank specific model should be literally the same model over layers while the model could be specific to rank. Prefer the following modification. layer common and rank specific (For a specific layer, different models are applied for different rank values) The performance of rank common and layer common AI/ML model deteriorate a bit from rank specific layer common AI/ML model. But it is acceptable considering its simplicity. Companies could just report their assumption. |
| Qualcomm | We support the intent, but the 4 options listed are not mutually exclusive. One way to address this is to use the 4 combinations to define the options: rank-{common/specific} and layer-{common/specific}. Another approach could be to split this into two proposals, one for rank and one for layer. |
| LG Electronics | Our preference is option 4, but can accept the proposal. |
| Fujitsu | We are fine with this proposal in general. @Moderator There is one case that needs further clarification. Consider an AI model with scalability of payload, which is to be applied to different layers (e.g., two layers with different payload sizes), does it belong to option 4? |

- Issue#3-8 (on hold) AI/ML model settings for rank>1-complexity report

Moderator note: For the “FFS on the reported complexity and storage” of Issue#3-8, Ericsson mentioned in its contribution [4] that the FLOPs is reported as the total FLOPs of all layer-specific models for the multi-rank case, while the storage/parameter is also the summary for all layer-specific models.

From Moderator’s understanding, as the number of models and the times of inferences over the options in Issue#3-8 are different, it is still meaningful to make a comparison over the options on the price of each one. E.g., the storage of the layer specific model may be the summary of all models; the FLOPs for layer-common or layer-specific model is the summary of the FLOPs for each layer; the FLOPs for rank-specific model is reported as the maximum over all models, etc.

Proposal 3.2.3: For the complexity and storage report for the AI/ML based CSI compression sub use cases with rank >=1,

- FLOPs

  - Option 1 (rank specific): maximum FLOP over each rank specific model

  - Option 3 (layer specific): summarized FLOPs over the layer specific models inferenced for each layer

  - Option 4 (layer common): summarized FLOPs for the layer common model inferenced for each layer

- Memory storage/number of parameters

  - Option 1 (rank specific): summarized Memory storage/number of parameters over the rank specific models

  - Option 3 (layer specific): summarized Memory storage/number of parameters over the layer specific models

| Support/Can accept | Vivo, OPPO, Qualcomm (comment) , Huawei/HiSi, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | For simplicity, we are supportive of Option 1 for FLOPs, but we are fine with Option 4 by layer common. |
| Qualcomm | We have the same comment as in the previous proposal - the 4 options listed are not mutually exclusive. One way to address this is to use the 4 combinations to define options: rank-{common/specific} and layer-{common/specific}. |

### 3.2-3: Other EVM for CSI compression

- Issue#3-9 (High priority) Payload size alignment

In the previous two meetings, for the CSI payload size alignment, there are different understandings, as summarized in the following options.

- Option 1: Payload size is calculated based on the maximum rank. - Option 2: Payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. FFS the following understandings - Option 2a: The above-mentioned “CSI payload” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank

After the clarification of the last meeting, at least companies can somehow align that the “maximum payload size” in the agreed evaluation metric applies at least to AI/ML based solution, while for whether it is also applied to the Type II CB, there is still controversy, considering there may be partial non-zero coefficients (NZC) reported by UE, which leads to mismatch between the actual reported CSI payload size and the maximum CSI payload size.

| Evaluation Metric | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| --- | --- |

In addition, for most of the companies taking Option 1 as 1st preference, it looks they can also live with Option 2b. Thus Option 1 is taken away from the following proposal, and two alternatives are provided, to collect the views from companies:

Proposal 3.2.4: For the CSI payload size calculation, between the two options:

- Option 2a: The above-mentioned “CSI payload” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank

- Alt.1: Option 2b is adopted for both AI/ML based solution and the legacy Type II CB

- Alt.2: Option 2b is adopted for AI/ML based solution, while whether Option 2a or Option 2b is adopted is reported by companies if partial NZC report is assumed for the legacy Type II CB.

- Alt.3: Other

| Alt.1 | Support/Can accept | MediaTek, vivo, CATT, Futurewei, OPPO, NTT DOCOMO, Qualcomm, ZTE, LG Electronics, Huawei/HiSi, Spreadtrum, AT&T, Fujitsu, Intel |
| --- | --- | --- |
|  | Object/Concern |  |
| Alt.2 | Support/Can accept | Huawei/HiSi |
|  | Object/Concern | Qualcomm |
| Alt.3 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | We have a concern on having Option 2a and Option 2b besides Option 1. Having both options is not good choices due to the misalignment of payloads between companies. If majority companies go to Option 1, we should take an agreement only on Option 1. |
| NTT DOCOMO | Option 1 is our previous preference. But it seems more companies prefer Option 2b. Could be fine with Option 2b just for evaluation. In this case, the payloads sized in general should be calculated based on the “max allowed bits at the given rank” as well as the distribution of different ranks during the SLS performance evaluation. |
| Qualcomm | The goal is to quantify the resource overhead for CSI feedback. The gNB cannot anticipate whether the UE will give a partial NZC report or not, and must allocate UCI resources without that knowledge. Hence, Option 2b is more relevant, and also aligned with previous agreement. Therefore Alt 1 should be selected. Wording suggestion: Since Alt 1 and Alt 2 are the same for the AI/ML based solution, the wording could be simplified by saying this proposal is about legacy Type II CB only. |
| ZTE | Alt 2 would incur misalignment for companies to calibrate than Alt 1. |

- Issue#3-10 (Low priority) CQI calculation

Moderator note: In this meeting, two companies discussed the CQI calculation methods as discussed in 9.2.2.2, wherein one provides the simulation results. An open question is then raised to collect the views from companies on whether/how to agree upon the EVM/simulation cases for CQI calculation, or allow companies to report the results of different CQI calculation methods.

Question 3.2.7: For the evaluation of CSI compression, do you think we need to agree upon the EVM/simulation cases or allow companies to report (e.g., by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g.,

- CQI is calculated based on the output CSI from the realistic channel estimation

- CQI is calculated based on the target CSI from the realistic channel estimation

- CQI is calculated based on traditional codebook

| Company | View |
| --- | --- |
| MediaTek | Support |
| ETRI | We propose to add one example as: - CQI is calculated based on the input CSI from realistic channel estimation The reason behind the following proposal is that the UE can have neither the output CSI (when the UE does not have the generation model) nor the target CSI (when the target CSI of the generation model is different to the input CSI). |
| OPPO | Up to companies report. |
| NTT DOCOMO | We prefer to allow companies to report the CQI determination method. |
| LG Electronics | Support. |
| AT&T | Support |

- Issue#3-11 (Low priority) RI calculation

Moderator note: In this meeting, Apple [30] simulated with different RI determination solutions for legacy Type II CB, where under an optimized/exhaustive search of RI&PMI determination solution, legacy Type II CB can outperform AI/ML based compression.

As per Moderator’s understanding, it is up to companies to consider the implementation based solution for RI determination, but whether/how to report such implementations can be further discussed. An open question is then raised to collect the views from companies on whether we need to report the RI calculation methods (or to be more generic, any implementation based solutions for benchmark) in the template.

Question 3.2.8: For the evaluation of CSI compression, do you think it is needed to allow companies to report (e.g., by introducing an additional field in the template to describe) the specific implementation (e.g., RI/PMI determination, etc.) for benchmark (e.g., Type II CB)?

| Company | View |
| --- | --- |

### 3.2-4: Template for simulation results collection

- Issue#3-12 (Medium priority) Template for the baseline table - CSI payload size

Moderator note: The initial template has been determined in the last meeting, and some companies discussed the table in this meeting. For the FFS the value or range of payload size X/Y/Z, the summarized CSI payload sizes can refer to Section 3.1-4 Template for simulation results collection.

Note that as the CSI payload sizes may hardly be exactly aligned over companies, it may be possible to define a CSI payload range, e.g., 40bits-80bits as low payload, 100bits-140bits as medium payload, and >200 bits as high payload. These ranges may more or less cover the submitted results of companies at least for rank 1/2.

In addition, some company raised that rank 3/4 should keep the comparable payload size as rank 2, as is the principle for R16 Type II CB. Therefore, for rank 3/4,

Proposal 3.2.5: For the initial template for AI/ML-based CSI compression without generalization/scalability verification, the range of the CSI feedback payload for per layer under rank 1/2 is determined as:

- X is 40bits-80bits

- Y is 100bits-140bits

- Z is >200bits

- FFS for rank 3/4

| Support/Can accept | MediaTek, OPPO, NTT DOCOMO, Huawei/HiSi, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | Generally okay to this proposal. But it is better that no CSI payload gap between X, Y and Z. For example, X is 40bits-100bits, Y is 100bits-200bits and Z is >200bits. |
| Intel | The range of 40 bits seems too wide. Can we support lower resolution (e.g. 20 bits)? |

- Issue#3-13 (High priority) Template for the baseline table – training types/cases

Moderator note: The initial template has been determined in the last meeting. For the training types/cases, there was a debate in the last meeting that whether the unmatched/restricted pairing of Type 2 (i.e., Type 2 with 1 NW to 1 UE) should be categorized into the baseline table, or a different table (see). Some companies think Type 2 with 1 NW to 1 UE and Type 1 are different since for Type 2, Network and UE don’t know the model structure of each other and thus there would be performance loss for Type 2.

Whether/how to model the restricted/unmatched model pairing is discussed also in Issue#3-4, and it seems difficult to align the combination of models over companies to cross check the performance loss due to this restricted/unmatched pairing.

In addition, some company raised in the contribution that even for Type 1 training, it may practically not achieve the ideal model pairing due to the compatibility. This will consequently lead to the joint training of 1 NW with M>1 UEs, which is similar to the multi-vendor case of Type 2.

Therefore, it is Moderator’s thinking that maybe we can avoid using “Type 1”/“Type 2” from the simulation perspective to distinguish the training cases and templates, but use the following terms to separate their templates:

- “1-on-1 joint training”, which means the CSI generation part and the CSI reconstruction part can be regarded to be jointly designed/developed/optimized as if by a single side) to represent the upper bound of the performance. For training Type 1/2 with 1 NW and 1 UE, it is subject to this case. Whether the model pairing is unrestricted/matched or restricted/unmatched, it can refer to Issue#3-4.

- “Multi-vendor joint training”, which means 1 NW to M>1 UEs, N>1 NWs to 1 UE, or N>1 NWs to M>1 UEs are jointly trained which may introduce additional loss due to the interaction with multi-vendors. Type 2 with multi-vendor training are subject to this case, while whether Type 1 also faces the multi-vendor case is FFS.

- “Separate training”. This is Type 3. The dataset sharing will lead to additional loss as compared to Type 1/2 joint training for the same pair of models.

The question is then raised to collect views from companies.

Question 3.2.9: For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, do you agree with the following:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

  - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate temple is introduced to capture the evaluation results for “training Type 3”

| Support/Can accept | MediaTek, vivo, IIT Kanpur, CATT, ETRI, OPPO, NTT DOCOMO, Qualcomm (comment), ZTE, LG Electronics, Huawei/HiSi, Fujitsu |
| --- | --- |
| Object/Concern | Lenovo |

| Company | View |
| --- | --- |
| Lenovo | We agree that we need to define different tables to better compare different case. But it is not limited to if it is Joint 1-1 or multi-vendor. As we discussed in our contribution, the important KPIs depends on at least the following for items. (Training scheme, Lifecycle stage, Training entity, Dataset type, Number of vendors, if_twosides_are_visible) where Training scheme: Type 1, Type 2, Type 3 Lifecycle stage: Initial Training, Model update, Fine-tuning Training entity: UE, gNB, both UE and gNB, cloud node(s) Dataset type: Simulated dataset, Offline field dataset, Online (near-real time) The template that is agreed in RAN1#111 is related to the case of: (Type1, Initial Training, A cloud node, 1 (one vendor), True (twosides_are_visible)) And it can be the base for other cases. |
| OPPO | We are general okay. But we have the following concerns: For Type 3 training, there are also 4 kinds of training cases including 1-to-1 separate training, 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, and N>1 MW part models to M>1 UE part models. For each case, we should consider both NW-first and UE-first methods. So this proposal means for Type 3 training, only one template is required for all above these conditions? |
| Qualcomm | We agree it would be more precise and clear to describe the evaluation approach in the title, instead of the training type number. For the third template also, it would be better to say “separate training” instead of “training Type 3”. |

- Issue#3-14 (Medium priority) Update of the baseline template (1)

Moderator note: In this meeting, a number of companies submitted the simulation results in the template. From the submitted results, Moderator finds that the table may need some updates and clarifications.

The first issue is that the title of the template includes the description of “traffic type”. However, as per our conclusion in RAN1#110bis-e, FTP model 1 with 0.5MB is assumed as the mandatory, so there seems to be no need to keep “traffic type” in the title.

Conclusion For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows: Traffic model; At least, FTP model 1 with packet size 0.5 Mbytes is assumed Other options are not precluded..

The suggested change is made as follows.

Question 3.2.10: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, the following update is made to the title of the template?

Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case]

| Support/Can accept | MediaTek, ETRI, NTT DOCOMO, LG Electronics, Huawei/HiSi |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Qualcomm | The conclusion also says that other options are not precluded. Removing the traffic type from the title will leave no place to capture the results of other options. Moreover, keeping it in the title can be informative, and the benefit of removing it is not clear. |

- Issue#3-14a (Medium priority) Update of the baseline template (2)

Moderator note: The second issue is that the description for Intermediate KPI#1 and Intermediate KPI#2 is missed.

Question 3.2.11: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, the following update is made to the “Common description”?

|  |  | Source 1 |
| --- | --- | --- |
| … |  |  |
| Common description | Input type |  |
|  | Output type |  |
|  | Quantization /dequantization method |  |
|  | Description of Intermediate KPI#1 |  |
|  | Description of Intermediate KPI#2 |  |
| … |  |  |

| Support/Can accept | MediaTek, ETRI, OPPO, Qualcomm, Huawei/HiSi, AT&T, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ETRI | We think this is necessary. |
| Intel | In our view at least one intermediate KPI should be common between companies (SGCS per layer). |

- Issue#3-14b (Medium priority) Update of the baseline template (3)

Moderator note: The third issue is that the CSI overhead (reduction) which is part of the “Evaluation metric” is missed.

| Evaluation Metric | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| --- | --- |

Question 3.2.12: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, the following update is made to the “Common description”?

| Gain for 5% UPT | CSI feedback payload X |  |  |  |
| --- | --- | --- | --- | --- |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| CSI feedback reduction (%) |  |  |  |  |
| … |  |  |  |  |

| Support/Can accept | MediaTek, ETRI, OPPO, NTT DOCOMO, Qualcomm (comment) , LG Electronics, Huawei/HiSi, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| NTT DOCOMO | We suggest to align the calculation method for CSI feedback reduction, if it is included in the template. |
| Qualcomm | We support adding this metric. However, the value may be different for different payload size settings. It might be useful to add a way to report multiple values at different points, similar to the UPT gain. |
| Intel | The methodology to calculate it shall be aligned between companies. Otherwise, we can’t compare it. |

- Issue#3-14c (On hold) Update of the baseline template (4)

Moderator note: The fourth issue is that the report of the rank>1 schemes (Option 1/2/3/4 in Issue#3-7) is missed.

Question 3.2.13: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, the following update is made?

|  |  | Source 1 |
| --- | --- | --- |
| … |  |  |
| Common description | Input type |  |
|  | Output type |  |
|  | Quantization /dequantization method |  |
|  | ... |  |
|  | Rank/layer adaptation settings for rank>1 |  |
| … |  |  |

| Support/Can accept | MediaTek, OPPO, Huawei/HiSi, AT&T |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

- Issue#3-14d (Medium priority) Update of the baseline template (5)

Moderator note: The fifth issue is that the some necessary explanation of the items in the template is missed, so that different companies may have different understandings to the template.

Question 3.2.14: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, the following update is made as the explanation of the template?

| … |  |  |  |  |
| --- | --- | --- | --- | --- |
| FFS others |  |  |  |  |

Note: “Benchmark” means the type of Legacy CB used for comparison.

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

| Support/Can accept | MediaTek, ETRI, OPPO, NTT DOCOMO, LG Electronics, Huawei/HiSi, AT&T, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

- Issue#3-14e (Low priority) Update of the baseline template (6)

Moderator note: Some suggested updates are raised by Intel [19], Xiaomi [13], and Lenovo [27].

Intel: The first point is due to that the number of output neurons for the CSI generation part can be significantly different for different CSI payload size.

The second point is due to that it is not clear how to calculate performance gain, i.e., it is not clear which eTpye II point shall be used as a reference for gain calculation if the point for AI-ML CSI lies in between of two eType II points w.r.t. the PMI overhead value. To Moderator’s understanding, it may be up to companies to find/determine the CSI feedback payload points for AI/ML and Type II CB as close as possible, or alternatively use some interpolation to align the payload point. But still

- For evaluation results collection table, - At least some fields of the table including number of parameters, storage and complexity of the AI-ML model shall be separately specified per CSI payload value - Remove fields corresponding to performance gains and support fields for absolute performance of eType II PMI codebook as well as AI-ML CSI (for intermediate metrics and UPT)

Xiaomi: Newly added KPIs provided in below

- The parameter and metric for complexity of AI model training e.g., the training related overhead, the required dataset size for convergence, number of epoch times, batch size and learning rate are captured in the template table, - AI model structure e.g., the number of layers is provided in the template table at least for case 2 of Type 3 separate training

Lenovo: Newly added KPIs provided in below

Data collection overhead Data collection latency Model Delivery overhead Model Delivery latency Model Training

Question 3.2.15: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, your view to the following points of changing?

- Change#1: The complexity related KPIs (number of parameters, storage and FLOPs) are separately specified per CSI payload value

- Change#2: Remove fields corresponding to performance gains and support fields for absolute performance of eType II PMI codebook as well as AI-ML CSI (for intermediate metrics and UPT)

- Change#3: Add a field to reflect the effort of training (e.g., the number of epoch times, batch size, learning rate)

- Change#4: Update the “AL/ML model backbone” to “AL/ML model description (e.g., backbone, number of layers, etc.)”

- Change#5: Add a field to capture data collection overhead (FFS training, monitoring, etc.)

- Change#6: Add a field to capture latency related KPI (e.g., data collection latency, data delivery latency, etc.)

| Change#1 | Support/Can accept | MediaTek, OPPO, Intel |
| --- | --- | --- |
|  | Object/Concern |  |
| Change#2 | Support/Can accept | MediaTek, OPPO, Intel |
|  | Object/Concern |  |
| Change#3 | Support/Can accept | MediaTek, OPPO |
|  | Object/Concern |  |
| Change#4 | Support/Can accept | MediaTek, OPPO |
|  | Object/Concern |  |
| Change#5 | Support/Can accept | MediaTek, OPPO |
|  | Object/Concern |  |
| Change#6 | Support/Can accept | MediaTek |
|  | Object/Concern | OPPO |

| Company | View |
| --- | --- |
| Moderator | Proposal of adding training type related field is postponed as a separate table is to be discussed in prior |
| MediaTek | We are not clear about the benefits of sharing info coming from change#3. |
| OPPO | Not clear about how to evaluate the latency related KPI. |

- Issue#3-14f (Low priority) Update of the baseline template (7)

Moderator note: Please input your views if you have other suggestions on the changes of the initial template.

Question 3.2.16: For the initial template of CSI compression achieved in the working assumption in the RAN1#111 meeting, do your have other proposals to the changes/updates of the table?

| Company | View |
| --- | --- |

- Issue#3-15 (Medium priority) Template for CSI compression-Table for multi-vendor joint training

Moderator note: As the FFS part in the last meeting for the initial template, that whether the results for different training types/cases are captured in separate tables need to be discussed.

Depending on the outcome of Issue#3-13, if a separate table is needed, we may further discuss the template as in below.

Proposal 3.2.6: The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for multi-vendor joint training and without generalization/scalability verification

- FFS whether to provide traffic types, UPT, overhead, and RU results

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression of [multi-vendor joint training] without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Train/k |  |  |
|  | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Case 1 (baseline): NW#1-UE#1 | UE part AL/ML model backbone/structure |  |  |
|  | Network part AL/ML model backbone/structure |  |  |
| ... (other NW-UE combinations for Case 1) |  |  |  |
| Case 2 (1 NW part to M>1 UE parts) | UE#1 part training dataset |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | NW part model backbone/structure |  |  |
| Case 3 (N>1 NW parts to 1 UE part) | NW#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | UE part model backbone/structure |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| Case 1: NW#1-UE#1: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other NW-UE combinations for Case 1) |  |  |  |
| Case 2: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

| Support/Can accept | MediaTek, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Whether we will have a separate table for this training case depends on the outcome of Issue#3-13, but you can provide your comments to the content of the proposal in parallel. |

- Issue#3-16 (Medium priority) Template for CSI compression-Table for training Type 3

Moderator note: As the FFS part in the last meeting for the initial template, that whether the results for different training types/cases are captured in separate tables need to be discussed.

Depending on the outcome of Issue#3-13, if a separate table for Type 3 is needed, we may further discuss the template as in below.

Proposal 3.2.7: The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for training Type 3 and without generalization/scalability verification

- FFS whether to provide traffic types, UPT, overhead, and RU results

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression of training Type 3 without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Shared output of CSI generation part/input of reconstruction part is before or after quantization |  |  |
| Dataset description | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| [Benchmark: NW#1-UE#1 joint training] | UE part AL/ML model backbone/structure |  |  |  |
|  | Network part AL/ML model backbone/structure |  |  |  |
|  | Training dataset size |  |  |  |
| ... (other NW-UE combinations for benchmark) |  |  |  |
| Case 1-NW first training | NW part AL/ML model backbone/structure |  |  |  |
|  | UE#1 part model backbone/structure |  |  |  |
|  | UE#1 part training dataset size |  |  |  |
|  | … |  |  |  |
|  | UE#M part model backbone/structure |  |  |  |
|  | UE#M part training dataset size |  |  |  |
| Case 1-UE first training | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset size |  |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset size |  |  |  |
|  | UE part model backbone/structure |  |  |
| Case 2-NW first training | NW#1 part model backbone/structure |  |  |  |
|  | … |  |  |  |
|  | NW#N part model backbone/structure |  |  |  |
|  | UE part model backbone/structure |  |  |  |
|  | UE part training dataset size |  |  |  |
| Case 3-UE first training | UE#1 part model backbone/structure |  |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE part AL/ML model backbone/structure |  |  |
|  | NW part training dataset size |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| [NW#1-UE#1 joint training: Intermediate KPI] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |
| Case 1-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 1-UE first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 2-NW first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

| Support/Can accept | MediaTek, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Whether we will have a separate table for this training case depends on the outcome of Issue#3-13, but you can provide your comments to the content of the proposal in parallel. |

- Issue#3-17 (High priority) Template for CSI compression-Table for Generalization/scalability

Moderator note: As the FFS part in the last meeting for the initial template, that whether the results for generalization/scalability are captured in a separate table need to be discussed.

Proposal 3.2.8: A separate table to capture the evaluation results of generalization/scalability verification is needed as given in the following initial template

- FFS whether to provide traffic types, UPT, overhead, and RU results

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression of training Type Y with model generalization/scalability, [traffic type], [Max rank value], [RU]

Table A.4 Template for CSI compression with model generalization

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| Intermediate KPI #1, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| Intermediate KPI #1, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other scenarios/configurations for Case 1) |  |  |  |
| Intermediate KPI #2, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| Intermediate KPI #2, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … |  |  |  |
| Generalization Case 2 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … (other scenarios/configurations for Case 2) |  |  |  |
| Generalization Case 3 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … (other scenarios/configurations for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting, size/k) |  |  |
|  | Fine-tune (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| FFS others |  |  |  |

| Support/Can accept | MediaTek, vivo, IIT Kanpur, CATT, OPPO, Qualcomm, ZTE, Huawei/HiSi, Spreadtrum |
| --- | --- |
| Object/Concern | Lenovo, Futurewei |

| Company | View |
| --- | --- |
| Lenovo | We suggest to first complete the temple for cases without generalization and then discuss this template. Especially Issue#3-14e. |
| Futurewei | The proposed table didn’t clearly indicate the following: - Assuming the intermediate KPI(s) are reported for the test dataset, there should also be an entry for the benchmark intermediate KPI(s) that are calculated when model was built using data from the test scenario/configuration. This benchmark can be considered as an upper bound to better understand the generalization performance. - There is no entry for eventual KPIs, i.e., mean UPT and 5% UPT. - If techniques other than fine-tuning is leveraged, e.g., using adaptation layers, additional pre/post processing, or others, there should be an entry for companies to report it. |

### 3.2-5: Quantization/dequantization

- Issue#3-18 (Medium priority) Quantization loss/quantization upper bound

Moderator note: In this meeting, Nokia [14] raised to define quantization loss as the difference between the reconstruction metric (SGCS or NMSE) obtained from a model trained without quantization and a model which incorporates quantization whether through quantization aware or unaware training. With a similar idea, Xiaomi [13] mentioned that the case of training without quantization and inference without quantization should be evaluated as the upper bound. Therefore, a question is given in below on whether an upper bound of CSI compression without quantization (i.e., float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training & inference) for comparing the quantization methods.

Question 3.2.17: For the evaluation of quantization aware/non-aware training, on top of the agreed cases of quantization non-aware training (Case 1) and quantization aware training (Case 2-1, Case 2-2), do you think it is needed to introduce an additional upper bound case of Case 0: non-quantized inference (i.e., float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training and inference), to reflect the performance loss due to quantization?

| Support/Can accept | MediaTek, NTT DOCOMO, Qualcomm (comment) , Huawei/HiSi, AT&T |
| --- | --- |
| Object/Concern | Vivo, OPPO |

| Company | View |
| --- | --- |
| MediaTek | We are okay with either measuring loss from the non-quantized upper bound or measuring gain over quantization-non-aware training. |
| ETRI | We think the non-quantization case can overestimates the performances and not suitable for the performance upper bound. |
| OPPO | Not necessary to evaluate the performance loss due to quantization. Because it is not possible to use an AI/ML model without quantization. |
| Qualcomm | For case 0, should non-quantized format be used only for inference? The following may be better: “Case 0: training and inference without quantization”. |
| Fujitsu | We share a similar view to that of OPPO. |
| Intel | We prefer to focus on the results with quantization. |

### 3.2-6: Evaluation on monitoring

- Issue#3-19 (Medium priority) Methodology of evaluation on monitoring

Moderator note: In this meeting, vivo [10], ZTE [5], and Qualcomm [31] raised the evaluation of the monitoring. But as the monitoring methodologies over the companies are still diverse and the proposals stay on the concept stage, it is still not clear to Moderator what can be considered as EVM/simulation cases.

E.g., vivo raised to consider to evaluate the relevance between eventual KPI and intermediate KPI, and test the performance fluctuation with environment changes; but it is Moderator’s understanding that the intermediate KPI/eventual KPI are anyway reported; and in addition such simulation cases can be reflected by the generalization Case 2 (as long as both intermediate KPI and throughput are both reported)?

As another example of the drift based monitoring, a framework needs to be determined firstly: 1) how to generate the datasets for training and drifting; 2) how to generate the distribution of the drift dataset; 3) how to quantize the bias of the drift dataset.

Therefore, an open question is raised.

Question 3.2.18: For the monitoring of the AI/ML based CSI compression, whether/how to construct the simulation cases? E.g.,

- For intermediate KPI/eventual KPI based monitoring, how to distinguish the methodology from the generalization Case 2 (change the scenario and trace the KPIs)?

- For drift based monitoring, at least the following issues may need to be elaborated:

  - How to generate the training dataset and the drift dataset? E.g., by modeling two datasets from different channel models, each of which includes eigenvectors.

  - How to generate the distributions of the drift dataset? E.g., reference sample(s) are selected from the training dataset, and the distributions of drift are generated by calculating the relevance of the drift dataset and the reference sample(s).

  - How to quantize the bias of the drift dataset? E.g., to set a threshold for the PDF of the drift dataset and therefore derive the probability of bias.

| Company | View |
| --- | --- |
| Moderator | This question does NOT necessarily need to be replied if you are not the proponent. Please proponents correct and elaborate the EVM for modeling monitoring. |
| MediaTek | Support. |
| vivo | Our understanding is that performance degradation/failure of a CSI compression model should be modelled, and then different monitoring methods can be evaluated on how well they can detect the performance degradation/failure. While generalization case2 (e.g., switching of scenario) can be considered for modelling performance degradation, more issues related to performance monitoring need to be discussed, such as how to define the accuracy of degradation/failure detection (e.g., correctly reporting a model switching). |

### 3.2-7: Generalization to the CSI compression use case

- Issue#3-20 (On hold) High-level observations on generalization cases

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### 3.2-8: Others

Question 3.2.19: For the evaluation of the AI/ML based CSI compression sub use cases, what other aspects related with evaluations do you think is necessary to be discussed?

| Company | View |
| --- | --- |

Specific evaluation methodology for CSI prediction sub use case

Summary of views from companies

### 4.1-1: Evaluation views/findings from companies

CSI prediction on time domain

- Views

  - OPPO: In R18 time domain CSI prediction, following two aspects should be studied to evaluate the performance gain and identify the potential spec impacts

    - Impact on throughput caused by scheduling delay and outdated CSI

    - Reduction of CSI-RS overhead

  - OPPO: For UE-sided CSI prediction, the following items should be considered

    - Ideal channel estimation for training stage and intermediate KPI calibration

    - Realistic channel estimation together with CSI feedback error for SLS performance evaluation

    - Possible CSI feedback enhancement for the output of CSI prediction (e.g. eType II based, AI based CSI feedback)

    - FFS: how to perform raw channel feedback in observation and prediction window

  - CEWiT: Time domain based prediction can be considered as  the data will be obtained for slot basis in real time scenario

  - Mavenir: For CSI prediction based on AI/ML model, we suggest that using at least 4 historical CSI samples to achieve a comparable performance as that of the nearest historical CSI

  - Mavenir: LSTM-CNN model is preferred for CSI prediction

  - NVIDIA: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Besides evaluating the CSI prediction at UE side, companies are encouraged to evaluate the CSI prediction at gNB side to understand the potential gains of performing CSI prediction at gNB side vs. UE side

- Findings on CSI prediction on time domain

  - Huawei, Hisilicon: For AI/ML-based CSI prediction

    - It outperforms nearest historical CSI with 2.5%-4% gain in terms of mean UPT and with 9%-17% gain in terms of 5% UPT.

    - With the same configuration of the observation window, it outperforms non-AI/ML prediction with 2%-3% gain in terms of mean UPT and with 6%-10% gain in terms of 5% UPT.

  - vivo: Without CSI prediction (scheme #2), using only AI/ML based CSI compression, there exist significant spectral efficiency loss due to the channel aging

  - vivo: The AI-based CSI prediction (scheme #3) can make up the spectral efficiency loss caused by channel aging

  - vivo: The AI-based CSI prediction (scheme #3) outperforms the non-AI based one (scheme #4).

  - vivo: The AI-based CSI prediction (#3) can predict CSIs of any future time, which has different performance.

    - Considering the spatial consistency, the AI-based CSI prediction still outperforms the non-AI approaches

    - AI-based CSI prediction can predict CSIs of any future time while the predicting time of AR-based method is limited to the periodic future time slot

  - Samsung: Gains in terms of SGCS are observed from AI/ML-based CSI prediction as compared to the sample-and-hold baseline over various UE speeds and prediction window (application instances)

    - The observed gains are different across UE speeds

    - The observed gains are different across different offset values between the CSI measurement and the predicted CSI application instance

  - ZTE: Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. Moreover, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied

  - ZTE: After eTypeII quantization on the predicated CSI, AI/ML based CSI prediction and Wiener filtering-based non-AI CSI prediction have similar SGCS performances, where both the methods completely outperform the CSI predication based on the nearest historical CSI

  - Nokia: Performance results for CSI prediction using the agreed evaluation methodology conditions continue to indicate promising performance for this use case

  - Nokia: CSI prediction using AI/ML has the potential for substantial performance gain at 5 and 10 ms prediction times compared with zero-order hold as measured by NMSE

  - Nokia: Preliminary system level simulation results with ideal channel estimation indicate that CSI prediction can provide throughput gains around 5%.

  - OPPO: Regarding AI based CSI prediction in single-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

    - AI achieves higher CSI prediction performance gain for UE with fast channel variation

    - AI achieves higher CSI prediction performance gain with father CSI prediction instance within the same CSI-RS feedback period

  - OPPO: Regarding AI based CSI prediction in multi-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

  - MediaTek: Depending on the requirements on CSI prediction, for example the required prediction length, AI/ML-based solutions provide superior performance compared to classical non-AI based methods

  - MediaTek: The AI/ML-based CSI prediction with Rel-18 codebook can effectively solve the CSI aging problem under specific scenarios

  - MediaTek: The AI/ML-based CSI prediction performance can approach the ideal prediction performance when the CSI-RS periodicity is within the coherence time.

  - CATT: AI/ML based CSI prediction outperforms the baseline using nearest historical CSI

  - Apple: LSTM based AI model achieves more than 10dB gain for CSI prediction use case

  - Apple: For sample and hold method, the SGCS performance is much better than NMSE since only Doppler effect is modeled in the channel

  - Xiaomi: AI based CSI prediction outperforms the benchmark scheme

  - CMCC: AI/ML based CSI prediction can achieve very high prediction accuracy compared with baseline non-prediction in terms of NMSE

  - CMCC: The performance of both baseline and AI based CSI prediction will decrease when UE moves faster

  - InterDigital: The SGCS performance of Layer 1 is higher than that of Layer 2 for all considered CSI prediction methods

  - InterDigital: The SGCS gain of LR over KF and S&H increases when the UE speed increases

  - InterDigital: For the same observation window and prediction window, the SGCS performance decreases when the UE speed increases

  - ETRI: AI/ML based CSI prediction improves performance compared to the baseline

  - Fujitsu: AI/ML-based eigenvector prediction outperforms the sample-and-hold method for temporal domain

  - Fujitsu: For AI/ML-based eigenvector predictions, the performance gain is decreased as the UE speed increases, compared to the sample-and-hold method

  - NVIDIA: Evaluation results show that AI/ML based CSI prediction significantly outperforms the baseline case without prediction (sample-and-hold)

  - CEWiT: For CSI prediction, the number of past observations are vital for prediction accuracy.  The result shows that with more number of past instances the prediction seems to perform better

  - CEWiT: For CSI prediction, the user mobility and  coherence time are important factors for AI/ML model’s prediction accuracy

  - CEWiT: For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds

  - CEWiT: When the coherence time is less than the CSI-RS periodicity, the CSI prediction performance will degrade rapidly

  - CEWiT: For AI/ML-based CSI prediction, an LSTM-based AI/ML model can be applied for training

  - Mavenir: Both LSTM and CNN-LSTM based CSI prediction can completely outperform the nearest historical CSI.

  - Mavenir: The prediction accuracy of both AI-based approaches improves with the rise of number of historical CSIs.

  - Mavenir: Both LSTM and CNN-LSTM based CSI prediction shows similar performance as nearest historical CSI when historical samples is 4.

  - Mavenir: CNN-LSTM-based CSI prediction can completely outperform LSTM-based CSI prediction.

### 4.1-2: AI/ML model settings

- Input/output CSI type

- Option 1: Raw channel matrix Huawei, Hisilicon, vivo, Nokia, ZTE, MediaTek, CMCC, NVIDIA Xiaomi InterDigital ETRI

  - Huawei, Hisilicon: The input/output type of CSI prediction is channel matrix, and SGCS is considered as the performance metric. SGCS is calculated based on the eigenvectors of the first layer decomposed from the predicted channel matrix, and the formula of SGCS calculation is same as the CSI compression sub use case… The frequency domain size of input/output channel matrix is 1RB in this simulation, after which the subband CSIs to be reported are calculated based on the predicted CSI per RB.

  - vivo: 15 raw historic channels in PRB

  - Nokia: Estimated downlink channel H, k time instance

  - MediaTek: each CSI instance is a complex-valued matrix with dimensions NRX×NTX×NF, where NRX and NTX are the numbers of RX and TX antennas, respectively, and NF is the number of elements in the frequency dimension, which could be on subcarrier or PRB level

  - ZTE: historical channel measurements (i.e., raw channel in time domain) are used as the input of AI models for training, validation and testing.

    - During the training process, to avoid high AI model complexity, the model input only includes 8 PRBs from a sub-band in frequency domain

  - CMCC: the CSI information is the full channel in one RB

- Option 2: Eigenvector OPPO Google ZTE CATT Fujitsu

  - ZTE: In order to evaluate the performance influenced by the diverse input types, we adopt the samples of historical sub-band eigenvectors processed from the raw channel matrices as input and the corresponding predicted eigenvectors as output

  - CATT: both the input and output are in the format of eigenvectors

  - OPPO: For eigenvector prediction, the input/output of AI/ML model would be the eigenvector W∈CNt×Nsb, where Nsb is the number of sub-band.

  - Google: The study of the input of CSI prediction should prioritize the input based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1

  - Fujitsu: the historical eigenvectors are used to predict the future eigenvectors at one or multiple slots

- Views on input/output types

  - ZTE: Further study the input and output types for the sub-use case of AI/ML based CSI prediction

  - ETRI: Further study the impact on different types of input on AI/ML based CSI prediction

  - Google: Study the following output of CSI prediction

    - Predicted RI/PMI based on Type1 codebook

    - Predicted CSI dwelling time

  - CEWiT: To  study the input and output types of the AI/ML based CSI prediction model

- Findings for comparison of input types

  - ZTE: The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input

  - ZTE: The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms

  - ETRI: AI/ML based CSI prediction as input of eigenvectors improves performance compared to the baseline. However, performance improvement is slightly reduced compared to the AI/ML model as input of RAW channel matrix

- Codebook type

  - ZTE: For non-AI/ML based CSI prediction and AI/ML based CSI prediction, Rel-16 codebook is used to report predicated CSIs in evaluations

- Observation window

- Descriptions on the Observation window

  - Huawei, Hisilicon: The observation window is the latest k=4 observation instances with 5 slots distance to each other

  - vivo: the prediction is with 15 historical CSIs as the input

  - Samsung: A CSI measured from K=3 CSI-RS resources are used for CSI prediction.

  - Nokia: The LSTM is trained to do a prediction of one time-step ahead after watching the last three time-steps

    - Time domain: We have a channel sample time of 0.5 ms and use either 20 or 30 time-domain input samples.

    - Frequency domain: Either 16 (1.44 MHz) or 100 (9 MHz) input frequency domain CSI reference signals

  - ZTE: We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions, respectively

    - we set diverse samples of historical CSIs as AI model inputs (3, 4, 6, 8 historical CSIs)

  - MediaTek: observation window is 10/15

  - CATT: In our design, we use the CSI in past three moments (i.e. t-2, t-1, t) to predict the CSI in one moment in the future (i.e. t+1)

  - Apple: Assuming CSI-RS periodicity of 5ms, the measurement window length 8

  - Xiaomi: In case 1, the input sample number is 8 and the prediction number is 1. In case 2, the input sample number is 4 and the prediction number is 1. And for case 3, the input sample number is 8 and the prediction number is 4, the 4 prediction samples are predicted one by one

  - OPPO: the input of CSI model includes K historic eigenvectors [Wt-5(K-1),…Wt-5,Wt] from K CSI-RS measurement

  - CMCC: we use 15 historic CSI samples to predict the 16th and 17th CSI in time domain

  - InterDigital:1~10ms

  - ETRI: we simulate 5/10 historical CSI information as the input of AI/ML model with CSI periodicity of 5ms/10ms/15ms

  - Fujitsu: The sampling interval of eigenvectors (v1, v2, v3, …) is 5ms, the slot number of AI/ML model input and output is 4 and 1, respectively.

  - NVIDIA: The raw channel matrices of the four latest CSI-RS measurement instances are used as the AI/ML model input. The raw channel matrices are associated with the first PRB

- Views on the Observation window

  - Nokia: Support high number of frequency-domain and of time-domain channel samples for training and inference of NNs for channel prediction

  - MediaTek: The tradeoff between the observation length and prediction length should be further studied

  - ZTE: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CI prediction and non-AI based CSI prediction

  - OPPO: Regarding the EVM on CSI prediction, evaluate the performance with different numbers of observation window K and prediction window T according to CSI-RS period

  - CEWiT: To study the trade-off between observation length and prediction length for different configurations

- Findings on the Observation window:

  - vivo: The observation window can be described by the number of historical CSIs and the spacing of the historical CSIs

  - vivo: The larger the number and the smaller the spacing of historical CSIs within the observation window, the better the prediction performance that can be achieved. However, this will in return increase the complexity and the storage (buffer) overhead of the model.

  - vivo: For different speeds, the requirement for the observation window is different

  - vivo: The prediction accuracy will decrease with the prediction length

  - Nokia: Channel prediction performance improves as the observation bandwidth increases and as the time step between measurements decreases.

  - Nokia: The complexity due to higher number of time domain and frequency domain channel samples increases only moderately

  - ZTE: The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increasing number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs

  - ZTE: AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increased number of historical CSIs in model input

  - Apple: LSTM based prediction achieve 20% SGCS performance gain at 5ms predict time, 23% at 7.5ms prediction time, and 10% at 10ms prediction time

  - Xiaomi: The prediction accuracy increased with larger observation window

- Prediction window

- Descriptions on the prediction window

  - Huawei, Hisilicon: The prediction window is 1 future slot

  - vivo: the future CSI at +4ms as output

  - Nokia: NMSE over the prediction time or, alternatively, cosine similarity in case of PMI prediction

  - ZTE: We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions (+5ms, +10ms, +15ms), respectively

  - OPPO: The output of CSI prediction model includes 4 future eigenvectors Wt+1,Wt+2,Wt+3,Wt+4 on the next 4 interval slots. Then the observation window is K and the prediction window is 4.

  - MediaTek: the length of the prediction window is 5/1/3;

  - Apple: Prediction window is 10ms, where we predict the channel 2.5ms, 5ms, 7.5ms and 10ms ahead of the last CSI-RS measurement

  - Samsung: The CSI prediction is made to two application instances. Application instance 1 and Application instance 2 which are 10ms and 25ms after the last CSI-RS resource used for measurement, respectively.

  - CATT: In our design, we use the CSI in past three moments (i.e. t-2, t-1, t) to predict the CSI in one moment in the future (i.e. t+1)

  - Xiaomi: In case 1, the input sample number is 8 and the prediction number is 1. In case 2, the input sample number is 4 and the prediction number is 1. And for case 3, the input sample number is 8 and the prediction number is 4, the 4 prediction samples are predicted one by one

  - CMCC: we use 15 historic CSI samples to predict the 16th and 17th CSI in time domain. The time interval between two CSIs is 5ms.

  - InterDigital: 1,…10ms

  - ETRI: to predict 3 future CSIs

  - Fujitsu: The sampling interval of eigenvectors (v1, v2, v3, …) is 5ms, the slot number of AI/ML model input and output is 4 and 1, respectively.

  - NVIDIA: The AI/ML model output is the predicted raw channel matrix at 4 ms ahead

- Views on the prediction window

  - ZTE: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CI prediction and non-AI based CSI prediction

  - MediaTek: The tradeoff between the observation length and prediction length should be further studied

- Company findings on the prediction window:

  - ZTE: The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction

  - Samsung: the SGCS gain is different over UE speeds and application instances. This suggests that the gNB can actively monitor the UE’s scenario, e.g., speed, and configure the number of CSI prediction instances and prediction window accordingly

  - Apple: Reduce the CSI-RS sample distance from 5ms to 2.5ms improve the SGCS by for LSTM based AI model. At 10ms prediction time, 17.5% SGCS improvement is observed over 5ms sample distance. Overall 30.6% SGCS improved at 10ms over sample and hold baseline

  - Xiaomi: The prediction accuracy decreased with larger prediction window

  - InterDigital: The SGCS performance decreases when the prediction time increases

- Other views/findings

AI/ML model architecture

- MediaTek: For AI/ML-based CSI prediction, both MIMO-based AI/ML model and SISO-based AI/ML model can be applied for training

- MediaTek: For AI/ML-based CSI prediction, both CNN-based AI/ML model and DNN-based AI/ML model can be applied for training

Complexity

- MediaTek: The number of FLOPs to perform the SISO-AI/ML-based CSI prediction is less than the MIMO-AI/ML-based CSI prediction

- MediaTek: The number of FLOPs to perform the DNN-based CSI prediction is less than the CNN-based CSI prediction

- MediaTek: Considering the performance results and the computation complexity, SISO-based DNN model is superior to other models

CSI-RS periodicity

- vivo: Using the preprocessing-based model scaling mechanism, the model trained at one speed can scales to other speeds

- MediaTek: The different CSI-RS periodicity should depend on the corresponding appropriate coherence time (UE speed and carrier frequency).

### 4.1-3: EVM related issues for CSI prediction

- KPI

- Intermediate KPI

  - OPPO: NMSE performance cannot completely match to the SGCS performance, especially for different UEs.

  - OPPO: Suggest to use multiple kinds of intermediate KPIs (e.g. SGCS, NMSE) for CSI prediction evaluation, conclusions should be drawn based on eventual KPI (e.g. SLS throughput) instead of intermediate KPIs

- Other KPIs

  - Execution latency Nokia

    - Nokia: An important metric which is not covered by the proposed reporting template is the execution latency

    - Nokia: One possible solution is to report a measure of the sequential calculation depth of the model, such as the number of layers in the model.

  - Processing complexity ZTE

    - ZTE: Further study how to reduce the model complexity for AI based CSI predication, e.g., reduce the input data size in frequency domain

- Other views on KPI

  - ZTE: Further evaluate various scenarios to justify that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced length of measurement window, reduced decorrelation distance, high UE speed, low LOS probability.

- Modeling for Spatial consistency

- vivo: The study on AI-based CSI prediction can rely on the channel with/without the spatial consistency. The channel assumptions with/without the spatial consistency should be reported by each proponent

- vivo: The spatial consistency procedure A with 50m decorrelation distance is used where the channel updating periodicity is assumed to be 1 ms.

- LG: For UE-sided AI/ML based CSI prediction, consider spatial consistency and companies can report one of spatial consistency procedures, Procedure A and/or B in TR 38.901

- Benchmark for evaluation results comparison

General views on benchamrk

- Huawei, Hisilicon: Confirm the working assumption on the benchmark of performance comparison for the CSI prediction sub use case

- Nokia: Identify the additional specification effects required by AI/ML-based CSI prediction beyond what is specified through the Rel-18 MIMO evolution work item

  - We support the adoption of the working assumption since it includes legacy CSI reporting as a baseline performance as well as the performance of conventional prediction algorithms, including Kalman filtering.  In addition, the working assumption takes into account the Rel-18 work currently in progress under the MIMO evolution work item

- InterDigital: To evaluate the AI/ML CSI prediction performance, a more realistic mobility model needs to be used

- AT&T: For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

  - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation-based AI/ML compatible with R18 MIMO as an example

  - It does not imply any restriction on future specification for CSI prediction

  - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Modeling of level x based CSI prediction in Benchmark#2

- Huawei, Hisilicon: For the CSI prediction sub use case, if collaboration level x is reported as the benchmark, the simulation methodology to evaluate the AI/ML-based CSI prediction enabled by LCM can emulate the fine-tuning case, while the simulation methodology to evaluate collaboration level x CSI prediction can emulate generalization Case 2.

- vivo: The comparison of level y/z AI-based CSI prediction and level x AI-based CSI prediction should be studied in the generalization aspects of AI-based CSI prediction.

  - Findings: For AI-based CSI prediction, with a level y/z collaboration, the speed-specific model can be switched according to the information associated with the UE change speed so as to guarantee the prediction performance for different speed. However, using a level x AI/ML model, it is hard to generalize well across different speeds.

- vivo: The performance impact of observation window and prediction window on the AI-based CSI prediction should be studied

  - Findings: The change of observation window and prediction window will impact the LCM of AI-based CSI prediction

- Template for simulation results collection

- Huawei, Hisilicon: For the AI/ML-based CSI prediction sub use case, the template of Table A.1 in the Appendix is considered for companies to report the evaluation results with model generalization/scalability.

Table A.1 Template for CSI prediction with model generalization

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … |  |  |  |
| Generalization Case 2 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … |  |  |  |
| Generalization Case 3 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … |  |  |  |
| Fine-tuning case (optional) | Train (setting, size/k) |  |  |
|  | Fine-tune (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … |  |  |  |
| FFS others |  |  |  |

- Other views

- ZTE: The progress of Rel-18 MIMO item should be tracked carefully

### 4.1-4: Generalization study

- Generalization methodology

- Huawei, Hisilicon: For the AI/ML-based CSI prediction sub use case, various UE speeds (e.g., 10km/h, 30km/h, 60km/h) can be added to the scenario list of generalization verification.

- Nokia: Consider specialized AI/ML models based on one or few generalized AI/ML models to achieve highest channel prediction performance with minimum number of AI/ML model versions

  - Nokia: With a specialized AI/ML model we refer to an AI/ML model which has been overfitted to the current radio channel evolution of the UE by using the channel estimates of the last 50 ms, 100 ms, or several hundreds of ms as a data set for fine tuning the AI/ML model.

Findings on fine-tuning of CSI prediction

  - Nokia: Additional fine tuning of the CSI prediction model has the potential to improve the performance of the CSI prediction for certain UEs

- AT&T: For the AI/ML based CSI prediction sub use case, to verify the generalization performance of an AI/ML model over various scenarios, the following CSI prediction specific scenarios are to be added to the set of scenarios for performing the generalization verification

  - Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, etc.)

  - Various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

- CEWiT: The solution to improve the generalization capability of AI model across different configurations/ scenarios could be further studied

- CEWiT: The generalization of AI/ML -based CSI prediction across various UE speeds should be studied

- Generalization over frequency PRBs

- Views

  - Huawei, Hisilicon: it seems the statistic channel characteristics over different PRBs would be similar, so the feature of the dataset would not make a big difference regardless of which PRB(s) are considered to construct the dataset. That is to say, there seems to be no strong motivation to verify the generalization performance from the PRB perspective.

  - vivo: Firstly, the AI model is trained using the data only collected from 1-st PRB. Then, the trained model is directly inferred on the 10-th, 20-th, 30-th, 40-th and 50-th PRB to evaluate the generalization performance

  - vivo: The generalization of AI-based CSI prediction over frequency granularities should be studied.

- Findings

  - vivo:  The generalization of AI-based CSI prediction with respect to PRBs is good.

- MediaTek: Further study the trade-off between single RB and joint RBs

  - For CSI prediction, the AI/ML model trained on a certain RB (or SB) can be generalized to other RBs (or SBs).

  - Compared with training at single RB, more complex models need to be considered when training at multiple RBs, otherwise the performance cannot be improved

  - The AI/ML model trained on joint RBs can be generalized and inferenced on other joint RBs

- NVIDIA: AI/ML training and inference are performed on the first PRB. Next, we use the AI/ML model trained on the first PRB to carry out inference/testing on different PRBs.

  - Findings: The AI/ML based CSI prediction model trained on a certain PRB can be generalized to perform inference on other PRBs

- Generalization over UE speeds

- Views/solution description

  - Huawei, Hisilicon: Generalization Case 1 assumes the same UE speed (30km/h or 60km/h) for both training and testing; generalization Case 2 assumes different UE speeds for both training and testing (30km/h for training and 60km/h for testing, or vice versa); generalization Case 3 assumes mixed dataset composed of 30km/h and 60km/h for training and either 30km/h or 60km/h is adopted for testing.

  - vivo: The generalization of AI-based CSI prediction across speeds should be studied, relying on, e.g., level y/z collaboration-based model switching and model scaling mechanisms.

  - vivo: The monitoring of the AI-based CSI prediction should be studied

  - MediaTek: UE speed is leveraged for evaluation with 10, 20, 30, 60, 120km/h

    - Case 1/2: Training at 30km/h, inference at 10/20/30/60/120km/h; Training at 120km/h, inference at 10/20/30/60/120km/h

    - Case 3: Training at mixed [10, 20, 30, 60, 120] km/h, inference at 10/20/30/60/120km/h

  - Samsung: For the evaluation of AI/ML based CSI prediction sub use case, the following evaluation cases are considered,

    - Case 1: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a dataset from the same UE speed A

    - Case 2: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

    - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple UE speeds including UE speed A and a different dataset than UE speed A, e.g., UE speed B, and then the AI/ML model performs inference/test on a dataset from a single UE speed from the multiple UE speeds, e.g., UE speed A, UE speed B.

      - Note: Companies to report the ratio for dataset mixing

    - Note: UE speed includes 10Kmphr, 20kmphr, 30kmphr, 60kmphr and 120kmphr

  - Apple: Case 3: the training dataset is a mixed dataset with both 30kmph speed and 60kmph speed UEs. The testing case is 30kmph data set and 60kmph data set separately.

  - CMCC: Case 3: we use a unified model trained with dataset containing samples with various speeds (10, 20, 30, 60 km/h), and during inference phase, this unified model is applied to separate test dataset with different UE speed

  - ETRI:

    - Case 1: Training at 30/20/10km/h, inference at the same UE speed of 30/20/10km/h.

    - Case 2: Training at 30/20/10km/h, inference at a different UE speed of 30/20/10km/h.

    - Case 3: Training at mixed dataset of 30/20/10km/h, and either 30/20/10km/h is adopted for testing.

  - ETRI: Further evaluate the AI/ML based CSI prediction over various UE speeds to overcome performance degradation in untrained UE speed

  - LG: Consider various UE speed for model generalization performance of CSI prediction

- Findings

  - Huawei, Hisilicon: For generalization of AI/ML-based CSI prediction over UE speeds of 30km/h and 60km/h,

    - AI/ML model trained by low UE speed has poor performance when tested on high UE speed.

    - AI/ML model trained by mixed dataset shows moderate performance when tested on each of the UE speeds.

  - vivo: The generalization of AI-based CSI prediction over speed is not good if the training set contains only one speed

  - vivo: The generalization of AI-based CSI prediction over speed can be improved using training set with mixed speed, whose prediction accuracy is still worse than that of speed-specific models

  - MediaTek: For CSI prediction, the Doppler effect and the coherence time are critical factors for AI/ML model’s prediction accuracy

  - MediaTek: When the coherence time is less than the CSI-RS periodicity, the CSI prediction performance will degrade rapidly

  - MediaTek: For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds

  - MediaTek: Using mixed datasets over UE speed for AI/ML model training is helpful to improve the generalization

  - Samsung: Severe performance degradation in terms of SGCS is observed when the AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

    - The degradation is sever when UE speed B < UE speed A.

  - Apple: LSTM based AI model performance has small performance degradation with mixed training dataset.  Model switching can be used to adapt to different mobility

  - CMCC: The unified AI model trained with mixed dataset achieve good generalization performance over different UE speeds for CSI prediction

  - ETRI: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds

  - ETRI: Mixed datasets of different UE speeds can mitigate performance degradation compared to datasets of single UE speed.

- Preliminary trends observed by more than one company

  - For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds. Huawei, Hisilicon, vivo, MediaTek, Samsung, ETRI

    - The generalization of AI-based CSI prediction over speed can be improved using training set with mixed speed. Huawei, Hisilicon, vivo, MediaTek, Samsung, Apple, CMCC, ETRI

- Generalization over carrier frequency

- MediaTek: Observe the performance changes under different carrier frequency values (including 2, 3 and 3.5GHz)

  - Case 1/2: Training at 3GHz, inference at 2GHz/3GHz/3.5GHz

  - Case 3: Training at mixed [2GHz, 3GHz, 3.5GHz], inference at 2GHz/3GHz/3.5GHz

Findings

  - For CSI prediction, the AI/ML model trained on a certain carrier frequency may not be generalized on other carrier frequencies

  - Using mixed datasets over Doppler frequency for AI/ML model training is helpful to improve the generalization

- Generalization over deployment scenarios

- vivo: The generalization and model switching/selection of AI-based CSI prediction over scenarios, e.g., LOS/NLOS, Uma/Umi, should be studied

- vivo: The generalization over the deployment scenarios, e.g., LOS/NLOS, Uma/Umi, is not good if the training set contains only one scenario

- MediaTek:

  - Case 1: Training at UMa, inference at UMa; Training at UMi, inference at UMi

  - Case 2: Training at UMa, inference at UMi; Training at UMi, inference at UMa

  - Case 3: Training at mixed [UMa, UMi], inference at UMa / UMi

Findings

  - The performance result of the UMi is better than the UMa, one of the reasons may be the multipath effect. For UMa development, because the NLOS ratio is higher than the UMi development, more multipath effects will be introduced in UMa channel, resulting in greater channel variation

  - For CSI prediction, the AI/ML model trained on a certain deployment (e.g., UMa/UMi) can be generalized and performed inference on other deployment (e.g., UMa/UMi)

1st round email discussions

### 4.2-1: EVMs for CSI prediction

For the CSI prediction, some EVMs are raised on top of the currently agreed generic EVM table.

- Issue#4-1 (High priority) Benchmark for performance comparison

Moderator note: In the last meeting, a WA was achieved for the benchmark of performance comparison for CSI prediction. From the inputs of this meeting, a large number of companies have submitted simulation results including the benchmarks following the guidance of the WA, and 3 companies propose to confirm the WA [3][14][28]. Therefore, the following proposal is given.

Proposal 4.2.1: Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

| Support/Can accept | vivo, CATT, Futurewei, ETRI, OPPO, Qualcomm, ZTE, LG Electronics, Huawei/HiSi, Spreadtrum, AT&T, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

- Issue#4-2 (Medium priority) Benchmark for performance comparison-modeling of level x

Moderator note: For the FFS part of the WA, two companies provide their views on how to model level x based CSI prediction and level y/z based CSI prediction in this meeting [3][10]. The ideas are similar, i.e., to reuse the EVM for the modeling of generalization, where level x based CSI prediction is modeled as generalization Case 2, i.e., the UE may not successfully trigger the model switching/updating when the environment changes, while level y/z based CSI prediction is modeled as generalization Case 1 (to emulate model switch)/fine-tuning (to emulate model updating). A proposal is given in below accordingly.

Proposal 4.2.2: For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

- E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2.

| Support/Can accept | ETRI, OPPO, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ETRI | In the generalization case such as variation of UE speed, collaboration level x based CSI prediction might show poor performance. Therefore, we agreed to evaluate generalized performance with other collaboration levels including information exchange between UE and NW. |

### 4.2-2: Generalization

- Issue#4-3 (High priority) CSI prediction specific generalization scenarios

Moderator note: From the inputs of companies, the generalization is performed over different environments, including different deployment scenarios, carrier frequencies, frequency PRBs, and UE speeds.

It is Moderator’s understanding that the agreement over scenarios in below apply to both CSI compression and CSI prediction, so we do not need to additionally discuss the generalization EVM for “Various deployment scenarios”, “Various outdoor/indoor UE distributions” and “Various carrier frequencies”.

Agreement For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification

In this meeting, 7 companies have provided the generalization results from the aspect of UE speeds, and 3 companies submitted the results from the aspect of PRBs. Therefore, it is suggested that we consider the various UE speeds into the generalization list for the study of CSI prediction.

Proposal 4.2.3: The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

| Support/Can accept | MediaTek, vivo, CATT, Futurewei, ETRI, OPPO, Qualcomm, LG Electronics, Huawei/HiSi, Spreadtrum, AT&T, Fujitsu |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | This should be added into the generalization scenario. After our studies, we observed that the UE speed has the biggest impact on model generalization due to the strong relation with the channel Doppler frequency. |
| CATT | UE speed is already in the agreement (in ‘Other aspects’). If the intention is to promote it as a sub-bullet, e.g. the same level as ‘Various deployment scenarios’, we are fine. |
| OPPO | We support. Generalization on different UE speeds can be evaluated without fine-tuning. |

### 4.2-3: Template for simulation results collection

- Issue#4-4 (Low priority) Template for the baseline table - latency

Moderator note: In this meeting, Nokia raised to reflect the execution latency to the template [14].

[Nokia] An important metric which is not covered by the proposed reporting template is the execution latency. In addition to the effect of the model architecture on latency, the execution latency is also affected by the processing power of the UE so that execution latency could be considered as a UE capability. The reporting of execution latency for a given model would then depend on the implementation platform assumed in the UE, if it is reported in time units. One possible solution is to report a measure of the sequential calculation depth of the model, such as the number of layers in the model. This measure is independent of time but can be used as an indicator of execution latency when coupled with an assumption on the UE processing capability.

It is Moderator’s understanding that a similar change with Issue#3-14e to update the “AL/ML model backbone” in the current template to “AL/ML model description (e.g., backbone, number of layers, etc.)”, so that companies are free to describe the details of the model.

Question 4.2.1: For the initial template of CSI prediction achieved in the working assumption in the RAN1#111 meeting, do you agree to make the following change?

| AI/ML model description | AL/ML model description ( e.g., backbone, number of layers, etc. |  |  |
| --- | --- | --- | --- |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |

| Company | View |
| --- | --- |
| MediaTek | Support |
| Huawei/HiSi | Support |
| AT&T | Support |

- Issue#4-4a (Low priority) Template for the baseline table – CSI payload size

Moderator note: In the template of CSI prediction achieved in the last meeting, the intermediate KPIs as well as eventual KPIs are captured without identifying/distinguishing the CSI feedback payload sizes. From some submitted simulation results of this meeting, some companies reported the eventual KPIs over different CSI feedback payload sizes. In Moderator’s understanding, there seems to be no need to separately report the intermediate KPIs per CSI payload sizes, as the predicted CSI for calculating SGCS/NMSE is the original CSI before quantization/feedback; on the other hand, report eventual KPIs (average UPT/5% UPT) may be meaningful to reveal more information on the performance of CSI prediction over payload sizes. A question is then raised in below.

Question 4.2.2: For the initial template of CSI prediction achieved in the working assumption in the RAN1#111 meeting, do you agree to make the following change to submit eventual KPIs per different CSI payload sizes?

- FFS the value or range of payload size X/Y/Z

| Gain for eventual KPI (Benchmark 1) | Mean UPT CSI feedback payload X |  |  |
| --- | --- | --- | --- |
|  | Mean UPT CSI feedback payload Y |  |  |
|  | Mean UPT CSI feedback payload Z |  |  |
|  | 5% UPT CSI feedback payload X |  |  |
|  | 5% UPT CSI feedback payload Y |  |  |
|  | 5% UPT CSI feedback payload Z |  |  |
| … |  |  |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT CSI feedback payload X |  |  |
|  | Mean UPT CSI feedback payload Y |  |  |
|  | Mean UPT CSI feedback payload Z |  |  |
|  | 5% UPT CSI feedback payload X |  |  |
|  | 5% UPT CSI feedback payload Y |  |  |
|  | 5% UPT CSI feedback payload Z |  |  |

| Support/Can accept | MediaTek |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | Support |
| Huawei/HiSi | Support |
| AT&T | Support |

- Issue#4-5 (High priority) Template for CSI prediction with generalization

Moderator note: The initial template for CSI prediction without generalization is achieved in the last meeting, while there is a “FFS the description and results for generalization/scalability may need a separate table”. It is Moderator’s thinking that it is neater to capture the generalization cases in a separate table, as there are different generalization cases to describe and submit results, which makes it hard to be merged into the basic template.

Proposal 4.2.4: The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification

- FFS whether to provide traffic types, UPT, overhead, and RU results

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI prediction with model generalization, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … (other scenarios/configurations for Case 1) |  |  |  |
| Generalization Case 2 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … (other scenarios/configurations for Case 2) |  |  |  |
| Generalization Case 3 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … (other scenarios/configurations for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting, size/k) |  |  |
|  | Fine-tune (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … |  |  |  |
| FFS others |  |  |  |

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### 4.2-4: Others

Question 4.2.3: For the evaluation of the AI/ML based CSI prediction sub use cases, what other aspects related with EVM do you think is necessary to be discussed?

| Company | View |
| --- | --- |
| Qualcomm | The scheme used to feedback the predicted CSI must be discussed. It must also captured in the results table template. |

Specific evaluation methodology for other sub use cases

1st round email discussions

Question 5.1.1: For the evaluation of the AI/ML based CSI feedback enhancement, is there any other sub use case that you think is necessary for EVM discussion and have not been discussed/captured in previous sections?

| Company | View |
| --- | --- |

Potential proposals for GTW/offline

Proposals for Mon. Offline

- Issue#2-1 (High priority) SGCS calculation for rank>1 – additional method

Proposal 6.1.1: For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method.

Proposal 6.1.1: For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, for the other two methods: - Method 1: Average over all layers - Method 2: Weighted average over all layers SGCS=E1Ni=1NjKλijk=1Kλik¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij. Option 3 is preferred: - Option 1: Company can report Method 2 as an additional metric. - Option 2: Company can optionally report either Method 1 or Method 2 as an additional metric. - Option 3: There is no consensus on whether to adopt an additional method. Option 1; Support/Can accept; CATT. Object/Concern. Option 2; Support/Can accept; Vivo, Futurewei, OPPO, NTT DOCOMO, LG Electronics. Object/Concern; Qualcomm. Option 3; Support/Can accept; MediaTek, vivo, IIT Kanpur, CATT, Futurewei, ETRI, ZTE, LG Electronics, Huawei/HiSi. Object/Concern.

- Issue#2-2 (High priority) Other intermediate KPIs-motivation

Proposal 2.2.2: Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

- Issue#2-2a (High priority) Other intermediate KPIs-options

Proposal 6.1.2: For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory.

- It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR)

Proposal 6.1.2: For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, consider option2 one of the following options: - Option 1: RAR is adopted as the additional intermediate KPI - Relative achievable rate (RAR) RARγ,L=EH1Ff=1FlogdetI+γPfHHfHHfPfEH1Ff=1FlogdetI+γPfHHfHHfPf where Hf~[NRx×NTx] is the complex channel matrix for RB f F is the total number of RBs, Pf~[NTx×L] is the reported precoding matrix for RB f, Pf~[NTx×L] is the optimal (SVD-based) precoding matrix for RB f, L is the number of MIMO layers the KPI is evaluated for, γ is the SNR-value; - the value of γ=1 is assumed for RAR calculation; Other values of γ are not precluded - Option 2: NO additional intermediate KPI is adopted as mandatory. - it is up to companies to optionally report other intermediate KPIs, e.g., Option 1. Option 1; Support/Can accept; Lenovo, Qualcomm (comment). Object/Concern. Option 2; Support/Can accept; MediaTek, vivo, CATT, Futurewei, OPPO, NTT DOCOMO, ZTE, LG Electronics, Huawei/HiSi. Object/Concern. Option 3; Support/Can accept. Object/Concern.

- Issue#2-3 (High priority) Provision of upper bound with ideal CSI

Proposal 6.1.3: For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison

Proposal 6.1.3: For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on ideal CSI without compression (e.g., ideal eigenvector from measured channel), which is taken as an upper bound for performance comparison Support/Can accept; MediaTek, Lenovo, vivo, IIT Kanpur, CATT, Futurewei, OPPO, NTT DOCOMO, Qualcomm (comment), ZTE, LG Electronics, Huawei/HiSi. Object/Concern.

- Issue#3-7 (High priority) AI/ML model settings for rank>1

Moderator note:

To QC: It is Moderator’s understanding that the 4 options are orthogonal. Please refer the rank common/rank specific examples in below, and refer to layer common and layer specific examples in CATT’s figure in below.

To DCM: For layer common rank specific, borrow CATT’s figure in below for demonstration.

To CATT: the intention here is the options are at least for inference; How to perform training can be reported by companies of course.

Proposal 3.2.2: For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

  - FFS: input/output type

- Option 3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is layer specific and rank common (For a specific layer, the same model is applied for all rank values), or layer specific and rank specific (For a specific layer, different models are applied for different rank values)

- Option 4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or layer common and rank specific (different models are applied for different rank values and the same model applied over all layers)

- Other options not precluded.

| Support/Can accept | MediaTek, vivo (support Option-2 and 4), CATT, Futurewei, ETRI, OPPO, NTT DOCOMO, Qualcomm (comment), LG Electronics, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

- Issue#3-9 (High priority) Payload size alignment

Proposal 3.2.4: For the CSI payload size calculation, between the two options:

- Option 2a: The above-mentioned “CSI payload” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank

- Alt.1: Option 2b is adopted for both AI/ML based solution and the legacy Type II CB

- Alt.2: Option 2b is adopted for AI/ML based solution, while whether Option 2a or Option 2b is adopted is reported by companies if partial NZC report is assumed for the legacy Type II CB.

- Alt.3: Other

| Alt.1 | Support/Can accept | MediaTek, vivo, CATT, Futurewei, OPPO, NTT DOCOMO, Qualcomm, ZTE, LG Electronics, Huawei/HiSi, Spreadtrum, AT&T, Fujitsu, Intel |
| --- | --- | --- |
|  | Object/Concern |  |
| Alt.2 | Support/Can accept | Huawei/HiSi |
|  | Object/Concern | Qualcomm |
| Alt.3 | Support/Can accept |  |
|  | Object/Concern |  |

- Issue#3-13 (High priority) Template for the baseline table – training types/cases

Moderator note: It is Moderator’s thinking that maybe we can avoid using “Type 1”/“Type 2” from the simulation perspective to distinguish the training cases and templates, but use the following terms to separate their templates:

- “1-on-1 joint training”, which means the CSI generation part and the CSI reconstruction part can be regarded to be jointly designed/developed/optimized as if by a single side) to represent the upper bound of the performance. For training Type 1/2 with 1 NW and 1 UE, it is subject to this case. Whether the model pairing is unrestricted/matched or restricted/unmatched, it can refer to Issue#3-4.

- “Multi-vendor joint training”, which means 1 NW to M>1 UEs, N>1 NWs to 1 UE, or N>1 NWs to M>1 UEs are jointly trained which may introduce additional loss due to the interaction with multi-vendors. Type 2 with multi-vendor training are subject to this case, while whether Type 1 also faces the multi-vendor case is FFS.

- “Separate training”. This is Type 3. The dataset sharing will lead to additional loss as compared to Type 1/2 joint training for the same pair of models.

To OPPO: note the achieved template in #111 WA is entitled with “1-on-1 joint training”. For separate training, regardless it is 1-on-1 or 1 to multiple, it is supposed to be potential loss compared to 1-on-1 joint training. Your understanding is correct, i.e., this table is used to capture all the cases (Case 1/2/3, both NW-first and UE-first) for separate training.

Proposed conclusion 6.1.2: For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

  - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate temple is introduced to capture the evaluation results for “separate training Type 3”

| Support/Can accept | MediaTek, vivo, IIT Kanpur, CATT, ETRI, OPPO, NTT DOCOMO, Qualcomm (comment), ZTE, LG Electronics, Huawei/HiSi |
| --- | --- |
| Object/Concern | Lenovo |

- Issue#3-17 (High priority) Template for CSI compression-Table for Generalization/scalability

Moderator note:

To Futurewei:

1) whether to capture UPT results pleases see the 1st FFS.

2) handling of scalability methods captured in “Scalability method description if applicable, e.g., truncation, adaptation layer, etc.” already.

3) Do we really need to evaluate the benchmark results over scenarios/configurations? They are supposed to have been evaluated at legacy releases.

To Lenovo: the plan is to start collecting the initial results at the next meeting, and as companies are more or less in-line with the observations for generalization results, it is Moderator’s hoping that we determine the template for generalization this meeting, and make observations next meeting.

Proposal 3.2.8: A separate table to capture the evaluation results of generalization/scalability verification is needed as given in the following initial template

- FFS whether to provide traffic types, UPT, overhead, and RU results

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression of training Type Y with model generalization/scalability, [traffic type], [Max rank value], [RU]

Table A.4 Template for CSI compression with model generalization

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| Intermediate KPI #1, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| Intermediate KPI #1, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other scenarios/configurations for Case 1) |  |  |  |
| Intermediate KPI #2, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| Intermediate KPI #2, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … |  |  |  |
| Generalization Case 2 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| … (other scenarios/configurations for Case 2) |  |  |  |
| Generalization Case 3 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| … (other scenarios/configurations for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting, size/k) |  |  |
|  | Fine-tune (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
| … |  |  |  |
| FFS others |  |  |  |

- Issue#4-1 (High priority) Benchmark for performance comparison

Proposal 4.2.1: Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

| Support/Can accept | vivo, CATT, Futurewei, ETRI, OPPO, Qualcomm, ZTE, LG Electronics, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

- Issue#4-3 (High priority) CSI prediction specific generalization scenarios

Moderator note: To CATT: yes, the intention is to promote the generalization verification from “others are not precluded” to the subbullet as a typical case.

Agreement For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification

Proposal 4.2.3: The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

| Support/Can accept | MediaTek, vivo, CATT, Futurewei, ETRI, OPPO, Qualcomm, LG Electronics, Huawei/HiSi |
| --- | --- |
| Object/Concern |  |

- Issue#4-5 (High priority) Template for CSI prediction with generalization

Moderator note: Sorry for missing the table for collecting companies’ view in the 1st round. Please provide your comments in the offline/online.

Proposal 4.2.4: The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification

- FFS whether to provide traffic types, UPT, overhead, and RU results

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI prediction with model generalization, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … (other scenarios/configurations for Case 1) |  |  |  |
| Generalization Case 2 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … (other scenarios/configurations for Case 2) |  |  |  |
| Generalization Case 3 (scenario/configuration, etc.) | Train (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … (other scenarios/configurations for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting, size/k) |  |  |
|  | Fine-tune (setting, size/k) |  |  |
|  | Test (setting, size/k) |  |  |
|  | Intermediate KPI #1 |  |  |
|  | Intermediate KPI #2 |  |  |
| … |  |  |  |
| FFS others |  |  |  |

- References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6-17, 2021.

- R1-2300044	Discussion and evaluation of AI/ML for CSI feedback enhancement FUTUREWEI

- R1-2300108	Evaluation on AI/ML for CSI feedback enhancement  Huawei, HiSilicon

- R1-2300154	Evaluations of AI-CSI	Ericsson

- R1-2300171	Evaluation on AI for CSI feedback enhancement	ZTE

- R1-2300211  Discussion on evaluation on AIML for CSI feedback enhancement  Spreadtrum Communications, BUPT

- R1-2300280	Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement  OPPO

- R1-2300348 	Evaluation on AI ML for CSI feedback enhancement	Mavenir

- R1-2300397  On Evaluation of AI/ML based CSI  Google

- R1-2300444	Evaluation on AI/ML for CSI feedback enhancement  vivo

- R1-2300501  Evaluation of AI/ML based methods for CSI feedback enhancement  Fraunhofer IIS, Fraunhofer HHI

- R1-2300530  Evaluation on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2300567	Discussion on evaluation on AI/ML for CSI feedback enhancement  xiaomi

- R1-2300604  Evaluation of ML for CSI feedback enhancement  Nokia, Nokia Shanghai Bell

- R1-2300671	Evaluation on AI/ML for CSI feedback enhancement  CATT

- R1-2300716	Evaluation on AI/ML for CSI feedback enhancement  China Telecom

- R1-2300744  Evaluation on AI/ML for CSI feedback enhancement  Fujitsu

- R1-2300841  Some discussions on evaluation on AI-ML for CSI feedback	 CAICT

- R1-2300941  Evaluation for CSI feedback enhancements  Intel Corporation

- R1-2300990  Discussion on evaluation on AI/ML for CSI feedback enhancement  CMCC

- R1-2301031  Evaluation on AI/ML for CSI feedback enhancement  Indian Institute of Tech (H)

- R1-2301041  Evaluation on AI/ML for CSI feedback enhancement  ETRI

- R1-2301097  Evaluation of joint CSI estimation and compression with AI/ML  BJTU

- R1-2301146  Evaluation on AI/ML for CSI Feedback Enhancement  IIT Kanpur

- R1-2301156	Evaluation on AI/ML for CSI feedback enhancement  InterDigital, Inc.

- R1-2301178  Evaluation of AI and ML for CSI feedback enhancement  NVIDIA

- R1-2301199  Evaluation on AI/ML for CSI feedback  Lenovo

- R1-2301223	Discussion on AI/ML for CSI feedback enhancement	AT&T

- R1-2301255	Evaluation on AI ML for CSI feedback enhancement	Samsung

- R1-2301337  Evaluation for AI/ML based CSI feedback enhancement  Apple

- R1-2301404  Evaluation on AI/ML for CSI feedback enhancement  Qualcomm Incorporated

- R1-2301466  Evaluation of AI/ML based methods for CSI feedback enhancement  SEU

- R1-2301485  Discussion on evaluation on AI/ML for CSI feedback enhancement  NTT DOCOMO, INC.

- R1-2301587  Evaluation on AI/ML for CSI feedback enhancement  MediaTek Inc.

- R1-2301688  Evaluation of AI and ML for CSI feedback enhancement  CEWiT

- Appendix I: Agreement list

### Agreements of the 109-e meeting

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement (modified by May 23rd post)

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

Note: Above agreement is updated as follows

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

  - Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

### Agreements of the 110 meeting

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

  - Note: Companies to report the ratio for dataset mixing

  - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted.

Agreement

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point:

- Various deployment scenarios (e.g., UMa, UMi, InH)

- Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

- Companies to report the selected scenarios for generalization verification

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

  - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

  - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

### Agreements of the 110bis-e meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | At least, FTP model 1 with packet size 0.5 Mbytes is assumed Other options are not precluded. |
| --- | --- |

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy,

- Use the target CSI from ideal channel and use output CSI from the realistic channel estimation

  - The target CSI from ideal channel equally applies to AI/ML based CSI feedback enhancement, and the baseline codebook

Note: there is no restriction on model training

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook (if it outperforms Type II Codebook) can be optionally considered for comparing AI/ML schemes up to companies

- Note: Type II Codebook is baseline as agreed

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, and if the AI/ML model outputs multiple predicted instances, the intermediate KPI is calculated for each prediction instance

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations:

- Raw channel matrixes

- Eigenvector(s)

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the evaluation of CSI prediction:

- Companies are encouraged to report the assumptions on the observation window, including number/time distance of historic CSI/channel measurements as the input of the AI/ML model, and

- Companies to report the assumptions on the prediction window, including number/time distance of predicted CSI/channel as the output of the AI/ML model

Conclusion

If ideal DL channel estimation is considered (which is optional) for the evaluations of CSI feedback enhancement, there is no consensus on how to use the ideal channel estimation for dataset construction, or performance evaluation/inference.

- It is up to companies to report whether/how ideal channel is used in the dataset construction as well as performance evaluation/inference.

Conclusion

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example:

- For each FP/BP loop,

  - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side

  - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side

  - Step 3: UE side trains the CSI generation part based on the BP information from NW side

- Note: the dataset between UE side and NW side is aligned.

- Other Type 2 training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training):

- Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly

- Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part

- Step3: UE side trains the UE side CSI generation part based on the received set of information

- Other Type 3 NW-first training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training):

- Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly

- Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part

- Step3: NW side trains the NW side CSI reconstruction part based on the received set of information

- Other Type 3 UE-first training approaches are not precluded and reported by companies

Working assumption

In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors

- Note: Eventual KPI can still be used to compare the performance

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2.

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Agreement

In CSI compression using two-sided model use case, evaluate and study quantization of CSI feedback, including at least the following aspects:

- Quantization non-aware training

- Quantization-aware training

- Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc.

- How to use the quantization methods

Agreement

For evaluating the performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, e.g., including at least the following options

- High resolution scalar quantization, e.g., Float32, Float16, etc.

    - FFS select one of the scalar quantization resolutions as baseline

- High resolution codebook quantization, e.g., R16 Type II-like method with new parameters

    - FFS new parameters

- Other quantization methods are not precluded

Agreement

For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement which is optionally considered by companies, the following case is taken

- The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations:

- Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side

- Case 2: Not aligned AI/ML model structures between NW side and UE side

    - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers)

- FFS different sizes of datasets between NW side and UE side

- FFS aligned/different quantization/dequantization methods between NW side and UE side

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

Agreement

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors,

- Case 1 (baseline): Type 2 training between one NW part model to one UE part model

- Case 2: Type 2 training between one NW part model and M>1 separate UE part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at UE part models, e.g., whether the same or different dataset(s) are used among M UE part models

- Case 3: Type 2 training between one UE part model and N>1 separate NW part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at NW part models, e.g., whether the same or different dataset(s) are used among N NW part models

- FFS N NW part models to M UE part models

- FFS different quantization/dequantization methods between NW and UE

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

- FFS: companies to report the training order of UE-NW pair(s) in case of M UE part models and/or N NW part models

- FFS: whether/how to report overhead

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations

- Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit

    - Companies to report the raw channel is in frequency domain or delay domain

- Precoding matrix

    - Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation)

- Other input/output types are not precluded

- Companies to report the combination of input (for CSI generation part) and output (for CSI reconstruction part),

    - Note: the input and output may be of different types

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for SLS, spatial consistency procedure A with 50m decorrelation distance from 38.901 is used (if not used, company should state this in their simulation assumptions)

- UE velocity vector is assumed as fixed over time in Procedure A modeling

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for the calculation of intermediate KPI, the following is considered as the granularity of the frequency unit for averaging operation

- For 15kHz SCS: For 10MHz bandwidth: 4 RBs; for 20MHz bandwidth: 8 RBs

- For 30kHz SCS: For 10MHz bandwidth: 2 RBs; for 20MHz bandwidth: 4 RBs

- Note: Other frequency unit granularity is not precluded and reported by companies

### Agreements of the 111 meeting

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS the value or range of payload size X/Y/Z

- FFS the description and results for different training types/cases may need a separate table

- FFS: training related overhead

Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#1, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#2, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#2, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Gain for Mean UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for 5% UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors

- Case 1 (baseline): Type 3 training between one NW part model and one UE part model

  - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model

  - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model

  - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different

  - FFS: different quantization methods between NW side and UE side

- Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models

  - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models

  - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model

  - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models

- Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models

  - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models

  - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models

  - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models

- FFS: whether/how to report overhead of dataset

Working Assumption

For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example

  - It does not imply any restriction on future specification for CSI prediction

- FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1.

- Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn.

- Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1.

- Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn.

- Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization.

- Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison.

Agreement

For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies:

- Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training

  - Fixed/pre-configured quantization method/parameters is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

- Case 2: Quantization aware training, where quantization/dequantization is involved in the training process

  - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

  - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase

    - Companies to report how to update the quantization method/parameters during the training

- Note: the above cases apply for training Type 1/2/3

- Others are not precluded.

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2

- For NW-first training

  - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization.

- For UE-first training

  - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

Working Assumption

For the AI/ML based CSI prediction sub use case, the following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction for the case without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS whether/how to capture the muliptle predicted CSI instances and their mapping to slots

Table X. Evaluation results for CSI prediction without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | UE speed |  |  |
|  | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Benchmark 1 |  |  |  |
| Intermediate KPI #1 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 1 |  |  |  |
| Intermediate KPI #2 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 1 |  |  |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| Benchmark 2 |  |  |  |
| Intermediate KPI #1 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 2 |  |  |  |
| Intermediate KPI #2 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 2 |  |  |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| FFS others |  |  |  |

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated

- Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions.

- Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions

- Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions

- Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, if R16 Type II-like method is considered, companies to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc.