3GPP TSG RAN WG1 #112							    R1-2302015

Athens, Greece, February 27th – March 3rd, 2023

Source:	Moderator (MediaTek)

Title:	[112-R15-NR] Summary of comments on R1-2301597 about SUL indicator and configuration

Agenda item:	7.1

Document for:	Discussion and Decision

Introduction

In RAN1#112 meeting, two contributions [1, 2, MTK], one discussion paper and one CR, are submitted to clarify the relation between SUL indicator and pusch-Config/pucch-Config for DCI 0_0.

As guided by the Chairman, this contribution provides summary of the submitted contributions (Section 4), discussion points (Section 2), and possible RAN1 consensus during this meeting (Section 3).

R1-2301597	On the relation between SUL indicator and pusch-Config/pucch-Config for DCI 0_0	MediaTek Inc.

R1-2301598	[R17] Draft 38.212 CR on SUL indicator and pusch-Config/pucch-Config for DCI 0_0	MediaTek Inc.

To be moderated by James (MediaTek)

Discussion points (phase 1 until 11-Oct)

In [1, MTK], it is mentioned that for DCI 0_1 in 38.212 [3] 7.3.1.1.2, PUSCH should first be transmitted on the UL carrier configured with pusch-Config if only one of the UL or SUL carrier in the cell is configured for PUSCH transmission.

However, for DCI 0_0 in 38.212 [3] 7.3.1.1.1, it seems the UL or SUL carrier with pucch-Config would be prioritized for PUSCH transmission if only one of the UL or SUL carrier in the cell is configured for PUSCH transmission, and this results in the PUSCH transmission behavior on NUL (normal uplink) or SUL shown in Table 1 below.

Table 1: PUSCH transmission (scheduled by DCI 0_0) on NUL (normal uplink) or SUL scheduled by DCI 0_0 implied in current spec

The detailed spec quote from 38.213 for the descriptions above can be found in Section 4.

In [1, 2, MTK], there is a R17 CR proposed to align the prioritization rule in DCI 0_0 with 0_1 (prioritize carrier configured with PUSCH). The proposed R17 CR is copied in the end of Section 4 in this document.

The following discussions points are devised to clarify the relation between SUL indicator and pusch-Config/pucch-Config for DCI 0_0.

Discussion point 1:

For PUSCH scheduled by DCI 0_0, if pusch-Config is only configured on NUL (normal uplink) and pucch-Config is only configured on SUL, which of NUL or SUL should the scheduled PUSCH be transmitted on?

Please assist to elaborate on the reasoning of your answer if possible.

If you have other answers or different views, please also elaborate.

| Company | NUL or SUL | Comment |
| --- | --- | --- |
| MTK | NUL | Our reason is, for PUSCH transmission, the carrier configured with pusch-Config should be prioritized for transmission, just like the case for DCI 0_1. |
| Qualcomm | Not supported | According to the RAN1#90bis agreement below, unless UE specific RRC signalling configures that PUSCH may be dynamically scheduled on the other (i.e. non-PUCCH) carrier in the same cell as the SUL, the PUSCH location has to be the carrier that PUCCH is located. Agreement: - UE specific RRC signalling (re-)configures the location of the PUCCH, either on the SUL carrier or on a non-SUL UL carrier in a SUL band combination - The default location of the PUSCH is the same carrier as used by PUCCH - UE specific RRC signalling may (de-)configure that PUSCH may be dynamically scheduled on the other (i.e. non-PUCCH) carrier in the same cell as the SUL - In this case, a carrier indicator field in the UL grant is used to indicate dynamically whether the PUSCH is transmitted on the PUCCH carrier or on the other carrier - Note: Simultaneous PUSCH transmission on the SUL carrier and non-SUL UL carrier is not supported according to existing RAN2 agreement - FFS in DCI discussion whether the SUL CIF is always present - There is one active BWP on the SUL carrier and one active BWP on the non-SUL UL carrier - SRS related RRC parameters are independently configured for SRS on the SUL carrier and SRS on the non-SUL UL carrier in the SUL band combination - SRS can be configured on the SUL carrier and non-SUL UL carrier, irrespective of the carrier configuration for PUSCH and PUCCH So, the rows highlighted in red are not supported. |
| Nokia, NSB | Not supported | This is new functionality, not a correction. |
| ZTE | Not supported | We don’t think there is any issue for this SUL field. Based on our understanding, the reason why different behaviors are introduced is because facllback DCI is used when UE is in poor coverage. Then in this case, PUSCH is better to be scheduled in the PUCCH carrier since usually PUCCH carrier is with better coverage. Non-fallback DCI is used when UE is in good coverage. In this case, PUSCH can be scheduled in the carrier configured with PUSCH-configure to boost throughput. |
| Spreadtrum | Not support | We agree with other companies that these configuration are not supported. |
| vivo |  | We think it should be fine to keep the current specification without change. |
| Apple | Not needed | We agree that this is not a correction rather an enhancement. There is no issue in the current spec. |
| Samsung |  | The cases where pusch-Config and pucch-Config are not on the same carrier can be avoided by gNB. Therefore, we don’t think this spec correction is essential. |

Discussion point 2:

For PUSCH scheduled by DCI 0_0, if pusch-Config is only configured on SUL and pucch-Config is only configured on NUL, which of NUL or SUL should the scheduled PUSCH be transmitted on?

Please assist to elaborate on the reasoning of your answer if possible.

If you have other answers or different views, please also elaborate.

| Company | NUL or SUL | Comment |
| --- | --- | --- |
| MTK | SUL | Our reason is, for PUSCH transmission, the carrier configured with pusch-Config should be prioritized for transmission, just like the case for DCI 0_1. |
| Qualcomm | Not supported | Please see our comment on Discussion point 1. |
| Nokia, NSB | Not supported | This case might exist with dynamic DCI-based indication, but the combination of DCI-based SUL/NUL indication with PUSCH-config only on NUL is pointless. With no SUL/NUL indication on the DCI the configuration should not be supported to begin with (see Qualcomm comment on #1). |
| ZTE | Not supported | Similar view as Nokia above. |
| Spreadtrum | Not support | We agree with other companies that these configuration are not supported. |
| vivo |  | We think it should be fine to keep the current specification without change. |
| Apple | Not Needed | Same reply as for Discussion point 1 |
| Samsung |  | Please refer to our comment on Discussion point 1. |

Discussion point 3:

For the proposed R17 CR to 38.212 [3] 7.3.1.1.1 (copied in the end of Section 4 in this document), do you support it? Revised text on the proposed CR is welcomed.

If you do not support, please elaborate your reason if possible.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | The proposed R17 CR is aiming to align the prioritization rule in DCI 0_0 with 0_1 (prioritize carrier configured with PUSCH) for SUL indicator. There was an agreement from RAN1 90-bis saying that: Agreement: - UE specific RRC signalling (re-)configures the location of the PUCCH, either on the SUL carrier or on a non-SUL UL carrier in a SUL band combination - The default location of the PUSCH is the same carrier as used by PUCCH To us, the agreement above does not intend to introduce different prioritization rules for DCI 0_0 and DCI 0_1 regarding PUCCH/PUSCH configuration. Hence, we think the proposed R17 CR can make current spec on SUL more complete and clear. |
| Qualcomm | Not support | We need to look at the overall RAN1#90bis agreement (not only the 1st bullet) to understand the intention. The intention was, unless the UE is configured with an indication of ULN or SUL for PUSCH transmission, PUSCH transmission takes place on the carrier where PUCCH is located. |
| Nokia, NSB | Not support | This is new functionality, at best could be seen as a TEI18 proposal, but it is not a Rel-15 correction (no matter from which spec release) |
| ZTE | Not support | Similar view as Qualcomm above. |
| Spreadtrum | Not support | Share same view as Qualcomm. |
| vivo |  | We think it should be fine to keep the current specification without change. |
| Apple | Not needed | Agree with QC |
| Samsung | Not support | We think this issue could be valid. However, such problematic cases can be avoided by gNB’s configuration; pusch-Config and pucch-Config are always on the same carrier. Therefore, we don't think this CR is vital and don’t support it. |

Resulted RAN1 conclusion/agreement (phase 2)

It seems hard to achieve RAN1 CR/conclusion in this meeting.

Moderator’s summary for each discussion point is provided below:

Discussion point 1 & 2 & 3:

- 7 out of 8 companies (QC, Nokia, ZTE, Spreadtrum, vivo, Apple, Samsung) prefer not to have the R17 CR proposed from [1, MTK]

  - QC quoted one agreement from RAN1 #90bis to argue:

    - Unless UE specific RRC signalling configures that PUSCH may be dynamically scheduled on the other (i.e. non-PUCCH) carrier in the same cell as the SUL, the PUSCH location has to be the carrier that PUCCH is located.

    - The mentioned issue of different PUCCH/PUSCH carrier for NUL/SUL is not supported

  - Nokia/Spreadtrum agree with QC

  - ZTE/vivo/Apple deem there is no issue in current spec (prioritizing PUCCH carrier for PUSCH transmission is acceptable)

  - Samsung thinks this issue could be valid, but such problematic cases can be avoided by gNB’s implementation, say configuring pusch-Config and pucch-Config on the same carrier

- Moderator comment: The proposed R17 CR from [1, MTK] is only supported by MTK and not preferred by other 7 companies. However, companies seem to have diverse understanding on the mentioned scenario (different PUCCH/PUSCH carrier for NUL/SUL):

  - 3 companies (QC, Nokia, Spreadtrum) think the mentioned scenario is not supported

  - 3 companies (ZTE, vivo, Apple) deem there is no issue in current spec (prioritizing PUCCH carrier for PUSCH transmission is acceptable)

  - 1 company (Samsung) thinks the issue is valid but can be avoided by gNB’s implementation

Another issue is that the scenario of only one carrier configured with pusch-Config but no carriers configured with pucch-Config is also not specified properly in current spec. It is possible that only cell-specific pucch-ConfigCommon is configured to UE.

Summary of contribution inputs

Summary for [1, 2, MTK]:

In [1, MTK], it is mentioned that for DCI 0_1 in 38.212 [3] 7.3.1.1.2, PUSCH should first be transmitted on the UL carrier configured with pusch-Config if only one of the UL or SUL carrier in the cell is configured for PUSCH transmission:

while for DCI 0_0 in 38.212 [3] 7.3.1.1.1, it seems the UL or SUL carrier with pucch-Config would be prioritized for PUSCH transmission if only one of the UL or SUL carrier in the cell is configured for PUSCH transmission:

and this results in the PUSCH transmission behavior on NUL (normal uplink) or SUL shown in Table 1 below.

Table 1: PUSCH transmission (scheduled by DCI 0_0) on NUL (normal uplink) or SUL scheduled by DCI 0_0 implied in current spec

Hence, in [1, 2, MTK], there is a CR proposed to align the prioritization rule in DCI 0_0 with 0_1 (prioritize carrier configured with PUSCH) as below:

Proposal 1: Adopt the following R17 CR to 38.212 [3] 7.3.1.1.1 about SUL indicator and pusch-Config/pucch-Config for DCI 0_0, to align the PUSCH transmission behavior on NUL/SUL with DCI 0_1:

7.3.1.1      DCI formats for scheduling of PUSCH

7.3.1.1.1      Format 0_0

…

-	UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and not configured on SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured;

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is configured on only one carrier, the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pusch-Config is configured;

…

- References

[1] R1-2301597, “On the relation between SUL indicator and pusch-Config/pucch-Config for DCI 0_0”, MediaTek, RAN1 #112

[2] R1-2301598, “[R17] Draft 38.212 CR on SUL indicator and pusch-Config/pucch-Config for DCI 0_0”, MediaTek, RAN1 #112

[3] 3GPP TS 38.213 V17.4.0, “NR; Multiplexing and channel coding”