3GPP TSG RAN WG1 #112		                                                      R1-2302121

Athens, Greece, February 27th – March 3rd, 2023

Source:	Moderator (CATT)

Title:	Summary of correction on SRS frequency hopping procedure

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

This document summarizes the discussions during RAN1#112 on the following CR.

R1-2300616	Correction on SRS frequency hopping procedure in Rel-15	CATT

To be moderated by Min (CATT)

- Discussion

  - 1st round discussion

In TS 38.214, Ns is used to denote the number of OFDM symbols of an SRS resource. In clause 6.2.1.1 in TS 38.214 for SRS frequency hopping procedure, “the N-symbol SRS resource” is used. There is no explanation or definition on symbol N for an SRS resource or N-symbol SRS resource in TS 38.214. It is unclear which SRS resource “the N-symbol SRS resource” is. Therefore, the CR of R1-2300616 is provided as following:

6.2.1.1 UE SRS frequency hopping procedure *** Unchanged text is omitted *** A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location(s) in each slot. For Ns=4, when frequency hopping is configured with R=2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across two pairs of R adjacent OFDM symbol(s) of the resource in each slot. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot. *** Unchanged text is omitted ***

Question 1: Do you agree with the following proposal?

Proposal:

Endorse the following text proposal in the alignment CRs for Rel-16/17 TS 38.214.

|  | Company |
| --- | --- |
| Agree | Samsung, |
| Not agree |  |

| Company | Comments |
| --- | --- |
| Samsung | We think shadow CRs are needed for Rel-16 and Rel-17. |

- Conclusion

Proposal:

Endorse the following text proposal in the alignment CRs for Rel-16/17 TS 38.214.

6.2.1.1 UE SRS frequency hopping procedure *** Unchanged text is omitted *** A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location(s) in each slot. For Ns=4, when frequency hopping is configured with R=2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across two pairs of R adjacent OFDM symbol(s) of the resource in each slot. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot. *** Unchanged text is omitted ***