3GPP TSG RAN WG1 #112			R1-2302127

Athens, Greece, February 27th – March 3rd, 2023

Title:	LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN1

To:	RAN4

Cc:

Contact person:	Florent Munier

Florent.munier@ericsson.com

Send any reply LS to:    3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

In the Rel-18 WI Expanded and Improved NR Positioning, in the agenda item “Positioning for redcap UEs”, RAN1 is discussing frequency hopping for the reception of the DL PRS and the transmission of UL SRS.

RAN1 made the following agreements:

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported. Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Conclusion The scope for RedCap positioning includes FR1 and FR2. Agreement For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance. - Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same. Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

# 2	Actions.

To RAN4

ACTION:

RAN1 kindly asks RAN4 for feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hop can be the same, and the Tx/Rx antennas used in all hops can be the same.

# 3	Dates of next TSG-RAN WG RAN1 meetings

RAN1#112bis-e	17th - 26th April 23, 					Electronic Meeting

RAN1#113			22th May – 26th May 2023,		Incheon, Korea