3GPP TSG RAN WG1 #112			R1-2302138

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.3.1

Source:	InterDigital, Inc.

Title:	Discussion on evaluation for NR duplex evolution

Document for:	Discussion and Decision

# Introduction

For Rel-18, a study item on evolution of NR duplex operation has been approved [1], where the objectives identified for the study item are as follows:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In RAN1#111 meeting [6], RAN1 continued discussions on this topic. As a result, some agreements and further discussion points were outlined which are listed below:

Conclusion For evaluation of SBFD Deployment Case 4, scenarios other than Urban Macro (FR1) and Dense Urban Macro layer (FR2-1) are low priority and it is up to companies to submit results for other scenarios. Agreement Consider following for the definition of coupling loss (CLp,uA,BwA,gB from Tx antenna port p of transmitter A to Rx antenna port u of receiver B: If both large scale fading and small scale fading are modelled, the coupling loss from Tx antenna port p of transmitter A to Rx antenna port u of receiver B is defined in formula (1) which is based on formula (B.1-2) in TR 37.910. CLp,uA,BwA,gB=PL∙SF∙α0,u,p2+n=1Nm=1Mαn,m,u,p2#1 If only large scale fading is modelled, the coupling loss from Tx antenna port p of transmitter A to Rx antenna port u of receiver B is defined in formula (2). CLp,uA,BwA,gB=PL∙SF∙α'0,u,p2#2 α'0,u,p=Frx,u,θθLOS,ZOA,ϕLOS,AOAFrx,u,ϕθLOS,ZOA,ϕLOS,AOAT∙100-1∙Ftx,p,θθLOS,ZOD,ϕLOS,AODFtx,p,ϕθLOS,ZOD,ϕLOS,AOD (3) Where - wA (wA=w1A,w2A,⋯,wNTA) represents a complex weight vector used for virtualization of Tx antenna port p of transmitter A, and gB (gB=g1B,g2B,⋯,gNRB) represents a complex weight vector used for virtualization of Rx antenna port u of receiver B. - Formula (3) can be understood according to equation (7.5-29) in TR38.901. Agreement Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that only large scale fading is modelled and small scale fading is not modelled for gNB-gNB co-channel channel model, CLBS A'→BS A can be modelled as below CLBS A'→BS A=1ST∙p=0ST-11SR∙u=0SR-1CLp,uA',AwA',gA wherein, - ST is the number of Tx antenna ports of BS A', and SR is the number of Rx antenna ports of BS A. Agreement Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, - For Ileakagen=HCLInWyn, it is up to companies to report other values of W and the corresponding applicable conditions. Agreement Regarding SLS calibration, consider the following metrics: - For CDF of gNB-UE coupling loss, only the coupling losses between each UE and its serving cell are collected for CDF statistic. - wA and gB are determined by selecting the best beam pair of the UE and its serving cell with the criteria of maximizing receive power of the UE. - For CDF of gNB-gNB coupling loss, - For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair - The two gNBs in each gNB pair should be from different sites. - Both wA and gB are randomly selected for calculating the coupling loss for each gNB pair. - Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. - For CDF of UE-UE coupling loss, - For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair - If the 2D distance between two UEs in a UE pair is larger than 50m, the UE pair is not considered for statistic. - For each UE, wA and gB is determined based on the best beam pair of the UE and its serving cell. - Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. - Note1: Formula (2) for CL with averaging across all the Tx/Rx ports is used for coupling loss calculation above, i.e., CLA,BwA,gB=1Sp=0S-11Uu=0U-1CLp,uA,BwA,gB=1Sp=0S-1PL∙SF∙1Uu=0U-1α'0,u,p2 - Note 2: The beams for above cases are selected based on a defined set of beams for FR1 and FR2 in the table for calibration assumptions. Agreement For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1, Alt 3 is deprioritized and the definition is updated as below. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 25% of the channel bandwidth.

In this contribution, we discuss evaluation assumption and methodology for study on NR-duplex evolution.

# Discussions

## Deployment scenarios and evaluation methodology

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-17, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-18, enhancements to support full duplex (FD) at least for gNB have been proposed and endorsed as the study item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on subband non-overlapping FD-gNB and HD-UEs in a cell

Considering a single UE perspective, a UE in a cell illustrated in Figure 2 can be informed of the mixed D/U transmission patterns across RBs per symbol/slot, where the UE can perform an UL Tx when the scheduled UL Tx is matched with an UL region of the mixed D/U transmission patterns on a given symbol/slot. For example, the UE can perform UL Tx on scheduled RBs across the adjacent 3 slots, as shown in Figure 2, which results in an UL coverage/capacity enhancement. As for the cell perspective, parallel DL transmissions toward other UEs over scheduled non-overlapping RBs can be performed by the FD-gNB.

In an SBFD scenario, the UEs that are close may experience inter-subband interference leakage. As such, it is necessary to investigate UE-to-UE CLI impacts to examine the potential benefits of SBFD. On the other hand, the studies have shown that scenarios based on fully-overlapping or partial-overlapping subband FD can be beneficial and achieve higher gains in specific traffic topologies, e.g., in small cells. Thus, the evaluations can be based on subband non-overlapping, followed by subband partial overlapping as well as subband overlapping scenarios.

Observation 1. Scenarios on subband non-overlapping (as for inter-subband CLI), subband partial overlapping and subband overlapping (as for intra-subband CLI) may achieve different gains based on at least traffic and/or cell sizes.

Proposal 1. Consider evaluating achieved gain and performance in subband non-overlapping scenario based on inter-subband CLI, followed by subband partial overlapping and subband overlapping scenarios based on intra-subband CLI.

Moreover, it has been agreed [2] that Rel-18 NR-duplex study considers both FR1 and FR2. Based on the above deployment scenarios, the study should take both intra- and inter-subband CLI effects into account for performance evaluation. Especially for the inter-subband CLI analysis, it should consider TX leakage power and effects on the rest of the RBs especially for adjacent RBs, depending on the actual resource allocation of the Tx in SBFD scenario.

Considering that the performance of the subband overlapping and subband non-overlapping scenarios depends on the traffic, cell size, etc., the evaluation can be performed on urban macro, micro, small cell, and indoor layouts. However, studies have shown [3] that the indoor scenarios where dropped UEs are in the same building can be the most important scenario which best presents the UE-to-UE CLI effects.

Proposal 2. Urban macro and indoor scenarios can be considered for evaluations in this study, where the indoor scenarios represent the most significant UE-to-UE CLI effects.

In the following sections, simulation assumptions and methodology based on our link-level and system-level simulation results are provided. In link-level simulations, we have addressed the impact of inter-subband and intra-subband CLI on the performance of the victim UE [4]. The results show the importance of such evaluations, as the results can be used to identify use cases and future target scenarios.

In system-level simulations, we have addressed the overall performance trade-off between DL and UL, in the presence of dynamic inter-cell CLI from neighbour cell(s) per slot basis. The results are provided in our companion contribution [5]. It is observed that while the performance depends on considered traffic model and DL/UL resource allocation scheme, application of CLI mitigation scheme is necessary for reliable communication in the presence of CLI.

## LLS simulation assumptions

In this subsection, we discuss LLS simulation assumptions and methodology based on our LLS results for study on NR-duplex. We conduct the LLS to address the potential negative impacts on DL reception on a subband (e.g., RBs), when intra-subband CLI on the same overlapped subband or inter-subband CLI on adjacent or non-overlapped subband exists due to nearby other UE’s uplink transmissions. Simulation assumptions for the LLS are summarized in Table 1.

We consider a duplexing scheme where the DL signal can be a couple of dBs less than, greater than or equal to the UL (CLI) signal. For a given UL (CLI) RB allocation, we evaluate the DL receiver performance for various RB allocations over the system bandwidth considering several degrees of intra and inter subband CLI overlaps as shown in Figure 3. For the initial results presented, no signal impairments due to PA nonlinearity or other imperfections are considered, where such impairments may further increase the adjacent inter-subband CLI level.

Table 1. LLS simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Antenna Configuration | 1Tx-2Rx |
| Carrier Frequency | 4GHz |
| SCS | 15kHz |
| MCS | 4, 10, 19 and 27 |
| System BW | 20MHz |
| Allocated no. of RBs for UL and DL signals | DL (Victim): 1RB, 2RBs, 5RBs, 10 RBs, 25RBs, 50RBs |
|  | UL (Aggressor): 25RBs |
| DL to UL power ratio (dB) | -6dB, 0dB, 6dB, 18dB |
| UL timing advance over DL | 0 , 1/2 of a symbol, 1/4 of a symbol |
| Propagation condition | TDL-A |
| Delay Spread | 30ns |
| UE Velocity | 3km/h |
| DMRS | DM-RS type-1, # of DMRS 1 + 1 |
| Channel estimation | Realistic |
| Receiver Type | MMSE |
| PA nonlinearity | None |

The DL throughput performance is compared with a corresponding DL throughput performance with no intra/inter-subband CLI.

Figure 3. Normalized signal spectrum of UL and DL signals

Figure 3 shows the normalized power spectrum of 5MHz (25-RBs) DL signal when the signal is scheduled at different subband locations (shown in pink, sky blue, red and blue colors) in a 20MHz channel bandwidth relative to a 5MHz UL signal (green color). The UL signal introduces intra-subband CLI on the DL signal when it is fully or partially overlapped with the UL signal as shown by the pink and sky blue colors respectively. On the other hand, an inter-band CLI is introduced on the DL signal, shown by the red and blue colors, which are adjacent to a UL signal.

Figure 4. DL Throughput performance under various intra-subband CLI

Figure 4 shows throughput performance curves of a DL signal for various degrees (100%, 50% and 25%) of overlap with an UL signal introducing intra-subband CLI. It can be seen that DL throughput performance suffers considerably, below 40% of maximum throughput value and may even get below 10%, as a result of intra-subband CLI when there is an overlap in DL and UL subbands. Further results for this evaluation can be found in our companion contribution [4].

In addition, a realistic scenario would be to consider a timing advance (TA) on the UL signal originating from a neighboring cell. The timing advance can be a fraction of an OFDM symbol as demonstrated in Figure 5. The resulting effect of the TA on the UL signal may be to introduce inter-carrier interference on the UL signal and as a result increase the level of the inter-subband CLI.

TA

Slot [1-ms]

Figure 5. An example demonstrating TA on a UL signal

Observation 2: DL throughput performance suffers considerably as a result of intra-subband CLI when there is an overlap in DL and UL subbands.

It is worth to investigate performance variation on victim (DL) signal across different number of PRB allocations (e.g., 2, 5, 10, 25, and 50) to the victim signal, while fixing a number of PRB allocation (25PRB) for the CLI (UL) signal. As discussed in our companion contribution [4], it can be observed that the normalized throughput performance curves for the victim (DL) signal, where the normalization is conducted relative to the number of PRBs allocated to it, are shown to be degraded significantly as the frequency allocation (number of PRBs) of the victim signal decreases. It can also be seen that the relative degradation with lower number of frequency (PRB) allocation is considerable when there is 0PRB frequency gap between DL signal and the CLI (UL) signal compared to a frequency gap of 1PRB.

Proposal 3. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands.

## SLS simulation assumptions

In this subsection, we discuss SLS simulation assumptions and methodology based on our initial SLS results for study on NR-duplex. We conduct the SLS to see how much of a negative impact on DL and UL reception in a cell can be observed, when inter-cell CLI from neighbour cell(s) may be present dynamically per slot basis. We consider an Indoor office scenario, details of which are provided in Table 2. For the evaluation, we compare performance of a baseline TDD scheme, where all gNBs/cells in the deployment use an aligned (common) TDD configuration, with a flexible duplex scheme where each gNB can independently and dynamically select the TDD configuration that will best serve its traffic needs.

Details regarding the TDD configurations can be found in Table 3. No CLI mitigation schemes are employed for these simulations. In addition, there is no UL power control and all UEs transmit at the maximum transmit power. The results for this evaluation can be found in our companion contribution [5]. Our results highlight the detrimental effects of CLI on both DL and UL UPT, highlighting the need for effective CLI management schemes.

Table 2. Indoor deployment scenario

|  | Indoor Sub-7GHz |
| --- | --- |
| Layout | (a,b,c,d)=(20,40,20,40) |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | 10 per gNB per 20MHz |
| SCS | 30 KHz |
| Channel Model | NR InH Mixed Office model |
| BS Tx Power | 23dBm (total across all TX antennas) |
| UE Tx Power | 23dBm (total across all TX antennas) |
| BS Antenna gain | 0dBi |
| UE Antenna gain | 0 dBi |
| BS Noise Figure | 5dB |
| UE Receiver Noise Figure | 9dB |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | According to 36.889 Table A.1.1. DL/UL FTP traffic – DL/UL traffic ratio: 1/1 and 2/1 FTP |
| UE to UE link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Table 3: TDD configuration parameters

|  | Baseline (static TDD) | Flexible duplex |
| --- | --- | --- |
| DL/UL subframe ratio | Semi-static subframe allocation - For DL:UL traffic ratio = 1:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 2:1, DL/UL subframe = 6:4 | Flexible UL/DL subframe ratio allocation. gNB can choose between following additional DL/UL subframe options: DL heavy: 9:1, 8:2, 7:3 UL heavy: 4:6 |

### Evaluation assumptions for SBFD

In our companion contribution [4], we conducted an SLS for performance comparison between SBFD and legacy TDD. We consider the SBFD deployment case 1, where a single carrier is considered with all cells using the same SBFD subband configuration. We consider the FR1 - Urban macro deployment. We applied Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD) from the agreement made in RAN1#109-e, which includes the following:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Alt. 2: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

  - SBFD: Alt. 4: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

Legacy TDD uses a shared Tx/Rx antenna array for downlink and uplink transmission/receptions, whereas SBFD utilizes separate Tx/Rx antenna array for simultaneous downlink and uplink transmission. The total number of TxRUs is the same for both legacy TDD and SBFD, however in the SBFD case, each of the downlink and uplink uses half of the total number of TxRUs.

Regarding antenna elements, following options are considered for the evaluation:

- SBFD Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- SBFD Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

Additionally, the effect of ratio of self-interference (RSI) at gNB receiver is considered. The effect of three different RSI values: 135 dB, 145 dB, 165 dB on UL performance are evaluated.

We consider two traffic ratios: (i) DL:UL ratio = 1:1, and (ii) DL:UL ratio = 2:1. Additionally, for both of these we consider varying DL:UL traffic arrival rates/loads. Simulation results are found in our contribution [4]. Details of simulations are listed in Table 4.

Table 4. SLS simulation assumptions for evaluations for SBFD

| Parameter | Deployment Scenario |
| --- | --- |
|  | Urban Macro (UMa) (from 38.913) |
| Layout | 21 cells with wraparound ISD: 500m |
| Channel Model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor |
| UE Mobility | 3 Km/hr |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |
| Open-loop power control | Default: P_0 = -92 dBm, alpha=1.0 |
| BS/UE TX power | BS: 49dBm, UE: 23dBm |
| BS antenna configuration | Baseline TDD: (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 1, 4, 4, 0.8, 0.5). Shared Tx/Rx antenna array with 32 TxRUs SBFD: Separate antenna array for Tx/Rx Option 1 (same # of antenna elements as TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 2, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. Option 2 (# of antenna elements for SBFD is two times that of TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. |
| Slot structure | Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] SBFD: Alt. 2 - Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD: Alt. 4 - Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Subband configuration#1 with {DUD} pattern, one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. All cells use the same SBFD slot configuration |
| gNB self-interference | RSI : 135 dB, 145 dB, 165 dB |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports 4 Tx, 2 Rx |
| Channel estimation | Realistic |
| Transmission scheme | 16/32 Tx Type 1 Codebook |
| Scheduler | SU-MIMO (with PF) |
| Target BLER | 10% first transmission BLER |
| HARQ/repetition | 3 HARQ retransmission |
| Metric | DL/UL User Perceived Throughput |
| Traffic model | FTP3 (0.5MB as packet size) for DL/UL Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) DL:UL traffic ratio 1:1 and 2:1 are considered |

One of our main observations for this SLS is a degraded downlink performance for SBFD in exchange for overall uplink performance improvements. For the applied simulation setup, legacy TDD offers four (full bandwidth) slots out of every five for downlink transmissions, while SBFD provides the same number slots, however only 80% of the bandwidth for each of these is available for downlink transmissions. One of main reasons on the degraded performance in downlink is because of the applied static/fixed subband partitioning, as [DUD] = [40 20 40] RB split all the time, which is not flexible in coping with varying traffic/channel conditions. Therefore, we suggest considering for SBFD performance analysis with dynamic indication schemes of flexible subband partitioning, e.g., [DUD] = [A B C] RB split for which values of A, B, and C can be flexibly selected by gNB and informed to UE dynamically, as discussed in [4].

Additionally, UL performance improvement can be observed, as RSI increases from 135 dB to 165 dB, which results from the fact that total residual interference on the UL subband decreases. Since RSI has no impact on DL, DL performance is unaffected.

Observation 3. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance.

Observation 4. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink, which is not reflecting a practical usefulness of SBFD.

Proposal 4. Evaluations on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study.

Proposal 5. To fairly reflect a practical usefulness of SBFD, the static/fixed subband partitioning assumption is not a proper assumption but is to be used as a baseline assumption for SBFD, where flexible/dynamic subband partitioning schemes should be further evaluated to overcome the degraded downlink performance for SBFD.

### SLS calibration for NR duplex

In this subsection we provide calibration results Indoor office and Uma scenarios for FR1. We provide gNB-UE, gNB-gNB, and UE-UE CL results for both scenarios. The assumptions are based on those provided in section 7 of R1-2212993. These results are based on 50 drops.

Figure 6. Indoor Office FR1 CL

Figure 7. Outdoor UMa FR1 CL

# Conclusion

In this contribution, we discussed link-level and system-level evaluation assumption and methodology for study on NR-duplex. From the discussions, we made following observations and proposals:

Observation 1. Scenarios on subband non-overlapping (as for inter-subband CLI), subband partial overlapping and subband overlapping (as for intra-subband CLI) may achieve different gains based on at least traffic and/or cell sizes.

Observation 2: DL throughput performance suffers considerably as a result of intra-subband CLI when there is an overlap in DL and UL subbands.

Observation 3. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance.

Observation 4. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink, which is not reflecting a practical usefulness of SBFD.

Proposal 1. Consider evaluating achieved gain and performance in subband non-overlapping scenario based on inter-subband CLI, followed by subband partial overlapping and subband overlapping scenarios based on intra-subband CLI.

Proposal 2. Urban macro and indoor scenarios can be considered for evaluations in this study, where the indoor scenarios represent the most significant UE-to-UE CLI effects.

Proposal 3. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands.

Proposal 4. Evaluations on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study.

Proposal 5. To fairly reflect a practical usefulness of SBFD, the static/fixed subband partitioning assumption is not a proper assumption but is to be used as a baseline assumption for SBFD, where flexible/dynamic subband partitioning schemes should be further evaluated to overcome the degraded downlink performance for SBFD.

# References

RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC.

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e, May 2022

R1-1611679, “Evaluation method of flexible duplex”, Huawei, HiSilicon.

R1-2300331, “Discussion on subband non-overlapping full duplex operation”, InterDigital.

R1-2300332, “Potential enhancements on dynamic/flexible TDD operation”, InterDigital.

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111, November 2022