3GPP TSG RAN WG1 #112	R1-2302153

# Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0400 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on channel measurement and interference measurement in FR2-2 in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia (Moderator), ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2023-03-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#108-e meeting, one agreement related to periodic CSI-RS validation is reached. The agreement and the corresponding CR for TS 38.213 are as follows. Agreement For unlicensed operation (or shared spectrum channel access), if gNB indicates to the UE this gNB-UE connection is operating in LBT mode, the periodic CSI-RS should be validated by COT duration or dynamically granted PDSCH or aperiodic CSI-RS over the same set of symbols. Similar to periodic CSI-RS validation, for unlicensed operation, we also need to introduce the same restrictions for channel/interference measurement in FR2-2. That is, only when the UE is provided ChannelAccessMode2-r17 = 'enabled', the UE operating with unlicensed channel access shall not average measurements across different transmission bursts. Otherwise, if the UE is not provided ChannelAccessMode2-r17 = 'enabled', UE shall apply the same operation as in licensed spectrum. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For unlicensed operation in FR2-2, introduce the following restrictions for channel/interference measurement: when the UE is provided ChannelAccessMode2-r17 = 'enabled', the UE shall not average measurements across different transmission bursts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description on channel measurement and interference measurement in unlicensed spectrum in FR2-2 when the UE is not provided ChannelAccessMode2-r17 = 'enabled'. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.4.2	Report Quantity Configurations

<Unchanged parts are omitted>

For operation with shared spectrum channel access in FR1, or in FR2-2 when the UE is provided ChannelAccessMode2-r17 = 'enabled', if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging two or more instances of any periodic or semi-persistent nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement or for interference measurement located in different DL transmissions,

-	the instances of the nzp-CSI-RS-Resources are not in the same channel occupancy duration indicated by DCI format 2_0, if the UE is provided at least one of SlotFormatIndicator or co-DurationList; or

-	the instances of the nzp-CSI-RS-Resources occur within a set of consecutive symbols which are not all occupied by PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats, if any, and the corresponding PDCCH(s), if the UE is neither provided with CO-DurationPerCell nor SlotFormatIndicator, but is provided with csi-RS-ValidationWith-DCI

-	the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

<Unchanged parts are omitted>