3GPP TSG RAN WG1 #112bis-e                                                                                 R1- 2302688

e-Meeting, April 17th – April 26th, 2023

Source:                CATT

Title:                     Low-Power WUS receiver architectures and its performance

Agenda Item:       9.11.2

Document for:     Discussion and Decision

- Introduction

In RAN1#112, the follows was agreed for studying and evaluating low-power wake-up receiver (LP-WUR) architectures [1]:

Agreement Study the parallel receiver architectures (as examples that can be captured in the TR) for FSK based on the following diagrams: - Parallel homodyne architecture receiver - The observations made for homodyne/zero-IF architecture with baseband envelope detection in RAN1#110b/111 are also applicable here. - Parallel heterodyne architecture receiver - The observations made for heterodyne architecture with IF envelope detection in RAN1#110b/111 are also applicable here. - Note: Other architectures are not precluded. - The OOK receiver architectures agreed for study in RAN1#110bis-e are also examples that can be captured in the TR Agreement Study the receiver architectures (as examples that can be captured in the TR) for FSK with frequency to amplitude conversion based on the following diagrams: - Homodyne architecture receiver with frequency to amplitude conversion - I/Q branches are required for frequency to amplitude conversion in digital BB. - Heterodyne architecture receiver with frequency to amplitude conversion - Companies provide the exact type FFS what type(s) of frequency to amplitude conversion being is studied. - Note: Other architectures are not precluded. Agreement For OFDMA-based signals/channels, study the receiver architectures based on the following diagrams: - I/Q branches are required for digital BB processing. - Digital BB processing may or may not include FFT (companies to provide details on how). - For sequence-based OFDM signals/channels, digital BB processing includes sequence correlation in either time domain (without FFT) or frequency domain (after FFT). - Proponent companies should at least provide details on power consumption reduction compared to the MR regarding the RF and digital BB processing. - Companies are encouraged to provide the break-down for the components. - The potential power reduction compared to the main radio may come from e.g.: - Lower performance LNA/amplifier - Oscillator/PLL with relaxed performance requirements - ADC with lower sampling rate and smaller bit-width - Reduced BB processing complexity compared to the MR - Companies are encouraged to provide the performance analysis corresponding to the considered power consumption considering the impact of e.g. phase noise, I/Q mismatch. - Companies to report whether the LP WUR is assumed to share components with MR. In case of component sharing, the potential impact on the MR ultra-deep sleep state should be considered. - Companies to report the possible number of information bits - In addition, companies should consider the power consumption in the OFF state and the transition energy. Agreement For the study on LP WUR architecture, power consumption relative to the deep sleep state of the MR is provided. - Deep sleep state of non-RedCap UE should be assumed.

In this paper, the waveform and modulation schemes for the LP-WUR are discussed and the OFDM-based receiver is analyzed.

- Waveform and Modulation Schemes for LP-WUR architectures

Multi-carrier (MC)-ASK, multi-carrier (MC)-FSK, OFDM-based signal are discussed for LP-WUS design. RF envelope detection architecture, heterodyne architecture and zero-IF architecture are supported as the candidate architecture for the evaluation of LP-WUR. The OOK modulation scheme, is the basic modulation scheme with simple receiver architecture for low power consumption and feasible for all three types of LP-WUR architectures. FSK modulation scheme is a traditional frequency detection with simple receiver architecture, similar to FM radio receiver, and can be used for different receiver architectures, e.g., the FSK receiver architecture based on parallel homodyne or heterodyne receivers can be represented by the diagrams in contribution [1].

Furthermore, the FSK receiver architectures based on parallel homodyne and heterodyne receiver architectures and homodyne and heterodyne with frequency to amplitude conversion were discussed in RAN1#112 and were captured in TR as examples. In general, a FSK receiver typically consumes more power compared to an OOK receiver with the similar architecture due to the additional components needed for FSK demodulation. For OFDM-based signal, the receiver brings higher power consumption than MC-ASK, MC-FSK due to the FFT block and baseband processing.

The target of the LP-WUR architecture study in 3GPP is to specify the waveform and the modulation scheme to achieve the low power consumption in UE wakeup mechanism and improve the UE power saving. The waveform and modulation is the critical aspect in assembly the low-power consumption components of wakeup receiver. In this section, the waveform and modulation are discussed and analysed.

2.1 Analysis of waveform to LP-WUR architectures

In RAN1#111, how to supress the adjacent channel interference (ACI) and adjacent subcarrier interference (ACSI) in three types of architecture was discussed, i.e. the architecture with RF envelope detection may require very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.   For homodyne/zero-IF architecture with baseband envelope detection and heterodyne architecture with IF envelope detection, it is more effective and less complex to use BB BPF/LPF and IF BPF to supress interferences, respectively, but with the requirements of higher frequency stability of local oscillator for down converter.

Detection complexity and Power consumption

In general, ASK modulation is the simple envelop detection and brings lower detection complexity and power consumption comparing to those of FSK modulation and OFDM. More details will be shown in section 2.2, in where the analysis of modulation schemes is discussed.

Adjacent channel leakage

For MC-ASK with CP-OFDM waveform, the adjacent channel leakage would result in the high PAPR for the LP-WUS.  The MC-ASK modulation is more sensitive for high PAPR. It would decrease the LP-WUS detection performance. While FSK with CP-OFDM waveform has relative low sensitive to the PAPR comparing to that of MC-ASK waveform. For OFDM-based signal waveform, the low PAPR sequence is configured and it has better detection performance than MC-ASK waveform.

Observation 1: For adjacent channel leakage, MC-ASK with CP-OFDM waveform is more sensitive to the PAPR comparing to that of MC-FSK with CP-OFDM waveform and OFDM-based signal waveform.

Interference rejection capability

From the perspective of adjacent channel interference rejection, the ASK and FSK modulation with CP-OFDM waveform has lower interference injection capability. For RF-envelop detection receiver, the LP-WUS signal modulated by ASK or FSK waveform is received and detected in RF.  If the interference signals superposing with the LP-WUS signals, the RF-envelop detection does not have any capability to single out the LP-WUS signals from interference signals. The RF filter has wider bandwidth than OFDM-based LP-WUS signal. So the ASK and FSK with RF-envelop detection receiver has serious adjacent channel interference comparing to that of OFDM-based LP-WUS signal. Similarly, for IF/BB-envelop detection receiver, the LP-WUS signal modulated by ASK or FSK is received and detected in IF. The IF/BB filter could be further used to supress the adjacent interference comparing to RF filter. But additional IF/BB filter would cost high power consumption since IF/BB-envelop detection needs high stability LO to perform coherent detection. The practical implementation in receiver would introduce additional interference through signal processing due to the noise from the receiver components, e.g., LO leakage and flicker noise.

For OFDM-based waveform, the signals are orthogonal with each other when the timing error is negligible. Thus, it has high interference rejection capability than that of OOK/FSK waveform. However, the interference rejection capability of the OFDM-based waveform requires the FFT process to filter out the superposition interference. The FFT process will increase the receiver complexity exponentially and bring more power consumption than OOK/FSK waveform.

Observation 2: For adjacent channel injection, OFDM-based signal waveform has better adjacent channel injection performance than that of MC-ASK/MC-FSK with CP-OFDM waveform but with extreme high receiver complexity and higher power consumption when FFT process is required at the LP-WUR.

Co-existence with NR channels/signals

For MC-ASK, it could reuse the existing NR OFDM structure and reuse NR resource by means of FDM and/or TDM. An example for MC-OOK is shown in Figure 1, where the NR channel/signals and LP-WUS are multiplexed at the NR bandwidth by one or separate IFFT process. If LP-WUS and NR signals are multiplexed with one IFFT process, the Low power wakeup receiver needs to perform FFT to separate LP-WUS signals from NR signals. If LP-WUS signals uses separated short IFFT embedded inside the long IFFT of NR signals/channels, the LP-WUS and NR signals are multiplexed after IFFT and remain orthogonal to each other. The LP-WUR does not need FFT process to separate out the LP-WUS from NR signals. Furthermore, FSK could perform by parallel OOK and FSK has the similar co-existence performance comparing with ASK. Thus, both MC-ASK and MC-FSK have good co-existence performance with NR channel/signal.

- CP-OFDM based LP-WUS with one IFFT

- CP-OFDM based LP-WUS with short IFFT

Figure 1:   CP-OFDM based LP-WUS

In summary, the Table 1 is listed for comparison between different waveforms to each receiver architecture.

Observation 3: For co-existence with NR channels/signals, all of the waveforms to each receiver architecture have good co-existence performance with NR channels/signals.

Table 1: The comparison of different LP-WUS waveforms in different receiver architectures

| Waveform | MC-ASK |  |  | MC-FSK |  |  | OFDM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Receiver type | RF-receiver | IF-receiver | BB-receiver | RF-receiver | IF-receiver | BB-receiver | BB-receiver with additional block |
| Detection complexity | Low | Low | High | Low | Low | High | Very High |
| Power consumption | Low | Medium | High | Low | Medium | High | Very High |
| Adjacent channel leakage | Sensitive | Sensitive | Sensitive | Insensitive | Insensitive | Insensitive | Insensitive |
| Interference rejection capability | Low | Medium | High | Low | Medium | High | High |
| Co-existence with NR channels/signals | Good |  |  |  |  |  |  |

2.2 Analysis of modulation schemes for LP-WUR architectures

Detection complexity and Power consumption

Non-coherent modulation such as ASK and frequency shift keying (FSK) could be used exclusively for ultra-low power receivers. From [2], each of the OOK and FSK modulations can be supported for three types of architecture (i.e. RF envelope detection, heterodyne, and zero-IF). In OOK modulation scheme, the information is embedded in the RF signals with the information bit of ‘1’s or ‘0’s representing by ON and OFF keying. The source node transmits a high amplitude carrier with or without FFT scrambling when it wants to send a ‘1’ and nothing is sending for ‘0’, i.e. the transmitter is turned off. At the receiver side, OOK demodulation does not require any channel equalization in the frequency and time domain, and therefore a non-coherent detection (e.g. envelop detection) is sufficient. The low-power wakeup receiver survey from 2005 to present [3] shows that OOK is widely used waveform for LP-WUS. The receivers have power consumption below 10uw while using OOK modulation [4]. To achieve higher data rate, ASK can be considered.

The advantage of FSK over OOK is that it is less susceptible to noise and fading. The RF amplifier design is straightforward for FSK modulation with no need of adaptable threshold of signal reception [4]. According to work in [2], FSK detection can be implemented by applying parallel OOK receivers and a comparator circuit or using a FM-AM converter. The low-cost receiver architecture may have the cost of more power over that of the conventional receiver for OOK modulation due to it requires accurate LO and I/Q signal path to demodulate the received signal. This brings the additional power consumption compared to OOK detection. In [5], an efficient direct modulation FSK transmitter and the simple ED based receiver are employed with the receiver power consumption of 420uW. As summarised in [3], the receiver with FSK modulation mostly brings high power consumption (e.g. 120uW~2100uW), which is substantially larger than OOK detection.

The OFDM reception requires good time/frequency synchronization which results in high power consumption compared to other modulations (OOK/FSK).

Observation 4: ASK modulation brings lower detection complexity and power consumption than FSK modulation and OFDM.

Capability in carrying the information bits

In 802.11ba, two data rates (250kbps and 62.5kbps) are support by WURs for achieving either better spectrum efficiency or better converge. For LP-WUS, the data rate that LP-WUS achieved is determined by the number of bits (M≥1) carried within one OFDM symbol, the SCS selection and the code rate. Thus, OOK and FSK modulations can bring same data rate with same assumption on M, SCS and encoding. For example, both OOK and FSK modulations can achieve 56 kbps with Manchester code (code rate =1), M=2, SCS=30 KHz.

Observation 5: OOK and FSK modulations can bring same data rate performance with same assumption on the number of bits (M≥1) carried within one OFDM symbol, SCS and encoding.

Signal detection for UE identification and information

Apart from the UE ID (i.e. 31bit), the LP WUS signal may bring other information e.g. cell-ID. The size of the signal will affect the signal detection. The larger the size is, the higher detection complexity gets. Thus, the detection complexity should be considered when designing the LP WUS.

Observation 6: The design of LP WUS would affect the signal detection.

Proposal 1: The waveform and modulation schemes should be selected with the target in minimizing power consumption of the low-power wakeup receiver.

2.3 Analysis of OFDM-based receiver

In RAN1#112, the OFDM-based architecture was discussed and agreed as one of the LP WUR architectures. The OFDM-based architecture is to have the OFDM receiver similar to the NR receiver of OFDM signal processing with FFT to convert the time domain signals to frequency domain signals. The OFDM-based architecture is notarized with its high complexity and high power consumption comparing to that of OOK/FSK based receivers due to the requirement of precise RF processing and high complexity digital baseband processing. The FFT operation in digital baseband processing is the component with high power consumption and cannot be replaced to maintain the orthogonality with other channels multiplexed in the same carrier. In addition, the OFDM-based architecture needs the higher sampling rate, a higher ADC bit-width, and better LO/PLL to obtain a high requirement than OOK/FSK based receivers, which consume more power. However, the OFDM-based architecture can use legacy SSB for synchronization and RRM measurement purpose. For RRM measurement, the standard effort of LP-WUS design in support of cell identification and interference rejection capability in RAN1, RAN2 and RAN4 would be large if the reference signal is LP-WUS, more details can be found in our companion paper [6]. The use of LP-WUS for RRM measurement also has the conflict with the RRM measurements by SSB and CSI-RS.

Proposal 2: The power model of the OFDM-based receiver needs to be addressed comparing to that of NR receiver before the OFDM-based receiver is captured in TR as an example.

- Conclusion

In this contribution, we have discussed the modulation and waveform of low-power wakeup receiver. We have the following observations and proposal:

Observation 1: For adjacent channel leakage, MC-ASK with CP-OFDM waveform is more sensitive to the PAPR comparing to that of FSK with CP-OFDM waveform and OFDM-based signal waveform.

Observation 2: For adjacent channel injection, OFDM-based signal waveform has better adjacent channel injection performance than that of MC-ASK/MC-FSK with CP-OFDM waveform but with extreme high receiver complexity and higher power consumption when FFT process is required at the LP-WUR.

Observation 3: For co-existence with NR channels/signals, all of the waveforms to each receiver architecture have good co-existence performance with NR channels/signals.

Observation 4: ASK modulation brings lower detection complexity and power consumption than FSK modulation and OFDM.

Observation 5: OOK and FSK modulations can bring same data rate performance with same assumption on the number of bits (M≥1) carried within one OFDM symbol, SCS and encoding.

Observation 6: The design of LP WUS would affect the signal detection.

Proposal 1: The waveform and modulation schemes should be selected with the target in minimizing power consumption of the low-power wakeup receiver.

Proposal 2: The power model of the OFDM-based receiver needs to be addressed comparing to that of NR receiver before the OFDM-based receiver is captured in TR as an example.

- References

- R1-2301818, ‘Summary #3 on LP WUR architecture’, Apple Inc., RAN1#112, February 27th – March 3rd, 2023.

- R1-2210909, “On architectures of LP-WUS receiver”, Huawei, HiSilicon, RAN1#111, November 2022.

- Low power radio survey, https:  //wics.engin.umich.edu/ultra-low-power-radio-survey.

- J. Blanckenstein, J. Klaue, and H. Karl, “A survey of low-power transceivers and their applications,” IEEE Circuits Syst. Mag., vol. 15, no. 3, pp. 6–17, 3rd Quart., 2015.

- B. Cook, A. Berny, A. Molnar, S. Lanzisera, and K. Pister, “Low-power 2.4 GHz transceiver with passive RX front-end and 400 mV supply,” IEEE J. Solid-State Circuits, vol. 41, no. 12, pp. 2757–2766, Dec. 2006.

- R1-2302689, ‘Physical layer signals and procedures for Low-Power WUS’, CATT, RAN1#112b-e, April 17th – April 26th, 2023.