3GPP TSG RAN WG1 #112bis-e			R1-2303236

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.4.1.2

Source: 	CMCC

Title: 	Discussion on physical channel design framework for sidelink on unlicensed spectrum

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new WID on NR sidelink evolution was approved [1], in which the application of NR sidelink is further expanded to the commercial use cases with increased sidelink data rate. To achieve the increased data rate, the support of sidelink over unlicensed spectrum was included as one of the objectives in Rel-18 sidelink evolution.

In this contribution, we further provide our views on the identified issues of SL-U physical channel design based on the agreements made in previous RAN1 meetings.

Physical channel design

- 2.1 SL BWP and resource pool configuration

Intra-cell guard band

In RAN1#110bis-e meeting, the following agreement was reached on SL BWP and resource pool configuration [2]:

Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

It was agreed that the intra-cell guard band of two adjacent RB sets can be used for PSSCH transmission if and only if multi-channel access procedure is successful on the respective corresponding RB sets. During the discussion, some companies raised concerns on the potential unequal sub-channel size caused by using the PRBs within intra-cell guard band, and whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as that for PSSCH transmissions in the two RB sets, etc., which were listed as an FFS point.

First of all, for the potential unequal sub-channel size, our understanding is that this is not particularly caused by using intra-cell guard band. As the number of RBs within an RB set could be from 100 to 110 for 15kHz SCS and from 50 to 55 for 30kHz SCS, even without intra-cell guard band, the unequal sub-channel size exists. Handling of unequal sub-channel size with and without intra-cell guard band, including TBS determination, can be separately discussed later.

Observation 1: The unequal sub-channel size is not a specific issue caused by using intra-cell guard band. Even without using intra-cell guard band, the unequal sub-channel size exists as the number of RBs within an RB set could be from 100 to 110 for 15kHz SCS and from 50 to 55 for 30kHz SCS.

On the other hand, regarding the index of PRB(s) in the intra-cell guard band, it was discussed during the last RAN1 meeting without consensus reached. The latest intermediate proposal was recapped as follows:

Proposal 1-1: Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - For interlaced RB based transmission, the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets. - PRBs within intra-cell guard band are not used for PSFCH/S-SSB transmission

Our view is that the PRB(s) in the intra-cell guard band should have the same interlace index(s) as the PRBs for PSSCH transmission in the two RB sets, which reuses NR-U design. Therefore, we propose that:

Proposal 1: Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- For interlaced RB based transmission, the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets.

RP includes sub-set of one RB set

In addition, there was an open issue of whether a SL-U RP can include sub-set of one RB set, which was not discussed during RAN1#110b-e meeting. The latest intermediate proposal (in RAN1#110 meeting) was recapped as follows:

Proposal 1-4: Do not support one SL resource pool includes sub-set of PRBs of one RB set.

In unlicensed spectrum, LBT is performed based on a 20MHz operation channel (i.e., an RB set defined in NR-U). In NR-U, a UE can be configured with one or more BWPs, each of them equals to or larger than 20MHz, with the limitation of the BWP shall include N continuous RB sets, where N is a positive integer as shown below.

Figure 1: NR-U BWP and RB sets

In NR sidelink, a resource pool in frequency domain consists of N continuous sub-channels, and each sub-channel consists of M continuous PRBs. The candidate values of the sub-channel size are {10, 15, 20, 25, 50, 75, 100} PRBs, which is (pre)configured per resource pool.

Figure 2: NR sidelink resource pool in frequency domain

For frequency domain resource pool configuration in SL-U, as it is over unlicensed spectrum, to follow the LBT operation on a 20MHz channel and to ensure the OCB requirement, a SL-U resource pool should consist integer number of RB sets, at least for the resource pool enabling interlace RB-based transmissions.

Proposal 2: For a SL-U resource pool with interlace RB-based transmission, support the resource pool to include integer number of RB sets by (pre)configuration.

On the other hand, for the contiguous RB-based transmissions, we think that the legacy way of configuring a resource pool is sufficient. Although the UE still needs to perform LBT based on a 20MHz channel, it does not seem necessary to limit the resource pool configuration to include integer number of RB sets. With such restriction on resource pool, the configuration flexibility of sub-channel size and number is lost. On the contrary, by reusing the legacy way of configuration, there may come across the case that some unavailable resource which are only included in an intra-cell guard band or in one of the two RB sets and the corresponding intra-cell guard band, when a resource pool spans two RB sets. We think that further enhancements on resource selection can be studied to solve this issue, nevertheless, we don’t think resource pool configuration restriction to include integer number of RB sets is needed.

Proposal 3: For a SL-U resource pool with contiguous RB-based transmission, R16/17 NR sidelink resource pool configuration method should be reused, i.e., the resource pool is not necessary to include integer number of RB sets.

Time domain resources

The time domain resources of SL-U BWP and resource pool configuration was still open to further discuss. The latest intermediate proposal (in RAN1#110 meeting) was recapped as follows:

Proposal 1-3: UE expects all slots of an unlicensed carrier can be used for SL-U transmission.

In NR sidelink, the set of slots that belong to a sidelink resource pool is determined by a bitmap after excluding slots with S-SSB and slots of which at least one of the symbols determined by startSLsymbols and lengthSLsymbols are not semi-persistently configured as UL. In other words, the logical slots with consecutive index may not be actually continuous from the perspective of physical.

Figure 3: NR sidelink resource pool in time domain

Note that the potential gaps between two logical slots with consecutive index are in slot level, and much longer than 16us or 25us, therefore, the discontinuity of the logical slots will prevent a UE to keep the occupancy of the channel, which is not feasible to maintain a COT.

Observation 2: The slots configured for NR sidelink resource pool in time domain may not be continuous in physical slots, and is difficult to maintain a COT in unlicensed spectrum.

To solve this problem, the most straightforward solution is to enhance the resource pool configuration in time domain, and ensure that a resource pool contains contiguous physical slots. Specifically, a SL-U resource pool is configured with a bitmap of all “1” by default, which implies that no NR-U co-exists with SL-U in the same carrier, in this case different SL-U resource pools can be FDMed.

Proposal 4: The resource pool configuration in time domain should be enhanced to ensure that a SL-U resource pool contains contiguous physical slots.

- By default, a SL-U resource pool can be configured with a bitmap of all “1”.

- 2.2 Slot structure

In NR sidelink, a minimum resource allocation unit in time domain is a slot. A UE can only transmit PSSCH in the symbols available for sidelink within a slot, which are (pre-)configured per SL BWP by higher layer parameters startSLsymbols and lengthSLsymbols, where startSLsymbols is the symbol index of the first symbol of lengthSLsymbols consecutive symbols.

According to the LBT procedures defined for NR-U, the basic unit for sensing (sensing granularity) is a duration of 9us. A device can occupy the channel if is the channel is sensed to be idle during the sensing duration of a specific LBT type. Note that the sensing granularity is much smaller than the duration of a sidelink slot, so it is possible that a channel is sensed to be idle in the middle of a sidelink slot. In such a case, if the legacy NR sidelink slot structure is reused, the UE is not able to access the channel until the next slot boundary, which is not smart from the resource utilization efficiency point of view, and may have potential risks to lose the available channel which will bring more transmission delay.

To solve this issue, the most straightforward way is to introduce multiple channel access opportunities in a single sidelink slot, as shown in the following figure. If a UE clears a channel at starting position #1, it follows the legacy slot structure. On the other hand, if LBT successes after starting position #1, the UE is still able to access the channel at starting position #2.

Figure 4: Illustration of multiple starting positions within a slot

In RAN1#111 meeting, it was agreed to support 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. In the last RAN1 meeting, the remaining open issues related to 2 candidate starting symbols were continually discussed, and the following agreement on locations of 1st and 2nd starting symbols and working assumption on TBS determination were achieved:

Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

In the following, we further discuss the remaining issues on TBS determination and AGC symbols for a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmissions.

TBS determination

The above working assumption on TBS determination was for COT initiation case, and an FFS bullet was left to further discuss whether a different number of symbols for TBS calculation is needed in a shared COT. During the discussion, it was argued that as type 2 channel access mechanism is performed for transmissions in a shared COT, it is more reasonable to use the first starting symbol to determine the TBS. From this perspective, we believe that similar enhancement can be introduced for MCSt, because if a UE has successfully accessed the channel in a previous slot, it can naturally start transmission from the first starting symbol of the subsequent slots.

Proposal 5:  If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the reference symbol length for TBS calculation is determined based on 1st starting symbol for transmissions in a shared COT and subsequent slots after a successfully accessed slot in case of MCSt.

AGC symbol(s)

In RAN1#111 meeting, the following agreement was made regarding symbols for AGC purpose. During RAN1#112 meeting, this issue was discussed without any consensus:

Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Regarding the AGC behaviour of Tx UE, the following example should be considered. That is, if a UE 1 has started transmission in a slot with the 1st starting symbol, and another UE 2 also has transmission in the same slot but started with the 2nd starting symbol, if UE 1 and UE 2 are hidden nodes with each other, i.e., the transmission of UE 2 cannot be blocked by UE 1, then the AGC performance of UE 3, which is in-between UE 1 and UE 2, has higher possibility to be degraded. Therefore, the most safe and straightforward way is adding an AGC symbol corresponding to the 2nd starting symbol, i.e., Option 1 is preferred.

Figure 5 An example for AGC performance degradation

On the other hand, regarding the AGC behaviour of Rx UE, we think it is more feasible leave some room for UE implementation, so Option C is preferred.

Proposal 6: Regarding the AGC behaviour when a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- For Tx UE, Option 1 is preferred;

- For Rx UE, Option C is preferred.

- 2.3 PSCCH and PSSCH

Contiguous and interlace RB-based transmission

In RAN1#110 meeting, the following agreement was achieved regarding the contiguous and interlace RB-based transmission for PSCCH/PSSCH [3]:

Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

On the support of interlace and contiguous RB-based transmission, there is still an open issue of whether one resource pool can be enabled either one or both of contiguous RB-based and interlace RB-based transmissions. During the discussion in the last RAN1 meeting, as companies shared different views on it, no consensus was reached. In our views, it can be supported that a resource pool to be (pre)configured to enable either interlace RB-based transmission or contiguous RB-based transmission in the first place. Regarding the method in which both contiguous and interlace RB-based transmissions are enabled in the same resource pool, it seems weird to us as whether the OCB requirement is definite and dependent on the requirement of region regulation, hence it is not clear when/whether this method is feasible and needed. On the other hand, even if it is feasible, we prefer to postpone such discussion until we have reached consensus on other issues such as the SL-U resource pool configurations and the resource allocation granularity in the frequency domain, and the corresponding respective applicable conditions, to avoid potential resource reservation ambiguities.

Proposal 7: For PSCCH and PSSCH in SL-U, either one of the interlace RB-based transmission and contiguous RB-based transmission can be enabled by (pre)configuration in a resource pool level.

Resource selection for contiguous RB-based transmission

In the last RAN1 meeting, the following agreements on the sub-channel definition for contiguous RB-based transmission of PSCCH/PSSCH were reached:

Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

With the above agreement that legacy configuration way in NR sidelink is reused, as shown in the following figure, the candidate resources may be categorized into 3 types:

- Type A: A candidate resource contains no guard band RBs;

- Type B: A candidate resource contains guard band RBs and occupies two RB sets;

- Type C: A candidate resource contains part of or all guard band RBs and occupies only one RB set.

Figure 6 Illustration of Type A,B,C resources w.r.t intra-cell guard band

To avoid selecting unavailable candidate resources caused by intra-cell guard band (i.e., Type C candidate resource), some enhancements on resource selection procedure can be further considered.

One potential solution is to enhance the resource (re)selection procedure in MAC layer. When a UE reports the candidate resource set SA to higher layer (i.e., MAC layer), MAC layer can only perform random selection within Type A or Type B candidate resources. On the other hand, the enhancement can also be considered in PHY layer procedure instead to determine the subset of resources to be reported to higher layer. In this case, the UE will exclude Type C resources from the candidate set SA. Moreover, in the resource pool configuration, higher layer parameters including sl-StartRB-Subchannel-r16, sl-SubchannelSize-r16 and sl-NumSubchannel-r16 can be configured to ensure that all guard band RBs are covered within one sub-channel, which avoids Type C resources by configuration.

Proposal 8: For contiguous RB-based transmission of PSCCH/PSSCH in SL-U, consider the following enhancements on resource selection procedure to exclude unavailable resources caused by intra-cell guard band:

- Alt. 1: The enhancement is introduced in MAC layer to avoid selecting unavailable resources from the reported set SA.

- Alt. 2: The enhancement is introduced in PHY layer to exclude unavailable resources from the candidate set SA.

- Alt. 3: Unavailable resources is avoided by setting proper higher layer parameters including sl-StartRB-Subchannel-r16, sl-SubchannelSize-r16 and sl-NumSubchannel-r16 in resource pool configuration.

Resource indication for interlaced RB-based transmission

In RAN1#112 meeting, the following agreement on the sub-channel definition for interlaced RB-based transmission was achieved:

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Based on the agreement, the issue on resource indication in the frequency domain for the interlaced RB-based transmissions should be further discussed, which were not touched due to the limited time in the last RAN1 meeting. The latest agreement made in RAN1#110bis-e meeting on such issue is recapped as follows, down-selection between Option A/B and Option 1/2 should be made.

Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details

Two different resource allocation mechanisms are adopted in R16/17 NR sidelink and NR-U, as shown in Figure 5(a) and 5(b).

Figure 7: Resource allocation mechanisms in (a) NR sidelink; (b) NR-U

In R16/17 NR sidelink, contiguous sub-channels are allocated to the PSCCH/PSSCH transmissions, and FRIV is used to indicated the starting sub-channel and/or number of allocated sub-channels of resources in the frequency domain. However, NR-U uses “X + Y” bits to provide the resource indication in the frequency domain, where “X” and “Y” bits are used to indicate the allocated interlaces and RB sets, respectively. Note that in NR-U, the multiple RB sets should be contiguously allocated, but the indicated multiple interlaces can be non-contiguous, which is different from the contiguous resource allocation mechanism in NR sidelink.

During previous RAN1 meetings, as some companies mentioned, using different interlace index(s) in different RB sets may cause some PAPR and uneven PSD issue, so in NR-U the allocated interlace index(s) in different RB sets should be same. In SL-U, we think reusing NR-U design principle is the most safe and straightforward way. Otherwise the RAN1 progress in some aspects, e.g., resource indication or resource selection, will be blocked since we may need to involve RAN4 into the discussion.

Observation 3:  Similar with NR-U, using same interlace index(s) in different RB sets is the most safe and straightforward way to avoid PAPR and uneven PSD issue; otherwise, RAN1 progress may be blocked.

In our views, the NR-U resource allocation mechanism of “X+Y” bit should be directly reused in SL-U resource allocation in the frequency domain for interlace RB-based PSCCH/PSSCH transmissions. The rationale behind is that: 1) Using “Y” bit can easily allocate 1 or multiple contiguous RB sets; 2) Using “X” bit first provides flexibility to support both contiguous and non-contiguous resource allocations, i.e., it can achieve contiguous sub-channel allocation same as R16/17 sidelink, and secondly, using “X” bit can also ensure that same interlace index(s) are allocated in different RB sets with the agreement on mapping between sub-channel and interlace.

Observation 4:  Only using FRIV for frequency domain resource indication may introducing some unavailable scheduling due to some combinations of sub-channels may lead the interlace index(s) across different PRBs to be different.

Proposal 9: For frequency domain resource indication for interlace RB-based PSSCH transmission, when more than one RB set is used for transmissions, Option A is supported, i.e.,

- Option A: The used interlace index(s) in different RB sets are always the same.

Proposal 10: For frequency domain resource indication for interlace RB-based PSSCH transmission,

- Option 1 is supported, i.e., explicitly indicating the used sub-channel index(s) and RB set index(s)

- Reuse the “X+Y” bit for NR-U resource allocation in frequency domain to allocate one or more sub-channels and RB sets.

Based on the proposed resource indication method (Option 1) above, there will also be some impacts on resource selection procedure. For instance, the resource selection granularity may be variable in the dimension of RB set , for example, if the resource pool includes 2 RB set, and 20 PRBs can fulfil the requirement of current SL packet’s transmission, one option is UE select a resource which contains 1 interlace across 2 RB set, but the other option is UE can select 2 interlace in 1 RB set, it seems that this kind of variable granularity will bring more complexity to mode 2 resource selection procedure. From our perspective, when performing resource selection procedure, the number of RB sets or actually used RB sets should be indicated by higher layer to PHY layer.

Proposal 11: When performing resource selection procedure, the number of RB sets or actually used RB sets should be indicated by higher layer to PHY layer.

PSCCH resource mapping in frequency domain

In NR sidelink, sub-channel size is per resource pool (pre-)configured, and the candidate values are {10, 15, 20, 25, 50, 75, 100} PRBs. The frequency domain size of PSCCH is also per resource pool (pre-)configured, and the candidate values are {10, 12, 15, 20, 25} PRBs. PSCCH is limited in only one sub-channel, and occupies the lowest PRBs of the lowest sub-channel of the associated PSSCH. In addition, the number of PSCCH symbols is per resource pool (pre-)configured, and the candidate values are {2, 3} symbols. PSCCH is transmitted in the same slot as the associated PSSCH, as shown below.

Figure 8: NR sidelink PSCCH/PSSCH multiplexing

For interlace RB-based PSCCH/PSSCH transmissions, the resource mapping of PSCCH in the frequency domain should be further investigated. In RAN1#110bis-e meeting, the following agreement was reached for this issue:

Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details

Regarding the FFS point on whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, and if it is the case, to allow UEs with different BW capabilities to communicate with each other, further enhancements on resource mapping of PSCCH, e.g., PSCCH locates in every RB set of corresponding PSSCH, can be studied. This issue was discussed during the last RAN1 meeting, but no agreement was achieved. The latest intermediate proposal was as follows:

Proposal 3-5: - Do not support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: For SL-U, if UEs support different bandwidths, they can use a narrow-bandwidth resource pool to communicate with each other as per legacy NR SL specification

To our understanding, we don’t think the scenario above is a valid case, as the resource pool and BW alignment between Tx and Rx UEs is the baseline for sidelink communication, otherwise, the whole system may not be workable and needs to be completely re-designed. Hence, regarding Option 2, the particular case of PSCCH locates in every RB set of corresponding PSSCH in a repeated way should be precluded.

Proposal 12: Do not support PSCCH locates in every RB set in a repeated way (PSCCH repetition across RB sets) for SL-U.

On the other hand, we are open to discuss some other cases of PSCCH locates in multiple RB sets of corresponding PSSCH. It should be noted that in NR sidelink, the frequency domain size of PSCCH can be flexibly (pre-)configured to ensure the coverage performance of the PSCCH. However, based on the agreement, 1 sub-channel equals at least K = 1 interlace for 30kHz SCS, and equals at least K = 1 and K = 2 interlaces for 15kHz SCS, which indicates the PRB size of PSCCH in SL-U may be limited. Some companies may argue that other K values can be introduced to solve this issue, however, we think that larger K values will decrease the flexibility of resource allocation, and we prefer to further study other alternatives of PSCCH resource mapping to solve the potential issue.

Observation 5: Reusing the PSCCH resource mapping principle in NR sidelink, i.e., Option 1, may limit the PSCCH size of SL-U and may impact the coverage performance.

One of the most straightforward method is to increase the number of interlaces occupied by PSCCH. One option is to occupy all the lowest interlaces of each RB set occupied by the corresponding PSSCH; The other one is to occupy more than one interlaces within only one RB set (lowest RB set). For the latter option, it may have risks of increasing the UE complexity to blind detect the PSCCH. For example, if the PSCCH can only occupy more than one interlaces within an RB set, as non-contiguous interlaces can be allocated to PSSCH/PSSCH transmissions, UE blind detection complexity is significantly increased compared to NR sidelink. Considering the case of 30kHz SCS, and a UE is allocated with 2 interlaces for PSCCH, there are C52=10 detection candidates for PSCCH. To address the UE complexity of blind detection, the former option is preferred, in which, the number of RB sets occupied by PSCCH can be (pre-)configured per resource pool, and the number of RB sets of the corresponding PSSCH should be equal to or larger than that (pre-)configured for PSCCH.

Proposal 13: PSCCH locates in multiple RB sets of the corresponding PSSCH should be further studied to ensure the (pre)configuration flexibility of the PSCCH size.

- The number of RB sets occupied by PSCCH can be (pre-)configured per resource pool to avoid increasing the UE complexity of blind decoding.

- 2.4 PSFCH and SL-HARQ

PSFCH format and resource determination

In NR sidelink, only one PSFCH format (PSFCH format 0) is specified which is a sequence-based physical channel. The sequence maps to the physical resource of 1 PRB in frequency domain and 1 symbol in time domain (The PSFCH contains two consecutive symbols in total as the first symbol is the duplication of the second symbol and used as AGC). Only 1 ACK/NACK bit can be carried by a PSFCH with format 0.

In RAN1#110 meeting, interlace RB-based transmission for PSFCH transmissions was agreed to support for 15kHz and 30kHz SCS, to meet OCB and PSD requirement. During the discussion, it was pointed out by many companies that if PSFCH is extended from 1 PRB to 1 interlace, it should further discuss the solutions to avoid the degradation of the PSFCH multiplexing capacity.

Considering that the target use case of SL-U is eMBB services with high data rate, in which the Tx UE may transmit multiple TBs to the Rx UE, then the Rx UE can naturally arrange the multiple HARQ-ACK bits in a HARQ codebook to the Tx UE, which is a more efficient way. In this sense, new PSFCH format should be further studied and designed, along with the enhancements on the design of PSFCH resource indication and HARQ codebook. To be specific, one potential solution is to introduce a PUCCH format 2 like new PSFCH format, and the size of the HARQ codebook can be determined based on the number of PSCCH/PSSCH transmissions corresponding to a same PSFCH slot. Instead of using source ID and/or member ID in R16/17 sidelink to determine the PSFCH resource index, destination ID and/or member ID can be used in such a case to determine the resource index, i.e., the feedback bits to one or multiple Tx UEs are contained in a HARQ codebook from a certain Rx UE, as shown in the following figure:

Figure 9: Illustration of new PSFCH format and resource determination

Proposal 14: New PSFCH format in addition to PSFCH format 0 should be studied to improve the PSFCH multiplexing capacity:

- A PUCCH format 2 like PSFCH format can be considered as a starting point.

Proposal 15: If the new PSFCH format is supported:

- (Pre-)configuration of PSFCH resources with fixed mapping timeline with respect to corresponding PSCCH/PSSCH transmissions can be considered as the starting point;

- Type 1 HARQ codebook can be considered as the starting point.

PSFCH due to LBT failure

In NR sidelink, PSFCH can only be dropped due to prioritization; while PSFCH dropping would be more frequent in SL-U due to LBT failure, which will be more harmful for reliability. In RAN1#112 meeting, it was agreed to support more than 1 PSFCH occasion per PSCCH/PSSCH transmission to address this issue:

Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

There is a remaining issue on whether such PSFCH occasions are (pre-)configured or are (pre-)configured and dynamically indicated. Considering dynamically indication of PSFCH occasion, first, it may introduce large specification impact on the PSFCH format and resource determination designs, on the other hand, it may lead to potential transmission holes (caused by a indicated occasion which is later than the occasion obtained by legacy mapping rule) and loss of COT. From this perspective, we prefer to prioritize the case that such PSFCH occasions are (pre-)configured.

Proposal 16: To support more than 1 PSFCH occasion per PSCCH/PSSCH transmission to address PSFCH transmission dropping due to LBT failure, the following option is prioritized:

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

In addition, with multiple PSFCH occasions for a PSSCH transmission in SL-U, the PSSCH-PSFCH mapping relationship should be further studied because only one-to-one mapping is supported in Rel-16 NR sidelink. The multiple PSFCH occasions can be introduced in time domain or frequency domain, it can be further studied either or both schemes are supported. To be specific, one potential solution is that the PSFCH resource in kth PSFCH occasion is the same as that in the 1st PSFCH occasion in frequency domain, where 1≤k≤K, and the K PSFCH occasions are between the nth (re)transmission and (n+1)th retransmission of a PSSCH. If the (k-1)th PSFCH occasion of a PSSCH transmission is not available due to LBT failure, the UE will send the PSFCH feedback using the same frequency and code domain in the kth PSFCH occasion if LBT success. Alternatively, the additional PSFCH occasions can be FDMed with the legacy PSFCH occasions, in which the PSFCH resource in the 1st PSFCH occasion is determined by the nth (re)transmission of the PSSCH and the PSFCH resource in the kth PSFCH occasion is determined by the (n+1)th retransmission of the PSSCH.

Figure 10: Illustration of multiple PSFCH occasions corresponding to one PSSCH transmission

Proposal 17: Consider the following PSSCH-PSFCH mapping relationships:

- Option 1: PSFCH resources in additional PSFCH occasions are determined by the nth PSSCH (re)transmission and are same with that of the 1st PSFCH occasion in frequency domain;

- Option 2: PSFCH resources in additional PSFCH occasion is FDMed with the legacy PSFCH occasion; PSFCH resources in the 1st PSFCH occasion is determined by nth PSSCH transmission, and PSFCH resources in additional PSFCH occasions are determined by the (n+1)th PSSCH retransmissionss

Conclusions

In this contribution, we provide our views on the physical channel design framework of SL-U, and the following observations and proposals are made:

Observation 1: The unequal sub-channel size is not a specific issue caused by using intra-cell guard band. Even without using intra-cell guard band, the unequal sub-channel size exists as the number of RBs within an RB set could be from 100 to 110 for 15kHz SCS and from 50 to 55 for 30kHz SCS.

Observation 2: The slots configured for NR sidelink resource pool in time domain may not be continuous in physical slots, and is difficult to maintain a COT in unlicensed spectrum.

Observation 3:  Similar with NR-U, using same interlace index(s) in different RB sets is the most safe and straightforward way to avoid PAPR and uneven PSD issue; otherwise, RAN1 progress may be blocked.

Observation 4:  Only using FRIV for frequency domain resource indication may introducing some unavailable scheduling due to some combinations of sub-channels may lead the interlace index(s) across different PRBs to be different.

Observation 5: Reusing the PSCCH resource mapping principle in NR sidelink, i.e., Option 1, may limit the PSCCH size of SL-U and may impact the coverage performance.

Proposal 1: Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- For interlaced RB based transmission, the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets.

Proposal 2: For a SL-U resource pool with interlace RB-based transmission, support the resource pool to include integer number of RB sets by (pre)configuration.

Proposal 3: For a SL-U resource pool with contiguous RB-based transmission, R16/17 NR sidelink resource pool configuration method should be reused i.e., the resource pool is not necessary to include integer number of RB sets.

Proposal 4: The resource pool configuration in time domain should be enhanced to ensure that a SL-U resource pool contains contiguous physical slots.

- By default, a SL-U resource pool can be configured with a bitmap of all “1”.

Proposal 5:  If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the reference symbol length for TBS calculation is determined based on 1st starting symbol for transmissions in a shared COT and subsequent slots after a successfully accessed slot in case of MCSt.Proposal 6: Regarding the AGC behaviour when a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- For Tx UE, Option 1 is preferred;

- For Rx UE, Option C is preferred.

Proposal 7: For PSCCH and PSSCH in SL-U, either one of the interlace RB-based transmission and contiguous RB-based transmission can be enabled by (pre)configuration in a resource pool level.

Proposal 8: For contiguous RB-based transmission of PSCCH/PSSCH in SL-U, consider the following enhancements on resource selection procedure to exclude unavailable resources caused by intra-cell guard band:

- Alt. 1: The enhancement is introduced in MAC layer to avoid selecting unavailable resources from the reported set SA.

- Alt. 2: The enhancement is introduced in PHY layer to exclude unavailable resources from the candidate set SA.

- Alt. 3: Unavailable resources is avoided by setting proper higher layer parameters including sl-StartRB-Subchannel-r16, sl-SubchannelSize-r16 and sl-NumSubchannel-r16 in resource pool configuration.

Proposal 9: For frequency domain resource indication for interlace RB-based PSSCH transmission, when more than one RB set is used for transmissions, Option A is supported, i.e.,

- Option A: The used interlace index(s) in different RB sets are always the same.

Proposal 10: For frequency domain resource indication for interlace RB-based PSSCH transmission,

- Option 1 is supported, i.e., explicitly indicating the used sub-channel index(s) and RB set index(s)

- Reuse the “X+Y” bit for NR-U resource allocation in frequency domain to allocate one or more sub-channels and RB sets.

Proposal 11: When performing resource selection procedure, the number of RB sets or actually used RB sets should be indicated by higher layer to PHY layer.

Proposal 12: Do not support PSCCH locates in every RB set in a repeated way (PSCCH repetition across RB sets) for SL-U.

Proposal 13: PSCCH locates in multiple RB sets of the corresponding PSSCH should be further studied to ensure the (pre)configuration flexibility of the PSCCH size.

- The number of RB sets occupied by PSCCH can be (pre-)configured per resource pool to avoid increasing the UE complexity of blind decoding.

Proposal 14: New PSFCH format in addition to PSFCH format 0 should be studied to improve the PSFCH multiplexing capacity:

- A PUCCH format 2 like PSFCH format can be considered as a starting point.

Proposal 15: If the new PSFCH format is supported:

- (Pre-)configuration of PSFCH resources with fixed mapping timeline with respect to corresponding PSCCH/PSSCH transmissions can be considered as the starting point;

- Type 1 HARQ codebook can be considered as the starting point.

Proposal 16: To support more than 1 PSFCH occasion per PSCCH/PSSCH transmission to address PSFCH transmission dropping due to LBT failure, the following option is prioritized:

Option 1: Such PSFCH occasion(s) are (pre-)configured

Proposal 17: Consider the following PSSCH-PSFCH mapping relationships:

- Option 1: PSFCH resources in additional PSFCH occasions are determined by the nth PSSCH (re)transmission and are same with that of the 1st PSFCH occasion in frequency domain;

- Option 2: PSFCH resources in additional PSFCH occasion is FDMed with the legacy PSFCH occasion; PSFCH resources in the 1st PSFCH occasion is determined by nth PSSCH transmission, and PSFCH resources in additional PSFCH occasions are determined by the (n+1)th PSSCH retransmissions

References

- RP-213678, New WID on NR sidelink evolution, OPPO, LG Electronics, RAN#94-e.

- Chair’s Notes, RAN1#110bis-e meeting v17.

- Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0.