3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2203398

# e-Meeting, April 17th - April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev | _ | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sidelink power control procedure in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-4-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-17, confusion occurs regarding current specification ‘expects to be provided with the same values of dl-P0-PSFCH’ since both dl-P0-PSFCH-r16 and dl-P0-PSFCH-r17 are supported in this release. Below are three different understandings towards this part of specification: Understanding 1: The same parameters for P0 should be configured for all resource pools, and the values of dl-P0-PSFCH-r16 and dl-P0-PSFCH-r17 should be the same. Understanding 2: The same parameters for P0 should be configured for all resource pools, the values of dl-P0-PSFCH-r16 should be the same if this parameter is provided for all resource pool, the values of dl-P0-PSFCH-r17, if provided, should be the same, but the values of dl-P0-PSFCH-r17 can be different from the values of dl-P0-PSFCH-r16 Understanding 3: Different parameters for P0 can be configured for each resource pool, i.e., some resource pools may only have dl-P0-PSFCH-r16, some only have dl-P0-PSFCH-r17, and some may have both, but the values of dl-P0-PSFCH-r16 and dl-P0-PSFCH-r17 should be the same. If understanding 3 is taken, UE adopts different PSFCH Tx power in different pools, violating previous RAN1 agreement. Between understanding 1 and understanding 2, understanding 2 is slightly preferred as it is aligned with the motivation of introducing the new parameter dl-P0-PSFCH-r17. The specification text is updated to reflect understanding 2 only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “expects to be provided with the same values of dl-P0-PSFCH” to “expects to be provided with the same values of dl-P0-PSFCH-r16, the same values of dl-P0-PSFCH-r17” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous description causes misunderstanding and erroneous UE behavior. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

    - 16.2.3	PSFCH

< Unchanged parts are omitted >

For resource pools configured with PSFCH resources overlapping in time, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH-r16, the same values of dl-P0-PSFCH-r17, and the same values of dl-Alpha-PSFCH for all the resource pools.

< Unchanged parts are omitted >