3GPP TSG RAN WG1 #112bis-e		                                                      R1-2304187

e-Meeting, April 17th – April 26th,2023

Source:	Moderator (NEC)

Title:	Summary of Corrections on Channel Structure for 2-step RACH

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

This document summarizes the discussions during RAN1#112bis-e on the following CRs.

R1-2303664	Corrections on Channel Structure for 2-step RACH in Rel-16	NEC Corporation

To be moderated by Xincai (NEC).

- Discussion

  - 1st round discussion

In TS 38.213 clause 8.1A, a mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration. However, the IE MsgA-PUSCH-Config in TS 38.331 can configure two PUSCH resource groups simultaneously which is given in the below, and if two PUSCH resource groups are configured through this IE, according to the description in TS 38.213, these two PUSCH resource groups in one PUSCH configuration will share one mapping relationship for these two groups. However, if two groups are configured, the mapping is separately performed for each group and each preamble group mapped to one PUSCH resource configuration. Besides, according to the field descriptions of msgA-PUSCH-ResourceGroupA and msgA-PUSCH-ResourceGroupB in TS 38.331, if UE performs MsgA transmission using  preambles group A PUSCH resources in group A should be used, and if UE performs MsgA transmission using  preambles group B then PUSCH resources in group B should be used. To ensure this, the mapping for each group should be separately performed, that is the mapping unit for preamble and PRU is per PUSCH resource group, not per PUSCH configuration as one PUSCH configuration may configure two PUSCH resource groups. Therefore, the misunderstanding will be introduced if per PUSCH configuration is used for the mapping unit in the spec. Therefore, CR in R1-2303664 correct the preamble to PRU mapping unit to per PUSCH resource group configuration as follows.

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH resource group configuration.

The IE MsgA-PUSCH-Config in TS 38.331 is copied in the below.

MsgA-PUSCH-Config-r16 ::= SEQUENCE { msgA-PUSCH-ResourceGroupA-r16 MsgA-PUSCH-Resource-r16 OPTIONAL, -- Cond InitialBWPConfig msgA-PUSCH-ResourceGroupB-r16 MsgA-PUSCH-Resource-r16 OPTIONAL, -- Cond GroupBConfigured … } MsgA-PUSCH-Resource-r16 ::= SEQUENCE { msgA-MCS-r16 INTEGER (0..15), nrofSlotsMsgA-PUSCH-r16 INTEGER (1..4), nrofMsgA-PO-PerSlot-r16 ENUMERATED {one, two, three, six}, ... } msgA-PUSCH-ResourceGroupA MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group A. msgA-PUSCH-ResourceGroupB MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group B.

Question 1: Do you agree with the intention of the CRs in R1-2303664? If not, please elaborate your reasons.

|  | Company |
| --- | --- |
| Agree | ZTE, Sanechips, Huawei, HiSi |
| Not agree | Xiaomi, Nokia, Nokia Shanghai Bell, vivo, Intel |

| Company | Comments |
| --- | --- |
| Xiaomi | Group A and group B have two separate PUSCH configurations from our point of view. Besides, the current spec is clear that different PUSCH resource groups with different PUSCH configurations have different mapping relationship with PRACH resources. Thus, no spec changed is needed. |
| Nokia, Nokia Shanghai Bell | The resource group separation should be implicitly understood as part of the preamble selection procedure in 38.321. If the gNB provided the preamble group configuration in RRC configuration, the UE would as part of the 38.213 procedure already know whether it selected from preamble group A or group B - and subsequently it would also know the PUSCH configuration that belongs to that preamble group. No specification changes needed. |
| vivo | The descriptions of mapping between preambles and PUSCH resources per PUSCH configuration in RAN1 spec. are for one preamble/PUSCH group, which was discussed in Rel-16 time and no agreement to introduce the term “group” in RAN1 spec.. Regarding how the preamble group and PUSCH group are determined, it’s described in RAN2. Therefore, this CR is not needed. |
| Intel | We share similar view as other companies that resource group is per RAN2 concept and RAN1 spec mainly discusses the mapping between preambles and PUSCH resources per PUSCH configuration. No need to change the current RAN1 spec. |
| ZTE, Sanechips | We basically understand the intention of the CR. It’s clear for companies that PUSCH configuration in RAN1 spec refers to MsgA-PUSCH-Resource-r16 instead of MsgA-PUSCH-Config-r16 in 331 since the latter one includes 2 resource groups, but it may cause confusion to readers who never participated in standard discussion, especially that PUSCH configuration is closer to MsgA-PUSCH-Config-r16 literally. We share other companies’ concern that resource group concept is introduced in RAN2 and we agree that no need to mention group concept in RAN1 spec, but to make the RAN1 and RAN2 spec more aligned, we think following revised TP is a better approach: A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by MsgA-PUSCH-Resource. |
| Huawei | Open to clarify the issue if the concept of group configuration is not preferable in RAN1 spec. |
| Samsung | First, we did not agree the assessment on “according to the description in TS 38.213, these two PUSCH resource groups in one PUSCH configuration will share one mapping relationship for these two groups.” By reasoning reading, the mapping will not mess with two groups together; Second, if majority wants some changes in spec to make it crystal clear, the suggestion by ZTE is acceptable to us. |
| Ericsson | ZTE explains it well. We have the same suggestion, with a minor amendment since one MsgA-PUSCH-Resource is used for the mapping. A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by a MsgA-PUSCH-Resource. |
| Moderator | Thanks for ZTE’s further explanations and that’s the intentions of this CR. To Nokia: Here we talk about the understanding of the sentence “A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration” not the procedure of the PUSCH resource group selection. If the text is not revised, the meaning of “per PUSCH configuration” will cause misunderstanding for the unit of the mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource. |

Question 2: Do you agree with the text proposals in R1-2303664? If not, please elaborate your reasons.

|  | Company |
| --- | --- |
| Agree |  |
| Not agree | Xiaomi, Nokia, Nokia Shanghai Bell, vivo, Intel, ZTE, Sanechips, SS |

| Company | Comments |
| --- | --- |
| Xiaomi | Same view as Question 1. |
| Nokia, Nokia Shanghai Bell | Same view as Question 1. |
| vivo | See our comments in response to Q1. |
| Intel | Same view as Question 1. |
| ZTE, Sanechips | We suggest a revised TP as explained in Q1. |
| Ericsson | See question 1 |

Question 3: Do you have any other comments on CRs in R1-2303664? If yes, please elaborate.

| Company | Comments |
| --- | --- |

Summary:

vivo and Intel worry about to introduce the concept of group in RAN1 spec and want to follow the discussion in Rel-16. And ZTE, Sanechips, Huawei, HiSi and Ericsson think the CR is needed, and the below suggested TPs are given. Samsung can accept the revised TP1.

Alt1for TP (ZTE):

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by MsgA-PUSCH-Resource.

Alt2 for TP (Ericsson):

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by a MsgA-PUSCH-Resource.

  - 2nd round discussion

Discuss whether to support the proposed TP given by ZTE/Ericsson and if support, which one do you prefer.

Question : Do you agree with the proposed TP given by ZTE/Ericsson? If not, please elaborate your reasons. If support, companies are also encouraged to check whether "a" is needed as suggested by Ericsson.

|  | Company |
| --- | --- |
| Agree | Xiaomi, ZTE, Sanechips, Ericsson |
| Not agree |  |

| Company | Comments |
| --- | --- |
| Xiaomi | We are fine with the updated CR for better understanding of the specification. We can’t see the necessity to add “a”, and we prefer Alt 1 for TP provided by ZTE. |
| ZTE, Sanechips | In our understanding, usually “provided by” is directly followed by RRC parameter name, we don’t see the need to add “a”. So Alt 1 is preferred. |
| Huawei, HiSi | OK with Alt1 |
| Intel | Alt. 1. Can be part of alignment CR if there is alignment CR. |
| Nokia, Nokia Shanghai Bell | Still, we do not see a need for this clarification. But if the majority insists on this we can compromise. |
| Vivo | Do not see the need of the clarification CR as we commented earlier. |
| Moderator | To vivo: If do not clarification, the meaning of “per PUSCH configuration” will cause misunderstanding for the unit of the mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource. According to the descriptions of the spec, the readers who never participated in standard discussion will think that these two PUSCH resource group configured in one PUSCH configuration share one common mapping relationship which is not the actually meaning of the spec. Therefore, the additional explanation for “PUSCH configuration” such as “provided by MsgA-PUSCH-Resource” is needed. After adding these explanation, the misunderstanding can be avoided. Can you reconsider the essential of the clarification? |
| Ericsson | To clarify our intention: one MsgA-PUSCH-Resource is used for the mapping of one of group A or group B preambles, hence the ‘a’. While this is more clear in our view, the spec is still understandable without the ‘a’, so we are fine to agree to Alt 1. |

- Conclusion

After the second round discussion and offline check with some companies,  all companies participated with this email discussion have reached a consensus that the CR for adding an explanation of “PUSCH configuration”  is needed and have no concern with the below TP proposed by ZTE. Therefore below TP can be included in the CR for 38.213 8.1A.

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by MsgA-PUSCH-Resource.