3GPP TSG RAN WG1 #112bis-e			R1-2302268

e-Meeting, April 17th – April 26th, 2023

3GPP TSG-RAN WG4 Meeting #106                                         R4-2303633

Athens, Greece, Feb 27 – Mar 03, 2023

Title:	LS on clarification on impact of SRS antenna switching for TDD-FDD band combinations

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name: 	Liu Ye

E-mail Address: 	leo.liuye@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

The impact on FDD Rx for the FDD band in an FDD+TDD combination caused by SRS antenna switching was initially discussed for LTE CA in RAN4. The identified issue has been sent to RAN1 in R4-1708772.

According the following discussion in RAN1, correction was made for the RAN1 spec TS 36.213 in clause 8.2 with clarification on the expected scheduling restriction and RAN2 introduced capability parameters of switched together band and switch impacted Rx band consequently. However, it is noted that the clarification adopted in LTE RAN1 spec is not reflected in the NR spec, and the clarification for the expected scheduling restriction in LTE spec is for UL only.

With widely application of NR SRS antenna switching from Rel-15, including both EN-DC and NR CA, it is observed that the specifications without clear clarifications similar to that in LTE spec may cause some ambiguity for better supporting the feature in the network. Therefore, some clarifications for the expected scheduling restrictions for both UL and DL in the affected band in an EN-DC/NR CA band combination would be helpful enabling the SRS antenna switching feature.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above observations and identified issues into account and consider possible clarification in specification if needed for better utilizing the feature of SRS antenna switching.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#106-bis-e				17th – 26th April 2023										Online

TSG-RAN4 Meeting#107 	 					22nd – 26th May 2023										Korea