3GPP TSG RAN WG1 #112bis-e			R1-2302287

e-Meeting, April 17th – April 26th, 2023

3GPP TSG RAN WG4 Meeting #106  	                                                                      R4-2303712

Athens, Greece, February 27 – March 3, 2022

Title:	Reply LS to RAN1 on LP-WUR architectures

Response to:	R1-2212999/ R4-2300011

Release:	Rel-18

Work Item:	FS_NR_LPWUS

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Ruixin Wang

ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 would like to thank RAN1 for the LS on low-power wake-up receiver architectures.

RAN4 approve the workplan in R4-2301565, and decide to provide feedback to RAN1 in multiple batches aligned with RAN4 progress.

In this meeting, to evaluate the RF impacts of LP-WUR architecture, RAN4 make some initial assumption and agreements, e.g.,

- RAN4 assume 1RX architecture for LP-WUR as starting point

- ACS values from current UE specifications are used as a starting point for discussion to evaluate LP-WUR performance

- Consider 1.4MHz and 5MHz WUS bandwidth for FR1 evaluation as the starting point

  - Guard band if needed, can be located within 1.4MHz and 5MHz RF bandwidth

- No impact of LP-WUS on the existing gNB emissions and compliance requirements is baseline

- RF envelop detection architecture is more appropriate for single-band operation

- IF/BB envelop detection is more appropriate for multi-bands operation. Multi-band here still means that only one band at a time is being received.

To further evaluate the RF aspects of LP-WUR architecture, RAN4 would like to know the following clarifications from RAN1:

- Whether IoT/wearables/smartphone UE types are all considered for LP-WUR design

- Power consumption, coverage and SNR targets

- Max occupied RB number in channel bandwidth for LP-WUS, for 1.4MHz and 5MHz RF bandwidth case

- Possible supported SCS for LP-WUS, if applicable

- Whether WUS can be located in a band separate from the UE’s NR band

- Whether FR1 is considered as first priority frequency range

- Whether in-band power boosting of LP-WUS is considered from RAN1 perspective

# 2	Actions

To RAN1:

ACTION: 	3GPP RAN4 respectfully asks RAN1 to take the above feedback into account, and provide feedback on the clarification questions.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #106-e	17-26 April, 2023 				E-meeting

TSG-RAN WG4 Meeting #107                           	 22-26 May, 2023    		     Incheon, KR