# 3GPP TSG RAN WG1 #113	                                                                                     R1-2304536

# Incheon, Korea, May 22nd – May 26th, 2023

# Agenda item:	9.2.3.1

# Title:	Evaluation on AI beam management

# Source:	ZTE Corporation

# Document for:	Discussion and Decision

Introduction

The evaluation methodology and KPIs for AI/ML based beam management were discussed in RAN1#112bis-e and many fruitful consensuses have been achieved as follows. Particularly, several realistic assumptions were considered for evaluation, including measurement error, practical data collection, and quasi-optimal Rx beam. The UCI payload overhead was agreed to be an additional KPI for evaluation and RS overhead reduction was further clarified. Additionally, some initial observations were draw for Tx beam prediction without generalization, different set B, and quantization error, while how to draw observations for beam pair prediction and BM-Case2 remains to be discussed. In this contribution, we will provide our views and preliminary simulation results of the AI/ML based beam prediction.

Agreement: For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, RS OH reduction%=1−NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - Companies report the assumption on baseline scheme - Companies report the assumption on T1 and T2 - Other options are not precluded and can be reported by companies. Agreement: To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead (e.g., number of UCI reports and UCI payload size) and/or UCI overhead reduction for inference of AI/ML model can be reported by company. - UCI overhead reduction = 1- Total UCI payload size for AI/ML/Total UCI payload size of baseline. - Companies to report detailed assumption of UCI for AI/ML and baseline, e.g., including quantization mechanism Agreement: At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Companies report how to select the “best” Rx beam(s) - Option 2: Measurements of specific Rx beam(s) - Companies report how to select specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies. Agreement: For performance evaluation of AI/ML based DL Tx beam prediction for BM-Case1 and BM-Case2, optionally study the performance with a quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the “best” Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - At least the following options can be considered: - Opt A: Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. - Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. - Companies report how to find the quasi-optimal Rx beam with “previous measurement” - FFS: Opt B: The Rx beams for measuring Set B/Set C consist of the X% of “best” Rx beam exhaustive Rx beam sweeping and (1-X%) of random Rx beams [or the adjacent Rx beam to the “best” Rx beam]. - X%= 80% or 90%, or other values reported by companies. - Note: X% is the percentage of measurements with “best” Rx beams out of all measurements - Other options are not precluded. - Companies report the measurement/RS overhead together with beam prediction accuracy. Agreement: For AI/ML in beam management, further study performance with different types of label, considering the following: - Option 1a: Top-1 beam(pair) in Set A - Option 1b: Top-K beam (pair)s in Set A - Option 2a: L1-RSRPs per beam of all the beams(pairs) in Set A - Option 2b: Top-K beam(pair)s in Set A and the corresponding L1-RSRPs - Option 2c: Top-1 beam(pair) in Set A and the corresponding L1-RSRP - Other options are not precluded and can be reported by companies. Conclusion: - It is optional to evaluate and compare the performance for BM Case-1 with different UE distribution assumptions: - Option 1: 80% indoor, 20% outdoor as in TR 38.901 - Option 2: 100% outdoor Observation: - At least for BM-Case1 for inference of DL Tx beam with L1-RSRPs of all beams in Set B, existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) causes [a minor loss x%~y%, if applicable] in beam prediction accuracy compared to unquantized L1-RSRPs of beams in Set B.

Evaluation for spatial domain beam prediction

General description:

The task of beam selection aims at aligning the best beam at both the transmitter and receiver, which is challenging due to mobility and large attenuation in mmWave communication systems. A typical way for beam selection is to exhaustively sweep all beam pairs to find the best beam pair. However, as the number of beams grows, the beam selection overhead and associated power consumption for measurement would be unacceptably high. Therefore, we utilize AI/ML technologies to assist beam selection and explore the spatial domain beam prediction with low overhead as shown in Figure 2-1 below. The measured RSRPs of partial beams (or beam pairs) are used as the AI input and the predicted RSRPs of all beams (or beam pairs) are used as the AI output, where each beam pair includes a transmit beam at the gNB and a receive beam at the UE. Besides, for the case of spatial domain beam prediction, the beam set for measurement (i.e., set B) is a subset of the whole beam set for prediction (i.e., set A) in our simulation.

Figure 2-1. Illustration of AI/ML based spatial domain beam prediction.

Dataset construction:

To evaluate performance of the AI/ML based method for spatial domain beam prediction, we generate the dataset for model training, model validation and model testing according to agreed parameters in RAN1#109e, which is summarized in Table I in the appendix. Specifically, 10 UEs per sector per drop are randomly generated with a total of 500 simulation drops. 80% of UEs are located in indoor and 20% of UEs are located in outdoor as in TR 38.901. The UE speed is fixed to be 3 km/h. For antenna configuration, the baseline agreed in RAN#110bis-e is adopted directly, where a single panel with (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ and 2 panels with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) are used by the gNB and UE, respectively. Besides, the gNB and UE are equipped with 64 Tx beams in total and 4 Rx beams per panel, respectively.

AI/ML model and training methodology:

For the spatial domain beam prediction, a fully connected network with 6 layers is used with measured RSRPs of partial beams (or beam pairs, i.e., Set B) being used as the AI/ML model input and predicted RSRPs of all beams (or beam pairs,  i.e., Set A) being used as the AI/ML model output. Both the Tx-Rx beam pair prediction and Tx beam prediction are considered for comparison, which apparently corresponds to the P1 initial beam selection procedure and P2 beam refinement procedure, respectively. For ease of understanding, we use a simple notation <Tx Ry> for AI/ML model input in the following, where x denotes the number of Tx beams picked out of 64 Tx beams and y denotes the number of Rx beams picked out of 8 Rx beams. Particularly, if multiple Tx or Rx beams are used for beam measurement, we assume all measured beams are obtained by sampling the whole beam space <T64 R8> with a pre-determined beam pattern. The detailed parameters are captured in Table I for spatial domain beam prediction based on the AI/ML model.

Table I. Description of the AI/ML model for spatial domain beam prediction

| Model architecture | Fully connected network |
| --- | --- |
| Number of layers | 6 |
| Model input | Measured RSRPs of partial beams/beam pairs |
| Model output | Predicted RSRPs of all beams/beam pairs |
| Dataset size | 100,000 |
| Percentage of training/ validity/test samples | 90%, 5%, 5% |
| Model input size | Tx-Rx beam pair prediction: 64×8 DL Tx beam prediction: 64×1 |
| Model output size | Tx-Rx beam pair prediction: 64×8 DL Tx beam prediction: 64×1 |
| Model size | Tx-Rx beam pair prediction: 2.888M DL Tx beam prediction: 0.0456M |

Performance KPIs:

To evaluate the performance of AI/ML in spatial domain beam prediction, we provide our preliminary simulation results by KPIs related to beam predicting accuracy, which include average L1-RSRP difference of Top-1 predicted beam, beam prediction accuracy with 1dB margin for Top-1 beam, beam prediction accuracy for Top-1 /Top-K beams, and predicted L1-RSRP difference. Particularly, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams (unless otherwise mentioned). The predicted L1-RSRP difference is L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam. The beam prediction accuracy (%) for Top-K beams is defined as the percentage that the Top-1 genie-aided beam is one of the Top-K predicted beams.

Rx beam assumption for beam pair prediction

In this section, the Tx-Rx beam pair prediction is evaluated in term of performance KPIs mentioned above. The sampling rate on the whole beam space is fixed to be 25%, which means that the associated RS overhead reduction would be 75%. Besides, corresponding non-AI cases are also simulated as baseline performance, where the optimal beam is obtained by measuring the uniformly sampled partial beams and directly choosing the one with the largest measured RSRP. For the Rx beam assumption of beam pair prediction, both the Tx down sampling and/or Rx down sampling are considered as follows.

- Case 1-1-1 (down sampling in both Tx and Rx beams): In the <T32 R4> case for Tx-Rx beam pair prediction, 32 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by 4 Rx beams picked out of 8 Rx beams by uniform sampling in the UE. In this case, RSRPs of selected beam pairs (32x4=128 beam pairs) are used as AI/ML model inputs and predicted RSRPs of all beam pairs (64x8=512 beam pairs) are used as AI/ML model outputs.

- Case 1-1-2 (down sampling in Tx beams only): In the <T16 R8> case for Tx-Rx beam pair prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by all 8 Rx beams in the UE. In this case, RSRPs of selected beam pairs (16x8=128 beam pairs) are used as AI/ML model inputs and predicted RSRPs of all beam pairs (64x8=512 beam pairs) are used as AI/ML model outputs.

The simulation results are shown in Table II. The spatial domain beam pair prediction achieves a sufficiently high performance with only 25% beam overhead being used. Besides, the Case 1-1-2 with down sampling in Tx beams only achieves better beam prediction accuracy than that of Case 1-1-1 with down sampling in both Tx and Rx beams. As can be concluded, the number of Rx beams is relatively small and any sampling in the Rx beam space may cause serve loss of the spatial channel characteristic. Thus, in the beam pair prediction, it’s better to take measurement results of all Rx beams as the AI/ML model input instead of performing any sampling in the Rx beam space. Other assistance information such as beam shape or beam usage are not evaluated since they are implementation-related information of the gNB or UE, which may be difficult to be disclosed across vendors.

Table II. Performance comparison of Tx-Rx beam pair prediction with different Rx beam assumptions

| Cases |  | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-1-1 | Non-AI | 3.099 | 42.80% | 25.95% | 25.95% | 3.099 |
|  | AI | 1.171 | 77.60% | 60.35% | 87.15% | 0.756 |
| Case 1-1-2 | Non-AI | 2.604 | 43.25% | 27.55% | 27.55% | 2.604 |
|  | AI | 0.357 | 91.10% | 75.55% | 96.55% | 0.352 |

The spatial-domain beam pair prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used.

With the same sampling rate on the whole beam space, Case 1-1-2 (without down sampling in the Rx beam space) obtains a better performance than Case 1-1-1 (with down sampling in the Rx beam space).

In the beam pair prediction, it’s better to take measurement results of all Rx beams as the AI/ML model input (i.e., down sampling in Tx beams only) instead of performing any sampling in the Rx beam space.

For the Rx beam assumption of beam pair prediction, consider both the Tx down sampling and/or Rx down sampling for evaluation.

Rx beam assumption for Tx beam prediction

For evaluation of the DL Tx beam prediction and providing input for AI/ML model for training and/or inference, an agreement about Rx beam assumptions was reached in RAN1#112bis-e.

Agreement: At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Companies report how to select the “best” Rx beam(s) - Option 2: Measurements of specific Rx beam(s) - Companies report how to select specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies.

As the AI/ML based beam prediction focuses exclusively on data-driven methods, more input information would generate better beam prediction performance. Therefore, if measurement results of Set B associated with different Rx beams are available, it’s better that all information of measurement results including Rx beam ID are fed to the AI/ML model to facilitate better prediction performance, which falls into the field of beam pair prediction. Therefore, in the case of Tx beam prediction, we prefer that only one Rx beam is used for providing input for AI/ML model for training and/or inference.  If only one Rx beam is used for beam measurement for Tx beam prediction, four Rx beam assumptions are considered: 1) assumption 1: the best Rx beam for the best Tx beam within Set B; 2) assumption 2: the best Rx beam searched for one Tx beam within Set B; 3) assumption 3:  the best Rx beam for the best Tx beam within Set A (named by the optimal Rx beam) ; 4) assumption 4: a specific Rx beam (e.g., the first Rx beam in the first UE panel by default unless otherwise mentioned). Besides, the best Rx beam for each Tx beam within Set B (i.e., Rx assumption 5) is also considered for performance comparison, where multiple Rx beams are used for one round of model inference.

At least for evaluation on the performance of DL Tx beam prediction, consider the following five Rx beam assumptions for providing input for AI/ML model for training and/or inference,

Rx beam assumption 1: the best Rx beam for the best Tx beam within Set B

Rx beam assumption 2: the best Rx beam searched for one Tx beam within Set B

Rx beam assumption 3:  the best Rx beam for the best Tx beam within Set A

Rx beam assumption 4: a specific Rx beam (e.g., the first Rx beam in the first UE panel)

Rx beam assumption 5: the best Rx beam for each Tx beam within Set B

Performance comparison with “best” Rx beam assumptions

Agreement: For performance evaluation of AI/ML based DL Tx beam prediction for BM-Case1 and BM-Case2, optionally study the performance with a quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the “best” Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - At least the following options can be considered: - Opt A: Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. - Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. - Companies report how to find the quasi-optimal Rx beam with “previous measurement” - FFS: Opt B: The Rx beams for measuring Set B/Set C consist of the X% of “best” Rx beam exhaustive Rx beam sweeping and (1-X%) of random Rx beams [or the adjacent Rx beam to the “best” Rx beam]. - X%= 80% or 90%, or other values reported by companies. - Note: X% is the percentage of measurements with “best” Rx beams out of all measurements - Other options are not precluded. - Companies report the measurement/RS overhead together with beam prediction accuracy.

To evaluate the performance of DL Tx beam prediction with different “best” Rx beam assumptions, the following six cases of best Rx beam and quasi-optimal beam are considered.

- Case 1-2-1: Measurements of the best Rx beam for the best Tx beam within Set B (i.e., Rx beam assumption 1).

- Case 1-2-2: Measurements of the best Rx beam searched for one Tx beam within Set B (i.e., Rx beam assumption 2).

- Case 1-2-3: Measurements of the best Rx beam for the best Tx beam within Set A (i.e., Rx beam assumption 3).

- Case 1-2-4: Measurements of the quasi-optimal Rx beam which consists of 80% of “best” Rx beam by exhaustive Rx beam sweeping and 20% of random Rx beams (i.e., Opt B)

- Case 1-2-5: Measurements of the quasi-optimal Rx beam which consists of 90% of “best” Rx beam by exhaustive Rx beam sweeping and 10% of random Rx beams (i.e., Opt B)

- Case 1-2-6: Measurements of the best Rx beam for the best Rx beam for each Tx beam within Set B (i.e., Rx beam assumption 5).

Detailed simulation results of the above six cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table III for reference. Accordingly, the best performance of DL Tx beam prediction can be achieved by measurements of the best Rx beam for the best Tx beam within Set A (i.e., Case 1-2-3), which is apparently an upper bound on beam prediction performance. Compared with Case 1-2-3 that the best Rx beam is used, there is a performance loss due to the adoption of quasi-optimal Rx beam in Case 1-2-4 and Case 1-2-5. Besides, Case 1-2-1 and Case 1-2-6 outperform Case 1-2-2 in terms of beam prediction accuracy due to more RS resource overhead for beam sweeping.

Table III. Evaluation results for BM-Case1 without AI/ML model generalization for DL Tx beam prediction

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-2-1 | Non-AI | 3.113 40.26 % 25.64 % 25.64 % 2.120 |  |  |  |  |
|  | AI | 1.269 85.16% 76.38% 92.06% 0.492 |  |  |  |  |
| Case 1-2-2 | Non-AI | 5.777 29.72% 24.80 % 24.80 % 5.383 |  |  |  |  |
|  | AI | 4.117 62.30% 54.06% 75.26% 3.078 |  |  |  |  |
| Case 1-2-3 | Non-AI | 2.429 40.11 % 24.53 % 24.53 % 2.417 |  |  |  |  |
|  | AI | 0.047 99.14 % 91.41 % 99.85 % 0.138 |  |  |  |  |
| Case 1-2-4 | Non-AI | 3.230 35.06% 22.32% 24.04% 3.318 |  |  |  |  |
|  | AI | 1.116 81.06% 69.22% 91.82% 0.588 |  |  |  |  |
| Case 1-2-5 | Non-AI | 2.872 37.70% 23.84% 24.80% 2.911 |  |  |  |  |
|  | AI | 0.561 90.16% 80.20% 95.58% 0.350 |  |  |  |  |
| Case 1-2-6 | Non-AI | 3.113 40.26% 25.64% 25.64% 2.120 |  |  |  |  |
|  | AI | 0.479 92.80% 81.16% 98.96% 0.242 |  |  |  |  |

For DL Tx beam prediction, compared with the Rx beam assumption that the best Rx beam searched for one Tx beam within Set B is used, a better beam prediction performance is achieved if the best Rx beam for the best Tx beam within Set B is used for measurement.

Among all Rx beam assumptions for Tx beam prediction, the best performance can be achieved by measurements of the best Rx beam for the best Tx beam within Set A (i.e., Case 1-2-3), which can be an upper bound on DL Tx beam prediction performance.

The realistic determination of quasi-optimal Rx beam causes a significant performance loss compared with that of the best Rx beam assumption, where the quasi-optimal Rx beam consists of X% (X=80 or 90) of “best” Rx beam by exhaustive Rx beam sweeping and (1-X) % of random Rx beams.

Performance comparison with specific Rx beam assumptions

In this sub section, three sub cases of <T16 R1> are considered for comparison. In particular, 16 beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is fixed (i.e., Rx beam assumption 4, a specific Rx beam per AI/ML model input sample or for all AI/ML model input sample) during the beam measurement procedure.

- Case 1-2-7: Measurements of a specific Rx beam (i.e., Rx beam assumption 4) per AI/ML model input sample, i.e., the specific Rx beam is changed from the first Rx beam to the fourth Rx beam on the first UE panel as the input sample changes, and followed by recycling.

- Case 1-2-8: Measurements of a specific Rx beam (i.e., Rx beam assumption 4) per AI/ML model input sample, i.e., the specific Rx beam is changed from the first Rx beam to the eighth Rx beam on the two UE panels as the input sample changes, and followed by recycling.

- Case 1-2-9: Measurements of a specific Rx beam (i.e., Rx beam assumption 4) for all AI/ML model input samples, i.e., the first Rx beam on the first UE panel is always used.

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). Detailed simulation results of the above 3 cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table IV for reference. Accordingly, Case 1-2-7, Case 1-2-8, and Case 1-2-9 have similar performance, which is lower than that of the best Rx beam. As can be concluded, the measured RSRPs corresponding to the best Rx beam are usually higher than that of other Rx beams, and thus more features can be extracted by the AI/ML model.

Table IV. Simulation results for DL Tx beam prediction with specific Rx beam assumptions

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-2-7 | Non-AI | 2.118 42.32 % 24.86 % 24.86 % 2.114 |  |  |  |  |
|  | AI | 0.161 96.06 % 82.29 % 98.34 % 0.279 |  |  |  |  |
| Case 1-2-8 | Non-AI | 2.086 43.35 % 25.58 % 25.58 % 2.073 |  |  |  |  |
|  | AI | 0.152 96.19 % 82.32 % 98.55 % 0.269 |  |  |  |  |
| Case 1-2-9 | Non-AI | 2.114 42.99 % 24.72 % 24.72 % 2.116 |  |  |  |  |
|  | AI | 0.129 96.50 % 83.07 % 98.72 % 0.268 |  |  |  |  |

For DL Tx beam prediction based on a specific Rx beam assumption, Option 2a (i.e., measurements of specific Rx beam(s) per model input sample) and Option 2b (i.e., measurements of specific Rx beam(s) for all model input samples) show similar beam prediction accuracy.

Performance comparison of different Set B pattern

In RAN1#112, both options agreed in RAN1#110b-e for the selection of Set B of beams (pairs) were kept open for further study. Besides, option D was added as follows to cover the issue when not all the beams that UE measured is reported, which is valid because now the definition of Set B is the input of the AI/ML model, not the set of beams that UE measures. According to the agreement achieved in RAN1#112, more comprehensive Set B design can be studied and evaluated, including joint designing the number of beams in Set B and their beam shape.

Proposal: - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/ report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured/pre-known patterns - Opt B: Set B is changed among pre-configured/ pre-known patterns - Opt C: Set B is changed among Set A beams (pairs) - Opt D: Set B is changed among a set of beams (pairs) for measurement - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded. Agreement: - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

In this section, performance comparison with different patterns of Set B is conducted. In addition, in order to evaluate the performance impact of beam prediction when Set B is changed among a set of beams (pairs) for measurement (i.e., Set C), only RSRPs of Set B, which are subset of Set C, are used as AI/ML model inputs. In other words, UE only reports a subset of Set C (e.g., Top-4, Top-8, or Top-16 of the RSRPs of Set C), which is fed into the NW-side model.

Performance comparison with different pattern of set B(s) for fixed Set B

In order to evaluate the performance impact resulted from different Set B pattern, six different Set B patterns are designed as showed in Figure 2-2. The sampling rate of Set B pattern 1/2/3 is 25%, which is two times of that of Set B pattern 4/5/6. Beam pairs prediction and DL Tx beam prediction are conducted by considering the following 12 different simulation cases.

- Case 1-3-1: <T16 R8> case for Tx-Rx beam pair prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 (Figure 2-2(a)) are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-2: <T16 R8> case for Tx-Rx beam pair prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 2 (Figure 2-2(b)) are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-3: <T16 R8> case for Tx-Rx beam pair prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 3 (Figure 2-2(c))  are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-4: <T8 R8> case for Tx-Rx beam pair prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 4 (Figure 2-2(d)) are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-5: <T8 R8> case for Tx-Rx beam pair prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 5 (Figure 2-2(e)) are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-6: <T8 R8> case for Tx-Rx beam pair prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 6 (Figure 2-2(f)) are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-7: <T16 R1> case for DL Tx beam prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 (Figure 2-2(a)) are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-8: <T16 R1> case for DL Tx beam prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 2 (Figure 2-2(b)) are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-9: <T16 R1> case for DL Tx beam prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 3 (Figure 2-2(c)) are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-10: <T8 R1> case for DL Tx beam prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 4 (Figure 2-2(d)) are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-11: <T8 R1> case for DL Tx beam prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 5 (Figure 2-2(e)) are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-12: <T8 R1> case for DL Tx beam prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 6 (Figure 2-2(f)) are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

Figure 2-2. Illustration of different Set B patterns.

Detailed simulation results of the above 12 cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table V for reference. As observed, for Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance can be achieved with different Set B patterns in a same sampling rate. In addition, a sufficient high performance of DL Tx beam prediction can also be achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%.

Table V. Simulation results for different Set B patterns

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-3-1 | Non-AI | 2.028 44.04 % 25.43 % 25.43 % 2.044 |  |  |  |  |
|  | AI | 0.105 97.09 % 83.83 % 98.72 % 0.261 |  |  |  |  |
| Case 1-3-2 | Non-AI | 0.937 63.43 % 39.09 % 39.09 % 0.970 |  |  |  |  |
|  | AI | 0.131 96.88 % 82.15 % 98.67 % 0.249 |  |  |  |  |
| Case 1-3-3 | Non-AI | 1.737 44.36 % 25.47 % 25.47 % 1.745 |  |  |  |  |
|  | AI | 0.087 97.77 % 85.66 % 99.10 % 0.225 |  |  |  |  |
| Case 1-3-4 | Non-AI | 2.924 32.21 % 18.55 % 18.55 % 2.926 |  |  |  |  |
|  | AI | 0.764 81.77 % 61.49 % 91.45 % 0.700 |  |  |  |  |
| Case 1-3-5 | Non-AI | 2.781 33.60 % 19.62 % 19.62 % 2.770 |  |  |  |  |
|  | AI | 0.795 80.48 % 59.68 % 90.76 % 0.707 |  |  |  |  |
| Case 1-3-6 | Non-AI | 5.384 14.55 % 6.42 % 6.42 % 5.409 |  |  |  |  |
|  | AI | 0.825 79.75 % 60.67 % 90.90 % 0.725 |  |  |  |  |
| Case 1-3-7 | Non-AI | 2.350 41.24 % 25.96 % 25.96 % 2.364 |  |  |  |  |
|  | AI | 0.057 98.93 % 91.66 % 99.81 % 0.149 |  |  |  |  |
| Case 1-3-8 | Non-AI | 0.996 61.70 % 39.96 % 39.96 % 1.014 |  |  |  |  |
|  | AI | 0.071 98.59 % 86.78 % 99.64 % 0.161 |  |  |  |  |
| Case 1-3-9 | Non-AI | 2.052 40.55 % 24.82 % 24.82 % 2.049 |  |  |  |  |
|  | AI | 0.057 98.61 % 89.70 % 99.73 % 0.160 |  |  |  |  |
| Case 1-3-10 | Non-AI | 3.444 29.75 % 19.20 % 19.20 % 3.456 |  |  |  |  |
|  | AI | 0.511 89.90 % 73.79 % 97.47 % 0.411 |  |  |  |  |
| Case 1-3-11 | Non-AI | 3.199 31.56 % 19.81 % 19.81 % 3.169 |  |  |  |  |
|  | AI | 0.492 90.11 % 74.08 % 97.58 % 0.420 |  |  |  |  |
| Case 1-3-12 | Non-AI | 5.988 11.75 % 6.51 % 6.51 % 6.022 |  |  |  |  |
|  | AI | 0.496 87.70 % 72.29 % 98.17 % 0.500 |  |  |  |  |

For Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance is achieved with different uniform Set B patterns in the same sampling rate if AI/ML model generalization is not considered.

For DL Tx beam prediction, a high performance is achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%.

Set B is changed among a set of beams (pairs) for measurement (Set C)

To reduce the overhead of UE feeding back beam measurements, evaluation of Set B changed among a set of beam for measurement (i.e., Set C) is considered in this section. For instance, as shown in Figure 2-3, Set C is used for measurement and Set B is used for AI/ML model input, which consists of Top-4 beams with highest L1-RSRPs optimally selected from Set C of beams.

To evaluate the performance of Set B changed among Set C,  six cases are considered as follows.

- Case 1-3-1: Tx-Rx beam pair prediction, Top 4 L1-RSRPs from 128 beam pair measurements are used as AI/ML model inputs, where the 128 beam pair measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and all 8 Rx beams in the UE.

- Case 1-3-2: Tx-Rx beam pair prediction, Top 8 L1-RSRPs from 128 beam pair measurements are used as AI/ML model inputs, where the 128 beam pair measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and all 8 Rx beams in the UE.

- Case 1-3-3: Tx-Rx beam pair prediction, Top 16 L1-RSRPs from 128 beam pair measurements are used as AI/ML model inputs, where the 128 beam pair measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and all 8 Rx beams in the UE.

- Case 1-3-4: DL Tx beam prediction, Top 4 L1-RSRPs from 16 Tx beam measurements are used as AI/ML model inputs, where the 16 beam measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-5: DL Tx beam prediction, Top 8 L1-RSRPs from 16 Tx beam measurements are used as AI/ML model inputs, where the 16 beam measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-6: DL Tx beam prediction, Top 16 L1-RSRPs from 16 Tx beam measurements are used as AI/ML model inputs, where the 16 beam measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

Figure 2-3. Illustration of selection from Set C pattern to Set B pattern.

Detailed simulation results of the above cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table VI for reference. For DL Tx beam prediction, marginal performance loss is observed when the AI/ML model inputs are Top 8 L1-RSRPs or Top 4 L1-RSRPs of all 16 Tx beam measurements. For Tx-Rx beam pair prediction, significant performance loss is observed when the AI/ML model inputs are Top 8 L1-RSRPs or Top 4 L1-RSRPs of all 128 beam pair measurements. As can be concluded from the simulation results, to reduce the overhead of UE feeding back beam measurement for DL Tx beam prediction, it is better to only report partial beams from a set of beams for measurement for a NW-side model.

Table VI. Simulation results for different Top N Set B patterns

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-3-1 | Non-AI | 2.064 42.51 % 24.84 % 24.84 % 2.045 |  |  |  |  |
|  | AI | 0.856 79.85 % 58.90 % 87.77 % 0.682 |  |  |  |  |
| Case 1-3-2 | Non-AI | 1.940 43.66 % 24.70 % 24.70 % 1.965 |  |  |  |  |
|  | AI | 0.496 86.13 % 66.29 % 92.21 % 0.554 |  |  |  |  |
| Case 1-3-3 | Non-AI | 2.062 42.76 % 25.05 % 25.05 % 2.077 |  |  |  |  |
|  | AI | 0.223 93.47 % 76.38 % 96.65 % 0.358 |  |  |  |  |
| Case 1-3-4 | Non-AI | 2.346 40.69 % 24.55 % 25.28 % 2.353 |  |  |  |  |
|  | AI | 0.157 96.15 % 83.18 % 99.03 % 0.228 |  |  |  |  |
| Case 1-3-5 | Non-AI | 2.407 40.34 % 25.28 % 25.28 % 2.423 |  |  |  |  |
|  | AI | 0.072 98.48 % 90.61 % 99.56 % 0.157 |  |  |  |  |
| Case 1-3-6 | Non-AI | 2.381 40.29 % 24.29 % 24.29 % 2.370 |  |  |  |  |
|  | AI | 0.044 99.18 % 91.52 % 99.79 % 0.138 |  |  |  |  |

For DL Tx beam prediction, marginal performance loss is observed when the AI/ML model inputs are Top 8 beams or Top 4 beams from the 16 measured beams compared with all measured beams are used as AI/ML model input.

For Tx-Rx beam pair prediction, significant performance loss is observed when the AI/ML model inputs are Top 8 beams or Top 4 beams from the 128 measured beams.

To reduce the overhead of UE feedback beam measurements for DL Tx beam prediction in BM-Case 1, it is better to only report partial beams from a set of beams for measurement for a NW-side model.

Analysis on UCI payload overhead reduction

In RAN1#112bis-e, the UCI report overhead and/or UCI overhead reduction was agreed to be an additional KPI to evaluate the performance of AI/ML in beam management at least for NW side beam prediction. The agreement is shown as below.

Agreement: To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead (e.g., number of UCI reports and UCI payload size) and/or UCI overhead reduction for inference of AI/ML model can be reported by company. - UCI overhead reduction = 1- Total UCI payload size for AI/ML/Total UCI payload size of baseline. - Companies to report detailed assumption of UCI for AI/ML and baseline, e.g., including quantization mechanism

- Clarification on UCI payload overhead and baseline

To evaluate the UCI report overhead, the legacy quantization method is assumed, where the largest measured value of RSRP is quantized to a 7-bit value and the differential RSRP is quantized to a 4-bit value. The differential RSRP value is computed with 2 dB step size with a reference to the largest measured RSRP value which is part of the same RSRP reporting instance. All measurement results are assumed to be reported in one reporting instance and the UCI contents include M CRIs and N RSRPs, where each CRI is represented by a 3-bit or 4-bit value based on the number of beams in the measured beam set. The non-AI case is taken as a baseline where the optimal beam is obtained by measuring the uniformly sampled partial beams (i.e., Set C with 16 beams inside) and directly choosing the one with the largest measured RSRP. The number of beams to be reported for the non-AI case is assumed to be 4 as with legacy.

- Performance comparison between different cases

The following two cases are considered for the evaluation of UCI payload overhead reduction:

- Case 1-4-1: Set B equals to the measured beam set (i.e., Set C). In this case, all beams in the measured beam set needs to be reported and served as the AI/ML model input, where the number of beams in Set C is set as 8 or 16 for comparison. Besides, as mentioned in our accompanied contribution, if all beams in the beam set are reported, only the CRI associated with the largest RSRP needs to be reported and all other CRIs can be implicitly obtained by the RSRP mapping order. As a result, the UCI payload overhead can be greatly reduced.

- Case 1-4-2: Set B is a subset of the measured beam set (i.e., Set C). In this case, only partial Top-K beams with larger measured RSRPs in the measured beam set are reported, where the number of beams in Set B is set as 4 or 8 for comparison. Besides, all RSRPs to be reported are associated with respective CRIs for differentiation. Compared with the first case, the UCI payload overhead can be reduced as only partial beams in the measured beam set are reported.

- All cases and corresponding simulation results are summarized in the following table for comparison. Generally, more UCI payload brings better beam prediction performance. As observed, compared with the case that Set B equals to Set C and all beams are reported (i.e., Case 1-4-1), only marginal performance loss in term of beam prediction accuracy is obtained for the case that Top-4 or Top-8 beams with larger RSRP are reported. The total UCI payload size is greatly reduced by only reporting partial beams in Case 1-4-2. Besides, if only the beam index associated with the largest RSRP is reported and other beam indices are not reported, the UCI payload size of Case 1-4-1 can be greatly reduced. Furthermore, we can conclude from the simulation results that reporting Top-K (K=4 or 8) beams with larger RSRP from all beams in the measured beam set achieves the best trade-off between beam prediction accuracy and UCI payload overhead.

| - Simulation cases | - Number of beams in Set B | - Beam prediction accuracy for Top-1 beam | - Beam prediction accuracy with 1dB margin for Top-1 beam | - Average L1-RSRP difference of Top-1 predicted beam | - Total UCI payload size | - Reporting overhead reduction |
| --- | --- | --- | --- | --- | --- | --- |
| - Baseline | - 4 | - 24.29 % | - 40.29 % | - 2.381 | - 35 | - 0 |
| - Case 1-4-1: AI method when Set B equals to Set C | - ¦Set B¦=8 with all CRI report | - 73.79 % | - 89.90 % | - 0.511 | - 59 | - -68.5% |
|  | - ¦Set B¦=8 with one CRI report | - / |  |  | - 38 | - -8.6% |
|  | - ¦Set B¦=16 with all CRI report | - 91.52 % | - 99.18 % | - 0.044 | - 131 | - -274.3% |
|  | - ¦Set B¦=16 with one CRI report | - / |  |  | - 71 | - -102.9% |
| - Case 1-4-2: AI method when Set B is a subset of Set C (¦Set C¦=16) | - Set B = Top-4 of Set C | - 83.18 % | - 96.15 % | - 0.157 | - 35 | - 0 |
|  | - Set B = Top-8 of Set C | - 90.61 % | - 98.48 % | - 0.072 | - 67 | - -91.4% |

Reporting Top-K (K=4 or 8) beams with larger RSRPs from all beams in the measured beam set achieves a good trade-off between beam prediction accuracy and UCI payload overhead.

If Set B equals to the measured beam set (i.e., Set C), the total UCI payload size can be greatly reduced by reporting all measured RSRPs in Set B and the beam index associated with the largest RSRP.

Evaluation for AI/ML model generalization

In RAN1#112, the following working assumption about generalization evaluation was confirmed with fine tuning being optionally considered.

Agreement: The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

Besides, a detailed agreement was reached with more examples being provided for generalization performance verification.

Agreement: For generalization performance verification, consider the following - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - e.g., same deployment, different cells with different configuration/assumption - e.g., gNB height and UE height - FFS: e.g., Carrier frequencies - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations (parameters and settings) are not precluded and can be reported by companies.

In this section, to evaluate the AI/ML model generalization capability, different number of beams in Set B, different Set B patterns, different Rx beam assumptions, different gNB settings, different number of UE Rx beams and different UE distributions are considered as follows.

Generalization on different number of beams in Set B

To evaluate the AI/ML model generalization capability on different number of beams in Set B, mixed data samples generated under <T8 R1>, <T16 R1>, and <T32 R1> cases are used as AI/ML model inputs to predict quality of all transmit beams. Besides, beam prediction performance of corresponding <T8 R1>, <T16 R1>, and <T32 R1> cases are also provided here for comparison.

- Case 1-4-1-1: Both training and testing with samples of mixed data samples, where the mixed data samples generated under <T8 R1>, <T16 R1>, and <T32 R1> cases are used as AI/ML model inputs to predict RSRPs of all Tx beams. The Rx beam is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure as above.

- Case 1-4-1-2: In the <T8 R1> case for Tx beam prediction, 8 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure. In this case, RSRPs of selected Tx beams (8x1=8 beams) are used as AI/ML model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI/ML model outputs.

- Case 1-4-1-3: In the <T16 R1> case for Tx beam prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure. In this case, RSRPs of selected beams (16x1=16 beams) are used as AI/ML model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI/ML model outputs.

- Case 1-4-1-4: In the <T32 R1> case for Tx beam prediction, 32 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure. In this case, RSRPs of selected beams (32x1=32 beams) are used as AI/ML model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI/ML model outputs.

Detailed simulation results of the above four cases are provided in the result collection excel [2]. Some of the simulation results are also provided in Table VII for reference. It can be observed that case 1-4-1-4 (i.e., <T32 R1>) outperforms all other cases in terms of beam prediction accuracy. Definitely, more beam measurement input to the AI/ML model will generate better beam prediction performance. For generalization evaluation, case 1-4-1-1 that AI/ML model training and testing are performed in mixed datasets of <T8 R1>, <T16 R1>, and <T32 R1> with different beam constructions achieves a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>. Additionally, the case of AI/ML model generalization can obtain significant performance gain over the non-AI case.

Table VII. Simulation results for generalization performance evaluation on different number of beams in set B

| Cases |  | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams |
| --- | --- | --- | --- | --- | --- |
| Case 1-4-1-1 | Non-AI | 3.405 | 44.17% | 30.03% | 30.03% |
|  | AI | 0.314 | 94.73% | 84.87% | 98.92% |
| Case 1-4-1-2 | Non-AI | 6.9198 | 19.80% | 12.60% | 12.60% |
|  | AI | 0.832 | 86.55% | 72.60% | 96.85% |
| Case 1-4-1-3 | Non-AI | 3.391 | 40.45% | 25.95% | 25.95% |
|  | AI | 0.096 | 98.10% | 88.25% | 99.75% |
| Case 1-4-1-4 | Non-AI | 0.627 | 72.25% | 51.55% | 51.55% |
|  | AI | 0.041 | 99.40% | 92.45% | 100.00% |

The case that AI/ML model training and testing are performed in mixed datasets of <T8 R1>, <T16 R1>, and <T32 R1> with different beam constructions achieves a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>.

The AI/ML model can provide a good generalization capability on different number of beams in Set B for Tx beam prediction, when the training dataset includes the data from the same setting of testing data.

Generalization on different Set B patterns

In order to evaluate the AI model’s generalization capability on different Set B patterns, Set B pattern 1, Set B pattern 2, and Set B pattern 3 showed in Figure 2-2 are used to generate data samples. DL Tx beam prediction based on the optimal Rx beam (i.e., Rx beam assumption 3) are conducted by the following four different variations of <T16 R1>.

- Case 1-4-2-1: The training data samples of the AI/ML model consists of mixed data samples, which are generated under Set B pattern 1, Set B pattern 2, and Set B pattern 3. The testing data samples of the AI/ML model are also the mixed data samples.

- Case 1-4-2-2: The training data samples of the AI/ML model consists of mixed data samples, which are generated under Set B pattern 1, Set B pattern 2 and Set B pattern 3. The testing data samples of the AI/ML model are generated under Set B pattern 1.

- Case 1-4-2-3: The training data samples of the AI/ML model are generated under Set B pattern 1. The testing data samples of the AI/ML model are generated under Set B pattern 2.

- Case 1-4-2-4: The training data samples of the AI/ML model are generated under Set B pattern 1. the testing data samples of the AI/ML model are generated under Set B pattern 1.

Detailed simulation results of the above four cases are provided in the result collection excel [2]. Some of the simulation results are also provided in Table VIII for reference. Accordingly, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed. However, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the AI model’s training phase and testing phase, the AI method can obtain significant performance gains over the non-AI case.

Table VIII. Simulation results for generalization performance evaluation on different set B patterns

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-2-1 | Non-AI | 1.826 46.02 % 28.70 % 28.70 % 1.831 |  |  |  |  |
|  | AI | 0.101 97.75 % 86.74 % 99.56 % 0.222 |  |  |  |  |
| Case 1-4-2-2 | Non-AI | 2.396 39.39 % 24.93 % 24.93 % 2.450 |  |  |  |  |
|  | AI | 0.073 98.29 % 88.10 % 99.71 % 0.225 |  |  |  |  |
| Case 1-4-2-3 | Non-AI | 1.068 58.97 % 38.21 % 38.21 % 10.342 |  |  |  |  |
|  | AI | 11.818 13.33 % 8.70 % 29.52 % 5.322 |  |  |  |  |
| Case 1-4-2-4 | Non-AI | 2.351 40.80 % 24.95 % 24.95 % 2.361 |  |  |  |  |
|  | AI | 0.044 99.12 % 91.33 % 99.79 % 0.144 |  |  |  |  |

For the AI/ML model generalization on different Set B patterns, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed.

For the AI/ML model generalization on different Set B patterns, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the AI/ML model training phase and testing phase, the AI method obtains significant performance gains over the non-AI case.

The AI/ML model can provide a good generalization capability on different Set B patterns for Tx beam prediction when the training dataset includes the data from the same setting of testing data.

Generalization on different Rx beam assumptions

To evaluate the AI model’s generalization capability on different Rx beam assumptions, Set B pattern 1 showed in Figure 2-2 is used to generate data samples. With different Rx beam assumptions during AI/ML model’s training phase and testing phase, the DL Tx beam prediction is conducted with the following 8 different variations of <T16 R1> being considered for comparison.

- Case 1-4-3-1: The training data samples of the AI/ML model are mixed data samples, which are generated based on the optimal Rx beam (i.e., Rx beam assumption 3) for each model sample or a fixed Rx beam (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI/ML model are also the mixed data samples as mentioned above.

- Case 1-4-3-2: The training data samples of the AI/ML model are the mixed data samples, which are generated based on the optimal Rx beam (i.e., Rx beam assumption 3) for each AI/ML model sample or a fixed Rx beam (i.e., Rx beam assumption 4) for all AI/ML model samples. The testing data samples of the AI/ML model are generated based on the optimal Rx beam for each model sample.

- Case 1-4-3-3: The training data samples of the AI/ML model are generated based on the optimal Rx beam (i.e., Rx beam assumption 3) for each AI/ML model sample. The testing data samples of the AI/ML model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples.

- Case 1-4-3-4: The training data samples of the AI/ML model are generated based on optimal Rx beam (i.e., Rx beam assumption 3) for each AI/ML model sample. The testing data samples of the AI/ML model are generated based on optimal Rx beam (i.e., Rx beam assumption 3) for each AI/ML model sample.

- Case 1-4-3-5: The training data samples of the AI/ML model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples. The testing data samples of the AI/ML model are generated based on the first Rx beam on the second UE panel (i.e., Rx beam assumption 4) for all model samples.

- Case 1-4-3-6: The training data samples of the AI/ML model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples. The testing data samples of the AI/ML model are generated based on the fourth Rx beam on the second UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples.

- Case 1-4-3-7: The training data samples of the AI/ML model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples. The testing data samples of the AI/ML model are generated based on the second Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples.

- Case 1-4-3-8: The training data samples of the AI/ML model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples. The testing data samples of the AI/ML model are generated based on the fourth Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all AI/ML model samples.

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). Detailed simulation results of the first four cases above are provided in the result collection excel [2]. Some of the simulation results are also provided in Table IX for reference. Accordingly, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions. Therefore, we conclude that the AI/ML model can provide good generalization capability on different Rx beam assumptions.

Table IX.  Simulation results for generalization performance evaluation on different Rx beam assumptions

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-3-1 | Non-AI | 2.183 41.43 % 25.05 % 25.05 % 2.181 |  |  |  |  |
|  | AI | 0.107 97.33 % 86.50 % 99.26 % 0.203 |  |  |  |  |
| Case 1-4-3-2 | Non-AI | 2.422 39.92 % 24.91 % 24.91 % 2.435 |  |  |  |  |
|  | AI | 0.054 98.80 % 90.46 % 99.83 % 0.170 |  |  |  |  |
| Case 1-4-3-3 | Non-AI | 2.138 42.74 % 24.11 % 24.11 % 2.161 |  |  |  |  |
|  | AI | 0.230 94.46 % 80.91 % 97.50 % 0.276 |  |  |  |  |
| Case 1-4-3-4 | Non-AI | 2.395 40.46 % 24.90 % 24.90 % 2.383 |  |  |  |  |
|  | AI | 0.053 98.90 % 90.88 % 99.68 % 0.145 |  |  |  |  |
| Case 1-4-3-5 | Non-AI | 2.069 44.27 % 25.89 % 25.89 % 2.073 |  |  |  |  |
|  | AI | 0.133 96.32 % 83.05 % 98.72 % 0.245 |  |  |  |  |
| Case 1-4-3-6 | Non-AI | 2.084 43.58 % 25.01 % 25.01 % 2.080 |  |  |  |  |
|  | AI | 0.123 97.01 % 83.10 % 98.86 % 0.259 |  |  |  |  |
| Case 1-4-3-7 | Non-AI | 2.081 44.32 % 25.50 % 25.50 % 2.087 |  |  |  |  |
|  | AI | 0.179 95.50 % 80.53 % 98.36 % 0.282 |  |  |  |  |
| Case 1-4-3-8 | Non-AI | 2.089 43.31 % 25.20 % 25.20 % 2.088 |  |  |  |  |
|  | AI | 0.150 96.32 % 82.88 % 98.51 % 0.269 |  |  |  |  |

For AI/ML model generalization on different Rx beam assumptions for Tx beam prediction, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions on AI/ML model training and AI/ML model testing.

The AI/ML model can provide good generalization capability on different Rx beam assumptions for Tx beam prediction.

Generalization on different gNB settings

To evaluate the AI model’s generalization capability on different gNB settings, two different gNB beam structures are used to generate data samples. For the first gNB beam structure, there are 16 beams in horizontal and 4 beams in vertical, denoted as H16V4 for simplicity. For the second gNB beam structure, there are 8 beams in horizontal and 8 beams in vertical, denoted as H8V8 for simplicity. With different gNB beam structures during the AI/ML model training phase and testing phase, the DL Tx beam prediction is conducted and the following 4 variations of <T16 R1> design with the optimal Rx beam (i.e., Rx beam assumption 3) are considered.

- Case 1-4-4-1: The training data samples of the AI/ML model are mixed data samples, which are generated based on the two different gNB beam layouts of H16V4 and H8V8. The testing data samples of the AI/ML model are also the mixed data samples as mentioned above.

- Case 1-4-4-2: The training data samples of the AI/ML model are mixed data samples, which are generated based on the two different gNB beam structures of H16V4 and H8V8. The testing data samples of the AI/ML model are generated based on the gNB beam structure of H16V4.

- Case 1-4-4-3: The training data samples of the AI/ML model are generated based on the gNB beam structures of H16V4. The testing data samples of the AI/ML model are generated based on the gNB beam structure of H8V8.

- Case 1-4-4-4: The training data samples of the AI/ML model are generated based on the gNB beam structures of H16V4. The testing data samples of the AI/ML model are generated based on the gNB beam structure of H16V4.

Detailed simulation results of the above four cases are provided in the result collection excel [2]. Some of the simulation results are also provided in Table X for reference. Accordingly, a significant performance loss is observed if the training data samples and testing data samples are generated under different gNB beam structures. Compared with the case that the same gNB beam structure is used for the AI/ML model training and testing, only marginal performance loss is observed if the training data samples of the AI/ML model are mixed data samples.

Table X.  Simulation results for generalization performance evaluation on different gNB setting

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-4-1 | Non-AI | 1.994 43.09 % 24.80 % 24.80 % 2.000 |  |  |  |  |
|  | AI | 0.086 97.73 % 87.31 % 99.52 % 0.230 |  |  |  |  |
| Case 1-4-4-2 | Non-AI | 2.380 40.91 % 24.82 % 24.82 % 2.369 |  |  |  |  |
|  | AI | 0.102 97.43 % 86.30 % 99.37 % 0.261 |  |  |  |  |
| Case 1-4-4-3 | Non-AI | 1.567 47.52 % 24.90 % 24.90 % 3.528 |  |  |  |  |
|  | AI | 11.693 34.10 % 24.74 % 72.93 % 1.819 |  |  |  |  |
| Case 1-4-4-4 | Non-AI | 2.390 39.41 % 24.13 % 24.13 % 2.394 |  |  |  |  |
|  | AI | 0.058 98.78 % 90.29 % 99.60 % 0.153 |  |  |  |  |

For AI/ML model generalization on different gNB settings, a significant performance loss is observed if the training data samples and the testing data samples are generated under different gNB beam structures.

Compared with the case that the same gNB beam structure is used for generating the training data samples and the testing data samples, only marginal performance loss is observed if the training data samples of AI/ML model inclue mixed data samples from both gNB beam layouts.

Generalization on different number of UE Rx beams

To evaluate the AI model's generalization capability on different number of UE Rx beams, two UE antenna configurations with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) and (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2) are used to generate data samples corresponding to 8 Rx beams and 4 Rx beams, respectively. With different number of UE Rx beams during the AI/ML model training phase and testing phase, the beam pair prediction is conducted with the following 5 different variations of <T16 R8> or <T16 R4> being evaluated for comparison.

- Case 1-4-5-1: The training data samples of the AI/ML model are mixed data samples, which are generated based on 8 Rx beams and 4 Rx beams. The testing data samples of the AI/ML model are also the mixed data samples as mentioned above.

- Case 1-4-5-2: The training data samples of the AI/ML model are mixed data samples, which are generated based on 8 Rx beams and 4 Rx beams. The testing data samples of the AI/ML model are generated based on 8 Rx beams.

- Case 1-4-5-3: The training data samples of the AI/ML model are generated based on 8 Rx beams. The testing data samples of the AI/ML model are generated based on 4 Rx beams.

- Case 1-4-5-4: The training data samples of the AI/ML model are generated based on 8 Rx beams. The testing data samples of the AI/ML model are generated based on 8 Rx beams.

- Case 1-4-5-5: The training data samples of the AI/ML model are generated based on 4 Rx beams. The testing data samples of the AI/ML model are generated based on 4 Rx beams.

Detailed simulation results of the above four cases are provided in the result collection excel [3]. Some of the simulation results are also provided in Table XI for reference. Accordingly, compared with the case that the training data samples and testing data samples are generated based on the same number of UE Rx beams, only marginal performance loss is observed for the case that different number of UE Rx beams are used for model training and testing. Therefore, we can conclude that the AI/ML model can provide good generalization capability on different number of UE Rx beams for beam pair prediction.

Table XI.  Simulation results for generalization performance evaluation on different number of UE Rx beams

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-5-1 | Non-AI | 2.018 43.14 % 24.42 % 24.42 % 2.025 |  |  |  |  |
|  | AI | 0.097 97.43 % 64.06 % 97.52 % 0.247 |  |  |  |  |
| Case 1-4-5-2 | Non-AI | 2.021 42.93 % 24.84 % 24.84 % 2.036 |  |  |  |  |
|  | AI | 0.121 96.70 % 82.48 % 98.63 % 0.304 |  |  |  |  |
| Case 1-4-5-3 | Non-AI | 2.060 43.33 % 25.05 % 25.05 % 2.129 |  |  |  |  |
|  | AI | 0.126 94.44 % 44.72 % 95.01 % 0.289 |  |  |  |  |
| Case 1-4-5-4 | Non-AI | 2.043 43.05 % 24.36 % 24.36 % 2.058 |  |  |  |  |
|  | AI | 0.094 97.49 % 84.86 % 98.86 % 0.242 |  |  |  |  |
| Case 1-4-5-5 | Non-AI | 1.974 43.75 % 24.63 % 24.63 % 1.998 |  |  |  |  |
|  | AI | 0.078 98.08 % 43.10 % 96.82 % 0.195 |  |  |  |  |

For AI/ML model generalization on different number of UE Rx beams for beam pair prediction, compared with the case that the training data samples and testing data samples are generated based on the same number of UE Rx beams, only marginal performance loss is observed for the case that different number of UE Rx beams are used for AI/ML model training and testing.

The AI/ML model can provide good generalization capability on different number of UE Rx beams for beam pair prediction.

Generalization on different UE distribution

To encourage companies to evaluate the impact of UE distribution, the following conclusion was obtained in RAN1#112bis-e. In this section, we evaluation the performance for BM Case-1 with different UE distribution assumptions for both the non-generalization and generalization cases.

Conclusion: It is optional to evaluate and compare the performance for BM Case-1 with different UE distribution assumptions: - Option 1: 80% indoor, 20% outdoor as in TR 38.901 - Option 2: 100% outdoor

In order to evaluate the AI/ML model’s generalization capability on different UE distributions, two UE distribution options are considered to generate the data samples, that is Option 1 with 80% indoor, 20% outdoor as in TR 38.901 and Option 2 with 100% outdoor. The Rx beam assumption for measurement is selected as the best Rx beam for each Tx beam within Set B (i.e., Rx beam assumption 5). To be more specific, the DL Tx beam prediction is conducted by emulating the following six variations of <T16 R1>.

- Case 1-4-6-1: The training data samples of the AI/ML model are generated under UE distribution of 20% outdoor and 80% indoor. The testing data samples of the AI/ML model are generated under UE distribution of 20% outdoor and 80% indoor.

- Case 1-4-6-2: The training data samples of the AI/ML model are generated under UE distribution of 100% outdoor. The testing data samples of the AI/ML model are generated under UE distribution of 100% outdoor.

- Case 1-4-6-3: The training data samples of the AI/ML model are generated under UE distribution of 20% outdoor and 80% indoor. The testing data samples of the AI/ML model are generated under UE distribution of 100% outdoor.

- Case 1-4-6-4: The training data samples of the AI/ML model consists of mixed data samples, which are generated under both the UE distributions. The testing data samples of the AI/ML model are generated under UE distribution of 20% outdoor and 80% indoor.

- Case 1-4-6-5: The training data samples of the AI/ML model consists of mixed data samples, which are generated under both the UE distributions. The testing data samples of the AI/ML model are generated under UE distribution of 100% outdoor.

- Case 1-4-6-6: The training data samples of the AI/ML model consists of mixed data samples, which are generated both the UE distributions. The testing data samples of the AI/ML model are also the mixed data samples.

Detailed simulation results of the above six cases are provided in the result collection excel [2]. Some of the simulation results are also provided in the following Table for reference. As observed, the AI/ML model can achieve better beam prediction performance with UE distribution of 100% outdoor than with UE distribution of 80% indoor and 20% outdoor. Besides, compared with the UE distribution of 100% outdoor, only marginal performance loss is observed if training data samples of the AI/ML model consists of mixed data samples. Therefore, we can conclude that the AI/ML model can provide good generalization capability on different UE distributions.

Simulation results for generalization performance evaluation on different UE distributions

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-6-1 | Non-AI | 3.304 36.32% 24.98% 24.98% 2.230 |  |  |  |  |
|  | AI | 0.502 90.74% 81.32% 99.02% 0.232 |  |  |  |  |
| Case 1-4-6-2 | Non-AI | 3.668 35.62% 24.94% 24.94% 3.115 |  |  |  |  |
|  | AI | 0.268 95.66% 88.82% 99.38% 0.202 |  |  |  |  |
| Case 1-4-6-3 | Non-AI | 3.304 36.32% 24.98% 24.98% 2.230 |  |  |  |  |
|  | AI | 0.686 87.44% 75.56% 98.42% 0.304 |  |  |  |  |
| Case 1-4-6-4 | Non-AI | 3.304 36.32% 24.98% 24.98% 2.230 |  |  |  |  |
|  | AI | 0.571 89.74% 79.98% 98.88% 0.267 |  |  |  |  |
| Case 1-4-6-5 | Non-AI | 3.668 35.62% 24.94% 24.94% 3.115 |  |  |  |  |
|  | AI | 0.321 94.92% 88.38% 99.16% 0.189 |  |  |  |  |
| Case 1-4-6-6 | Non-AI | 3.547 36.30% 25.72% 25.72% 2.718 |  |  |  |  |
|  | AI | 0.457 92.32% 83.52% 99.08% 0.235 |  |  |  |  |

The AI/ML model can achieve a better beam prediction performance with UE distribution of 100% outdoor than with UE distribution of 80% indoor and 20% outdoor.

The AI/ML model can provide a good generalization capability on different UE distributions.

Analysis on the predicted L1-RSRP difference

In RAN1#112bis-e, there was a debate on whether/how to evaluate the feasibility on the performance metric(s) for AI/ML model monitoring and the following proposal was initial discussed.

Proposal: Strive to evaluate the feasibility on the performance metric(s) for AI/ML model monitoring, at least for the following: - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - FFS on The L1-RSRP difference definition in Alt 4. - Note: No potential feasibility issues have been identified for Alt1 and Alt2

For Alt.4, the controversial part is whether the predicted L1-RSRP difference can be adopted as the performance metric(s) for AI/ML model monitoring. As can be observed from the simulation results in this section (from Table II to Table XI), the performance of predicted L1-RSRP difference shows strong correction with other intermediate KPI (such as the beam prediction accuracy of Top-1/K beam(s)). Although there exists corner cases that lower beam prediction accuracy is associated with lower predicted L1-RSRP difference if the beam prediction accuracy is very close. The predicted L1-RSRP difference is consistent with the beam prediction accuracy in most cases and thus can well reflect whether the beam is accurately predicted or not. Besides, the mechanism of performance monitoring with Alt.1 and Alt.4 would be similar. Specifically, for Alt.1, a pre-defined threshold can be set for performance monitoring, where the model/functionality failure occurs if the beam prediction accuracy is lower than the pre-defined threshold. With Alt.4, a similar threshold-based performance monitoring mechanism can be used due to the strong correlation between Alt.4 and the beam prediction accuracy in Alt.1. Thus, Alt.4 is feasible to be adopted as the performance metric for AI/ML model monitoring.

The performance of predicted L1-RSRP difference shows strong correction with other intermediate KPI (such as the beam prediction accuracy of Top-1/K beam(s)).

Alt.4 (i.e., the L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP) is feasible to be adopted as the performance metric for AI/ML model monitoring.

Evaluation for temporal beam prediction

General description:

In high mobility environment, the task of fast beam tracking is more challenging. A typical way for beam tracking is periodic measurement and reporting, which introduces significant overhead that scales with the number of beams. Therefore, we utilize AI/ML technology to realize temporal beam prediction in this section. Specifically, recurrent neutral network is leveraged to implement the temporal beam prediction with assumption of a simple UE mobility pattern with random direction straight-line trajectories. The measured RSRPs of all or partial DL beams (or beam pairs) of some past time instances are used as the AI/ML input and the predicted RSRPs of all DL beams (or beam pairs) of multiple future time instances are used as the AI/ML output. That is, the beam set for measurement and the beam set for prediction are the same, or the beam set for measurement is a subset of the beam set for prediction. Besides, for each time instance within the prediction window, Top-K beams predicted by the adopted AI/ML model may or may not be used for a refined beam sweeping.

Figure 3-1. Illustration of AI/ML based temporal beam prediction.

Dataset construction:

The dataset for AI/ML model training and evaluation is generated according to the agreed parameters in RAN1#109-e, which is summarized in Table I in the appendix. Specifically, one UE per sector per drop is randomly generated with a total of 20 simulation drops. For antenna configuration, the baseline agreed in RAN#110bis-e is adopted directly, where a single panel with (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ and 2 panels with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) are used by the gNB and UE, respectively. Besides, the gNB and UE are equipped with 32 Tx beams in total and 4 Rx beams per panel, respectively. In addition, more simulation assumptions for the temporal beam prediction are summarized in Table XII.

For temporal beam prediction, the random direction straight-line trajectories in Option #4 is adopted for modelling UE mobility, which means that multiple UEs are randomly dropped in multiple cells and move in a straight-line trajectory along an initially random direction with fixed speed. More details about Option #4 is provided in the appendix for reference. Compared with other UE trajectory options, Option #4 is simpler and beneficial for AI/ML model generalization. The modelling of UE orientation and rotation may cause the adopted AI/ML model to be overly complex with excessive training overhead. Thus, it has not been considered for the time being for simplicity. Additionally, spatial consistency procedure B in TR38.901 is modeled along the mobility trajectory to ensure that propagation parameters maintain continuity across multiple realizations.

Table XII. Simulation assumptions for the temporal beam prediction

| UE distribution | 1 UE per sector per drop with a total of 21 sectors and 20 simulation drops |
| --- | --- |
| TTI number per simulation drop | 150000 |
| BS Antenna Configuration | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| UE Antenna Configuration | Two panel: (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) |
| Spatial consistency | Procedure B in TR38.901 |
| UE Speed | 30 km/h, 60 km/h, or 90 km/h |
| UE trajectory model | Option #4: Random direction straight-line trajectories |
| UE orientation | Initially random and keeps fixed |
| UE rotation speed | 0 |

AI/ML model and training methodology:

For cross-checking purpose, the adopted AI/ML model for temporal beam prediction is captured in Table XIII. The LSTM network is used, where the AI/ML model input is measured RSRPs of all or partial DL beams (or beam pairs) of past 5 time instances, and the AI/ML model output is predicted RSRPs of all DL beams (or beam pairs) of future 5 time instances. The sampling interval between two adjacent time instances is 160ms. For the evaluation of generalization performance, data samples generated by mixed UE speeds of 30km/h and 60km/h are used for AI/ML model training and testing. The corresponding dataset size is 60,000 and 180,000 for the case of UE speed of 30/60km/h and for the case of mixed UE speeds of 30km/h and 60km/h, respectively. Besides, AI/ML model complexity is also taken into consideration, which includes the AI/ML model size and the amount of computation (measured in FLOPs). Among them, the former describes the required memory size and the latter describes the required computing complexity, which directly impacts the inference time of the model.

Table XIII. Description of the adopted AI/ML model for temporal beam prediction

| Model architecture | LSTM, FC layer |
| --- | --- |
| Number of layers | 4 |
| Model input | Measured RSRPs of past time instances |
| Model output | Predicted RSRPs of future time instances |
| Observation window length | 5 |
| Prediction window length | 5 |
| Dataset size | For performance comparison of fixed Set B and variable Set B across training and inference: 140000 |
|  | For performance comparison of Tx-Rx beam pair prediction and Tx beam prediction: 140000 |
|  | For evaluation of generalization: 60,000 (for a single UE speed of 30km/h or 60km/h or 90km/h) 180,000 (for mixed UE speeds of 30km/h, 60km/h, and 90km/h) |
| Percentage of training/validity/testing samples | 90%, 5%, 5% |
| Model size | 2.758M |
| FLOPs | 7.250M |

Performance KPIs:

To evaluate the performance of AI/ML in temporal beam prediction, we provide our preliminary simulation results in terms of beam prediction accuracy for Top-K beams. All adopted KPIs are appropriately defined according to the agreement achieved in RAN1#109e. Particularly, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams (unless otherwise mentioned). The beam prediction accuracy (%) for Top-K beams is defined as the percentage that the Top-1 genie-aided beam is one of the Top-K predicted beams. As with the spatial domain beam prediction, a second stage beam sweeping over these Top-K predicted beams is not considered for RS overhead counting. Otherwise, the RS overhead for the second stage beam sweeping would be the product of the number of UEs per sector and the number of Top-K beams. Thus, the RS overhead of the temporal beam prediction method may be even higher than that of a cell-specific exhaustive sweeping over all beams.

Performance comparison of fixed Set B and variable Set B

To study the selection of Set B of beams, performance comparison of fixed Set B and variable Set B during the observation window is conducted, where RSRPs of all measured beams in Set B are used as AI/ML model inputs. In particular, as with the spatial domain beam prediction, Set C of beams can be measured and Top-4, Top-2, or Top-1 beams of Set C are used as AI/ML model inputs (i.e., Set B) to reduce the reporting overhead for a NW-side model. More specifically, the following six different cases are considered for comparison.

- Case 2-1-1: Set B is changed following a set of pre-configured patterns (total 4 beam patterns)

- Case 2-1-2: Set B is changed among pre-configured patterns (one beam pattern is randomly selected from 4 beam patterns in each past time instance)

- Case 2-1-3: Set B consists of Top-4 beams of the beam set for measurement in each past time instance, on the basis of Case 2-1-1

- Case 2-1-4: Set B consists of Top-2 beams of the beam set for measurement in each past time instance, on the basis of Case 2-1-1

- Case 2-1-5: Set B consists of Top-1 beams of the beam set for measurement in each past time instance, on the basis of Case 2-1-1

- Case 2-1-6: Set B is fixed during the observation window

Table XIV.  Performance comparison of fixed Set B and variable Set B across observation slots (UE speed: 30km/h)

| Cases |  | Top-K | Top-K Prediction Accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-1-1 | Non-AI | 4 | 65.09 | 64.46 | 63.83 | 63.27 | 61.66 |
|  | AI | 4 | 98.05 | 98.27 | 98.10 | 98.03 | 97.53 |
| Case 2-1-2 | Non-AI | 4 | 62.83 | 62.29 | 61.78 | 60.75 | 60.39 |
|  | AI | 4 | 95.49 | 94.95 | 93.92 | 93.24 | 92.37 |
| Case 2-1-3 | Non-AI | 4 | 65.25 | 64.61 | 63.65 | 62.47 | 61.78 |
|  | AI | 4 | 97.87 | 97.97 | 97.79 | 97.39 | 96.82 |
| Case 2-1-4 | Non-AI | 4 | 62.10 | 60.72 | 59.19 | 57.92 | 56.51 |
|  | AI | 4 | 96.78 | 96.22 | 96.29 | 95.34 | 94.51 |
| Case 2-1-5 | Non-AI | 4 | 53.80 | 51.60 | 49.93 | 48.24 | 46.63 |
|  | AI | 4 | 93.06 | 92.18 | 91.38 | 90.76 | 89.47 |
| Case 2-1-6 | Non-AI | 4 | 25.82 | 26.09 | 25.75 | 25.66 | 25.45 |
|  | AI | 4 | 98.02 | 98.07 | 97.69 | 97.73 | 96.67 |

The simulation results are shown in Table XIV. As observed, a significant performance gain over the non-AI method can be achieved by the AI based temporal beam prediction in terms of beam prediction accuracy for Top-4 beams. Additionally, compared with the case that Set B is fixed across training and inference (i.e., Case 2-1-6), a slight performance gain is observed if Set B is changed following a set of pre-configured patterns (i.e., Case 2-1-1).  Compared with the case that the beam pattern of Set B is randomly selected from 4 beam patterns in each past time instance (i.e., Case 2-1-2), a slight performance gain is observed if Set B is changed following a set of pre-configured patterns (i.e., Case 2-1-1). Besides, as can be seen from the simulation results of Case 2-1-3, Case 2-1-4, and Case 2-1-5, only a little performance loss is observed if Top-4 or Top-2 beams of all measured beams are used as AI/ML model inputs. Therefore, it can be concluded that it is better to take partial beams from the beam set for measurement as the AI/ML model input to reduce the beam reporting overhead for a NW-side model.

In BM-Case2, compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, compared with the case that the Set B pattern is randomly selected from a total of 4 beam patterns in each past time instance, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, if Top-4 or Top-2 beams of all measured beams are used as AI/ML model inputs in each past time instance, only a little performance loss is observed compared with the case that all measured beams are used as AI/ML model input.

To reduce UE report overhead in BM-Case 2 for a NW-side model, it is better to take partial beams from the beam set for measurement as AI/ML model input.

Performance comparison of Tx-Rx beam pair prediction and Tx beam prediction

To compare the performance of Tx-Rx beam pair prediction and Tx beam prediction based on the optimal or fixed Rx beam (i.e., Rx beam assumption 3 or 4), four different cases of AI/ML model inputs for each past time instance and outputs for each future time instance are considered as follows.

- Case 2-2-1: RSRPs of all Tx beams of 5 past time instances are used as the AI/ML model input and RSRPs of all Tx beams of 5 future time instances are used as the AI/ML model output, where the optimal Rx beam (i.e., Rx beam assumption 3) is assumed for measurement

- Case 2-2-2: RSRPs of all Tx-Rx beam pairs of 5 past time instances are used as the AI/ML model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as the AI/ML model output, where the optimal Rx beam (i.e., Rx beam assumption 3) is assumed for measurement

- Case 2-2-3: RSRPs of all Tx beams of 5 past time instances are used as the AI/ML model input and RSRPs of all Tx beams of 5 future time instances are used as the AI/ML model output, where the specific (i.e., first) Rx beam (i.e., Rx beam assumption 4) is assumed for measurement

- Case 2-2-4: RSRPs of all Tx-Rx beam pairs of 5 past time instances are used as AI/ML model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as AI/ML model output, where the specific (i.e., first) Rx beam (i.e., Rx beam assumption 4) is assumed for measurement

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). Detailed simulation results of the first two cases above are provided in the result collection excel [4]. Some of the simulation results are also provided in Table XV for reference. As observed, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction under the optimal Rx beam or the fixed Rx beam. In addition, compared with the case of the fixed Rx beam, a slight performance gain is observed if the Tx beam prediction or Tx-Rx beam pair prediction is performed under the optimal Rx beam.

Table XV.  Performance comparison of Tx-Rx beam pair prediction and Tx beam prediction (UE speed: 30km/h)

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-2-1 | Non-AI | 4 | 97.06 | 95.48 | 94.05 | 92.19 | 90.37 |
|  | AI | 4 | 99.17 | 98.99 | 98.85 | 98.50 | 98.25 |
| Case 2-2-2 | Non-AI | 4 | 90.35 | 86.07 | 83.22 | 80.67 | 77.77 |
|  | AI | 4 | 96.70 | 96.74 | 96.50 | 96.26 | 94.96 |
| Case 2-2-3 | Non-AI | 4 | 96.61 | 94.98 | 93.13 | 91.29 | 89.35 |
|  | AI | 4 | 98.62 | 98.32 | 98.13 | 98.09 | 97.61 |
| Case 2-2-4 | Non-AI | 4 | 21.42 | 21.05 | 20.54 | 20.13 | 19.85 |
|  | AI | 4 | 95.28 | 95.88 | 95.28 | 94.91 | 93.47 |

In BM-Case2, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction with the assumption of the optimal Rx beam or the specific Rx beam being used for measurement.

Evaluation for AI/ML model generalization

To evaluate the generalization performance of AI model, RSRPs of all measured beam pairs of past time instances are used as AI/ML model input, and the beam set for measurement and the beam set for prediction are the same. Datasets for training and testing are generated with a UE speed of 30km/h or 60km/h or with mixed UE speeds. According to the type of datasets for training and testing, the following seven different cases are considered for comparison.

- Case 2-3-1: Training with samples of mixed UE speeds, testing with samples of UE speed of 60km/h

- Case 2-3-2: Training with samples of mixed UE speeds, testing with samples of UE speed of 30km/h

- Case 2-3-3: Both training and testing with samples of mixed UE speeds

- Case 2-3-4: Both training and testing with samples of UE speed of 60km/h

- Case 2-3-5: Both training and testing with samples of UE speed of 30km/h

Detailed simulation results for the 30km/h and 60km/h cases above are provided in the result collection excel [5]. Some of the simulation results are also provided in Table XVI for reference. As can be seen from the simulation results in Table XVI, if the AI/ML model is trained with samples of a single UE speed or mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 60km/h. The AI based method in case 2-3-1, case 2-3-2, case 2-3-3, and case 2-3-4 can also achieve a better performance than that of the non-AI method in scenarios with mixed UE speeds. Additionally, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, average performance is observed if both the training data samples and the testing data samples of AI/ML model are the mixed data samples.

Table XVI.  Evaluation of generalization performance on different UE speeds

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-3-1 | Non-AI | 4 | 93.76 | 88.64 | 83.34 | 78.49 | 74.24 |
|  | AI | 4 | 95.27 | 93.94 | 93.16 | 92.27 | 90.67 |
| Case 2-3-2 | Non-AI | 4 | 95.86 | 92.36 | 89.01 | 86.36 | 83.27 |
|  | AI | 4 | 96.69 | 96.03 | 95.70 | 95.14 | 93.47 |
| Case 2-3-3 | Non-AI | 4 | 93.79 | 88.58 | 83.99 | 79.76 | 75.62 |
|  | AI | 4 | 95.38 | 94.87 | 94.61 | 93.63 | 91.98 |
| Case 2-3-4 | Non-AI | 4 | 93.33 | 88.40 | 83.17 | 79.27 | 74.63 |
|  | AI | 4 | 95.37 | 95.33 | 95.47 | 95.27 | 93.50 |
| Case 2-3-5 | Non-AI | 4 | 96.60 | 93.50 | 90.53 | 86.47 | 83.67 |
|  | AI | 4 | 97.40 | 97.20 | 97.37 | 96.80 | 96.60 |

In BM-Case2, if the AI/ML model is trained with samples of mixed UE speeds, a minor performance loss is observed as the UE speed in testing samples increases from 30km/h to 60km/h.

In BM-Case2, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, a minor performance loss is observed if both the training data samples and the testing data samples of the AI/ML model are mixed data samples.

Conclusions

In this contribution, we discuss the potential specification impacts and enhancements for AI/ML based beam management. We have the following observations and proposals.

Observations:

The spatial-domain beam pair prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used.

- With the same sampling rate on the whole beam space, Case 1-1-2 (without down sampling in the Rx beam space) obtains a better performance than Case 1-1-1 (with down sampling in the Rx beam space).

For DL Tx beam prediction, compared with the Rx beam assumption that the best Rx beam searched for one Tx beam within Set B is used, a better beam prediction performance is achieved if the best Rx beam for the best Tx beam within Set B is used for measurement.

Among all Rx beam assumptions for Tx beam prediction, the best performance can be achieved by measurements of the best Rx beam for the best Tx beam within Set A (i.e., Case 1-2-3), which can be an upper bound on DL Tx beam prediction performance.

The realistic determination of quasi-optimal Rx beam causes a significant performance loss compared with that of the best Rx beam assumption, where the quasi-optimal Rx beam consists of X% (X=80 or 90) of “best” Rx beam by exhaustive Rx beam sweeping and (1-X) % of random Rx beams.

For DL Tx beam prediction based on a specific Rx beam assumption, Option 2a (i.e., measurements of specific Rx beam(s) per model input sample) and Option 2b (i.e., measurements of specific Rx beam(s) for all model input samples) show similar beam prediction accuracy.

For Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance is achieved with different uniform Set B patterns in the same sampling rate if AI/ML model generalization is not considered.

For DL Tx beam prediction, a high performance is achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%.

For DL Tx beam prediction, marginal performance loss is observed when the AI/ML model inputs are Top 8 beams or Top 4 beams from the 16 measured beams compared with all measured beams are used as AI/ML model input.

For Tx-Rx beam pair prediction, significant performance loss is observed when the AI/ML model inputs are Top 8 beams or Top 4 beams from the 128 measured beams.

Reporting Top-K (K=4 or 8) beams with larger RSRPs from all beams in the measured beam set achieves a good trade-off between beam prediction accuracy and UCI payload overhead.

If Set B equals to the measured beam set (i.e., Set C), the total UCI payload size can be greatly reduced by reporting all measured RSRPs in Set B and the beam index associated with the largest RSRP.

The case that AI/ML model training and testing are performed in mixed datasets of <T8 R1>, <T16 R1>, and <T32 R1> with different beam constructions achieves a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>.

The AI/ML model can provide a good generalization capability on different number of beams in Set B for Tx beam prediction, when the training dataset includes the data from the same setting of testing data.

For the AI/ML model generalization on different Set B patterns, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed.

For the AI/ML model generalization on different Set B patterns, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the AI/ML model training phase and testing phase, the AI method obtains significant performance gains over the non-AI case.

The AI/ML model can provide a good generalization capability on different Set B patterns for Tx beam prediction when the training dataset includes the data from the same setting of testing data.

For AI/ML model generalization on different Rx beam assumptions for Tx beam prediction, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions on AI/ML model training and AI/ML model testing.

The AI/ML model can provide good generalization capability on different Rx beam assumptions for Tx beam prediction.

For AI/ML model generalization on different gNB settings, a significant performance loss is observed if the training data samples and the testing data samples are generated under different gNB beam structures.

Compared with the case that the same gNB beam structure is used for generating the training data samples and the testing data samples, only marginal performance loss is observed if the training data samples of AI/ML model inclue mixed data samples from both gNB beam layouts.

For AI/ML model generalization on different number of UE Rx beams for beam pair prediction, compared with the case that the training data samples and testing data samples are generated based on the same number of UE Rx beams, only marginal performance loss is observed for the case that different number of UE Rx beams are used for AI/ML model training and testing.

The AI/ML model can provide good generalization capability on different number of UE Rx beams for beam pair prediction.

The AI/ML model can achieve a better beam prediction performance with UE distribution of 100% outdoor than with UE distribution of 80% indoor and 20% outdoor.

The AI/ML model can provide a good generalization capability on different UE distributions.

The performance of predicted L1-RSRP difference shows strong correction with other intermediate KPI (such as the beam prediction accuracy of Top-1/K beam(s)).

In BM-Case2, compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, compared with the case that the Set B pattern is randomly selected from a total of 4 beam patterns in each past time instance, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, if Top-4 or Top-2 beams of all measured beams are used as AI/ML model inputs in each past time instance, only a little performance loss is observed compared with the case that all measured beams are used as AI/ML model input.

In BM-Case2, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction with the assumption of the optimal Rx beam or the specific Rx beam being used for measurement.

In BM-Case2, if the AI/ML model is trained with samples of mixed UE speeds, a minor performance loss is observed as the UE speed in testing samples increases from 30km/h to 60km/h.

In BM-Case2, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, a minor performance loss is observed if both the training data samples and the testing data samples of the AI/ML model are mixed data samples.

Proposals:

In the beam pair prediction, it’s better to take measurement results of all Rx beams as the AI/ML model input (i.e., down sampling in Tx beams only) instead of performing any sampling in the Rx beam space.

- For the Rx beam assumption of beam pair prediction, consider both the Tx down sampling and/or Rx down sampling for evaluation.

At least for evaluation on the performance of DL Tx beam prediction, consider the following five Rx beam assumptions for providing input for AI/ML model for training and/or inference,

Rx beam assumption 1: the best Rx beam for the best Tx beam within Set B

Rx beam assumption 2: the best Rx beam searched for one Tx beam within Set B

Rx beam assumption 3:  the best Rx beam for the best Tx beam within Set A

Rx beam assumption 4: a specific Rx beam (e.g., the first Rx beam in the first UE panel)

Rx beam assumption 5: the best Rx beam for each Tx beam within Set B

To reduce the overhead of UE feedback beam measurements for DL Tx beam prediction in BM-Case 1, it is better to only report partial beams from a set of beams for measurement for a NW-side model.

Alt.4 (i.e., the L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP) is feasible to be adopted as the performance metric for AI/ML model monitoring.

To reduce UE report overhead in BM-Case 2 for a NW-side model, it is better to take partial beams from the beam set for measurement as AI/ML model input.

References

BMCase1_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase1_Txbeam_Generalization_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase1_BeamPair_Generalization_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase2_v000, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase2_Generalization_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

- Appendix

For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in the following table for Dense Urban scenario for SLS.

Table I. Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | For spatial domain beam prediction, 3km/h For time domain beam prediction: 30km/h (baseline), 60km/h (optional) Other values are not precluded |
| UE distribution | FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. For spatial domain beam prediction: FFS: Option 1: 80% indoor ,20% outdoor as in TR 38.901 Option 2: 100% outdoor For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | · [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline] · [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional] · Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M, N, P) = (1,4,2)] · 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline · Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: Option 1: Full buffer Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

UE trajectory model Option #4: Random direction straight-line trajectories. - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed. - The initial UE location should be randomly drop within the following blue area where d1 is the minimum distance that UE should be away from the BS. - Each sector is a cell and that the cell association is geometry based. - During the simulation, inter-cell handover or switching should be disabled. - For training data generation - For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance. - The value of T (or D) can be further discussed - The trajectory sampling interval granularity depends on UE speed and it can be further discussed. - UE can move straightly along the entire trajectory, or - UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°] - If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated. - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded. - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.