3GPP TSG-RAN WG1 #113	R1-2304779

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.2.2.1

Source:	InterDigital, Inc.

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

During RAN#94e, a study item (SI) on AI/ML for NR Air Interface was approved, with the revised study item description in [1].

For the CSI compression sub-use case, RAN1 #112 [2] provided initial templates for evaluation of 1-on-1 joint training, model generalization/scalability, multi-vendor joint training without model generalization/scalability as well as separate training without model generalization/scalability.

For the CSI prediction sub-use case, RAN1 #112 [2] confirmed the WA achieved in RAN1 #111 ([4]), shown below for convenience, and provided initial templates for evaluation of CSI prediction with model generalization.

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

For CSI prediction, RAN1 #112bis-e further agreed on the following simulation assumptions [3].

Agreement For the AI/ML based CSI prediction, for the submission of simulation results to the RAN1#113 meeting, - for Table 6. Evaluation results for CSI prediction without model generalization/scalability, companies are encouraged to take the following assumptions as baseline for the calibration purpose: - UE speed: 10km/h, 30km/h, 60km/h; - Others can be additionally submitted, e.g., 120km/h. - Input/Output type: Raw channel matrix - Other can be additionally submitted, e.g., eigenvectors. - Observation window: 5/5ms, 10/5ms - Other observation window configurations can be additionally submitted for comparison, e.g., 3/5ms, 4/5ms, 8/2.5ms, 10/4ms, etc. - Prediction window: 1/5ms/5ms - Other prediction window configurations can be additionally submitted for comparison, e.g., 3/5ms/5ms, 5/5ms/5ms, 4/2.5ms/2.5ms, 5/4ms/4ms, etc. - Performance metric for intermediate KPI: SGCS - NMSE can be additionally submitted. - Spatial consistency configuration (optional): procedure A with 50m decorrelation distance and channel updating periodicity of 1 ms. - for Table 7. Evaluation results for CSI prediction with model generalization, companies are encouraged to take the following assumption as baseline for the calibration purpose: - Performance metric for intermediate KPI: SGCS - NMSE can be additionally submitted.

This contribution presents simulation results for CSI compression for the initial template assumed in RAN1 #112 ([2]) and compares the performance of different AI/ML models (transformers vs. CNN) as well as provides a performance comparison of full matrix vs. EV-based CSI compression. For EV-based compression, we provide simulation results for model generalization over layers. Additionally, evaluation results for CSI compression with model generalization and model scalability are provided. Lastly, the contribution also presents updated evaluation results for CSI prediction as per the RAN1 #112bis-e agreement ([3]).

# CSI compression

## AI/ML Model considerations

For the CSI compression use case, we utilize multiple neural network models with varying configurations to accomplish the compression task. The discussed neural network models can be broadly classified as CNN based or transformer based.

### CNN based models

For CNN based models, we utilize two different architectures. The first is based on the CSI-Net model proposed in [9] and the second is based on the EVCsiNet model proposed in [8]. Next, we provide architectural details of the two models.

- Input CSI Type:

  - raw channel matrix estimated by UE

  - eigenvectors derived from the raw channel matrix

- Output CSI type: compressed channel matrix and compressed eigenvectors

- Pre-processing – for the full channel compression

To effectively compare the performance of the existing CSI feedback and precoding methods with the deep learning-based methods, we utilize the following pre-processing strategy:

  - we average the channel matrix across 2 resource blocks and across 1 time slot. Therefore, if the size of the raw channel is 624x2x16, representing 52 RBs, Nt = 16 and Nr = 2, the pre-processed channel will have a dimension of 26x2x16, respectively.

  - Additionally, we normalize the channels to zero mean and unit variance.

- Pre-processing – for the eigenvector based compression

  - Similar to the full channel case, first, we average the channel across 2 resource blocks and across 1 time slot. Thus, getting a channel of dimensions 26x2x16.

  - Next, we evaluate the SVD of the channel for each of the 26 sub-bands to evaluate the Nt ×1 = 16×1 dimensional eigenvectors. The Nr = 2, eigenvectors are then ordered based on their eigenvalues, from the largest to the smallest. Depending on the training configuration, the eigenvectors corresponding to a specific ordering, or a group of ordering may be selected for training.

- Quantization

The output of the encoder is quantized using a uniform quantizer. The encoder output is passed through a tanh layer to restrict the range of the encoder output for uniform quantization. The quantization operation is included during the training so that the encoder and decoder can learn appropriate weights while taking into account the quantization impact.

- Model Information:

- CSI-Net based Model

  - Base Model: For our evaluation we use the CSI-Net [9] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: CNN->BN->FC->FC

    - Quantizer: Linear quantization

    - Decoder architecture: FC-> FC -> RN->RN->CNN, where:

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

      - RN: CNN->BN->CNN->BN->CNN->BN with a skip connection from the input to the RN block

  - Feedback size: 64, 96, 128, 192 or 256 bottleneck elements with 2 bits per element

  - Loss function

    - We utilize the mean squared error loss function for training. The mean squared error is calculated between the input to the encoder and the output of the decoder.

  - Additional information:

    - Optimizer: ADAM with adaptive learning rate starting from a rate of = 0.0001 and scale it down by a factor of 0.87 every 5 epochs.

- EVCsiNet based Model

  - Base Model: For our evaluation we use a modified version of the EVCsiNet [9] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: FC->BN->FC

    - Quantizer: Linear quantization

    - Decoder architecture: FC-> followed by CNN layers and skip connections,

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

### Transformer based models

  - Base Model: For our evaluation we use a modified version of the Transformer architecture proposed in [9]. The utilized model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: CNN->Tf-block-> Tf-block -> Tf-block -> Tf-block ->FC

    - Decoder architecture: FC->Tf-block-> Tf-block -> Tf-block -> Tf-block ->CNN

    - Quantizer: Linear quantization

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - Tf-block: Multiheaded-attention block->FC->FC->LN with skip connections.

### Datasets for model training

To evaluate the performance of the AI/ML models, we consider the dataset as agreed upon in the prior RAN1 meetings #110 and #110bis-e ([7][8]). More details on the dataset and the configurations used can be found in the Appendix.

For training, a dataset containing 210k channel samples has been utilized; 80% of the dataset is used for model training, while 20% is used for validation.

In this contribution, we discuss multiple different CSI compression setups ranging from full channel compression to eigenvector compression. Although the data used for each of these setups is the same, the input and output dimensionality of the model, as well as the pre-processing steps may vary depending on the setup.

- Dataset F used to train the FCSIModel: This is the standard CSI compression use case wherein the entire channel matrix is compressed.

- Dataset L1 used to train the EVModel – L1 (Layer-specific): In this setup the eigen vector compression use case is considered. As a pre-processing step, the singular value decomposition of the channel matrix is evaluated and only the first eigenvector is utilized for training the model. The model is configured and trained to compress only one eigenvector at a time.

- Dataset L2 used to train the EVModel – L2 (Layer specific): In this setup the eigenvector compression use case is considered. As a pre-processing step, the singular value decomposition of the channel matrix is evaluated and only the second eigenvector is utilized for training the model. The model is configured and trained to compress only one eigenvector at a time.

- Dataset L12 used to train EVModel – L12 (Layer common-rank specific): In this setup the eigenvector compression use case is considered. As a pre-processing step, the singular value decomposition of the channel matrix is evaluated and both, the first as well as the second eigen vector are utilized for training the model. Although both the eigenvectors are used for training, the model is configured and trained to compress only one eigenvector at a time.

- Dataset R2 used to train the EVModel – R2 (Rank Two specific): In this setup the eigenvector compression use case is considered. Unlike the earlier setups aimed at compressing only one eigenvector at a time, this setup assumes that the channel has a rank of two and compresses both the eigenvectors together. As a pre-processing step, the singular value decomposition of the channel matrix is evaluated and both, the first as well as the second eigenvector are utilized together for training.

## CSI compression baseline: Rel-16 Type II

Rel-16 type II codebook will be used as the performance baseline to show the gain of AI/ML based CSI compression. Table 6.3.2.1.2-1 of TS 38.212 shows the details about how to calculate PMI payload of each rank for Rel-16 Type II codebook. Given N1 = 4, N2 = 2, O1 = O2 = 4, and the number of subbands = 13, the PMI overhead of paramCombination from 1 to 6 is shown in Table 1.

Table 1 PMI overhead for Rel-16 Type II codebook

| paramCombination | Rank 1 | Rank 2 | Rank 3 | Rank 4 |
| --- | --- | --- | --- | --- |
| 1 | 61 | 111 | 98 | 109 |
| 2 | 89 | 167 | 154 | 165 |
| 3 | 107 | 203 | 183 | 203 |
| 4 | 164 | 315 | 295 | 315 |
| 5 | 221 | 427 | 467 | 507 |
| 6 | 275 | 535 | 523 | 563 |

For the rank > 1 transmission cases in system-level simulation, we use the weighted overhead based on the rank distribution during the simulation to find the final overhead.

## CSI compression simulation configuration

The common parameters for dataset generation and for performance evaluation are as agreed in [5], [7] and [8]; for convenience, the common parameters table is also included in the Appendix.

## CSI compression simulation results

### Transformers compared to CNN based CSI compression

In our previous contributions, a CSI-Net [9] based autoencoder model with CNN used as a backbone has been utilized for comparison against Rel-16 Type II. To assess how different model architectures can impact the results, two additional models are considered and used for comparison against the CsiNet model. The two additional models are referred to as Transformer and EVCsiNet [10] (see Section 2.1 for the detailed structure). For the three models, the input is the spatial-frequency domain CSI, and the feedback size is set to 256 bits. For the Rel-16 Type II baseline, the parcomb 4 configuration is used with feedback size equal to 315 bits under the rank two transmission case. This results in 24% overhead increase over the considered AE models which are all using 256 bits.

Table 3 shows the average SGCS results for Layer 1 and Layer 2 separately. For the first layer, the EVCsiNet and Transformer models outperform the CsiNet model with 15% and 24% gain in SGCS, respectively, while for the second layer, AE models EVCsiNet and Transformer outperform the Rel-16 Type II baseline with 18% and 31% SGCS gain, respectively. While the EVCsiNet outperforms the CsiNet in terms of SGCS performance, the latter has much lower computational and memory requirements. For instance, Table 12 shows that the encoder of EVCsiNet has double the number of parameters and memory size of the CsiNet. This is due to the large number of fully connected layers used in the EVCsiNet. On the other hand, the transformer model is not only achieving the best performance relative to both CNN based models but also has comparable complexity and memory requirements compared to the CsiNet model, as shown in Table 12.

Observation 1: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of SGCS.

Observation 2: The AI/ML-based CSI compression using transformer model achieves a favorable performance-complexity-memory tradeoff.

The system level simulation results of the average throughput for the different AI/ML-based CSI compression models and the Rel-16 Type II codebook using full buffer traffic are provided in Table 2.  With the same feedback overhead of 256 bits, the EVCsiNet and transformer models have performance gain of 4.5% and 5.5%, respectively, over the CsiNet model.

Table 2 Average throughput results of the different AI/ML CSI compression models

|  | CsiNet | EVCsiNet | Transformer |
| --- | --- | --- | --- |
| Mean Throughput (Mbps) | 4.5779 | 4.785 | 4.827 |

Observation 3: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of average throughput.

Observation 4: The SGCS gain of the Transformer-based model over the CNN-based model (CsiNet) is 28% while the throughput gain is only 5%. The significant gap between the throughput and SGCS gain may suggest that using SGCS as an intermediate KPI is not a good indicator of the throughput gain.

Relative to the Rel-16 Type II codebook under the rank two transmission case, Table 3 shows that, for the first layer, AE models EVCsiNet and Transformer achieves a performance gain of 15% and 23%, respectively, in terms of SGCS, while for the second layer, AE models EVCsiNet and Transformer outperform the Rel-16 Type II baseline with 6% and 18% SGCS gain, respectively. Figure 1 shows that the Rel-16 Type II outperforms the CNN based models in the lower part of the CDF for the second layer. The Transformer model, however, is consistently better than the Rel-16 Type II for both the first and second layer.

Table 3 Average SGCS results of the different AE models and Rel-16 Type II for layer 1 and layer 2

|  | CsiNet | EVCsiNet | Transformer | Rel-16 Type II (parcomb4) |
| --- | --- | --- | --- | --- |
| Layer 1 | 0.558 | 0.667 | 0.717 | 0.582 |
| Layer 2 | 0.405 | 0.48 | 0.533 | 0.451 |

Figure 1 CDF of the SGCS for the different AE models and Rel-16 Type II

Observation 5: For the rank two case, the AI/ML-based CSI compression using transformer model achieves higher SGCS than the Rel-16 Type II with 20% reduction in overhead. Also, the achievable gain over Rel-16 Type II on the first layer is higher than that of the second layer.

### EV-based CSI compression: generalization over layers

This section provides evaluation results of the AI/ML based CSI compression with models trained on using eigenvectors of channels as the input and output of the AI/ML models. Four different eigenvector (EV) models are considered with three different feedback sizes for each model; 128, 192 and 256 bits. For each feedback size, each model is retrained with changing the bottleneck size. The four models are labeled as follows.

- EVModel-L1: this is a model trained in a layer-specific mode. That is, trained only on the first layer as an input, where each training sample is constructed from the principal eigenvectors across all subbands of the channel sample.

- EVModel-L2: this is a model trained in a layer-specific mode. That is, trained only on the second layer as an input, where each training sample is constructed as the second eigenvector across all subbands of the channel sample.

- EVModel-L12: this is a model trained in a layer-common-rank specific mode. That is, a unified model trained on samples from both layers, where the first and the second eigenvectors of each channel sample are derived and then used as two separate training samples. The inference is then done separately for each layer in case of rank two transmission.

- EVModel-R2: this is a model trained in a rank-specific mode. That is, the model is trained with the two right singular vectors of rank two channel samples.

Figure 2a illustrates the SGCS comparison between the different EV models for different feedback sizes under the rank one transmission case. It can be seen that, for rank one transmission, the layer-common model (EVModel-L12), purple bar, consistently outperforms all the other models for all feedback sizes. Interestingly, the layer-common model (EVModel-L12) even achieves higher SGCS relative to the layer-specific (EVModel-L1). This suggests that considering samples from both layers introduces more diversity in the data which in turn helps the model to generalize well. Figure 2b shows the SGCS comparison of the considered EV models and the Rel-16 Type II baseline for different feedback sizes. It can be seen that the all the considered EV models achieve higher SGCS relative to the Rel-16 Type II baseline for different feedback sizes. In particular, the layer-common model (EVModel-L12) achieves around 25% gain over the Rel-16 Type II in SGCS at the same feedback size.

(a) 							(b)

Figure 2 Average SGCS performance of the different EV models for rank one transmission

Figure 3 SGCS CDF performance of EV models

Figure 3 shows the CDF results for the different EV models. It can be seen that the layer-common model (EVModel-L12) consistently outperforms the layer-specific and rank-specific models for feedback sizes 128 and 256 bits, for all UEs.

Observation 6: For the rank one case, the AI/ML-based CSI compression using eigenvectors of channels as the input outperforms the Rel-16 Type II at similar overhead.

Observation 7: For the rank one case, the AI/ML-based CSI compression using the eigenvector layer-common model achieves the best SGCS performance relative to the layer-specific and rank-specific models.

For the rank two transmission case, the layer-specific models, EVModel-L1 and EVModel-L2, and the layer-common model (EVModel-L12) should be used twice during inference for layer 1 and layer 2, while the rank-specific model (EVModel-R2) is used once as it operates on both layers simultaneously. Thus, for fair comparison in the rank two transmission case, we use the feedback size of 128 bits for the models EVModel-L1, EVModel-L2, EVModel-L12 while we use the feedback size of 256 bits for the EVModel-R2. This makes the total number of bits for all models to be 256 bits under the rank two case.

Figure 4 illustrates the SGCS performance of the different EV models under the rank two case. It can be seen that all the EV models achieve comparable SGCS performance, with the layer-common model (EVModel-L12) slightly outperforming all the other EV models for both layer 1 and layer 2 in terms of SGCS. This suggests that the layer-common model generalizes well for both layers as it achieves the best performance for both rank one and rank two transmission relative to the layer-specific and rank-specific model.

Figure 4 SGCS performance of the different EV models under rank two transmission

Observation 8: The AI/ML-based CSI compression using the eigenvector layer-common model generalizes well when tested under two layers, and achieves the best SGCS performance relative to the layer-specific and rank-specific models for both rank one and rank two transmission.

### EV compared to full channel based CSI compression

This section provides evaluation results of the AI/ML based CSI compression for comparison between models trained with full CSI and models trained with eigenvectors as an input. For fair comparison, we fix the architecture of all models to the EVCsiNet described in Section 2.1. For the eigenvector models, we use the layer-common model (EVModel-L12) and rank-specific model (EVModel-R2), and for the full channel model we adopt the EVCsiNet model - hereafter referred to as FCSIModel. Additionally, it is worth emphasizing that the only difference between the EV rank-specific model (EVModel-R2) and the full CSI model (FCSIModel) is the input domain, i.e., eigenvectors vs. full channel.

Figure 5 shows the SGCS comparison between the EV models and the full CSI model for different feedback sizes under the rank one transmission case. It can be seen that, for rank one transmission, both EV models outperform the full CSI model (FCSIModel) for all feedback sizes. In particular, the EV layer-common model (EVModel-L12) achieves 20% gain in SGCS relative to the full channel model (FCSIModel) while the EV rank-specific model (EVModel-R2) is slightly better than the full CSI model. This suggests that, under rank one transmission, it may be better to perform compression with eigenvectors used as an input as opposed to full channel.

Figure 6 shows the SGCS CDF results for the EV models, full channel model and Rel-16 Type II under the rank one case. For the EV and full CSI models, we adopt the feedback sizes with 128 and 256 bits, while for the Rel-16 Type II, both paracomb 4 with 164 bits and paracomb 6 with 275 bits are adopted for comparison. This represents 28% increase in overhead of the Rel-16 Type II (paracomb 4) relative to the 128 bits models and 7.4% increase (paracomb 4) in overhead relative to the 256 bits models. It can be seen that the EV layer-common model (EVModel-L12) consistently outperforms the Rel-16 Type II in terms of SGCS and with additional overhead for the latter. On the other side, the FCSIModel and the rank-specific model are comparable to the Rel-16 Type II, where the latter is better in the lower part of the CDF, as shown in Figure 6. In addition, from the CDF curves in Figure 6, both EVModel-R2 and FCSIModel achieve comparable performance in terms of SGCS.

Figure 5 Average SGCS performance of the EV and full CSI models for rank one transmission

Figure 6 SGCS CDF performance of EV and full CSI based models under rank one transmission

Observation 9: The AI/ML-based CSI compression using eigenvectors as an input for the layer-common model consistently outperforms the AI/ML model using full CSI as an input and the Rel-16 Type II, in terms of SGCS under the rank one transmission case.

For the rank two transmission case, the layer-common model (EVModel-L12) should be used twice during inference for layer 1 and layer 2, while the rank-specific model (EVModel-R2) and the full CSI model (FCSIModel) are used once as they both operate on both layers simultaneously. To ensure that the total overhead is the same for all models, we use the 128 bits feedback size for the EV layer-common model (EVModel-L12) while we use the 256 bits feedback size for the rank-specific model (EVModel-R2) and the full CSI model (FCSIModel).

Figure 7 illustrates the average SGCS performance of the EV models and full CSI model under the rank two case for the feedback size of 256 bits. It can be seen for the rank two case, the EV layer-common model (EVModel-L12) achieves the best performance relative to the full CSI model and Rel-16 Type II.

Figure 7 Average SGCS results for full channel versus EV models under rank two transmission

Observation 10: For the rank two case, the AI/ML-based CSI compression using eigenvectors as an input for the layer-common model outperforms the AI/ML model using full CSI as an input and the Rel-16 Type II, in terms of SGCS.

### CSI compression of 1-on-1 joint training without model generalization

This section provides simulation results for 1-on-1 joint training without model generalization, using transformer-based models for CSI eigenvector-based compression.

The simulation results are presented in Table 4 below, for FTP and full buffer traffic, and max rank = 2.

Table 4 CSI compression of 1-on-1 joint training without model generalization

| Assumptions |  | FTP Max rank = 2 | Full buffer Max rank = 2 |
| --- | --- | --- | --- |
| CSI generation part | AI/ML model backbone | Transformers | Transformers |
|  | Pre-processing | Freq domain | Freq domain |
|  | Post-processing | N/A | N/A |
|  | FLOPs/M | 23.6 | 23.6 |
|  | Number of parameters/M | 0.625 | 0.625 |
|  | [Storage /Mbytes] | 5.534 | 5.534 |
| CSI reconstruction part | AI/ML model backbone | Transformers | Transformers |
|  | [Pre-processing] | N/A | N/A |
|  | [Post-processing] | N/A | N/A |
|  | FLOPs/M | 23.6 | 23.6 |
|  | Number of parameters/M | 0.627 | 0.627 |
|  | [Storage /Mbytes] | 5.572 | 5.572 |
| Common description | Input type | EV | EV |
|  | Output type | Compressed EV | Compressed EV |
|  | Quantization /dequantization method | Training-aware SQ, Uniform | Training-aware SQ, Uniform |
|  | Rank/layer adaptation settings for rank>1 | Option 3-2, Layer-common/ Rank-specific | Option 3-2, Layer-common/ Rank-specific |
| Benchmark |  | Rel-16 Type II | Rel-16 Type II |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  | Weighted average | Weighted average |
| Gain for Mean UPT (for a specific CSI feedback overhead) | [X*Max rank value], RU<=39% | 30% |  |
|  | [Y*Max rank value], RU<=39% | 25% |  |
|  | [Z*Max rank value], RU<=39% | 21% |  |
|  | [X*Max rank value], RU 40%-69% | 23% |  |
|  | [Y*Max rank value], RU 40%-69% | 21% |  |
|  | [Z*Max rank value], RU 40%-69% | 8% |  |
|  | [X*Max rank value], RU >=70% | 10% | 4% (full buffer) |
|  | [Y*Max rank value], RU >=70% | 9% | 4% (full buffer) |
|  | [Z*Max rank value], RU >=70% | 5% | 0.80% (full buffer) |
| Gain for 5% UPT | [X*Max rank value], RU<=39% | 9% |  |
|  | [Y*Max rank value], RU<=39% | 7% |  |
|  | [Z*Max rank value], RU<=39% | 2% |  |
|  | [X*Max rank value], RU 40%-69% | 5% |  |
|  | [Y*Max rank value], RU 40%-69% | -2% |  |
|  | [Z*Max rank value], RU 40%-69% | -8% |  |
|  | [X*Max rank value], RU >=70% | -5% |  |
|  | [Y*Max rank value], RU >=70% | -10% |  |
|  | [Z*Max rank value], RU >=70% | -10% |  |
| [CSI feedback reduction (%)] |  | (40%, 20%, 7%) | (40%, 20%, 7%) |

Observation 11: The gain of the AI/ML compression over the baseline appears to decrease as the RU utilization increases. This trend is similar for both Mean UPT and 5% UPT.

Observation 12: The gain of the AI/ML compression over the baseline appears to be smaller for low geometry UEs.

### CSI compression with model generalization

This section provides the evaluation of the generalizability of the AI/ML models under different channel models. In particular, we considered three eigenvector based models with Transformer backbone and feedback sizes 64, 128 and 256 bits. Each of three models is trained with UMa channel samples, UMi channel samples and mixed samples from both UMa and UMi.

Table 5, Table 6 and Table 7 show the average SGCS results for layers 1 and 2 under feedback sizes 64, 128, 256 bits, respectively. It can be seen that the performance of the AI/ML model slightly degrades when trained and tested on different scenarios (UMa/UMi). Additionally, the mixed model trained on both UMa and UMi channel samples generalizes well and outperforms the models trained and tested on the same scenario.

Table 5 Generalization performance under different UMa and UMi scenarios: SGCS  for a Transformer model with 64 bits.

| Train/Test | UMa (L1 -- L2) | UMi (L1 -- L2) |
| --- | --- | --- |
| UMa | 0.623 – 0.444 | 0.628 – 0.449 |
| UMi | 0.618 – 0.434 | 0.652 – 0.469 |
| Mixed | 0.636 – 0.444 | 0.659 – 0.472 |

Table 6 Generalization performance under different UMa and UMi scenarios: SGCS for a Transformer model with 128 bits.

| Train/Test | UMa (L1 -- L2) | UMi (L1 -- L2) |
| --- | --- | --- |
| UMa | 0.71 – 0.536 | 0.72 – 0.544 |
| UMi | 0.698 – 0.518 | 0.741 -- 0.57 |
| Mixed | 0.723 – 0.547 | 0.753 – 0.58 |

Table 7 Generalization performance under different UMa and UMi scenarios: SGCS for a Transformer model with 256 bits.

| Train/Test | UMa (L1 -- L2) | UMi (L1 -- L2) |
| --- | --- | --- |
| UMa | 0.803 – 0.671 | 0.814 – 0.681 |
| UMi | 0.774 – 0.619 | 0.829 – 0.694 |
| Mixed | 0.818 – 0.686 | 0.845 – 0.721 |

Observation 13: The AI/ML model trained on mixed datasets from UMa and UMi channel samples generalizes well when tested under each individual dataset. This suggests that using one model trained under mixed datasets can provide both performance gains and memory savings relative to using a separate model for each scenario.

### CSI compression with model scalability

This section provides the evaluation of the scalability of the AI/ML models under different input sizes. We considered one eigenvector based models with Transformer backbone and feedback size 128 bits. To evaluate the performance of a single AI/ML model under different input size, we use a model trained on a maximum bandwidth of 10 MHz and tested on 5 MHz, 3 MHz and 1 MHz bandwidth configurations. The samples for the 5 MHz, 3 MHz and 1 MHz configurations are obtained using a zero padding approach. To quantify the performance loss, we use a baseline that considers the entire 10 MHz channel samples without zero-padding but the SGCS is measured on the truncated input and output that match the 5 MHz, 3 MHz and 1 MHz configurations.

Table 8 shows SGCS values for the scalability performance of the 10 MHz AI/ML model. It can be seen that the model generalizes well when tested under the 5 MHz configuration with a slight performance degradation (1 %). Additionally, we observe that the performance degrades more when we increase the number of zero padded inputs (e.g., 3 MHz and 1 MHz).

Table 8 SGCS values for the scalability performance of AI/ML model under different bandwidth configurations

|  | L1-- L2 (5 MHz) | L1-- L2 (3 MHz) | L1 – L2 (1 MHz) |
| --- | --- | --- | --- |
| Baseline | 0.683 -- 0.525 | 0.675 – 0.513 | 0.659 – 0.493 |
| Zero-padded | 0.666 -- 0.513 | 0.561 – 0.434 | 0.369 – 0.302 |

Observation 14: The AI/ML model trained on 10 MHz bandwidth generalizes well when tested under 5 MHz bandwidth. Further, we observed that increasing the number of zero padded input results in more performance degradation relative to the baseline.

### Evaluation of quantization impacts

This section provides evaluation results of different quantization techniques that can be used for CSI compression. Figure 8 illustrates the distribution, i.e., density and CDF, of the values of all latents of the autoencoder, whereas Figure 9 illustrates the distribution for specific latents. From Figure 8 and Figure 9, it can be seen that the distribution of latent values is non-uniform and values of the latents are mostly in the range of 0.4 to 0.6. Based on this observation we evaluate clustering-based and CDF-based non-uniform quantizers and compare with traditional uniform quantizers for the case of quantization non-aware training. We further provide results on quantization aware training.

Figure 8: Density and CDF for All Latents Combined

Figure 9: Density and CDF for a Sample of Latents

Observation 15: The latent values have a non-uniform distribution.

We present the evaluation results based on the following quantization methods for quantization non-aware training. The quantization is applied to latents during inference after the model is trained with full precision. The number of bits used for quantization is denoted by NQ.

- Clustering-based / All Latents: Single set of clusters is computed and used for the quantization of all latents. The number of clusters is given by 2NQ.

- Clustering-based / Per Latent: Different sets of clusters are computed and used for the quantization each latent separately. The number of clusters per latent is given by 2NQ.

- CDF-based / All Latents: Single empirical CDF is constructed and used for the quantization of all latents.  The CDF transform is followed by a uniform quantization with 2NQ levels.

- CDF-based / Per Latent: Different empirical CDFs are constructed and used for the quantization of each latent separately.  The CDF transform is followed by a uniform quantization with 2NQ levels.

- Uniform Quantization: A uniform quantization with 2NQ levels is applied to all latents.

For quantization aware training, the latents are applied a uniform quantization with 2NQ levels during the training and different models are trained for given bits per quantization and number of latents. For quantization non-aware training, one model is trained for a given number of latents.

The evaluation results for quantization non-aware training and quantization aware training are provided in Table 9 and Table 10, respectively.

Table 9: SGCS Results on Quantization Non-Aware Training

|  |  | 64 Latents |  | 128 Latents |  |
| --- | --- | --- | --- | --- | --- |
|  | Quantization | 2-bit | 3-bit | 2-bit | 3-bit |
|  | Payload size | 128 | 192 | 256 | 384 |
| Clustering All Latents | SGCS Layer-1 | 0.3679 | 0.4273 | 0.4853 | 0.5668 |
|  | SGCS Layer-2 | 0.2351 | 0.2893 | 0.3259 | 0.4170 |
| Clustering Per Latent | SGCS Layer-1 | 0.3751 | 0.4364 | 0.4938 | 0.5767 |
|  | SGCS Layer-2 | 0.2367 | 0.2942 | 0.3286 | 0.4230 |
| CDF-based All Latents | SGCS Layer-1 | 0.3561 | 0.4192 | 0.4779 | 0.5596 |
|  | SGCS Layer-2 | 0.2161 | 0.2725 | 0.3050 | 0.3941 |
| CDF-based Per Latent | SGCS Layer-1 | 0.3775 | 0.4281 | 0.4930 | 0.5664 |
|  | SGCS Layer-2 | 0.2300 | 0.2792 | 0.3164 | 0.3997 |
| Uniform Quantization | SGCS Layer-1 | 0.2291 | 0.2750 | 0.3046 | 0.3493 |
|  | SGCS Layer-2 | 0.1453 | 0.1716 | 0.1792 | 0.2095 |

Table 10: SGCS Results on Quantization Aware Training

|  | 64 Latents |  | 128 Latents |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantization | No Quantization | Q-aware 2-bit | No Quantization | Q-aware 1-bit | Q-aware 2-bit |
| Payload size | NA | 128 | NA | 128 | 256 |
| SGCS Layer-1 | 0.4701 | 0.4304 | 0.6244 | 0.4457 | 0.5464 |
| SGCS Layer-2 | 0.3329 | 0.2752 | 0.4878 | 0.2708 | 0.3832 |

The SGCS results on the case with full precision CSI compression, i.e., no quantization, is also provided in Table 10 that sets the upper bound for SGCS in our evaluations. The results in Table 9 show that non-uniform quantization improves the SGCS performance significantly compared to uniform quantization. The results further show that SGCS for per latent quantization is always higher than the all-latent quantization counterpart due to the differences in the distribution of latent values, but this may incur additional overhead. Clustering and CDF-based non-uniform quantization shows similar SGCS performance where clustering-based method slightly outperforms the other.

The quantization aware training method shows around 18% and 10% increase in SGCS performance compared to non-uniform quantization for 128 and 256 bits of payload size, respectively. Quantization aware training has the potential to provide better results in terms of SGCS while lacking generalization on payload size. On the other hand, quantization non-aware training has the potential on payload size generalization while lagging on SGCS performance.

Observation 16: For quantization non-aware training: non-uniform quantization improves the SGCS performance significantly compared to uniform quantization.

Observation 17: For quantization non-aware training: the per-latent quantization outperforms the all-latent quantization.

Observation 18: For quantization non-aware training: the clustering and CDF-based non-uniform quantization show similar SGCS performance.

Observation 19: Quantization aware training outperforms (in terms of SGCS) the quantization non-aware training, at the expense of lacking generalization on the payload size.

Observation 20: Quantization non-aware training has more flexibility on payload size generalization, at the expense of some SGCS performance degradation.

### Type 2 multi-vendor joint training

This section presents the evaluation of Type 2 multi-vendor joint training. The results are shown in Table 11, where the index ‘1’ refers to a EVCsiNet model, and index ‘2’ refers to a Transformer model; thus, “E1” refers to an EVCsiNet encoder, “D1” refers to an EVCsiNet Decoder, “E2” refers to a transformers based encoder, and “D2” a transformers based decoder. The models have the same output size of 256 bits.

Table 11 SGCS results for Type 2 multi-vendor joint training

| Train/Test | E1-D1 | E2-D1 | E1-D2 | E2-D2 |
| --- | --- | --- | --- | --- |
| 1-1 | 0.793 -- 0.678 | 0.799 -- 0.687 | 0.781 -- 0.645 | 0.791 -- 0.638 |
| 2-1 (E1, E2, D2) | N/A | N/A | 0.781 -- 0.641 | 0.806 -- 0.678 |
| 2-2 (E1, E2, D1, D2) | 0.786 – 0.664 | 0.801 – 0.688 | 0.784 – 0.659 | 0.799 – 0.682 |

The baseline models trained with M=1, N=1 have similar performance to those trained with M=2, N=2 and M=2, N=1 setups. In terms of the intermediate KPI (SGCS), no degradation was seen with the multi-vendor case. A marginal increase in SGCS, (order of 10-2 to 10-3) was seen for the Type 2 multi-vendor joint training, but given the improvement is very small and the training was carried out over one random initialization of weights only, further analysis over multiple random training initializations (higher Monte Carlo iterations) and with much larger M and N and may be required to draw a conclusion on the precise percentage gain or loss associated with multi-vendor training.

Observation 21: For the scenario analysed (M=2, N=2), only a marginal difference in the SGCS performance was found with respect to 1-on-1 joint training.

Observation 22: further analysis over multiple random training initializations (higher Monte Carlo iterations) and with much larger M and N and may be required to draw a conclusion on the relative performance of Type 2 multi-vendor joint training.

### Summary of results

Table 12 Evaluation results for CSI compression without model generalization/scalability, [Full Buffer], [Max rank =2], [training type 1]

|  |  |  | InterDigital 1 | InterDigital 2 | InterDigital 3 |
| --- | --- | --- | --- | --- | --- |
| CSI generation part |  | AL/ML model backbone | CNN (CsiNet) | Transformers | CNN (EVCsiNet) |
|  |  | Pre-processing | Freq domain | Freq domain | Freq domain |
|  |  | Post-processing | N/A | N/A | N/A |
|  |  | FLOPs/M | 1.9 | 40.1 | 3.68 |
|  |  | Number of parameters/M | 0.918 | 1.15 | 1.84 |
|  |  | [Storage /Mbytes] | 3.64 | 5.71 | 7.13 |
| CSI reconstruction part |  | AL/ML model backbone | CNN (CsiNet) | Transformers | CNN (EVCsiNet) |
|  |  | [Pre-processing] | N/A | N/A | N/A |
|  |  | [Post-processing] | N/A | N/A | N/A |
|  |  | FLOPs/M | 10.0 | 40.1 | 1973.9 |
|  |  | Number of parameters/M | 0.925 | 1.15 | 1.4 |
|  |  | [Storage /Mbytes] | 5.61 | 5.75 | 6.05 |
| Common description |  | Input type | Raw channel matrix | Raw channel matrix | EV, Raw channel matrix |
|  |  | Output type | Compressed channel matrix | Compressed channel matrix | Compressed EV, Compressed channel matrix |
|  |  | Quantization /dequantization method | uniform | Uniform | uniform |
| Dataset description |  | Train/k | 210 | 210 | 210 |
|  |  | Test/k | 20 | 20 | 20 |
|  |  | Ground-truth CSI quantization method | N/A | N/A | N/A |
| Benchmark |  |  |  |  |  |
| Intermediate KPI l#1 of benchmark, [layer 1]: SGCS KPI1: SGCS Rank one case | CSI feedback payload 164 (Rel-16 config4) |  | 0.5825 |  |  |
|  | CSI feedback payload 221 (Rel-16 config5) |  | 0.5914 |  |  |
|  | CSI feedback payload 275 (Rel-16 config6) |  | 0.6656 |  |  |
| Gain for intermediate KPI I#1, [layer 1] Rank one case | CSI feedback payload 164 (Rel-16 config4) |  | 16% (22% less overhead) |  |  |
|  | CSI feedback payload 221 (Rel-16 config5) |  | 25% (13% less overhead) |  |  |
|  | CSI feedback payload 275 (Rel-16 config6) |  | 18% (7% less overhead) |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1]: SGCS KPI1: SGCS Rank two case | CSI feedback payload 315 (Rel-16 config4) |  | 0.5825 |  |  |
| Intermediate KPI I#1 of benchmark, [layer 2] Rank two case | CSI feedback payload 317 (Rel-16 config4) |  | 0.451 |  |  |
| Gain for intermediate KPI I#1, [layer 1] Rank two case | CSI feedback payload 317 (Rel-16 config4) |  | N/A | 23% | 15% |
| Gain for intermediate KPI#1, [layer 2] Rank two case | CSI feedback payload 317 (Rel-16 config4) |  | N/A | 18% | 6% |

# CSI prediction

## AI/ML Model considerations

### Model architecture

The AI/ML model adopted for CSI prediction is the linear regression (LR), in which the relation between the historical CSI (input of the model) and future CSI (output of the model) is modelled using linear predictor functions. The model parameters are estimated from the dataset using the least-square estimation technique.

The AI/ML model adopted for CSI prediction has a LSTM backbone with 3 LSTM layers and 1 fully connected layer, with a total of 707 k parameters. Additionally, there is a weighted bypass that connects the input to the output. The input to the model is a (NRx×NTx)MIMO channel associated with a single sub-band and thus the prediction for each sub-band is performed independently. SGCS is used as the loss function for the training.

### Dataset for model training

In the context of CSI prediction, let L denote the number of historical CSI to be fed to the CSI predictor (either the AI/ML model or the baselines), and let N denote the time slot index of the future CSI to be predicted. In the construction of the dataset, Nsamples channel samples are generated, where each two consecutive channel samples are 1 ms a part. The size of channel sample is NRx×NTx×Nsb×NUEs, where NRx, NTx, Nsb, and NUEs denote the number of receive antenna ports, the number of transmit antenna ports, the number of sub-bands, and the number of UEs, respectively. The time samples of each channel element in the 4D tensor are then grouped together to create NRx×NTx×Nsb×NUEs sequences of size Nsamples each. Afterwards, for each channel coefficient, Nseq= Nsamples-L+N+1 pair of sequences are constructed from its initial sequence of size Nsamples time samples. Specifically, for all i ∈ 1, Nseq: the two sequences are defined as follows:

- The first sequence contains the time samples of the channel element from i-1+1 to i-1+L, which is the sequence of features (or sequence of historical CSI) that will be fed to the CSI predictor,

- The second sequence contains the time samples of the same channel element from i-1+L+1 to i-1+N+1, which represents the sequence of labels (or sequence of future CSI) to be predicted by the CSI predictor.

Finally, the dataset consists of the NRx×NTx×Nsb×NUEs×Nseq distinct sequences of features, where each sequence has L features, and their associated NRx×NTx×Nsb×NUEs×Nseq sequences of labels, where each sequence has N labels.

## CSI prediction baseline

### Kalman filter architecture for CSI prediction

The Kalman filter is an algorithm for doing inference in discrete linear dynamic systems in which the state space of the latent variables is discrete and where all latent and observed variables have a Gaussian distribution. In the context of CSI prediction, the CSI evolution over time can be modelled as a noisy dynamic system, and therefore, the Kalman filter can be used to predict the future CSI. Specifically, the Kalman Filter is a two-stage algorithm that assumes there is a smooth trend-line within the data that represents the true value of the CSI before being disturbed by noise. In the first stage, a few historical CSI values are fit to a model, which are then extrapolated to the next time value to generate a CSI prediction along with its error variance. In the second stage, the corresponding CSI measurement is read, and a new CSI trend value is computed as a weighted average of the predicted CSI and the measured CSI. The weights are based on the relative amounts of noise in the data and predictions. The filter then repeats this cycle of prediction and correction as each new CSI value is observed.

Moreover, let hk denote the true (or corrected) CSI state at time k, which includes the historical CSI from k-L+1 to k, hk+N denote the estimated (or observed) CSI state at time k+N, and hk+N denote the predicted CSI state of time k+N. The goal of the Kalman Filter is to predict the CSI state  hk+N of time k+N at time k, using the true CSI state hk at time k, and then correct this prediction using the estimated CSI state hk+N at time k+N.

In this case, the Kalman Filter assumes that the CSI state at time k+N evolves from the CSI state at time k according to

hk+N=Ak+N hk+ qk+N,

Where, Ak+N is the state transition matrix, and qk+N is the process noise with wk+N∼ ℵ 0, Qk+N, in which Qk+N is the covariance matrix of the process noise This equation is called the dynamic (or plant) equation or the channel. Moreover, at time k+N, an estimation of the CSI is made, and the estimated CSI state at time k+N follows

hk+N= hk+N+ vk+N,

Where vk+N is the observation noise with vk+N∼ ℵ 0, Vk+N, in which Vk+N is the covariance matrix of the observation noise. The initial state h0 is generally modelled as a Gaussian random variable, with known mean and covariance. The two noisy sequences and the initial state are mutually independent and the matrix Ak+N depends on the adopted channel model.

The Kalman Filter uses the previous corrected state and the current observed CSI state to predict and correct the current CSI state, hence it is a recursive estimator. Two variables need to be predicted and corrected, which are hk+N, and Pk+N, which is the error covariance matrix or in simple words the measure of the estimated accuracy of the CSI state estimate. There are two stages in the Kalman Filter algorithm, namely, prediction and correction.

Prediction:

The predicted CSI state estimate is given by hk+N= Ak+N hk and the predicted covariance is given by Pk+N= Ak+NPk Ak+NT.

Correction:

After performing the prediction, the measurement residual at time k+N, is given by ek+N= hk+N- hk+N, and its covariance is given by Ek+N= Pk+N+ Vk+N. Hence, the filter gain is given by Gk+N= Pk+NEk+N-1. Based on this, the corrected CSI state is computed as hk+N= hk+N+ Gk+Nek+N, and the corrected covariance is computed as Pk+N= Pk+N- Gk+N Ek+N Gk+NT.

Finally, at each time slot k+N, the CSI state transition matrix Ak+N and the CSI process covariance matrix Qk+N are obtained and updated online along with the prediction and correction procedures.

## CSI prediction simulation configuration

In generating the channel samples, we consider two scenarios:

- Spatial consistency off (Doppler only), and

- Spatial consistency on, procedure A

The simulation configuration specific to CSI prediction is shown below.

- Traffic type: Full Buffer

- Max rank value = 2

- Intermediate KPI#1 = SGCS

- Channel model: UMa

- UE distribution: 100% outdoor UE (per RAN1 #110 conclusion)

- O2I car penetration loss (as per 38.901, section 7.4.3.2 – per the RAN1 #110b conclusion)

- Number of drops for training: 25

- Number of drops for the testing: 5

- Number of UEs per drop: 84

- Number of time samples N_samples (5 ms period): 60

- Observation window: 5/5 for Section 3.4.1, and 1..10/1 for Section 3.4.3

- Prediction window: 1/5/5 for Section 3.4.1, and 1..10/1/1 for Section 3.4.3

The common parameters for dataset generation and for performance evaluation are as agreed in [5], [7] and [8]; for convenience, the common parameters table is also included in the Appendix.

## CSI prediction simulation results

To evaluate the CSI prediction performance, two baselines are considered, as follows:

- Baseline #1: the nearest historical CSI without prediction, also known as sample and hold (S&H) baseline.

- Baseline #2: the Kalman Filter (KF).

### Impact of spatial consistency on the CSI prediction performance

To evaluate the impact of mobility on the performance of CSI prediction (AI/ML model and the baseline), we present simulation results for the case of spatial consistency off (Doppler only) and compare to the results when the spatial consistency (procedure A) is enabled.

The SGCS performance of the AI/ML model, the S&H and the Kalman filter baselines is shown in Figure 10 and Figure 11 for spatial consistency off and on, respectively.

Figure 10 SGCS Performance versus speed, spatial consistency off

In Figure 10 it can be seen that the AI/ML model outperforms the S&H baseline, while the Kalman filter outperforms the AI/ML model. Additionally, for each of the models, the SGCS performance of Layer 1 is better than the SGCS performance for Layer 2.

Figure 11 SGCS Performance versus speed, spatial consistency on

When spatial consistency is enabled (Figure 11), the SGCS performance of Layer 1 is still better than the SGCS performance for Layer 2, for all models considered.  The relative performance of Layer 1 versus Layer 2 is consistent regardless of spatial consistency.

It is important to note that when spatial consistency is enabled (see Figure 11), the Kalman filter still offers the best performance and outperforms both the AI/ML based CSI prediction and the S&H baseline. The AI/ML based CSI prediction outperforms the S&H baseline for 10 km/h, and it is slightly worse than S&H for 30 and 60 km/h.

While it is possible to further improve the performance of the AI/ML CSI prediction, the relative gain over the S&H baseline will likely be smaller than the relative gain observed when spatial consistency is off.  This illustrated in Figure 12, where the performance of Layer 1 of the AI/ML based CSI prediction, the Kalman filter and the S&H baseline is presented for both spatial consistency on and off.

Figure 12 Impact of mobility on the CSI prediction SGCS performance

Observation 23: Enabling spatial consistency (procedure A) results in SGCS performance degradation for all models considered (AI/ML based CSI prediction, Kalman filter and S&H baselines).

Observation 24: The relative gain of the AI/ML based CSI prediction over the S&H baseline decreases when spatial consistency (procedure A) is enabled in the simulations. This is because the dataset becomes much more complex as more channel parameters are involved in impacting the correlation between samples in time, and hence, AIML model performance will be highly impacted.

Observation 25: Kalman filter seems to be performing the best across all methods regardless of the spatial consistency being enabled or not. This suggests that Kalman filter should be considered as the non AI/ML baseline to assess the actual gains of the AI/ML solution, otherwise the gains of the AIML solution may be overestimated if compared to the S&H baseline.

Observation 26: If spatial consistency (procedure A) is not used for evaluation, the relative gains of AI/ML based CSI prediction may be overestimated.

Proposal 1: For a realistic evaluation of the AI/ML based CSI prediction performance gains over the baseline, spatial consistency (procedure A) should be used for channel generation.

#### Results in tabular form

The AI/ML CSI prediction results without generalization are presented in Table 13.

Table 13 CSI prediction without generalization

|  |  | InterDigital 1 | InterDigital 2 | InterDigital 3 |
| --- | --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | LSTM |  |  |
|  | [Pre-processing] | Freq domain avg |  |  |
|  | [Post-processing] | Freq domain |  |  |
|  | Input type | raw channel matrix | raw channel matrix | raw channel matrix |
|  | Output type | channel matrix | channel matrix | channel matrix |
| Assumption | UE speed | 10 km/h | 30 km/h | 60 km/h |
|  | CSI feedback periodicity | 5 ms | 5 ms | 5 ms |
|  | Observation window (number/distance) | 5 | 5 | 5 |
|  | Prediction window (number/distance) | 1/5 | 1/5 | 1/5 |
|  | Whether/how to adopt spatial consistency | Yes | Yes | Yes |
| Dataset size | Train/k | 210 k | 210 k | 210 k |
|  | Test/k | 210 k | 210 k | 210 k |
| Benchmark 1 (the nearest historical CSI w/o prediction, i.e. S&H) |  |  |  |  |
| Intermediate KPI #1 (SGCS) of Benchmark 1 | Layer 1 | 0.653 | 0.46 | 0.407 |
| Gain for intermediate KPI#1 over Benchmark 1 | Layer 1 | 20% | -3%% | -3% |
| Intermediate KPI #1 (SGCS?) of Benchmark 1 | Layer 2 | 0.585 | 0.394 | 0.351 |
| Gain for intermediate KPI#1 over Benchmark 1 | Layer 2 | 27% | -4% | -4% |
| Benchmark 2: Kalman filter based non-AI/ML CSI prediction |  |  |  |  |
| Intermediate KPI #1 of Benchmark 2 | Layer 1 | 0.894 | 0.577 | 0.434 |
| Gain for intermediate KPI#1 over Benchmark 2 | Layer 1 | -11% | -22% | -9% |
| Intermediate KPI #1 of Benchmark 2 | Layer 2 | 0.857 | 0.501 | 0.376 |
| Gain for intermediate KPI#1 over Benchmark 2 | Layer 2 | -13% | -25% | -10% |

### Generalization performance

This section presents evaluation results for the model generalization over speeds.

#### Evaluation results for generalization Case 2

For Case 2 testing, the relative SGCS gain over Case 1 for Layer 1 of the AI/ML model is shown in Table 14, for the case of spatial mobility off.

Table 14 Relative SGCS performance (%) for Case 2 compared to Case 1, Spatial consistency off

| Train Speed (km/h) | Test speed 10 km/h | Test speed 30 km/h | Test speed 60 km/h |
| --- | --- | --- | --- |
| 10 | 0.0 | -16.5 | -9.6 |
| 30 | -1.7 | 0.0 | -15.4 |
| 60 | -17.0 | -32.0 | 0.0 |

Table 14 (spatial consistency off) shows that a model trained for 10 km/h does not generalize well for 30 and 60 km/h operation; similarly, the model trained at 60 km/h does not generalize well for 10 and 30 km/h. The only model that has marginally acceptable generalization is the model trained at 30 km/h, that operates with relatively small degradation at 10 km/h. However, that same model (trained at 30 km/h) does not generalize well at 60 km/h.

For Case 2 testing, the relative SGCS gain over Case 1 for Layer 1 of the AI/ML model is shown in Table 15, for the case of spatial mobility on.

Table 15 Relative SGCS performance (%) for Case 2 compared to Case 1, Spatial consistency on

| Train Speed (km/h) | Test speed 10 km/h | Test speed 30 km/h | Test speed 60 km/h |
| --- | --- | --- | --- |
| 10 | 0.0 | -9.5 | -8.1 |
| 30 | -24.0 | 0.0 | -0.5 |
| 60 | -33.0 | -2.5 | 0.0 |

Similar to the observations for the spatial consistency off case, the simulations in Table 15 (spatial consistency on) show that model trained for 10 km/h does not generalize well for 30 and 60 km/h operation.

Observation 27: The AI/ML CSI prediction model does not appear to generalize well across speeds (for Case 2).

Lastly, it is interesting to evaluate the relative performance of the AI/ML model with respect to the S&H baseline.  The relative SGCS performance of the AI/ML model compared to S&H, for Layer 1, with spatial consistency off and on, is shown in Table 16 and Table 17, respectively.

Table 16 Relative SGCS performance (%) for the AI/ML model vs S&H, Spatial consistency off

| Train Speed (km/h) | Test speed 10 km/h | Test speed 30 km/h | Test speed 60 km/h |
| --- | --- | --- | --- |
| 10 | 6.5 | 13.0 | -12.5 |
| 30 | 4.6 | 35.4 | -18.1 |
| 60 | -11.6 | -8.0 | -3.2 |

Table 17 Relative SGCS performance (%) for the AI/ML model vs S&H, Spatial consistency on

| Train Speed (km/h) | Test speed 10 km/h | Test speed 30 km/h | Test speed 60 km/h |
| --- | --- | --- | --- |
| 10 | 21.0 | -12.4 | -10.7 |
| 30 | -8.1 | -3.2 | -3.3 |
| 60 | -19.0 | -5.6 | -2.9 |

The results from Table 16 and Table 17 indicate that S&H is better than the AI/ML CSI prediction model for Case 2 (out of distribution) and spatial consistency on.  Moreover, the in-distribution gain reduces when spatial consistency is enabled: specifically, the AI/ML model (train/test at 30/30, Table 16) shows 35.4% improvement over S&H when spatial consistency is off, but the improvement decreases to 21% (for the train/test 10/10, Table 17) once spatial consistency is enabled.

Observation 28: Spatial consistency is needed for a realistic evaluation of the AI/ML CSI prediction compared to the baseline.

#### Evaluation results for generalization Case 3

To evaluate the relative performance (over Case 1) of the AI/ML model for generalization Case 3 across speeds, the model was trained on a mixed dataset based on 10, 30 and 60 km/h UE speeds. The simulation results of the mixed model are shown in Table 18.

Table 18 Relative SGCS performance (%) for Generalization Case3 across speeds

| Spatial consistency | Test speed 10 km/h | Test speed 30 km/h | Test speed 60 km/h |
| --- | --- | --- | --- |
| Off | -0.6 | -2.6 | -13.6 |
| On | -3.8 | -1.5 | 0.6 |

From Table 18 above it can be seen that the mixed model (trained on the mixed dataset) generalizes better across the tested speeds as compared to models trained on a single speed.

Observation 29: The mixed model for Case 3 (model trained on the mixed dataset) generalizes better compared to models trained on datasets comprised of single speed (Case 2).

# Conclusion

In this contribution, we presented a simulation-based comparison of AI/ML CSI compression of the raw channel matrix vs eigenvector compression as well as the baseline Rel-16 Type II codebook.  We also provided simulation results for CSI compression with model generalization and scalability. Additionally, we evaluated the AI/ML performance for CSI prediction and compared it to the the nearest historical CSI w/o prediction and to the non-AI/ML based CSI prediction.

We provide the following observations and proposals:

Observation 1: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of SGCS.

Observation 2: The AI/ML-based CSI compression using transformer model achieves a favorable performance-complexity-memory tradeoff.

Observation 3: The AI/ML-based CSI compression using transformer model outperforms the CNN based models in terms of average throughput.

Observation 4: The SGCS gain of the Transformer-based model over the CNN-based model (CsiNet) is 28% while the throughput gain is only 5%. The significant gap between the throughput and SGCS gain may suggest that using SGCS as an intermediate KPI is not a good indicator of the throughput gain.

Observation 5: For the rank two case, the AI/ML-based CSI compression using transformer model achieves higher SGCS than the Rel-16 Type II with 20% reduction in overhead. Also, the achievable gain over Rel-16 Type II on the first layer is higher than that of the second layer.

Observation 6: For the rank one case, the AI/ML-based CSI compression using eigenvectors of channels as the input outperforms the Rel-16 Type II at similar overhead.

Observation 7: For the rank one case, the AI/ML-based CSI compression using the eigenvector layer-common model achieves the best SGCS performance relative to the layer-specific and rank-specific models.

Observation 8: The AI/ML-based CSI compression using the eigenvector layer-common model generalizes well when tested under two layers, and achieves the best SGCS performance relative to the layer-specific and rank-specific models for both rank one and rank two transmission.

Observation 9: The AI/ML-based CSI compression using eigenvectors as an input for the layer-common model consistently outperforms the AI/ML model using full CSI as an input and the Rel-16 Type II, in terms of SGCS under the rank one transmission case.

Observation 10: For the rank two case, the AI/ML-based CSI compression using eigenvectors as an input for the layer-common model outperforms the AI/ML model using full CSI as an input and the Rel-16 Type II, in terms of SGCS.

Observation 11: The gain of the AI/ML compression over the baseline appears to decrease as the RU utilization increases. This trend is similar for both Mean UPT and 5% UPT.

Observation 12: The gain of the AI/ML compression over the baseline appears to be smaller for low geometry UEs.

Observation 13: The AI/ML model trained on mixed datasets from UMa and UMi channel samples generalizes well when tested under each individual dataset. This suggests that using one model trained under mixed datasets can provide both performance gains and memory savings relative to using a separate model for each scenario.

Observation 14: The AI/ML model trained on 10 MHz bandwidth generalizes well when tested under 5 MHz bandwidth. Further, we observed that increasing the number of zero padded input results in more performance degradation relative to the baseline.

Observation 15: The latent values have a non-uniform distribution.

Observation 16: For quantization non-aware training: non-uniform quantization improves the SGCS performance significantly compared to uniform quantization.

Observation 17: For quantization non-aware training: the per-latent quantization outperforms the all-latent quantization.

Observation 18: For quantization non-aware training: the clustering and CDF-based non-uniform quantization show similar SGCS performance.

Observation 19: Quantization aware training outperforms (in terms of SGCS) the quantization non-aware training, at the expense of lacking generalization on the payload size.

Observation 20: Quantization non-aware training has more flexibility on payload size generalization, at the expense of some SGCS performance degradation.

Observation 21: For the scenario analysed (M=2, N=2), only a marginal difference in the SGCS performance was found with respect to 1-on-1 joint training.

Observation 22: further analysis over multiple random training initializations (higher Monte Carlo iterations) and with much larger M and N and may be required to draw a conclusion on the relative performance of Type 2 multi-vendor joint training.

Observation 23: Enabling spatial consistency (procedure A) results in SGCS performance degradation for all models considered (AI/ML based CSI prediction, Kalman filter and S&H baselines).

Observation 24: The relative gain of the AI/ML based CSI prediction over the S&H baseline decreases when spatial consistency (procedure A) is enabled in the simulations. This is because the dataset becomes much more complex as more channel parameters are involved in impacting the correlation between samples in time, and hence, AIML model performance will be highly impacted.

Observation 25: Kalman filter seems to be performing the best across all methods regardless of the spatial consistency being enabled or not. This suggests that Kalman filter should be considered as the non AI/ML baseline to assess the actual gains of the AI/ML solution, otherwise the gains of the AIML solution may be overestimated if compared to the S&H baseline.

Observation 26: If spatial consistency (procedure A) is not used for evaluation, the relative gains of AI/ML based CSI prediction may be overestimated.

Observation 27: The AI/ML CSI prediction model does not appear to generalize well across speeds (for Case 2).

Observation 28: Spatial consistency is needed for a realistic evaluation of the AI/ML CSI prediction compared to the baseline.

Observation 29: The mixed model for Case 3 (model trained on the mixed dataset) generalizes better compared to models trained on datasets comprised of single speed (Case 2).

Proposal 1: For a realistic evaluation of the AI/ML based CSI prediction performance gains over the baseline, spatial consistency (procedure A) should be used for channel generation.

Proposal 2: The trade-off between the CSI prediction performance for various UE speeds and the model complexity and prediction parameters (observation window, prediction window) needs to be studied.

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RAN1 #112 Chairman’s note

RAN1 #112bis-e Chairman’s note

RAN1 #111 Chairman’s note

RAN1 #109-e Chairman’s note

R1-2207840, “Summary#5 for CSI evaluation of [110-R18-AI/ML]”, Moderator (Huawei)

RAN1 #110 Chairman’s note

RAN1 #110bis-e Chairman’s note

C. Wen, W. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.

W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu and J. Shen, "EVCsiNet: Eigenvector-Based CSI Feedback Under 3GPP Link-Level Channels," in IEEE Wireless Communications Letters, vol. 10, no. 12, pp. 2688-2692, Dec. 2021, doi: 10.1109/LWC.2021.3112747.

A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A. N. Gomez, Ł. Kaiser, and I. Polosukhin. "Attention is all you need." Advances in neural information processing systems 30 (2017).

# Appendix

The common parameters used for the considered suite of simulations are based on the assumptions agreed in [2] and is shown below for convenience.

Table 20 Common parameters used in all Scenarios/Configurations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Uma, Umi |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports ; configuration specific |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30 kHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank adaptation |
| Number of users |  | 6 UE per BS |
| Max number of MU layers |  | 12 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | Full buffer, Non full buffer (FTP Model 1), packet size 0.5 Mbytes |
| UE distribution |  | (0.5, 0.5) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | 4ms delay, ideal |
| Channel estimation |  | ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. |
| Baseline for performance evaluation |  | Rel-16 Type II CSI |