3GPP TSG RAN WG1 #113			R1-2304891

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	     Xiaomi

Title:		     Discussion on UE features for MC enhancements

Agenda item:    9.16.9

Document for:  Discussion

Introduction

In RAN1#112bis e-meeting, UE features for MC enhancements were heatedly discussed and the framework was settled. [1]

In this contribution, we provided our views on controversial issues listed in feature lead summary. [2]

Discussion

UE features for multi-cell PUSCH/PDSCH scheduling with a single DCI

FG 49-1

Component 2): It is related to the question whether FG49-1a should be separated from FG49-1 or not. If the answer is yes, component 2) is no longer needed under FG49-1. After the back-and-forth discussion, we tend to agree that there is no difference from UE complexity point of view between two options. The key point of UE complexity is how to handle the BD/CCE. Considering same SCS is applied to both scheduling cell and co-scheduled cells, and reference cell is clearly defined for BD/CCE handling, there should no difference to include scheduling in cell set or not. Hence, we prefer to keep component 2) in FG49-1.

Component 3): We are okay to let’s UE report the carrier type. The proposed candidate values is quite aligned with current UE features, e.g. FG22-6/22-6a/22-7. Therefore, we support the current version of component 3).

Component 4): There is one controversial issue on whether to report the max number of co-scheduled cells per set of cells supported by UE per carrier type. RAN1 doesn’t differentiate carrier types except SCS. On the other word, the entire UE procedure, e.g. PDCCH monitoring, data channel reception/transmission, HARQ/ACK feedback, reference signal measurement and report, etc., is carrier type agnostic. We don’t see the necessity to differentiate different carrier type.

Component 5): We understand the intention to report maximum number of cell sets per PUCCH group is to address the following RAN1 agreement. If we don’t explicitly capture the maximum number should be reported per PUCCH group, it may break per PUCCH group limitation as a PUCCH group may not contain all the cells belongs to the band combination. We are fine with capture per PUCCH group in component 5).

Agreement Following is supported in Rel-18 multi-cell scheduling - A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X. - Up to 4 sets of cells can be configured per PUCCH group. - When multiple sets of cells are configured, - a cell in one set of cells can’t be included in another set of cells. - n_CI value is independently configured for each set of cells. - reference cell for counting DCI size and BD/CCE of DCI format 0_X/1_X is independently determined for each set of cells. - search space configuration of DCI format 0_X/1_X is independently configured for each set of cells. - DCI size of DCI format 0_X is independently determined for each set of cells. - DCI size of DCI format 1_X is independently determined for each set of cells. - The multiple sets of cells can be scheduled by DCI format 0_X/1_X from different scheduling cells. - Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell. - The value of N is reported as UE capability. - An indicator is included in the DCI to indicate the scheduled set of cells, - The size of the indicator is equal to ceil(log2(N)), where N is the number of sets of cells. - Unique n_CI value is configured for each set of cells.

Regarding to whether to report different values for primary and secondary PUCCH group, it may lead to different implementation at UE side, i.e.:

- If UE reports the maximum number of cell sets for primary and secondary PUCCH group respectively, the maximum number can be same or different. More precise UE capability can be achieved.

- If UE reports the maximum number of cell sets per BC, the maximum number should be same across different PUCCH groups if applicable.

To be honest, we don’t have strong preference on whether to allow UE to report the maximum number of cell sets respectively for primary PUCCH group and secondary PUCCH group. We slightly to keep the UE feature simple to avoid unexpected scheduling complexity at gNB side.

Same reason as mentioned in component 4), we don’t see the necessity to report the maximum number of cell sets per UE per PUCCH group for different carrier types separately.

Regarding to the max total number of cells across different sets, it can be derived from the following reported values.

- Max number of co-scheduled cells per set of cells supported by UE

- Max number of sets of cells supported by UE per PUCCH group

One argument is that if the max total number of cells across different sets of cells is derived by Tmax, cell×Tmax, set, UE may need to report conservative values in order to guarantee gNB doesn’t configure something beyond UE’s capability. However, we don’t quite understand the logic as it is UE’s responsibility to report something it is capable.

Component 6): Same comments as aforementioned, we have the following preference on component 6):

- This component is uniquely reported across the reported values in component 3

- There is no need to report max total number of cells, across different sets of cells, supported by UE for a same scheduling cell

- There is no need to report max number of sets of cells supported by UE across PUCCH groups

Component 7): For MC scheduling, Type-1 codebook is exactly the same as legacy. It is natural for a MC UE to support Type-1 CB in order to achieve completed function. On the other hand, Type-2 codebook is enhanced in order to accommodate single DCI scheduling multiple carriers, e.g. a sub-CB is defined for MC DCI HARQ-ACK. From this perspective, we prefer to keep Type-2 codebook as a separate UE feature.

Note: One note is currently capture in FG49-1 as below:

[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]

The note doesn’t seem to provide any useful information as it is well known that 1) SCell scheduling PCell/PSCell is excluded for Rel-18 MC 2) SCell can have only single scheduling cell in the specification.

Hence, we slightly prefer to delete this note to keep the UE feature more readable.

FFS point#1: Regarding to whether to separate this FG for the case when scheduling cell is not the reference cell for the set, we don’t see the necessity. The purpose of defining reference cell is to properly handle BD/CCE budget and DCI size budget within introducing DCI format 0_3 and DCI format 1_3. No matter scheduling cell is reference cell or not, there is no different on BD/CCE determination and DCI size budget sustaining. Same reasons as aforementioned, FG49-1 should not be split because of different carrier types.

FFS point#2: Although DCI format 0_3 and 1_3 are used to schedule multiple UL carriers and DL carriers respectively, they are UE-dedicated DCI format which is scrambled with C-RNTI/MCS-C-RNTI. From this perspective, it is still unicast DCI. Accordingly, DCI format 0_3 and DCI format 1_3 should be treated as unicast DCI.

FFS point#3: Considering DCI format 0_3 and DCI format 1_3 are newly introduced DCI formats for multi-carrier scheduling, they are different from legacy DCI formats. It’s better to introduce new FG for configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set.

Based on the above analyses, we have the following proposal:

Proposal 1: Update FG 49-1 with the following modifications:

| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] FFS whether to report max number of sets of cells supported by UE across PUCCH groups 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

FG 49-1a

As analysed aforementioned, FG 49-1a can be merged to FG 49-1.

FG 49-1b

Component 3b): One concern on bitmap based indication is the signalling overhead would be huge. For example, there are five carrier types for scheduling cell and scheduled cell, i.e. {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}. Bitmap is the most flexible mechanism to indicate the scheduling cell and scheduled cell combination. Regarding to the overhead, it highly depends on the signalling design. For example, the following two bitmap-based options can achieve similar flexibility while signalling overhead is quite different.

- Option 1: each bit in the bitmap indicates whether the related combination is supported between scheduling cell and scheduled cell. In this case, 25 bits are required in order to cover all the combinations.

- Option 2: bitmap is used to indicate which kind(s) of carrier type is supported for scheduling cell and scheduled cell respectively. In this case, 10 bits are required.

As mentioned above, we think option 2 should be the way to go. However, the detail signalling design is up to RAN2. Considering bitmap-based mechanism can significantly improve flexibility, we support bitmap-based indication.

Regarding to the relation between component 3a) and component 3b), we think it is UE’s responsibility to report a proper combination. For example, if UE reports a capability of ‘Scheduling cell of lower SCS and scheduled cells of higher SCS’ in component 3a), it should not report any scheduling cell and scheduled cell combination like {FR2-2, FR1 licensed TDD }. From gNB point of view, it certainly takes both component 3a) and 3b) into account. Hence, we think the current component 3a) and component 3b) is OK.

Component 4) to component 9): Same comments as FG49-1 can be applied here.

Accordingly, we have the following proposal for FG 49-1b:

Proposal 2: Update FG 49-1b with the following modifications:

| 49. NR_MC_enh | 49-1ba | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}. FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

FG 49-2/49-2a/49-2b

For the UE features related to DCI format 0_3, we have similar views as FG 49-1/49-1a/49-1b. Hence, we have the following proposals:

Proposal 3: Update FG 49-2 with the following modifications:

| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}.] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

Proposal 4: Update FG 49-2b with the following modifications:

| 49. NR_MC_enh | 49-2ba | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

For FG 49-3, FG 49-4a, FG 49-4b, FG 49-5, FG 49-5a and FG 49-5b, we are fine with the current version.

UE features for multi-carrier UL Tx switching scheme

Two FG were agreed for supporting UL Tx switching across more than two bands. FG 49-x is quite stable and RAN1 agree to leave the remaining issues to RAN2. For FG 49-Y, the following agreement was achieved:

Agreement:

Introduce FG 49-Y as follows

| 49. NR_MC_enh | 49-Y | Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots | If two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching, and X us is reported with a candidate value set of {[0us], 500us} - The reported value X is applied to both one TAG case and two-TAG case (if UE supports two-TAG case) FFS: Note: If the UE reports [0us], the minimum separation time is not applied | 49-X | Yes |  | [two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands] | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

One of open issue is the consequence if UE doesn’t report FG 49-Y.

There are two interpretations which were explained during email discussion by FL very well:

- Case 3-1: if UE does not report FG49-Y, when two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, the minimum separation time is not applied i.e., same as for the case where two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are only on two bands.

- Case 3-2: if UE does not report FG49-Y, two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands (but the UE can expect two uplink switching can be triggered in two consecutive reference slots for UL transmissions on 2 bands as well as in Rel-16/17).

  - This is rapporteur’s proposed consequence if the FG49-Y is not supported.

The basic logic of UE capability reporting is UE indicateS gNB whether it has something advanced, instead of the other way around. From this perspective, we agree with moderator and support case 3-2.

To be specific, if FG 49-Y is not reported, 1) UE doesn’t expect two uplink swtichings are triggered in two consecutive reference slots for UL transmission on more than 2 bands 2) Rel-16/17 UL Tx switching between two consecutive reference slots is supported without any restriction.

Proposal 5: If UE doesn’t report FG 49-Y, two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands.

Conclusion

In this contribution, we provide our views on UE features for MC. We have the following proposals:

Proposal 1: Update FG 49-1 with the following modifications:

| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] FFS whether to report max number of sets of cells supported by UE across PUCCH groups 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

Proposal 2: Update FG 49-1b with the following modifications:

| 49. NR_MC_enh | 49-1ba | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}. FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

Proposal 3: Update FG 49-2 with the following modifications:

| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}.] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

Proposal 4: Update FG 49-2b with the following modifications:

| 49. NR_MC_enh | 49-2ba | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1,2,3,4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1,2,3,4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |
| --- | --- | --- | --- |

Proposal 5: If UE doesn’t report FG 49-Y, two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands.

Reference

  - RAN1#112bis e-meeting chairman notes

  - R1-2304213, Summary on UE features for MC enhancements, Moderator (NTT DOCOMO, INC.)