3GPP TSG RAN WG1#113	        	R1-2305030

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:					    9.1.4.2

Source:	Sony

Title:	Considerations on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion

# Introduction

Objective 5: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

-	Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

According to WID for Rel-18 MIMO enhancement [1], enhancement for SRI/TPMI is required to enable 8 Tx UE transmission.

In this contribution, dual codeword (CW) transmission in PUSCH with rank>4 and antenna port group assumption in UL 8 Tx are discussed.

# Discussion

## Dual CW transmission in PUSCH with rank>4

Working Assumption

For uplink transmission with rank>4, support dual CW transmission.

In RAN1#110bis meeting [2], the following working assumption was made.

For NR DL, dual CW transmission is supported in the case of rank>4. In higher rank, since different layer tends to be different channel quality, dual CW transmission would be beneficial. By dual CW transmission, different MCS for each CW transmission can be used depending on channel quality on each layer. Therefore, also in NR UL with rank > 4, dual CW transmission should be supported.

Proposal 1: RAN1 should confirm the working assumption

- For uplink transmission with rank>4, support dual CW transmission.

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated.

- FFS: Details on how to signal

In RAN1#112 meeting [3], the following agreement regarding NDI and RV indication to the second CW was made but FFS for details on how to signal was remained.

RAN1#111 meeting made the agreement to strive to reuse Rel-15 NR DL schemes where possible for dual CW transmission on PUSCH. Based on this agreement, a second set of NDI and RV fields for the second CW should be explicitly indicated by UL grant.

Proposal 2: A second set of NDI and RV fields for the second CW should be explicitly indicated by UL grant.

Dual CW transmission would be also beneficial for configured grant PUSCH, assuming that configured grant PUSCH is especially beneficial for a UE for stationary device such as CPE or FWA. If dual CW transmission would be supported for dynamic grant PUSCH, it should be also supported for configured grant PUSCH.

For Type-1 CG, which is fully RRC configured, a second MCS parameter has to be configured by RRC, because there is no DCI indication. For Type-2 CG, which is activated by DCI, a second MCS parameter could be added in the activation DCI for Type-2 CG.

Proposal 3: Configured grant PUSCH with dual CW transmission should be supported

- For Type-1 CG: A second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant

- For Type-2 CG: A second MCS field is added in DCI format activating a Type-2 CG-PUSCH

## Antenna port on 8 Tx partial-coherent UE

Agreement

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7}

- Note: Other alternatives which are not foreseen are not precluded

In RAN1#110bis meeting [2], the following agreement regarding assumption of port coherency was made.

By following the legacy rule of coherent group in DL 8 Tx and UL 4 Tx, a coherent group should contain 2 or 4 closest antennas with different polarizations. In that sense, alt 2 for when Ng=2 and alt 1 for when Ng=4 should be supported.

Proposal 4: For codebook design of an 8TX partial-coherent UE, the following alternative for coherent groups should be supported

- For when Ng=2, the following convention for assumption of port coherency scheme is used

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

- For when Ng=4, the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

# Conclusions

The following Proposals were made in this contribution.

Proposal 1: RAN1 should confirm the working assumption

- For uplink transmission with rank>4, support dual CW transmission.

Proposal 2: A second set of NDI and RV fields for the second CW should be explicitly indicated by UL grant.

Proposal 3: Configured grant PUSCH with dual CW transmission should be supported

- For Type-1 CG: A second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant

- For Type-2 CG: A second MCS field is added in DCI format activating a Type-2 CG-PUSCH

Proposal 4: For codebook design of an 8TX partial-coherent UE, the following alternative for coherent groups should be supported

- For when Ng=2, the following convention for assumption of port coherency scheme is used

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

- For when Ng=4, the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

# References

[1]			 RP-223276	, “WID Update: MIMO Evolution for Downlink and Uplink,” 3GPP TSG RAN Meeting #98-e, December 2022.

[2] RAN1#110bis meeting chairman’s note, October 2022

[3] RAN1#112 meeting chairman’s note, February 2022