3GPP TSG RAN WG1 #113			R1-2305044

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item: 	9.5.3

Source: 	Sony

Title: 	Discussion on LPHAP

Document for: 	Discussion / Decision

# Introduction

A new work item on expanded and improved NR positioning was approved in RANP#98e meeting [1]. One of the objectives related to supporting Low Power High Accuracy Positioning (LPHAP) is given below:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

We made the following selected agreements in RAN1#112b-e meeting [3]:

Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, sequenceID in SRS for positioning configuration is commonly configured across cells within the validity area. For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state: - When the spatial relation information is absent in the configuration, the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources; - When the spatial relation information is provided in the configuration, it is applicable across the cells within the SRS positioning validity area. Further study the configuration details. - FFS: spatial relation information validity criteria, and whether/how to determine UE fallback behavior if validity criteria for spatial relation of the configured RS is not met. For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS. For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support one or multiple of the following alternatives on how to configure pathloss RS (down-selection to be made in RAN1#113 meeting): - Alt. 2-1: Reuse the configuration of pathloss RS in Rel-17; - Reuse the validity criteria of OPLC for SRS transmissions by RRC_INACTIVE UEs in Rel-17, i.e. when the pathloss RS cannot be accurately measured. When the validity criteria of OPLC fail, pathloss is calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB. - Alt. 2-4: Configure a list of candidate pathloss RSs or candidate cells per SRS for positioning resource set. The UE transmits SRS resources in a SRS resource set using a pathloss RS determined from the candidate pathloss RSs based on its DL measurements on the candidate pathloss RSs; - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail. - Alt. 2-5: Reuse the configuration of pathloss RS in Rel-17. The UE only transmits SRS resources in a SRS resource set associated with pathloss RS that can be accurately measured. - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail. - Alt. 2-6: Reuse the configuration of pathloss RS in Rel-17; - A cell-agnostic DL RS can be the pathloss RS in the validity area and the cell-agnostic DL RS itself. - Note: UE power consumption should be considered Working assumption For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the following options: - Option 1: Pathloss RS is absent in the configuration. - Option 2: Pathloss RS is provided in the configuration. For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support that spatial relation information can be absent or present in the configuration.

In this contribution, we discuss some considerations on UL and DL+UL positioning for UEs in RRC_INACTIVE state.

# Discussions

A UE supporting LPHAP is expected to save a significant power consumption feature compared to the legacy UE in NR. Hence, the LPHAP target battery life as discussed during study item phase can be met. On the other hand, some limitations are expected in the operation of LPHAP UE. There are many potential use-cases, for example, on the operation of LPHAP UE in a factory / warehouse with a certain number of base-stations. The position of the LPHAP UE needs to be tracked by the system for certain purposes, such as factory automation, tracking, logistic, etc. The LPHAP UE is expected to only have mobility function within the factory / warehouse. Hence, it will interact with those base-stations (gNB) as illustrated in Figure 1. Here, the LPHAP UE supports mobility with a limited number of gNBs. We assume once the LPHAP UE is outside the factory then the LPHAP UE no longer can utilize the low power capability or may not be operated at all.

In this illustrated indoor scenario, the LPHAP UE transmits uplink sounding reference signal (SRS) for positioning purposes to the gNBs. Each gNB performs positioning measurements, such as UL-TDOA, UL-AoA. The positioning measurements results are transmitted to the LMF. Furthermore, the LMF performs the legacy positioning estimation based on the obtained positioning measurements and the geographical position of the gNBs.

Figure 1: Illustration of LPHAP UE in an indoor factory

During Rel-18 study phase, the target battery life required by LPHAP use case 6 cannot be satisfied when the existing Rel-17 positioning techniques are used for UEs in RRC_INACTIVE state. The enhancements on SRS for positioning have been recommended as one of the solutions to meet the LPHAP target battery life for UL and DL+UL positioning.

As described earlier, the LPHAP is expected to be operated in a certain location (e.g., warehouse, indoor factory) in which there are a few numbers of gNBs. The location or the operation area of a few numbers of gNBs can also be called as validity area. Here, we can consider the operation of LPHAP UE is limited and only interact with those gNBs. Since the LPHAP UE operation is limited and can also be predicted, especially if we consider LPHAP UE operates with certain dedicated function, then the LPHAP UE can be preconfigured with UL SRS configuration. There are limited number of gNBs, so that there could be multiple UL SRS configuration that can be provided to the UE. In the legacy positioning, the UE retrieves the UL SRS configuration when the UE is in CONNECTED mode. When the UE moves to other cell then the UE would require to be in CONNECTED mode to receive the UL SRS configuration of a new cell. It would be beneficial if the UE receives the multiple UL SRS configuration for positioning in advance and the UE can use that when the UE operates in RRC_INACTIVE in that validity area.

Proposal 1: Support multiple UL SRS configuration for positioning for the UE to operates UL and DL+UL positioning in RRC_INACTIVE state.

The configuration can be provided when the UE is in the connected mode via dedicated RRC signalling or it can also be provided using system information. The details of the signalling for conveying the multiple UL SRS configuration can be further discussed in RAN2.

Proposal 2: Multiple UL SRS configuration can be provided to the UE via dedicated RRC (e.g., when the UE is in connected mode) or system information (SI). The details are to be discussed in WG RAN2.

When the LPHAP UE operates in RRC_INACTIVE state, it would be beneficial if the LPHAP UE can be triggered to perform positioning procedure, such as transmitting UL SRS for positioning. In the case of the trigger comes from the network (e.g., LMF), the trigger can be included in downlink signal / channel in RRC_INACTIVE state, such as the paging message. The SRS transmission should be designed in relatively short time after the reception of paging message, such as relative to the start of DRX active time. In this case, after the paging reception, the UE performs RACH procedure in which the timing advance acquiring information can be used to validate the SRS configuration. If the SRS configuration is still valid, the UE continues to perform SRS for positioning.

Proposal 3: Support network activation of UL SRS transmission via downlink signal / channel (e.g., via paging message).

Proposal 4: Support aligning the DRX on duration and UL SRS transmission for positioning

As also shown in Figure 1, the LPHAP UE moves within a validity area that consists of multiple cells. During study item phase, there may be some issues related to the UL SRS transmission in multiple cells scenario. The issues are such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

One of the issues that we would like to consider on the timing advance (TA). In legacy NR, timing advance plays important role for uplink transmission, including UL SRS transmission. The TA value is obtained during random access procedure. The UE transmits preamble (Msg1), gNB calculates the TA, then gNB provides the TA value to the UE via Msg2 during RACH procedure. The UE uses the TA for subsequent transmissions. The TA value is related to the serving cell it was released from.

LPHAP UE can be considered as a moving device within the validity area. Re-obtaining the TA value may require the LPHAP UE to perform RACH procedure, in which it may cost a non-negligible power consumption. On the other hand, not re-obtaining TA value could lead the LPHAP UE to use inaccurate TA and resulting the unsynchronized UL SRS reception at the gNB side. Hence, it would be beneficial for the UE to update the TA value without performing RACH procedure. In RAN1#112 meeting, it was discussed that for the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, there are multiple options on determining the UL timing advance value. We propose the UE to be able to adjust the TA value based on the downlink timing measurement. In the legacy NR, this approach has been used for the UE to be able determine the timing advance of legacy uplink transmission. As for positioning, we consider it may have tighter requirements as an error in timing advance could lead to positioning accuracy degradation. In order to minimize the error in timing advance, the downlink measurement for TA adjustment should be based on DL PRS. DL PRS is specially designed for positioning purpose (e.g., wider bandwidth, good correlation properties). Hence, the downlink timing measurement based on DL-PRS reception could minimize the error in timing advance. The LPHAP UE can perform timing measurement toward one of the gNBs. Regarding the specification impact, it can be further discussed. One potential specification impact is on UE capability, supporting timing measurement based on DL-PRS reception for timing advance.

Observation 1: Re-obtaining the TA value may require the LPHAP UE to perform RACH procedure, in which it may cost a non-negligible power consumption. On the other hand, without re-obtaining TA value could lead the LPHAP UE to use inaccurate TA and resulting the unsynchronized UL SRS reception at the gNB side.

In RAN1#112b-e meeting, there were a related discussion had been discussed in multiple rounds, in which the latest round proposal is as follows [4]:

- For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: - By default, UE maintains the TA obtained from the last serving cell within the validity area, - Note: This assumes that the geometric cell coverage is small. - When the DL reference timing difference exceeds a threshold, UE autonomously adjusts the TA up to its implementation. - The threshold can be provided by the network to control the UE behaviour of autonomous TA adjustment. - The DL reference timing follows the DL timing of current camping cell.

In principle, we support the above feature lead proposal as it is aligned with our view.

Proposal 5: Support UE to be able to autonomously adjusts the TA. The UE can use the timing measurement based on DL-PRS reception.

We should consider the mechanism for the UE to adjust the TA value should still be controlled by the gNB. For example, some gNBs may not allow the UE to adjust the TA by itself, or the adjusted TA should be still within the valid range, etc.

Proposal 6: The gNB control the adjustment of TA value by the UE, by providing the threshold indicating when the UE can perform autonomous adjustment.

As part of the SRS configuration, we consider it would be beneficial if the spatial relation information is provided in the SRS configuration. The UE is configured with the specific spatial direction behavior, such as gNB SRS for positioning reception behavior. In this case, there can be an association between SSB resources and UL-SRS resources. This association can be for each gNB. Here, the UE initially performs the legacy SSB measurements for multiple cells. Once the UE has identified the best spatial direction of the received SSB, the UE is expected to use the same beam for the UL-SRS for positioning transmission. In RAN1#112b-e meeting, a working assumption was made so that for the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support that spatial relation information can be absent or present in the configuration. We are fine with the working assumption. It may have a flexibility whether the configuration can be provided or not. The UE may use the default configuration if it absent (i.e., use a fixed spatial domain filter) and follow the configuration in case it is provided.

Proposal 7: Confirm the working assumption on spatial relation of an SRS for positioning information can be absent or present in the configuration.

# Conclusion

In this contribution we provide our considerations on LPHAP. We made the following observations and proposals:

Observation 1: Re-obtaining the TA value may require the LPHAP UE to perform RACH procedure, in which it may cost a non-negligible power consumption. On the other hand, without re-obtaining TA value could lead the LPHAP UE to use inaccurate TA and resulting the unsynchronized UL SRS reception at the gNB side.

Proposal 1: Support multiple UL SRS configuration for positioning for the UE to operates UL and DL+UL positioning in RRC_INACTIVE state.

Proposal 2: Multiple UL SRS configuration can be provided to the UE via dedicated RRC (e.g., when the UE is in connected mode) or system information (SI). The details are to be discussed in WG RAN2.

Proposal 3: Support network activation of UL SRS transmission via downlink signal / channel (e.g., via paging message).

Proposal 4: Support aligning the DRX on duration and UL SRS transmission for positioning

Proposal 5: Support UE to be able to autonomously adjusts the TA. The UE can use the timing measurement based on DL-PRS reception.

Proposal 6: The gNB control the adjustment of TA value by the UE, by providing the threshold indicating when the UE can perform autonomous adjustment.

Proposal 7: Confirm the working assumption on spatial relation of an SRS for positioning information can be absent or present in the configuration.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, RANP #98e, December 2022

- TR 38.859, “Study on expanded and improved NR position (Release 18)”, December 2022

- RAN1 #112b-e Meeting Chairman’s notes, e-Meeting, April 2023.

- R1-2304002, Summary #3 for low power high accuracy positioning, April 2023