3GPP TSG RAN WG1 #113	R1-2305531

# Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:		9.9.3

Source:				Samsung

Title:					Disabling of HARQ feedback for IoT NTN

Document for:	Discussion

# Introduction

A WID on further enhancements for NB-IoT/eMTC in non-terrestrial networks (NTN) was approved in RAN#92-e [1]. One objective is to specify disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2].

Good progress was achieved in RAN1#112/112bis-e [2] for the signaling. This contribution discusses remaining issues.

# Discussion

Signaling for disabling HARQ feedback

In RAN1#112bis-e [2], the following was agreed:

Agreement For Option 3 DCI indication: - Option A: when both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration - For single TB scheduled by DCI, the DCI based overridden indication is applied to one of the following options (to be down-selected): - Option A-1: only applied to semi-statically HARQ disabled processes - Option A-4: applied to both semi-statically HARQ disabled and enabled processes - FFS for multiple TBs scheduled by single DCI - Option B: DCI-based HARQ enabling/disabling direct indication in case DCI solution enabling/disabling signaling is configured and per-HARQ process bitmap signaling is not configured (i.e. no bitmap is configured) - DCI-based mechanism is DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission - For single TB scheduled by DCI, DCI-based direct indication is applied to the scheduled TB - FFS for multiple TBs scheduled by single DCI Agreement For single TB scheduled by DCI, For DCI-based direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc - Option 1: Indication by adding one field in DCI (e.g., 1-bit) - Note: Other fields in DCI are the same as legacy. - Option 2: Indication by reusing/reinterpreting existing field in DCI - Option 2A: HARQ-ACK related field - For eMTC CE mode B, one state of “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource. - FFS: detailed state - For NBIoT, one state of “HARQ-ACK resource” field in DCI format N1 is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource. - FFS: detailed state - Option 2B: MCS or repetition number field - Reduce 1bit of legacy MCS or repetition number field and add 1bit new field in DCI format 6-1B and N1 to indicate the HARQ feedback enabled/disabled - FFS: detailed for interpreting of the reduced MCS or repetition number field - Option 2C: HARQ-ACK related field v2 - For eMTC CE mode B, reduce 1bit of legacy “HARQ-ACK resource offset” field and add 1bit new field in DCI format 6-1B to indicate the HARQ feedback enabled/disabled - FFS: detailed for interpreting of the reduced “HARQ-ACK resource offset” field - For NBIoT, reduce 1bit of legacy “HARQ-ACK resource” field and add 1bit new field in DCI format N1 to indicate the HARQ feedback enabled/disabled - FFS: detailed for interpreting of the reduced “HARQ-ACK resource” field - Option 2D: Other indication by reusing/reinterpreting existing field

Regarding Option A, for the down selection between Option A-1: only applied to semi-statically HARQ disabled processes, and Option A-4: applied to both semi-statically HARQ disabled and enabled processes, for the sake of progress and to conclude this discussion, Option A-4 can be adopted. Option A-1 can be realized by implementation if Option A-4 is adopted (up to eNB whether to send the indication or not for any of the HARQ processes).

Regarding Option B, since the DCI-based signaling is a direct indication (not a switch) it would apply independently of other configurations. Thus, there is no need for Option B.

Proposal 1: For Option 3 DCI indication, Option A-4 is adopted.

Proposal 2: For Option 3 DCI indication, Option B is not needed.

In the first agreement above from RAN1#112bis-e, it is FFS for multiple TBs scheduled by single DCI. Unless a reason is identified to discuss different solutions for single and multiple TBs, the solution that will be agreed for single TB can be applied also to multiple TBs.

Proposal 3: The solution for Option 3 DCI indication that will be agreed for single TB also applies to multiple TBs scheduled by single DCI.

Regarding the indication in the DCI format, it was agreed in RAN1#112bis-e to select between adding one field in the DCI format and one of the several solutions for reusing/reinterpreting an existing field in DCI, based on criteria of DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc. Adding a new 1-bit field in DCI may seem to be a clean solution, but it adds an overhead that may not be justified for such simple feature, and creates the need for further DCI design that would have a larger specification impact and also increase UE complexity. Thus, reusing existing fields is preferred.

Number of HARQ feedbacks to disable

The number of HARQ processes for NB-IoT can be 1 or 2, hence the benefits of disabling HARQ-ACK reports for NB-IoT might be smaller than for eMTC or NR. Whether to allow disabling the HARQ-ACK reports for the case of single HARQ process can be further discussed. Also whether to allow disabling HARQ-ACK reports for all processes when more than one HARQ process is configured can be further discussed. A disadvantage of disabling HARQ-ACK reporting is an inability of the gNB to perform open-loop link adaptation and also having to rely on costly higher layer ARQ for retransmissions and/or higher PDSCH reception reliability while an advantage can be some UE power savings. That can be left to gNB implementation and enabling/disabling all/some of the HARQ-ACK reports for PDSCH receptions would be a gNB choice.

Proposal 4: For IoT NTN, enabling/disabling HARQ feedback for downlink transmission is supported for single HARQ process and for multiple HARQ processes.

Potential enhancements to assist the network for enabling/disabling HARQ feedback

Another aspect to consider is to report additional information by a UE in order to assist the network in configuring enabling/disabling HARQ-ACK reporting. As discussed during the Rel-17 SI, the additional information can be signaling that a sufficient number of repetitions has already been transmitted, requesting a number of repetition, reporting buffer status for HARQ operation, explicit indication to request enabling/disabling HARQ reports, etc.

Proposal 5: Further discuss the reporting of additional information by the UE.

# Conclusion

This contribution discusses remaining aspects related to timing relationship enhancements. Proposals made in this contribution are summarized below.

Proposal 1: For Option 3 DCI indication, Option A-4 is adopted.

Proposal 2: For Option 3 DCI indication, Option B is not needed.

Proposal 3: The solution for Option 3 DCI indication that will be agreed for single TB also applies to multiple TBs scheduled by single DCI.

Proposal 4: For IoT NTN, enabling/disabling HARQ feedback for downlink transmission is supported for single HARQ process and for multiple HARQ processes.

Proposal 5: Further discuss the reporting of additional information by the UE.

# References

- RP-220979, WID on IoT NTN enhancements, RAN#95-e, March 2022

- RAN1 Chair’s Notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023

- RAN1 Chair’s Notes, RAN1#112bis-e, April 17th – April 26th, 2023