3GPP TSG-RAN WG1 Meeting #113 	 R1-2305746

Incheon, Korea, May 22nd – 26th, 2023

Title:	Draft reply LS to RAN4 on RS supported for group-based reporting

Response to:	R4-2306394/R1-2304311

Release:	Release 18

Work Item:	NR_feMIMO-Core

Source:	RAN1

To:	RAN4

CC: 	RAN2

Contact Person:

#### Name:	<name>

E-mail Address:	<email address>

1. Overall Description:

RAN1 would like to thank RAN4 for the question in LS in R4-2306394 on RS supported for group-based reporting.

RAN1 has the following reply to the RAN4 questions:

Question 1: RAN4 understands that two resource sets consisting of SSB + SSB or CSI-RS + CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report. However, RAN4 is not clear whether the two resource sets configured for Rel-17 group-based L1-RSRP measurement and report can contain a mix of SSB and CSI-RS. Can RAN1 clarify whether a mix of SSB and CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report?

Answer 1

Yes, a mix of SSB and CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report. In this respect, there is the following specification support:

[38.331] If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourceForChannelMeasurement, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2).

Question 2: When CSI-RS resources are configured as resource set for Rel-17 group-based L1-RSRP measurement and report, can the CSI-RS resource with repetition ON be configured for Rel-17 group-based L1-RSRP report?

Answer 2

No, CSI-RS resource with repetition ON cannot be configured for Rel-17 group-based L1-RSRP report.

[38.214, section 5.2.1.4] When the higher layer parameter repetition for a CSI-RS Resource Set for channel measurement is set to 'on', CRI for the CSI-RS Resource Set for channel measurement is not reported.

Question 3: Does RAN1 specification define UE reporting behaviour for the case when NW configures UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report (e.g., the measured L1-RSRP of at least one of the beams of any beam pair is lower than a certain value, e.g., -140dBm)?

Answer 2

No, there is no UE behaviour defined for the mentioned scenario. The condition under which reporting is performed, is described by the following:

[38.214, section 5.2.1.4.2]  if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance nrofReportedGroups, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

2. Actions:

To RAN4 group.

ACTION: 	RAN1 requests RAN4 to kindly take the above information into account.

3. Date of Next TSG-RAN WG1 Meeting:

RAN1#114	August 21th – 25th, 2023			Toulouse, FR

RAN1#114-bis	October 09th – 13th, 2023			Xiamen, CN