3GPP TSG RAN WG1 Meeting #113  			    			             R1-2305796

Incheon, Korea, May 22nd – May 26th, 2023

# Source:	ETRI

# Title:		Discussion on co-channel coexistence for LTE sidelink and NR sidelink

# Agenda item:	9.4.2

# Document for:	Discussion

Introduction

In the last RAN1#112bis-e meeting, the following agreements for co-channel coexistence for LTE sidelink and NR sidelink have been made [1]:

Agreement In NR SL resource (re)selection procedure, for dynamic resource pool sharing, the list of initial SL RSRP thresholds is separately (pre)configured (i.e., Alt 3 in the agreement of RAN1#112 meeting) for the PHY layer of NR SL module to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE. - FFS: whether a different initial SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH. Agreement In NR SL resource (re)selection procedure, for determining the LTE SL periodic reserved resources by other LTE SL UE, the formula of Q in Section 8.1.4 in TS 38.214 is used. - FFS: whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas Note: The reservation period and the location of the LTE SL periodic reserved resources are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213. Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe (in which UE has not monitored due to its transmission) in LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - The same formula of Q for determining the LTE SL periodic reserved resources by other SL UE is used - Note: The reservation period and the location of the LTE SL resources repeated Q times in the above procedure are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213 - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether the set SA is initialized to the set of all the candidate single-slot resources excluding NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframes when the amount of candidate single-slot resources is not sufficient as specified in Step 5a in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in NR SL slots that overlap with LTE SL resource pool where NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframes of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: The NR SL module excludes NR SL candidate resources overlapping with NR SL resources (including NR SL resource reservation period(s) (pre)configured in a NR SL resource pool) associated with non-monitored slot as specified in Section 8.1.4 of TS 38.214 (i.e., NR SL slot in which UE has not monitored due to its own transmission), which is the existing Rel-16/17 NR SL behavior - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module Agreement In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in a NR SL slot where NR SL periodic resources are in the NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR SL module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - Alt 1: The above procedure is applied at least when the priority of LTE SL transmission is higher than the priority of NR SL transmission - It is up to UE implementation whether or not to apply the above procedure when the priority of LTE SL transmission is not higher than the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module Agreement For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot Agreement When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing - FFS: whether/how to differently handle the case when different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected. Agreement RAN1 does not pursue further enhancements except for the following to handle NR SL candidate resources overlapping with LTE PSCCH for non-adjacent LTE SL resource pool in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. - NR SL module uses LTE SL RSRP measurement results (as specified in TS 36.213) shared from LTE SL module to determine excluding NR SL candidate resources overlapping with LTE PSCCH and/or LTE PSSCH resources reserved by other LTE SL UE in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain, Down-selection one of followings: - Option 1: - When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - The list of the above initial SL RSRP thresholds is separately (pre)configured - FFS: whether this initial SL RSRP threshold list can be (pre)configured per subset of PSFCH resources - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR SL module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE and the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether to apply the above procedure based on the priority of the LTE SL reserved resources and/or the priority of the NR SL transmission - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE in time domain according to Step 5 in Section 8.1.4 of TS 38.214. - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module Agreement The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module. - The starting LTE SL subframe is no later than the time (n-T_start) - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214 - Down-selection one of followings for T_start: - Option 1-1: T_start is T_0 as defined in clause 8.1.4 of TS 38.214 - Option 1-2: T_start is 1100ms - Option 1-3: T_start is up to UE implementation - The ending LTE SL subframe is no earlier than the time (n-T_valid2) - Down-selection one of followings for T_valid2: - Option 2-1: T_valid2 is T - Option 2-2: T_valid2 <= T+4ms - Option 2-3: T_valid2 is up to UE implementation

Based on the above agreements, we introduce our views on the co-channel coexistence of LTE sidelink and NR sidelink. According to the endorsed proposals in [2], the coexistence objective should be completed by June 2023. The revised WID limits the supporting SCSs to 15 and 30kHz, and provides solutions for supporting the SCSs as shown in the above. Therefore, we focus on the remaining issues on dynamic resource sharing in this contribution.

Co-channel coexistence of LTE sidelink and NR sidelink

Higher SCS

In addition to 15kHz, only 30kHz is allowed for dynamic resource pool sharing between LTE SL and NR SL. For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, it is up to UE implementation how to ensure the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. The remaining one is whether same or different frequency allocation may be used in the second overlapping slot. We do not see any critical reason for different handling depending on frequency allocation.

Proposal 1: For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe regardless of frequency locations for the NR PSCCH/PSSCH transmissions.

When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is also UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe. Even when different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected, we do not see any specific reason to handle the case differently.

Proposal 2: When different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe, it is also up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe.

Candidate resource determination of NR SL module

Based on the previous agreements, for determination of candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE(s) in NR SL resource (re-)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission. For the list of the above initial SL RSRP threshold, it was agreed that the list of initial SL RSRP thresholds is separately (pre)configured. For determining the LTE SL periodic reserved resources by other LTE SL UE, it was also agreed that the formula of Q in Section 8.1.4 in TS 38.214 is used. One remaining FFS point is whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas. Given that pre-emption and re-evaluation are supported in NR SL, further optimization is not necessary in order to minimize specification impact.

Proposal 3: It is not necessary to use the formula of Q in Section 14.1.1.6 in TS 36.213 additionally to derive the largest value between two formulas.

Regarding whether the set SA is initialized to the set of all the candidate single-slot resources excluding NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframes when the amount of candidate single-slot resources is not sufficient as specified in Step 5a in Section 8.1.4 of TS 38.214, it seems not necessary to be adopted. This may result in infinite loop in resource selection procedure, and the NR SL performance may degrade seriously. This would also provide excessive protection to LTE.

Proposal 4: It is not necessary to apply any additional condition for resource excluding in Step 5a in Section 8.1.4 of TS 38.214.

Regarding the candidate information shared by LTE SL module for NR SL resource (re-)selection procedure, it is not finalized yet. In our opinion, at least following parameters should be included in the candidate information shared by LTE SL module to perform the agreed NR SL resource selection procedure for dynamic resource pool sharing. Regarding the LTE logical subframe related information we also agree that it is RAN1’s understanding that the NR SL module is aware of the LTE SL resource pool configurations as per Rel-16 in-device coexistence.

Proposal 5: At least the following parameters should be included in the candidate information shared by LTE SL module:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

PSFCH handling

Regarding NR SL PSFCH handling, two options were agreed as shown in the above, and it needs to be down-selected. In case of Option 1, the same procedure regardless of PSFCH handling can be applied. Only different thing is that the separate initial SL RSRP is configured. In case of Option 2, additional procedure needs to be defined, and it may also cause over-exclusion of NR SL resources. Therefore we prefer to Option 1.

Proposal 6: In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain according to Option 1.

Timeline of Information Shared by LTE SL Module

Regarding the timeline to pass the candidate information from LTE SL module to NR SL module, it was agreed that the latest time for sharing is n-T which T ≤ Tmax ms, and Tmax is 4ms considering Rel-16 in-device coexistence framework. In addition to the latest time, the agreement on the timeline for the shared information, such as the starting and ending LTE SL subframes, was made in the last meeting. However, there are still several options for the starting LTE SL subframe and the ending LTE SL subframe respectively which need to be down-selected. Regardless of actual NR sensing window e.g., 1100ms or 100ms, the staring LTE SL subframe, i.e., Tstart should be determined to cover LTE sensing window since the shared information is on the LTE SL reserved resources. Among the options, only Option 1-2 can guarantee to cover LTE sensing window. Regarding Tvalid2 which is the time for the ending LTE SL subframe used to determine the information shared with the NR SL module, we already have the latest time constraint, i.e., T ≤ Tmax. and the latest time will be determined within the constraint by UE implementation. Therefore, we think no need to define Tvalid2 and it can be also determined by UE implementation considering the latest time.

Proposal 7: The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-Tstart)

  - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214

  - Option 1-2: Tstart is 1100ms

- The ending LTE SL subframe is no earlier than the time (n-Tvalid2)

  - Option 2-3: Tvalid2 is up to UE implementation

Summary

In this contribution, we made the following proposals to support co-channel coexistence between LTE sidelink and NR sidelink.

Proposal 1: For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe regardless of frequency locations for the NR PSCCH/PSSCH transmissions.

Proposal 2: When different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe, it is also up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe.

Proposal 3: It is not necessary to use the formula of Q in Section 14.1.1.6 in TS 36.213 additionally to derive the largest value between two formulas.

Proposal 4: It is not necessary to apply any additional condition for resource excluding in Step 5a in Section 8.1.4 of TS 38.214.

Proposal 5: At least the following parameters should be included in the candidate information shared by LTE SL module:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

Proposal 6: In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain according to Option 1.

Proposal 7: The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-Tstart)

  - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214

  - Option 1-2: Tstart is 1100ms

- The ending LTE SL subframe is no earlier than the time (n-Tvalid2)

  - Option 2-3: Tvalid2 is up to UE implementation

References

- Chairman’s notes, RAN1#112bis-e, April 2023.

- RP-230769, “Way forward on R18 SL-Evo WI objectives,” OPPO