3GPP TSG-RAN WG1 Meeting #113	 R1-2306078

Incheon, Korea, May 22 – May 26, 2023

Agenda Item:	9.1.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion and Decision

- Introduction

For 8TX UL operation, the agreement achieved in last meeting can be found in [1].

In this contribution, we discuss SRI and TPMI enhancements to enable 8TX UL transmission.

- SRI/SRS enhancement for enabling 8TX UL transmission

This section discusses the enhancement of SRI for NCB PUSCH.

  - NCB based UL transmission

Agreement For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively. Agreement For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - Support Option 2 where a legacy-based solution is used by extending the existing SRI indication tables to include NSRS=8 and lmax=8 Agreement For 8TX UE supporting dual CW PUSCH (Maximum number of layers configured for the UE is larger than 4) - Alt1 – DL principle is reused for disabling transmission of a transport block, where - The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable (when transmission rank<=4) transmission of its corresponding TB - The enabled transport block is mapped to the first CW. - Note: When the transmission of a transport block is disabled, the number of layers is ≤ 4. - Note: the first CW refers to the enabled CW.

For NCB based UL transmission, SRI is used to indicate one or more SRS resources from NSRS SRS resources to UE, where UE uses the same precoder of selected SRS resource to transmit PUSCH. For 4TX UE, the UE can only be configured with up to 4 single-port SRS resource for an SRS resource set. For 8TX UE, the number of candidate precoders of 8TX UE is up to 8. In RAN#110bis-e meeting [2], it was agreed to support a single SRS resource set configured with up to 8 single-port SRS resources for SRS configuration. In addition, configuration of up to two, or four SRS resource sets is FFS. However, we fail to see the benefits to use multiple SRS resource sets. Moreover, there will be several issues if multiple SRS resource sets are used. For example, considering two SRS resource sets each with up to 4 SRS resources, if NSRS=6, there can be multiple possibilities in configuration across two SRS resource sets, such as 3+3, 2+4, or 4+2, which needs to be further discussed. Hence, we think it is not reasonable to support multiple SRS resource sets.

Proposal 1: For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, configuration of up to two, or four SRS resource sets is not supported.

In legacy, k (1≤k≤NSRS) SRS resources can be selected from NSRS SRS resource, thus the SRI overhead is  bits, where Lmax is given by the maximum number of layers. For 4TX UE, is up to 4, and the maximum SRI overhead is 4 bits. For 8TX UE, it is agreed in RAN#112 meeting [3] that all SRS port combinations are supported with NSRS>4. For SRI indication, it is agreed in RAN#112bis meeting that a legacy-based solution is used by extending the existing SRI indication tables to include NSRS=8 and Lmax=8. Hence, the overhead of SRI is also  bits, which is up to 8 bits.

In RAN#112bis meeting, it is agreed to reuse DL principle disabling transmission of a transport block, where the combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable (when transmission rank<=4) transmission of its corresponding transport block. Note that when the transmission of a transport block is disabled, the number of layers is ≤ 4, otherwise the number of layers is > 4. Furthermore, when the number of layers is ≤ 4, the 1-bit NDI field corresponding to disabled transport block is not used, which can be considered to reduce SRI overhead.

Proposal 2: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce SRI overhead.

- UL 8TX codebook

In this section, we discuss the codebook design for fully coherent UEs and partially/non-coherent UEs.

  - Fully coherent codebook

In RAN1#112 meeting, Rel-15 DL Type I single-panel codebook with (O1, O2) = (1, 1) is supported to generate fully coherent precoding matrices. There are two remaining issues to be discussed. The first one is how to solve phase alignment error between antennas, and the second one is how to indicate fully coherent precoders.

    - Phase alignment error between antennas

The Rel-15 DL type I single-panel codebook is optimized for highly correlated dual-polarized antennas, which is based on DFT matrix. It is agreed in RAN1 #109-e [5] that 8 TX antenna layout follows the regular structure of ULA or UPA. For ULA structure, Rel-15 DL type I single-panel codebook assumes that there is a linear phase ramp across the 4 antenna ports in one polarization. For UPA structure, Rel-15 DL type I single-panel codebook assumes that the phase difference between the former 2 antenna ports is equal to that between the latter 2 antenna ports. However, as discussed in RAN1#110bis-e meeting, such assumption may not be satisfied in practice due to impairment of UE implementation, where each of the 4 antenna ports in one polarization has a random phase offset. According to the simulation results reported in RAN1#111 meeting [6], such random phase offset leads to large performance loss when using Rel-15 DL type I single-panel codebook. Hence, Rel-15 DL type I single-panel codebook needs to be enhancement to compensate for the impairment of random phase offset.

A straightforward way to solve this issue is phase pre-compensation. Specifically, for an 8TX codeword w=[w1,w2,w3,w4,φw1,φw2,φw3,φw4]Tselected from Rel-15 DL type I single-panel codebook, we can introduce three phase pre-compensation coefficients σ1, σ2 and σ3 to constitute a new codeword as

w'=[w1,σ1w2,σ2w3,σ3w4,φw1,φσ1w2,φσ2w3,φσ3w4]T

where σ1, σ2 and σ3 are respectively used to pre-compensate the phase offset between 2nd, 3rd, 4th antenna ports and 1st antenna port. Typically,

σ1=exp(jθ1),   θ1=0:2πN1:(N1-1)2πN1

σ2=exp(jθ2),   θ2=0:2πN2:(N2-1)2πN2

σ3=exp(jθ3),   θ1=0:2πN3:(N3-1)2πN3

where N1, N2 and N3 are used to adjust the phase granularity.

In the following evaluation, we denote the UL 8TX fully coherent codebook based on Rel-15 DL type I single-panel codebook as “DL type I”, and that based on enhanced Rel-15 DL type I single-panel codebook as “DL type I with phase error compensation”.

System-level simulation is performed to compare the performance of two candidate codebooks for UL 8TX. In the simulation, the UE antenna layouts agreed in RAN1#109-e meeting are used, as shown in Table 1. In RAN1#110bis-e meeting, fully-coherent precoders with Ng=1 is agreed. Hence, we just consider the Case 1 with Ng=1 in Table 1.

Table 1 Exemplary cases of UE antenna layout

| Case | Ng | (N1, N2, P) per group | Antenna Layout |
| --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2) (4, 1, 2) |  |
| 2 | 2 | (2, 1, 2) |  |
| 3 | 4 | (1, 1, 2) |  |

For “DL type I” and “DL type I with phase error compensation”, we adopt O1=1，O2=1, codebookMode=1 for antenna layout 1-a and 1-b. For “DL type I with phase error compensation”, the number of phase pre-compensation coefficients are N1=N2=N3=4. The detailed simulation assumptions can be found in Appendix A, where the antenna setup and port layouts at TRP is (12,4,2,1,1,4,4). To explicitly analyze the effect of phase alignment error, we consider each UE is rank-one.

Figure 1 and 2 show the throughput comparison between “DL type I” and “DL type I with phase error compensation” with random phase offset for antenna layout 1-a and 1-b, respectively. Here, the phase offset values of each antenna port is assumed uniformly distributed over [-μ,μ], where μ takes 0, 45, 90, 135 and 180 degrees. In Figure 1 and 2, we take “DL type I” with μ=0 as the baseline. It can be observed that in all considered cases, the throughput of two codebooks decrease with the increase of μ. However, “DL type I with phase error compensation” achieves larger throughput than “DL type I”, especially for large μ. For “DL type I” codebook, there is 9.28%~15.44% throughput loss with μ=135 and 12.09%~18.44% throughput loss with μ=180. With the help of phase pre-compensation, the throughput loss of “DL type I with phase error compensation” codebook with μ=180 reduces to 1.81%~4.75%. Table 2 summarizes the throughput gains of “DL type I with phase error compensation” codebook than “DL type I” codebook.

(a) P0 = -80 dBm                                    (b) P0 = -50 dBm

Figure 1 Throughput comparison between “DL type I” and “DL type I with phase error compensation” with random phase offset for antenna layout 1-a

(a) P0 = -80 dBm                                   (b) P0 = -50 dBm

Figure 2 Throughput comparison between “DL type I” and “DL type I with phase error compensation” with random phase offset for antenna layout 1-b

Table 2 Throughput gains of “DL type I with phase error compensation” codebook than “DL type I” codebook

| Phase error | μ=0 | μ=45 | μ=90 | μ=135 | μ=180 |
| --- | --- | --- | --- | --- | --- |
| 1-a with P0 = -80 dBm | 102.60% | 103.99% | 108.05% | 111.50% | 112.51% |
| 1-a with P0 = -50 dBm | 103.63% | 105.20% | 111.50% | 118.99% | 119.73% |
| 1-b with P0 = -80 dBm | 102.21% | 103.17% | 106.48% | 108.43% | 108.84% |
| 1-b with P0 = -50 dBm | 102.97% | 103.92% | 108.35% | 110.62% | 111.78% |

Observation 1: For fully coherent 8TX codewords based on “DL type I” codebook, there will be large throughput loss due to phase alignment error, such as 9.28%~15.44% throughput loss with μ=135 and 12.09%~18.44% throughput loss with μ=180.

Observation 2: For fully coherent 8TX codewords, the throughput gains of “DL type I with phase error compensation” than that of “DL type I” are 2.21%~3.63% for μ=0, 3.17%~5.20% for μ=45, 6.48%~11.50% for μ=90, 8.43%~18.99% for μ=135 and 8.84%~19.73% for μ=180.

Since fully coherent 8TX codewords based on DL type I codebook has large throughput loss with phase alignment error, and such throughput loss can be compensated by using enhanced DL type I codebook with phase pre-compensation. Hence, we have the following proposal.

Proposal 3：For fully coherent precoder by an 8TX UE, support to enhance DL Type I SP codebook to mitigate the impact of phase alignment error.

    - Fully coherent precoder indication

The following Table 3 summarizes the number of fully coherent precoders of “DL type I” with O1=1，O2=1.

Table 3 Number of fully coherent precoders

| Rank | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total number of precoders |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| “DL type I” with N1=4，N2=1 | 16 | 32 | 24 | 24 | 8 | 8 | 4 | 4 | 120 |
| “DL type I” with N1=2，N2=2 | 16 | 32 | 24 | 24 | 8 | 8 | 8 | 8 | 128 |

For UL fully coherent precoder indication, one option is to extend legacy 2TX/4TX solution to 8TX, which requires 7-bit indication overhead. Specifically, rank and TPMI are jointly coded, and a large indication table with 128 entries will be specified.

Another option is to separately indicate rank and TPMI. Firstly, similar to the above discussion for NCB, the number of layers≤ 4 or > 4 can be indicated to UE according to the enabling/disabling status of transport blocks. Then, a 2-bit RI (rank indicator) field and a 5-bit TPMI field can be configured in DCI. Such scheme also requires 7-bit indication overhead, and a large indication table covering all ranks is not needed. Hence, we have the following proposal.

Proposal 4：For PUSCH transmission by an 8TX UE, support to separately indicate rank and TPMI for fully coherent precoder.

Proposal 5：For the indication of fully coherent precoder by an 8TX UE, the #layer in {5, 6, 7, 8} is indicated if two TBs are enabled, and #layers in {1, 2, 3, 4} otherwise.

Furthermore, similar to the above discussion for NCB, when the number of layers is ≤ 4, the 1-bit NDI field corresponding to disabled transport block is not used, which can also be considered to reduce the indication overhead of UL 8TX fully coherent precoder.

Proposal 6: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce the indication overhead of UL 8TX fully coherent precoder.

  - Partially/non-coherent codebook

For partially/non-coherent precoders, it is agreed in RAN1#112 meeting to support NR Rel-15 UL 4TX/2TX codebooks for Ng=2/4 respectively.

    - Partially coherent precoders with Ng=2

In RAN1#112 meeting, it is agreed to design partial coherent precoders with Ng=2 based on Rel-15 UL 4TX codebook. A typical precoding structure can be shown as follows:

- For rank = 1

  - A04×18×1 or  04×1A8×1 where A is a rank-1 fully coherent precoding matrix taken from Rel-15 UL 4TX codebook

- For rank = 2, 3, 4

  - A04×rank8×rank or 04×rankA8×rankor A104×rank(A2)04×rank(A1)A28×rank , where A, A1 and A2 are fully coherent precoding matrices taken from Rel-15 UL 4TX codebook, and rank= rank(A) or rank(A1) + rank(A2) is one of 2, 3 or 4

- For rank >4,

  - A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are fully coherent precoding matrices taken from Rel-15 UL 4TX codebook, the rank=rank(A1) + rank(A2) is one of 5, 6, 7 or 8,

where only fully coherent Rel-15 UL 4TX precoders are considered to save overhead.

      - Layer splitting schemes

Furthermore, the layer splitting schemes for each rank needs to be discussed. The following FL proposal agreed in RAN1#112 meeting list all possible layer splitting schemes for each rank.

Proposal 3.3: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated.

In RAN1#112bis meeting, some companies propose to remove some layer splitting cases, as shown in the following working assumption.

Working Assumption For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3).

In addition, some companies propose to further remove (2,1) for rank 3, (3,2) for rank 5 and (4,3) for rank 7, as shown in the following Proposal 3.2.

Proposal 3.2: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with updates: Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3).

To evaluate the performance of full layer splitting and partial layer splitting in various layouts, two antenna groups are arranged as two 4TX ULA arrays with different directions. To accommodate various UE layouts, the direction gap θ can be 0 (pointing the same direction), or 0~180 (with angle between two antenna groups). The detailed simulation assumptions can be found in Appendix A, where the antenna setup and port layouts at TRP is (12,4,2,1,1,4,4), and UE adopts antenna layout 2-b.

Three precoder sets are evaluated:

- Full: precoders with all layer splitting

- Partial 1: precoders with layer splitting in the working assumption

- Partial 2: precoders with layer splitting in proposal 3.2

It can be observed that when θ=0, “Full” scheme has slight gains than “Partial 1” and “Partial 2” schemes. When θ=60,  “Partial 1” and “Partial 2” schemes respectively have 5.63% and 7.92% throughput loss than “Full” scheme. Therefore, the performance is degraded when more layer splitting is removed, especially when there’s angle between panels.

Figure 3 Throughput comparison between three layer-splitting alternatives

Observation 3: For partially coherent 8TX codewords with Ng=2, the throughput loss of two partial layer splitting schemes than full layer splitting scheme are respectively 5.63% and 7.92% when direction gap between two antenna groups is 60.

Hence, to accommodate various UE layouts and dynamically varied channel environments, we prefer to support all possible layer splitting schemes for each rank.

In addition, the motivation of most companies to remove some layer splitting cases is to reduce the number of precoders, and thus to reduce the indication overhead. The following Table 4 summarizes the number of precoders for all layer splitting schemes in the agreement and partial layer splitting schemes in Working Assumption and Proposal 3.2.

Table 4 Number of partially coherent precoders with Ng=2

| r | Number of PC precoders with Ng=2 for all layer splitting schemes | Number of PC precoders with Ng=2 for partial layer splitting schemes in WA | Number of PC precoders with Ng=2 for partial layer splitting schemes in Proposal 3.2 |
| --- | --- | --- | --- |
| 1 | 32 | 32 | 32 |
| 2 | 272 | 272 | 272 |
| 3 | 264 | 264 | 136 |
| 4 | 196 | 68 | 68 |
| 5 | 128 | 64 | 32 |
| 6 | 48 | 16 | 16 |
| 7 | 16 | 16 | 8 |
| 8 | 4 | 4 | 4 |
| ≤4 | 764 | 636 | 508 |
| ≤8 | 960 | 736 | 568 |

It can be seen that there is a reduction of the number of precoders for some ranks by removing some layer splitting schemes, but it still needs 10-bit indication overhead, which means such pruning of layer splitting schemes is unnecessary. Hence, we propose to support all layer splitting schemes.

Proposal 7：For partially coherent precoder with Ng=2 by an 8TX UE, support all possible layer splitting schemes for each rank.

| r | Full | Partial 1 | Partial 2 |
| --- | --- | --- | --- |
| 1 | 32 | 32 | 32 |
| 2 | 272 | 272 | 272 |
| 3 | 264 | 264 | 136 |
| 4 | 196 | 68 | 68 |
| 5 | 128 | 64 | 32 |
| 6 | 48 | 16 | 16 |
| 7 | 16 | 16 | 8 |
| 8 | 4 | 4 | 4 |
| ≤8 | 960 | 736 | 568 |

      - UL precoder indication

For indication of partially coherent precoders with Ng=2, an option is to extend legacy 2TX/4TX solution to 8TX, which requires 10-bit indication overhead and a large indication table. However, this will introduce a very large table and large spec impact.

Another option is to separately indicate rank and TPMI. However, even though the number of layers≤ 4 or > 4 can be indicated to UE according to the enabling/disabling status of transport blocks, 11-bit indication overhead consisting of 2-bit RI and 9-bit TPMI is required.

Note that the partially coherent precoder with Ng=2 by an 8TX UE is based on UL 4TX fully coherent precoder. A more straightforward option is to indicate two 4TX fully coherent precoder using two sub-fields. For each sub-field, a total of 30 4TX fully coherent precoders for rank 1~4 requires 5-bit overhead. Hence, the third option requires the same indication overhead to the first scheme, but it does not need to design a large indication table.

Therefore, we have the following proposal.

Proposal 8：For PUSCH transmission by an 8TX UE, support to indicate two 4TX fully coherent precoder for partially coherent precoder with Ng=2.

Furthermore, for the third scheme, when the number of layers is ≤ 4, the 1-bit NDI field corresponding to disabled transport block is not used, which also can be considered to reduce the indication overhead of UL 8TX partially coherent precoder with Ng=2.

Proposal 9: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce the indication overhead of UL 8TX partially coherent precoder with Ng=2.

    - Partially coherent precoders with Ng=4

Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, Alt1 is supported where - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Further study codebook size reduction

In RAN1#112bis meeting, it is agreed to design partial coherent precoders with Ng=4 based on Rel-15 UL 4TX codebook. A typical precoding structure can be shown as follows:

A102×rankA202×rankA1A202×rankA302×rankA402×rankA302×rankA402×rankA102×rankA202×rankA102×rankA2A302×rankA402×rankA3A4

where rank=rank(A1) + rank(A2) + rank(A3) + rank(A4) is one of 1~ 8, rank(A1), rank(A2), rank(A3) and rank(A4) can be one of 0~2, and A1, A2, A3, and A4 are empty matrices or fully coherent precoding matrices taken from Rel-15 UL 2TX codebook.

      - Layer splitting schemes

Furthermore, the layer splitting schemes for each rank needs to be discussed. The following agreements in RAN1#112bis meeting list all possible layer splitting schemes for each rank.

Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - The following rank and layer splitting cases are supported, Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups. 1; (1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1). 2; (2,0,0,0), (0,2,0,0), (0,0,2,0), (0,0,0,2). 2; Transmission by 2 of the 4 antenna groups: (1,1,0,0), (1,0,1,0), (1,0,0,1) (0,1,1,0), (0,1,0,1), (0,0,1,1). 4; (1,1,1,1). 4; Transmission by 2 of the 4 antenna groups: (2,2,0,0), (2,0,2,0), (2,0,0,2) (0,2,2,0), (0,2,0,2), (0,0,2,2). 8; (2, 2, 2, 2). Note: Above is not relevant to how precoders are indicated. Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - In addition to the previously agreed cases, down-select from the rank and layer splitting cases listed below Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2).

Table 5 lists the number of partially coherent precoders with Ng=4 for all layer splitting cases, which leads to 12-bit indication overhead. Intuitively, it is not necessary to use so much overhead for partially-coherent precoders with Ng=4, where only two antenna ports can be performed with coherent transmission.

Table 5 Number of partially coherent precoders with Ng=4 for all layer splitting cases

| Rank | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | <=4 | <=8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC precoders with Ng=4 | 16 | 104 | 352 | 664 | 704 | 416 | 128 | 16 | 1136 | 2400 |

To reduce DCI overhead, we can consider to prune layer splitting. It can be seen from the above Table 5 that rank 4-6 have larger number of precoders. Hence, we prefer to prioritize the pruning the layer splitting of rank 4, rank 5 and rank 6.

For rank 4, we prefer to remove all unbalanced layer splitting cases of transmission by 3 of the 4 antenna groups, which can reduce to 280 precoders.

For rank 5, considering the probability that one antenna group out of the four has bad channel quality, it should be enough to support the first 4 layer splitting cases (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1) of transmission by 3 of the 4 antenna groups in the agreement. For transmission by 4 of the 4 antenna groups, we prefer to support only two layer splitting cases, such as the first 2 layer splitting cases (1,1,2,1), (1,1,1,2). With such pruning, the number of precoders of rank 5 can reduce to 320 precoders.

For rank 6, all layer splitting cases of transmission by 3 of the 4 antenna groups need to be supported. For transmission by 4 of the 4 antenna groups, we prefer to support only four layer splitting cases, such as the first 4 layer splitting cases (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2). With such pruning, the number of precoders of rank 6 can reduce to 288 precoders.

To sum up, Table 6 lists the number of partially coherent precoders with Ng=4 for partial layer splitting cases. It can be seen that 1-bit indication overhead can be saved through the above layer splitting pruning.

Table 6 Number of partially coherent precoders with Ng=4 for partial layer splitting cases

| Rank | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | <=4 | <=8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC precoders with Ng=4 | 16 | 104 | 352 | 280 | 320 | 288 | 128 | 16 | 752 | 1504 |

Proposal 10: For partially coherent precoder with Ng=4 by an 8TX UE, for rank 1-3 and 7-8, supporting all layer splitting cases; for rank 4-6, supporting to prune layer splitting cases as

- Rank 4: remove all layer splitting cases of transmission by 3 of the 4 antenna groups

- Rank 5: only support (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1) of transmission by 3 of the 4 antenna groups and (1,1,2,1), (1,1,1,2) of transmission by 4 of the 4 antenna groups

- Rank 6: only support all layer splitting cases of transmission by 3 of the 4 antenna groups and (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2) of transmission by 4 of the 4 antenna groups

      - UL precoder indication

For indication of partially coherent precoders with Ng=4, we can consider the options of joint, separate indication of rank and TPMI, or indication 4 TPMIs similar to Ng=2. Therefore, further discussion is needed.

    - Non-coherent precodes

Agreement For non-coherent uplink precoding with rank≤8 by an 8TX UE, down-select from - Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported - Alt2. – Only a subset of Alt1. is supported, striving for a substantial reduction in the number of precoders

In RAN1#112bis meeting, it is agreed to design non-coherent precoders by down selecting from two alternatives. We prefer Alt 2 because we think it is not necessary to support all 255 combinations. However, a lot of effort will be required to identify which precodes can be removed. Considering the workload, we prefer to apply the above precoding structures for Ng=2 with slight change to non-coherent precoders, where A, A1 and A2 are non-coherent precoding matrices taken from Rel-15 UL 4TX codebook. Furthermore, the rank 2 Rel-15 UL 4TX non-coherent precoding matrices can be restricted to reduce indication overhead.

Proposal 11：For non-coherent precoder by an 8TX UE, support to design precoders based on Rel-15 UL 4TX non-coherent codebook.

- UL precoder indication enhancement

In this section, we discuss the precoder indication.

  - Discussion on field size of DCI precoder indication field

In the existing NR specification, TPMI and TRI are jointly encoded to indicate the selected codeword and rank to UE, which is indicated by the “Precoding information and number of layers” field in DCI. For 4TX UE in Rel-15, the UL transmission layer per UE is up to 4. When the UL transmission layer for 8TX UE is increased to 8, the indication of TPMI and TRI will require a larger field than the legacy. In the existing NR specification, the maximum overhead of “Precoding information and number of layers” field is 6 bits. If UL 8TX codebook is designed based on Rel-15 UL 4TX codebook and reuse the legacy TPMI and TRI indication rule, the maximum size of “Precoding information and number of layers” field may reach up to 12 bits. If UL 8TX codebook is designed based on Rel-15 DL type I codebook with antenna selection vector and reuse the legacy PMI indication rule, the maximum indication overhead may exceed 12 bits.

Observation 4: To support 8TX CB based UL transmission, the field size of TPMI indication field is increased by at least twice than legacy.

Although the indication overhead can be reduced by reducing the number of codewords, such as lower oversampling ratios for DL type I and fixed rank for UL 4TX, the performance will be degraded.

  - Discussion on high-resolution UL precoder

To reduce the DCI payload, the existing Rel-15 UL 2TX/4TX codebook and Rel-15 DL Type I codebook, as well as the above-mentioned UL 8TX codebook have limited number of codewords, resulting in a low-resolution precoder. 8TX introduces more spatial degree of freedom compared to 2TX and 4TX, which can improve the UL throughput. To maximize the throughput improvement introduced by 8TX, high-resolution precoder should be considered.

Figure 4 and 5 show UL system-level simulation results for 8TX UE with up to 8 layers. The detailed simulation assumptions can be found in Appendix A, where the antenna setup and port layout at UE is (2,2,2,1,1,2,2). Network layout is 1*3 cell, 10 UE per cell. Here we consider DL type 1 single-panel codebook with as a basis with O1=4，O2=4 and codebookMode=1. The label of EigenVector denotes that the precoder of each UE is from the eigenvectors of SVD decomposition of the channel matrix, which represents the upper bound of high-resolution precoder. It can be observed that “EigenVector” precoder can obtain 20~33% throughput gain compared with codebooks of “DL type I”.

(a) 64QAM                                           (b) 256QAM

Figure 4 Throughput for DL type I and EigenVector precoder at 32R

(a) 64QAM                                           (b) 256QAM

Figure 5 Throughput for DL type I and EigenVector precoder at 64R

Observation 5: High-resolution precoder such as eigenvector precoder can obtain 20~33% throughput gain compared with that based on DL type I for UL 8TX.

  - Beamformed CSI-RS based UL precoder indication

As we have discussed above, the field size of DCI indication field for indicating 8TX codebook is increased by at least twice than legacy, that is at least 12 bits, which is a large overhead. In addition, high-resolution precoder will lead to unacceptable indication overhead with the legacy DCI indication method. Hence, it is essential to consider new UL precoder indication method to resolve the above issue.

Beamformed CSI-RS can resolve the issues of large DCI overhead, where CSI-RS is used to indicate the selected precoder by TRP to UE, instead of traditional DCI. Specifically, TRP transmits a beamformed CSI-RS with precoder WDL. Then after the TRP-UE channel H, UE can receive the CSI-RS as a vector PUL with dimension nRxx1 , where nRx is the number of receive antenna ports at UE. Then the received vector can be used as a UL precoder for UE.

PUL=HWDL

By choosing the precoder WDL of CSI-RS, the selected precoder PUL can be indicated to UE with high precision. Because the CSI-RS overhead is not related to the number of codewords, we can design a high-resolution codebook with large codebook size. Even TRP can directly indicate a high-resolution precoder (such as “EigenVector” precoder) to UE without codebook. Note that the considered beamformed CSI-RS based UL precoder indication requires reciprocity between UL and DL channels, which can be used for TDD networks.

To indicate the precoder of one layer, the above beamformed CSI-RS based UL precoder indication requires 1 CSI-RS port. For 8TX UL transmission with up to 8 layers, 8 CSI-RS ports are required, which occupies 8 resource elements. If we use the 8 resource elements to transmit DCI in PDCCH with QPSK modulation, the number of bits to be transmitted is typically 8*2*0.2=3.2, where 0.2 is a typical value of code rate in PDCCH. Compared to the DCI indication overhead in the above-mentioned UL 8TX codebook, the overhead of beamformed CSI-RS based UL precoder indication is greatly reduced.

With the beamformed CSI-RS, the overhead can be reduced and the resolution of indication can be increased. Therefore, we have the following proposal.

Proposal 12: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

- Conclusions

The contribution discusses the SRI, TPMI (including codebook) enhancement for enabling 8TX UL transmission, based on which the following observations and proposals are made.

Observation 1: For fully coherent 8TX codewords based on “DL type I” codebook, there will be large throughput loss with phase alignment error, such as 9.28%~15.44% throughput loss with μ=135 and 12.09%~18.44% throughput loss with μ=180.

Observation 2: For fully coherent 8TX codewords, the throughput gains of “DL type I with phase error compensation” than that of “DL type I” are 2.21%~3.63% for μ=0, 3.17%~5.20% for μ=45, 6.48%~11.50% for μ=90, 8.43%~18.99% for μ=135 and 8.84%~19.73% for μ=180.

Observation 3: For partially coherent 8TX codewords with Ng=2, the throughput loss of two partial layer splitting schemes than full layer splitting scheme are respectively 5.63% and 7.92% when direction gap between two antenna groups is 60.

Observation 4: To support 8TX CB based UL transmission, the field size of TPMI indication field is increased by at least twice than legacy.

Observation 5: High-resolution precoder such as eigenvector precoder can obtain 20~33% throughput gain compared with that based on DL type I for UL 8TX.

Proposal 1: For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, configuration of up to two, or four SRS resource sets is not supported.

Proposal 2: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce SRI overhead.

Proposal 3：For fully coherent precoder by an 8TX UE, support to enhance DL Type I SP codebook to mitigate the impact of phase alignment error.

Proposal 4：For PUSCH transmission by an 8TX UE, support to separately indicate rank and TPMI for fully coherent precoder.

Proposal 5：For the indication of fully coherent precoder by an 8TX UE, the #layer in {5, 6, 7, 8} is indicated if two TBs are enabled, and #layers in {1, 2, 3, 4} otherwise.

Proposal 6: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce the indication overhead of UL 8TX fully coherent precoder.

Proposal 7：For partially coherent precoder with Ng=2 by an 8TX UE, support all possible layer splitting schemes for each rank.

Proposal 8：For PUSCH transmission by an 8TX UE, support to indicate two 4TX fully coherent precoder for partially coherent precoder with Ng=2.

Proposal 9: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce the indication overhead of UL 8TX partially coherent precoder with Ng=2.

Proposal 10: For partially coherent precoder with Ng=4 by an 8TX UE, for rank 1-3 and 7-8, supporting all layer splitting cases; for rank 4-6, supporting to prune layer splitting cases as

- Rank 4: remove all layer splitting cases of transmission by 3 of the 4 antenna groups

- Rank 5: only support (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1) of transmission by 3 of the 4 antenna groups and (1,1,2,1), (1,1,1,2) of transmission by 4 of the 4 antenna groups

- Rank 6: only support all layer splitting cases of transmission by 3 of the 4 antenna groups and (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2) of transmission by 4 of the 4 antenna groups

Proposal 11：For non-coherent precoder by an 8TX UE, support to design precoders based on Rel-15 UL 4TX non-coherent codebook.

Proposal 12: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

# References

- Chairman’s Notes RAN1#112bis-e, April 17th –April 26th, 2023.

- Chairman’s Notes RAN1#110bis-e, October 10th – 19th, 2022.

- Chairman’s Notes RAN1#112, February 27th – March 3rd, 2023.

- Chairman’s Notes RAN1#111, November 14th – 18th August 22th – 26th, 2022.

- Chairman’s Notes RAN1#109-e, May 9th – 20th, 2022.

- R1-2210917, Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Huawei, HiSilicon, RAN1#111, 2022.

# Appendix A

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5G |
| Channel Model | According to the TR 38.901 |
| Scenario | Uma with 500 m ISD |
| Antenna setup and port layouts at TRP | (12,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (12,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| Antenna setup and port layouts at UE | (2,2,2,1,1,2,2) , (1,2,2,2,1,1,2) , (1,1,2,2,2,1,1) (1,4,2,1,1,1,4) , (1,2,2,1,2,1,2) , (1,1,2,1,4,1,1) (𝑑H, 𝑑V) = (0.5, 0.5)𝜆 (𝑑G-H, 𝑑G-V) = (2, 2)𝜆 |
| TRP antenna height | 30m |
| TRP receiver noise figure | 5dB |
| UE Tx power | 23 dBm |
| UE antenna height | 3m |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| Modulation | up to 256QAM |
| Bandwidth | 20 MHz |
| MIMO scheme | SU-MIMO |
| Rank | Rank adaptation with up to 8 layers |
| Traffic model | Full buffer |
| UE distribution | 100% outdoor (3km/h) |
| BS receiver | MMSE-IRC |
| Network Layout | 1*3 cell, 30 UE per cell |
| Precoder granularity | Wideband |
| Scheduling granularity | 4RB |
| Power control | Open loop; alpha = 0.8; P0 = -50, -80 dBm |

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5G |
| Channel Model | According to the TR 38.901 |
| Scenario | Uma with 500 m ISD |
| Antenna setup and port layouts at TRP | (12,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| Antenna setup and port layouts at UE | (1,2,2,1,2,1,2) (𝑑H, 𝑑V) = (0.5, 0.5)𝜆 (𝑑G-H, 𝑑G-V) = (2, 2)𝜆 |
| TRP antenna height | 30m |
| TRP receiver noise figure | 5dB |
| UE Tx power | 23 dBm |
| UE antenna height | 3m |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| Modulation | up to 256QAM |
| Bandwidth | 20 MHz |
| MIMO scheme | SU-MIMO |
| Rank | Rank adaptation with up to 8 layers |
| Traffic model | Full buffer |
| UE distribution | 100% outdoor (3km/h) |
| BS receiver | MMSE-IRC |
| Network Layout | 1*3 cell, 30 UE per cell |
| Precoder granularity | Wideband |
| Scheduling granularity | 4RB |
| Power control | Open loop; alpha = 0.8; P0 = -50 dBm |