3GPP TSG RAN WG1 #113		                                   					       												                          R1-2306164

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.16.11

Agenda Item:	9.16.11

Document for: 	Endorsement

### UE features for IoT NTN enhancements

Initial input on UE features for IoT NTN to be provided by rapporteur.

Agreement: Introduce the following new FGs

| 2. IoT_NTN_enh | 2-1a | Semi-static HARQ feedback disabling for eMTC CE mode B | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process 2. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Candidate values: {dynamic, semi-static, dynamic + semi-static} Note: HARQ disabling with Option 1 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1b | Dynamic HARQ feedback disabling for eMTC CE mode B | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 in CE mode B Note: this applies to single-TB case FFS: whether to merge 2-1a and 2-1b FFS: whether to have a separate FG for the combination of FGs 2-1a and 2-1b [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1d | Semi-static HARQ feedback disabling for eMTC CE mode A | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode A cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-2 | Semi-static HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | UE reports ACK/NACK for the first SPS PDSCH after activation if enabled, and follow per-process HARQ feedback enabled/disabled configuration otherwise | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE Mode A cannot disable HARQ feedback for SPS PDSCH | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A FFS: whether to merge 2-1d and 2-2 [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

Proposal: Introduce the following new FG

| 2. IoT_NTN_enh | 2-1e | Semi-static HARQ feedback disabling for NB-IoT | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process 2. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Candidate values: {dynamic, semi-static, dynamic + semi-static} Note: HARQ disabling with Option 1 Note: this applies to single-TB case FFS: whether to merge 2-1f and 2-1e FFS: whether to have a separate FG for the combination of FGs 2-1e and 2-1f [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1f | Dynamic HARQ feedback disabling for NB-IoT | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

Agreement: Introduce the following new FGs

| 2. IoT_NTN_enh | 2-3a | GNSS position fix in RRC Connected state for eMTC—triggered | 1. UE reports GNSS position fix time duration for measurement and validation duration at least [during the initial access stage and in connected mode] 2. UE receives eNB GNSS measurement trigger 3. UE re-acquires GNSS in RRC Connected state 4. UE re-acquires GNSS position fix within a configured gap | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] Note: This applies to non-DRX | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-4a | GNSS position fix in RRC Connected state for eMTC—autonomous | [1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger] 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | [Rel. 18 2-3a] | Yes | N/A | Release 18 eMTC UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX FFS: merge 2-4a with 2-3a [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

Agreement: Introduce the following new FGs

| 2. IoT_NTN_enh | 2-3b | GNSS position fix in RRC Connected state for NB-IoT—triggered | 1. UE reports GNSS position fix time duration for measurement and validation duration at least [during the initial access stage and in connected mode] 2. UE receives eNB GNSS measurement trigger 3. UE re-acquires GNSS in RRC Connected state 4. UE re-acquires GNSS position fix within a configured gap | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot get triggered GNSS position fix in RRC Connected state [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-4b | GNSS position fix in RRC Connected state for NB-IoT—autonomous | [1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger] 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | [Rel. 18 2-3b] |  |  | Release 18 NB-IoT UE cannot get autonomous GNSS position fix in RRC Connected state [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] FFS: merge 2-3b with 2-4b | Optional with capability signalling |

R1-2304669	UE features for IoT NTN enhancements	Huawei, HiSilicon

R1-2304944	Summary of UE features for IoT NTN enhancements	Moderator (AT&T)

R1-2305281	On UE Features for Rel-18 IoT NTN Enhancements	Apple

R1-2305374	UE features for IOT NTN	Qualcomm Incorporated

R1-2305447	Discussion on UE features for IoT NTN enhancements	OPPO

R1-2305562	Discussion on the UE feature for IoT-NTN	ZTE

R1-2305725	Initial views on UE features for IoT NTN enhancements	Nokia, Nokia Shanghai Bell

R1-2305807	Initial input on UE features for Rel-18 IoT NTN enhancements	Rapporteur (MediaTek Inc.)

R1-2305809	On UE features for IoT NTN enhancements	Ericsson