| 3GPP TR 38.858 V0.4.1 (2023-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Evolution of NR Duplex Operation (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Introduction	10

5	Objectives of study	10

6	Subband non-overlapping full duplex (SBFD)	11

6.1	General aspects of SBFD schemes	11

6.1.1	SBFD Operations	11

6.1.1.1 Semi-static configuration of SBFD subbands	12

6.1.1.2 SBFD operation in symbols configured as DL in TDD-UL-DL-ConfigCommon	12

6.1.1.3 SBFD operation in symbols configured as flexible in TDD-UL-DL-ConfigCommon	12

6.1.2	Impact and potential enhancements for transmissions and receptions	13

6.2	Inter-gNB CLI handling schemes specific for SBFD	14

6.3	Inter-UE CLI handling schemes specific for SBFD	14

7	Performance evaluation and its feasibility for SBFD	15

7.1	Deployment scenarios	15

7.2	Evaluation methodologies	16

7.2.1	System level simulation	16

7.2.2	Link level evaluation	18

7.3	Performance evaluation results	20

7.3.1	System level simulation results	20

7.3.2	Link level evaluation results	20

8	Potential enhancements and analysis for dynamic/flexible TDD	21

8.1	Deployment scenarios	21

8.2	Evaluation methodologies	21

8.2.1	System level simulation	21

8.3	Inter-gNB CLI handling schemes	22

8.3.x	Inter-gNB CLI scheme x	22

8.3.x.1 Description of scheme	22

8.3.x.2 Performance evaluation or analysis	22

8.3.x.3 RAN1 specification impacts	23

8.4	Inter-UE CLI handling schemes	23

8.4.x	Inter-UE CLI scheme x	23

8.4.x.1 Description of scheme	23

8.4.x.2 Performance evaluation or analysis	23

8.4.x.3 RAN1 specification impacts	23

9 	Implementation feasibility of SBFD	23

9.1 	Background for analysis	23

9.2.1	Self-interference analysis	23

9.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis	23

9.2.3	Co-channel inter-sub-band inter-site interference analysis	24

9.2.4	Summary	24

9.3 	Feasibility of FR1 Medium Range BS aspects	24

9.3.1	Self-interference analysis	24

9.3.2	Co-channel inter-sub-band co-site inter-sector interference analysis	24

9.3.3	Co-channel inter-sub-band inter-site interference analysis	24

9.3.4	Summary	24

9.4 	Feasibility of FR1 Local Area BS aspects	24

9.4.1	Self-interference analysis	24

9.4.2	Co-channel inter-sub-band inter-site interference analysis	24

9.4.3	Summary	24

9.5 	Feasibility of FR2 BS aspects	24

9.5.1	Self-interference analysis	24

9.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis	24

9.5.3	Co-channel inter-sub-band inter-site interference analysis	25

9.5.4	Summary	25

9.6. 	FR1 Feasibility of UE aspects	25

9.6.1	Interference analysis	25

9.6.1.1	UE-UE co-channel inter-subband CLI modeling	25

9.6.1.2	UE-UE adjacent channel CLI modeling	25

9.6.2	Summary	25

9.7 	FR2 Feasibility of UE aspects	25

9.7.1	Interference analysis	25

9.7.1.1	UE-UE co-channel inter-subband CLI modeling	25

9.7.1.2	UE-UE adjacent channel CLI modeling	25

9.7.2	Summary	25

9.8 	Summary	25

10	 Impact on RF requirements	26

10.1	Impact on BS RF requirements	26

10.2	Impact on UE RF requirements	26

11	Adjacent channel co-existence evaluation results	26

12	Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum	26

12.1	Region 1	26

12.2	Region 2	26

12.3	Region 3	26

13	Conclusions and recommendations	26

Annex <A>: Evaluation methodologies	27

A.1	Layout and UE distribution	27

A.1.1	Layout	27

A.1.2	UE distribution	31

A.2	Interference modelling	32

A.2.1	gNB self-interference modelling	33

A.2.2	Co-site inter-sector co-channel inter-subband CLI	34

A.2.3	Inter-site gNB-gNB co-channel inter-subband CLI	34

A.2.4	UE-UE co-channel inter-subband CLI	36

A.2.5	Co-site gNB-gNB adjacent-channel CLI	38

A.2.6	Inter-site gNB-gNB adjacent-channel CLI	39

A.2.7	UE-UE adjacent-channel CLI	39

A.2.8	BS noise figure model	40

A.3	Channel modelling	40

A.4	Performance metrics	45

A.4.1	UPT related performance metrics	45

A.4.2	Latency related performance metrics	45

A.4.3	RU and Unfinished/dropped FTP packets rate	46

A.5	gNB antenna configuration and transmit power for SBFD	46

A.6	Traffic model	50

A.7	SBFD subband and slot configurations	51

A.8	Coupling loss definition	52

Annex <B>: System level simulation	52

B.1	System level simulation assumptions	52

B.2	System level simulation results	55

Annex <C>: System level simulation calibration	56

C.1	SLS calibration methodology	56

C.2	SLS calibration assumptions	57

Annex <D>: Link level evaluation for coverage performance	60

D.1	Link level simulation assumptions	60

D.2	Link budget template	62

D.3	Link level evaluation results	62

Annex <E>: Adjacent channel co-existence evaluation	62

E.1	RAN4 co-existence simulation scenarios	62

E.2	RAN4 co-existence simulation assumption	62

E.3	RAN4 co-existence simulation methodology	62

Annex <X>: Change history	63

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures the results and findings from the study item "Study on Evolution of NR Duplex Operation " [2]. The purpose of this TR is to document the follows for evolution of NR duplex operation:

-	applicable and relevant deployment scenarios.

-	evaluation methodology and assumptions.

-	possible schemes/enhancements, feasibility and performance evaluation results of subband non-overlapping full duplex and dynamic/flexible TDD.

-	summary of the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum.

This activity involves the Radio Access work area of the 3GPP studies and has potential impacts both on the Mobile Equipment and Access Network of the 3GPP systems.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-213591, New SI: Study on evolution of NR duplex operation.

[3]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[4]	3GPP TR 36.843: "Study on LTE Device to Device Proximity Services; Radio Aspects".

[5]	3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

[6]	3GPP TR 36.873: "3D channel model for LTE".

[7]	3GPP TR 36.814: "Further advancements for E-UTRA physical layer aspects".

[8]	3GPP TR 36.889: "Study on Licensed-Assisted Access to Unlicensed Spectrum".

[9]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[10]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[11]	Report ITU-R M.2412: "Guidelines for evaluation of radio interface technologies for IMT-2020".

[12]	RP-180524, Summary of calibration results for IMT-2020 self evaluation.

[13]	3GPP TR 38.830: "Study on NR coverage enhancements".

[14]	R1-2304212, Summary on SLS calibration results for NR duplex evolution

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACIR	Adjacent Channel Interference Ratio

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AOA	Azimuth angle Of Arrival

AOD	Azimuth angle Of Departure

AS	Angular Spread

ASA	Azimuth angle Spread of Arrival

ASD	Azimuth angle Spread of Departure

CDF	Cumulative Distribution Function

CLI	Cross link interference

EIRP	Effective Isotropic Radiated Power

IBE	In-Band Emission

ICS	In Channel Selectivity

ISD	Intersite Distance

LOS	Line Of Sight

MIMO	Multiple-Input-Multiple-Output

NLOS	Non-LOS

O2I	Outdoor-to-Indoor

O2O	Outdoor-to-Outdoor

OFDM	Orthogonal Frequency-Division Multiplexing

PRB	Physical Resource Block

RMa	Rural Macro

RSI 	Ratio of self-interference

RSRP	Reference Signal Received Power

RU	Resource Utilization

Rx	Receiver

SBFD	Subband non-overlapping Full Duplex

SI	Self-Interference

SINR	Signal-to-Interference-plus-Noise Ratio

SLS	System Level Simulation

TBoMS	TB processing over multi-slot PUSCH

TRP	Transmission Reception Point

Tx	Transmitter

TxRU	Transceiver Unit

UMa	Urban Macro

UMi	Urban Micro

UPT	User Perceived Throughput

ZOA	Zenith angle Of Arrival

ZOD	Zenith angle Of Departure

ZSA	Zenith angle Spread of Arrival

ZSD	Zenith angle Spread of Departure

# 4	Introduction

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.

The NR TDD specifications allow the dynamic/flexible allocation of downlink and uplink in time and CLI handling and RIM for NR were introduced in Rel-16. Nevertheless, further study may be required for CLI handling between the gNBs of the same or different operators to enable the dynamic/flexible TDD in commercial networks. The inter-gNB CLI may be due to either adjacent-channel CLI or co-channel-CLI, or both, depending on the deployment scenario. One of the problems not addressed in the previous releases is gNB-to-gNB CLI.

This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

# 5	Objectives of study

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

-	Duplex enhancement at the gNB side

-	Half duplex operation at the UE side

-	No restriction on frequency ranges

The detailed objectives are as follows:

-	Identify applicable and relevant deployment scenarios (RAN1).

-	Develop evaluation methodology for duplex enhancement (RAN1).

-	Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

-	Identify possible schemes and evaluate their feasibility and performances (RAN1).

-	Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

-	Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

-	Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

-	Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

-	Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

-	Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

-	Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

# 6	Subband non-overlapping full duplex (SBFD)

Editor's note: This section captures the schemes of SBFD and their feasibility from specification design point of view. gNB self-interference handling schemes (e.g., spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc.) and their feasibilities can be discussed in RAN4 and captured in section 9. Subsections may be added or removed under this section depending on the discussion in RAN1.

## 6.1	General aspects of SBFD schemes

Editor's note: This section captures the general aspects of SBFD schemes except self-interference, inter-gNB and inter-UE CLI handling schemes.

### 6.1.1	SBFD Operations

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction. For discussion purpose, SBFD symbol is defined as symbol with subbands that gNB would use for SBFD operation.

SBFD operation within a TDD carrier is studied and SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

-	The UL subband can be located at one side of the carrier.

-	The UL subband can be located at the middle part of the carrier

Whether SBFD operation in SSB symbols is supported or not is studied in RAN1.

RAN1 studies whether or not a slot can consist of both SBFD and non-SBFD symbols including

-	Benefits

-	Use cases

-	Scheduling flexibility

-	Implementation complexity

-	Compatibility with legacy TDD DL/UL configuration

The time and frequency location of subbands within a TDD carrier are not fixed in the specification. Regarding whether to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation, the following alternatives are studied with Alt 4 prioritized at least for RRC_CONNECTED state.

-	SBFD operation Alt 1:

-	Time and frequency locations of subbands for SBFD operation are not known to UEs.

-	UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

-	SBFD operation Alt 2:

-	Time and frequency locations of subbands for SBFD operation are not known to UEs.

-	UE behaviors for non-SBFD aware UEs follow existing specifications.

-	From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

-	SBFD operation Alt 3:

-	Only time location of subbands for SBFD operation is known to SBFD aware UEs.

-	UE behaviors for non-SBFD aware UEs follow existing specifications.

-	From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

-	SBFD operation Alt 4:

-	Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

-	UE behaviors for non-SBFD aware UEs follow existing specifications.

-	From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Among the four alternatives, SBFD operation Alt 4 is agreed as the baseline for SBFD operation at least for RRC_CONNECTED state.

#### 6.1.1.1	Semi-static configuration of SBFD subbands

For indication of subband locations for SBFD operation, semi-static configuration of subband time and frequency location is studied as baseline. For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline. For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. Furthermore, for the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. For semi-static configuration of subband location, same subband frequency resources across different SBFD symbols are considered as baseline.

#### 6.1.1.2	SBFD operation in symbols configured as DL in TDD-UL-DL-ConfigCommon

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

-	UL transmissions within UL subband are allowed in the symbol

-	UL transmissions outside UL subband are not allowed in the symbol

-	Frequency locations of DL subband(s) are known to the SBFD aware UE

-	The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

-	DL receptions within DL subband(s) are allowed in the symbol

-	Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

In addition, whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon for SBFD aware UEs are studied based on the following options:

-	Option 1 (semi-static SBFD): DL receptions outside semi-statically configured DL subband(s) are not allowed

-	Option 2: (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed

#### 6.1.1.3	SBFD operation in symbols configured as flexible in TDD-UL-DL-ConfigCommon

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following alternatives are studied for SBFD aware UEs,

Alt 1:

-	UL transmissions within UL subband are allowed in the symbol

-	UL transmissions outside UL subband are not allowed in the symbol

-	Frequency locations of DL subband(s) are known to the SBFD aware UE

-	DL receptions within DL subband(s) are allowed in the symbol

-	FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Alt 2:

-	UL transmissions within UL subband are allowed in the symbol

-	The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

-	FFS: SBFD aware UE behaviours

-	FFS: Whether or not signalling of guardband(s) is needed

-	FFS: Whether or not the symbol can be converted to a DL-only symbol

-	Frequency locations of DL subband(s) are known to the SBFD aware UE

-	DL receptions within DL subband(s) are allowed in the symbol

Note:	UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

In addition, whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in a symbol configured as flexible in TDD-UL-DL-ConfigCommon for SBFD aware UEs are studied based on the following options:

-	Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

-	Option 2 (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed

-	UL transmissions outside the semi-statically configured UL subbands are not allowed

-	Option 3 (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed

-	UL transmissions outside the semi-statically configured UL subbands are allowed

### 6.1.2	Impact and potential enhancements for transmissions and receptions

Impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following, are studied:

-	PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

-	Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

-	Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

-	Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

-	Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note:	Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

-	Resource allocation in frequency domain including frequency hopping

-	Resource allocation in time domain

-	Power domain

-	Spatial domain

RAN1 studies the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

-	RBG for PDSCH RA type 0

-	CSI reporting configuration

-	CSI-RS resource configuration

-	PRG of PDSCH

For SBFD-aware UEs, at least the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands are studied. For an RBG that overlaps the subband boundary,

-	Option 1:

-	Part of the DL RBG inside the DL subband can be used

-	Part of the UL RBG inside the UL subband can be used

-	Option 2:

-	Part of the DL RBG inside the DL subband cannot be used

-	Part of the UL RBG inside the UL subband cannot be used

Frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs are studied considering the following options:

-	Option 1: Two contiguous CSI-RS resources that are linked

-	Option 2: One CSI-RS resource

-	Option 2-1: Non-contiguous CSI-RS resource allocation

-	Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

For SBFD-aware UEs, at least the following issues for PDSCH are studied:

-	PRG(s) with size of 2 and 4 that overlaps with subband boundary

-	Wideband precoder in case of non-contiguous DL subbands

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following options are studied for SBFD-aware UEs:

-	Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

-	Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

-	PDSCH/PUSCH/PUCCH repetitions

-	SPS PDSCH/CG PUSCH

-	TBoMS

-	Multi-PUSCH/PDSCH scheduled by a single DCI

-	Periodic/semi-persistent SRS/CSI-RS/PUCCH

-	PDCCH

For SBFD-aware UEs, the following options are studied for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

-	Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

-	Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, at least the followings are studied:

-	Whether/how to have separate resources

-	Whether/how to have separate FH parameters

-	Whether/how to have separate UL power control parameters

-	Whether/how to have separate beam/spatial relation

## 6.2	Inter-gNB CLI handling schemes specific for SBFD

Editor's note: This section captures the potential inter-gNB CLI handling schemes that are specific for SBFD.

## 6.3	Inter-UE CLI handling schemes specific for SBFD

Editor's note: This section captures the potential inter-UE CLI handling schemes that are specific for SBFD.

For inter-UE inter-subband CLI measurement, at least the following methods are studied:

-	Method#1: victim UE measures RSSI within DL subband

-	Method#2: victim UE measures RSRP of aggressor UE within UL subband

-	Method#3: victim UE measures RSSI within UL subband

-	Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

# 7	Performance evaluation and its feasibility for SBFD

## 7.1	Deployment scenarios

Editor's note: This section captures a summary of the identified applicable deployment scenarios for feasibility and performance evaluation, covering both FR1 and FR2, as well as non-coexistence scenarios, co-channel co-existence scenarios and adjacent-channel co-existence scenarios.

The following deployment cases are considered for evaluation:

-	SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

-	SBFD Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations.

-	SBFD Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration.

-	Deployment Case 3-1: Only 1-layer is considered

-	Deployment Case 3-2: 2-layer is considered. Layer 1 uses legacy static TDD operation, Layer 2 uses SBFD operation. All the gNBs in Layer 2 use the same SBFD subband configuration.

-	SBFD Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration.

Note that SBFD subband configuration is from gNB perspective.

For SBFD Deployment Case 1, the following scenarios are considered:

-	FR1

-	1-layer scenario

-	Indoor office

-	Urban Macro

-	(Optional) Dense Urban Macro layer

-	2-layer scenario

-	(Optional) Dense Urban with 2-layer

-	FR2-1

-	1-layer scenario

-	Indoor office

-	Dense Urban Macro layer

-	(Optional) Dense Urban Micro layer

For SBFD Deployment Case 3-2, the following scenarios are considered:

-	FR1

-	2-layer Scenario B

-	Layer 1: Urban Macro

-	Layer 2:

-	Baseline: Indoor office

-	Optional: Indoor factory

For SBFD Deployment Case 4, the following scenarios are considered:

-	FR1

-	1-layer scenario

-	Urban Macro, considering 0% and 100% grid shift between two networks.

-	FR2-1

-	1-layer scenario

-	Dense Urban Macro layer, considering 0% and 100% grid shift between two networks.

The layouts and UE distributions for these scenarios can be found in Annex A.1.

## 7.2	Evaluation methodologies

### 7.2.1	System level simulation

Editor's note: This section captures evaluation methodologies and assumptions of system level simulation for SBFD operation.

Interference Modelling

The modelling methods for the following interference types can be found in Annex A.2.

-	gNB Self-Interference (SI)

-	co-site inter-sector co-channel inter-subband CLI

-	inter-site gNB-gNB co-channel inter-subband CLI

-	UE-UE co-channel inter-subband CLI

-	inter-site gNB-gNB adjacent-channel CLI

-	co-site gNB-gNB adjacent-channel CLI

-	UE-UE adjacent-channel CLI

Channel model

The details of gNB-UE channel model, gNB-gNB channel model, and UE-UE channel model can be found in Annex A.3.

For gNB-gNB and UE-UE channel model in RAN1 SLS, large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies to report whether small scale fading (e.g., fast fading including antenna gain) is also modelled. Antenna gain is calculated based on the gNB-gNB LOS direction instead on the multi-path directions if small scale fading is not modelled.

Performance metrics

The following metrics are considered. The detailed definitions can be found in Annex A.4.

-	UPT related performance metrics

-	Mean/5%/50%/95% Average-UPT, Average-UPT CDF

-	Mean/5%/50%/95% Tail-UPT, Tail-UPT CDF

-	Mean/5%/50%/95% Median-UPT, Median-UPT CDF

-	Latency related performance metric

-	Baseline: Mean/5%/50%/95% Packet-Latency, Packet-Latency CDF

-	Optional: Mean/5%/50%/95% UE-Average-Latency, UE-Average-Latency CDF

-	Unfinished/dropped Packet Rate

-	RU

-	Type-1 RU

-	Type-2 RU

gNB Antenna configuration

The detailed gNB antenna configurations for SBFD evaluation can be found in Annex A.5.

Traffic model

FTP model 3 is used and the details can be found in Annex A.6.

SBFD subband and slot configurations

The following SBFD subband configurations are considered for SBFD evaluation:

-	SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth.

-	SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

The detailed SBFD subband configurations as well as the SBFD/legacy TDD slot configurations for evaluation can be found in Annex A.7.

SLS assumptions

The SLS assumptions common to SBFD and dynamic/flexible TDD can be found in Table B.1-1 in Annex B.

The SLS assumptions specific to SBFD Deployment Case 1 can be found in Table B.1-2 in Annex B.

The SLS assumptions specific to SBFD Deployment Case 3-2 can be found in Table B.1-3 in Annex B.

The SLS assumptions specific to SBFD Deployment Case 4 can be found in Table B.1-4 in Annex B.

### 7.2.2	Link level evaluation

Editor's note: This section captures evaluation methodologies and assumptions of link level evaluation for SBFD. Link level evaluation can at least be used to evaluate coverage performance for SBFD operation.

Link level evaluation is performed to evaluate coverage performance of SBFD. It is up to companies to use SLS as a tool to evaluate coverage performance of SBFD, and it is also up to companies to perform link level evaluation for other purposes.

For link level evaluation of coverage performance, RAN1 focuses on Urban Macro scenario for FR1 and Dense Urban Macro Layer scenario for FR2-1. Regarding the target uplink channel for coverage evaluation, RAN1 focuses on PUSCH with 1Mbps target data rate for FR1 and 5Mbps target data rate for FR2-1.

For link level evaluation of coverage performance, single slot PUSCH transmission is assumed for baseline legacy TDD, and the following schemes of coverage enhancement can be assumed for SBFD:

-	Scheme-1: SBFD with PUSCH repetition type A

-	Scheme-2: SBFD with TBoMS PUSCH

-	Scheme-3: SBFD with PUSCH repetition type A and joint channel estimation

-	Scheme-4: SBFD with TBoMS PUSCH and joint channel estimation

-	For Scheme-3 and Scheme-4, two options are considered:

-	Option 1 (baseline): joint channel estimation is applied only for the same symbol type

-	Option 2: joint channel estimation is applied across SBFD and non-SBFD slots

For link level evaluation of coverage performance, MPL, MCL and MIL defined in TR38.830 are used as the performance metrics. Similar evaluation methodology as defined in TR38.830 (i.e., LLS + Link budget analysis) can be used, and the performance metrics are obtained using link budget analysis and TDD/SBFD required SINR for target data rate.

For link level simulation of coverage performance, RAN1 should consider self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference in TDD system and SBFD system. The following modelling methods can be considered.

Option-1:

-	The modelling method is as below:

-	For TDD UL slot, additive white Gaussian noise with variance of NTDD is generated, where NTDD=IUE-gNB+N0

-	IUE-gNB is UE-gNB interference and N0 is noise (in linear scale).

-	For SBFD slot, additive white Gaussian noise with variance of NSBFD is generated, where NSBFD=ISI+Ico-site+IgNB-gNB+IUE-gNB+N0

-	ISI, Ico-site, IgNB-gNB, IUE-gNB are self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference (in linear scale), respectively

-	Companies to report the details of deriving NTDD and NSBFD. Some examples are as below:

-	Example-1: IUE-gNB and IgNB-gNB are derived based on a certain assumption of the topology of gNBs and UEs. In this example, the interference is pre-receiver interference.

-	Note: link budget analysis can be applied in this example

-	Example-2: ∆=10*log10NSBFD-10*log10NTDD is derived based on statistic in SLS, and then ∆ is used in LLS to increase the Gaussian noise power in SBFD symbol compared to TDD UL symbol. In this example, the interference is post-receiver interference.

-	Example-3: NTDD and NSBFD can be derived based on statistic in SLS. In this example, the interference is post-receiver interference.

-	Companies to report the RU assumption for the interference.

-	Note: For simplicity, the interference is independently updated/generated in each slot.

-	Note: Companies are encouraged to report whether and how channel estimation and interference estimation will be impacted by IUE-gNB and IgNB-gNB.

-	Based on the modelling method, the following high-level evaluation method can be used as an example for coverage performance evaluation:

-	Step 1: For legacy TDD system, assume the SNR in UL only slot is SNRTDD=SNTDD, perform LLS to get the required SNR (SNRTDD) with which UE can achieve a certain bit rate in UL

-	Step 2: For SBFD system with frame structure XXXXU, assume the SNR in UL only slot is SNRSBFD-UL=SNTDD and the SNR in SBFD slot is SNRSBFD-X=SNSBFD. Perform LLS to get the required SNR (SNRSBFD-UL) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.)

-	Step 3: Use Link budget template to obtain MPL, MCL and MIL for legacy TDD and SBFD.

-	For legacy TDD, the required SNR (SNRTDD) obtained in Step 1 is used to calculate MPL, MCL, MIL.

-	For SBFD, the required SNR (SNRSBFD-UL) obtained in Step 2 is used to calculate MPL, MCL, MIL.

Option-2:

-	The UE-gNB interference and inter-site gNB-gNB co-channel inter-subband CLI in LLS coverage evaluation are explicitly modelled based on a given topology of aggressor UEs and gNBs. The UE-gNB and gNB-gNB fast fading channels are explicitly modelled in LLS. The signal model is as follows

-	Y=HkSk +n≠kHnSn +HgSg+HCLI,mWmSm+Iself+N

-	Y is the received signal vector at the victim gNB

-	Hk is the channel matrix from target UE to gNB, Sk is the transmitted signal of the target user

-	Hn, Sn, n≠k, are the channel matrix and transmitted signal of the UE in the same cell as the target user

-	Hg and Sg are the channel matrix and transmitted signal of the UEs in the adjacent cell

-	HCLI,m, Wm and Sm are the channel matrix, the precoding matrix, and leakage CLI signal from aggressor gNB m to the victim gNB.

-	The power of the signal and interference is included in the channel matrix respectively

-	Iself and N are the self-interference vector of the co-site sectors and the thermal noise signal vector on the receiving antennas

-	Companies to report the topology of gNBs and UEs to derive the detailed signals and interferences above. One example is as below

-	Based on the above modelling, the following high-level evaluation method can be used as an example for coverage performance evaluation:

-	Step 1: For legacy TDD system, perform LLS to get the required SNR (SNRTDD=SN0) with which UE can achieve a certain bit rate in UL

-	Step 2: For SBFD system with frame structure XXXXU, perform LLS to get the required SNR (SNRSBFD-UL=SN0) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.)

-	Step 3: Use Link budget template to obtain MPL, MCL and MIL for legacy TDD and SBFD.

-	For legacy TDD, the required SNR (SNRTDD) obtained in Step 1 is used to calculate MPL, MCL, MIL.

-	For SBFD, the required SNR (SNRSBFD-UL) obtained in Step 2 is used to calculate MPL, MCL, MIL.

For the two options above, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense similar to SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation αco-site.

The link level simulation assumptions for coverage evaluation are provided in Annex D.1. The link budget template for coverage evaluation is provided in Annex D.2.

## 7.3	Performance evaluation results

Editor's note: This section captures the results and observations of performance comparison between baseline legacy TDD network and SBFD operation for each identified scenario of each SBFD deployment case, as well as the feasibility of SBFD from performance point of view.

### 7.3.1	System level simulation results

### 7.3.2	Link level evaluation results

# 8	Potential enhancements and analysis for dynamic/flexible TDD

## 8.1	Deployment scenarios

Editor's note: This section captures a summary of the identified applicable deployment scenarios for feasibility and performance evaluation, covering both FR1 and FR2, as well as non-coexistence scenarios, co-channel co-existence scenarios and adjacent-channel co-existence scenarios.

The following scenarios are considered for dynamic/flexible TDD evaluation for single operator case:

-	FR1

-	1-layer scenario

-	Indoor office

-	(Optional) Urban Macro

-	2-layer Scenario B

-	Layer 1: Urban Macro

-	Layer 2:

-	Baseline: Indoor office

-	Optional: Indoor factory

-	FR2-1

-	1-layer scenario

-	Indoor office

-	(Optional) Dense Urban Macro layer

The layouts and UE distributions for these scenarios can be found in Annex A.1.

## 8.2	Evaluation methodologies

Editor's note: This section captures some common evaluation metrics and assumptions of system level simulation for evaluating the potential enhancement schemes for dynamic/flexible TDD (e.g., inter-gNB CLI handling schemes captured in section 8.3 and inter-UE CLI handling schemes captured in 8.4). It does not imply all the potential enhancement schemes have to be evaluated by system level simulation. If additional assumptions are needed for a specific enhancement scheme, they can be captured in section 8.3.x.2 or 8.4.x.2.

### 8.2.1	System level simulation

Channel model

The channel model for dynamic/flexible TDD evaluation is the same as that for SBFD evaluation which can be found in Annex A.3.

Performance metrics

The following metrics are considered. The detailed definitions can be found in Annex A.4.

-	UPT related performance metrics

-	Mean/5%/50%/95% Average-UPT, Average-UPT CDF

-	Mean/5%/50%/95% Tail-UPT, Tail-UPT CDF

-	Mean/5%/50%/95% Median-UPT, Median-UPT CDF

-	Latency related performance metric

-	Baseline: Mean/5%/50%/95% Packet-Latency, Packet-Latency CDF

-	Optional: Mean/5%/50%/95% UE-Average-Latency, UE-Average-Latency CDF

-	Unfinished/dropped Packet Rate

-	RU: Type-1 RU

Traffic model

FTP model 3 is used and the details can be found in Annex A.6.

Assumption on baseline and target dynamic/flexible TDD operation for comparison

|  |  | Target dynamic/flexible TDD operation | Baseline operation for comparison |
| --- | --- | --- | --- |
| 1-layer scenario |  | Using dynamic TDD UL/DL assignment based on potential enhancements | Using dynamic TDD UL/DL assignment based on Rel-17 specifications |
| 2-layer Scenario (NOTE 1) | Option 1 | Layer 2 using legacy static TDD {DSUUU} based on potential enhancements | Layer 2 using legacy static TDD {DDDSU} based on Rel-17 specifications |
|  | Option 2 | Layer 2 using dynamic TDD UL/DL assignment based on potential enhancements | Layer 2 using dynamic TDD UL/DL assignment based on Rel-17 specifications |
| NOTE 1: For 2-layer Scenario, layer 1 uses legacy static TDD {DDDSU} for both target and baseline operation NOTE 2: - For legacy static TDD {DDDSU} and {DSUUU}, S=[12D:2G:0U] is assumed. - For dynamic TDD UL/DL assignment, {FFFFF} is assumed and companies to report the guard symbols assumed. Other configurations for dynamic TDD are not precluded and can be reported by companies. |  |  |  |

SLS assumptions

The SLS assumptions common to SBFD and dynamic/flexible TDD can be found in Table B.1-1 in Annex B.

The SLS assumptions specific to dynamic/flexible TDD can be found in Table B.1-5 in Annex B.

## 8.3	Inter-gNB CLI handling schemes

Editor's note: This section captures the potential inter-gNB CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD, as well as performance evaluation/analysis, observations and RAN1 specification impacts for each scheme.

### 8.3.x	Inter-gNB CLI scheme x

#### 8.3.x.1	Description of scheme

#### 8.3.x.2	Performance evaluation or analysis

Editor's note: The feasibility from performance point of view for the scheme can also be captured in this section.

#### 8.3.x.3	RAN1 specification impacts

Editor's note: The feasibility from specification design point of view for the scheme can also be captured in this section.

## 8.4	Inter-UE CLI handling schemes

Editor's note: This section captures the potential inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD, as well as performance evaluation/analysis, observations and RAN1 specification impacts for each scheme.

### 8.4.x	Inter-UE CLI scheme x

#### 8.4.x.1	Description of scheme

#### 8.4.x.2	Performance evaluation or analysis

Editor's note: The feasibility from performance point of view for the scheme can also be captured in this section.

#### 8.4.x.3	RAN1 specification impacts

Editor's note: The feasibility from specification design point of view for the scheme can also be captured in this section.

# 9	Implementation feasibility of SBFD

Editor's note: This section captures the feasibility considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE, as well as feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

## 9.1	Background for analysis

Editor's note: This section captures the overall description of all potential approaches and key enablers for SBFD

## 9.2	Feasibility of FR1 wide area BS aspects

### 9.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 9.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results.

### 9.2.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

### 9.2.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 9.3	Feasibility of FR1 medium range BS aspects

### 9.3.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results.

### 9.3.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results

### 9.3.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

### 9.3.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 9.4	Feasibility of FR1 local area BS aspects

### 9.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 9.4.2	Co-channel inter-sub-band inter-site interference analysis

This section captures the typical assumption of RF requirements and analysis results

### 9.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 9.5	Feasibility of FR2 BS aspects

### 9.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability and analysis results.

### 9.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

### 9.5.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

### 9.5.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 9.6	FR1 feasibility of UE aspects

### 9.6.1	Interference analysis

#### 9.6.1.1	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

#### 9.6.1.2	UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

### 9.6.2	Summary

Editor's note: This section captures the conclusion of feasibility.

## 9.7	FR2 feasibility of UE aspects

### 9.7.1	Interference analysis

#### 9.7.1.1	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

#### 9.7.1.2	UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

### 9.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

## 9.8	Summary

# 10		Impact on RF requirements

## 10.1	Impact on BS RF requirements

Editor's note: This section captures the impact on BS RF requirements

## 10.2	Impact on UE RF requirements

Editor's note: This section captures the impact on UE RF requirements

# 11	Adjacent channel co-existence evaluation results

Editor's note: This section will also capture adjacent channel co-existence simulation results, i.e. ACLR, ACS, ACIR. About simulation parameters and methodology, they are suggested to be moved into Annex E.

# 12	Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum

Editor's note: This section captures the summary of the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

## 12.1	Region 1

## 12.2	Region 2

## 12.3	Region 3

# 13	Conclusions and recommendations

Annex <A>:
Evaluation methodologies

# A.1	Layout and UE distribution

Editor's note: This section captures the layout and UE distribution for SBFD and dynamic/flexible TDD evaluation.

## A.1.1	Layout

Table A.1.1-1: Layout for SBFD and dynamic/flexible TDD evaluation

|  | Layout | Inter-BS (2D) distance | Minimum BS-UE (2D) distance | Minimum UE-UE (2D) distance | BS antenna height |
| --- | --- | --- | --- | --- | --- |
| Indoor office | Single layer (see Figure A.1.1-1) - Indoor floor: (12BSs per 120m x 50m) | 20m | 0m | 1m | 3 m |
| Urban Macro/ Dense Urban Macro layer | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around (see Figure A.1.1-2) Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | 500m for Urban Macro, 200m for Dense Urban Macro layer | 35m | 1m | 25 m |
| Dense Urban with 2-layer | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around (see Figure A.1.1-2) - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer (see Figure A.1.1-3): - Step 1: Randomly drop X micro TRP centers within one macro cell geographical area considering the minimum distance between micro TRP centers (Dinter-micro-center) and the minimum distance between macro TRP and micro TRP center (Dmacro-to-micro-center). - X=1, 3, 6 or 9 - Step 2: Randomly deploy one micro TRP on the area circle around each micro TRP center with the radius of half of Dinter-micro-center - Step 3: Determine the horizontal angle of the micro TRPs with the planer facing to the micro TRP center. | Macro-to-Macro: 200m Minimum Macro-to-micro-center distance: 42m Minimum Micro-center-to-micro-center distance: 40m | Macro-to-UE: 35m Micro-to-UE: 10m | 1m | 25m for Macro cells and 10m for Micro cells |
| Dense Urban Micro layer | Single layer Only consider the Micro TRPs of Dense Urban with 2-layer network. All users communicate with micro TRPs, i.e. macro cell is only used for determining position of micro TRP. |  |  |  |  |
| 2-layer Scenario B | Two layer (see Figure A.1.1-4) - Layer 1: Urban Macro - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=500m (see Figure A.1.1-2) - Layer 2: Indoor office (baseline) or indoor factory (optional) - Only one building randomly dropped in the whole network as in figure A.1.1-4. The building has to be confined within one macro cell area. - For Indoor office: 12 (baseline) or 3 (optional) TRPs per 120m x 50m x 3m - For indoor factory: 12 TRPs per 120m x 50m x 3m. Other simulation assumptions follow InF-SL in Table 7.8-7 (Simulation assumptions for large scale calibration for the indoor factory scenario) in TR 38.901 - The orientation of the building is fixed as in Figure A.1.1-4 (i.e., the long side of the rectangular is along the x-axis) | Macro-to-Macro: 500m Minimum Macro-to- indoor office center distance: 100m Indoor TRP -to- Indoor TRP: 20m for 12 TRPs, 40m for 3 TRPs | Macro-to-UE: 35m Indoor TRP-to-UE: 0m | 1m | 25m for Macro cells and 3m for Indoor TRP |
| NOTE 1: Distance-based wrap-round is used. |  |  |  |  |  |

Figure A.1.1-1: Layout for indoor

Figure A.1.1-2: Layout for hexagonal grid with 7 macro sites and 3 sectors per site

Figure A.1.1-3: Layout for dense urban (3 Micro TRPs per Macro TRP)

(a) 12 TRPs in one building

(b) 3 TRPs in one building

Figure A.1.1-4: Layout for 2-layer Scenario B

For SBFD Deployment Case 4, consider 0% and 100% grid shift between two networks for Urban Macro and Dense Urban Macro layer scenarios as shown in Figure A.1.1-5.

Figure A.1.1-5: 0% and 100% grid shifts between two networks

For SLS in RAN1, use the following BS antenna layout (see Figure A.1.1-6) for indoor office scenario (referring to Table 1 in RP-180524), wherein,

-	X-axis is pointing down to the floor

-	The antenna array is mounted in the Y-Z plane with boresight along the X-axis (ceiling mounted with boresight towards the floor)

-	The X-axis/Y-axis/Z-axis refer to LCS

Figure A.1.1-6: Top view of the BS antenna layout for indoor office scenario

## A.1.2	UE distribution

Table A.1.2-1: UE distribution for SBFD and dynamic/flexible TDD evaluation

|  |  | UE distribution |
| --- | --- | --- |
| FR1 | Indoor office | - 10 users per TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m |
|  | Urban Macro / Dense Urban Macro layer | - Baseline: UE clustering distribution (NOTE 1) - Baseline: M=20, X=2; Optional: M=10, X=1. - R’ = 25 m. Evaluation results for other values of R’ can be submitted. - Dmacro-to-cluster = 35m+ R’, Dinter-cluster = 2R’ m - UEs dropped within the UE cluster(s) are indoor with 3km/h. UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h. - UE height is 1.5m for all UEs. - Optional: Uniform UE distribution - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell - 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
|  | Dense Urban with 2-layer | - 2/3 users randomly and uniformly dropped around micro TRP centers with radius of R (R = [28.9m]), 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area. - 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
|  | 2-layer Scenario B | - Layer 1: Urban Macro - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell outside the Indoor office / Indoor factory - 100% outdoor without car penetration loss (3km/h), UE height is 1.5m - Layer 2: Indoor office (baseline) - 10 users per indoor TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m |
| FR2-1 | Indoor office | - 10 users per TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m |
|  | Dense Urban Macro layer | - Baseline: UE clustering distribution (NOTE 1) - Baseline: M=10, X=1; Optional: M=20, X=2. - R’ =20m. Lower values of R’ are not excluded. - Dmacro-to-cluster = 35m+R’. - For X=2, Dinter-cluster = 2R’ m. - All the UEs (including UEs in the clusters and out of the clusters) are outdoor UEs without car penetration loss (3km/h). - UE height is 1.5m. - Optional: Uniform UE distribution - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell. - Baseline: 100% outdoor without car penetration loss (3km/h). Outdoor UE height is 1.5 m. - Optional: 20% outdoor in cars with 30km/h, 80% indoor in houses with 3km/h. - Outdoor UE height: 1.5 m; - Indoor UE height: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
|  | Dense Urban Micro layer | - All users are randomly and uniformly dropped around Micro TRP center with the radius of R (R = [28.9m]) |
| NOTE 1: The UE clustering distribution is assumed as below: - Randomly drop X (X =1 or 2) UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and for X=2, the minimum distance between two UE cluster centers as Dinter-cluster - Assuming M (M=10 or 20) users per macro TRP per direction, 80% UEs are randomly and uniformly dropped within the UE clusters with the radius of R’, 20% users randomly and uniformly dropped in the macro geographical area outside the clusters. If each UE is assigned both UL traffic and DL traffic, there are 8 UEs in one UE cluster. If each UE is either assigned UL traffic or DL traffic, there are 8 UEs with DL traffic and 8 UEs with UL traffic in one UE cluster. - The UEs in the same cluster are in the same building for FR1. For the case with M=20 and X=2, the UEs in different clusters are in different buildings for FR1. - Note that the UE cluster is totally confined within the macro cell geographical area (i.e. a cluster cannot be partially overlap with adjacent cell area). NOTE 2: UE clustering distribution for SBFD Deployment Case 4 with 0% or 100% grid shift is assumed as below: - The UE cluster centers of the first operator are the same as that of the second operator. - For 100% grid shift, the minimum distance requirement between the UE cluster center and macro TRP should be satisfied for both operators. - Minimum UE-UE 2D distance is 1m regardless the serving operator. - For each operator, the UE cluster distribution method for SBFD deployment Case 1 is reused. |  |  |

# A.2	Interference modelling

Editor's note: This section captures the interference modelling methods for duplex evolution study. The details of interference modelling described in Annex A.2.1~A.2.8 will be further revised based on RAN4 reply LS and more progress in RAN1 and RAN4.

Consider the following definitions of interference types for duplex evolution study.

Table A.2-1: Definition of interference types

| Interference types |  |  | Definition |
| --- | --- | --- | --- |
| Co-channel interference | Co-channel intra-subband interference | gNB-UE co-channel intra-subband interference | This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. |
|  |  | UE-gNB co-channel intra-subband interference | This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. |
|  |  | (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI | CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier. |
|  |  | (inter-cell) co-site inter-sector co-channel intra-subband CLI | CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier. |
|  |  | (inter-cell) UE-UE co-channel intra-subband CLI | CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier. |
|  | Co-channel inter-subband interference | gNB self-interference (SI) | Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency (see Annex A.2.1 for detailed modelling). |
|  |  | (inter-cell) co-site inter-sector co-channel inter-subband CLI | CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency (see Annex A.2.2 for detailed modelling). |
|  |  | (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI | CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency (see Annex A.2.3 for detailed modelling). |
|  |  | (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI | CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency (see Annex A.2.4 for detailed modelling). |
| Adjacent channel interference |  | Co-site gNB-gNB adjacent-channel CLI | CLI caused by DL transmission of the aggressor co-site gNB in a carrier to UL reception of the victim gNB in another adjacent carrier(see Annex A.2.5 for detailed modelling). |
|  |  | Inter-site gNB-gNB adjacent-channel CLI | CLI caused by DL transmission of the aggressor inter-site gNB in a carrier to UL reception of the victim gNB in another adjacent carrier (see Annex A.2.6 for detailed modelling). |
|  |  | UE-UE adjacent-channel CLI | CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier (see Annex A.2.7 for detailed modelling). |
| NOTE: This does not imply all the above interference types are needed in evaluation for SBFD. Some of the interferences may not be used according to the deployment scenarios, e.g., whether the SBFD subband configurations are the same or different across gNBs. |  |  |  |

## A.2.1	gNB self-interference modelling

For gNB self-interference modelling in SLS of SBFD in RAN1, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference.

For SLS of SBFD in RAN1, the RSI is modelled as frequency flat within the UL subband. The residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power is computed by

ISImax=PtxmaxαSI*NULRBNDLRB#A-1

where

-	ISImax is the residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power (in linear scale).

-	Ptxmax is the maximum gNB DL Tx Power on the two DL subbands (in linear scale).

-	NDLRB is the total number of DL RBs in the DL subbands.

-	NULRB is the total number of UL RBs in the UL subband.

-	αSI is the overall RSI value provided by RAN4 in linear scale. RAN1 assumes frequency isolation value in the overall RSI value ranges provided by RAN4 is based on the assumption of SBFD subband configuration with {DUD=40MHz:20MHz:40MHz} at least for FR1 and all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power. Candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. Companies shall report what values of the individual components are assumed in order to achieve the αSI value corresponding to 1 dB desense. Other approaches of determining values for αSI are not precluded and can be used and reported by companies.

Assuming αSI doesn’t change when DL RBs are not fully allocated for DL transmission, the residual self-interference power on one UL RB when DL RBs are not fully allocated for DL transmission is computed by

ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB#A-2

where

-	ISIper-RB is the residual self-interference power on one UL PRB when all the DL RBs in the DL subbands are not fully allocated for DL transmission (in linear scale).

-	Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB.

-	Nused-DL-RB is the number of DL RBs allocated for DL transmission.

## A.2.2	Co-site inter-sector co-channel inter-subband CLI

For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows.

Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB#A-3

Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB#A-4

where

-	Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB.

-	Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale).

-	NDLRB is the total number of DL RBs in the DL subbands.

-	Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x.

-	αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI.

-	10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

-	Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for Tx leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment.

-	Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived.

## A.2.3	Inter-site gNB-gNB co-channel inter-subband CLI

For discussion of gNB-gNB co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for gNB-gNB co-channel channel model, the power of inter-site gNB-gNB co-channel inter-subband CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as

IInter-Site-CLIA'→A,per-RB=PtxBS A',per-RB*Nused-DL-RBBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS*1NDLRB#A-5

where

-	IInter-Site-CLIA'→A,per-RB is the power of inter-site gNB-gNB co-channel inter-subband CLI from aggressor gNB A' to victim gNB A on each receiver chain at one UL RB (linear value).

-	PtxBS A',per-RB is DL transmission power of gNB A' across all transmit chains per RB (linear value). PtxBS A',per-RB=Ptxmax/NDLRB.

-	Nused-DL-RBBS A' is the number of DL RBs allocated for DL transmission by gNB A'.

-	CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB.

-	CLBS A'→BS A=1ST∙p=0ST-11SR∙u=0SR-1CLp,uA',AwA',gA, wherein, ST is the number of Tx antenna ports of gNB A', and SR is the number of Rx antenna ports of gNB A. CLp,uA',AwA',gA is coupling loss from Tx antenna port p of transmitter BS A’ to Rx antenna port u of BS A and is given by equation (A-16) in Annex A.8.

-	NDLRB is the total number of DL RBs in the DL subbands.

-	Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e., ACLRBS) is provided as the candidate for Tx leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment.

-	Note: the model is based on the assumption that the same transmission power across different DL RBs is used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS.

-	Note: This model is not applicable to the RBs in the guardband.

-	Note: This model is not applicable for some candidate gNB-gNB CLI handling schemes (for example, spatial digital beam coordination, advanced receivers).

For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB A can be modelled as

IInter-Site-CLIn=Ileakagen+Iselectivity#A-6

where

-	Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB A'.

-	HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn.

-	NT is the number of Tx chains at aggressor gNB.

-	NR is the number of Rx chains at victim gNB.

-	yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB.

-	NT is the number of Tx chains at aggressor gNB.

-	ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise.

-	σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB  is the total leakage power at UL RB n at aggressor gNB.

-	Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB.

-	Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB.

-	NDLRB is the total number of DL RBs in the DL subbands.

-	W is the NT×NT normalized identity matrix with unit norm, WF=1.

-	It is up to companies to report other values of W and the corresponding applicable conditions.

-	Iselectivity is the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB A. It can be modelled as

Iselectivity=z0n,z1n,…,zNR-1nT#A-7

-	zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise.

-	σz,n2=1ICSBS×NDLRB*1NR*m∈Used DL RBsHCLImWmsm2

-	HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm.

-	Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1.

-	sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB.

-	NDLRB is the total number of DL RBs in the DL subbands.

-	For FR1, the value range of 10*log10(ICSBS) (in channel selectivity) recommended from RAN4 is {46dB, [62]dB}. The following two options are recommended to be used in RAN1 simulation. Companies to report the value of ICSBS used in their simulations.

-	Option-1: 10*log10(ICSBS)=46dB

-	Option-2: 10*log10(ICSBS)=62dB

-	For FR2-1, RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS until further input is received from RAN4.

-	Note: ICSBS is in linear scale.

-	Note: The piece wise BS noise figure model at least for FR1 should be used. FFS for FR2-1.

-	Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment.

-	Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS.

-	Note: This model is not applicable to the RBs in the guardband.

## A.2.4	UE-UE co-channel inter-subband CLI

For discussion of UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g., receiver blocking at the victim, overload of the receiver dynamic range, etc)

For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as

IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB#A-8

where

-	IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value).

-	PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value)

-	CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE

-	NULRB is the total number of UL RBs in the UL subband

-	ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

-	10*log10IUE-UE,IBEn=max10*log10PtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1max10*log10PtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1

wherein,

-	For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored

-	10*log10IGeneral=10*log10PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-10*log10PtxUE B',per-RBfor FR110*log10PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-10*log10PtxUE B',per-RBfor FR2-1

-	PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'.

-	NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1.

-	NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS

-	NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS

-	ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs)

-	EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs.

For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as:

IInter-UE-CLIn=Ileakagen+Iselectivity#A-9

where

-	Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE,

-	HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn

-	NR is the number of Rx chains and NT is the number of Tx chains

-	W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1.

-	yn=y0n,y1n,…,yRnT,

-	 ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise

-	IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled

-	Iselectivity is modelled as frequency flat

Iselectivity=z0n,z1n,…,zNR-1nT

-	zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

-	σz,n2=1ICSUE×NULRB*1NR*m∈Used UL RBsHCLImWsm2

-	HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm,

-	W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1

-	sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB.

-	PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled

-	NULRB is the total number of UL RBs in the UL subbands,

-	ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

## A.2.5	Co-site gNB-gNB adjacent-channel CLI

For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows.

Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB#A-10

Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj#A-11

where

-	Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj.

-	Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale).

-	NDLRBadj is the total number of DL RBs in adjacent channel.

-	Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel.

-	αadj,co-site-x is the interference suppression capability of co-site gNB-gNB adjacent-channel CLI between the aggressor sector x and the victim sector.

-	10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

-	FFS the concrete value of αadj,co-site-x

-	ACLRBS and ACSBS are in linear scale.

## A.2.6	Inter-site gNB-gNB adjacent-channel CLI

Regarding gNB-gNB adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g., receiver blocking at the victim, overload of the receiver dynamic range, etc).

For inter-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modelling with gNB ACLR for TX leakage and gNB ACS for Receiver impairment.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for inter-site gNB-gNB adjacent-channel channel model, the power of inter-site gNB-gNB adjacent-channel CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as

IgNB-gNB-adjacent-channel-CLIA'→A,per-RB=PtxBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS*1NRBtotal#A-12

where

-	IgNB-gNB-adjacent-channel-CLIA'→A,per-RB is the power of inter-site gNB-gNB adjacent-channel CLI from gNB A' to gNB A on each receiver chain at one UL RB (linear value).

-	PtxBS A' is DL transmission power of gNB A' across all transmit chains over the scheduled DL RBs (linear value).

-	CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB.

-	NRBtotal is the total number of RBs of the channel bandwidth (e.g., 100MHz for FR1) of the aggressor gNB.

-	Note: ACLRBS  (i.e., gNB ACLR) and ACSBS (i.e., gNB ACS) are in linear scale. With this assumption, in absence of further RAN4 inputs, gNB ACLR and gNB ACS in current specification are used for both inter-site gNB-gNB co-channel inter-subband CLI modelling and inter-site gNB-gNB adjacent-channel CLI modelling. The values of ACLRBS and ACSBS used in inter-site gNB-gNB co-channel and adjacent-channel CLI modelling can be revisited based on further RAN4 inputs.

-	Note: This model is not applicable to the RBs in the guard band between the two adjacent channels.

## A.2.7	UE-UE adjacent-channel CLI

Regarding UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

-	Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

-	Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

For UE-UE adjacent-channel CLI modelling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modelling with UE ACIR used in Rel-16 CLI study.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE adjacent-channel channel model, the power of UE-UE adjacent-channel CLI experienced by the victim UE on each receiver chain at one DL RB can be modelled as

IUE-UE-adjacent-channel-CLIB'→B,per-RB=PtxUE B'*CLUE B'→UE B*1ACIRUE*1NRBtotal#A-13

where

-	IUE-UE-adjacent-channel-CLIB'→B,per-RB is the power of UE-UE adjacent-channel CLI from UE B' to UE B on each receiver chain at one DL RB (linear value).

-	PtxUE B' is UL transmission power of UE B' across all transmit chains over all UL RBs (linear value).

-	CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE.

-	NRBtotal is the total number of RBs of the channel bandwidth (e.g., 100MHz for FR1) of the aggressor UE.

-	Note: ACIRUE are in linear scale. In RAN4 reply LS, RAN4 agree on UE ACLR based model on Tx and UE ACS based model on Rx which is the same ACIR model as Rel-16 CLI study as starting point.

-	Note: This ACIR-based model is not applicable to the guard band between the two adjacent channels.

## A.2.8	BS noise figure model

For SLS of duplex evaluation in RAN1, the BS noise figure is modelled as piece wise linear based on the total received power (P) as

NF dB= C ,  P<A (D- C)(B-A)P-A+C,  A≤P<B#A-14

-	P is in dB scale. The linear value of total received power is the linear sum of all received power, including wanted signal, co-channel and adjacent-channel UE-gNB interference, self-interference, co-channel and adjacent-channel co-site inter-sector interference and co-channel and adjacent-channel inter-site gNB-gNB interference.

-	adjacent-channel interference is only used for SBFD deployment case 4

-	If P is larger than B, the receiver will be blocked.

-	For FR1 WA BS without sub-band filter, A = -43dBm, B = -25dBm, C = 5dB, D = 14dB

-	For FR1 WA BS with sub-band filter, A = -35dBm, B = -17dBm, C = 5dB, D = 14dB

-	For FR1 MR BS, A = -38dBm, B = -20dBm, C = 10dB, D = 19dB

-	For FR1 LA BS, A = -35dBm, B = -17dBm, C = 13dB, D = 22dB

-	For FR2-1 BS, A = -58dBm, B = -40dBm, C = 10dB, D = 19dB

# A.3	Channel modelling

Editor's note: This section captures the channel modelling for SBFD and dynamic/flexible TDD evaluation, including gNB-UE channel model, gNB-gNB channel model and UE-UE channel model.

Table A.3-1: gNB-UE channel model

|  | Urban Macro, Dense Urban | Indoor office | 2-layer Scenario B |
| --- | --- | --- | --- |
| Large-scale channel parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - O2I penetration loss follows TR 38.901 (NOTE 1, NOTE 2) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - O2I penetration loss follows TR 38.901 (NOTE 1) | FR1: - TRP-to-UE: InH-Office in TR 38.901 - Penetration loss is not modelled. FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - Penetration loss is not modelled. | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 (NOTE 1, NOTE 2) - Indoor TRP to Outdoor UE: - Option 1: - UMi-Street canyon in TR 38.901 (hBS =3 m) - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - For both options, O2I penetration loss between indoor TRP and outdoor UE follows Table A.2.1-12 in TR 38.802 (d2D-in is the distance between the indoor TRP and the building boundary along the direction from Indoor TRP to outdoor UE. The d2D-in may be different for different indoor-TRP-outdoor-UE links associated with the same indoor TRP) (NOTE 1, NOTE 2) |
| Fast fading parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 | FR1: - TRP-to-UE: InH-Office in TR 38.901 FR2-1: - TRP-to-UE: InH-Office in TR 38.901 | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - Option 1: - UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA - Option 2: - For Indoor office layer: InH-Office (NLOS) in TR 38.901 - For Indoor factory layer: InF (NLOS) in TR 38.901 |
| NOTE 1: For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; Otherwise the building type is high loss. NOTE 2: The O2I car penetration loss is modelled with μ = 9, and σP = 5. |  |  |  |

Table A.3-2: gNB-gNB channel model

|  | Urban Macro, Dense Urban | Indoor office | 2-layer Scenario B |
| --- | --- | --- | --- |
| Large-scale channel parameters (NOTE 2, NOTE 3) | FR1: - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) | FR1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) - Penetration loss is not modelled. FR2-1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) - Penetration loss is not modelled. | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - For Indoor factory layer: InF in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 (NOTE 1) - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| Fast fading parameters | FR1: - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 | FR1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| NOTE 1: For Macro-gNB-indoor-gNB O2I building penetration loss, adopt the high loss and low loss O2I building penetration loss model in Table 7.4.3-2 in TR 38.901: - If InF is used as Layer-2 for 2-layer Scenario B, 100% high-loss model is considered. - Otherwise,80% low-loss model and 20% high-loss model is considered. - The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; Otherwise the building type is high loss. NOTE 2: LOS probability of gNB-gNB channel: - For Macro-to-Macro, if the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to 75%; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901. NOTE 3: Regarding gNB-gNB channel model with UMa, extend the applicability range of hUT from 13m=<hUT<=23m to 13m=<hUT<=25m in the formula to get C(d2D, hUT) in Table 7.4.1-1 (Pathloss models) in TR 38.901. |  |  |  |

Table A.3-3: UE-UE channel model

|  | Urban Macro, Dense Urban | Indoor office | 2-layer Scenario B |
| --- | --- | --- | --- |
| Large-scale channel parameters (NOTE 1, NOTE 2, NOTE 3, NOTE 4, NOTE 5) | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR 36.843 (NOTE 6) - Option 2 (baseline): UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m) (NOTE 7) FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m) (NOTE 7) | FR1: - Option1: UE-to-UE: A.2.1.2 in TR 36.843 (NOTE 1) - Option 2 (baseline): UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) | FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR 36.843 (NOTE 6) - Option 2 (baseline): UMi-Street canyon in TR 38.901 (hBS =1.5m) (NOTE 7) - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR 36.843 (NOTE 6). - Option 2 (baseline): - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR 36.843 (NOTE 6). - Option 2 (baseline): UMi-Street canyon in TR 38.901 (hBS =1.5m) (NOTE 7). |
| Fast fading parameters | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR 36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2 (baseline): UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR 36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2 (baseline): UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. | FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2 (baseline): UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR 36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| NOTE 1: For UE-UE penetration loss, Table A.2.1-12 in TR 38.802 is used with modification on the criterion used to determine whether two indoor UEs are in the same building or not as below: - For indoor office scenario in FR1/FR2-1, all the UEs are in the same building. - For Urban Macro or Dense Urban Macro layer scenario in FR1, - (baseline) if UE clustering distribution is used, two indoor UEs are considered in the same buildings if they are in the same UE cluster; two indoor UEs are considered in different buildings if they are in different UE clusters. - (optional) if uniform UE distribution is used, two indoor UEs are considered in the same building if the inter-user 2D distance ≤ 50m; two indoor UEs are considered in different buildings if the inter-user 2D distance > 50m. - For Dense Urban with 2-layer scenario in FR1, two indoor UEs are considered in the same building if the inter-user 2D distance ≤ 50m; two indoor UEs are considered in different buildings if the inter-user 2D distance > 50m. - For Dense Urban Macro layer scenario in FR2, baseline is that all the UEs are outdoor UEs. - For the optional case that 20% UEs are outdoor UEs and 80% UEs are indoor UEs, two indoor UEs are considered in the same building if the inter-user 2D distance ≤ 50m; two indoor UEs are considered in different buildings if the inter-user 2D distance > 50m. - For Dense Urban Micro layer scenario in FR2, all the UEs are outdoor UEs. - For 2-layer Scenario B in FR1, all the indoor UEs are in the same building. NOTE 2: For UE-UE O2I building penetration loss, the following is used to generate d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O2I building penetration loss. - d2D-in =Uniform(0,min⁡{25m,d2D2}) NOTE 3: For UE-UE O2I building penetration loss, adopt the high loss and low loss O2I building penetration loss model in Table 7.4.3-2 in TR 38.901. - If InF is used as Layer-2 for 2-layer Scenario B, 100% high-loss model is considered. - Otherwise, 80% low-loss model and 20% high-loss model is considered. - The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; Otherwise, the building type is high loss. - For UEs determined in the same building, each UE selects high loss/low loss building type independently. NOTE 4: For UE-UE LOS probability, when channel model of UMi-Street canyon in TR 38.901 is used for UE-to-UE link, d2D-out in LOS probability formula can be interpreted as follow: - For outdoor UE to outdoor UE, d2D-out=d2D - For indoor UE to outdoor UE and outdoor UE to indoor UE, d2D-out=d2D-d2D-in - For indoor UE to indoor UE in different buildings, it is always NLOS. - d2D is the UE-UE 2D distance NOTE 5: When two indoor UEs are in different buildings, the LOS probability is always NLOS, and the standard deviation of shadow fading for NLOS in TR 38.901 is used. NOTE 6: Regarding Option 1 of UE-UE channel model for FR1, - For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR 36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR 36.843, and the penetration loss is derived according to Table A.2.1-12 in TR 38.802. - For Indoor to Indoor case, additional 6dB should be added in pathloss to support 4GHz carrier frequency. NOTE 7: For UE-UE path loss computation based on TR 38.901 (Option 2), extend the applicability range of the equations down to 1m (minimum distance between UEs). - PLUMi-LOS=PL11m ≤d2D≤dBP'PL2dBP'≤d2D≤5km |  |  |  |

# A.4	Performance metrics

Editor's note: This section captures the definition of performance metrics for SBFD and dynamic/flexible TDD evaluation.

## A.4.1	UPT related performance metrics

UPT is defined as the size of an FTP packet divided by the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver [Refer to TR36.814]. Unfinished FTP packets should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished FTP packet by the end of the simulation is divided by the served time (simulation end time – file arrival time) [Refer to TR36.889]. Consider zero bit for dropped FTP packets.

Mean/5%/50%/95% Average-UPT, Average-UPT CDF:

-	Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

-	Average-UPT CDF: the CDF of the Average-UPTs for all users.

-	Mean/5%/50%/95% Average-UPT: the mean/5%/50%/95% value of Average-UPTs for all users.

Mean/5%/50%/95% Tail-UPT, Tail-UPT CDF:

-	Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

-	Tail-UPT CDF: the CDF of the Tail-UPTs for all users.

-	Mean/5%/50%/95% Tail-UPT: the mean/5%/50%/95% value of Tail-UPTs for all users.

Mean/5%/50%/95% Median-UPT, Median-UPT CDF:

-	Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

-	Median-UPT CDF: the CDF of the Median-UPTs for all users.

-	Mean/5%/50%/95% Median-UPT: the mean/5%/50%/95% value of Median-UPTs for all users.

## A.4.2	Latency related performance metrics

Packet latency is defined as the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver. HARQ re-transmission should be considered for latency evaluation. Unfinished/dropped FTP packets are not incorporated in the packet latency calculation.

Mean/5%/50%/95% Packet-Latency, Packet-Latency CDF:

-	Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

-	Packet-Latency CDF is the CDF of the packet latencies of all the packets from all the UEs.

-	Mean/5%/50%/95% Packet-Latency is the mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

Mean/5%/50%/95% UE-Average-Latency, UE-Average-Latency CDF:

-	Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

-	UE-Average-Latency CDF is the CDF of the UE-Average-Latency for all users.

-	Mean/5%/50%/95% UE-Average-Latency is the mean/5%/50%/95% value of UE-Average-Latency for all users.

## A.4.3	RU and Unfinished/dropped FTP packets rate

Type-1 RU:

-	DL/UL Type-1 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time.

Type-2 RU:

-	DL/UL Type-2 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of RBs per cell available for traffic for the given link direction over observation time.

For both type-1 RU and type-2 RU, in case of MU-MIMO, one RB allocated to N users within a cell is only counted as used once.

Unfinished/dropped Packet Rate:

-	Unfinished/dropped Packet Rate is defined as the number of the unfinished packets for all users divided by the total number of generated packets for all users

# A.5	gNB antenna configuration and transmit power for SBFD

Editor's note: This section captures the gNB antenna configuration and transmit power for SBFD evaluation.

For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption.

The separate-Tx/Rx antenna array for description of evaluation assumption can be modelled by two panel groups as in Figure A.5-1.

-	Legacy parameters M,N,P,Mg,Ng, dH,dV and dg,H,dg,V are used for description of each panel group:

-	M: Number of vertical antenna elements within a panel, on one polarization

-	N: Number of horizontal antenna elements within a panel, on one polarization

-	P: Number of polarizations

-	Mg: Number of panels in a column within a panel group.

-	Ng: Number of panels in a row within a panel group.

-	dg,H: Antenna panel spacing in horizontal direction within a panel group.

-	dg,V: Antenna panel spacing in vertical direction within a panel group.

-	Companies are to report the separation of the two panel groups. Introduce new parameters da,H,da,V as illustrated in the following Figure A.5-1.

-	da,H: Panel group spacing in the horizontal direction. Typically, da,H = 0.

-	da,V: Panel group spacing in the vertical direction.

Figure A.5-1: Separate-Tx/Rx antenna array model

For evaluation and comparison between SBFD and legacy TDD, three options can be used.

-	SBFD antenna configuration option-1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

-	SBFD antenna configuration option-2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

-	SBFD antenna configuration option-3: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD.

These options are further clarified with examples in the following:

As shown in Figure A.5-2 (a), for legacy TDD with shared-Tx/Rx antenna array, assume the antenna configuration is M,N,P,Mg,Ng;Mp,Np. The total number of TxRUs is K=PMpNpMgNg, and the total number of antenna elements is L=PMNMgNg.

For SBFD antenna configuration option-1, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). One method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols is illustrated as below. Other methods are not precluded and can be reported by companies.

-	Method 1 (see Figure A.5-2 (b)):

-	In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Tx chains in TxRU group#2.

-	In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2.

-	In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2.

For SBFD antenna configuration option-2, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is 2L=2PMNMgNg(two times of that for legacy TDD). Two methods on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies.

-	Method 2-1 (see Figure A.5-2 (c)):

-	In DL slots, L antenna elements on panel group#1 are connected to K Tx chains.

-	In UL slots, L antenna elements on panel group#2 are connected to K Rx chains.

-	In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains.

-	Method 2-2 (see Figure A.5-2 (d)):

-	In DL slots, L antenna elements on panel group#1 are connected to K Tx chains.

-	In UL slots, L antenna elements on panel group#1 are connected to K Rx chains.

-	In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains.

For SBFD antenna configuration option-3, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K/2=PMpNpMgNg/2 (half of that for legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). The method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies.

-	Method 3-1 (see Figure A.5-2 (e)):

-	In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains.

-	In UL slots, L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains.

-	In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains.

-	Method 3-2 (see Figure A.5-2 (f)):

-	In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1.

-	In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1.

-	In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#1.

(a) Legacy TDD with shared-Tx/Rx antenna array

(b) SBFD antenna configuration Option-1 (Method 1)

(c) SBFD antenna configuration Option-2 (Method 2-1)

(d) SBFD antenna configuration Option-2 (Method 2-2)

(e) SBFD antenna configuration Option-3 (Method 3-1)

(f) SBFD antenna configuration Option-3 (Method 3-2)

Figure A.5-2: Illustration of antenna configuration of legacy TDD and SBFD

For SBFD evaluation, assume the maximum BS transmit power is proportional to the number of Tx chains used for transmission

-	For SBFD antenna configuration Option-1,

-	in DL-only symbols, the maximum BS transmit power for SBFD is the same as that for legacy TDD

-	in SBFD symbols, the maximum BS transmit power for SBFD is half of that for legacy TDD

-	For SBFD antenna configuration Option-2, in both DL-only symbols and SBFD symbols, the maximum BS transmit power for SBFD is always the same as that for legacy TDD

-	For SBFD antenna configuration Option-3, in both DL-only symbols and SBFD symbols, the maximum BS transmit power for SBFD is always half of that for legacy TDD

For BS transmit power for SBFD, take Option-1 as baseline. Option-2 can also be evaluated.

-	Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems), i.e., BS transmit power spectrum density per Tx chain is kept the same for SBFD symbols and DL-only symbols

-	Option-2: Power boosting is assumed for SBFD symbols compared to DL-only symbols, i.e.,

-	DL symbols in SBFD operation have the same PSD as used in TDD DL symbols

-	For SBFD symbols, its PSD is scaled according to the number of RBs in DL subband(s), e.g.,

PSDSBFDPer Tx-chain=PSDDL-onlyPer Tx-chain∙BWcarrierBWDL subbands

-	PSDSBFDPer Tx-chain is the BS transmit power spectrum density per Tx chain in SBFD symbols

-	PSDDL-onlyPer Tx-chain is the BS transmit power spectrum density per Tx chain in DL-only symbols

-	BWcarrier is the system bandwidth and BWDL subbands is the total bandwidth of DL subbands

# A.6	Traffic model

Editor's note: This section captures the traffic model for SBFD and dynamic/flexible TDD evaluation.

Table A.6-1: Traffic model of FTP model 3 for SBFD evaluation

|  | 1-layer scenario for SBFD Deployment Case 1 | 2-layer scenario for SBFD Deployment Case 1 and Case 3-2 | SBFD Deployment Case 4 |
| --- | --- | --- | --- |
| General | UL and DL are simulated simultaneously. Companies to report which option is used (NOTE 1). - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL - Option 2: Each UE is assigned both UL traffic and DL traffic. |  |  |
| FTP packet size | Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL |  |  |
| UL/DL arrival rate for baseline legacy TDD case | The UL/DL packet arrival rates are selected so that baseline legacy TDD case achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). | The UL/DL packet arrival rates are selected for each layer independently so that each layer of baseline legacy TDD case achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). | The UL/DL packet arrival rates are selected for each operator independently so that each operator of baseline legacy TDD case achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). |
| UL/DL arrival rate for SBFD case | The packet arrival rates for SBFD case are the same as that for baseline legacy TDD case. | The packet arrival rates are kept the same for each corresponding layer in baseline legacy TDD case (i.e., legacy TDD for both layer-1 and layer-2) and SBFD deployment case (i.e., SBFD for both layer-1 and layer-2 for SBFD Deployment Case 1, or layer-1 with legacy TDD and layer-2 with SBFD for SBFD Deployment Case 3-2) respectively. | The packet arrival rates are kept the same for each corresponding operator in baseline legacy TDD case (i.e., legacy TDD for both Operator#1 and Operator#2) and SBFD deployment case 4 (i.e., legacy TDD for Operator#1 and SBFD for Operator#2) respectively. |
| NOTE 1: For M UEs distributed per direction, - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M UEs, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M UEs. |  |  |  |

Table A.6-2: Traffic model of FTP model 3 for dynamic/flexible TDD evaluation

|  | 1-layer scenario | 2-layer scenario |
| --- | --- | --- |
| General | UL and DL are simulated simultaneously. Companies to report which option is used (NOTE 1). - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL Option 2: Each UE is assigned both UL traffic and DL traffic. |  |
| FTP packet size | Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL |  |
| UL/DL arrival rate determination method | The UL/DL arrival rates are selected so that network using legacy static TDD {DDDSU} achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). | The UL/DL arrival rates are selected for each layer independently so that each layer using legacy static TDD {DDDSU} achieves a certain level of Type-2 RU (i.e., <10%, 20%-40% and ≥50% for low, medium and high load). |
| NOTE 1: For M UEs distributed per direction, - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M UEs, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M UEs. |  |  |

# A.7	SBFD subband and slot configurations

Editor's note: This section captures the SBFD subband and slot configurations for SBFD evaluation.

For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation, one or more of the following alternatives can be considered:

-	Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

-	Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

-	Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 25% of the channel bandwidth.

-	Note: Alt 3 is deprioritized.

-	Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

-	Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

-	SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

Regarding the SBFD subband configuration for Alt 1/2/4, SBFD Subband configuration#1 with {DUD} pattern as following is assumed.

-	For FR1

-	Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5>

-	Optional: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <106, 51, 5>

-	For FR2-1

-	Optional: 100MHz channel bandwidth and 120kHz SCS (66 PRB) < ND, NU, NG > = <25, 14, 1>

-	Baseline: 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <52, 26, 1>

wherein,

-	ND is the number of RBs in one DL subband.

-	NU is the number of RBs in one UL subband.

-	NG is the number of RBs in one guard band between one UL subband and one DL subband.

-	Other values of < ND, NU, NG > are not precluded and can be reported by companies.

Regarding the SBFD slot configurations, companies should report the guard symbols assumed in the SBFD operation.

# A.8	Coupling loss definition

Editor's note: This section captures the coupling loss definition for SBFD evaluation.

Consider following for the definition of coupling loss CLp,uA,BwA,gB from Tx antenna port p of transmitter A to Rx antenna port u of receiver B:

If both large scale fading and small scale fading are modelled, the coupling loss from Tx antenna port p of transmitter A to Rx antenna port u of receiver B is defined in formula (1) which is based on formula (B.1-2) in TR 37.910.

CLp,uA,BwA,gB=PL∙SF∙α0,u,p2+n=1Nm=1Mαn,m,u,p2#A-15

If only large scale fading is modelled, the coupling loss from Tx antenna port p of transmitter A to Rx antenna port u of receiver B is defined in formula (2).

CLp,uA,BwA,gB=PL∙SF∙α'0,u,p2#A-16

α'0,u,p=Frx,u,θθLOS,ZOA,ϕLOS,AOAFrx,u,ϕθLOS,ZOA,ϕLOS,AOAT∙100-1∙Ftx,p,θθLOS,ZOD,ϕLOS,AODFtx,p,ϕθLOS,ZOD,ϕLOS,AOD#A-17

where

-	wA (wA=w1A,w2A,⋯,wNTA) represents a complex weight vector used for virtualization of Tx antenna port p of transmitter A, and gB (gB=g1B,g2B,⋯,gNRB) represents a complex weight vector used for virtualization of Rx antenna port u of receiver B.

-	Formula (A-17) can be understood according to equation (7.5-29) in TR38.901.

Annex <B>:
System level simulation

# B.1	System level simulation assumptions

Table B.1-1: SLS assumptions common to SBFD and Dynamic/flexible TDD

| Parameters | FR1 | FR2-1 |
| --- | --- | --- |
| Carrier frequency | 4GHz | 30GHz |
| BS antenna array configuration | Companies to report the BS antenna configurations |  |
| UE antenna configuration | Companies to report the UE antenna configurations |  |
| BS antenna radiation pattern | - InH: reuse the ceiling-mount antenna pattern in Table 10 in Report ITU-R M.2412 - Urban Macro/ Dense Urban Macro layer / Dense Urban Micro layer: reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) Companies can also consider evaluation with other realistic BS antenna radiation pattern |  |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | Reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) |
| Channel Model | Refer to Annex A.3 |  |
| Traffic model | Refer to Annex A.6 |  |
| Layout | Refer to Annex A.1.1 |  |
| UE distribution | Refer to Annex A.1.2 |  |
| System bandwidth | 100MHz | Optional: 100MHz Baseline: 200MHz |
| Numerology | 14 OFDM symbol slot, SCS = 30kHz | 14 OFDM symbol slot, SCS = 120kHz |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Open loop power control parameters | Companies to report power control parameters. |  |
| BS receiver noise figure | Refer to Annex A.2.8 |  |
| UE receiver noise figure | 9 dB | 10 dB |
| UE receiver | MMSE-IRC as the baseline receiver. NOTE: Advanced receiver is not precluded. |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Companies to report the option used. Option 1: Ideal Option 2: Realistic |  |
| UE processing capability | UE processing capability 1 as baseline |  |
| Handover margin | 3 dB |  |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |
| DL/UL Modulation | Up to 256QAM |  |
| Transmission scheme | Companies to report transmission schemes (e.g., SU-MIMO, MU-MIMO, maximum layers for SU-MIMO/MU-MIMO, etc) |  |
| Scheduling | PF |  |
| Overhead | Companies to report the overhead assumption |  |

Table B.1-2: SLS assumptions specific to SBFD Deployment Case 1

|  | FR1 |  |  | FR 2-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters | Indoor office | Urban Macro / Dense Urban Macro layer | Dense Urban with 2-layer | Indoor office | Dense Urban Macro layer | Dense Urban Micro layer |
| BS transmit power for legacy TDD | 24 dBm for 100MHz | Urban Macro - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz Dense Urban Macro layer - Option 1: 53 dBm for 100MHz - Option 2: 44 dBm for 100MHz | Dense Urban Macro layer - Option 1: 53 dBm for 100MHz - Option 2: 44 dBm for 100MHz Dense Urban Micro layer - 38 dBm for 100MHz | 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm. | Option-1: 30 dBm for both 100MHz and 200MHz. Option-2: 40 dBm for both 100MHz and 200MHz. | 30 dBm for both 100MHz and 200MHz. EIRP should not exceed 68 dBm. |
| BS transmit power for SBFD | Refer to Annex A.5. Companies to report the assumption of BS transmit power on DL slots and SBFD slots in SBFD operation. |  |  |  |  |  |
| SBFD subband and slot configurations | Alt 1/2/4 in Annex A.7 are used for comparison between baseline legacy TDD network and SBFD deployment case 1. |  |  |  |  |  |

Table B.1-3: SLS assumptions specific to SBFD Deployment Case 3-2 (FR1)

| Parameters | 2-layer Scenario B |
| --- | --- |
| BS transmit power for legacy TDD | For 2-layer scenario B in FR1, reuse the BS transmission power of Urban Macro scenario for Layer 1, and reuse the BS transmission power of Indoor office scenario for Layer 2, e.g., Layer 1: Urban Macro - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz Layer 2: Indoor office - 24 dBm for 100MHz |
| BS transmit power for SBFD | Refer to Annex A.5. Companies to report the assumption of BS transmit power on DL slots and SBFD slots in SBFD operation. |
| SBFD subband and slot configurations | Layer 1; Layer 2. baseline legacy TDD network (Baseline for comparison with SBFD Deployment Case 3-2); Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]. SBFD Deployment Case 3-2; Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]; Companies to report which option is used: - Option 1: SBFD Frame structures in Alt2(XXXXU) in Annex A.7 - Option 2: SBFD Frame structures in Alt4(XXXXX) in Annex A.7. |
| UE attachment | UE selected macro TRP or indoor TRP is determined based on the RSRP from port 0, i.e., the UE in the indoor office/indoor factory can select the Macro cell as serving cell, and the UE outside the indoor office/indoor factory can select the indoor TRP as serving cell |

Table B.1-4: SLS assumptions specific to SBFD Deployment Case 4 (FR1 & FR2)

| Parameters | Urban Macro (FR1) | Dense Urban Macro layer (FR2-1) |
| --- | --- | --- |
| BS transmit power | - Different power levels in adjacent carriers can be simulated, and it is up to company to report the power levels. - Company to report the power levels on DL slots and SBFD slots in SBFD operation. |  |
| SBFD subband and slot configurations | Operator#1; Operator#2. baseline legacy TDD network (Baseline for comparison with SBFD Deployment Case 4); Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]. SBFD Deployment Case 4; Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]; Companies to report which option is used: - Option 1: SBFD Frame structures in Alt2(XXXXU) in Annex A.7 - Option 2: SBFD Frame structures in Alt4(XXXXX) in Annex A.7. |

Table B.1-5: SLS assumptions specific to dynamic/flexible TDD

| Parameters | FR1 |  |  | FR2-1 |  |
| --- | --- | --- | --- | --- | --- |
|  | Indoor office | Urban Macro | 2-layer Scenario B | Indoor office | Dense Urban Macro layer |
| BS transmit power | 24 dBm for 100MHz | - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz | Urban Macro - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz Indoor office/indoor factory - 24 dBm for 100MHz | 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm. | Option-1: 30 dBm for both 100MHz and 200MHz. Option-2: 40 dBm for both 100MHz and 200MHz. |

Table B.1-6: SLS assumptions for interference modelling

| Parameters | FR1 | FR2-1 |
| --- | --- | --- |
| gNB self-interference - αSI | Candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. Companies shall report what values of the individual components are assumed in order to achieve the αSI value corresponding to 1 dB desense. Other approaches of determining values for αSI are not precluded and can be used and reported by companies. |  |
| Co-site inter-sector co-channel inter-subband CLI - αco-site | For co-site inter-sector co-channel inter-subband CLI modelling, RAN1 assume the following only for evaluation: - FR1: - 75dB for spatial isolation (RAN4 typical value). - 93dB for spatial isolation (RAN4 best value). - 100dB for spatial isolation - FR2: - 88dB for spatial isolation (RAN4 typical value). - 98dB for spatial isolation (RAN4 best value). - 105dB for spatial isolation - In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible. - The feasibility of these values is up to RAN4. These values can be revisited based on further RAN4 inputs. - The 100dB/105dB isolation values for FR1 and FR2 are not from RAN4, but based on RAN4 input that some companies have proposed that isolating material could be added between sectors to increase the isolation. RAN4 has not yet discussed the details whether such approaches can be applied to outdoor sites. |  |
| BS ACLR - ACLRBS | 45 dB | 28 dB |
| BS ACS - ACSBS | 46 dB | 23.5 dB |
| UE ACLR - ACLRUE | 30 dB | 23 dB |
| UE ACS - ACSUE | 33 dB | 23 dB |

# B.2	System level simulation results

Annex <C>:
System level simulation calibration

# C.1	SLS calibration methodology

Calibration scenarios

The SLS calibration focuses on the following scenarios of SBFD Deployment Case 1:

-	FR1

-	Urban Macro

-	Indoor office

-	FR2-1

-	Dense Urban Macro layer

-	Indoor office

Calibration metrics

The following metrics are considered for SLS calibration:

-	CDF of gNB-UE coupling loss

-	CDF of Inter-gNB coupling loss

-	CDF of Inter-UE coupling loss

-	Optional: CDF of DL SINR for legacy TDD/ DL SINR in DL-only slots for SBFD

-	Optional: CDF of DL SINR in SBFD slots

-	Optional: CDF of UL SINR for legacy TDD/ UL SINR in UL-only slots for SBFD

-	Optional: CDF of UL SINR in SBFD slots

Table C.1-1: Statistic method for SLS calibration

| SLS calibration metrics | Statistic method |
| --- | --- |
| gNB-UE coupling loss | Only the coupling losses between each UE and its serving cell are collected for CDF statistic. - wA and gB are determined by selecting the best beam pair of the UE and its serving cell with the criteria of maximizing receive power of the UE. |
| Inter-gNB coupling loss | For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair - The two gNBs in each gNB pair should be from different sites. - Both wA and gB are randomly selected for calculating the coupling loss for each gNB pair. Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. |
| Inter-UE coupling loss | For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair - For all of Urban Macro, Dense Urban Macro Layer and Indoor office scenarios, if the 2D distance between two UEs in a UE pair is larger than 50m, the UE pair is not considered for statistic. - For each UE, wA and gB is determined based on the best beam pair of the UE and its serving cell. Companies to run enough SLS drops and report the number of SLS drops when plotting the CDF using the collected coupling losses. |
| NOTE 1: For calibration, only large scale fading is modelled. Formula (A-16) in Annex A.8 for CL with averaging across all the Tx/Rx ports is used for coupling loss calculation above, i.e., CLA,BwA,gB=1Sp=0S-11Uu=0U-1CLp,uA,BwA,gB=1Sp=0S-1PL∙SF∙1Uu=0U-1α'0,u,p2 - Not modelling fast fading doesn’t impact the calculation of path loss PL and shadowed fading SF - The antenna pattern related part (α'0,u,p) is calculated based on the LOS direction between the two nodes, i.e., θLOS,ZOA/ϕLOS,AOA/θLOS,ZOD/ϕLOS,AOD NOTE 2: The beams for above cases are selected based on a defined set of beams for FR1 and FR2 in the table for calibration assumptions. |  |

# C.2	SLS calibration assumptions

Table C.2-1: SLS calibration assumptions

|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- |
| Carrier frequency | 4 GHz | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| BS transmit power for SBFD | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 23 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |  |  |  |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  | 12 TRPs per 120m x 50m x 3m |  |
| Inter-BS (2D) distance | 500m | 200m | 20m |  |
| Minimum BS-UE (2D) distance | 35m | 35m | 0m |  |
| Minimum UE-UE (2D) distance | 1m | 1m | 1m |  |
| BS antenna height | 25m | 25m | 3m |  |
| UE distribution | UE Clustering | UE Clustering | Uniform |  |
| UE number per macro/indoor TRP (per direction) (M) | 20 | 10 | 10 |  |
| UE cluster number per macro cell (X) | 2 | 1 | - |  |
| UE number per cluster | 8 | 8 | - |  |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 100% outdoor without car penetration loss: 3km/h | 100% indoor in houses: 3km/h |  |
| Indoor UE height (m) | 1.5m | 1.5m | 1.5m |  |
| Outdoor UE height (m) | 1.5m | 1.5m | - |  |
| Radius of cluster (R) | 25m | 20m | - |  |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m | 55m | - |  |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m | - | - |  |
| gNB-UE Channel model | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  | TRP-to-UE: InH-Office in TR 38.901 Penetration loss is not modelled. |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) Penetration loss is not modelled. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  |
| BS antenna array configuration for Legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) =(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,16,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: (M,N,P,Mg,Ng) = (16,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna radiation pattern | reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) |  | reuse Table 10 in Report ITU-R M.2412 (Table A.2.1-7 in TR 38.802) |  |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) |
| BS receiver noise figure | 5dB | 7 dB | 5dB | 7 dB |
| UE receiver noise figure | 9 dB | 13 dB | 9 dB | 13 dB |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -86 dBm, alpha = 0.9 | P0= -60 dBm, alpha = 0.6 |  |
| Handover margin (dB) | 3 dB | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 180° in GCS (pointing to the ground) | 180° in GCS (pointing to the ground) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {5*pi/8, 7*pi/8} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {pi/4, 3*pi/4} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

# C.3	SLS calibration results

The SLS calibration results can be found in R1-2304212.

Annex <D>:
Link level evaluation for coverage performance

# D.1	Link level simulation assumptions

This clause describes the link level simulation assumptions for FR1 and FR2-1. Table D.1-1 shows the general parameters for FR1. Table D.1-2 shows PUSCH specific parameters for FR1. Table D.1-3 shows the general parameters for FR2-1. Table D.1-4 shows PUSCH specific parameters for FR2-1.

Table D.1-1: General parameters for FR1

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Urban Macro: 4GHz |
| Frame structure for TDD | TDD: DDDSU (S: 10D:2G:2U) SBFD: XXXXU, where X denotes SBFD slot. - For SBFD slot, {DUD} pattern is assumed. - 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> |
| Target data rates for eMBB | UL 1Mbps |
| Pathloss model (select from LoS or NLoS) | gNB-UE: NLOS gNB-gNB (if modelled in LLS): LOS: NLOS = 3:1 |
| BWP | 100MHz |
| Channel model for link-level simulation | gNB-UE: TDL-C, CDL-C Note: Company can provide simulation results based on either TDL channel or CDL model Note: Companies can report gNB-gNB channel model if modelled in LLS. |
| Delay spread | 300ns Note: Other values can be reported by companies. |
| UE velocity | 3km/h for indoor |
| Number of antenna elements for BS | SBFD antenna configuration option-2, - 192 antenna elements - (M,N,P,Mg,Ng) = (12,8,2,1,1) - (optional) 128 antenna elements - (M,N,P,Mg,Ng) = (8,8,2,1,1) - Note: it is the same for both SBFD and non-SBFD slots Note: Companies to report the details if other antenna configurations are used. |
| Number of TxRUs for BS | gNB architectures to study: SBFD antenna configuration option-2, - 64 TxRUs - Note: it is the same for both SBFD and non-SBFD slots Note: Companies to report the details if other antenna configurations are used. gNB modelling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |

Table D.1-2: Channel-specific parameters for PUSCH for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM |
| SCS | 30kHz |
| PUSCH duration | 14 OS |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |

Table D.1-3: General parameters for FR2-1

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Dense Urban Macro: 30GHz |
| Frame structure for TDD | TDD: DDDSU (S: 10D:2G:2U) SBFD: XXXXU where X denotes SBFD slot. - For SBFD slot, {DUD} pattern is assumed, - 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <52, 26, 1> |
| Target data rates for eMBB | UL: 5Mbps |
| BWP | 200MHz |
| Pathloss model (select from LoS or NLoS) | gNB-UE: NLOS gNB-gNB (if modelled in LLS): LOS: NLOS = 3:1 |
| Channel model for link-level simulation | gNB-UE: CDL- A, TDL-A Note: Company can provide simulation results based on either TDL channel or CDL model Note: Companies can report gNB-gNB channel model if modelled in LLS. |
| Delay spread | 100ns Note: Other values can be reported by companies. |
| UE velocity | 30 km/h for outdoor |
| Number of antenna elements for BS | SBFD antenna configuration option-2, - 256 antenna elements - (M,N,P,Mg,Ng) = (16,8,2,1,1) - Note: it is the same for both SBFD and non-SBFD slots |
| Number of TxRUs for BS | 2 Note: Analog beamforming is assumed. |
| Number of UE antenna elements | 8, one panel:(M, N, P) = (2,2,2) |

Table D.1-4: Channel-specific parameters for PUSCH for FR2-1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER, Optional: companies report iBLER. w/o HARQ, 10% iBLER. |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| DMRS configuration | For 30km/h: Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM |
| SCS | 120kHz. |
| PUSCH duration | 14 OS |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 26 PRBs for 5Mbps for PUSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |

# D.2	Link budget template

For coverage performance evaluation for SBFD, the link budget template in Table A.3 in TR 38.830 is reused with the following modifications.

| (10) Number of receive antenna elements | SBFD antenna configuration option-2, FR1: - 192 antenna elements - (M,N,P,Mg,Ng) = (12,8,2,1,1) - (optional) 128 antenna elements - (M,N,P,Mg,Ng) = (8,8,2,1,1) FR2: - 256 antenna elements - (M,N,P,Mg,Ng) = (16,8,2,1,1) Note: Companies to report the details if other antenna configurations are used. |
| --- | --- |
| (10a) Number of receive TxRUs | SBFD antenna configuration option-2, FR1: - 64 TxRUs FR2: - 2 TxRUs Note: Companies to report the details if other antenna configurations are used. |

# D.3	Link level evaluation results

Annex <E>:
Adjacent channel co-existence evaluation

# E.1	RAN4 co-existence simulation scenarios

# E.2	RAN4 co-existence simulation assumption

# E.3	RAN4 co-existence simulation methodology

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN1#109-e | R1-2205399 |  |  |  | Skeleton | 0.0.1 |
| 2022-05 | RAN1#109-e | R1-2205692 |  |  |  | Endorsed skeleton | 0.0.2 |
| 2022-11 | RAN1#111 | R1-2212374 |  |  |  | Capture evaluation methodology agreements until RAN1#110-b meeting | 0.1.0 |
| 2023-02 | RAN1#112 | R1-2300997 |  |  |  | Capture evaluation methodology agreements until RAN1#111 meeting | 0.2.0 |
| 2023-04 | RAN1#112b-e | R1-2303230 |  |  |  | Version not formally endorsed by RAN1. Capture evaluation methodology agreements until RAN1#112 meeting; Update TR skeleton of RAN4 part | 0.3.0 |
| 2023-05 | RAN1#113 | R1-2305091 |  |  |  | Capture agreements and TP until RAN1#112b-e meeting | 0.4.0 |
| 2023-06 | RAN1#113 | R1-2306344 |  |  |  | MCC clean-up - no changes compare to v0.4.0 | 0.4.1 |