3GPP TSG RAN WG1 #113 		                                    		       R1-2306216

Incheon, Korea, May 22nd – May 26th, 2023

Title:	LS to RAN4 on SRS and PRS bandwidth aggregation for positioning

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

Name:	Chuangxin Jiang

E-mail Address:	jiang.chuangxin1@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

With regard to SRS and PRS bandwidth aggregation for NR positioning, RAN1 made the following agreements

Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA. Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed. Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: - A request to indicate UE which two or three PFLs to be used for performing joint measurement - A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements - Support at least the values of k={-1,-2} - FFS other values e.g. -3, -4, -5, -6 - Send RAN4 an LS to confirm the feasibility

Based on the first agreement, SRS for bandwidth aggregation may be configured within an UL CC without PUSCH/PUCCH. So, RAN1 would like RAN4 to provide the retuning time values, i.e. the guard period values as described in the second agreement,

With regard to the third agreement, RAN1 would like RAN4 to check the feasibility of the negative k values.

2. Actions:

To RAN4:

ACTION: RAN1 respectfully asks RAN4 to take the above information into account, and provide the feedback to RAN1.

3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #114              21 August – 25 August, 2023        Toulouse, France

TSG RAN WG1 Meeting #114b            9 Oct. – 13 Oct., 2023        Xiamen, China