3GPP TSG RAN WG1 #114		                           R1- 2306896

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.12.3

Source:	LG Electronics

Title:	Discussion on dynamic waveform switching for NR coverage enhancement

Document for:	   Discussion and Decision

- Introduction

The following agreements were made for dynamic switching between DFT-S-OFDM and CP-OFDM of Rel-18 NR coverage enhancement WI during the RAN1 #113 meeting. [1]:

Agreement Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2. Agreement For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. - Details FFS. - Note: Any MAC CE related decision is up to RAN2 - Option 4: No enhancement.

In this contribution, dynamic waveform switching between DFT-S-OFDM and CP-OFDM for Rel-18 NR coverage enhancement is discussed.

- Discussion

According to the current specification, the gNB can determine either the DFT-S-OFDM or CP-OFDM as the UL waveform of PUSCH via higher layer parameter, such as transformPrecoder. This means that if the gNB wants to change the UL waveform for UE, the gNB could only change it by updating the higher layer parameter. To improve this inefficient operation, dynamic waveform switching is introduced for Rel-18 coverage enhancement.

Applicability of dynamic waveform switching

During the previous meetings, there were discussions on whether to support the dynamic waveform switching in the scenario of indicating not only for single carrier but also for multi-carrier multiple PUSCH transmissions scheduling with single DCI. Many companies agreed that it is possible and enough to indicate waveform with 1-bit DWS field for the multiple PUSCH transmissions scheduled across multi-carrier, which monitored by DCI format 0_3 newly introduced in the multi-carrier work item. However, it is not necessary to discuss the detail of DCI composition for dynamic waveform switching support in the Rel-18 UL Coverage WI. Since the DWS field is one of components of DCI format 0_3, further discussion will be unnecessarily repeated in the multi-carrier WI in case other information fields are dependent by the DWS field. Therefore, if dynamic waveform switching is supported in a multi-carrier scheduling scenario, details of DCI components need to be specified in the corresponding multi-carrier WI.

Observation 1: Even if dynamic waveform switching is supported for multi-carrier scheduled PUSCH, the detailed discussion regarding the component of DCI format 0_3 should be conducted within the multi-carrier WI.

Configuration of dynamic waveform switching

After agreeing to support the dynamic waveform switching configuration for each bandwidth part in the RAN1#112b-e, there was a further agreement in the last RAN1#113 meeting that separate configuration of the DWS field between DCI format 0_1 and DCI format 0_2 is possible within the bandwidth part. This means the advantage of efficiently adjusting the payload size of DCI format 0_2 according to the needs of the information field in the DCI.

For example, when the dynamic waveform switching is supported, DCI format 0_1 indicates dynamic waveform switching via the DWS field, while DCI format 0_2 may not need dynamic waveform switching for a specific URLLC purpose. In this case, a separate configuration by higher layer parameter can be useful to exclude the DWS field from DCI format 0_2.

To make aforementioned things possible, higher layer parameters that configure the support for dynamic waveform switching as well as presence of DWS field for DCI format 0_2 should be defined. Firstly, a new RRC parameter, e.g., dws-Config = {enabled, disabled}, can be introduced to enable/disable dynamic waveform switching. This allows the availability of dynamic waveform switching to be configured while maintaining the existing semi-static waveform configuration through the trasnformPrecoder. Additionally, a specific RRC parameter for DCI format 0_2, e.g., dwsField-DCI-0-2 = {enabled, disabled}, can be introduced. This parameter allows to configure the presence or absence, e.g., enable or disable respectively, of the DWS field in DCI format 0_2.

Furthermore, decision of which waveform that UE needs to follow when the DWS field is not present in DCI format 0_2 should also be discussed. For example, the simplest way is the UE just following the RRC configured waveform, e.g., transformPrecoder in PUSCH-Config.

Proposal 1: Introduce new higher layer parameters to enable/disable dynamic waveform switching, e.g., dws-Config = {enabled, disabled}, and presence of DWS field in DCI format 0_2, e.g., dwsField-DCI-0-2 = {enabled, disabled}, in PUSCH-Config.

Proposal 2: Further discussion is necessary on UE behavior when the DWS field is not presented in DCI format 0_2 if dynamic waveform switching is enabled by higher layer signaling.

When UE applies CP-OFDM, there is no restriction on FDRA/DMRS configuration type, but when transform precoding is enabled, there is a limitation on the configuration. That is, when DFT-S-OFDM is applied, only FDRA type 1 and DMRS type 1 can be applied. This is problematic if UE is indicated to be dynamically switched to DFT-S-OFDM but the configuration is not the above type. In the RAN1#113 meeting, there was a discussion how to handle the configuration type of FDRA or DMRS according to the dynamic waveform switching operation, but could not proceed with further down-select since the preferred option was divided by half for each company. Option 2 aims to allow the use of FDRA/DMRS configuration type without any restriction. UE follows the higher layer parameter resourceAllocation for FDRA and dmrs-Type for DMRS when CP-OFDM is applied while limited only to FDRA/DMRS configuration type 1 if DFT-S-OFDM is indicated. Therefore, UE is necessary to interpret FDRA/DMRS configuration type differently according to the indicated waveform.

On the other hand, Option 1 is a much simpler solution that generally applies FDRA/DMRS configuration type 1 regardless of the waveform considering the infrequent use of dynamic waveform switching and the main purpose of uplink coverage compensation using DFT-S-OFDM. It seems that there will be no critical issue even if FDRA/DMRS configuration type 1 is uniformly configured for both waveforms only for the situations that support dynamic waveform switching.

Proposal 3: Configuration restriction via higher layer signaling can sufficiently handle the FDRA/DMRS error case regardless of the indicated waveform as described in the Option 1.

In the last RAN1#113 meeting, there was a discussion on the BWP switching case, especially when the UE is indicated to switch from the source BWP that does not support DWS to the target BWP that supports DWS. According to clause 12 of TS 38.213 in the Table 2.2-1, the DWS field of the target BWP is prepended with a 1-bit size ‘0’ value in that situation. This ‘0’ value is automatically generated according to the specification, so it is not desirable to interpret directly, and some companies proposed that the UE could follow the RRC configured waveform.

# 12 Bandwidth part operation … If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the DCI format - if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively …

Table 2.2-1. TS 38.213 description about zero prepending case during UL BWP switching

Prior to this, there was a discussion on how to interpret the value of the DWS field. One of proposals is to align ‘0’ and ‘1’ with transform precoding ‘enabled’ and ‘disabled’ respectively as in the FL proposal 2-1.

Following this proposal, the interpretation of the value of the DWS field results in only using transform precoding ‘enabled’, i.e., DFT-S-OFDM, for BWP switching situation with zero prepending, so some companies proposed to follow the RRC configured value for that case.

There is a proposal to solve this unintentional UE behavior effectively at once. That is, instead of hard-coding ‘0’ and ‘1’ as DFT-S-OFDM and CP-OFDM, respectively, link each bit-value to the value of a higher layer parameter, e.g., msg3-transformPrecoder. In this way, the bit-value ‘0’ maps to same transform precoding, and otherwise ‘1’ maps to the other transform precoding that leads to the alternative statement, Alt 2, provided as follows.

Alt 1: (Latest FL proposal 2-1) Value “0” of dynamic waveform switching indicator field maps to transform precoding enabled. Value “1” of dynamic waveform switching indicator field maps to transform precoding disabled. Alt 2: (A different way of suggestion) Value “0” of dynamic waveform switching indicator field maps to the same transform precoding as higher layer parameter (e.g., msg3-transformPrecoder). Value “1” of dynamic waveform switching indicator field maps to the different transform precoding as higher layer parameter (e.g., msg3-transformPrecoder).

Proposal 4: Consider the bit-value of the DWS field refers to the higher layer parameter, e.g., msg3-transformPrecoder.

- ‘0’ maps to the same transform precoding as higher layer parameter.

- ‘1’ maps to the different transform precoding as higher layer parameter.

Assistance information of dynamic waveform switching

In the last RAN1#113 meeting, there was an in-depth discussion on PHR enhancement as assistance information of dynamic waveform switching [2]. When calculating PCMAX, one of PH information based on the reference PUSCH assumes MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB, ΔTC = 0 dB, and the PUSCH scheduled with 1 RB and 15kHz. Some companies concerned that the reference PUSCH is not suitable for computing PCMAX with a target waveform comparing with that actual PUSCH when the dynamic waveform switching is supported. To address this problem, the concept of an assumed PUSCH was discussed and defined to calculate the PH information for the target waveform based on the assumed PUSCH with the same RB, MCS, and etc. of actual PUSCH.

On the other hand, there was a consistent objection that PHR enhancement is not necessary for the dynamic waveform switching, i.e., legacy PHR mechanism is sufficient, and there was a guidance from the RAN#100 plenary meeting to decide whether to define any PHR enhancement at the beginning of the next RAN1#114 meeting [3].

The gNB scheduler operates the UL power control based on the PH information in PHR including PCMAX, PHR value, and MPE if any in FR2 of the actual PUSCH, but if the existing PHR is applied as it is, the dynamically indicated waveform might be determined and indicated without proper timely information. By providing the PH information about the assumed PUSCH of the target waveform as discussed in Option 1, at least reflecting the PCMAX and PHR values for both actual and assumed PUSCH together, the accuracy of dynamic waveform decision and UL power control can be increased for dynamic waveform switching scenario.

Additionally, in the last meeting, there were two alternatives on how to compose the information when the PH information of the assumed PUSCH is decided to be reported. One is to report the PH information of the assumed PUSCH and legacy Type 1 PH together. The other is to report only PH information of the assumed PUSCH. It is important to point out that this is about enhancements to the dynamic waveform switching between the different waveforms as we have discussed continuously, so as Alt 1, it is more desirable to fully report both PH information of the assumed and actual PUSCH together.

Moreover, the UE transmission power must be appropriately adjusted according to the dynamically indicated waveform from the gNB. With the PHR value per its waveform reported by the UE, the gNB can estimate how much available power left to the UE.

Considering the UL power control technique when the UL waveform is dynamically indicated as mentioned above, there may be a need for an additional procedure, such as closed-loop power control, for the gNB to indicate that UE should modify its transmission power level. In other words, the UE transmission power is adjusted after the waveform is changed subsequently, which may result in a delay during the closed-loop power control procedure.

Therefore, in order for the UE to immediately change the transmission power for the dynamic waveform, the gNB can consider providing and individually managing open-loop power control parameters for each waveform, i.e. CP-OFDM/DFT-S-OFDM, respectively. For example, open-loop power control related parameters, P0 offset for the target received power or  offset for the path-loss compensation can be accepted.

Proposal 5: Providing both PH information based on the assumed PUSCH and legacy Type 1 PH together is beneficial to the scheduler to make a timely decision on dynamic waveform switching.

Proposal 6: Introduce additional open-loop power control parameters for both waveforms, i.e. , P0 for each waveform, that can enhance accuracy of waveform determination for both actual and assumed PUSCH transmission.

- Conclusion

In this contribution, we discussed on the dynamic waveform switching between DFT-S-OFDM and CP-OFDM for Rel-18 NR coverage enhancement with following proposals and the observation.

Observation 1: Even if dynamic waveform switching is supported for multi-carrier scheduled PUSCH, the detailed discussion regarding the component of DCI format 0_3 should be conducted within the multi-carrier WI.

Proposal 1: Introduce new higher layer parameters to enable/disable dynamic waveform switching, e.g., dws-Config = {enabled, disabled}, and presence of DWS field in DCI format 0_2, e.g., dwsField-DCI-0-2 = {enabled, disabled}, in PUSCH-Config.

Proposal 2: Further discussion is necessary on UE behavior when the DWS field is not presented in DCI format 0_2 if dynamic waveform switching is enabled by higher layer signaling.

Proposal 3: Configuration restriction via higher layer signaling can sufficiently handle the FDRA/DMRS error case regardless of the indicated waveform as described in the Option 1.

Proposal 4: Consider the bit-value of the DWS field refers to the higher layer parameter, e.g., msg3-transformPrecoder.

- ‘0’ maps to the same transform precoding as higher layer parameter.

- ‘1’ maps to the different transform precoding as higher layer parameter.

Proposal 5: Providing both PH information based on the assumed PUSCH and legacy Type 1 PH together is beneficial to the scheduler to make a timely decision on dynamic waveform switching.

Proposal 6: Introduce additional open-loop power control parameters for both waveforms, i.e. , P0 for each waveform, that can enhance accuracy of waveform determination for both actual and assumed PUSCH transmission.

- Reference

[1] RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #113

[2] R1-2306255, “Summary #5 on dynamic switching between DFT-S-OFDM and CP-OFDM”, Moderator (InterDigital, Inc.)

[3] RP-231498, “Summary for discussion on RAN1 UL Coverage”, NTT DOCOMO