3GPP TSG RAN WG1 #114			R1-2307040

Toulouse, France, August 21st – August 25th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on HARQ-ACK for multicast SPS PDSCH and for unicast SPS PDSCH multiplexed on the same PUCCH with same priority |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current specification, when unicast SPS PDSCH and multicast SPS PDSCH with HARQ-ACK information are multiplexed on the same PUCCH with same priority, the UE determines the PUCCH resource from the SPS-PUCCH-AN-List configured for unicast. However, if SPS-PUCCH-AN-List for unicast is not configured, the UE behaviour on PUCCH resource determination is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the case if SPS-PUCCH-AN-List for unicast is not configured, the UE determine PUCCH resource from SPS-PUCCH-AN-ListMulticast configured for multicast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When unicast SPS PDSCH and multicast SPS PDSCH with HARQ-ACK information are multiplexed on the same PUCCH with same priority, and SPS-PUCCH-AN-List for unicast is not configured, the UE can’t determine the PUCCH resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

< Unchanged parts are omitted >

f a UE multiplexes in a PUCCH first HARQ-ACK information associated with unicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information in a same PUCCH, the UE determines the PUCCH resource from

-	if provided, SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1;

-	else, if provided, PUCCH-Config/PUCCH-ConfigurationList for multicast PDSCH receptions;

-	else, PUCCH-Config/PUCCH-ConfigurationList for unicast PDSCH receptions.

If a UE multiplexes in a PUCCH first HARQ-ACK information associated with unicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast SPS PDSCH receptions and having same priority value as the first HARQ-ACK information in a same PUCCH, the UE determines the PUCCH resource from

- If provided, SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1;

- else, sps-PUCCH-AN-ListMulticast for multicast SPS PDSCH recetions as described in clause 9.2.1.

< Unchanged parts are omitted >