3GPP TSG RAN WG1 #114                                          R1-2307079

Toulouse, France, August 21st – August 25th, 2023

Source: 	CATT

Title:	Discussion on other aspects for AI/ML beam management

Agenda Item:	9.2.3.2

Document for:	Discussion and Decision

- Introduction

In RAN1#113 meeting, the following agreements and conclusion were made for other aspects on AI/ML for beam management [1].

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study potential spec impact(s) from the following aspects in addition to those included in previous agreements: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - FFS: definition of an event and the performance metric(s) used to identify it - Indication from NW for UE to do LCM operations Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding performance monitoring, study the necessity and potential spec impact(s) of the mechanism that facilitate UE to detect whether the functionality/model is suitable or no longer suitable. Conclusion For the study of DL beam pair prediction of BM-Case1 and BM-Case2 with a UE-side AI/ML model, RAN1 has no consensus to support the reporting of the predicted Rx beam(s) (e.g., Rx beam ID, Rx beam angle information, etc) from UE to network. Agreement For BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - Reporting information about measurements of multiple past time instances in one reporting instance for BM-Case2 - Note: only applicable to network-side AI/ML model - Note: The potential performance gains of measurement reporting should be justified by considering UCI payload overhead Agreement For BM-Case1 and BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - How to perform beam indication of beams in Set A not in Set B - Note: the legacy mechanism may be sufficient Agreement Regarding data collection for BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the benefits, necessity and potential specification impact of the following aspect on top of those we have agreed in previous meeting: - Assistance information from NW to UE for UE data collection for categorizing the data for the purpose of differentiating characteristics of data - The assistance information should preserve privacy/proprietary information. Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) at least from the following aspects: - information regarding model inference - Set A / Set B configuration - performance monitoring - data collection - assistance information

In this contribution, we will discuss the potential specification impacts for above BM-Case1 and BM-Case2.

- Discussion

  - Feasibility of beam pair prediction

For the type of beam prediction, one debating point in previous meetings is whether the DL beam pair prediction is feasible since it may require the gNB and UE to disclose the beam information. In RAN1#112bis-e meeting, the following proposal has been discussed [2].

Proposal 3.2.1: For DL beam pair prediction of BM-Case1 and BM-Case2 with a network-side AI/ML model, study the feasibility and potential spec impacts (if feasible) from the following aspects as a starting point - Whether/How to align the common understanding between NW and UE on the mapping between beam pairs and UE’s associated Rx beams - Association/mapping of beams/beam pairs within Set A and beams within Set B - Whether/How to indicate a beam pair / Tx beam /Rx beam from NW to UE - Whether/How Rx beam related information corresponding to a Tx beam reported from UE to NW - Generalization aspects, e.g., different UE Rx beam shapes/directions, different UE orientation/location - Potential assistance information - Note1: The potential down-selection/prioritization (if any) on the types of beam prediction is a separate discussion - Note2: The performance, overhead and spec impacts should be considered. - Note3: Potential reporting and assistance information should not disclose proprietary/privacy information

For NW-side model, it is not practical to train a model for each UE due to large model training and storage overhead, so that the AI/ML model would be used to predict the best beam pairs for different UEs. Since the Rx beam patterns are implementation dependent, different UEs can have different Rx beam patterns. The model generalization capability of different Rx beam patterns should be considered when training the model. One key issue for beam pair prediction is how to map correct Tx beam ID and Rx beam ID as model input when a model could be used for beam pair prediction for different UEs. For Tx beam ID, the NW can generate an association between SSB/CSI-RS and Tx beam IDs, when UE reports the L1-RSRP and the RS indicator, the corresponding Tx beam ID can be obtained by the NW. For Rx beam ID, the UE can associate each Rx beam with an ID, that is local Rx beam ID. Then, UE can report the association relationship between relative Rx beam info and local Rx beam ID to NW, and NW can decide how to map the local Rx beam ID from each UE to global Rx beam ID and the mapping rule between local Rx beam ID and global Rx beam ID for each UE will be stored by the NW. When UE reports the RSRP, the corresponding local Rx beam ID should also be reported. Then, NW can map the local Rx beam ID to global Rx beam ID for the reported RSRP and the global ID can be explicitly or implicitly used as model input. It should be noted that different UEs may have different association relationships between relative Rx beam info and local Rx beam ID. Hence, the reporting overhead and storage overhead of the association relationships should be considered. After model inference, the NW can map the predicted best Rx beam ID to local Rx beam ID and send the beam info to UE for providing the spatial Rx parameters of DL reception. The procedure for beam pair prediction of NW-side model is shown in Figure 1.

Figure 1 Procedure for beam pair prediction of NW-side model

In RAN1#112 meeting, there is no consensus on directly reporting UE Rx beam shape/direction. To facilitate the NW mapping of the correct Rx beam IDs as model input, the contents of relative Rx beam information with preserving sensitive proprietary information need to be studied. One intuitive way is dividing the angular coverage into multiple grids, and the coverage of each grid can be determined by considering the reporting overhead and accuracy requirement. Then, UE can report the association between the local Rx beam IDs and the corresponding grids to NW. This method is similar as providing beam information of TRP in Rel-17 positioning.

Proposal 1: For DL beam pair prediction with a NW-side model, considering the generalization of different UE Rx beam shapes/patterns, study how to report/send relative Rx beam information when preserving sensitive proprietary information and considering the reporting overhead and storage overhead.

For UE-side model, the AI/ML model can be trained by NW and transferred/delivered to UE, or trained by UE side. If the model is trained by NW, the model can be a cell-specific/gNB-specific/area-specific model. The model training procedure is the same as that of NW-side model shown in Figure 1. For model inference at UE side, the mapping rules of Tx beam ID and Rx beam ID used by NW when model training should be known by the UEs. Otherwise, the UEs have no idea how to put the measured L1-RSRP values in correct order for model inputs. For Tx beam ID, the gNB and UE can define a mapping rule between RS ID and Tx beam ID to align the understanding between the NW and UE. Or, the mapping rules used when model training can be indicated from NW to UE. After model inference, UE can report the predicted best Tx beam ID to NW. The procedure for beam pair prediction of UE-side model trained by NW is shown in Figure 2.

Figure 2 Procedure for beam pair prediction of UE-side model trained by NW

For the model trained by UE side, the corresponding Tx beams of the training data set may be from various gNBs, and the number and patterns of Tx beams from various gNBs may be different. In such a case, UE can train a model for each Tx beam pattern, but the model storage overhead may be unacceptable. Another way is to train a model that can be used for several Tx beam patterns, and this way is a trade-off between model performance and storage overhead. Different gNBs may have different rules for associating the Tx beams with local Tx beam IDs, to facilitate the UE mapping the measurement from different gNBs with correct Tx beam ID as model input, NW can first send the association relationship between relative Tx beam info and local Tx beam ID to UE, then UE can decide how to map the local Tx beam ID to global Tx beam ID. During model training and model inference, the local Tx beam ID of the RS for measurement could also be provided. Then, UE could map the local Tx beam IDs to global Tx beam IDs. After model inference, UE can map the predicted best Tx beam ID to local Tx beam ID and report the local Tx beam ID to NW. The procedure for beam pair prediction of UE-side model trained by UE side is shown in Figure 3. It should be noted that the methods for providing the association relationship between relative Tx beam info and local Tx beam ID to UE can also be used in UE-side DL Tx beam prediction to train a model that can be used for different Tx beam patterns, and the association relationship between relative Tx beam info and local Tx beam ID can be viewed as the assistance information.

Figure 3 Procedure for beam pair prediction of UE-side model trained by UE-side

In the discussion of previous meetings, some companies thought that for UE-side model, there is no difference between Tx beam prediction and beam pair prediction from the specification perspective. We think the difference may be the CSI-RS configuration. For beam pair prediction, the CSI-RS/SSB will be configured to support the Rx beam sweeping. For Tx beam prediction, Rx beam sweeping may not be needed if the Tx beam prediction is with quasi-optimal Rx beam or a specific Rx beam.

Based on the above analysis, we propose the following proposals.

Proposal 2: For DL beam pair prediction with a UE-side model, study the following aspects:

- For model training at NW side, study how to report relative Rx beam information when preserving sensitive proprietary information

- For model training at UE side, study how to send/report relative Tx beam information when preserving sensitive proprietary information.

Proposal 3: For DL beam pair prediction with model training at NW side and inference at UE side, study how to align the mapping rule for Tx beam ID and Rx beam ID between the NW and UE.

  - Spec impacts

In this section, we will discuss the details of spec impacts of AI/ML-based beam management.

    - Model inputs

For the sub use case BM-Case1 and BM-Case2, the following conclusions for the AI/ML inputs was achieved in RAN1#109-e meeting [3] and RAN1#112 [4].  .

Conclusion@RAN1#109-e Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion @RAN1#109-e Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion@RAN1#112 Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction Conclusion@RAN1#112 Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement

Based on the simulation results in our contribution [5], the input of L1-RSRP measurement based on Set B with fixed pattern has the best performance. For Set B with pre-configured pattern, if beam (pair) ID is implicitly used as model input, the model performance is also acceptable.

Regarding using CIR based on Set B as AI/ML inputs, if AI/ML training or inference is at NW side, it will occupy significantly more UL feedback resources than using L1-RSRP measurement without significant performance improvement.

Different gNBs/UEs have different antenna patterns. It is impractical to train a model for each antenna pattern due to model training and model storage cost. To train an AI/ML model that can be used for different antenna patterns, the training data should include data collected under different antenna patterns, and the assistance information can be used as additional model input. The assistance information can be relative Tx/Rx beam info or dataset ID, which is helpful for identifying different antennas patterns.

Proposal 4: For the AI/ML mode input for BM-Case1 and BM-Case2, suggest to support at least following：

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID.

Proposal 5: For the AI/ML mode input for BM-Case1 and BM-Case2, suggest to further study using assistance information as model input when considering the generalization over different antenna/beam patterns.

    - Data collection

In RAN1#112bis-e [6], the following agreement regarding data collection for NW-side model was approved.

Agreement Regarding data collection for NW-side AI/ML model, study necessity, benefits and beam-management specific potential specification impact from RAN1 point of view on the following additional aspects - Mechanism related to the reporting - Additional information for content of the reporting - FFS: Information associated with or configured for the reported data samples, e.g., timestamps, SNR, data quality, etc. - Reporting overhead reduction - Note1: non-3GPP based solution is a separate issue. - Note2: The framework corresponding to higher layer(s) are up to the associated WG(s) - Note 3: Overhead, UE complexity and power consumption should be considered

During the discussion, several reporting overhead reduction mechanisms have been proposed. The contents of the collected data may include the L1-RSRPs with the indications/index of the beams (beam pairs). To reduce the reporting overhead, the first way is to reduce the number of reported beams (beam pairs). As example, when reporting the L1-RSRPs of Set A or Set B, UE can only report the beams with L1-RSRP larger than a certain threshold, and the L1-RSRPs of other beams can be set as a particular value (e.g.,0) at NW. The second way is to reduce the number of reported index. For example, if differential RSRP reporting method is used, UE can report the index of the largest measured L1-RSRP, and the other differential L1-RSRP values are mapped in a defined order, e.g., in an ascending order of the RS resources in the RS set for measurement. In this way, if M resources are configured for measurement, UE can report one index and M L1-RSRPs (one largest L1-RSRP and M-1 differential L1-RSRPs). The third way can be considered for reducing the reporting overhead is to use less bits to quantize the L1-RSRP. In current spec, the largest measured L1-RSRP is quantized to 7 bits and the differential L1-RSRP is quantized to 4 bits, whether less bits can be used when reporting the L1-RSRP can be evaluated in AI 9.2.3.1.

Proposal 6: Regarding data collection for NW-side model, study the following options for reporting overhead reduction:

- Option 1: Reducing the number of reported beams (beam pairs)

- Option 2: Reducing the number of reported index

- Option 3: Reducing the number of bits for RSRP quantization.

For model training at UE side, the UE can use the measurement results as model inputs and labels for model training. The details of model inputs and labels for model training at UE sided can be left to UE implementation.

In RAN1#112bis-e [7], the following agreement regarding data collection for UE-side model was approved.

Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact of UE reporting to network from the following aspect - Supported/preferred configurations of DL RS transmission - Other aspect(s) is not precluded Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact (if any) to initiate/trigger data collection from RAN1 point of view by considering the following options as a starting point - Option 1: data collection initiated/triggered by configuration from NW - Option 2: request from UE for data collection - FFS: details

According to the agreements, data collection can be initiated/triggered by NW or UE. If the data collection is triggered by NW, the NW should let UE know the intention of the RS transmission. For instance, if the RS transmission is for generating training dataset of AI/ML model at UE side, it is not expected to report the measurement results from UE to NW. For data collection initiated/triggered by UE, the NW may response or confirm UE’s request, and the details of the response or confirmation should be studied. When the network resources are limited, the NW can refuse UE’s request. Moreover, if multiple UEs request RS transmissions corresponding to totally different Set B patterns, the RS overhead will be large, and this will go against the original purpose of using AI for beam management. Another issue is the configuration of RS should be aligned with Rx beam assumption of Set B and Set A at the UE side. If the UE assumes using best Rx beam from certain Rx beams (e.g. with a panel) for measuring Set B, the corresponding number of RS resources should be transmitted in a repetition manner. How to align the RS measurement capability of UE (i.e., the maximum number of configurable RS resources for Set B and Set A), and the RS transmission manner to align with Rx beam assumption at the UE side need to be further studied. Besides, the preferred size of training dataset and minimum periodicity of the RS can be indicated from the UE to network. The UE can indicate a preferred dataset size when considering both training complexity and model performance. The minimum periodicity of RS is related to the UE speed. For example, if the minimum RS periodicity for low-speed UEs is too small, the UE may get the same L1-RSRP results from several subsequent measurements.

Proposal 7: Regarding the training data collection for AI/ML model training at UE side, study the potential specification impact on the following aspects:

- RS transmission to align/determine beam pattern of Set A/Set B, including RS transmission to align with Rx beam assumption at UE side

- Indication of the preferred size of training dataset

- Indication of the minimum periodicity of the RS transmission.

    - Model/functionality identification

Model identification or functionality identification is a process/method of identifying an AI/ML model/functionality for the common understanding between the NW and the UE. At least for collaboration Level y, gNB doesn’t know the exact model at UE side, if the model deployed at UE is proprietary. Thus, sufficient information of the AI/ML model shall also be provided to enable LCM within 3GPP network. In RAN1#113 meeting, the following agreement on conditions/additional conditions for AI-BM functionality(ies) and/or model(s) was approved [1].

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) at least from the following aspects: - information regarding model inference - Set A / Set B configuration - performance monitoring - data collection - assistance information

In our view, the conditions/additional conditions shall tell the network how to use and monitor the AI/ML functionality or AI/ML model. The following aspects can be considered for BM-specific conditions/additional conditions.

- Model inputs

  - e.g., the Set B pattern information, the periodicity and measurement time instances of T1 for BM-Case2

  - based on this information, gNB can decide how to transmit the RS for Set B

- Model outputs

  - e.g., the model outputs can be Top-K predicted beams (pairs) or predicted L1-RSRP, the periodicity and prediction time instances of T2 for BM-Case2

  - based on this information, gNB can decide and configure proper beam reporting quantities for UE

- Model performance metric

  - e.g., beam prediction accuracy, or predicted L1-RSRP difference

  - based on this information, gNB can know how to configure RS for performance monitoring

Proposal 8: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) at least from the following aspects:

- Model inputs, e.g., the Set B pattern information, the periodicity and measurement time instances of T1 for BM-Case2

- Model outputs, e.g., the model outputs can be Top-K predicted beams (pairs) or predicted L1-RSRP, the periodicity and prediction time instances of T2 for BM-Case2

- Model performance metric, e.g., beam prediction accuracy, or predicted L1-RSRP difference

Another open issue is how to distinguish different functionalities. For a given sub-use-case, one or more functionalities may be provided by the UE, and UE can use more than one model to realize a functionality. How to distinguish different functionalities is not clear so far. One possible way is to use large granularity characteristics such as input type and/or output type, allowing network have correct understanding on ‘what is required’ and ‘what can be provided’. For example, we have two models, i.e., model 1 and model 2. Model 1 is for beam pair prediction and model 2 is for DL Tx beam prediction on a specific Rx beam. For model 1 inference, UE needs to sweep the Rx beams to measure the L1-RSRP of Set B. But for model 2 inference, the Rx beam sweeping is not needed for Set B measurement. Obviously, it requires different RS configurations at the gNB side, hence model 1 and model 2 cannot be within a functionality. For another example, we have model 3 and model 4, which are both used for Tx beam prediction. The output of model 3 is Top-K beam ID while the output of model 4 is the predicted L1-RSRP. For model 3, UE could provide the Top-K beam ID to gNB, and for model 4, UE could provide the Top-K beam ID and corresponding predicted L1-RSRPs to gNB, which requires different report quantities and resources configuration. Hence, it is not proper to group model 3 and model 4 into one functionality.

Proposal 9: Regarding the functionality identification of BM-Case1 and BM-Case2, study distinguishing different functionalities by large granularity characteristics, e.g. input type and output type.

    - Model inference

In RAN1#113 meeting, the following agreement on beam indication was approved [1].

Agreement For BM-Case1 and BM-Case2, study necessity, benefit(s) and potential specification impact from the following additional aspects for AI model inference: - How to perform beam indication of beams in Set A not in Set B Note: the legacy mechanism may be sufficient

For AI-based beam management, if the predicted Top-K beams are in Set A not in Set B, which means the predicted beams are unmeasured beams, the following two options can be considered for beam indication.

Opt.1: The gNB sweeps over the predicted Top-K beams, UE measures and reports the L1-RSRP of the Top-K beams, and gNB choose a beam from the reported beams for indication.

Opt.2: The gNB determines the best beam from the predicted Top-K beam and choose a measured beam close to the best beam for beam indication.

For Opt.1, after measuring the predicted Top-K beam (pairs), the unmeasured beams are turned into measured beams, then legacy TCI indication mechanism can be used. For Opt.2, gNB can choose a measured beam close to the predicted best beam for beam indication. For Opt.1, sweeping over the predicted Top-K beam (pairs) can cause additional latency and RS transmission overhead, and for Opt.2, it may cause performance degradation since the indicated beam may be sub-optimal.

For BM-Case2, it has been proposed to study how to indicate beams for multiple future time instances. From our perspective, indicating beams for multiple future time instances will limit the beam indication flexibility. For example, gNB indicates the beams for future 320ms, which is a typical prediction time in the evaluation of BM-Case2, then before the indicated beams have been applied, if gNB decides to change the beams, it has to send a new beam indication command, which may need more indication resources than legacy beam indication mechanism.

Proposal 10: For BM-Case1 and BM-Case2, for beam indication of the predicted Top-K beams, the following options can be considered:

- Opt1: Reusing legacy TCI indication mechanism after sweeping over the Top-K predicted beams

- Opt2: Indicating a measured beam close to the predicted best beam

Proposal 11: For BM-Case2, indicating beams for multiple future time instances will limit the beam indication flexibility of the network.

In RAN1#112 [4], the following agreement for UE-side model was approved.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

Based on the agreements, the indication of the associated Set A from network to UE can be further studied. In our view, the indication of the associated Set A is for UE reporting the predicted Top-K beams. In legacy L1-RSRP reporting, the UE shall report the L1-RSRP of the CSI resource setting associated with the CSI reporting. For AI-based BM, the resource setting associated with the CSI reporting shall include the resources for measurement (Set B) and the virtual resources or index for reporting (Set A). The virtual resources for Set A will not be transmitted and measured but be used for reporting the beam prediction result. If Set B is the subset of Set A, the gNB can configure the resources for Set A and indicate the resources for Set B in an explicit or implicit way. For example, gNB can indicate the resources for Set B by a bitmap or defining a rule like the resources for Set B is the first N resources of Set A.

Proposal 12: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding how to indicate the association/mapping of Set A and Set B:

- Study indicating the resources of Set A and Set B associated with CSI reporting

- Study both explicit indication and implicit indication methods.

    - Performance monitoring

The propagation environment in the system may change due to various factors, e.g. moving of UE and new obstacles. Due to the large change of propagation environment, the performance of AI/ML based beam management may deteriorate dramatically. In order to avoid long time performance degradation, performance monitoring of AI/ML model or functionality is needed, and some actions should be taken when the AI/ML model or functionality becomes invalid.

For UE-side performance monitoring, UE needs to calculate the KPIs and make decision of model selection/activation/ deactivation/switching/fallback operation. The RS configuration/beam reporting/beam indication for to be activated AI/ML models and for legacy beam management may be different from that of to be deactivated models, hence UE should report the decision of model activation/ deactivation/switching/fallback to the network, and the decision shall be applied after UE receiving the acknowledgement from the network.

Proposal 13: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for Alt.1 UE-side model monitoring, study the potential specification impacts on the following aspects:

- Reporting the decision of model activation/ deactivation/switching/fallback to the network

- Acknowledgement mechanism of model activation/ deactivation/switching/fallback from the network.

Regarding the performance metric(s) of AI/ML performance monitoring and the benchmark/reference for performance comparison, the following agreements were approved in RAN1#112 [7] and RAN1#112 bis [5].

Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered Agreement For AI/ML performance monitoring for BM-Case1 and BM-Case2, study potential specification impact of at least the following alternatives as the benchmark/reference (if applicable) for performance comparison: - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A) - FFS: gNB configures one or multiple sets for one or multiple benchmarks/references - Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams) - FFS: - Alt.3: The beam corresponding to some or all the indicated/activated TCI state(s) - Other alternative is not precluded.

For the performance metrics, Alt.1 and Alt.4 are the intermediate KPIs of the AI/ML model. The intermediate KPIs are more straightforward than Alt.2 link quality related KPIs, since link quality KPIs may be influenced by other factors. For Alt.3, the specific metrics and the model monitoring performance should be studied in AI 9.2.3.1 to evaluate whether it is valid to take Alt.3 for model monitoring. For performance metric of Alt.1, the beam prediction accuracy includes Top-1 beam prediction accuracy, Top-K/1 beam prediction accuracy, Top-1/K beam prediction accuracy and Top-1 beam prediction accuracy within 1dB margin. If the performance metric is calculated by the UE, the beam prediction accuracy can be calculated based on the measurement results of Set A. If the performance metric is calculated by the network, UE should report the corresponding information to NW. The information needed for different performance metrics is illustrated in Table 1

Table 1 Reporting contents for different performance metrics of Alt.1

| Performance metrics | Reporting contents |
| --- | --- |
| Top-1 beam prediction accuracy | The RS indicator corresponding to the ideal Top-1 beam |
| Top-K/1 beam prediction accuracy, | The RS indicator corresponding to the ideal Top-1 beam |
| Top-1/K beam prediction accuracy | The RS indicators corresponding to the ideal Top-K beams |
| Top-1 beam prediction accuracy within 1dB margin | The RS indicators corresponding to the beams within 1dB margin of ideal Top-1 beam, or one bit indicating whether the predicted Top-1 beam is within 1dB margin of the ideal Top-1 beam |

Observation 1: For BM-Case1 and BM-Case2 with NW-side performance monitoring, different reporting contents from UE are needed if different kinds of beam prediction accuracy (e.g. Top-K/1 beam prediction accuracy, Top-1/K beam prediction accuracy, Top-1 beam prediction accuracy within 1 dB margin) are used as the performance metric.

For performance metric of Alt.4, to obtain the actual L1-RSRP of predicted Top-1/K beams, gNB needs to transmit the corresponding RS of the predicted Top-1/K beams for measurement. Regarding when to transmit the RS, the following two options can be considered.

- Opt.1 : transmit the RS corresponding to beams in Set A before model inference

- Opt.2: transmit the RS corresponding to predicted Top-1/K beams after model inference

For Opt.1, since the gNB does not know the prediction result before model inference, gNB can transmit the RS corresponding to Set A. After model inference, the UE can report the actual L1-RSRP of predicted Top-1/K beams to gNB if the performance metric is calculated by the NW. For Opt.2, the RS is transmitted after model inference, it should be ensured that the beam qualities are almost unchanged between the model inference time and the RS measurement time, otherwise the performance metric will not be calculated accurately.

Proposal 14: For AI/ML performance monitoring for BM-Case1 and BM-Case2, study when to transmit the RS corresponding to the predicted Top-1/K beams if the L1-RSRP difference (Alt.4) is used as performance monitoring metric.

For the benchmark for performance comparison, one potential method is to measure the beams from the Set A and decide whether the AI/ML model or functionality is still valid by comparing the model outputs and best beam(s) of Set A. To get the best beam(s) of Set A, gNB still needs to transmit RS with beams in Set A. Thus, the power consumption and RS overhead are large. To save the power consumption and RS resources, it’s better to study how to calculate the intermediate KPIs only based on Set B, so that gNB doesn’t need to transmit RS in Set A at all, for example, UE measures multiple Set Bs to constitute Set A or nearly constitute Set A, so that UE can get the best beam(s) of Set A based on the measurement of multiple Set Bs.

Besides, the model switch/fallback events should be defined to ensure the robustness. For example, the beam failure event is defined as the number of beam failure instance within a timer is greater than a threshold. A similar event can be defined for model update/switching/fallback to consider both robustness and latency.

Proposal 15: For AI/ML performance monitoring for BM-Case1 and BM-Case2, study potential specification impact of the following alternatives as the benchmark/reference (if applicable) for performance comparison

- The best beam(s) obtained by measuring beams of multiple sets that nearly constitutes set A

- AI/ML solution subject to an inactive model, to make the decision of switching/selection based on the performance comparison with the AI/ML solution being monitored.

Proposal 16: Regarding the performance monitoring for BM-Case1 and BM-Case2, study the specification impacts on the following aspects:

- Model update/switching/fallback procedures based on model monitoring results, including the signaling exchange between the gNB and UE

- Trigger condition for model update/switching/fallback.

- Conclusion

In this contribution, the related spec impacts for AI/ML based beam management are discussed. The observation and proposals are summarized as follows:

Observation 1: For BM-Case1 and BM-Case2 with NW-side performance monitoring, different reporting contents from UE are needed if different kinds of beam prediction accuracy (e.g. Top-K/1 beam prediction accuracy, Top-1/K beam prediction accuracy, Top-1 beam prediction accuracy within 1 dB margin) are used as the performance metric.

Proposal 1: For DL beam pair prediction with a NW-side model, considering the generalization of different UE Rx beam shapes/patterns, study how to report/send relative Rx beam information when preserving sensitive proprietary information and considering the reporting overhead and storage overhead.

Proposal 2: For DL beam pair prediction with a UE-side model, study the following aspects:

- For model training at NW side, study how to report relative Rx beam information when preserving sensitive proprietary information

- For model training at UE side, study how to send/report relative Tx beam information when preserving sensitive proprietary information.

Proposal 3: For DL beam pair prediction with model training at NW side and inference at UE side, study how to align the mapping rule for Tx beam ID and Rx beam ID between the NW and UE.

Proposal 4: For the AI/ML mode input for BM-Case1 and BM-Case2, suggest to support at least following：

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID.

Proposal 5: For the AI/ML mode input for BM-Case1 and BM-Case2, suggest to further study using assistance information as model input when considering the generalization over different antenna/beam patterns.

Proposal 6: Regarding data collection for NW-side model, study the following options for reporting overhead reduction:

- Option 1: Reducing the number of reported beams (beam pairs)

- Option 2: Reducing the number of reported index

- Option 3: Reducing the number of bits for RSRP quantization.

Proposal 7: Regarding the training data collection for AI/ML model training at UE side, study the potential specification impact on the following aspects:

- RS transmission to align/determine beam pattern of Set A/Set B, including RS transmission to align with Rx beam assumption at UE side

- Indication of the preferred size of training dataset

- Indication of the minimum periodicity of the RS transmission.

Proposal 8: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) at least from the following aspects:

- Model inputs, e.g., the Set B pattern information, the periodicity and measurement time instances of T1 for BM-Case2

- Model outputs, e.g., the model outputs can be Top-K predicted beams (pairs) or predicted L1-RSRP, the periodicity and prediction time instances of T2 for BM-Case2

- Model performance metric, e.g., beam prediction accuracy, or predicted L1-RSRP difference

Proposal 9: Regarding the functionality identification of BM-Case1 and BM-Case2, study distinguishing different functionalities by large granularity characteristics, e.g. input type and output type.

Proposal 10: For BM-Case1 and BM-Case2, for beam indication of the predicted Top-K beams, the following options can be considered:

- Opt1: Reusing legacy TCI indication mechanism after sweeping over the Top-K predicted beams

- Opt2: Indicating a measured beam close to the predicted best beam

Proposal 11: For BM-Case2, indicating beams for multiple future time instances will limit the beam indication flexibility of the network.

Proposal 12: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding how to indicate the association/mapping of Set A and Set B:

- Study indicating the resources of Set A and Set B associated with CSI reporting

- Study both explicit indication and implicit indication methods.

Proposal 13: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for Alt.1 UE-side model monitoring, study the potential specification impacts on the following aspects:

- Reporting the decision of model activation/ deactivation/switching/fallback to the network

- Acknowledgement mechanism of model activation/ deactivation/switching/fallback from the network.

Proposal 14: For AI/ML performance monitoring for BM-Case1 and BM-Case2, study when to transmit the RS corresponding to the predicted Top-1/K beams if the L1-RSRP difference (Alt.4) is used as performance monitoring metric.

Proposal 15: For AI/ML performance monitoring for BM-Case1 and BM-Case2, study potential specification impact of the following alternatives as the benchmark/reference (if applicable) for performance comparison

- The best beam(s) obtained by measuring beams of multiple sets that nearly constitutes set A

- AI/ML solution subject to an inactive model, to make the decision of switching/selection based on the performance comparison with the AI/ML solution being monitored.

Proposal 16: Regarding the performance monitoring for BM-Case1 and BM-Case2, study the specification impacts on the following aspects:

- Model update/switching/fallback procedures based on model monitoring results, including the signaling exchange between the gNB and UE

- Trigger condition for model update/switching/fallback.

- References

- Chair’s Notes RAN1#113, Incheon, Korea, May 22nd - 26th, 2023.

- R1-2303970, Summary#5 for other aspects on AI/ML for beam management, RAN1#112bis e-meeting, April 17th – April 26th, 2023.

- Chair’s Notes RAN1#109-e, e-Meeting, May 9th – 20th, 2022.

- Chair’s Notes RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2307078, Evaluation and discussion on AI/ML for beam management, CATT, RAN1#114 meeting, August 21st – August 25th, 2023.

- Chair’s Notes RAN1_112bis-e, e-Meeting, April 17th - 26th, 2023.