3GPP TSG RAN WG1 #114			R1-2307312

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.16.5

Source:	Apple Inc.

Title:	Discussion on UE Features for Rel-18 NR NTN Enhancements

Document for:	Discussion/Decision

# Introduction

The Release-18 NR NTN enhancement work item was approved in RAN #94e meeting [1] and updated in RAN #100 meeting [1]. RAN1-related objectives are shown below.

Coverage enhancement:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To specify, if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

Network verified UE location:

- Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied.

In RAN1 #113 meeting [3], the UE features on NR NTN enhancements have been discussed, and some high-level agreements have been made:

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-1 | PUCCH repetition for Msg4 HARQ-ACK | 1. Support repetition transmission of PUCCH for Msg4 HARQ-ACK on common PUCCH resource 2. Support receiving repetition factor in system information [3. Support receiving repetition factor in DCI scheduling Msg4 PDSCH] [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition] [5. Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration] |  | Yes | No | UE does not support PUCCH repetition for Msg4 HARQ-ACK | Per Band | N/A | N/A | N/A | A UE that includes [RAN2 parameter name] in [RRC Setup Request] must support FG 44-1 [Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-2 | NTN DMRS bundling enhancement for PUSCH | 1. Support of DM-RS bundling for PUSCH over consecutive slots 2. Support of pre-compensation to keep phase rotation due to timing drift within the phase difference limit [3. Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.] | 30-4a/b | Yes | No | UE does not support DM-RS bundling enhancement for PUSCH in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-3 | UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN | 1. Support UE Rx-Tx Measurement and Report for Multi-RTT positioning with single satellite in NTN [2. Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN] | 13-4, 13-8 | Yes | No | UE does not support Multi-RTT positioning with single satellite in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In this contribution, we discuss the UE features on NR NTN enhancements and give our proposals.

# Discussion on NR NTN UE features

For NR NTN coverage enhancement objective, there are two tasks: PUCCH enhancements for Msg 4 HARQ-ACK and enhancements for DMRS bundling for PUSCH taking into account NTN-specifics.

For PUCCH enhancements for Msg4 HARQ-ACK, the main scheme designed in RAN1 is PUCCH repetition. This includes UE sending repetition request or capability report to gNB to indicate either it has PUCCH repetition capability for Msg4 HARQ-ACK or it additionally requests PUCCH repetition for Msg4 HARQ-ACK based on its own downlink RSRP measurements. This repetition request or capability report is via Msg3 PUSCH higher layer signaling based on working assumption. If more than one repetition factor is configured by network, network will dynamically indicate the number of repetitions for Msg4 HARQ-ACK to a UE via DCI scheduling Msg4 PDSCH.

Overall, the new UE feature group (i.e., FG 44-1) of PUCCH repetition for Msg4 HARQ-ACK should include two more components: 1). support sending Msg 3 to indicate repetition request or capability report for PUCCH repetition for Msg4 HARQ-ACK; 2). support receiving repetition factor in DCI scheduling Msg4 PDSCH.

Regarding another potential new component “Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration”, we think the discussions should be held, depending on the conclusion to be made in RAN#101 meeting. This is because according to the agreement made in RAN #100 [4], it is open whether to support repetitions for PUCCH transmission when dedicated PUCCH resource configuration is not provided.

Proposal 1: In FG 44-1, introduce the following components:

- Support sending Msg3 to indicate repetition request or capability report for PUCCH repetition for Msg4 HARQ-ACK.

- Support receiving repetition factor in DCI scheduling Msg4 PDSCH.

For enhancements for DMRS bundling for PUSCH taking into account NTN-specifics, it was working assumption [3] that for NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS 38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information. Furthermore, a list of options for potential UE capability report was identified, including

- Option 1a: No new capability except for FG 44-2.

- Option 1b: Maximum TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account.

- Option 1c: Support of antenna switching with DMRS bundling in NTN

- Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account

- Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account

- Option 1f: Whether to support actual TDW across pre-compensation segments

- Option 1g: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit

Since the discussions on additional UE capability report (besides FG 44-2) are still on-going in RAN1, we think it should be kept open on whether or not to introduce additional UE capability for NTN-specific PUSCH DMRS bundling.

Proposal 2: Depending on the outcome of RAN1 discussions, determine whether or not to introduce additional UE capability (besides FG 44-2) for NTN-specific PUSCH DMRS bundling.

For the objective of network verified UE location, the existing RAN1 discussions focus on multi-RTT positioning method. There are no discussions on DL-TDoA positioning method. A new UE feature group of supporting network verified UE location based on multi-RTT with single satellite in NTN was introduced as FG 44-3.

In FG 44-3, it is open whether it contains a component “Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN”. At this stage, we do not see the need of UE reporting assistance information. Hence, we propose not to include this component for FG 44-3.

Proposal 3: In FG 44-3, do not include the following component:

- Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN.

# Summary

In this contribution, we discuss the Rel-18 NR NTN enhancement UE capability, and have the following proposals:

Proposal 1: In FG 44-1, introduce the following components:

- Support sending Msg3 to indicate repetition request or capability report for PUCCH repetition for Msg4 HARQ-ACK.

- Support receiving repetition factor in DCI scheduling Msg4 PDSCH.

Proposal 2: Depending on the outcome of RAN1 discussions, determine whether or not to introduce additional UE capability (besides FG 44-2) for NTN-specific PUSCH DMRS bundling.

Proposal 3: In FG 44-3, do not include the following component:

- Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN.

# References

- RP-213690, “New WI: NR NTN enhancements,” Dec. 2021.

- RP-231484, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements,” Jun. 2023.

- Chairman’s Notes, 3GPP TSG RAN WG1 #113 Meeting, May 2023.

- Chairman’s Notes, 3GPP TSG RAN #100 Meeting, Jun. 2023.