3GPP TSG RAN WG1 #114		                                           R1-2307557

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.11.3

Source: 	OPPO

Title:	Design consideration on lower power wake-up signal and procedure

Document for:	Discussion & Decision

Introduction

In this contribution, we discuss the aspect of LP-WUS content, signal modulation methods, different RRC modes, measurement, procedure, and coverage with potential extension related to study of the WUS solutions.

The discussion is based on the agreements of previous meetings cited in the end.

Waveform Parameters

In earlier discussion, different options of waveforms are proposed to be evaluated. The main options are OOK-1 to 4, FSK-1/2, OFDM. For perspective of the low power envelop detection in receiver side, time and frequency parameters should be given.

For evaluation we also have agreement in post meeting#112 email discussion.

Agreement Configuration for LP-WUS signal; For OOK/FSK waveform, # of segments in one OFDM symbol (M) - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report. LP-WUS BW; Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4,8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS.

Based on the agreed wave form options, with rang of bit rate, we can then decide the further details of parameter. We basically consider bit rates from existing WUS design. In WiFi, the range of WUS information bit rates are from HDR 250k to LDR 62.5k bps. Thus, 56k bps and 112k bps would be considered. Those will result in M = {4, 8} @ 15kHz SCS for both OOK and FSK. M=16 can be also considered if higher data rate required. For 30kHz, the M = can be considered as {2,4,8}. In frequency, we can consider 8~24 PRB @ 15kHz SCS or 4~12 PRB @ 30kHz SCS for wake-up signal transmission bandwidth.

1RB or 2RBs of guard band for each side can be reserved to protect frequency/timing error, especially for reference receiver architectures for OOKs. Also, this help for multiplexing with NR OFDM signal. The guard band can be included in the transmission bandwidths.

Both OOK and FSK is considered to be compatible with NR OFDM signal. The CPs in time domain can be inserted same as NR, to help multiplexing with other NR signals.

Figure 1. Multi-bits OOK/FSK with NR signal

Proposal 1: Wake-up signals consider OOK or FSK as basic detection wave form. NR OFDM symbol duration without CP symbol is M times of WUS symbol duration for multi-bit schemes. The candidate value could be start from M = {2,4,8}. Transmission bandwidth can be 6~24 PRB @ 15kHz SCS or 4~12 PRB @ 30kHz SCS.

Compatible NR CP is also inserted.

Guard band for WUS of 2 RBs for 15kHz SCS and 1RB for 30kHz SCS are introduced in 2 sides of WUS signal.

WUS signals

Additional periodic LP-SS is considered to be introduced to facilitate LP-WUS processing. The LP-SS periodicity is observed to be necessary for that purpose. The LP-SS should be design with the same waveform as for LP-WUS carrying wake-up information. It could be pseudorandom sequences modulated in OOK/FSK.

In another option, the legacy PSS/SSS can also be used for synchronization. The legacy resource of SSB will carry the PSS/SSS. However, some low-power receiver can measure the PSS/SSS with some degraded accuracy and sensitivity, comparing with the normal NR receiver.

We consider few functionality of those periodic assisting signal. The timing/frequency inaccuracy of WUS receiver is due to the limited internal clock/component. Those synchronization signals will help for LP-WUS demodulation. The time drifting can also be corrected by that periodic signals.

The synchronization signals can also assist AGC of WUR before the payload detection. For OOK, the amplitude training would be important. Thus, synchronization signal before the wake-up information can serve the purpose.

The WUR may also measure LP-SS or PSS/SSS to facilitate main radio.

For AGC and other training of following LP-WUS, PSS/SSS may not be used. It has different frequency/time resource than that of LP-WUS. The waveform and power may also be different. Then, LP-SS should be introduced regardless the availability of PSS/SSS. In that case, the PSS/SSS may also be additional used for better measurement as this is existing signal.

Proposal 2: Introducing periodically transmitted LP-SS to assist the WUS time/frequency tracking, amplitude adjustment and demodulation.

Pseudorandom sequences modulated in the LP-SS with same waveform as LP-WUS.

Study whether WUS synchronization signal measurement can also facilitate the main radio wake-up and supporting the mobility of UE.

As example of time domain structure is proposed for the study. Synchronization signal(LP-SS) can be 4 symbols before the WUS information payload. As discussed before, this part can be used for synchronization, training and even measurement. The modulation scheme should be same as for payload part. However, the synchronization part should be sequence based. In addition, the cell group ID could be included in that part for measurement. It will facilitate the wake-up procedure when main-radio is activated. Cell related information can also allow receiver maintain mobility.

Figure 2. Time domain WUS structure

The WUS payload bits should at least include UE group ID information. The low-power receiver may not support complex FEC Coding scheme, considering hardware and power consumption. The 8 symbols per subframe only contains 64 bits if M=8 The bit size may not have high coding gain. Simple coding should be considered. For evaluation, Manchester code can be used. Still, CRC can be included in the payload.

Proposal 3: Wake-up signals compose of LP-SS part and LP-WUS part.

UE group ID information is in LP-WUS payload part with Manchester code and CRC.

Cell related information could be carried by LP-SS.

Coverage and Measurement

LP-WUS signal coverage and procedure

The coverage of WUS would be much smaller, as one of the costs of very low power consumption. Coverage of LP-WUS is evaluated to show the potential coverage gap to the NR channels. Ideal synchronization is assumed in the evaluation done among the candidate waveforms. In the figure 3, link level performance is shown for OOK1, OOK2, OOK4 and FSK1. The simulation assumption can be found in the Appendix.

Figure 3. Link level simulation results for different waveforms

We observed that SNR = 2.6dB @ BLER=1% for OOK1, SNR = 13.35dB @ BLER=1% for OOK2, SNR = 4.89dB @ BLER=1% for OOK4, and SNR = 6.98dB @ BLER=1% for FSK1.

In the simulated cases, the OOK1 can achieve best performance at the cost of low data rate. The OOK4 is worse than OOK1. However, the payload/data rate can be much higher. Further, we provide the coverage comparison between LP-WUS and legacy NR signals. The results are based on updated MIL template and summarized as following.

Table 1: Coverage performance for LP-WUS and legacy NR signals

|  | PDCCH AL8 | PUSCH | OOK1 | OOK2 | OOK4 | FSK1 |
| --- | --- | --- | --- | --- | --- | --- |
| Urban (2.6GHz) | 163.54 dB | 135.88 dB | 143.78 | 133.03 | 141.49 | 139.40 |

Still, coverage gap cannot be totally removed. It is considered that a wake-up signal should be associate with a cell ID. In that one to one mapping, coverage gap will result in WUS outage area in the cell. Thus, the gap should be reduced as much as possible.

Target of coverage enhancement could be PDCCH channel, which represent the best coverage of a cell. In the above evaluation, around 20dB enhancement is to be reached if the LP-WUS is using OOK4.

Proposal 4:  The coverage target of LP-WUS is up to PDCCH with aggregation level 8.

Figure 4. Coverage mismatch of normal cell signal and very low power WUS

Regarding the solution of the coverage enhancement, repetition would be the baseline scheme to meet that target. In case of 8 symbols WUS transmission, the duration of signal would be less than 0.5ms for 30kHz SCS.  Repetition of 10 times will results in 5ms duration, which is acceptable to the overall latency.

Proposal 5: LP-WUS support repetition in time to enhance the coverage in cell.

Even under the coverage enhancement, LP-WUS will not have exactly the same coverage as that for normal NR. The activation and deactivation of LP-WUS monitoring is under study in Idle/Inactive mode to deal with that mismatch. One of the alternative is to perform activation and deactivation of LP-WUS monitoring in a cell is based on signaling. However, it is unclear that how the signaling will be done. The signaling can not reach UE for deactivating through LP-WUS, if it is in the coverage outage. Instead, measurements based on LP-SS/WUS and NR signal should be used for criteria of UE activation and/or deactivation of LP-WUS monitoring. The criteria can be pre-configured by gNB signaling.

For UE in RRC CONNECTED mode, similar criteria should be introduced as the UE may not be able to demodulate LP-WUS, while camping in the cell.

Proposal 6:

Measurements based on LP-SS/WUS and NR signal should be used for criteria of UE activation and/or deactivation of LP-WUS monitoring. The criteria can be pre-configured by gNB signaling.

UE activation and/or deactivation of LP-WUS monitoring for UE in RRC CONNECTED mode is also based on a pre-configured criteria.

WUS measurement and procedure

In WUS measurement, we consider the measurement should be firstly based on LP-SS. The UE may measure LP-SS to facility not only for WUS but also for MR synchronization and acquire channel state information. Additionally, RRM relaxation can also be based on the LR measurement in UE’s RRC Idle/Inactive mode. This RRM relaxation would be the key issue for LP-WUS power saving as evaluated in [3].

For the mobility, the WUS signal should also support or facility the function. Even the use case is in low mobility, it cannot be avoided for handing over between cells. UE in low power sleeping state should also aware the camping cell, at least in less frequent way.

We consider the RRM measurement procedure under different conditions. When LP-SS based measurements by LP-WUR is above a pre-configured threshold, relaxed periodicity serving cell RRM measurement in MR is performed. When LP-SS based measurements by LP-WUR is below a pre-configured threshold, normal serving cell RRM measurement in MR is performed.

MR perform neighboring cell RRM measurement, only when LP-SS based measurements by LP-WUR is below a pre-configured threshold.

Proposal 7:

When LP-SS based measurements by LP-WUR is above a pre-configured threshold, relaxed periodicity serving cell RRM measurement in MR is performed. When LP-SS based measurements by LP-WUR is below a pre-configured threshold, normal serving cell RRM measurement in MR is performed.

MR perform neighboring cell RRM measurement, only when LP-SS based measurements by LP-WUR is below a pre-configured threshold.

Conclusion

In this contribution, we had discussed and analyzed lower power WUS design for NR.

Regarding the LP-WUS signals, we have following proposals:

Proposal 1: Wake-up signals consider OOK or FSK as basic detection wave form. NR OFDM symbol duration without CP symbol is M times of WUS symbol duration for multi-bit schemes. The candidate value could be start from M = {2,4,8}. Transmission bandwidth can be 6~24 PRB @ 15kHz SCS or 4~12 PRB @ 30kHz SCS.

Compatible NR CP is also inserted.

Guard band for WUS of 2 RBs for 15kHz SCS and 1RB for 30kHz SCS are introduced in 2 sides of WUS signal.

Proposal 2: Introducing periodically transmitted LP-SS to assist the WUS time/frequency tracking, amplitude adjustment and demodulation.

Pseudorandom sequences modulated in the LP-SS with same waveform as LP-WUS.

Study whether WUS synchronization signal measurement can also facilitate the main radio wake-up and supporting the mobility of UE.

Proposal 3: Wake-up signals compose of LP-SS part and LP-WUS part.

UE group ID information is in LP-WUS payload part with Manchester code and CRC.

Cell related information could be carried by LP-SS.

Proposal 4:  The coverage target of LP-WUS is up to PDCCH with aggregation level 8.

Proposal 5: LP-WUS support repetition in time to enhance the coverage in cell.

Proposal 6:

Measurements based on LP-SS/WUS and NR signal should be used for criteria of UE activation and/or deactivation of LP-WUS monitoring. The criteria can be pre-configured by gNB signaling.

UE activation and/or deactivation of LP-WUS monitoring for UE in RRC CONNECTED mode is also based on a pre-configured criteria.

Proposal 7:

When LP-SS based measurements by LP-WUR is above a pre-configured threshold, relaxed periodicity serving cell RRM measurement in MR is performed. When LP-SS based measurements by LP-WUR is below a pre-configured threshold, normal serving cell RRM measurement in MR is performed.

MR perform neighboring cell RRM measurement, only when LP-SS based measurements by LP-WUR is below a pre-configured threshold.

Reference

[1] RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

[2] R1-2305965, Design consideration on lower power wake-up signal and procedure, OPPO

[3] R1-2305450, Evaluation for lower power wake-up signal, OPPO

Appendix

Table A. Simulation assumptions for candidate waveforms

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| Waveform | OOK-1 / OOK-2 / OOK-4 / FSK-1 |
| Channel structure | Option 3: Payload (48bits) + CRC (16bits) |
| SCS of OFDM generator for NR signal | 30kHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, o Option 1b: M =1 for SCS = 30 kHz (same as NR signal) |
|  | o Option 2b: M =8 for SCS = 30 kHz (same as NR signal) |
|  | o Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 |
|  | Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: 8/64 symbols |
| MDR/FAR assumption |  The miss-detection rate (MDR) of LP-WUS 1%, |
|  |  The false-alarm rate (FAR) of LP-WUS |
|  | o 0.1% |
|  |  Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold |
|  | o Companies to report the selected reference time duration values and the associated number of WUS attempts/trials |
| Code scheme | Manchester code, code rate (1/2) |
| gNB Channel BW | 20MHz |
| LP-WUS BW | Option 1: |
|  | - 5MHz including subcarriers for guard band |
|  | - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS |
|  | GB is symmetrically placed on each side of LP-WUS |
| Filter | 5-th Order filter (e.g. Butterworth, Chebyshev, …) with 4.32 MHz bandwidth |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; |
|  | EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB |
| Sampling Rate | - 3.84MHz |
| ADC bit width | 1,8-bit |
| Channel Model | TDL-C, Delay Spread = 300 ns, UE velocity = 3 km/h |

Agreements

RAN1 #113 L1 signal design and procedure for low power WUS

Agreement

- For at least RRM serving cell measurement performed by LP-WUR based on reference signal(s), RAN1 identified at least the following metrics for further study and evaluation (including feasibility, complexity, power consumption, etc)

  - LP-RSSI or Energy detection: linear average of total received power over a RSSI resource.

    - FFS RSSI resource.

  - LP-RSRP: linear average of received power of resource of reference signal(s) or signal(s) parts.

    - FFS resource of reference signal(s) or signal(s) parts

  - LP-SINR = LP-RSRP/(power of interference and noise)

    - FFS how to define “power of interference and noise”

  - LP-RSRQ= [N x] LP-RSRP/LP-RSSI, where N is the factor of resource size difference for evaluation LP-RSRP and LP-RSSI.

  - Accounting AGC accuracy, ADC of at least 4 bits is required.

- Note: Reference signal for performing measurements can be e.g. SSB (PSS/SSS/PBCH DMRS), LP-WUS-waveform sequence, LP-SS

- Note: The definition of metrics could be further refined based on future study

Agreement

Power pooling between OFDM symbols is not assumed for evaluation purposes. Average EPRE is defined per OFDM symbol.

Agreement

The following observations are to be captured in the TR

- At least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionality.

  - RRM measurements by LP-WUR, if supported

  - at least coarse time synchronization of LP-WUR.

  - at least coarse frequency synchronization of LP-WUR.

- Additional periodic LP-SS system overhead depends on LP-SS periodicity, system BW, # of beams, and resource required to fulfil the target functionality, etc. Periodic signal if used for coarse synchronization may reduce overhead of signal preceding LP-WUS, if any. LP-SS can be designed to be common among UE groups (cell-specific) and such further reduce system overhead.

- For LP-WUR that can receive existing PSS/SSS potentially assisted by PBCH DMRS/TRS for synchronization, existing PSS/SSS potentially assisted by PBCH DMRS/TRS may be used for above functionality.

- Periodic LP-SS coverage should be equal or better than that of LP-WUS.

- For fine time and frequency synchronization, a signal (e.g. preamble) preceding or part of LP-WUS may be used.

Agreement

- For Idle/Inactive mode, study offloading of RRM measurements of serving cell to LP-WUR under certain conditions, if any, and relaxation of serving/neighboring cell RRM measurements in MR considering

- Periodic reference signal(s) is/are used for LR measurements.

- FFS: reference signal(s) to measure, e.g. PSS/SSS/PBCH DMRS, LP-WUS waveform sequence, LP-SS

- FFS: periodicity, content

- MR performs measurements

- Alt2: with relaxed periodicity if RRM measurement in MR is relaxed.

    - FFS: Condition for relaxation if any

    - Can apply for both neighboring and serving cell

- Alt3: only when reference signal(s) based measurements by LP-WUR satisfy certain condition(s), e.g. are below threshold.

    - FFS threshold.

    - Above MR measurement under certain conditions can apply for both neighboring and serving cell

    - Potentially with relaxation methods for MR neighboring cell measurement

- Other alternatives are not precluded

- FFS: Feasibility of RRM measurements of neighbor cells by LP-WUR

Agreement

- For waveform generation the following observations are made

  - Flat spectrum in frequency domain provides robustness against frequency selective fading compared to concentrated energy in frequency domain.

    - for OOK-4, sequence before DFT/LS with variation in phase via such as ZC, M-sequence or QAM sequence can achieve more flattened spectrum.

  - Sequences(s) used in LP-WUS symbol generation with different pulse shape or spectral shape may have different performance.

  - Knowledge of sequence(s) used in LP-WUS waveform generation may improve performance for at least a receiver with I/Q branches

- Further discuss the following potential observations for waveform generation:

  - When DFT is employed in OOK-4 (M>=2), -1/1 alternation in time or frequency shift in frequency domain may be needed to match CP-OFDM generation.

  - Pre-storing of the generated frequency domain samples at gNB may reduce complexity of waveform generation at gNB with memory requirement depending on number of possible combination. This may be up to gNB implementation.

  - quantization of generated waveform in frequency domain to existing constellation (e.g. 64QAM) has low impact on performance and reduces complexity. This may be up to gNB implementation.

  - Repetition of a sequence(s) used in LP-WUS generation in frequency can be used to improve diversity for MC-OOK and robustness against frequency offsets for MC-FSK.

For companies to consider for providing evaluation results

- Cross-waveform-comparison

  - OOK-1 M=1 and OOK-4 M=1 (may not need to be simulated, difference can be only in frequency domain sequence used)

  - OOK-1 with M x higher SCS than NR, and OOK-4 M

    - M=2,4

  - OOK-4 M=2 and OOK-2 M=2

  - OOK-3 M=1 and OOK-1 M=1

  - OOK-1 and OOK-2 M=2 with further reduced coderate/increased sequence length

  - OOK-1 and OOK-4 M=2 with further reduced coderate/increased sequence length

  - FSK1/2 M=1 (1bit per OFDMA symbol) and OOK-1 M=2

  - FSK1/2 M=2 (2bits per OFDMA symbol) and OOK-2 M=4

  - FSK1/2 M=2 (2bits per OFDMA symbol) and OOK-4 M=4

  - OFDMA and other waveforms with roughly matching T-F resources

  - Note: Above cases should result in same length of LP-WUS in OFDMA symbols and BW for both compared waveforms

- Manchester coding 1/2 is applied to OOK for at least encoded bits (payload).

- At least time and frequency impairments should be included.

  - residual time offset 0, 1, 2 and 4 us

  - residual frequency offset 0, 1, 2, 5 and 10

    - optional 50, 100 ppm

  - showing tolerance higher than above values is not precluded

- If further improvement of the signal generation for the agreed waveforms is applied, companies are to provide relevant details

- For evaluation of LP-SS accuracy, assume SNR at [-3dB] and LP-WUR noise figure should be reported

Agreement

- Study the following techniques/mechanisms to enhance coverage performance of LP-WUS

  - low complex channel coding

    - FEC

    - spreading code in time domain

    - time domain repetition

    - with combining before or after ED

    - time-domain interleaving

    - Note: Also Manchester coding can be considered as channel code

  - non-contiguous transmission in the frequency domain

  - frequency domain repetition

  - frequency-hopping

  - power-boosting

  - transmit diversity

  - study whether any above techniques could be transparent to UE.

Agreement

- For Idle/Inactive mode, following options for activation and deactivation of LP-WUS monitoring by LP-WUR for a UE can be considered for study

- Alt 1a:

  - gNB transmits legacy paging indication and LP-WUS

  - UE activation and/or deactivation of LP-WUS WUS monitoring is up to UE implementation.

  - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient.

- Alt 1b:

  - gNB transmits legacy paging indication and LP-WUS

  - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria

  - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient.

- Alt 2:

  - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling.

- Paging misdetection performance shall not be impacted.

Agreement

- In RRC CONNECTED mode, study benefit of LP-WUS over existing Rel-15, R16, and R17 power saving techniques for following functionalities:

  - LP-WUS with similar functionality as R16 DCP.

  - LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received.

    - interaction with legacy power saving techniques, if any

  - other functionalities are not precluded

  - for evaluation

    - companies to report

      - assumption on MR sleep state when LP-WUR is monitoring LP-WUS

        - deep sleep,

        - light sleep,

        - micro sleep

      - how to activate/deactivate LP-WUS monitoring and deactivate/activate PDCCH monitoring

      - LP-WUS waveform

- In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of

  - by gNB RRC signaling, with or without UE assistance.

  - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance.

  - based on pre-configured condition(s), such as timer.

  - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB.

other options are not precluded.