3GPP TSG RAN WG1 #114			R1-2307593

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	9.9.1

Source: 	Hyundai Motor Company

Title:	Discussion on coverage enhancement for NR NTN

Document for:	Discussion and Decision

Introduction

In the Rel-18 WID for NR NTN coverage enhancements, an objective to study and specify support of coverage enhancement for NR NTN is provided in [1].

In the recent meeting (RAN1#113), the following working assumptions and agreements were made [2]:

Working assumption For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information (if needed). - i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined based on UE capability(ies) signaling. - FFS: which UE capability(ies) signaling is(are) used - FFS: whether/how to use UE assistance information, if supported Agreement For NTN-specific PUSCH DMRS bundling, one or more of the following is down-selected for actual TDW determination. - Actual TDW is determined by the existing events and, - Alt A: No additional event - i.e., no spec impact is assumed for actual TDW determination. - Alt B: New event of TA pre-compensation timing dynamically indicated by gNB - i.e., TA pre-compensation timing can be dynamically indicated by gNB - Note: UE can perform TA pre-compensation update at the indicated timing - FFS: detailed indication - Alt C: as dynamically indicated by gNB - i.e., actual TDW can be dynamically indicated by gNB - FFS: detailed indication - Alt D: New event based on epoch time - FFS details - Alt E: New event based on antenna switching Agreement For NTN-specific PUSCH DMRS bundling, - As UE capability report (in addition to FG 44-2), one or more of the following is down-selected. - Option 1a: No new capability except for FG44-2 - Note: FG 30-4 is reported [in consideration of pre-compensation to keep phase rotation due to timing drift within the phase difference limit and without taking TA pre-compensation update into account] - Option 1b: Max TDW size when pre-compensation to keep phase rotation due to timing drift within the phase difference limit is performed and without taking TA pre-compensation update into account - Note: FG 30-4 is not reported for NTN band - Option 1c: Support of antenna switching with DMRS bundling in NTN - Option 1d: Max TDW size per NTN platform (e.g., LEO, MEO, GEO) with taking TA pre-compensation update into account - FFS details - Option 1e: Max TDW size per elevation angle with taking TA pre-compensation update into account - Option 1f: Whether to support actual TDW across pre-compensation segments - Segments defined in R17 IoT-NTN is baseline, FFS details - Option 1g: Whether to support TA pre-compensation update within an actual TDW that does not violate the phase difference limit - As UE assistance information (i.e., report by signaling other than UE capability report (FFS details)), one or more of the following is down-selected. - Option 2a: No assistance information - Option 2b: Max TDW size based on reporting timing - FFS which timing is referred - Option 2c: TA adjustment timing - Option 2d: Antenna switching interval

Discussion

PUSCH DMRS bundling for NTN

In RAN1#112bis-e, it has been agreed that only gNB-centric TDW determination rather than UE-centric TDW determination is supported for NR NTN DMRS bundling. Many companies have suggested several NTN-specific events that would determine actual TDW such as TA pre-compensation, antenna switching and so on. Especially, TA pre-compensation should be considered one of NTN-specific events due to the high rate change of distance between UE and its serving satellite with low elevation angle. Thus, we propose to introduce a new indication for TA pre-compensation timing and discuss what kind of information for the indication is appropriate.

Proposal #1

- Introduce a new indication for TA pre-compensation timing that would be a new event to determine TDW.

Proposal #2

- Discuss what kind of information is appropriate to indicate TA pre-compensation timing.

Based on release 17 DMRS bundling, if pusch_WindowRestart is enabled, the number of actual TDW in a nominal TDW (= #aTDW) depends on the number of semi-static/dynamic events (=#events) in the nominal TDW, i.e. #aTDW = #event+1. If TA pre-compensation is agreed to be one of new events in release 18 and a new indication for the TA pre-compensation timing is introduced, the number of the new events by TA pre-compensation within a nominal TDW is a component that determines actual TDW and the number of actual TDWs in a nominal TDW.

In common sense, we can expect that minimizing the number of actual TDWs (or events including TA pre-compensation) in a nominal TDW would show a high joint channel estimation gain. Thus, we propose UE to perform TA pre-compensation whenever every event (or a part of events) occur(s). Figure 1 shows an example, where the maximum tolerable duration without TA pre-compensation is 13 slots and the nominal TDW is 20 slots. If another event occurs after 10th slot, we can expect that gNB indicate that UE performs TA pre-compensation after 13th slot, and three actual TDWs are generated by the event and TA pre-compensation as shown in Case 1. If we assume that UE performs TA pre-compensation at the timing of the event (Rel-17 event in Figure1) regardless of TA pre-compensation indication by gNB, UE does not have to perform another TA pre-compensation within the nominal TDW shown as Case2 and we can reduce the number of actual TDWs in the nominal TDW.

Figure 1. An example to show that the number of actual TDW can be reduced by controlling TA pre-compensation timing

Proposal #3

- Discuss whether UE performs TA pre-compensation not only at the events that gNB indicates for TA pre-compensation but also at the other events.

Conclusion

In this contribution, the following conclusions were made:

Proposal #1

- Introduce a new indication for TA pre-compensation timing that would be a new event to determine TDW.

Proposal #2

- Discuss what kind of information is appropriate to indicate TA pre-compensation timing.

Proposal #3

- Discuss whether UE performs TA pre-compensation not only at the events that gNB indicates for TA pre-compensation but also at the other events.

Reference

[1] RP-222654, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements,” Thales

[2] “Chair’s Notes RAN1#113,”