3GPP TSG RAN WG1 #114			R1-2308012

Toulouse, France, August 21st – August 25th, 2023

Source:	CAICT

Title:	Discussion on TA management to reduce latency

Agenda Item:	9.10.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #113 meeting several agreements of Timing advance management to reduce latency were accepted during the meeting [1]:

On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS

- UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved

- whether/how to reset the counter

Note:

From RAN1 perspective, when reception of RAR is configured, there may have 4 alternatives to determine the random access response window

- Alt1: Postpone the starting point of the random access response window

- Alt2: Extend the length of the random access response window

- Alt3: Length and offset of the starting point of RAR window can be configured by RRC

- Alt4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed

[Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.]

Note:

In addition to TA, when reception of RAR is configured, RAN1 discussed the following alternatives:

- Alt 1: when there is only one ongoing RACH procedure at each time, the identification of candidate cell is not needed

- Alt 2: when more than one RACH procedures are allowed at each time, the identification of candidate cell is contained in RAR

In this contribution, we further discuss more details of the PDCCH-order based PRACH including RACH based TA acquisition and TA updating mechanism.

- PRACH transmission power with no RAR reception

As has been agreed, on the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, if retransmission is indicated, UE will increase the power with the value of power ramping counter increased until the max allowed power is achieved, and the power ramping counter needs to be reset or suspended as in legacy specification [2].

Proposal 1: On the determination of the PRACH transmission power when reception of RAR is not configured, UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved, the power ramping counter needs to be reset or suspended as in legacy specification.

- RACH with RAR reception

Considering RACH with RAR reception, the procedure is that PDCCH order of source gNB triggers UE to send msg1 to one of candidate gNB, the candidate gNB will send TA value and maybe cell ID to source cell after reception of msg1, and UE will monitor the RAR of candidate cell in serving cell to get the TA value of candidate cell.

When reception of RAR is configured, the first issue to be discussed is that whether MAC PDU(like RAR MAC PDU in 4 step RACH) or MAC CE is used to carry TA of candidate gNB.

If the RAR is carried by MAC PDU, there are some issues to be solved. The first one is about the RA response window, because the NW may need longer time to send the RAR response to the msg1 transmission when candidate cell and source cell are in different DUs. To solve this problem the random access response window for candidate cell(s) needs to be separately configured from the normal RAR window, the length and offset of the starting point of RAR window can be configured by RRC.

Proposal 2: If the RAR is carried by MAC PDU, the random access response window for candidate cell(s) needs to be separately configured from the normal RAR window, the length and offset of the starting point of RAR window can be configured by RRC.

Because the RAR response is to be monitored by UE with a DCI scrambled with RA-RNTI over the Type-1 common search space of source cell, the second issue is serving cell may not know the RA resource of UE transmitting to candidate cell, which is necessary to get RA-RNTI value, and it can be provided by the candidate cell along with the TA value to the serving cell.

Proposal 3: If the RAR is carried by MAC PDU, the random access resource information of candidate cell can be provided by the candidate cell to the serving cell.

The third issue is the collision between RAR reception of different candidate cells, when more than one RACH procedures are allowed at each time, for example, one UE at the source cell has triggered a PRACH transmission to one candidate cell, and the other UE at the same source cell has also triggered a PRACH transmission to other candidate cells or to the source cell with the same RA-RNTI. To  avoid this type of problem, the identification of candidate cell can be contained in RAR.

Proposal 4: If the RAR is carried by MAC PDU, the candidate cell ID can be provided by the candidate cell to the serving cell.

If the RAR is carried by MAC CE, the RAR response is to be monitored with a DCI scrambled with C-RNTI over a UE specific search space. In such cases, RAR window, RA-RNTI and candidate cell ID are not needed, but a new UE procedure needs to be specified.

Proposal 5: If the RAR is carried by MAC CE, a new UE procedure needs to be specified.

- TA updating Mechanism

An agreement has been made that the serving cell can trigger the UE to update the TA for a candidate cell by updating the TA using RACH procedure. There are two options of TA updating mechanism, the first option is UE based updating, which means the UE may be configured to maintain the timer for TA validity after sending preamble or receiving the RAR message, upon expiration of the timer, the UE may inform the source cell to trigger TA update. The second option is network based updating, which means the candidate cell may share the time duration of the TA validity timer with the serving cell, and the  serving cell may maintain the timer for TA validity.

For this issue, at least for PDCCH ordered based TA acquisition scheme, as mentioned above, we think the TA indication at serving cell is better than candidate cell, so for the TA updating mechanism, we also think it is better the TA validity timer is maintained in the serving cell. When the Timer expires, the source cell can trigger the PDCCH ordered PRACH transmission to acquire new TA information.

Proposal 6: For TA updating mechanism, the network-based updating is supported.

- Conclusion

In this contribution, we give following proposals on PDCCH ordered RACH for candidate cell(s):

Proposal 1: On the determination of the PRACH transmission power when reception of RAR is not configured, UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved, the power ramping counter need to be reset or suspended as in legacy specification.

Proposal 2: If the RAR is carried by MAC PDU, the random access response window for candidate cell(s) needs to be separately configured from the normal RAR window. And length and offset of the starting point of RAR window can be configured by RRC.

Proposal 3: If the RAR is carried by MAC PDU, the random access resource information of candidate cell can be provided by the candidate cell to the serving cell.

Proposal 4: If the RAR is carried by MAC PDU, the candidate cell ID can be provided by the candidate cell to the serving cell.

Proposal 5: If the RAR is carried by MAC CE, a new UE procedure needs to be specified.

Proposal 6: For TA updating mechanism, the network-based updating is supported.

- References

- Chairman notes of RAN1#113

- 3GPP TS 38.213 V 17.6.0 (2023-06)