3GPP TSG RAN WG1 Meeting #114					R1-2308043

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.16.15

Source:	MediaTek Inc.

Title:	UE Features for NR NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#97 [1], the revised WID on NR NTN enhancements was endorsed.  In this contribution, UE features for Release 18 NR_NTN_enh WI are discussed.

- 2 FG 44-1 PUCCH repetition for Msg4 HARQ-ACK

RAN1#113 made the following agreements on 44-1 PUCCH repetition for Msg4 HARQ-ACK [2]

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-1 | PUCCH repetition for Msg4 HARQ-ACK | 1. Support repetition transmission of PUCCH for Msg4 HARQ-ACK on common PUCCH resource 2. Support receiving repetition factor in system information [3. Support receiving repetition factor in DCI scheduling Msg4 PDSCH] [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition] [5. Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration] |  | Yes | No | UE does not support PUCCH repetition for Msg4 HARQ-ACK | Per Band | N/A | N/A | N/A | A UE that includes [RAN2 parameter name] in [RRC Setup Request] must support FG 44-1 [Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

On components [3 “Support receiving repetition factor in DCI scheduling Msg4 PDSCH] and [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition] highlighted in yellow, based on RAN1#111 working assumption copied below we note the following:

- If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

- If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

Working assumption (RAN#111)

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

Hence, we think the component [3] in FG 44-1 should be revised to reflect the working assumption in red text in proposal 1 below

Proposal 1:  Revise components 3 and 4 in FG 44-1 PUCCH repetition for Msg4 HARQ-ACK in “red text”

- [3 “Support receiving repetition factor in DCI scheduling Msg4 PDSCH if multiple factors are configured in system information]

- [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition if repetition factor is configured in system information]

# 4 Conclusion

In this contribution, the following proposal was made:

Proposal 1:  Revise components 3 and 4 in FG 44-1 PUCCH repetition for Msg4 HARQ-ACK in “red text”

- [3 “Support receiving repetition factor in DCI scheduling Msg4 PDSCH if multiple factors are configured in system information]

- [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition if repetition factor is configured in system information]

# 6 References

- RP-231484, NR NTN (Non-Terrestrial Networks) enhancements WID, RP#100, Taipei, June, 2023

- R1-230163, Session Notes of AI 9.16.5, Ad-Hoc Chair (AT&T), RAN1#113, Incheon, May 2023