3GPP TSG RAN WG1 #114		                                                      R1-2308463

Toulouse, France, August 21st – August 25th, 2023

Source:	Moderator (CATT)

Title:	Summary of O_ACK determination for Type-2 HARQ-ACK codebook

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

This document summarizes the discussions during RAN1#114 on the following CRs.

R1-2307025	Correction on O_ACK determination for Type-2 HARQ-ACK codebook	CATT

R1-2307026	Correction on O_ACK determination for Type-2 HARQ-ACK codebook	CATT

- Discussion

  - Summary of the changes

The CRs above include the following two changes:

- Change OACK to include HARQ-ACK information for SPS PDSCH receptions;

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits and OACK=OACK+1. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits and OACK=OACK+OSPS-ACK, where OSPS-ACK is the number of HARQ-ACK information bits for the multiple activated SPS PDSCH receptions.

- Change notation OACK to OACK in clause 9.1.3.1.

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACKOACK=2⋅4⋅j+Vtemp2 else OACKOACK=4⋅j+Vtemp2 end if oiACK=NACK for any i∈0,1,⋯,OACKOACK-1\Vs If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACKOACK HARQ-ACK information bits. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACKOACK HARQ-ACK information bits.

  - Discussion

Question 1: Do you agree that OACK is incremented to include SPS HARQ-ACK bits for Type-2 HARQ-ACK codebook?

|  | Company |
| --- | --- |
| Agree | ZTE, Qualcomm, Spreadtrum, Nokia, LGE, Ericsson |
| Not agree |  |

| Company | Comments |
| --- | --- |

Question 2: Do you think “appends it to the OACK HARQ-ACK information bits” in current specification means that OACK is incremented to include SPS HARQ-ACK bits?

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits and OACK=OACK+1. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits and OACK=OACK+OSPS-ACK, where OSPS-ACK is the number of HARQ-ACK information bits for the multiple activated SPS PDSCH receptions.

|  | Company |
| --- | --- |
| Yes | ZTE, Qualcomm, Spreadtrum, Nokia, LGE’, Ericsson |
| No |  |

| Company | Comments |
| --- | --- |

Question 3: Do you agree with the following changes?

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits and OACK=OACK+1. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits and OACK=OACK+OSPS-ACK, where OSPS-ACK is the number of HARQ-ACK information bits for the multiple activated SPS PDSCH receptions.

|  | Company |
| --- | --- |
| Agree |  |
| Not agree | ZTE, Qualcomm, Spreadtrum, Nokia, LGE, Ericsson |

| Company | Comments |
| --- | --- |
| ZTE | Seems the current wording is clear. No need to add more. |
| Qualcomm | Agree with ZTE. |
| Spreadtrum | Agree with ZTE. |
| Nokia | Agree with ZTE |
| LGE | Agree with ZTE |
| Ericsson | Same view as other comapnies |

Question 4: Do you agree with the following changes?

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACKOACK=2⋅4⋅j+Vtemp2 else OACKOACK=4⋅j+Vtemp2 end if oiACK=NACK for any i∈0,1,⋯,OACKOACK-1\Vs If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACKOACK HARQ-ACK information bits. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACKOACK HARQ-ACK information bits.

|  | Company |
| --- | --- |
| Agree | ZTE, Qualcomm, Spreadtrum, Nokia, LGE |
| Not agree | [Ericsson] |

| Company | Comments |
| --- | --- |
| ZTE | Can be an alignment CR and treated by editor. |
| Qualcomm | Agree with ZTE. |
| Nokia | Agree with ZTE |
| LGE | Agree with ZTE |
| Ericsson | Even for alignment CR, would be cat F. If most companies see it is important, we can be flexible but we appreciate to avoid these kind of correction. |

- Conclusion

Summary of discussion:

Companies agree that OACK is incremented to include SPS HARQ-ACK bits for Type-2 HARQ-ACK codebook and “appends it to the OACK HARQ-ACK information bits” in current specification means that OACK is incremented to include SPS HARQ-ACK bits, thus the following proposed changes in R1-2307025 and R1-2307026 are not necessary.

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits and OACK=OACK+1. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits and OACK=OACK+OSPS-ACK, where OSPS-ACK is the number of HARQ-ACK information bits for the multiple activated SPS PDSCH receptions.

For the change of O_ACK from superscript to subscript, companies think it can be included in alignment CR.

Proposed Conclusion:

The specification is already clear with regard to inclusion of SPS HARQ-ACK bits for Type-2 HARQ-ACK codebook as proposed in R1-2307025 and R1-2307026. No further clarification in TS 38.213 is needed.

For the editors:

The following text proposal is provided to improve the clarity of the RAN1 specifications. Please include it in the Rel-17 alignment CR (TS38.213) section 9.1.3.1.

#### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel <Unchanged parts are omitted> if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACKOACK=2⋅4⋅j+Vtemp2 else OACKOACK=4⋅j+Vtemp2 end if oiACK=NACK for any i∈0,1,⋯,OACKOACK-1\Vs If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception based on downlinkHARQ-FeedbackDisabled if provided [12, TS 38.331], including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACKOACK HARQ-ACK information bits. If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI and excluding the ones that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if downlinkHARQ-FeedbackDisabled is provided, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACKOACK HARQ-ACK information bits. <Unchanged parts are omitted>