3GPP TSG RAN WG1 #115	R1-2311149

Chicago, USA, November 13th – 17th, 2023

Source: 	Intel Corporation

Title: 	Discussion on AI/ML for CSI feedback

Agenda item:	8.14.2

Document for: 	Discussion and Decision

# Introduction

The new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study objectives includes the analysis of solutions for CSI feedback enhancements.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project

In this contribution, we express our views on remaining issues for potential specification support of CSI compression using two-sided AI model sub-use case and CSI prediction using one-sided AI model at the UE side sub-use case.

Discussion

## CSI compression using two-sided AI model

### Pros/Cons of training collaboration types

At the last RAN1 meeting significant progress was achieved for discussion on pros and cons of different training collaboration types. Majority of rows for the corresponding tables were agreed. A few rows from the tables are marked in yellow and require further discussion.

Type 1 training: Feasibility of allowing UE side and NW side to develop/update models separately.

Type 1 training assumes that joint training is done at one side (either NW side or UE side), CSI generation part and/or CSI reconstruction part of the jointly trained two-sided model is shared with the other side. For NW side Type 1 training, separate development/update of CSI generation part at the UE side is not possible considering the above definition of Type 1 training. Thus, it is not feasible to allow UE side to develop/update model separately for this case. Similar logic is applied for development/update of CSI reconstruction part model for UE side Type 1 training.

At the same time, for NW side Type 1 training, development/update of CSI reconstruction part model is feasible since NW side has all the tools to allow compatibility of a NW side model with a UE side model with the best performance possible. Similar logic is applied for development/update of CSI generation part model for UE side Type 1 training.

Type 1 training: Extendibility.

After training procedure is finished and the corresponding models are deployed, it is still possible to update one of the models for Type 1 training. For example, for NW side training, CSI reconstruction part model can be updated such that it is still compatible with the CSI generation part model in use. It can be done by using CSI generated by the deployed CSI generation part model for training of the new/updated NW side model. Similar logic is applied to UE side training for update of CSI generation part model.

However, extendibility of CSI generation part model for NW side training is limited due to requirement on compatibility with NW side model in use. Updated CSI generation part can be trained at the UE side by using dataset generated with the CSI generation part in use (trained at the NW side). However, in this case there may be potential issues with dataset alignment for different CSI generation parts training which may lead to degradation of system performance. Similar logic is applied to UE side training for update of CSI reconstruction part model.

Proposal 1:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration type 1.

| Training type Characteristics | Type1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Feasibility of allowing UE side and NW side to develop/update models separately | UE side: Infeasible. NW side: Feasible. | UE side: Infeasible. NW side: Feasible. | UE side: Feasible. NW side: Infeasible. | UE side: Feasible. NW side: Infeasible. |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Limited | Limited | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Support | Support | Limited | Limited |

Type 2 sequential training: Feasibility of allowing UE and NW side to develop/update models separately.

For Type 2 sequential training it is assumed that NW side holds trained CSI reconstruction part and UE side can train CSI generation part by obtaining FP/BP (Forward/Backward propagation) information from the NW side. In this case CSI reconstruction part model can be developed/updated at the NW side separately from the development/update of CSI generation part model at the UE side (i.e., models may have an arbitrary model structure except alignment on input/output formats and quantization). After model is developed/updated UE side or NW side shall initiate/request model training procedure. Thus, in our view, it is feasible to allow UE and NW side to develop/update models separately for Type 2 sequential training.

Extendibility for type 3 training.

For NW first Type 3 training, after training procedure is completed and both NW side and UE side models are deployed, it is still possible to update NW side model while maintaining compatibility with the deployed UE side model. This can be done by using training dataset and CSI generation part used at the first step of separate training procedure. Also, in this case NW can evaluate the performance of the new NW side model by using a testing dataset and decide whether to deploy the new model or not. Thus, extendibility to train new NW-side model compatible with UE-side model in use is supported for NW first Type 3 training. Similar logic is applied for  extendibility to train new UE-side model compatible with NW-side model in use in case of UE first Type 3 training.

Proposal 2:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration types 2 and type 3.

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (note 1) | NW first | UE first |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | Support | Support |

### Model performance monitoring and data collection

Efficiency of NW-side model performance monitoring based on intermediate KPI calculated using ground truth CSI report depends on the accuracy of the corresponding ground truth CSI report. Based on the evaluation results presented in [2] for Option 1 evaluation methodology, it is observed that new CSI report formats improve the accuracy of the corresponding KPI. However, the evaluation methodology does not consider all the aspects to suggest if enhancements are beneficial for the system performance. First, model performance monitoring based on ground truth CSI reporting for one time instance is not robust to channel variations in time as shown in [2]. Second, the aspect of CSI overhead for ground truth CSI is not considered. For instance, ground truth CSI report transmitted every 100 ms with high accuracy (e.g., ~500 bits) will significantly increase the total CSI overhead which will have negative impact on system performance.

Observation 1:

- At least the following aspects require further study for NW-side model performance monitoring based on ground truth CSI quantization

  - Robustness of model performance monitoring against channel variations in time

  - Efficiency of model performance monitoring considering the corresponding CSI feedback overhead

As it was mentioned above, new CSI report format for ground truth CSI quantization improves the accuracy of the corresponding KPI for model performance monitoring. However, considering the above observation introduction of new CSI report format is not fully justified. Support of certain CSI report format for ground truth CSI quantization should be further studied considering total CSI overhead including the periodicity of such CSI report with potential averaging to make the measured KPI more robust to channel variations in time.

Proposal 3:

- The following conclusion is made for intermediate KPI-based model performance monitoring with ground truth CSI reporting

  - Whether/how to support new CSI report format for ground truth CSI quantization should be decided in future releases considering the corresponding CSI overhead

For AI/ML-based CSI compression, data for model training can be collected at the NW-side by using conventional (non-AI/ML) CSI feedback. Considering that the AI/ML-based CSI compression is aiming to improve CSI accuracy and/or decrease CSI overhead comparing to conventional PMI codebooks, ground truth CSI quantization with number of bits A  is required to train AI/ML model corresponding to CSI report with B bits, where A > B. SGCS performance evaluation results are presented in [2] for different assumptions on quantization of the training dataset (including scalar quantization and eType II-like quantization) for AI/ML model with B = 128 CSI bits. According to the evaluation results, performance degradation is observed for ground truth CSI quantization with less than 3329 bits which is much larger comparing to number of bits for CSI report with existing Type II PMI codebooks. Hence, it can be concluded that specification enhancements are required for data collection based on CSI feedback to support ground truth CSI quantization with higher accuracy comparing to the existing PMI codebooks.

Proposal 4:

- Conclude that specification enhancements are required for data collection based on CSI feedback

  - Support codebook-based (eType II-like) and/or scalar ground truth CSI quantization with higher CSI accuracy and larger CSI reporting overhead comparing to existing Type II codebooks

### AI/ML model input and inference

At the RAN1#110 meeting the following agreements were made w.r.t. AI/ML model input and inference.

Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination Agreement In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least - Model output type/dimension/configuration and potential post processing

Since an AI/ML model is trained assuming a particular algorithm for pre-processing, it doesn’t make sense to have separate configuration of pre-processing algorithm and AI/ML model. Thus, if an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE. Since the pre-processing and AI/ML model are highly related it is important to provide clear definition of pre-processing algorithm. To describe pre-processing at least linear transforms (DFT across different dimensions), down selection of matrix elements and normalization operations should be defined as well as the CSI-RS ports mapping and mapping of subbands to the input of the pre-processing.

The parameters for the model input can be defined like parameters L and M for the Enhanced Type II PMI codebook, where parameter L controls the number of inputs for spatial domain and parameter M controls the number of inputs for frequency domain, so the dimensions of input matrix are L × M.

Information from the pre-processing step may be required at the gNB side, otherwise it may not be possible to reconstruct the precoding matrix at the gNB, e.g., if oversampling (SD basis rotation factor) of Spatial Domain (SD) vectors is used and/or selection of SD or Frequency Domain (FD) vectors is used at the pre-processing step. In such case, information from pre-processing step shall be reported by the UE together with CSI bits generated by the AI/ML model.

Proposal 5:

- Pre/post-processing may include at least linear transforms (e.g., DFT across different dimensions), down selection of matrix elements and normalization

- The dimensions of the input matrix are defined by parameters similar to parameters L/M for Enhanced Type II PMI codebook (input matrix corresponds to the AI/ML model input after pre-processing)

- Information from pre-processing step can be reported by the UE together with CSI bits generated by the AI/ML model (e.g., selected basis vectors, basis rotation factor, etc.)

## CSI prediction using one-sided AI model at the UE side

The following agreement was made at RAN1#111 meeting for CSI prediction using one-sided AI model at the UE side.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

In Rel-18 MIMO WI UE-side channel prediction support is enabled to enhance performance for UEs with high/medium velocities, new codebooks for predicted PMI with the corresponding prediction timing configuration and CQI determination mechanism are specified. To avoid redundant work, AI/ML based CSI prediction shall reuse the specified CSI reporting mechanism of the new codebooks for predicted PMI.

Proposal 6:

- CSI prediction with one-sided AI model at the UE side should be based on CSI reporting with the following PMI codebooks

  - Enhanced Type II codebook for predicted PMI

  - Further enhanced Type II port selection codebook for predicted PMI

Performance of CSI prediction depends on many factors. For a given prediction algorithm (e.g., for AI/ML CSI prediction with a particular trained model) prediction accuracy may vary significantly for different signal propagation conditions and UE speed. In some cases, CSI accuracy corresponding to CSI with prediction may be worse comparing to CSI accuracy corresponding to CSI without prediction (sample-and-hold approach or channel averaging). Thus, performance monitoring is very important to avoid CSI performance degradation. The above statement is applicable for both AI/ML CSI prediction and non-AI/ML CSI prediction.

The following agreement on model performance monitoring for CSI prediction was achieved at RAN1#114.

Agreement For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculate the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculate the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report. - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

The above agreement provides a list of basic approaches for CSI prediction performance monitoring which can be further considered for normative work with potential down-selection. However, there is no conclusion which suggests that performance monitoring feature is required for AI/ML CSI prediction. Also, as performance monitoring is beneficial for both AI/ML and non-AI/ML CSI prediction, solution for model performance monitoring shall be applicable to non-AI/ML CSI prediction as well.

Proposal 7:

- Conclude that model performance monitoring feature should be supported for CSI prediction with one-sided AI model at the UE side

  - Solution for the model performance monitoring shall be applicable to non-AI/ML CSI prediction

Conclusion

In this contribution, we have provided our views on sub use cases for AI/ML CSI including potential specification impact. The following proposals and observations were made.

Proposal 1:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration type 1.

| Training type Characteristics | Type1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Feasibility of allowing UE side and NW side to develop/update models separately | UE side: Infeasible. NW side: Feasible. | UE side: Infeasible. NW side: Feasible. | UE side: Feasible. NW side: Infeasible. | UE side: Feasible. NW side: Infeasible. |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Limited | Limited | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Support | Support | Limited | Limited |

Proposal 2:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration types 2 and type 3.

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (note 1) | NW first | UE first |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | Support | Support |

Observation 1:

- At least the following aspects require further study for NW-side model performance monitoring based on ground truth CSI quantization

  - Robustness of model performance monitoring against channel variations in time

  - Efficiency of model performance monitoring considering the corresponding CSI feedback overhead

Proposal 3:

- The following conclusion is made for intermediate KPI-based model performance monitoring with ground truth CSI reporting

  - Whether/how to support new CSI report format for ground truth CSI quantization should be decided in future releases considering the corresponding CSI overhead

Proposal 4:

- Conclude that specification enhancements are required for data collection based on CSI feedback

  - Support codebook-based (eType II-like) and/or scalar ground truth CSI quantization with higher CSI accuracy and larger CSI reporting overhead comparing to existing Type II codebooks

Proposal 5:

- Pre/post-processing may include at least linear transforms (e.g., DFT across different dimensions), down selection of matrix elements and normalization

- The dimensions of the input matrix are defined by parameters similar to parameters L/M for Enhanced Type II PMI codebook (input matrix corresponds to the AI/ML model input after pre-processing)

- Information from pre-processing step can be reported by the UE together with CSI bits generated by the AI/ML model (e.g., selected basis vectors, basis rotation factor, etc.)

Proposal 6:

- CSI prediction with one-sided AI model at the UE side should be based on CSI reporting with the following PMI codebooks

  - Enhanced Type II codebook for predicted PMI

  - Further enhanced Type II port selection codebook for predicted PMI

Proposal 7:

- Conclude that model performance monitoring feature should be supported for CSI prediction with one-sided AI model at the UE side

  - Solution for the model performance monitoring shall be applicable to non-AI/ML CSI prediction

References

- RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021.

- R1-2306832 Evaluation on AI/ML for CSI feedback enhancement, Intel Corporation