3GPP TSG-RAN WG1 Meeting #116	R1-2400515

26 Feburary – 01 March 2024, Athens, GR

Agenda item:	9.5.1

Source:	Dell Technologies

Title:	On-demand SSB delivery of NES cells

Document for:	Decision, Discussion

# 1	Introduction

Based on the latest agreements of RAN#102 plenary meeting, a WI on network energy efficiency (NES) (for Release-19) has been approved [1], with the following objectives:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

This contribution presents our views on the on-demand SSB triggering signaling options.

# 2	On-demand SSB triggering

Network energy saving (NES) solutions are vital for saving the OPEX cost of the radio gNBs, which is the major cost contributing element of the entire cellular networks. Thus, solutions for NES cells to avoid transmitting the always-ON SSB and/or SIB signals clearly lead to an optimized energy cion performance.

Observation 1: Avoiding to always or occasionally transmit the SSB and/or SIB signals clearly offers a network energy saving (NES) gain of NES cells.

Coverage issues and NES coverage provisioning

Per the latest agreements, the NES capable devices should only be able to camp on anchor cells, where they assume receiving SSBs, and/or SIBs, and/or paging signals from. This clearly does not add additional power consumption overhead of devices, due to avoiding to camp on multiple cells at the same time. For collocated anchor and NES cells, no coverage issues are identified. However, for non-collocated cells, it may negatively impact the coverage performance, in case the anchor and NES cells have a large received coverage gap.  For instance, when the NES capable device received much better coverage level from NES cell than the anchor cell.

Observation 2: For collocated anchor versus NES cells, no coverage issues are identified, thus, it is reasonable that NES devices only camp on anchor cells.

Observation 3: For non-collocated anchor versus NES cells, coverage issues can be exhibited in case the coverage level of NES cells is much better than the received level from anchor cells. Thus, for generality of considered SSB/SIB-less solutions, coverage provisioning step can be added.

Thus, for generality of SSB/SIB-less NES solutions, we suggest considering additional coverage provisioning step for checking if the NES cell offers much better received coverage level to the NES capable device than the currently camped anchor cell.

Proposal 1: For generality of SSB/SIB-less NES solutions, to include cases of NES cells not only purposed for capacity bosting, or non-collocated anchor versus NES cells, studying and specifying coverage provisioning of NES cells is important, specifically, for NES devices, reporting poor coverage levels, when camping on anchor cells.

As shown by overall Fig. 1, NES coverage provisioning can only be triggered when an anchor cell receives a camping NES capable device with a poor coverage level, e.g., below a predefined coverage threshold. Therefore, the anchor cell transmits a request indication to associated NES cell(s) for transmitting an on-demand SSB block, for the receiving device to provision the received coverage level from the NES cell. The transmitted SSB can be a full SSB block transmission or light SSB block transmission. The latter option enables NES cells not to transmit the full SSB block since in this case, it can be considered as measurement-only SSB, where the rest of NES access information can be transmitted, or piggybacked with signaling, from anchor cells.

Figure 1: NES coverage provisioning.

Proposal 2: When anchor cell receives a camping NES capable device with highly poor coverage level, i.e., below a threshold, NES coverage provisioning is triggered.

Proposal 3: On condition of activating NES coverage provisioning, anchor cells transmit on-demand SSB/light SSB transmission request towards associated NES cells, over XN/backhaul interface.

Proposal 4: On condition of activating NES coverage provisioning, anchor cells configure camping NES capable devices with on-demand SSB/light SSB measurement of NES cells while halting their RRC connection establishment.

Proposal 5: (Send an LS to) RAN3 to support the on-demand SSB/light SSB transmission request over XN/backhaul interface from anchor cells to NES cells.

Device behavior

For the baseline NES scenario, of an anchor cell transmitting an SSB/SIB on behalf of collocated or surrounding capacity-boosting NES cell, the device expects to receive SSB, SIB, RACH and paging information of both the anchor and NES cells from the anchor cell during connection establishment. Thus, without NES coverage provisioning, NES devices can still perform RACH on NES cell, blindly without determining the received coverage from NES cells. This option is mainly appropriate only when both NES and anchor cells are closely located or collocated.

On the paging handling of the NES devices in coverage of one or more NES cell, there would be no problem, for NES devices to always monitor paging on anchor cells if both the anchor and NES cells are on the same radio notification area (RNA), since paging indications will be still transmitted within entire RNA, including the anchor cells. In case of anchor and NES cells of different RNAs, the NES devices shall trigger an RNA update and hence, paging resources will be updated. Therefore, it is reasonable that paging is always monitored from the anchor cell.

Proposal 6: NES devices expect to receive SSB, SIB, paging and RACH information for both NES and anchor cells, from the anchor cells.

Proposal 7: RAN1 to study and specify signaling procedures for carrying SIB and RACH information of NES cells, from the anchor cell, assuming there can be multiple NES cells associated with an anchor cell, i.e., anchor cell is piggybacking SSB and RACH information of NES cells.

With the NES coverage provisioning, new device behaviours are also expected. When the NES capable device attempts camping the anchor cell, with a coverage level that below a predefined threshold, it may receive an on-demand SSB/light SSB measurement request from the anchor cell, to measure the on-demand SSB of the adjacent NES cells. Thus, it halts its RRC connection establishment, i.e., extend its connection establishment timers, while measuring the requested SSB/light SSBs of the NES cells.

Proposal 8: With NES coverage provisioning, the NES capable devices measure the on-demand SSB/light SSB of the NES cells while extending the RRC connection timers of the current anchor cell.

# 4	Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: Avoiding to always or occasionally transmit the SSB and/or SIB signals clearly offers a network energy saving (NES) gain of NES cells.

Observation 2: For collocated anchor versus NES cells, no coverage issues are identified, thus, it is reasonable that NES devices only camp on anchor cells.

Observation 3: For non-collocated anchor versus NES cells, coverage issues can be exhibited in case the coverage level of NES cells is much better than the received level from anchor cells. Thus, for generality of considered SSB/SIB-less solutions, coverage provisioning step can be added.

Proposal 1: For generality of SSB/SIB-less NES solutions, to include cases of NES cells not only purposed for capacity bosting, or non-collocated anchor versus NES cells, studying and specifying coverage provisioning of NES cells is important, specifically, for NES devices, reporting poor coverage levels, when camping on anchor cells.

Proposal 2: When anchor cell receives a camping NES capable device with highly poor coverage level, i.e., below a threshold, NES coverage provisioning is triggered.

Proposal 3: On condition of activating NES coverage provisioning, anchor cells transmit on-demand SSB/light SSB transmission request towards associated NES cells, over XN/backhaul interface.

Proposal 4: On condition of activating NES coverage provisioning, anchor cells configure camping NES capable devices with on-demand SSB/light SSB measurement of NES cells while halting their RRC connection establishment.

Proposal 5: (Send an LS to) RAN3 to support the on-demand SSB/light SSB transmission request over XN/backhaul interface from anchor cells to NES cells.

Proposal 6: NES devices expect to receive SSB, SIB, paging and RACH information for both NES and anchor cells, from the anchor cells.

Proposal 7: RAN1 to study and specify signaling procedures for carrying SIB and RACH information of NES cells, from the anchor cell, assuming there can be multiple NES cells associated with an anchor cell, i.e., anchor cell is piggybacking SSB and RACH information of NES cells.

Proposal 8: With NES coverage provisioning, the NES capable devices measure the on-demand SSB/light SSB of the NES cells while extending the RRC connection timers of the current anchor cell.

# References

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Moderator (Huawei), 3GPP RAN#102, Dec 2023.