3GPP TSG RAN WG1 #116			     R1-2400575

Athens Greece, February 26th – March 1st, 2024

Agenda item:	9.7

Source:	Xiaomi, AT&T

Title:	Discussion on CR to introduce channel model for ISAC

Document for:	Decision

- Introduction

In the RAN#102 meeting, a new study item on channel modelling for ISAC was approved with the following objectives [1].

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

Based on the above SID, a CR to TR 38.901 [2] will be prepared as the output of the study item. Instead of discussing and agreeing every aspect of the CR right before the end of the study item, a more manageable approach is to maintain a running CR, which can be updated after each RAN1 meeting. Subject to RAN1 agreement, this CR can be endorsed as a final CR to TR 38.901 at the end of the study item. In this contribution, we discuss the options for a skeleton CR which outlines how the additions for ISAC channel modelling can be incorporated into TR 38.901. This document is submitted together with the proposed work plan on ISAC channel model in [3].

- Discussion

The existing TR 38.901 defines the channel model primarily for communication purposes. To enable the evaluation for sensing techniques and related operations, it is necessary for the channel model to address target modelling and sensing, and define distinct characteristics to identify one or more targets by differentiation from the background environment. It is anticipated such new functions will result in a significant update to the TR 38.901. Almost all existing sections/sub-sections in TR 38.901 will be impacted by introducing ISAC channel model. Multiple options can be considered to organize the CR to TR 38.901.

- Option 1: Update details of ISAC channel model into existing sections/sub-sections in TR 38.901

Since it is expected that large amounts of updates are necessary, the sections/sub-sections may become unreadable. Additionally, ISAC channel modelling as output of this SI would require tight coordination with the CR for the outcome of SI “Study on Channel Modelling Enhancements for 7-24 GHz for NR”, creating unnecessary work.

- Option 2: Creating new sections/subsections for ISAC channel model in TR 38.901

  - Option 2A: ISAC channel model is updated into a subsection, e.g., 7.6.13 under “7.6 Additional modelling components”.

The ISAC channel model involves many aspects especially scenarios and calibrations which may not be suitable to be included in a subsection under “7.6 Additional modelling components”.

  - Option 2B: For a section/sub-section which is expected to be updated a lot, create a new section by appending ‘A’ to the existing section/sub-section number, e.g., section 7.2A, 7.4A, 7.5A, 7.6.3A, 7.7A, 7.8A, etc. For other sections/sub-sections, it would be fine to directly update the existing sections/sub-sections.

This option is logically fine. The only concern is that we had to duplicate almost all existing subsections in section 7.

  - Option 2C: For section 7 and if agreed section 8, create a new section by appending ‘A’ to the existing section number, e.g., section 7A, 8A. For other sections, it would be fine to directly update the existing sections/sub-sections.

Compared with Option 2B, this option is the cleaner approach for the CR.

Based on some early discussions, most companies are OK with Option 2C for the CR. Accordingly, the rapporteurs are providing a skeleton CR in the ANNEX of this contribution. Note: In the current skeleton CR, only the high-level sections for ISAC channel model are proposed. Additional clauses and sub-clauses may be added based on RAN1 progress/agreements.

Proposal: RAN1 to endorse the skeleton CR in the Annex and update the running CR after each RAN1 meeting.

- Conclusion

This contribution provides a skeleton CR to TR 38.901 which can be used to update agreements/conclusions after each RAN1 meeting. It is proposed to endorse the skeleton and maintain the running CR after each RAN1 meeting.

Proposal: RAN1 to endorse the skeleton CR in the Annex and update the running CR after each RAN1 meeting.

References

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Nokia, Nokia Shanghai Bell

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz, V17.1.0 (2024-01)”

- R1-2400574, “Proposed work plan on channel modelling for ISAC”, Xiaomi, AT&T

ANNEX Skeleton CR to TR 38.901

3GPP TSG RAN WG1 #116	R1-240xxxx

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | Draft | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to introduce channel model for ISAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing_NR |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN #102, the study item for channel modelling for ISAC was agreed. The output of the study item was agreed to incorporate into TR 38.901. This CR will be continuously updated to capture the agreements/conclusions after each RAN1 meeting. Subject to RAN1 agreement, this CR can be endorsed as a final CR to TR 38.901. The proposed draft CR is only a skeleton which indicates how the additions for ISAC channel modelling can be incorporated into TR 38.901. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sections for the channel model of ISAC are added if a large part of an existing section is expected to be updated to improve readability. For other additional small updates for ISAC channel modelling, some of the existing sections will be re-used (not shown for the moment). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing channel model for communications in TR 38.901 are not sufficient to address evaluation of sensing techniques for Integrated Sensing and Communications use cases. The existing models in TR38.901 are not designed for sensing, in particular they do not address target modelling and sensing, background environment modelling and differentiation from targets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Impact to all/most clauses expected |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1	Scope

[Rapporteur’s note: To be updated if a necessary change is identified]

2	References

[Rapporteur’s note: To be updated if a necessary change is identified]

3	Definitions, symbols and abbreviations

[Rapporteur’s note: To be updated if a necessary change is identified]

4	Introduction

[Rapporteur’s note: To be updated if a necessary change is identified]

5	Void

6	Status/expectation of existing information on high frequencies

[Rapporteur’s note: To be updated if a necessary change is identified]

7	Channel model(s) for 0.5-100 GHz

7A	Channel model(s) for 0.5-100 GHz for ISAC

[Rapporteur’s note: This clause is to capture the general principles on ISAC channel modelling, e.g., target modelling and sensing, background environment modelling and differentiation from targets]

- 7A.1	Coordinate system

[Rapporteur’s note: This clause can refer to section 7.1, and/or may reflect any necessary changes specific to ISAC, if identified]

- 7A.2	Scenarios

[Rapporteur’s note: This clause is to capture the agreed scenarios for ISAC, which can be used for channel model calibration]

- 7A.3	Antenna modelling

[Rapporteur’s note: This clause can refer to section 7.3 and/or may reflect any necessary changes specific to ISAC, if identified]

- 7A.4	Sensing target modelling

[Rapporteur’s note: this clause is to capture the agreements on the modelling of sensing target, e.g., single or multiple points modelling, RCS of target, etc.]

- 7A.5	Pathloss, LOS probability and penetration modelling

[Rapporteur’s note: this clause is to capture the agreements on slow fading modelling for ISAC.]

- 7A.6	Fast fading model

[Rapporteur’s note: this clause is to capture the agreements on the fast fading channel model for i.e., bistatic and monostatic sensing mode.]

- 7A.7	Additional modelling components for ISAC

[Rapporteur’s note: this clause is to capture the agreements on additional modelling components.]

- 7A.7.1	Spatial consistency

[Rapporteur’s note: this clause is to capture the agreements on spatial consistency for ISAC.]

- 7A.8	Channel models for link-level evaluations

[Rapporteur’s note: this clause is to capture the agreements on LLS channel model for ISAC.]

- 7A.9	Channel model calibration

[Rapporteur’s note: this clause is to capture the calibration assumptions/results on the channel model for ISAC.]

8	Map-based hybrid channel model (Alternative channel model methodology)

[8A	 Map-based hybrid channel model (Alternative channel model methodology) for ISAC]

[rapporteur’s note: if agreed, this clause is to capture the agreements on map-based hybrid channel model for ISAC.]