3GPP TSG RAN WG1 #116		R1-2400580

Athens, Greece, February 26th – March 1st, 2024

Source:	OPPO

Title:	Text Proposals on Expanded and Improved NR Positioning

Agenda Item:	8.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will provide a few text proposals for the Rel-18 expanded and improved NR positioning

- Text Proposals

2.1 Text Proposal #1

Proposal 1: Adopt the following TP for TS38.214:

- Reason for change: (1) A few high layer parameter names are in [] now and are not aligned with TS 37.355. (2) The description of “DL carrier phase measurement” is not clear. - Summary of change: (1) Align the high layer parameter names in 214 with that in 37.355. (2) Clarify that the UE can measure DL RSCP or DL RSCPD in RRC_INACTIVE or RRC_IDLE state. - Consequences if not approved: The description of carrier phase positioning measurement is ambiguous.

TS 38.214 ##### 5.1.6.5.2 PRS for carrier phase positioning *** Unchanged parts are omitted *** If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be provided with nr-PRU-RSCP-MeasInfo or nr-PRU-DL-TDOA-MeasInfo which contains DL RSCP/RSCPD measurements together with DL RSTD, DL PRS-RSRP, and/or DL PRS-RSRPP measurement(s) associated with the RSCP/RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] the timestamps associated with the measurements, and the location information of the PRU. The UE may be configured to report quality metrics NR-PhaseQuality corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]: - phaseQualityValue which provides the uncertainty of the measurement - phaseQualityResolution which specifies the resolution levels used in the [phase quality index] field. The UE in RRC_INACTIVE or RRC_IDLE mode is expected to perform the DL RSCP or DL RSCPD measurement from the bandwidth of a DL PRS resource including outside of the initial downlink bandwidth part.

2.2 Text Proposal #2

Proposal 2: Adopt the following TP for TS38.214:

- Reason for change: The following RAN1 agreement is implemented in 38.214 but the specification description is not accurate: Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions.. - Summary of change: Correct the specification description to accurately implement the agreement. - Consequences if not approved: The corresponding UE behavior is not clear.

TS 38.214 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements *** Unchanged parts are omitted *** When an SRS resource configured in a component carrier without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of a component carrier with PUSCH and/or PUCCH, there should be a guard period before and after the SRS transmissions for bandwidth aggregation, during which the UE is not expected to transmit or receive any signals or channels other than the SRS transmissions for bandwidth aggregation. If the UE receives an activation or deactivation command of semi-persistent SRS resource set(s) for positioning in up to three aggregated carriers or SRS resource set(s) for positioning in up to two aggregated carriers as specified in [10, TS 38.321] and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation or deactivation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission or cessation corresponding to the SRS resource set(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where µ is the SCS configuration for the PUCCH.

- Conclusion

In this contribution, we presented a few text proposals for Rel-18 expanded and improved NR positioning:

Proposal 1: Adopt the following TP for TS38.214:

- Reason for change: (1) A few high layer parameter names are in [] now and are not aligned with TS 37.355. (2) The description of “DL carrier phase measurement” is not clear. - Summary of change: (1) Align the high layer parameter names in 214 with that in 37.355. (2) Clarify that the UE can measure DL RSCP or DL RSCPD in RRC_INACTIVE or RRC_IDLE state. - Consequences if not approved: The description of carrier phase positioning measurement is ambiguous.

TS 38.214 ##### 5.1.6.5.2 PRS for carrier phase positioning *** Unchanged parts are omitted *** If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be provided with nr-PRU-RSCP-MeasInfo or nr-PRU-DL-TDOA-MeasInfo which contains DL RSCP/RSCPD measurements together with DL RSTD, DL PRS-RSRP, and/or DL PRS-RSRPP measurement(s) associated with the RSCP/RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] the timestamps associated with the measurements, and the location information of the PRU. The UE may be configured to report quality metrics NR-PhaseQuality corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]: - phaseQualityValue which provides the uncertainty of the measurement - phaseQualityResolution which specifies the resolution levels used in the [phase quality index] field. The UE in RRC_INACTIVE or RRC_IDLE mode is expected to perform the DL RSCP or DL RSCPD measurement from the bandwidth of a DL PRS resource including outside of the initial downlink bandwidth part.

Proposal 2: Adopt the following TP for TS38.214:

- Reason for change: The following RAN1 agreement is implemented in 38.214 but the specification description is not accurate: Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions.. - Summary of change: Correct the specification description to accurately implement the agreement. - Consequences if not approved: The corresponding UE behavior is not clear.

TS 38.214 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements *** Unchanged parts are omitted *** When an SRS resource configured in a component carrier without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of a component carrier with PUSCH and/or PUCCH, there should be a guard period before and after the SRS transmissions for bandwidth aggregation, during which the UE is not expected to transmit or receive any signals or channels other than the SRS transmissions for bandwidth aggregation. If the UE receives an activation or deactivation command of semi-persistent SRS resource set(s) for positioning in up to three aggregated carriers or SRS resource set(s) for positioning in up to two aggregated carriers as specified in [10, TS 38.321] and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation or deactivation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission or cessation corresponding to the SRS resource set(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where µ is the SCS configuration for the PUCCH.

- Reference

TS 38.214 V18.1.0