3GPP TSG RAN WG1 #116	R1-2400659

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.2.4

Source:	China Telecom, ZTE

Title:	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion

- Introduction

The new Rel-19 WI of NR MIMO Phase 5 was approved in RAN#102 meeting, and the following objective is included in [1].

5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 a. Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Asymmetric DL sTRP/UL mTRP deployment scenarios can be beneficial from the aspects of network energy saving, uplink coverage and throughput improvement. Further enhancements are needed to ensure the performance of such heterogeneous network, especially the SRS power control and UL pathloss calculation. In this contribution, we provide our views on the essential enhancements for symmetric DL sTRP/UL mTRP network deployment.

- Considerations on enhancement demand of asymmetric DL sTRP/UL mTRP deployment

Based on the study and evaluation in 3GPP Rel-17 coverage enhancement SI [2], the performance of UL channel is the bottleneck of NR network. By deploying heterogeneous network as illustrated in Fig 1, where more micro BSs can be deployment for providing UL services, the UL performance can be guaranteed and the energy consumption of macro gNB can be reduced.

Fig 1 illustration of asymmetric DL sTRP/UL mTRP deployment scenario

According to the evaluation results provided in [3], with DL/UL decoupling, i.e., introducing more micro TRPs only providing UL services on some occasions, more than 20% average UPT gain can be acquired, and the maximum UPT gain can be up to 31% for cell-edge UEs. Thus, the asymmetric network DL sTRP/UL mTRP is beneficial for UL performance, especially for the cell-edge UE.

Observation 1: Asymmetric DL sTRP/UL mTRP is especially beneficial for cell-edge UEs.

However, in some scenarios, since the distance from UE to macro and micro gNB can be quite different (like UE1 shown in Fig 1), and there can be no DL signals transmitted from macro UL TRP to UE. Thus the mechanism such as close-loop power control and path loss calculation should be enhanced.

Besides the mechanisms included in the WID, from our prospective, 2TAs for DL sTRP/UL mTRP may also need to be enhanced in ‘asymmetric DL sTRP/UL mTRP’ scenarios using the legacy PRACH resources. In Rel-18 eMIMO WID, the TA enhancement has been discussed for multi-TRP scenarios with mDCI. But in the ‘asymmetric DL sTRP/UL mTRP’ scenario, since there is no DCI transmitted from UL TRP to UE, the 2TA with mDCI introduced in Rel-18 should be extended to 2TA with one single DCI.

Proposal 1: Support to specify 2TAs for ‘asymmetric DL sTRP/UL mTRP’ with single DCI in Rel-19 MIMO WI.

- Considerations on two separate closed-loop PC adjustment state for SRS

As discussed in last section, the two separate closed-loop PC adjustments are needed for SRS, of which one is used for the CSI acquisition of DL TRP, the other for the beam management of UL TRP. In current specs [4], only one separate closed loop is supported for the SRS power adjustment. And the configuration of SRS PC can be indicated either by DCI format 0_1/1_1, for by DCI format 2_3.

However, since there may be no PUCCH/PUSCH scheduled for DL sTRP or a PDCCH/PDSCH scheduled for UL mTRPs in the asymmetric scenarios. What’s more, our initial intention for specifying the SRS close-loop PC is to decouple it with PUSCH/PDSCH. Thus, the DCI format 0_1/1_1, the main usage of which is for scheduling UL/DL transmission, are not suitable for such scenario. While DCI format 2_3 can be used to indicate the SRS power of UE without DL/UL transmission and not tied with PUSCH power control, indicating the SRS power with DCI format 2_3 can be applied for asymmetric DL sTRP/UL mTRPs scenarios.

Proposal 2: Support to indicate the two separate close-loop PC adjustment states for SRS with DCI format 2_3.

In current specs, when DCI format 2_3 is applied, the srs-TPC-PDCCH-Group in high layer parameter is always configured, which means SRS-CarrierSwitching IE is configured. From our understanding, there is no need to couple these two scenarios for gNB. Thus, if DCI format 2_3 can be adopted to indicate the two separate close-loop SRS PC, the restriction of srs-TPC-PDCCH-Group is configured with high layer should be released.

Proposal 3: The restriction of srs-TPC-PDCCH-Group configuration for DCI format 2_3 needs to be released when DCI format 2_3 is used for indicating the two separate close-loop SRS PC adjustment.

If DCI format 2_3 is adopted for indicating the two separate close-loop SRS PC adjustments, it should be specified how to indicate the relationship between DCI and SRS. Basically, explicit indication and implicit indication can both be taken to consideration for addressing the issue.

For explicit indication, the simplest method is to introduce a new field in DCI format 2_3. The field can be used to indicate the relationship. It is worth noting that, in current specs, the size of DCI format 2_3 is limited to no larger than that of DCI format 0_1. Which means, if such a field is introduced, the number of block can be indicated will be reduced.

Observation 2: If an extra 1-bit field is introduced in DCI format 2_3 for indicating the TRP the DCI is associated with, the number of block indicated by the DCI may be reduced.

While for implicit indication, multiple methods can be considered. For example, it can be specified that the DCI transmitted on odd frame is related to UL TRP, while even frame is related to DL TRP; or to reuse the specified/reservation bit in exist DCIs. But such mechanism will reduce the flexibility of network to realize the SRS PC.

Observation 3: If the implicit method is adopted to indicate the relation between TRP and DCI, the flexibility can be reduced.

In all, we are okay to discuss both explicit and implicit method for indication the association between DCI format 2_3 and TRP, with slight preference to the explicit method, but the details are to be further discussed.

Proposal 4: Both explicit and implicit indication about the relationship between DCI format 2_3 and its associated TRP can be adopted, details can be further discussed.

- PL enhancement

Since there is no DL pathloss RS from UL TRP(s), the pathloss measured from the pathloss RS from DL TRP is not accurate to represent the actual path loss between UL TRP and UE, especially if the UE is at the edge of DL macro TRP but near the center of UL micro TRP, as shown in fig 2.

Fig 2 illustration of asymmetric DL macro gNB and UL micro gNB deployment

Without 2 separate PL estimation for different TRPs, the site selection for UE may be wrong and result in a degradation on performance. Thus, it is necessary to configure an offset for UE to calculate the pathloss of UL TRP(s). Then the actual PL for micro UL TRP PSRS, UL, actual can be reformed as:

PSRS, UL, actual=PSRS, UL, calculate+PSRS, offset

Where the PSRS, UL, calculate can be calculated as described in [5].

Basically, the offset can be transmitted to UE with RRC/MAC CE or DCI. Considering the latency demand for UE to adjust the transmission power, where the acquire of PL is necessary, it is better to transmit the pathloss offset via DCI or MAC CE, the details can be further studied and discussed.

Proposal 5: Support to transmit the configuration of the pathloss offset for UL micro TRP with DCI or MAC CE from DL macro TRP.

- Conclusion

In this paper, we discuss the issues for asymmetric DL sTRP/UL mTRP scenarios and have the following observations and proposals.

Observation 1: Asymmetric DL sTRP/UL mTRP is especially beneficial for cell-edge UEs.

Observation 2: If an extra 1-bit field is introduced in DCI format 2_3 for indicating the TRP the DCI is associated with, the number of block indicated by the DCI may be reduced.

Observation 3: If the implicit method is adopted to indicate the relation between TRP and DCI, the flexibility can be reduced.

Proposal 1: Support to specify 2TAs for ‘asymmetric DL sTRP/UL mTRP’ with single DCI in Rel-19 MIMO WI.

Proposal 2: Support to indicate the two separate close-loop PC adjustment state for SRS with DCI format 2_3.

Proposal 3: The restriction of srs-TPC-PDCCH-Group configuration for DCI format 2_3 needs to be released when DCI format 2_3 is used for indicating the two separate close-loop SRS PC adjustment.

Proposal 4: Both explicit and implicit indication about the relationship between DCI format 2_3 and its associated TRP can be adopted, details can be further discussed.

Proposal 5: Support to transmit the configuration of the pathloss offset for UL micro TRP with DCI or MAC CE from DL macro TRP.

- References

3GPP RP-234007, “New WID: NR MIMO Phase 5”, Samsung (Moderator), RAN#102, Dec 11th- 15th, 2024.

3GPP TR 38.830, “Study on NR coverage enhancements (Release 17)”.

3GPP RP-233609, “Views on MIMO evaluation in Rel-19”, ZTE, RAN #102 meeting, Dec., 2023.

3GPP TS 38.212, “"NR; Multiplexing and channel coding" (Release 18)”.

3GPP TS 38.214, “Physical layer procedures for data (Release 18)”.