3GPP TSG RAN WG1 #116	R1-2400907

Athens, Greece, February 26th - March 1st, 2024

Agenda Item:	9.1.1

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Specification support for beam management

Document for:	Discussion & Decision

# Introduction

A new work item on AI/ML for NR air interface was approved in the last RAN plenary meeting RAN#102 [1] and the support of AI for beam management by the specification was agreed. As a starting point for the RAN1#116 discussion, the following guidance was given:

- Beam management – DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In the AI/ML SI [2], two use cases for beam management have been studied, BM-Case 1 and BM-Case 2. Furthermore, NW-sided models and UE-sided models have been considered for these use cases. As per the WID, it was agreed to specify both flavors having different requirements to the specification. Thus, we structure our contribution according to the WID and will highlight aspects for NW-sided and UE-sided models in the next sections.

# Support of NW-sided models

The SI studied beam management use cases, where the DL Tx beam is predicted by an AI. To achieve this, the AI/ML model can be located at the UE or in the NW. However, as the DL Tx beam is under control of the network, it is natural to consider NW-sided AI/ML models. These offer the advantage that the prediction result is almost immediately available at the gNB and further, allows the NW to have more insight when a prediction makes sense and when not.

## Enhanced measurement framework

In a recent study on AI/ML for NR air interface [2], the following items have been considered for studying the potential specification impact in relation to NW-sided models:

Data collection: At NW side for NW-side AI/ML model: - Mechanism related to the reporting. - Additional information for content of the reporting. - Reporting overhead reduction. - Signalling/configuration/measurement/report for data collection, e.g., signalling aspects related to assistance information (if supported), Reference signals. Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following approaches have been identified for overhead reduction: - the omission/selection of collected data - the compression of collected data - Note1: For the different purposes of data collection, the overhead reduction mechanisms and corresponding specification impacts may be different. - Note2: Support of any mechanism(s) (if necessary) for each LCM purpose and the potential spec impact (if any) are separate discussions - Note 3: UE complexity and power consumption should be considered Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following reporting signalling for beam-specific aspects maybe applicable: - L1 signalling to report the collected data - Higher-layer signalling to report the collected data - At least not applicable to AI/ML model inference - Note1: Higher layer signalling design is up to RAN2 - Note2: Whether each signalling applicable to each LCM purpose is a separate discussion - Note3: The legacy signalling principle (e.g. RSRP reporting for L1) can be re-used Model inference related: For BM-Case1 and BM-Case2 with a NW-side AI/ML model: - L1 beam reporting enhancement for AI/ML model inference: - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

For the NW-side model for both BM-Case 1 and BM-Case-2, the gNB configures the measurement resources and instructs the UE to provide the measurement results back to the gNB. According to recent evaluations conducted by the companies, enabling efficient and accurate AI/ML model’s performance especially for a spatial-domain DL Tx beam prediction case, might require measuring more beams than in the legacy framework. In case of a larger number of measurements, e.g., a set B of 16 beams, the given approach may contribute to a significant amount of signaling that needs to be reported regularly. Moreover, the gNB can use one or more AI/ML models at the NW-side simultaneously, which constitutes to even higher amount of reporting feedback. The current CSI reporting framework supports reporting for up to 4 beams per report setting. However, to collect training data or to perform performance monitoring, the NW requires measurement results for all beams of the label data, i.e. Set A. Hence, 4 beams do not meet this demand and the maximum number of reported beams needs to be determined based on the Set A configuration.

Proposal 1: Increase the number of reported beams per report setting depending on the Set A configuration.

Given the direction of having an increased number of beams, resulting in a larger number of measurements and reporting overhead, it is obvious that there is a need in a legacy reporting enhancement design for both data collection and inference data. Several methods that can bring payload size reduction to the measurement overhead are:

- feedback compression,

- quantization, e.g., reporting of quantized L1-RSRP or applying different quantization configurations with varying levels of precision,

- partial reporting of data, e.g., reporting of a selection of beams,

- time series compression, e.g., based on temporal sequence of L1-RSRP.

In the following table, we provide a high-level overview of the methods, including their main pros and cons, in relation to the signaling reduction:

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Compression | - Preservation of data integrity (with lossless compression). | - Processing overhead for compression and decompression. - Potential loss of information with lossy compression. |
| Quantization | - Reduction in data size - Lower transmission and storage requirements | - Loss of precision, impacting measurement accuracy and therefore, quality of input and output performance of the AI/ML model - Selection of quantization levels might be challenging. - Multiple quantization configurations might be required for different scenarios (LOS/NLOS, etc.) |
| Partial reporting of data, e.g. subset B | - Reduced data transmission overhead. | - Potential loss of context if crucial information is omitted. - Requires careful definition of conditions for partial reporting. |
| Time series compression, e.g. difference | - Reduced data volume while capturing trends. - Efficient use of resources for trend-based predictions. | - May not perform optimally in situations with rapidly changing patterns. |

Each method offers a trade-off between data reduction and preserving essential information. The choice of which method to use depends on specific use cases, the acceptable level of information loss, and the available computational resources. Implementing a combination of these methods can provide a more robust solution, striking a balance between accuracy and efficiency.

Proposal 2: For network-side AI/ML models, study methods for overhead reduction of L1 measurement reporting.

## Performance monitoring of NW-sided models

In the TR [2], the following potential enhancements for model monitoring of NW-sided models have been identified:

For BM-Case1 and BM-Case2 with a NW-side AI/ML model - Beam measurement and report for model monitoring - UE reporting of beam measurement(s) based on a set of beams indicated by gNB. - Signalling, e.g., RRC-based, L1-based. - Note: This may or may not have specification impact. - NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Note: Performance and UE complexity, power consumption should be considered.

Performance monitoring is required to ensure an adequate performance of AL/ML. However, it comes at an increased overhead. The increased overhead that occurs due to a larger monitoring report transmission can be addressed by the scenario with different monitoring windows configured by the gNB, which is illustrated in Fig.1. The given configurations comprise multiple phases. During the first ‘Monitoring’ phase the UE might experience similar channel conditions over a certain time period that will have almost no impact on the AI/ML model’s performance. In the given phase UE can send less frequent reports to the gNB with a smaller number of measurements or KPIs. In general, the given configuration favors reduced radio resource overhead by avoiding or minimizing reporting slightly changing information. Furthermore, in the monitoring phase the overhead due to reference signals is reduced compared to the validation phase.

However, when required the UE switches to a validation phase where more frequent and more detailed reporting is performed. This phase covers scenarios, when the UE experiences changes in the network conditions that affect the performance of AI/ML model at the gNB. This configuration comes with more detailed and more frequent measurements followed by the report from the UE. The data quality of the transmitted measurements in the given phase can be improved by higher quantization granularity.

Moreover, some measurements can be reported over L1 signaling reduce latency and have early awareness of the AI/ML model failure by the gNB.

Figure 1 Monitoring phases with less measurements, and validation phase with increased number of measurements, e.g., CSI measurements.

Proposal 3: For network-side AI/ML models, support 2-phase monitoring with varying frequencies and reporting detail.

# Support of UE-sided models

In the WID, it is stated that both UE-sided and NW-sided models for beam management shall be supported. In general, UE-sided models offer multiple merits over NW-sided models. In particular, the measurement signaling overhead is significantly reduced. This is due to the UE rather processing the measurements directly instead of transmitting them to the NW. Essentially, this also allows a shorter measurement-to-inference delay as the step of transmitting the measurements to the NW is omitted. Finally, the UE can use more measurements as input to the beam prediction as there is no cost associated to transferring measurements over the air.

## LCM operation for UE-sided models

In the TR [2], the following has been agreed for functionality-based LCM and model-ID-based LCM:

In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE. In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), are studied, where the applicable models may be a subset of all identified models. Applicable models can be reported by the UE. How to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature is to be studied. Note: it does not preclude any existing solutions. For functionality/model-ID based LCM, once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

Essentially, the major difference of the two LCMs is how much control the gNB has over the actual models running on the UE. Functionality LCM supporting model IDs can be considered a flavor of model-ID-based LCM. Regardless of this issue, the major reason for the gNB to maintain control over the models at the UE stems from generalization issues of the AI/ML models. In particular, if models do not perform well under certain conditions, the gNB needs control over which particular model is used at the given time by the UE in the given site, scenario, or area. Thus, at least for BM-Case 1 changing parameters, such as the deployment scenarios and the gNB settings, result in huge performance fluctuations. Hence, the gNB, which is aware of these parameters, wants to keep control over the AI/ML models used at the UE.

Proposal 4: Functionality LCM with model-ID or model-ID LCM should be supported for UE-sided models, at least for BM-Case 1.

In both cases, the UE capabilities report is used as a basis to configure the AI/ML functionality or model. In particular, the UE indicates in the UE capabilities report what conditions it does support. For example, this can be information about the Set A/B, AI/ML model input and output or sites, scenarios, or areas that are supported by the UE.

Proposal 5: The UE capabilities report may indicate its supporting condition, in order to allow configuration of functionality LCM or model-ID LCM.

## Inference operation for UE-sided models

In the TR [2], the following potential enhancements for inference of UE-sided models have been identified:

In order to facilitate the AI/ML model inference: - Enhanced or new configurations/UE reporting/UE measurement, e.g., enhanced or new beam measurement and/or beam reporting - Enhanced or new signalling for measurement configuration/triggering - Signalling of assistance information (if applicable) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception, which may or may not have additional specification impact (e.g., legacy mechanism may be reused), particularly: - how to perform beam indication of beams in Set A not in Set B. Note: also applicable to NW-side AI/ML model. Note: At least for BM-Case1 with a UE-side AI/ML model, the legacy TCI state mechanism can be used to perform beam indication of beams - Note: For DL beam pair prediction, there is no consensus to support the reporting of the predicted Rx beam(s) (e.g., Rx beam ID, Rx beam angle information, etc) from the UE to the network. - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams)

By the nature of the problem, when the inference is performed at the UE-side, the prediction result has to be signaled to the gNB so that it can take the prediction into account. The NR Rel-17 specification already offers a sophisticated CSI reporting framework that supports reporting on a per-beam basis. This framework can be used as a basis for reporting the inference result to the gNB.

The major question to potentially extend the CSI reporting framework is the content of the prediction itself. In the study phase, mainly, models that predict an L1-RSRP have been considered. However, also models that directly output a beam ID can be used or further a combination of both. Depending on the actual output the format of the inference reporting of the UE has to be adapted. Both types of models have their merits and hence, should be considered when designing an inference reporting format.

Proposal 6: For UE-sided models, take the CSI reporting framework as a basis to indicate the predicted beam ID or L1-RSRPs to the gNB.

An additional issue arises due to different processing capabilities of UEs and the complexity of the used models. Depending on these parameters, the time required at the UE to obtain the prediction can vary significantly. Nevertheless, one can apply a worst-case inference time which fits all models. However, this unnecessarily sacrifices the overall performance due to delayed reporting of the inference outcome. Hence, the UE needs be able to report is inference time to the gNB, so that the inference reporting procedure can be adapted to the UE’s performance.

Proposal 7: For UE-sided models, support the UE reporting its inference time to the gNB.

Finally, reporting of confidence intervals or reliability for the inference results have been discussed in the SI. Such a report could potentially improve the final decision taken at the gNB. In particular, the gNB can decide to ignore low reliability predictions, if they contradict other information at the gNB. For example, for BM-Case 2, the gNB might have more detailed information on the direction of UE movement and/or its beam geometry, which do not match the UE prediction. In such a scenario, the gNB can also use some reliability information to decide whether it wants to follow the UE prediction or not. Nevertheless, confidence or reliability metrics are concepts that are hard to grasp. Hence, it has to be studied how they can be incorporated in the specification.

Proposal 8: Examine whether and how to report confidence of prediction.

## Configuration of Set A and model monitoring of UE-sided models

In the TR [2], the following potential enhancements for model monitoring of UE-sided models have been identified:

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

Model monitoring is an extremely crucial aspect for employing AI/ML in the field. While AI models perform very well under theoretical or well-defined testing conditions, their practical performance can also be significantly worse than that. Hence, monitoring and validating the model performance is of uttermost importance.

In Section 2.2, we discussed a 2-phase monitoring approach in which the UE adapts its monitoring depending on the phase. In a monitoring phase, the monitoring requirements can be more relaxed. Thus, the overhead of reference signals are reduced compared to the validation phase. In the validation phase, the UE reports more frequently and possibly more measurements or KPIs to the network. For example, the UE measures all beams of Set A in the validation phase. Thus, the UE can reliably assess the performance of its AI model and further, gather measurements for training of another model.

Proposal 9: For UE-side AI/ML models, support 2-phase monitoring with varying frequencies and reporting detail.

As mentioned earlier, the CSI measurement framework provides a basis for the required evaluation. The UE can be configured to periodically monitor all beams of Set A. This allows the UE to obtain measurements for the model monitoring and at the same time let the UE derive the Set A of beams. For example, such a monitoring configuration is only applied in the validation phase allowing the UE to collect ground truth data for training a new model or evaluating the performance of inactive models.

Proposal 10: Support a model monitoring configuration that defines measurement resources for all Set A beams.

In some cases, the Set A of beams can be the resources for radio link monitoring or resources for radio link quality measurements. Based on the RS resources configured for the two procedures, the RS resources configured for radio link monitoring can be used for model monitoring procedures associated with CSI beam reporting. The linking between the two can be determined based on the beams configured for the two purposes, the TCI-states of the beams and beam prediction methods used. If such linking for model monitoring between the two frameworks is possible, a separate set of configured resources for model monitoring is not necessary.

Proposal 11: Explore the possibility of using the resources configured for radio link monitoring for model monitoring purposes with respect to CSI beam reporting.

Furthermore, such a model monitoring configuration can also be used to collect data, which allows the UE to train a new AI/ML model. In addition, the UE also determines the performance of other inactive models and can use these results to decide whether switching to another beam management model is beneficial or not.

Proposal 12: Support a model monitoring configuration that allows for collecting data for model training and monitoring of inactive models.

## Assistance information for BM-Case 1

In the TR [2], the following parameters regarding assistance information for BM UE-sided models have been identified. However, no consensus has been reached across the companies.

Assistance information: Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement

Although there was no consensus to support explicit assistance information from the network in the study phase, we believe that reporting Tx and Rx beam shape information as well as information on the scenario can improve system performance. Thus, providing assistance information is beneficial for UE-sided models as it offers significant context for the AI/ML models to refine its spatial-domain DL beam predictions. This enhances both performance and accuracy of models. In this context, the assistance information may be used jointly with L1-RSRP measurements as inputs for BM-Case 1.

Assistance information can be configured for the AI/ML model by the NW. By transmitting important parameters once during AI/ML functionality/model configuration, the system will reduce overhead. This technique suffers when the network changes dynamically. However, a dynamic strategy can solve the adaptation problem.

Proposal 13: For UE-side models, support signaling of assistance information from the NW to the UE, at least for BM-Case 1.

## Set B configuration for BM-Case 1

As stated in the Section 3.2, the CSI beam reporting framework can be reused for predicted beam and/or L1-RSRP values. In the case of spatial beam prediction that the measured set B of beams is a subset A of beams. In this case the set A of beams can be configured as the channel measurement resource set, and the set B of beams as a subset of set A. The beams in set A that are not in set B are ‘disabled’, meaning that these beams are not transmitted by the network. The set B of beams are the ‘enabled’ resources in the channel measurement resource set – the beams that are transmitted by the network and are measured by the UE.

Proposal 14: The Set B of beams is provided as a subset of the CSI resource configuration.

## Radio link monitoring

Like CSI beam reporting, radio link monitoring and link recovery procedures also involve the reporting of L1-RSRP values or status messages associated with L1-RSRP measurements to higher layers. Considering the possibility of temporal and spatial AI/ML beam prediction models deployed at the UE side, RS configuration and reporting mechanisms for radio link monitoring or link recovery exploiting such prediction models need to be studied by RAN1.

Proposal 15: Study the support for radio link monitoring and link recovery procedures considering spatial and temporal beam prediction at the UE-side.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: Increase the number of reported beams per report setting depending on the Set A configuration.

Proposal 2: For network-side AI/ML models, study methods for overhead reduction of L1 measurement reporting.

Proposal 3: For network-side AI/ML models, support 2-phase monitoring with varying frequencies and reporting detail.

Proposal 4: Functionality LCM with model-ID or model-ID LCM should be supported for UE-sided models, at least for BM-Case 1.

Proposal 5: The UE capabilities report may indicate its supporting condition, in order to allow configuration of functionality LCM or model-ID LCM.

Proposal 6: For UE-sided models, take the CSI reporting framework as a basis to indicate the predicted beam ID or L1-RSRPs to the gNB.

Proposal 7: For UE-sided models, support the UE reporting its inference time to the gNB.

Proposal 8: Examine whether and how to report confidence of prediction.

Proposal 9: For UE-side AI/ML models, support 2-phase monitoring with varying frequencies and reporting detail.

Proposal 10: Support a model monitoring configuration that defines measurement resources for all Set A beams.

Proposal 11: Explore the possibility of using the resources configured for radio link monitoring for model monitoring purposes with respect to CSI beam reporting.

Proposal 12: Support a model monitoring configuration that allows for collecting data for model training and monitoring of inactive models.

Proposal 13: For UE-side models, support signaling of assistance information from the NW to the UE, at least for BM-Case 1.

Proposal 14: The Set B of beams is provided as a subset of the CSI resource configuration.

Proposal 15: Study the support for radio link monitoring and link recovery procedures considering spatial and temporal beam prediction at the UE-side.

# References

- 3GPP RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG-RAN #102, Edinburg, Scotland, Dec 11th – Dec 15th, 2023.

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, version 18.0.0.