3GPP TSG RAN WG1 #116bis			R1-2402125

Changsha, Hunan Province, China, April 15th – 19th, 2024

Agenda Item:	9.3.2

Source:	InterDigital, Inc.

Title:	Enhancements on SBFD random access operations

Document for:	Discussion and Decision

# Introduction

In Rel-19, a work item on evolution of NR duplex operation for Sub-band full duplex (SBFD) was approved and updated [1], where the objectives identified for the work item are as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Support UE-to-UE CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

In RAN1 #116, the issues on random access operations in SBFD systems were discussed and following agreements were made:

Agreement For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Conclusion No new PRACH format is introduced in Rel-19 duplex WI. Agreement: For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options: - Option 1: Use one single RACH configuration with possible enhancement. - The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE. - FFS: Further details - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration. - The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE. - FFS: Further details Agreement: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH) Conclusion If PRACH e allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following: - The benefits include at least one or more of the following: - reduced random access latency. - reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots. - improved coverage of PRACH with sparse UL resources - increased cell range of PRACH with sparse UL resources - PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario. For future meetings Companies to consider whether the existing random-access configuration tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2) need to be enhanced. Agreement For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols. Agreement For SBFD-aware UEs in RRC CONNECTED state, further study the following two options: - Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols. - a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid. - Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots. RAN1 to leverage the study in Rel-18 as baseline. Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1. For future meetings In RAN1#116bis meeting, at least the following issues will be discussed: - Whether to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode. - Details of the two options for configuring ROs for SBFD aware UEs in RRC CONNECTED mode, including RO validation rules, SSB-RO mapping rules, whether/how to allow SBFD aware UE and non-SBFD aware UE to use different PRACH preamble formats. - Whether/how to support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols. - Whether/how to enhance the existing random access configuration tables for unpaired spectrum. - Whether/how to support PRACH repetition.

In this contribution, we discuss SBFD operation to support random access in SBFD symbols in RRC_CONNECTED and RRC_IDLE/INACTIVE modes and provide initial performance analysis for repetition based PRACH transmissions across SBFD symbols.

# Discussions

## Subband non-overlapping FD operations

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-17, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-18, enhancements to support full duplex (FD) at least for gNB were proposed and endorsed as the study item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on SBFD configurations including DL/UL subbands

## Random access occasions in SBFD systems

Figure 3. Example for available ROs versus valid/invalid ROs for SBFD-aware UEs

The UE in RRC-CONNECTED mode can receive indications to perform random access procedure, for example based on a received PDCCH order from the gNB. The PDCCH order could include the preamble to be used and the SSB index, based on which random access is performed. Also, during initial access, the UE could perform initial access procedure by selecting a preamble and the best SSB detected and measured from a selected cell.

### Random-Access Configurations in SBFD Symbols

In random-access procedure, the UE selects the RACH occasions (RO) to transmit the PRACH from the sets of configured ROs. The UE selects the ROs based on the selected or indicated SSB and according to the association of SSBs and ROs. In NR, the UE determines the time and frequency resources associated to the selected ROs. As for the time resources, the UE determines the ROs in time domain based on a received index (prach-ConfigurationIndex or msgA-PRACHConfigurationIndex) that maps to configuration tables. Table 6.3.3.2-2, Table 6.3.3.2-3, and Table 6.3.3.2-4 in 38.211 provide configurations on RACH configurations in time-domain for NR-FDD FR1, NR-TDD FR1, and NR-TDD FR2, respectively. The tables include indications on the slot number to be used for the configured ROs.

In NR-TDD, the UE considers an RO that coincides in time with UL symbols as valid. In other words, the ROs in DL slots are considered as invalid. However, in SBFD systems, the ROs need to be allocated to the UL subbands in the DL slots. As such, there would be potentially more ROs available for SBFD-aware UEs for random access transmissions. For example, a separate parameter indicating an entry of one of the configuration tables can be additionally configured only for SBFD-aware UE, where the NR-FDD FR1 table can be used as it fits well with the SBFD systems constituting FDD-like subband partitions within a carrier. Alternatively, to simplify the SBFD configurations supporting random access operations, a time-domain offset parameter can be considered to allow for an additional set of ROs that are time-domain shifted versions of the legacy ROs in UL-only slots.

Observation 1. The time resources where random-access preambles can be transmitted are different in systems with SBFD slots compared to legacy TDD systems with UL-only and DL-only slots.

Proposal 1. Decide how to indicate valid ROs that can coincide in SBFD symbols, flexible symbols, or UL-only symbols for SBFD-aware UEs.

In NR RAN1 #116, following agreements were made regarding random-access configurations:

Agreement: For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options: - Option 1: Use one single RACH configuration with possible enhancement. - The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE. - FFS: Further details - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration. - The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE. - FFS: Further details For future meetings Companies to consider whether the existing random-access configuration tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2) need to be enhanced.

In the agreement, two options are provided, where Option 1 is based on using a single RACH config for both SBFD and legacy UEs. That is, the SBFD and legacy UEs determine the valid ROs based on conditions or criteria that may be specific to SBFD and legacy UEs, respectively. In Option 2, two separate RACH configurations are considered, where the first RACH configuration indicates the ROs in UL symbols for legacy UEs and the second RACH configuration indicates the ROs in UL subbands in SBFD symbols for SBFD UEs.

In our opinion, both Option 1 and Option 2 can be used as long as NW indicates the option that is used in the cell, to the UEs. In an example, in NW with higher and more complex traffic and scheduling demands, NW could use Option 2 to configure two separate sets of RACH configurations for SBFD and legacy UEs to make the traffic management and scheduling manageable and more flexible at the NW-side. Alternatively, in NW with lower traffic or scheduling demands, NW could use Option 1 with a single RACH configuration, where UEs determine the validity of configured ROs in SBFD or UL-only symbols based on configured conditions or criteria.

Observation 2. In RACH configurations for SBFD UEs, NW can achieve benefits from both Option 1 (single config) and Option 2 (two separate config), based on the NW’s scheduling, flexibility, and traffic requirements.

Proposal 2. Support both Option 1 (single config) and Option 2 (two separate config), for RACH configurations for SBFD-aware UEs.

In case of Option 1 with single RACH config, the legacy UEs only consider the ROs that are in UL-only symbols as valid ROs. However, SBFD-aware UEs can consider the configured ROs in UL-only symbols as valid non-SBFD ROs that is similar to ROs used by legacy UEs. Moreover, the SBFD-aware UEs consider the ROs that are in SBFD symbols as valid SBFD ROs.

In case of Option 2 with separate RACH config, the legacy UEs are configured with only the first RACH config and consider the ROs being configured based on the first RACH config and that are in UL-only symbols as valid ROs. However, SBFD-aware UEs consider the ROs configured based on the first RACH config as valid non-SBFD ROs, and the ROs configured based on second RACH config as valid SBFD ROs.

Observation 3. SBFD-aware UEs can use the RACH config (based on either Option 1 or Option 2) for determining valid SBFD and non-SBFD ROs, where the non-SBFD ROs may be the same as legacy ROs.

While the SBFD-aware UE could potentially have more available ROs, there may be limitations from gNB on which available ROs can be used by the UE and which ROs are valid or invalid. One straightforward observation is that random access preamble transmission is not possible in DL-only slots in the legacy system. However, the UE may need new indications or guidance on whether to use SBFD symbols, flexible symbols, or UL-only symbols for RACH transmissions. As an example, see Figure 3 where eight RO slots (ROS) are considered as available ROs that can be configured to the SBFD-aware UE. Obviously, ROS1 and ROS2 are invalid ROs for SBFD-aware UEs as they overlap with DL-only slots, and this can be identified by the UE as the UE can receive SBFD time/frequency subband location information as well. Then, the UE can determine that RO slots ROS3, ROS4, …, ROS8 are potentially valid for random access preamble transmission. Furthermore, it can be considered that, based on NW configurations or preferences with a restriction signalling, the UE may not be allowed to use some of these potentially valid RO slots. For example, NW could limit the UE to only use ROs that are in SBFD slots, due to scheduling and traffic handling considerations.

In NR RAN1 #116, following agreement was made regarding valid ROs for SBFD-aware UEs:

Agreement For SBFD-aware UEs in RRC CONNECTED state, further study the following two options: - Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols. - a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid. - Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots. RAN1 to leverage the study in Rel-18 as baseline.

In this agreement, two options are provided, where Option 1 is based on limiting SBFD-aware UEs to only use either valid SBFD ROs or valid non-SBFD ROs. Option 2 is based on allowing SBFD-aware UEs to use both of SBFD and non-SBFD ROs, even in the same slot or across slots.

In Option 1, separating may result in lower opportunities for SBFD-aware UEs to send PRACH and limiting ROs across non-SBFD (UL-only) symbols only to the legacy UEs. This is definitely not in line with SBFD objective and the limitation could increase the latency and degrade the performance for SBFD UEs. On the other hand, Option 2 allows SBFD-aware UEs to use both SBFD and non-SBFD ROs that allows reducing the latency, increasing UL capacity, and coverage for SBFD-aware UEs.

Observation 4. The UE may need new indications on whether configured ROs in SBFD symbols, flexible symbols, or UL-only symbols are valid to be used for random access preamble transmission.

Proposal 3. For SBFD-aware UEs in RRC CONNECTED state, support Option 2, where a valid RO can be configured across SBFD and non-SBFD symbols in the same slot or across slots.

### PRACH Power Control in SBFD Symbols

In NR RAN1 #116, following agreements were made regarding PRACH power control in SBFD symbols:

For future meetings In RAN1#116bis meeting, at least the following issues will be discussed: - Whether/how to support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols.

In NR, PRACH power control parameters are calculated based on configured parameters such as power ramping step, maximum number of PRACH preamble transmission (e.g., TransMax), etc. Considering the potential CLI caused by PRACH transmission with increased PRACH power due to power ramping, the SBFD-aware UEs may be configured with different PRACH power control parameters for SBFD and non-SBFD ROs.

The SBFD-aware UEs may also be configured with different PRACH power control parameters for (FDM-ed) SBFD ROs that are closer or farther to the DL subband edges.

Observation 5. Considering the potential CLI caused by PRACH transmission with increased PRACH power due to power ramping, the SBFD-aware UEs may be configured with different PRACH power control parameters for SBFD and non-SBFD ROs.

Proposal 4. Support configuring different PRACH power control parameters for SBFD and non-SBFD ROs, also for FDM-ed SBFD ROs that are closer or farther from DL subband edges, in consideration of potential CLI caused by PRACH transmission.

### RACH Repetition Configurations in SBFD Symbols

In NR RAN1 #116, following agreements were made regarding RACH repetition in SBFD symbols:

Agreement For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

RACH repetition was added in Rel. 18 for increasing the Random-Access coverage. Enhancing the UL coverage is one of the objectives in SBFD as well. So, NW could increase the benefits of PRACH repetition by supporting PRACH repetition in SBFD ROs.

Moreover, in cases where legacy (non-SBFD) ROs do not provide adequate RO sets for X-rep repetition, using SBFD along with non-SBFD ROs could allow applying higher X-reps.

Observation 6. Supporting PRACH repetition across SBFD symbols and non-SBFDs symbols could enable increased coverage for random access, as well as higher X-rep repetitions.

Proposal 5. Support PRACH repetition across SBFD symbols and non-SBFDs symbols.

### 4-Step and 2-Step Random-Access in SBFD Symbols

In NR RAN1 #116, following agreements were made regarding Type-1/2 random access procedure in SBFD symbols:

Agreement: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

Based on the agreement, 4-step RACH is supported in SBFD ROs, and 2-step RACH is left for FFS. Supporting 2-step RACH could result in reducing the latency. For example, in systems with small data transmission, 2-step RACH can improve the performance with lower latency. Considering the objectives of SBFD systems in reducing the latency, it is worth to consider 2-step RACH in SBFD symbols.

In our opinion, in case the measured RSRP of the selected SSB is higher than corresponding MsgA-RSRP threshold, 2-step RACH could be also supported in SBFD ROs.

In case of SBFD ROs, in addition to RSRP, the CLI could also be used as the condition to apply 2-step RACH. That is, if the measured RSRP is higher than corresponding threshold and measured CLI is lower than corresponding threshold, then 2-step RACH could be used in SBFD ROs.

Proposal 6. Support 2-step RACH in SBFD ROs.

### Indication of support for SBFD

In NR RAN1 #116, the following agreement was made, and it was left for FFS on how gNB identifies whether a UE is SBFD aware UE or non-SBFD aware UE:

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

Moreover, the SBFD-aware UE in RRC-IDLE/INACTIVE/CONNECTED modes can decide on whether to connect to the selected cell based on SBFD operation or connect to the selected cell based on non-SBFD operation, during initial access. For example, in case the measured CLI (based on received SSB) is low, the UE could connect to the cell based on SBFD operation. Otherwise, if the measured CLI (based on received SSB) is high, the UE could connect to the cell based on non-SBFD operation.

The SBFD-aware UE that has determined to connect to a cell that supports SBFD and has decided to operate based on SBFD operation can implicitly indicate such decision as part of random-access procedure. For example, the UE can select a preamble from a set that corresponds to SBFD operation. In another example, the UE can transmit random access preamble in ROs that coincide with SBFD symbols. As such, upon reception of the preamble, the gNB will recognize that the UE is SBFD-aware, and that the UE is operating based on SBFD operation. So, gNB can further transmit RAR in SBFD symbols.

The SBFD-aware UE that has decided to perform RA based on non-SBFD ROs, could indicate its support for SBFD operation later and as part of Msg3.

Observation 7. During random access procedure, the SBFD-aware UE can indicate its capability and support for SBFD operation based on transmitted random-access preamble, or ROs used for RA preamble transmission.

Proposal 7. Support SBFD-aware UE indicating its support of SBFD operation as part of random-access, for example, via

- SBFD-specific preamble selection, or

- PRACH transmission in SBFD ROs, or

- Indication as part of Msg3.

## Random Access Operation in RRC-IDLE/INACTIVE mode

In NR RAN1 #116, following conclusion was made regarding random-access operations in SBFD symbols in RRC_IDLE/INACTIVE modes:

Conclusion If PRACH is allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following: - The benefits include at least one or more of the following: - reduced random access latency. - reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots. - improved coverage of PRACH with sparse UL resources - increased cell range of PRACH with sparse UL resources - PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario.

In our opinion, performing random-access procedure in SBFD symbols for UEs in RRC_IDLE/INACTIVE can have a huge impact in reducing the latency and preventing undesired further cell switching for UEs in RRC_CONNECTED mode. Consider a UE that connects to a cell based on non-SBFD ROs, without knowing about cell’s SBFD configurations, CLI, etc. Then when the UE gets connected to the cell and determines to operate based on SBFD operation, the SBFD-aware UE may need to switch the cell due to low RSRP, CLI, or other SBFD-specific limitations.

On the other hand, the UE can indicate its intention to operate based on SBFD operation via random access (as described in 2.2.5), and even report the measured CLI (e.g., via Msg3). In this case, the UE can receive a confirmation or rejection from gNB on whether the UE can connect to the cell based on SBFD operation. The received confirmation or rejection can also affect UE’s decision to connect to that cell in the early steps rather than later on, when UE is in RRC_CONNECTED mode, requiring undesired cell switching or HO with increased power consumption and complexity.

Proposal 8. Support random access in SBFD symbols for UEs in RRC-IDLE/INACTIVE mode.

An SBFD-aware UE in RRC-IDLE/INACTIVE mode that determines to select a cell for connecting to the NW can prioritize cells with SBFD operation over cells without SBFD operation. The UE could receive information on the neighbour cells’ support for SBFD operation based on stored cell information or as part of SIB associated with the detected SSB from the corresponding cell. The cell information on SBFD could include whether the cell supports SBFD as well as time and frequency resources where SBFD is applied.

Observation 8. During cell selection or initial access, the SBFD-aware UE in RRC-IDLE/INACTIE mode could check cells’ information on supporting SBFD operation, so that UE could prioritize cells with SBFD operation.

Proposal 9. In cell selection or initial access, support indicating cells’ support on SBFD operation (e.g., in SIB1) to be used for cell ranking by SBFD-aware UEs.

In SI phase, it was agreed that SSB could be transmitted in SBFD symbols in cells that support SBFD operation. As such, the UEs could have a measurement or estimate on the received power (e.g., RSRP) and potential CLI that they could expect after switching to RRC-CONNECTED Mode in the corresponding cell. The UE may then decide if the measured received power or CLI are within acceptable respective ranges and whether the UE can camp on or connect to the corresponding cell. This will avoid latency and ping-ponging effects. That is, in case the UE connects to the cell blindly and without considering limitations (e.g., potential CLI or lower RSRP) in cells with SBFD operation, the UE may need to switch cells after changing to RRC-Connected mode that results in latency and undesired complexity.

Observation 9. During initial access and cell selection, the SBFD-aware UE can measure the received power and potential CLI based on SSBs that are transmitted in SBFD symbols.

Observation 10. During cell selection, the SBFD-aware UE can determine to connect to the selected cell based on SBFD or non-SBFD operation according to the measured received power and potential CLI. This prevents latency and ping-ponging affects.

Proposal 10. Support prioritization rules for selecting the cells that support SBFD operation based on RSRP or potential CLI, during initial access or cell selection procedures for SBFD-aware UEs in RRC-IDLE/INACTIVE modes.

## Initial performance analysis for repetition based PRACH transmissions across SBFD symbols

In this section, we provide preliminary link level simulation performance results for repetition based PRACH transmissions in SBFD symbols relative to legacy TDD. The simulation assumptions corresponding to the performance results presented are provided in Appendix.

In the performance results provided, interference was modelled as additive white Gaussian noise with the inter-site gNB-gNB co-channel inter-subband CLI assumed to be the dominant interference. Furthermore, the interference power was set to a fixed -7dB. The performance results aim to present the RACH detection performance for SBFD using Type A PUSCH repetition over adjacent slots relative to the baseline TDD scheme where RACH is transmitted in UL slots only. Furthermore, both contention-free and contention-based random access were considered to evaluate the link level performance. For contention-free random access, since UE is transmitting dedicated preambles, the receiver at the gNB can combine the received waveforms during each repeated occasions before applying the detection of the RACH. The advantage in such approach for the contention free random access comes from the incremental gain in the received SNR as the gNB receives the repeated RACH occasions before applying detection. For contention based random access, the gNB tries to detect each of the repeated RACH occasions such that the network will only have to detect the preamble in at least one of the RACH occasions. Therefore, the probability of detection of a repeated RACH occasion will increase by the number of repeated RACH occasions.

Figure 4. Probability of detection of random access preamble for legacy TDD vs SBFD with type A UL repetition

Figure 4 shows the RACH detection performance in terms of the probability of detection as a function of SNR for both legacy TDD system and for SBFD system employing type A like RACH repetition. The co-channel inter-subband CLI for the SBFD system is modelled as AWGN with a variance of -7dB. It can be seen from Figure 4 that random access occasions on SBFD system clearly provides performance improvement over legacy TDD by increasing the probability of detection. For contention free RACH occasions, the improvement in probability of detection over legacy TDD system can be as high as 7dB in SNR in SBFD systems with repetitions of five RACH occasions while for contention-based RACH occasions, the improvement in probability of detection can be up to 3dB.

Observation 11. PRACH preamble repetition in SBFD symbols enhances RACH detection performance in both contention-based and contention-free RACH occasions, where the RACH detection performance improves as the number of PRACH repetition occasions increases.

Proposal 11. Support PRACH repetition in SBFD symbols, for both contention-based and contention-free RACH occasions.

# Conclusion

In this contribution, we discussed issues on random access procedures in SBFD systems. From the discussions, we made following observations and proposals:

[2.2.1 Random-Access Configurations in SBFD Symbols]

Observation 1. The time resources where random-access preambles can be transmitted are different in systems with SBFD slots compared to legacy TDD systems with UL-only and DL-only slots.

Observation 2. In RACH configurations for SBFD UEs, NW can achieve benefits from both Option 1 (single config) and Option 2 (two separate config), based on the NW’s scheduling, flexibility, and traffic requirements.

Observation 3. SBFD-aware UEs can use the RACH config (based on either Option 1 or Option 2) for determining valid SBFD and non-SBFD ROs, where the non-SBFD ROs may be the same as legacy ROs.

Observation 4. The UE may need new indications on whether configured ROs in SBFD symbols, flexible symbols, or UL-only symbols are valid to be used for random access preamble transmission.

Proposal 1. Decide how to indicate valid ROs that can coincide in SBFD symbols, flexible symbols, or UL-only symbols for SBFD-aware UEs.

Proposal 2. Support both Option 1 (single config) and Option 2 (two separate config), for RACH configurations for SBFD-aware UEs.

Proposal 3. For SBFD-aware UEs in RRC CONNECTED state, support Option 2, where a valid RO can be configured across SBFD and non-SBFD symbols in the same slot or across slots.

[2.2.2 PRACH Power Control in SBFD Symbols]

Observation 5. Considering the potential CLI caused by PRACH transmission with increased PRACH power due to power ramping, the SBFD-aware UEs may be configured with different PRACH power control parameters for SBFD and non-SBFD ROs.

Proposal 4. Support configuring different PRACH power control parameters for SBFD and non-SBFD ROs, also for FDM-ed SBFD ROs that are closer or farther from DL subband edges, in consideration of potential CLI caused by PRACH transmission.

[2.2.3 RACH Repetition Configurations in SBFD Symbols]

Observation 6. Supporting PRACH repetition across SBFD symbols and non-SBFDs symbols could enable increased coverage for random access, as well as higher X-rep repetitions.

Proposal 5. Support PRACH repetition across SBFD symbols and non-SBFDs symbols.

[2.2.4 4-Step and 2-Step Random-Access in SBFD Symbols]

Proposal 6. Support 2-step RACH in SBFD ROs.

[2.2.5 Indication of support for SBFD]

Observation 7. During random access procedure, the SBFD-aware UE can indicate its capability and support for SBFD operation based on transmitted random-access preamble, or ROs used for RA preamble transmission.

Proposal 7. Support SBFD-aware UE indicating its support of SBFD operation as part of random-access, for example, via

- SBFD-specific preamble selection, or

- PRACH transmission in SBFD ROs, or

- Indication as part of Msg3.

[2.3 Random Access Operation in RRC-IDLE/INACTIVE mode]

Observation 8. During cell selection or initial access, the SBFD-aware UE in RRC-IDLE/INACTIE mode could check cells’ information on supporting SBFD operation, so that UE could prioritize cells with SBFD operation.

Observation 9. During initial access and cell selection, the SBFD-aware UE can measure the received power and potential CLI based on SSBs that are transmitted in SBFD symbols.

Observation 10. During cell selection, the SBFD-aware UE can determine to connect to the selected cell based on SBFD or non-SBFD operation according to the measured received power and potential CLI. This prevents latency and ping-ponging affects.

Proposal 8. Support random access in SBFD symbols for UEs in RRC-IDLE/INACTIVE mode.

Proposal 9. In cell selection or initial access, support indicating cells’ support on SBFD operation (e.g., in SIB1) to be used for cell ranking by SBFD-aware UEs.

Proposal 10. Support prioritization rules for selecting the cells that support SBFD operation based on RSRP or potential CLI, during initial access or cell selection procedures for SBFD-aware UEs in RRC-IDLE/INACTIVE modes.

[2.4 Initial performance analysis for repetition based PRACH transmissions across SBFD symbols]

Observation 11. PRACH preamble repetition in SBFD symbols enhances RACH detection performance in both contention-based and contention-free RACH occasions, where the RACH detection performance improves as the number of PRACH repetition occasions increases.

Proposal 11. Support PRACH repetition in SBFD symbols, for both contention-based and contention-free RACH occasions.

# References

RP-240789, “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

“Report of 3GPP TSG RAN WG1 #116”, 3GPP RAN1 Meeting #116, February 26 - March 1, 2024

# Appendix – LLS assumptions for repetition based PRACH transmissions

| Parameter | Value |
| --- | --- |
| Scenario and frequency range | FR1, 4GHz carrier frequency |
| Subcarrier spacing | 30 kHz |
| Frame structure | - Baseline (TDD): DDDSU, where S=[12D:2G:0U]/S=[10D:2G:2U] - SBFD: XXXXU, DXXXU, DDDXU (X: SBFD slot), with type-A repetition |
| Channel model for link-level simulation | TDL-C |
| Delay spread | 300ns |
| UE velocity | 3km/h |
| Number of Tx/Rx chains | FR-1: 1Tx (UE) and 4Rx (gNB) |
| CLI models | -7dB AWGN |
| Frequency hopping | Disabled |
| Timing error, frequency offset | 0, 0 |