3GPP TSG RAN WG1 Meeting #116-bis		R1-2402840

Changsha, Hunan Province, China, April 15th – 19th, 2024

Agenda item:       9.2.3

Source: 	Nokia

Title: 	On the support for 3-antenna-port codebook based transmissions

Document for:     Discussion and Decision

# 1	Introduction

In RAN#102 one objective was agreed for 3Tx UL MIMO enhancements in Rel-19 WID [1]. The specific objective in the WID RP-234007 is:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported.

The last RAN1 meeting have reached many agreements [2], including the 3Tx non-coherent codebook. EVM for LLS and SLS are also agreed for 3Tx performance evaluation. PT-RS and SRS are discussed in the last RAN1 meeting and there are still open issues remaining. This contribution is addressing these open issues with our proposals and some observations.

# 2	Non-coherent 3Tx codebook and performance

The 3Tx non-coherent codebook is agreed in the last RAN1 meeting with a total of 7 precoders, as listed below:

Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission. 13100,13010,13001 Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission. 13100100, 13100001, 13 001001 Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission. 13100010001

The EVM for SLS is also agreed during the RAN1 discussion:

Agreement For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation. - Companies may provide additional evaluation results per their case of interest. Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 CP-OFDM symbol slot SCS , 30 KHz. Scenario; eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz. UE Outdoor/Indoor (%); eMBB: 80%, 20% Outdoor FWA: 100%, 0%. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°]. gNB antenna radiation pattern parameters; - Outdoor/Indoor Per 38.901, Table 7.3-1. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; - Up to 64 QAM - Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE antenna config; eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi. Traffic model; - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional). Suggested benchmarking; Rel-15 2Tx non-coherent. Precoder granularity; Wideband. Power control; Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0. Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT.

Together with the EVM, the 3Tx antenna configuration is also agreed, as:

Agreement For performance evaluation of 3TX UE, consider following reference configurations, - A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ, - For example: ¦←0.5λ→¦←0.5λ→¦ - A configuration of a cross-polarized and a single-polarized antennas, - For example: ×←0.5λ → ⁄

One motivation of using 3Tx over the existing 2Tx and 4Tx would be that the 3Tx can be considered as an “enhancement” over 2Tx UE implementation. The question is how much benefit can be achieved from using 3Tx vs 2Tx, given that there were few performance studies for 3Tx uplink. To answer this question, we studied the 3Tx system-level performance, where we used the SLS simulation assumptions used for Rel-18 UL 8Tx as a starting point. The simulation assumptions are listed below, largely following the eMBB portion of the agreed SLS EVM.

Table 1    System level simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS , 30 KHz |
| Scenario | UMa (ISD = 200 m), 80% Indoor |
| Channel model | 38.901 |
| System bandwidth | 20 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| gNB antenna radiation pattern parameters | 38.901 Table 7.3-1, 8 dBi , 65° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation: For 2TX UEs: max rank=2. For 3TX UEs: max rank = 3 |
| UE speed | 3 Km/h |
| UE TX antenna configuration | 2TX UEs: UEx: two x-pol antennas UEvv: two vertically polarized elements 3TX UEs: UExv: x-pol plus a vertically polarized element; UEvvv: three vertically polarized elements |
| Traffic model | Full buffer |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -80 dBm |
| UE power rating | 23 dBm (UE, 38.101): Full power mode-0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

The detailed performance results are captured in this table and in the subsequent two figures:

Table 2    System level simulation results

|  | Sector SE | Edge UE SE | Mean Rank |
| --- | --- | --- | --- |
| 2TX: comparing coherent vs non-coherent |  |  |  |
| Gain of coherent 2TX over non-coherent 2TX: (vv) | 13.8% | 9.2% | 0.1% |
| Gain of coherent 2TX over non-coherent 2TX: (x) | 2.0% | -3.6% | -1.2% |
| 2TX comparing (x) vs (vv) |  |  |  |
| Coherent 2TX: gain of (x) over (vv): | 8.7% | 4.8% | 11.3% |
| Noncoherent 2TX: gain from (x) over (vv): | 21% | 19% | 13% |
| Noncoherent: comparing 3TX with 2TX |  |  |  |
| Gain of noncoherent 3TX (xv) over noncoherent 2TX (x): | 6.7% | 4.0% | 35.6% |
| Gain of noncoherent 3TX (x/) over noncoherent 2TX (x): | 8.8% | 9.0% | 37.4% |
| Gain of noncoherent 3TX (vvv) over noncoherent 2TX (vv): | 7.2% | -0.9% | 29.1% |
| Comparing non-coherent 3TX with coherent 2TX |  |  |  |
| gain of noncoherent 3TX (xv) over coherent 2TX (x): | 4.6% | 7.9% | 37.3% |
| gain of noncoherent 3TX (x/) over coherent 2TX (x): | 6.7% | 13.0% | 39.1% |
| gain of non-coherent 3TX (x/) over coherent 2TX (vv): | 15.9% | 18.4% | 54.7% |
| gain of noncoherent 3TX (vvv) over coherent 2TX (vv): | -5.8% | -9.2% | 28.9% |

Figure 1: Relative mean UE spectral efficiency and Edge UE spectral efficiency: full buffer traffic.  Comparing 2TX with 3TX.

Figure 2: Mean scheduled rank per UE: full buffer traffic.  Comparing 2TX with 3TX.

Comparing coherent CBs and non-coherent CBs: the results are dependent on the UE configuration, but some trends can be seen from the cases that were run so far.  For Section SE, these are our observations:

Observation 1: Gain of 3TX non-coherent over 2TX non-coherent is less than 10%.

Observation 2: Gain of 3TX non-coherent over 2TX coherent:  ranges between a 6% loss to an 18% gain depending on the UE antenna configuration.

Note that the performance was evaluated with full power mode 0 for all UE cases. The full power mode 0 for 3Tx is supported following the RAN1 agreement in RAN1 #116.

# 3	PT-RS for 3Tx

There are three agreements related to 3Tx PTRS design:

Agreement For a 3TX UE, down-select one of the following options for the number of PTRS ports, - Option-1: A single PTRS port is supported. - Option- 2: Up to 2 PTRS port may be configured. Agreement For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association. Agreement For codebook-based transmission by a 3TX UE, - Only PUSCH antenna ports 1000, 1001, 1002 are used - Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig, - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication. Above is only for single panel transmission.

The FFS point within the agreement is directly related to 2 PTRS port configuration with the related UE capability. The issue of using either 1 bit or 2 bits for PTRS-DMRS association is only applicable for 2 PTRS port configuration.

The related PT-RS procedures is defined in 38.214:

For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1, 0_2 or 0_3 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - for PUSCH transmission with 2 or 4 ports, PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1, 0_2 and 0_3 described in Clause 7.3.1 of [5, TS38.212].

The related PTR-DMRS association field of DCI is defined in 38.212:

- PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and equals "10" and "11" and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

Based on the existing 38.214 and 38.212 spec, 2-bits are needed for PTRS-DMRS association. During the RAN1 discussion in the last meeting, single one bit was proposed, indicating that the 2nd bit is not needed because the 2nd PT-RS port will have only one layer. However, there is no need to change the legacy definition of using 2-bit for PTRS-DMRS association for 2Tx and 4Tx operation.

Proposal 1: For 3Tx uplink, the PTRS-DMRS association indication shall use 2-bit.

# 4	SRS for 3Tx

For 3Tx SRS operations, there are three agreements in RAN1 #116 meeting:

Agreement For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of - Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted - Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set, The value for X is FFS, and it will be determined according to the selected alternative. Agreement For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH. Agreement For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

From these agreements, there are several options to consider for configuring SRS resources, all of which have various advantages and disadvantages.  For the first agreement above, Alt 1 proposes the idea of supporting one or more 4-port SRS resource with one port being muted, where we assume that if more than one SRS resource is used, then SRI feedback will be necessary, but the TPMI would be applicable to the SRS resource selected by the SRI.  For Alt 2, the proposal is to support the configuration of multiple SRS resources with equal and/or unequal numbers of ports per resource.  The (1+1+1) option in Alt 2 involves the configuration of three single-port SRS resources and the three resources are transmitted in a TDM fashion, where all three single-port SRS resources can be transmitted with full UE TX power (because there would be no splitting of the total TX power across the multiple UE SRS ports within a single OFDM symbol).  The (2+1) option in Alt 2 involves the configuration of a single two-port SRS resource and a single one-port SRS resource, where the two-port SRS resource shares the total UE TX power, but the one-port SRS resource would need to be transmitted at half power if it is to have the same TX power as the SRS ports in the two-port SRS resource.  An advantage of the Alt 2 options is that no SRS port muting is needed, which means no SRS resources are wasted.  A possible advantage of using the 2+1 option over the 1+1+1 option is the use of 2 symbols rather than 3 symbols, plus an even number of symbols is probably easier to manage.

Based on the discussion of Alt 1 and Alt 2, it is possible to support both alternatives, subject to UE capability. With the indication of UE capability, gNB can configure the 3Tx SRS resource accordingly.

Proposal 2: Support 3Tx SRS operation with configurations of 4-port SRS resource, or 2-port/1-port SRS resource, subject to UE capability.

Despite the advantages described above for Alt 2, using multiple SRS resources in Alt 2 rather than using a single SRS resource poses two difficulties given certain existing assumptions present in the NR MIMO framework:

Problem 1: Currently in NR, SRS power control is setup on a per SRS resource basis.  Therefore, Alt 2 then requires multiple SRS resources, which according to the current NR specs would require multiple SRS power control settings (one per SRS resource), which would be inefficient since the same settings would likely be used across all resources being used for UL3TX operation.

Problem 2: Also, PMI (DL) and TPMI (UL) is assumed to be applicable to the ports within one resource (CSI-RS for DL, SRS for UL) and not applicable to ports that belong to multiple resources.  Generally in NR, the channels measured from ports belonging to different resources (CSI-RS or SRS) are not assumed to be phase coherent, which means that the channel measured on ports that span multiple CSI-RS or SRS resources cannot be assumed to stay coherent between the time of the measurement and the time of the data transmission (which means the PMI (DL) or TPMI (UL) would be unreliable).  The options in Alt 2 go against this basic assumption in NR.

One possible solution to Problem 1 and Problem 2 is to configure an SRS resource set containing two or more SRS resources (this covers the 2+1 or 1+1+1 options), subject to UE capability. With this configuration, the SRS power control setting can be considered for the first SRS resource in the configured resource set. At the UE side, the UE having the UL 3Tx capability will assume the power control settings configured for the first SRS resource apply to all ports that belong to all SRS resources contained in the SRS resource set configured for that UE.  And the UE will transmit SRS on the ports belonging to all the resources within the SRS resource set according to the power control settings configured for the first SRS resource.

Proposal 3: For Alt 2, one SRS resource set containing two or more SRS resources can be configured to 3Tx SRS operation, subject to UE capability.

From this proposal, the SRS power control and TPMI applicability, as indicated in Problem 1 and Problem 2, shall be based on the SRS resource set, instead of legacy operation that is based on one SRS resource. There are two options to address this issue:

Option 1: With an indicator: The configuration for the 3Tx SRS resource set shall contain an indicator, which indicates that the ports across all of the ports of the resources within the SRS resource set are to be treated as if they belong to one resource for the purpose of SRS power control and TPMI applicability.

Option 2: Without an indicator: if the number of ports across all resources with the 3Tx resource set is equal to 3, the UE shall assume all the ports across all of the ports of the resources within the SRS resource set are to be treated as if they belong to one resource for the purpose of SRS power control and TPMI applicability.  If the number of ports across all resources within the resource set is not equal to 3, then the legacy assumption on per-resource SRS power control and TPMI applicability are to be used by the UE.

Another solution is to define one 3Tx SRS resource, as a combination of two or more 1Tx or 2Tx SRS resources. The legacy assumption on per-resource SRS power control and TPMI applicability are to be used by the UE. If the total number of ports of SRS resource is equal to three, then the legacy SRS power control configuration will be assumed by the UE to be applicable to all the 1Tx and/or 2Tx ports belonging to all the combined resources of the 3-port SRS resource. Otherwise, the legacy assumption on per-resource SRS power control and TPMI applicability are to be used by the UE. If the total number of ports of SRS resource is equal to three, then the UE will assume the TPMI sent from the gNB will correspond to the ports that are applicable to 1Tx and/or 2Tx SRS ports belonging to the combining resources of the 3-port SRS resource.

Proposal 4: For Alt 2, one 3Tx SRS resource can be defined as a combination of two or more 1Tx/2Tx SRS resources.

# 5	Conclusions

In our contribution, system level simulation results are presented following the agreed SLS EVM. The remaining issues on PT-RS and SRS are discussed with our proposals. Here is the list of our observations and proposals.

For system level simulations, we have these observations:

Observation 1: Gain of 3TX non-coherent over 2TX non-coherent is less than 10%.

Observation 2: Gain of 3TX non-coherent over 2TX coherent:  ranges between a 6% loss to an 18% gain depending on the UE antenna configuration.

On PTRS-DMRS association indication, we have:

Proposal 1: For 3Tx uplink, the PTRS-DMRS association indication shall use 2-bit.

On 3Tx port SRS, we have:

Proposal 2: Support 3Tx SRS operation with configurations of 4-port SRS resource, or 2-port/1-port SRS resource, subject to UE capability.

Proposal 3: For Alt 2, one SRS resource set containing two or more SRS resources can be configured to 3Tx SRS operation, subject to UE capability.

Proposal 4: For Alt 2, one 3Tx SRS resource can be defined as a combination of two or more 1Tx/2Tx SRS resources.

# References

- RP-234007, “New WID: NR MIMO Phase 5,” RAN#102, Edinburgh, Scotland, Dec 11-15, 2023

- RAN1 #116 meeting Chair’s note, Athens, Greece, Feb 26 – March 1, 2024

# RAN1 Previous Agreements

- RAN1 #116 agreements

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of

- Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

- Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set,

The value for X is FFS, and it will be determined according to the selected alternative.

Agreement

For a 3TX UE, down-select one of the following options for the number of PTRS ports,

- Option-1: A single PTRS port is supported.

- Option- 2: Up to 2 PTRS port may be configured.

Agreement

For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association.

Agreement

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

Agreement

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Conclusion

There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 3Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% Outdoor FWA: 100%, 0% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°] |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | Rel-15 2Tx non-coherent |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreement

For performance evaluation of 3TX UE, consider following reference configurations,

- A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ,

  - For example:		 |←0.5λ→|←0.5λ→|

- A configuration of a cross-polarized and a single-polarized antennas,

  - For example:		 ×←0.5λ → ⁄

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

Agreement

For codebook-based transmission by a 3TX UE,

- Only PUSCH antenna ports 1000, 1001, 1002 are used

- Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig,

  - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication.

Above is only for single panel transmission.